3GPP TSG RAN WG1 #100-e	R1-2000766

e-Meeting, February 24th – March 6th, 2020

Agenda Item:	7.2.4.5

Source: 	NEC

Title:	Remaining issues on physical layer procedure

Document for:	Discussion/Decision

- Introduction

RAN plenary had discussion the task RAN1 needs to focus on in RAN1#100 for 5G V2X. RP-193198 captures the discussion outcome. For sidelink physical layer procedure, the remaining issues are:

| B1 | Details of “when to prioritize which transmission” in case of simultaneous transmission of UL and SL across difference carriers |
| --- | --- |
| B2 | Details on PSFCH candidate resource determination |
| B3 | Whether or not to introduce restriction on the size of group in groupcast HARQ feedback option 2 |
| B4 | Details of calculating TX-RX distance for groupcast HARQ feedback option 1 |
| B5 | Details of transmission power of PSCCH |
| B6 | How to derive reference PSSCH DMRS power for SL pathloss estimation |
| B7 | Details of sidelink CSI measurement |
| B8 | Contents of 2nd SCI format, including down-selection of the following options: - Option 1: The same 2nd stage SCI format is used for groupcast HARQ feedback option 1 and option 2. - SCI indicator to indicate between groupcast Option 1 and groupcast Option 2 is in the 2nd-stage SCI. - Option 2: Different 2nd stage SCI formats are used in groupcast HARQ feedback option 1 and option 2. - 1st stage SCI indicates which format is used. |

In this contribution, we share our views on B3: whether or not to introduce restriction on the size of group in groupcast HARQ feedback option 2.

- Discussion

In last RAN1#99 meeting, following working assumption are made:

Working assumption: - For the PSFCH candidate resource set with Z PRBs and Y cyclic shift pairs in each PRB, - Each PSFCH resource is indexed in the manner of frequency first and cyclic shift second. - FFS the order of cyclic shift indexing in a PRB. - PSFCH resource with the index ((K+M) mod (Z*Y)) is used for PSFCH transmission of a RX UE. - K is the L1 source ID of the associated PSCCH/PSSCH. - M is 0 for unicast and groupcast feedback option 1 and M is the member ID of the RX UE for groupcast feedback option 2. - FFS whether to have the following restriction. - Groupcast HARQ feedback option 2 is not used if X > Z*Y (Y denotes the number of PSFCH in a PRB). - Note: RAN1 assumes that the member ID M is an integer between 0 and X-1.

Regarding the FFS point: whether or not to introduce restriction on the size of group in groupcast HARQ feedback option 2. In our opinion, when the feedback resources are not sufficient for the receiver UEs within the group, the feedback option 2 should be disabled.

Proposal 1: Support that groupcast HARQ feedback option 2 is not used if X > Z*Y (Y denotes the number of PSFCH in a PRB).

- Conclusion

In this contribution, we shared our views on B3: whether or not to introduce restriction on the size of group in groupcast HARQ feedback option 2 and proposed that:

Proposal 1: Support that groupcast HARQ feedback option 2 is not used if X > Z*Y (Y denotes the number of PSFCH in a PRB).

- Reference

[1] RP-193198, Task list for 5G V2X in RAN1#100, 3GPP TSG RAN #86, December 9th – 12th, 2019

[2] RAN1 Chairman’s Notes, 3GPP TSG RAN 1#99, November 18th – 22nd, 2019