3GPP TSG RAN WG1 #100	                                                                                                      R1-2000801

e-Meeting, February 24th – March 6th, 2020

Source:	vivo

Title:	Summary#3 of UL inter UE Tx prioritization/multiplexing

Agenda Item:	7.2.5.5

Document for:	Discussion and Decision

- Introduction

The document provides a summary for discussion based on the contribution submitted to agenda item 7.2.5.5-UL inter UE Tx prioritization/multiplexing.

Summary of the open issues

| Section | FL suggestion |
| --- | --- |
| 3.1 RRC parameters | - To include Issue #1~#6 in section 3.1 in the email discussion#1 |
| 3.2 UE behaviours | - To include Issue #3, #5,#6, #7, #8, #9 in section 3.2 in the email discussion #2 - Issue #1, #2, #4, #10, #11 in section 3.2 are not considered as high criticality, thus not included in email discussion |
| 3.3 Text proposals | - To include text proposals #1,#2, #3, #5, #6 in section 3.3 in the email discussion #3 - Text proposal #4 in section 3.3 is not included in email discussion |

Discussions

RRC parameters

Issue 1: Maximum CI monitoring periodicity

Agreement: The maximum monitoring periodicity for UL CI is [5] slots

Discussion:

- Confirm the 5 slots as the maximum monitoring periodicity?

  - Yes: vivo

  - No: Ericsson (support up to 2 slots), CMCC (additionally support 8,10,16,20 as monitoring periodicity)

- FL suggestion: A decision is needed in RAN1#100-E (Criticality-HIGH) –to be included in email discussion #1

Issue 2: BD limitations for UL CI monitoring

Agreement

- Up to X BDs can be configured for UL CI

  - FFS per UL CI monitoring occasion or per span

  - The value of X is to be concluded during this week

  - Note: UE is not expected to be configured with search space configuration for UL CI with AL and number of candidates exceeding X BDs

Discussion:

- The BD limitation is defined as per CI monitoring occasion, or per PDCCH monitoring span?

  - Per CI monitoring occasion: Ericsson, vivo

  - Per PDCCH monitoring span: Apple

- The X value

  - X=2 (same as SFI): Samsung, Huawei

  - X can be configured to from {1, 2, 3, …, 7}: Intel (New RRC parameter?)

  - X is defined as UE capability: vivo

- FL suggestion: A decision is needed in RAN1#100-E (Criticality-HIGH) –to be included in email discussion #1

Issue 3: timedurationforCI for sub-slot level UL CI monitoring

Agreements:

- Possible values for RRC parameter timedurationforCI can be:

  - If the configured UL CI monitoring periodicity is >1 slot or 1-slot with only one monitoring occasion

    - At least the same as the configured UL CI monitoring periodicity

      - FFS whether or not to additionally support multiple of UL CI monitoring periodicity

  - Otherwise (i.e., >1 monitoring occasion within 1 slot when 1-slot is the configured UL CI monitoring periodicity)

    - {2, 4, 7, [14]} OS, which SCS is used when determine the time duration

      - SCS for the DL BWP carrying UL CI

      - FFS The UE is not expected to be configured with a time duration for CI less than the time different (in symbols) between any adjacent monitoring occasions in a slot

Discussion: In case of sub-slot-level CI monitoring (i.e., >1 monitoring occasion within 1 slot when 1-slot is the configured UL CI monitoring periodicity)

- Confirm that 14OS can be configured for timedurationforCI

  - Yes: Ericsson, vivo, CATT, LG

- Can the UE be configured with a time duration to be less than the gap between any adjacent UL CI monitoring occasion?

  - Yes: Ericsson,

  - No: vivo, CMCC

- FL suggestion: suggest the following proposal (Criticality-HIGH) –to be included in email discussion #1

- Confirm that 14OS can be configured for timedurationforCI

- UE is not expected to be configured with a time duration to be less than the gap between any adjacent UL CI monitoring occasion

Issue 4: Possible values for CI-PayloadSize

Agreements:

  - Possible values (16 values) for RRC parameter CI-PayloadSize are

    - {[1],2,4,[5],7,8,[10],14,16,[20],[25],28,32,[35],56,112}

Discussion:

- Confirm the all the possible values for CI-PayloadSize in the previous agreement

  - Yes: Ericsson, vivo, Nokia, CATT

- FL suggestion Confirm the all the possible values for CI-PayloadSize in the previous agreement (Criticality-HIGH) –to be included in email discussion #1

Issue 5: Clarification of RRC parameter offsetToCarrier

[3] raised the issue about which offsetToCarrier parameter (the one from FrequencyInfoUL-SIB or from FrequencyInfoDL-SIB) in case of different SCS is used in DL and UL. Following text proposal is given

-----------------------------------------Text Proposal 1 for Section 11.2A in TS38.213 [2]------------------------------------

11.2A Cancellation indication <---------------------------Other parts are omitted -------------------------------> For a group of symbols, bits from each set of bits have a one-to-one mapping with NBI groups of PRBs where each of the first groups includes PRBs and each of the remaining groups includes PRBs. A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as NRFRsize=LRB from frequencyRegionforCI that indicates an offset and a length as RIV according to [6, TS 38.214], and from offsetToCarrier in FrequencyInfoDL-SIB that indicates for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. <---------------------------Other parts are omitted ------------------------------->

- FL suggestion: It was agreed to use the DL SCS as the reference to derive the UL frequency region, so seems OK to also use offsetToCarrier in IE FrequencyInfoDL-SIB to determine the UL frequency region (Criticality-HIGH)–to be included in email discussion #1

Issue 6: For enhanced OL PC, use of P0-PUSCH-Set in case both DCI formats (0_1, 0_2) are configured for the UE and only one format is configured with SRI.

Since configuration of P0-PUSCH-Set is common to both DCI formants, multiple P0-PUSCH-Set corresponding to different SRI value will be configured to the UE due to presence of SRI in one DCI format. Thus for the other DCI without SRI, [10] proposed to use the P0-PUSCH-Set with lowest ID from the multiple sets.

Proposal:

- For 1 bit OLPC parameter indication, if OLPC parameter set indication in DCI is set to ‘1’

  - P0-PUSCH-Set having the lowest p0-PUSCH-SetId is used.

- For 2 bit OLPC parameter indication, if OLPC parameter set indication in DCI is set to ‘01’ or ‘10’

  - P0-PUSCH-Set having the lowest p0-PUSCH-SetId among P0-PUSCH-Set having two P0-PUSCH is used

- FL suggestion: the 1st part of the proposal seems needed, the 2nd part seems an optimization, should be possible to use the same behaviour for the 1bit and 2bits case, i.e. use the P0-PUSCH-Set having the lowest p0-PUSCH-SetId for the DCI which does not have SRI field. (Criticality-HIGH)–to be included in email discussion #1

| Company | View |
| --- | --- |

UE behaviours

Issue 1: Misaligned start time of reference region due to different TA across UEs

In current specification, the start time of reference region is derived based on the ending symbol of PDCCH carrying UL CI and the processing time, as shown in the relevant 38.213 text below

“An indication by a DCI format 2_4 for a serving cell is applicable to PUSCH or SRS transmissions on the serving cell. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2 from the end of a PDCCH reception where the UE detects the DCI format 2_4. Tproc,2 corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=0 with μ being the smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of an SRS transmission on the serving cell.”

[1] raised the issue about misaligned start time of reference region due to different TA across UEs, as shown in the following figure. It is proposed to revise definition of reference region starting position in time.

[6] [13] proposed to have additional RRC configuration for the start time of the UL reference region, i.e. having an RRC configurable X symbol offset relative to the end of a PDCCH reception where the UE detects the DCI format 2_4.

Feature lead comment:

Ideally it would be nice to align the start of RUR across different UEs as much as possible, such that different UEs can have aligned behaviour for cancellation. However, due to the misaligned timing advance (most likely sub-OFDM-symbol level), there may be an time difference for the start of RUR for different UEs, as shown in the figure below.

In this example, there are three UEs, UE#1 (eMBB) and UE#3 (URLLC) uses TA=0 thus has aligned UL symbol/slot boundary, while UE#2(eMBB) uses TA=0.5 OS. As a result, the start of RUR of UE#2 is later than that of UE#1.In order to protect the URLLC transmission from UE#3, which happens in symbol #2 and #3 of the slot, gNB should sent UL CI bitmap to indicate UL cancellation of symbol#1, #2 and #3 for both UE#1 and UE#2. Consequently, UE#1 is overshot with one additional symbol (symbol#1 of UE#1 should not have been cancelled as it does not overlap with UE#3 URLLC transmission). However, it does not seem to be a big problem which requires significant spec change.

- FL suggestion: The TA issue exists but seems possible to handle by proper gNB implementation. (Criticality-LOW/MED)

The necessity of introducing additional RRC configurable X symbol offset for the start time of UL reference region was explicitly discussed in RAN1#99 with no consensus, suggest not to discuss again.

Issue 2: Issues due to excluding of DL and SSB symbols from reference time region

[3] Raised the issue about unused bits in the bitmap indication caused by the time domain partitioning after excluding DL and SSB symbols. And proposed the solution to reduce the number of wasted indication bits and potentially achieving finer granularity in frequency domain, as proposal below

Proposal: NR should support to take the minimum value between the number of partitions configured by timeGranularityforCI and the number of remaining symbols after excluding DL and SSB symbols from the time region for UL CI as the actual number of partitions.

--------------------------------------------Text Proposal 3 for Section 11.2A in TS38.213 [2]---------------------------------

11.2A Cancellation indication <---------------------------Other parts are omitted -------------------------------> For a serving cell having an associated field in DCI format 2_4, for the field denote by - NCI a number of bits provided by CI-PayloadSize - BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion - TCI a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon,provided by timeDurationforCI in timeFrequencyRegion - a number of partitions for the TCI symbols provided by timeGranularityforCI in timeFrequencyRegion - min {, } GCI sets of bits from the NCI bits have a one-to-one mapping with GCI groups of symbols where each of the first groups includes symbols and each of the remaining groups includes symbols. A UE determines a symbol duration with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. For a group of symbols, NBI=NCIGCI bits from each set of bits have a one-to-one mapping with NBI groups of PRBs where each of the first groups includes PRBs and each of the remaining groups includes PRBs. A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as NRFRsize=LRB from frequencyRegionforCI that indicates an offset and a length as RIV according to [6, TS 38.214], and from offsetToCarrier that indicates for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. <---------------------------Other parts are omitted ------------------------------->

[15] also proposed a method to reuse the “wasted” bits to indicate more frequency regions and provides the following text proposal

For a serving cell having an associated field in DCI format 2_4, for the field denote by - NCI a number of bits provided by CI-PayloadSize - BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion - TCI a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon, provided by timeDurationforCI in timeFrequencyRegion - GCI a number of partitions for the TCI symbols provided by timeGranularityforCI in timeFrequencyRegion If GCI≤TCI, - GCI sets of bits from the NCI bits have a one-to-one mapping with GCI groups of symbols where each of the first GCI-TCI+TCIGCI⋅GCI groups includes TCIGCI symbols and each of the remaining TCI-TCIGCI⋅GCI groups includes TCIGCI symbols. - For a group of symbols, NBI=NCIGCI bits from each set of bits have a one-to-one mapping with NBI groups of PRBs where each of the first NBI-BCI+BCINBI⋅NBI groups includes BCINBI PRBs and each of the remaining BCI-BCINBI⋅NBI groups includes BCINBI PRBs. A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as NRFRsize=LRB from frequencyRegionforCI that indicates an offset RBstart and a length LRB as RIV according to [6, TS 38.214], and from offsetToCarrier that indicates Ocarrier for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. If GCI>TCI, - TCI sets of bits from the NCI bits have a one-to-one mapping with the TCI symbols, where each of the first NCI-NCITCI⋅TBI set of bits includes NBI=NCITCI bits, and each of the remaining TCI-NCI+NCITCI⋅TBI sets include NBI=NCITCI bits - For a symbol, the corresponding set of bits has a one-to-one mapping with NBI RB groups (RBGs) of PRBs, where each of the first NBI-BCI+BCINBI⋅NBI groups includes BCINBI PRBs and each of the remaining BCI-BCINBI⋅NBI groups includes BCINBI PRBs. A UE determines a symbol duration with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as NRFRsize=LRB from frequencyRegionforCI that indicates an offset RBstart and a length LRB as RIV according to [6, TS 38.214], and from offsetToCarrier that indicates Ocarrier for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

[7] proposed that the DL symbols indicated by SFI are excluded from the reference time region for UL CI.

- FL suggestion:  Proposals above are optimizations targeting more effective usage of the bitmap in some regions (with DL and/or SSB symbols). (Criticality-LOW/MED)

Issue 3: Issues related to the interaction between intra-UE UL prioritization/multiplexing and inter-UE UL cancellation

Issue 3-1: A UL channel with which priority level can be cancelled by UL CI? ([2],[4])

Discussion:

  - Option 1-1: For a given UE, UL CI is only applicable to the UL transmissions indicated/configured as low priority level [4][17][18]

  - Option 1-1: A UE is not expected to receive a UL CI indicating a cancellation of its high priority UL transmission. [15]

  - Option 2: For a given UE, UL CI is applicable to UL transmission irrespective of its priority level. [2]

  - Option 3: RRC configuration between option 1and option 2 [4][15]

Issue 3-2: UE behaviour in case of simultaneous UL prioritization/multiplexing for intra-UE and inter-UE cancellation

[2] [15] [18] discussed the UE behaviour in case of simultaneous UL prioritization/multiplexing for intra-UE and inter-UE cancellation, and observed ambiguous operation if the UE behaviour is not specified. [6] proposed that both UL CI and priority indication should be followed by the UE for cancellation. Following options can be discussed

- Option 1: Handling of intra-UE prioritization/multiplexing for overlapping UL transmissions is performed firstly and handling of inter-UE cancellation for UL transmission overlapping with resources by UL CI is performed secondly [15]

- Option 2: Handling of inter-UE cancellation for UL transmission overlapping with resources by UL CI is performed firstly and handling of intra-UE prioritization/multiplexing for overlapping UL transmissions is performed secondly [18]

- Option 3: UE performs intra-UE prioritization/multiplexing or inter-UE cancellation for the overlapped UL channels according to the time order which is determined by the receiving time order of PDCCH carrying DCI scheduling high priority transmission or DCI for UL CI.

- FL suggestion: A decision for above issues is needed in RAN1#100-E (Criticality-HIGH)-to be included in email discussion #2

Issue 4: Condition of UE UL CI monitoring

This issue is related to when a UE starts/ends to monitor UL CI and how BD/CCEs are counted by the UE, followings has been proposed [2][6][12], however [9] [10]proposed to not specify it.

Proposal 1: For UL transmission with associated PDCCH, UE monitors UL CI at least at the latest monitoring occasion ending no later than X symbols before the start of the UL transmission, and X is related to UL CI processing time.

Proposal 2: For UL transmission without associated PDCCH, UE monitors UL CI at least at the latest monitoring occasion that ends no later than X symbols before the start of the UL transmission, and X is related to UL CI processing time.

Furthermore, [2] proposed the following

Proposal 3: For a UE configured with UL CI, the number of BD/CCE for the monitoring occasions for UL CI in a slot or a monitoring span should be counted in the total number of BD/CCE for the monitoring occasions for PDCCH in the slot or monitoring span, even when the UE does not monitor a UL CI.

- FL suggestion:  To discuss whether any specification impact is necessary. (Criticality-LOW/MED)

Issue 5: Whether another UL transmission can be scheduled in non-pre-empted resource

In case of inter-UE prioritization, when a UE is scheduled with a transmission on a resource that is overlapping with URLLC transmission, the UE cancels the scheduled transmission on the overlapped resource according to the indication by UL CI, together with the non-overlapping parts afterwards due to “cancellation without resuming”. Can the UE expect another UL transmission to be scheduled on the non-overlapping cancelled resource? Note that similar issue has been discussed in the intra-UE prioritization with following agreement:

Agreement:

When a high-priority UL transmission overlaps with a low-priority UL transmission in a slot,

  - The UE is not expected to be scheduled to transmit in the non-overlapping canceled symbols

Following the same principle as intra-UE prioritization, the following is proposed [2][5][9][15]

Proposal: In case of UL inter-UE prioritization, when a UL transmission from a UE is cancelled by UL CI, the UE is not expected to be scheduled to transmit in the cancelled symbols that do not overlap with the resource indicated by UL CI.

- FL suggestion: Adopt the above proposal (Criticality-HIGH)–to be included in email discussion #2

Issue 6: overlapping reference time region for multiple UL CIs

In case of sub-slot UL CI monitoring, the timedurationforCI can be configured to be larger than the distance between adjacent CI monitoring occasions thus causes (full or partial) overlapping reference time region for the consecutive UL CIs.

[6] proposed to specify that UE follows the latest UL CI indication. [8] Proposed to specify that UE cancels over the resource that is commonly indicated to be cancelled in both UL CIs. [10][14][18] Proposed to not specify any special UE behaviour, i.e. UE cancels following the indication from any detected UL CI.

- FL suggestion: A decision is needed in RAN1#100-E on whether any additional handling is needed for the overlapping region (Criticality-HIGH)–to be included in email discussion #2

Issue 7: [9] proposed the following

Proposal: A canceled PUSCH transmission by a UE is counted towards the number of PUSCH that a UE can support per slot.

- FL suggestion: Adopt the above proposal (Criticality-HIGH)–to be included in email discussion #2

Issue 8: scheduling and cancellation at the same time

[15] proposed that UE should not expected to receive a scheduling DCI and UL CI which cancels the scheduled transmission at the same time, propose the following text

Proposal: The UE is not expected to detect a DCI format scheduling a PUSCH/SRS and a DCI format 2_4 that indicates the cancellation of the same PUSCH/SRS in the same monitoring occasion of a search space set or overlapping monitoring occasions of different search space set.

- FL suggestion: Adopt the above proposal (Criticality-HIGH)–to be included in email discussion #2

Issue 9: Cancellation behaviour for intra-band UL CA

[15] raised the issue: A UE that supports UL-CA and in case it is scheduled with multiple PUSCHs on different intra-band carriers, if PUSCH is cancelled on one serving cell, the phase continuity for other PUSCHs on other carriers cannot be maintained. And proposed that if UE cancels UL transmisison on at least one serving cell, the overlapping UL transmisison on other intra-band serving cells are also cancelled, with the text proposal below

If the UE indicates a capability to cancel overlapping PUSCHs on different intra-band serving cells by [xxxxx], and if the PUSCH on at least one serving cell is cancelled, the UE cancels the (repetition of the) PUSCHs transmission on all other intra-band serving cells. The cancellation of the (repetition of the) PUSCH transmission on a serving cell includes all symbols from the earliest symbol that is overlapping with the first cancelled symbol of the PUSCH on a serving cell for which the DCI format 2_4 is applicable to.

- FL suggestion: A decision is needed in RAN1#100-E (Criticality-HIGH)–to be included in email discussion #2

Issue 10: UL CI monitoring during BWP switching

[16] discuss the UL CI monitoring issue during BWP switching and has following observation

Observation: In case of a BWP switch,

- When K2 is equal to the BWP switch delay, i.e. when the eMBB PUSCH would start directly after the BWP switch delay, the eMBB UE cannot receive UL CI and cannot cancel its PUSCH.

- When K2 is larger than the BWP switch delay, following the Rel-15 rule, the eMBB UE cannot receive UL CI during the BWP switch delay and neither during the “K2-BWP switch delay” slots before the start of the eMBB PUSCH.

The proposal is to require UE start monitoring the PDCCH already at the start of the next slot after the BWP switch delay, it does not need to wait until the indicated slot-offset value for its uplink transmission. Text proposal is given as the following for Subclause 6.1.1.1 in TS 38.214 v16.0.0.

Text Proposal for 38.213: If a UE detects a DCI format 0_1 indicating an active UL BWP change for a cell, the UE is not required to receive or transmit in the cell during a time duration from the end of the third symbol of a slot where the UE receives the PDCCH that includes the DCI format 0_1 in the scheduling cell until the beginning of a slot after the end of the BWP switch delay

[18] also discussed the same issue and proposed to do not have additional spec impacts

- FL suggestion: This is an optimization for the case of UL CI monitoring during BWP switching, to discuss whether special handling is needed (Criticality-MED)

Issue 11: single or different closed loop for different open loop power control parameters?

  - Support single closed loop: vivo, OPPO

- FL suggestion: Shared closed loop is already assumed for different open loop parameters in the current specification. No action needed.

| Company | View |
| --- | --- |

Text proposals for specification clarifications/improvements

Text proposal 1: [1] raised an text proposal as the following to clarify that whenever UE processing capability 2 is not defined, use UE processing capability 1 to derive the UL CI processing time. Proposed text proposal as the following

TP for section 11.2A of TS38.213:

An indication by a DCI format 2_4 for a serving cell is applicable to PUSCH or SRS transmissions on the serving cell. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2 from the end of a PDCCH reception where the UE detects the DCI format 2_4. Tproc,2 corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=0 with μ being the smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of an SRS transmission on the serving cell. Processing capability 1 should be used if processing capability 2 is not defined for a pair of Frequency Range and SCS in [6, TS 38.214].

- FL suggestion: Adopt the text proposal above (Criticality-HIGH)–to be included in email discussion #3

Text proposal 2:  [3] raised an issue about which UL SCS is used to derive the UL CI processing time in case of different SCS between DL and UL. Note that the starting time of reference time region is depending on the processing time. [3] Proposed to use the smallest SCS provided in the FrequencyInfoUL-SIB, the relevant TP as the following. While [10] proposed to use only the SCS configurations of the PDCCH to derive the processing time, i.e not take the smallest SCS between the DL and UL.

-----------------------------------------Text Proposal 2 for Section 11.2A in TS38.213 [2]------------------------------------

11.2A Cancellation indication <---------------------------Other parts are omitted -------------------------------> An indication by a DCI format 2_4 for a serving cell is applicable to PUSCH or SRS transmissions on the serving cell. For the serving cell, the UE determines the first symbol of the symbols to be the first symbol that is after from the end of a PDCCH reception where the UE detects the DCI format 2_4. corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=0 with being the smallest SCS configuration between the SCS configurations of the PDCCH and SCSs provided in FrequencyInfoUL-SIB on the serving cell. <---------------------------Other parts are omitted ------------------------------->

- FL suggestion: A decision is needed in RAN1#100-E (Criticality-HIGH)–to be included in email discussion #3

Text proposal 3 (Editorial corrections)

- [4] [7]Proposed the following editorial correction to 38.213

TP to TS 38.213, Sec. 11.2A: Correction of SUL description 11.2A Cancellation indication If a UE is provided UplinkCancellation, the UE is provided a CI-RNTI by ci-RNTI for monitoring PDCCH candidates for a DCI format 2_4 [5, TS 38.212]. UplinkCancellation additionally provides to the UE - a set of serving cells, by ci-ConfigurationPerServingCell, that includes a set of serving cell indexes and a corresponding set of locations for fields in DCI format 2_4 by positionInDCI - a number of fields in DCI format 2_4, by positionInDCI-forSUL, for each serving cell for a SUL carrier for a SUL carrier, if the serving cell is configured with a SUL carrier for SUL of a serving cell if the serving cell configured with SUL - an information payload size for DCI format 2_4 by dci-PayloadSize-forCI - an indication for time-frequency resources by timeFrequencyRegion ## <Unchanged text is omitted>

- [7] proposed an 38.213 text proposal as the following

For a group of symbols, NBI=NCIGCI bits from each set of bits have a one-to-one mapping with NBI groups of PRBs where each of the first NBI-BCI+BCINBI⋅NBI groups includes BCINBI PRBs and each of the remaining BCI-BCINBI⋅NBI groups includes BCINBI PRBs. A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as NRFRsize=LRB NRFRsize=BCIfrom frequencyRegionforCI that indicates an offset RBstart and a length LRBBCI as RIV according to [6, TS 38.214], and from offsetToCarrier that indicates Ocarrier for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

- [4] proposed the following correction to 38.212

TP to TS 38.212, Sec. 7.3.1.3.5: Correct reference to UL cancelation section 7.3.1.3.5 Format 2_4 DCI format 2_4 is used for notifying the PRB(s) and OFDM symbol(s) where UE cancels the corresponding UL transmission from the UE according to Clause 11.2A5 of [5, TS 38.213]. The following information is transmitted by means of the DCI format 2_4 with CRC scrambled by CI-RNTI: - Cancellation indication 1, Cancellation indication 2, …, Cancellation indication indication N. The size of DCI format 2_4 is configurable by higher layers parameter dci-PayloadSize-forCI up to 126 bits, according to Clause 11.2A5 of [5, TS 38.213]. The number of bits for each cancellation indication is configurable by higher layer parameter CI-PayloadSize. For a UE, there is at most one cancellation indication for an UL carrier. ## <Unchanged text is omitted>

- FL suggestion: The above text proposals are to be checked in email discussion #3

Text proposal 4: [6] proposed an text proposal as the following

For a group of symbols, NBI=NCIGCI bits from each set of bits have a one-to-one mapping with NBI groups of PRBs where each of the first NBI-BCI+BCINBI⋅NBI groups includes BCINBI PRBs and each of the remaining BCI-BCINBI⋅NBI groups includes BCINBI PRBs. A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as NRFRsize=LRB from frequencyRegionforCI that indicates an offset RBstart and a length LRB as RIV according to [6, TS 38.214], and from offsetToCarrier that indicates Ocarrier for a SCS configuration of an active UL BWP. A UE determines a PRB with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

- FL suggestion: The current text seems clear. The agreement was to used DL SCS as the reference SCS to derive the UL frequency region.

Text proposal 5: text proposal for the 2D-bitmap

[11] proposed the following text proposal to improve the spec description about 2D-bitmap.

-------------------------------------------- Text proposal starts for TS 38.213, Section 11.2A ---------------------------------------- ***** omitted text **** A UE that detects a DCI format 2_4 for a serving cell cancels a PUSCH transmission, or a repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with repetitions, or an SRS transmission on the serving cell if, respectively, - a group of symbols, from the TCI symbols, has at least one bit value of '1' in the corresponding set of NBI bits in the DCI format 2_4 and includes a symbol of the (repetition of the) PUSCH transmission or of the SRS transmission, and - a group of PRBs, from the BCI PRBs, has a corresponding bit value of '1' in the set of bits corresponding to the group of symbols in the DCI format 2_4 and includes a PRB of the (repetition of the) PUSCH transmission or of the SRS transmission, where - the cancellation of the (repetition of the) PUSCH transmission includes all symbols from the earliest symbol of the (repetition of the) PUSCH transmission that is in a group of symbols having at least one bit value of '1' in the corresponding set of NBI bits in the DCI format 2_4; - the cancellation of the SRS transmission includes only symbols that are in one or more groups of symbols having at least one bit value of '1' in the corresponding set of NBI bits in the DCI format 2_4. -------------------------------------------- Text proposal ends for TS 38.213, Section 11.2A ------------------------------------------

- FL suggestion: The text proposal above seems OK, to be checked in email discussion#3

Text proposal 6: proposal by [14]

Whole text proposal incorporating text proposal 1,2, and 3 ## 11.5 Cancellation indication If a UE is provided UplinkCancellation, the UE is provided a CI-RNTI by ci-RNTI for monitoring PDCCH candidates for a DCI format 2_4 [5, TS 38.212]. UplinkCancellation additionally provides to the UE - a set of serving cells, by ci-ConfigurationPerServingCell, that includes a set of serving cell indexes and a corresponding set of locations for fields in DCI format 2_4 by positionInDCI - a number of fields in DCI format 2_4, by positionInDCI-forSUL, for each serving cell for a SUL carrier for a SUL carrier, if the serving cell is configured with a SUL carrier for SUL of a serving cell if the serving cell configured with SUL - an information payload size for DCI format 2_4 by dci-PayloadSize-forCI - an indication for time-frequency resources by timeFrequencyRegion For a serving cell having an associated field in DCI format 2_4, for the field denote by - NCI a number of bits provided by CI-PayloadSize - BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion - TCISCI a number of symbolsprovided by timeDurationforCI in timeFrequencyRegion - GCI a number of partitions TCIprovided by timeGranularityforCI in timeFrequencyRegion GCI sets of bits from the NCI bits have a one-to-one mapping with GCI groups of symbols where each of the first GCI-TCI+TCIGCI⋅GCI groups includes TCIGCI symbols and each of the remaining TCI-TCIGCI⋅GCI groups includes TCIGCI symbols. A UE determines a symbol duration with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. For a group of symbols, NBI=NCIGCI bits from each set of bits have a one-to-one mapping with NBI groups of PRBs where each of the first NBI-BCI+BCINBI⋅NBI groups includes BCINBI PRBs and each of the remaining BCI-BCINBI⋅NBI groups includes BCINBI PRBs. A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as NRFRsize=LRB from frequencyRegionforCI that indicates an offset RBstart and a length LRB as RIV according to [6, TS 38.214], and from offsetToCarrier that indicates Ocarrier for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. The resulting set of PRBs includes a number of PRBs fully or partially overlapping the PRBs derived by the RIV that is denoted as BCI. An indication by a DCI format 2_4 for a serving cell is applicable to PUSCH or SRS transmissions on the serving cell. For the serving cell, the UE determines the first symbol of the TCI2μDL-μUL*SCI symbols to be the first symbol that is after Tproc,2 from the end of a PDCCH reception where the UE detects the DCI format 2_4, where μDL is the SCS configuration of the PDCCH, μUL is the SCS configuration of a PUSCH transmission or of an SRS transmission on the serving cell. If timedurationforCI is not configured, then SCI=TINT*Nsymb, TINT is is the PDCCH monitoring periodicity provided by the value of monitoringSlotPeriodicityAndOffset, and Nsymb is the number of symbol per slot of the serving cell. Otherwise, SCI is provided by timedurationforCI. Tproc,2 corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=0 with μ being the smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of an SRS transmission on the serving cell. If the UE is provided tdd-UL-DL-ConfigurationCommon, symbols indicated as downlink by tdd-UL-DLConfigurationCommon are excluded from the 2μDL-μUL*SCI symbols. If the UE is provided ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, symbols indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon are excluded from the 2μDL-μUL*SCI symbols. The resulting set of symbols includes a number of symbols that is denoted as TCI. A UE that detects a DCI format 2_4 for a serving cell cancels a PUSCH transmission, or a repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with repetitions, or an SRS transmission on the serving cell if, respectively, - a group of symbols, from the TCI symbols, has a corresponding bit value of ‘1’ in the DCI format 2_4 and includes a symbol of the (repetition of the) PUSCH transmission or of the SRS transmission, and - a group of PRBs, from the BCI PRBs, has a corresponding bit value of ‘1’ in the DCI format 2_4 and includes a PRB of the (repetition of the) PUSCH transmission or of the SRS transmission where - the cancellation of the (repetition of the) PUSCH transmission includes all symbols from the earliest symbol of the (repetition of the) PUSCH transmission that are in one or more groups of symbols having corresponding bit values of ‘1’ in the DCI format 2_4 - the cancellation of the SRS transmission includes only symbols that are in one or more groups of symbols having corresponding bit values of ‘1’ in the DCI format 2_4

- FL suggestion: The yellow highlighted parts in the above text proposal are necessary clarifications, other changes do not seems very essential. – to be checked in email discussion#3

| Company | View |
| --- | --- |

Previous agreements

- RAN1#96bis

Working assumption:

- PDCCH is used for UL cancelation indication

  - The Working assumption can be revisited if the DCI for cancelation indication only carry very small number of information bits, e.g. 1 bit.

Agreements:

- Upon detecting an UL cancelation indication, at least stop without resuming is supported

  - FFS whether and how to support stop with resume

Agreements:

- Further discuss which UL transmissions that can potentially be cancelled by the UL cancelation indication, including

  - Dynamic scheduled UL transmissions, including PUSCH, PUCCH, SRS

  - Semi-persistent UL transmissions, including PUSCH, PUCCH, SRS

  - Periodic UL transmissions, including configured grant PUSCH, PUCCH, SRS

  - PRACH

Agreements:

- Further discuss, aiming for down-selection, the group common DCI and UE-specific DCI for UL cancelation indication

  - For group common DCI (different from Rel-15 SFI)

    - UE is configured to monitor a group common DCI which indicates the time/frequency region on which the UL cancellation indication applies

  - For UE specific-DCI

    - When applicable, UE is configured to monitor a second UL grant for the same TB as an earlier PUSCH indicating UL cancellation before the end of the earlier PUSCH transmission. In this case, the UE follows the UL cancellation indication.

Conclusion:

- Further discuss the following power control enhancements

  - Increased TPC range

    - FFS details, e.g. supported value range, number of TPC bits, accumulated and/or absolute TPC, configurability of the TPC tables, applicability to SRS/PUCCH.

  - Indication of open-loop parameter sets based on scheduling DCI without using SRI

  - Indication of open-loop parameter sets based on GC-PDCCH

- RAN1#97

Agreements:

- Support at least group common DCI for cancelation indication

  - FFS whether or not to additionally support UE-specific DCI for cancelation indication

Conclusion:

To down-select from the following options for enhanced power control

- Option 1: Indication of open-loop parameter sets by DCI

- For DG-PUSCH, an open-loop parameter set indicated to the UE by scheduling DCI without using SRI is applied to the scheduled transmission

- FFS At least for single active CG-PUSCH, an open-loop parameter set is indicated to the UE by a UE-specific field in group common DCI

    - FFS for the case of multiple active CG-PUSCH

- FFS For a UE, the open-loop parameter sets for DG-PUSCH and CG-PUSCH may be same or different

- Option 2: Indication of TPC with increased range by DCI

- For DG-PUSCH, a TPC with increased range is indicated to the UE by the TPC field in scheduling DCI

- FFS At least for single active CG-PUSCH (and potentially also for DG-PUSCH), a TPC with increased range is indicated to the UE by a UE-specific TPC field in group common DCI

    - FFS for the case of multiple active CG-PUSCH

- At least for DG-PUSCH, for a UE, the number of TPC entries (4 or 8) and power adjustment value for each entry is higher layer configured

- FFS For a UE, the TPC configuration for DG-PUSCH and CG-PUSCH may be same or different

- Option 3:

- For DG-PUSCH, use either the solution from option 1 or option 2 for DG-PUSCH as above

    - To down-select from option 1 and 2

- FFS At least for single active CG-PUSCH, UE derives the transmissions power based on the time/frequency resource indicated by a group common DCI

    - If a CG-PUSCH transmission overlaps with the indicated time/frequency resource, UE use one open-loop parameter set with higher power for the transmission

    - If a CG-PUSCH transmission does NOT overlap with the indicated time/frequency resource, UE use another open-loop parameter set with lower power for the transmission

    - FFS for the case of multiple active CG-PUSCH

    - Note: some companies have concern that this was not captured in the TR as one potential solutions

- RAN1#98

Agreements:

- Reuse the existing methods for search space configuration to support UL CI monitoring

  - FFS possible restrictions

  - Note: this means both symbol level and slot level monitoring periodicities are possible from specification perspective

Agreements:

- The UE DCI size budget is not increased by UL CI monitoring

- Further discuss methods to reduce the UE monitoring for UL CI, e.g.

  - The number of aggregation levels and/or candidates for the UL CI monitoring should be limited

  - Conditions for eMBB UE UL CI monitoring:

    - For UL transmission with associated PDCCH,

      - Option 1: UE starts UL CI monitoring after the PDCCH is decoded

      - Option 2: UE monitors UL CI at least at the latest monitoring occasion ending no later than X symbols before the start of the UL transmission, and X is related to UL CI processing time.

    - For UL transmission without associated PDCCH, UE monitors UL CI at least at the latest monitoring occasion that ends no later than X symbols before the start of the UL transmission, and X is related to UL CI processing time.

    - Other conditions?

  - Others?

- FFS the enhancement of UE capability (number of non-overlapping CCE and/or blind decodes) for UL CI monitoring

Agreements:

- Upon detecting an UL cancelation indication, for the transmission of UL signal/channels, “stop with resuming” is not supported

  - Except:

    - SRS can still be transmitted on the non-cancelled symbols (conditioned on if SRS can be pre-empted)

    - FFS for the PUSCH repetition (Rel-15 & Rel-16) case

    - FFS for the PUCCH repetition case (conditioned on if PUCCH can be pre-empted)

  - FFS whether another PUSCH can be scheduled in non-pre-empted resource

  - FFS impact (e.g. phase continuity issue) to a different carrier due to UL cancelation

R1-1909774

Agreements:

- The following UL channel/signals can be cancelled by UL cancelation indication

  - PUSCH (including DG-, CG- and SP-)

  - FFS for SRS

  - FFS for PUCCH

    - Option 1: PUCCH (all types) can be cancelled

    - Option 2: Some PUCCH can be cancelled, e.g. PUCCH carrying CSI

    - Option 3: PUCCH cannot be cancelled

  - FFS for PRACH (preamble and/or MSG 3 PUSCH)

Agreements:

- The UE processing time requirement for UL cancelation indication based on N2 defined in Rel-15 UE cap#2 is supported

  - FFS whether the processing time requirement for UL cancelation indication larger than N2 as defined in Rel-15 UE cap#2 can also be supported as an UE capability

  - FFS whether the processing time requirement for UL cancelation indication shorter than N2 as defined in Rel-15 UE cap#2 as can also be supported an UE capability

Agreements:

- For a DG-PUSCH, an open-loop parameter set indicated to the UE by scheduling DCI using a separate field than SRI is supported.

  - FFS number of bits for the indication

- RAN1#98bis

Agreements:

- Regarding UL CI monitoring, support the following:

  - A new RNTI (e.g. CI-RNTI) is used for UL CI

  - FFS: Monitoring periodicity larger than [5] slot is not supported for UL CI

  - The aggregation level(s) and the number of PDCCH candidates configured by RRC

    - FFS possible restrictions, e.g., the ones associated with SFI

  - The DCI payload size for UL CI  is configured by RRC

    - FFS possible values

Agreements:

- SRS can be cancelled by UL CI

- PUCCH cannot be cancelled by UL CI

- RACH related UL transmissions cannot be cancelled by UL CI, including MSG 1/3 in case of 4-step RACH, MSG A in case of 2-step RACH.

Agreements:

- Cross-carrier UL cancelation indication is supported using the same way as Rel-15 SFI/DL PI

  - The indication field position in DCI for each cross-carrier indicated serving cell is configured by RRC

Agreements:

- Different UE processing time capability for UL CI (i.e. shorter or longer than T_proc2 for cap#2 UE) is not considered in Rel-16

  - d2,1=0 also when DMRS and UL-SCH (for the PUSCH to be cancelled) are multiplexed in the 1st symbol

Agreements:

- In case of PUSCH repetitions, UL CI is applied to each repetition individually (actual repetition in case of Rel-16 PUSCH repetition) that overlaps with the resource (in time and frequency) indicated by UL CI.

Agreements:

  - The reference time region where a detected UL CI is applicable is determined by the following:

      - The reference time region starts from X symbols after the ending symbol of the PDCCH CORESET carrying the UL CI, where X is at least equal to the minimum processing time for UL cancelation

        - FFS X can be configured to be larger than the minimum processing time for UL cancelation

      - The duration of the reference time region is configured by RRC

        - FFS Possible values (e.g. 2OS, 4OS, 7OS, 14OS, 28OS?)

      - FFS DL symbols are excluded from the reference time region

Agreements:

    - The reference frequency region where a detected UL CI is applicable is configured by RRC

Agreements:

Support the following for UL CI

  - Each UL cancelation indicator per serving cell has a RRC configurable field size of  X bits

      - One value of X is 14

      - FFS other values (e.g. X can be N (N>0) times of 7)

  - The time domain granularity for the reference time region is configured by RRC

      - FFS the possible values  (e.g. the time region can be divided into [1],[2],[4],[7],[14],…portions)

      - FFS valid configurations according to the duration of the time reference region

  - The frequency domain granularity is determined based on the configured time domain granularity and the configured bit field size of each indicator

  - The time and frequency resource for cancellation is jointly indicated by a 2D-bitmap (i.e. similar as DL PI) over the time and frequency partitions within the reference region

      - FFS dynamic 2D-bitmap

Agreements:

- For DG-PUSCH, one bit (separately from SRI) in UL grant is used to indicate the open loop power control parameter set

  - Introduce one new RRC parameter that contains one additional P0-PUSCH-Set per SRI

  - The one bit indication is present in the UL grant when the above new RRC parameter is configured

  - If present, the one bit in the DCI is used to switch between the P0 value from the existing P0-PUSCH-AlphaSet and the P0 value from the newly configured P0-PUSCH-Set

Conclusion:

No enhancement for CG-PUSCH power control in Rel-16 for inter-UE multiplexing

- RAN1#99

Agreements:

- There is no enhancement to PDCCH monitoring capability (number of BD and non-overlapping CCEs) specifically for UL CI monitoring purpose

Agreements:

- The maximum monitoring periodicity for UL CI is [5] slots

Agreements:

- Up to X BDs can be configured for UL CI

  - FFS per UL CI monitoring occasion or per span

  - The value of X is to be concluded during this week

  - Note: UE is not expected to be configured with search space configuration for UL CI with AL and number of candidates exceeding X BDs

Agreements:

- The maximum size for dci-PayloadSize-forCI is 126

Agreements:

- Possible values for RRC parameter timedurationforCI can be:

  - If the configured UL CI monitoring periodicity is >1 slot or 1-slot with only one monitoring occasion

    - At least the same as the configured UL CI monitoring periodicity

      - FFS whether or not to additionally support multiple of UL CI monitoring periodicity

  - Otherwise (i.e., >1 monitoring occasion within 1 slot when 1-slot is the configured UL CI monitoring periodicity)

    - {2, 4, 7, [14]} OS, which SCS is used when determine the time duration

      - SCS for the DL BWP carrying UL CI

    - FFS The UE is not expected to be configured with a time duration for CI less than the time different (in symbols) between any adjacent monitoring occasions in a slot

Agreements:

  - Possible values (16 values) for RRC parameter CI-PayloadSize are

    - {[1],2,4,[5],7,8,[10],14,16,[20],[25],28,32,[35],56,112}

  - timeGranularityforCI is defined as number of partitions within the time region, and possible values are

    - {1,2,4,7,14,28}

  - The configured value of CI-PayloadSize shall be a multiple integer of the configured value of timeGranularityforCI

Agreements:

  - The frequency region for UL CI is derived by the following

    - A RIV indication configured by RRC within value range of (0..37949) (i.e. the same way as IE “locationAndBandwidth” for BWP configuration ), the configuration is per serving cell specific

      - The reference point is derived based on the RRC parameter offsetToCarrier (existing parameter, same way as BWP configuration)

    - A reference SCS (no RRC configuration) for a serving cell (to handle the case where a UE is configured with multiple BWPs using different SCSs on the serving cell),

      - Use the SCS for the DL BWP carrying UL CI as the reference SCS

Agreements:

- Support per serving cell configuration for the following parameters

- CI-PayloadSize

- timedurationforCI

- timeGranularityforCI

- frequencyRegionforCI

Agreements:

- If a serving cell is configured with SUL, each UL carrier (SUL and non-SUL) can be configured with different positionInDCI.

Agreements:

- The DL symbols indicated by tdd-UL-DL-ConfigurationCommon are excluded from the reference time region for UL CI

  - The partition of reference time region is done after excluding the DL symbols

  - The symbols used for SSB are also excluded

Agreements:

- Clarification of 2D-bitmap

  - 2D-bitmap is to use X bits for bitmap indication over a time/frequency region with M partitions in time and N partitions in frequency, and X=M x N

Agreements:

Regarding “FFS whether or not to additionally support multiple of UL CI monitoring periodicity”

- If the configured UL CI monitoring periodicity is >1 slot or 1-slot with only one monitoring occasion, no additionally support that the time duration to be multiple of UL CI monitoring periodicity

Agreement

To determine the P0 value in case SRI is not configured in the DCI

- Option 1A: The open-loop power control parameter set indication field in the DCI can be configurable to be 1 or 2bits

  - P0-PUSCH-Set can provide up to two P0 values

    - UE uses the P0 values according to open loop power control indication field in DCI

    - UE use P0 from P0-PUSCH-AlphaSet when

      - open-loop power control parameter set indication field is 1bit and “0” is indicated, or

      - open-loop power control parameter set indication field is 2bits and “00” is indicated

  - Open-loop power control parameter set indication field can be separately configurable for DCI format 0_1 and DCI format 0_2

    - If open-loop power control parameter set indication field is not present for a DCI format, use P0 from P0-PUSCH-AlphaSet

  - A single configuration of P0-PUSCH-Set applies to both DCI format 0_1 and DCI format 0_2

- TR 38.824

- 7.2 Potential enhancements In the following sub-sections, potential enhancements for UL inter UE Tx prioritization/multiplexing are presented. It is recommended to specify both UL cancelation scheme and enhanced UL power control scheme in the work item phase. - 7.2.1 UE UL cancelation mechanisms UE UL cancelation mechanisms are considered as one potential enhancement for UL inter-UE Tx prioritization/multiplexing and are studied from several aspects, including the potential mechanisms (e.g. UE UL cancelation/pausing indication, UL continuation indication, UL re-scheduling indication), physical channel/signal used for the UL cancelation indication, UE processing timeline for the UL cancelation indication, UE monitoring behaviours for the UL cancelation indication, UE PDCCH monitoring capability if the UL cancelation indication is by PDCCH, methods to ensure the reliability of the indication for UE UL cancelation. Either PDCCH or sequence can be considered as potential options for the UL cancelation indication. If PDCCH is used, either group common DCI or UE-specific DCI can be considered as potential options. If sequence is used, either group common sequence or UE-specific sequence can be considered. The monitoring periodicity for the UL cancelation indication should be configurable by the gNB and UE supporting UL cancelation indication should be able to support more than one monitoring occasions for the UL cancelation indication in a slot. If PDCCH is used, whether the UE PDCCH monitoring capability (number of CCEs/BDs per slot) should be increased is to be further investigated. The UE processing time for UL cancelation indication should be equal or shorter than N2 defined in Rel-15 UE capability#2. Upon detecting an UL cancelation indication, UE cancels the corresponding UL transmission. The corresponding UL transmission may include an on-going UL transmission, or an UL transmission that has not been started. After cancelation, the UE may resume the transmission afterwards as one option, or may not resume the transmission afterwards as another option. - 7.2.2 Enhanced UL power control Enhanced UL power control is considered as one potential enhancement for UL inter-UE Tx prioritization/multiplexing and the study mainly focuses on enhanced dynamic power boost for URLLC UE, including dynamic change of power control parameters (e.g. P0 and alpha without SRI configured) and enhanced TPC (e.g. increased TPC range and finer granularity). The need of URLLC UE power change during one transmission instance is not envisioned. It is assumed that there is no change of eMBB UE power control scheme in this study item. Enhanced dynamic power boost for URLLC UE are studied from several aspects, including feasibility of boosting UE power in power limited or interference limited scenarios, physical channel/signal used for the signalling, UE processing timeline for the signalling, UE monitoring behaviours for the signalling, UE PDCCH monitoring capability if the signalling is by PDCCH and methods to ensure the reliability of the signalling. It is concluded that the potential enhanced UL power control may include UE determining the power control parameter set (e.g. P0, alpha) based on scheduling DCI indication without using SRI, or based on group-common DCI indication. Increased TPC range compared to Rel-15 may also be considered. Power boosting is not applicable to power limited UEs.

List of contributions and proposals

| 1 | R1-2000234 | Remaining Issue of Inter-UE Prioritization and Multiplexing of UL Transmissions | Ericsson |
| --- | --- | --- | --- |
| 2 | R1-2000329 | UL inter UE Tx prioritization for URLLC | vivo |
| 3 | R1-2000359 | Remaining issues on UL inter-UE multiplexing between eMBB and URLLC | ZTE |
| 4 | R1-2000433 | Maintenance of Rel-16 URLLC Enhanced inter UE Tx prioritization/multiplexing | Nokia, Nokia Shanghai Bell |
| 5 | R1-2000453 | Remaining issues on enhanced inter UE Tx prioritization/multiplexing | MediaTek Inc. |
| 6 | R1-2000483 | Inter UE Tx prioritization and multiplexing | OPPO |
| 7 | R1-2000531 | Remaining issues on inter-UE UL multiplexing | CATT |
| 8 | R1-2000586 | On the granularity of overlapping Reference Region for UL CI | Sony |
| 9 | R1-2000629 | Remaining issues for inter-UE multiplexing | Samsung |
| 10 | R1-2000674 | Remaining issues of UL inter UE Tx prioritization | LG Electronics |
| 11 | R1-2000738 | Remaining details on enhanced inter-UE multiplexing | Intel Corporation |
| 12 | R1-2000753 | Remaining issues on inter UE Tx prioritization/multiplexing | CMCC |
| 13 | R1-2000857 | Remaining Details on Enhanced Inter-UE Prioritization/Multiplexing | Apple |
| 14 | R1-2000938 | Remaining issues on UL cancellation indication for NR URLLC | WILUS Inc. |
| 15 | R1-2000972 | Remaining issues on uplink Inter-UE Tx Multiplexing and Prioritization | Qualcomm Incorporated |
| 16 | R1-2001025 | Corrections on UL inter-UE multiplexing | Huawei, HiSilicon |
| 17 | R1-2001094 | Inter-UE prioritization/multiplexing | InterDigital, Inc. |
| 18 | R1-2000774 | Enhanced inter UE Tx prioritization/multiplexing | ETRI |