3GPP TSG RAN WG1 Meeting #100-e	R1-2000822

e-Meeting, February 24th-March 6th, 2020

Agenda Item:	7.2.4.7

Source:	FUTUREWEI

Title:	Summary of critical issues for NR Uu scheduling LTE sidelink

Document for:	Discussion and decision

# Introduction

A WID on NR-V2X was approved at RAN#83, with one of the objectives as follows:

2. Specify support for NR Uu to provide control for LTE sidelink

- Sidelink mode 4 as per the study outcome [RAN2, RAN1]; and

- Sidelink mode 3-like RRC-configured SPS scheduling with either RRC-based activation/deactivation as per the study outcome or DCI-based activation/deactivation [RAN1, RAN2].

  - RAN1 to make a decision on which option is supported until RAN#84.

- 13 companies submitted contributions for this AI ([1]-[7], [9]-[14]), although [12] addresses a different AI. This summary aims to collect issues brought in submitted contributions.

# Issues

## Summary

The issues to be addressed for this AI are listed in Table 1. More details for each issue are given in Sections 2.2-2.5.

Table 1. Summary of issues

| Issue | Criticality | FL proposal | Comments |
| --- | --- | --- | --- |
| Timing of the application of a DCI 3_1 | HIGH | Email discussion A | Issue discussed and resolved for mode-1 at RAN1#99 |
| DCI format size | HIGH | Email discussion B | Which NR Uu DCI size to match 3_1 with |
| X Values | Medium | Email discussion C | Decide whether new X values are allowed, and whether to remove the brackets around [1.25] and [1.5]ms |
| Capture necessary changes on LTE side in TS36.213 | HIGH | Up to the Editor | Can be handled by the Editor with a cat-B CR during this e-meeting |

## Timing

At RAN1#99, for sidelink mode-1 resource allocation, the timing at which a dynamic grant/CG type-2 grant applies was agreed, with an FFS for cross-RAT scheduling to be resolved here:

- For dynamic grant and configured grant type-2, the slot of the first sidelink transmission is the in the first SL slot of the corresponding resource pool that starts not earlier than (working assumption for the formular)  where TDL is starting time of the slot carrying the corresponding DCI, TTA is the timing advance value and m is the slot offset (based on the SL numerology) between DCI and the first sidelink transmission scheduled by DCI, Tc is as defined in 38.211, and Tslot is the SL slot duration.

  - FFS the case of CG type 1

- FFS the application of the above formula to cross-RAT scheduling.

Contributions [1], [2], [3], [7], [9], [13] discuss the FFS on the applicability of this formula (or an equivalent one) for LTE sidelink scheduled by NR Uu. The timing of a grant must be without ambiguity to ensure all UEs understand apply the same timing, therefore this issue must be resolved.

## DCI format size

At RAN1#97, the following was agreed:

- DCI-based activation/deactivation is supported

  - Support of LTE PC5 scheduling by NR Uu (mode 3-like ) is based on UE capability

  - NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling

  - The size of DCI for activation/deactivation is one of the DCI size(s) that will be defined for NR Uu scheduling NR V2V

    - FFS whether the DCI format is the same as one of the DCI formats that will be defined for NR Uu scheduling NR V2V

The DCI size issue was brought up by multiple companies: [2], [4], [7], [8], [9], [11], [14]. This is another critical issue that must be addressed unambiguously by RAN1. Note: it is expected that a similar issue will be discussed in the mode-1 sub agenda item.

## Values of X

Values of X were discussed at RAN1#99, with a first set of values agreed:

The supported values of X signaled in the DCI are:

- 0.75ms, 1ms, [1.25ms], [1.5ms], 2ms, 4ms, 5ms, 8ms, 10ms, 20 ms

  - Additional value(s) can be discussed during the Feb. meeting

- Spare values are reserved for future deployments

Several companies discuss this aspect ([1], [5], [6], [13]). While this issue is less critical, is should be feasible to conclude this aspect by email discussion.

## Capture of the decisions in TS36.213

In [13], it is brought up that the decisions in this sub-AI were not captured in the TS36.213 LTE specification. This aspect needs to be corrected. However, it can be handled by the Editor with a cat-B CR during this e-meeting

# Decisions at previous meetings

## RAN1#96

Agreements:

- Scheduling by gNB using RRC for LTE sidelink scheduled mode is supported from RAN1 perspective under the premise that there is sufficient time for coordination between the NR and LTE modules. No DCI to activate/release

  - RRC message delivers the SPS grant configuration and releases the SPS configuration.

  - Support of this scheduling mode is subject to UE capability (may or may not have capability for both LTE & NR)

  - Note: some specification LTE change is needed to support the reception of a grant through RRC

    - RRC message contains mode 3 grant content and timing

    - Up to the Editor to capture it as mode 3 or new LTE sidelink mode

  - No intention to have additional NR & LTE specification change (other than those described above) for this function in Rel-16

- RAN1 studied the feasibility of SPS scheduling by gNB for LTE sidelink with DCI activation/release, but there is no consensus to support it

## RAN1#96bis

Agreements:

Regarding RRC-based versus DCI-based activation/release of LTE sidelink SPS, RAN1 agrees to make the choice on the basis of at least:

- Spec impact

- Flexibility

- Performance, including latency

- Implementation complexity

- Timing of the activation/deactivation

## RAN2#105bis

Agreements:

For scheduling LTE SL UEs, the gNB uses RRC messages to deliver the SPS grant configuration.

Separate system information block should be designed to support LTE resource pool configuration via NR Uu. It will be defined as a container (OCTET STRING) and actual information follows what defined in LTE RRC.

gNB should be able to configure the LTE V2X mode 4 sidelink resource pool via dedicated signalling. In addition, gNB should be able to configure mode3 SL resources via dedicated signaling. It will be defined as a container (OCTET STRING) and actual information follows what defined in LTE RRC.

## RAN1#97

Agreements:

- DCI-based activation/deactivation is supported

  - Support of LTE PC5 scheduling by NR Uu (mode 3-like ) is based on UE capability

  - NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling

  - The size of DCI for activation/deactivation is one of the DCI size(s) that will be defined for NR Uu scheduling NR V2V

    - FFS whether the DCI format is the same as one of the DCI formats that will be defined for NR Uu scheduling NR V2V

  - Activation/deactivation applies to the first LTE subframe after Z+X ms after receiving the DCI

    - Z is the same timing offset in current LTE V2X specs

    - X>0. FFS value(s) of X, and if one or multiple values of X are possible

## RAN1#98

Agreements:

- A new RNTI is introduced to scramble the NR DCI used for scheduling LTE PC5.

- X is dynamically indicated using a field in the DCI

  - FFS whether the DCI field provides an index to a table or the value of X

  - The minimum value of X is subject to UE capability

    - UE reports a single value subject to UE capability

## RAN1#98b

Agreements:

- The NR DCI field to indicate X provides an index to a table of values

  - The table of values is configurable, and has 8 values

  - The size of the DCI field is fixed at 3 bits

## RAN1#99

Agreements:

- Use a separate PDCCH monitoring configuration (as configured in Rel-15) for NR DCI scheduling LTE SL

  - The per-CC and across-CC blind decoding budget and the maximum number of non-overlapped CCEs for channel estimation are not increased.

  - The per-CC and across-CC maximum number of search spaces is not increased.

  - The per-CC and across-CC maximum number of CORESETs is not increased

    - When in the same slot, there is both PDCCH monitoring for Uu SL and PDCCH monitoring for SL for the same CC, the search space(s) for LTE SL is configured to be the same or a subset of those for Uu for the same CC or vice versa

Agreements:

- The minimum value of X signalled in the UE capability is one of the values (excluding spare values) that can be signalled in DCI 3_1

Agreements:

The supported values of X signaled in the DCI are:

- 0.75ms, 1ms, [1.25ms], [1.5ms], 2ms, 4ms, 5ms, 8ms, 10ms, 20 ms

  - Additional value(s) can be discussed during the Feb. meeting

- Spare values are reserved for future deployments

# References

| No. | Tdoc | Type | Source | Title |
| --- | --- | --- | --- | --- |
| 1 | R1-2000189 | discussion | Huawei, HiSilicon | Remaining details of NR Uu control for LTE sidelink |
| 2 | R1-2000323 | discussion | vivo | Remaining issues on support of NR Uu controlling LTE sidelink |
| 3 | R1-2000389 | discussion | Nokia, Nokia Shanghai Bell | Remaining details of NR Uu controlling LTE sidelink |
| 4 | R1-2000450 | discussion | MediaTek | Support of NR-Uu controlling LTE sidelink |
| 5 | R1-2000497 | discussion | OPPO | On DCI signalling to control LTE sidelink |
| 6 | R1-2000526 | discussion | CATT | Remaining issues on NR Uu controlling LTE sidelink |
| 7 | R1-2000571 | decision | Futurewei | Remaining details on the support of NR Uu controlling LTE sidelink |
| 8 | R1-2000623 | discussion | Samsung | On NR Uu Controlling LTE Sidelink |
| 9 | R1-2000787 | discussion | LGE | Discussion on NR Uu controlling LTE sidelink |
| 10 | R1-2000822 | decision | Futurewei | Summary of critical issues for NR Uu scheduling LTE sidelink |
| 11 | R1-2000854 | discussion | Apple | On size alignment of DCI format 3_1 |
| 12 | R1-2000967 | discussion | Qualcomm | Syncronisation details for NR-V2X |
| 13 | R1-2001014 | discussion | Ericsson | NR UU controlling LTE sidelink transmissions |
| 14 | R1-2001010 | discussion | Ericsson | Other outstanding resource allocation issues |