3GPP TSG RAN WG1 #100-e			R1-2000854

e-Meeting, February 24th – March 6th, 2020

Agenda Item:	7.2.4.7

Source:	Apple

Title:	On Size Alignment of DCI Format 3_1

Document for:	Discussion/Decision

# Introduction

The Release-16 NR V2X specifications were approved in Dec. 2019 [1]. There are still some remaining tasks for NR V2X, which were identified in [2].

In this contribution, we discuss the size alignment of DCI format 3_1.

# Discussion

It was agreed [3] to use a separate PDCCH monitoring configuration for NR DCI scheduling LTE sidelink. When in the same slot, there is both PDCCH monitoring for Uu and PDCCH monitoring for sidelink for the same carrier, the search space(s) for LTE sidelink is configured to be the same or a subset of those for Uu for the same carrier or vice versa.

If a UE monitors both DCI format 3_0 and DCI format 3_1, i.e., the UE supports both LTE sidelink and NR sidelink, then it is beneficial to align the payload size of DCI format 3_1 with that of DCI format 3_0 via zero padding. This maintains DCI size budget and reduces blind decoding complexity. Our companion contribution [4] proposes to align DCI format 3_0 payload size to DCI format 0_0 to further reduce the blind decoding complexity.

If a UE monitors only DCI format 3_1 for sidelink, i.e., the UE only supports LTE sidelink, then it is beneficial to align the payload size of DCI format 3_1 with that of DCI format 0_0, due to their similar payload sizes. Again, this payload size alignment is achieved by zero padding.

Proposal 1: If a UE monitors only DCI format 3_1 for sidelink, then payload size of DCI format 3_1 is aligned with payload size of DCI format 0_0 by zero padding.

# Conclusion

In this contribution, we discussed the size alignment of DCI format 3_1. Our proposal is as follows:

Proposal 1: If a UE monitors only NR DCI format 3_1 for sidelink, then payload size of DCI format 3_1 is aligned with payload size of DCI format 0_0 by zero padding.

# References

- Chairman’s Notes, 3GPP TSG RAN #86 Meeting, Sitges, ES, Dec. 2019.

- RP-193198, Task list for 5G V2X in RAN1 #100, Sitges, ES, Dec. 2019.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #99 Meeting, Reno, USA, Nov. 2019.

- R1-2000851, Remaining details on mode 1 resource allocation, Apple, Feb. 2020.