3GPP TSG RAN WG1 Meeting #100-e	R1-2000877

e-Meeting, February 24th – March 6th, 2020

Source:	Sharp

Title:	Remaining issues on resource allocation mode 1 for NR sidelink

Agenda Item:	7.2.4.2.1

Document for:	Discussion and Decision

Introduction

In this document, we share our views on a few remaining issues on resource allocation mode 1 for NR sidelink.

Discussion

Size alignment for DCI format 3_0 and 3_1

The following were agreed in RAN1#98bis and RAN1#99 regarding handling of the sizes of DCI format 3_0 and 3_1.

RAN1#98bis agreements: - Two different UE-specific SL RNTIs are introduced for Mode-1 scheduling: one for CRC scrambling in DCI for a dynamic grant and the other one for CRC scrambling in DCI for a configured grant type-2. - The two above DCIs have the same size RAN1#99 agreements: - Existing DCI size budget is maintained when the UE is configured with SL - (working assumption): The size of the new DCI format and the size of one of the existing NR DCI formats are aligned.

The above RAN1#98bis agreements (i.e. size alignment between DCI format 3_0 and 3_1) were specified in TS 38.212 as follows,

##### 7.3.1.4.2 Format 3_1 DCI format 3_1 is used for scheduling of LTE PSCCH and LTE PSSCH in one cell. The following information is transmitted by means of the DCI format 3_1 with CRC scrambled by SL-L-CS-RNTI: - Timing offset – 3 bits determined by higher layer parameter TimeOffsetLTESL, as defined in clause x.x.x of [x] […] If the UE is configured to monitor DCI format 3_0 and the number of information bits in DCI format 3_1 is less than the payload of DCI format 3_0, zeros shall be appended to DCI format 3_1 until the payload size equals that of DCI format 3_0.

However, the above RAN1#99 agreements (i.e. size alignment between DCI format 3_0/3_1 and other DCI formats) have not been specified in TS 38.212. And it is still unclear what “one of the existing NR DCI formats” in the RAN1#99 agreements mean. In our view, the size alignment between DCI format 3_0/3_1 and other DCI formats should be done after (or at the end of) the DCI size alignment procedure as specified in sub clause 7.3.1.0 of TS 38.212, and DCI format 3_0/3_1 should be padded to the closest value among the set of DCI sizes in the cell (i.e. outcome of the DCI size alignment procedure). A TP is provide for this proposal in section 5.1.

Timing determination of PUCCH for SL A/N to gNB

Regarding timing determination of PUCCH, the following working assumption was made in RAN1.

Working assumption: - The timing of the PUCCH used for conveying SL HARQ is indicated in DCI or RRC (only for transmissions without a DCI) in terms of PSFCH-to-PUCCH physical slots, where the slot duration is defined based on the PUCCH SCS. - Note: it is not intended to define any new sync requirements for gNBs

The corresponding spec text was captured as follows,

##### 16.5 UE procedure for reporting HARQ-ACK on uplink ##### […] With reference to slots for PUCCH transmissions and for a number of PSFCH reception occasions ending in slot n, the UE provides the generated HARQ-ACK information in a PUCCH transmission within slot n+k, subject to the overlapping conditions in Clause 9.2.5, where k is a number of slots indicated by a PSFCH-to-HARQ_feedback timing indicator field, if present, in a DCI format indicating a slot for PUCCH transmission to report the HARQ-ACK information, or k is provided by sl-ACKToUL-ACK. k=0 corresponds to a last slot for a PUCCH transmission that would overlap with the last PSFCH reception occasion. ##### […]

According to the working assumption that the indicated slot duration is in unit of physical slots with reference to PUCCH SCS, it implies slot n+k and slot n both refer to slots in Uu carrier. Considering the timing between sidelink and Uu carrier may not align in some cases, it is not clear enough which slot the PSFCH reception occasions end, i.e. it is ambiguous for the UE to determine slot n. Hence, we propose to clarify how the UE determines slot n.

Corresponding TP is provide for this proposal in section 5.2.

Conclusion

In this contribution, we discuss some remaining issues on resource allocation mode 1 for NR sidelink, and provide corresponding TPs in section 5.

References

RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei, 3GPP RAN#83.

- Appendix

TP for subclause 7.3.1.0 and 7.3.1.4.2 in TS 38.212 on DCI size alignment

#### < Unchanged parts are omitted >

#### 7.3.1.0	DCI size alignment

If necessary, padding or truncation shall be applied to the DCI formats according to the following steps executed in the order below:

Step 0:

-	Determine DCI format 0_0 monitored in a common search space according to clause 7.3.1.1.1 where  is the size of the initial UL bandwidth part.

-	Determine DCI format 1_0 monitored in a common search space according to clause 7.3.1.2.1 where  is given by

-	the size of CORESET 0 if CORESET 0 is configured for the cell; and

-	the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

-	If DCI format 0_0 is monitored in common search space and if the number of information bits in the DCI format 0_0 prior to padding is less than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_0 until the payload size equals that of the DCI format 1_0.

-	If DCI format 0_0 is monitored in common search space and if the number of information bits in the DCI format 0_0 prior to truncation is larger than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, the bitwidth of the frequency domain resource assignment field in the DCI format 0_0 is reduced by truncating the first few most significant bits such that the size of DCI format 0_0 equals the size of the DCI format 1_0.

Step 1:

-	Determine DCI format 0_0 monitored in a UE-specific search space according to clause 7.3.1.1.1 where  is the size of the active UL bandwidth part.

-	Determine DCI format 1_0 monitored in a UE-specific search space according to clause 7.3.1.2.1 where  is the size of the active DL bandwidth part.

-	For a UE configured with supplementaryUplink in ServingCellConfig in a cell, if PUSCH is configured to be transmitted on both the SUL and the non-SUL of the cell and if the number of information bits in DCI format 0_0 in UE-specific search space for the SUL is not equal to the number of information bits in DCI format 0_0 in UE-specific search space for the non-SUL, a number of zero padding bits are generated for the smaller DCI format 0_0 until the payload size equals that of the larger DCI format 0_0.

-	If DCI format 0_0 is monitored in UE-specific search space and if the number of information bits in the DCI format 0_0 prior to padding is less than the payload size of the DCI format 1_0 monitored in UE-specific search space for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_0 until the payload size equals that of the DCI format 1_0.

-	If DCI format 1_0 is monitored in UE-specific search space and if the number of information bits in the DCI format 1_0 prior to padding is less than the payload size of the DCI format 0_0 monitored in UE-specific search space for scheduling the same serving cell, zeros shall be appended to the DCI format 1_0 until the payload size equals that of the DCI format 0_0

Step 2:

-	Determine DCI format 0_1 monitored in a UE-specific search space according to clause 7.3.1.1.2.

-	Determine DCI format 1_1 monitored in a UE-specific search space according to clause 7.3.1.2.2.

-	For a UE configured with supplementaryUplink in ServingCellConfig in a cell, if PUSCH is configured to be transmitted on both the SUL and the non-SUL of the cell and if the number of information bits in format 0_1 for the SUL is not equal to the number of information bits in format 0_1 for the non-SUL, zeros shall be appended to smaller format 0_1 until the payload size equals that of the larger format 0_1.

-	If the size of DCI format 0_1 monitored in a UE-specific search space equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 0_1.

-	If the size of DCI format 1_1 monitored in a UE-specific search space equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 1_1.

Step 2A:

-	Determine DCI format 0_2 monitored in a UE-specific search space according to clause 7.3.1.1.3.

-	Determine DCI format 1_2 monitored in a UE-specific search space according to clause 7.3.1.2.3.

-	For a UE configured with supplementaryUplink in ServingCellConfig in a cell, if PUSCH is configured to be transmitted on both the SUL and the non-SUL of the cell and if the number of information bits in format 0_2 for the SUL is not equal to the number of information bits in format 0_2 for the non-SUL, zeros shall be appended to smaller format 0_2 until the payload size equals that of the larger format 0_2.

-	[If the size of DCI format 0_2 monitored in a UE-specific search space equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 0_2.]

-	[If the size of DCI format 1_2 monitored in a UE-specific search space equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 1_2.]

-	If DCI format 3_0 and/or DCI format 3_1 is monitored in a UE-specific search space and the size of the DCI format 3_0/3_1 does not equal that of any other DCI format configured to monitor for the cell, zeros shall be appended to the DCI format 3_0 and/or DCI format 3_1 until the payload size equal the smallest value among the sizes of other DCI formats configured to monitor for the cell which are larger than that of the DCI format 3_0/3_1 prior to padding.

Step 3:

-	If both of the following conditions are fulfilled the size alignment procedure is complete

-	the total number of different DCI sizes configured to monitor is no more than 4 for the cell

-	the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell

Step 4:

-	Otherwise

Step 4A:

-	Remove the padding bit (if any) introduced in step 2 and step 2A above.

-	Determine DCI format 1_0 monitored in a UE-specific search space according to clause 7.3.1.2.1 where  is given by

-	the size of CORESET 0 if CORESET 0 is configured for the cell; and

-	the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

-	Determine DCI format 0_0 monitored in a UE-specific search space according to clause 7.3.1.1.1 where  is the size of the initial UL bandwidth part.

-	If the number of information bits in the DCI format 0_0 monitored in a UE-specific search space prior to padding is less than the payload size of the DCI format 1_0 monitored in UE-specific search space for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_0 monitored in a UE-specific search space until the payload size equals that of the DCI format 1_0 monitored in a UE-specific search space.

-	If the number of information bits in the DCI format 0_0 monitored in a UE-specific search space prior to truncation is larger than the payload size of the DCI format 1_0 monitored in UE-specific search space for scheduling the same serving cell, the bitwidth of the frequency domain resource assignment field in the DCI format 0_0 is reduced by truncating the first few most significant bits such that the size of DCI format 0_0 monitored in a UE-specific search space equals the size of the DCI format 1_0 monitored in a UE-specific search space.

Step 4B:

-	If the total number of different DCI sizes configured to monitor is more than 4 for the cell after applying the above steps, or if the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell after applying the above steps

-	If the number of information bits in the DCI format 0_2 prior to padding is less than the payload size of the DCI format 1_2 for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_2 until the payload size equals that of the DCI format 1_2.

-	If the number of information bits in the DCI format 1_2 prior to padding is less than the payload size of the DCI format 0_2 for scheduling the same serving cell, zeros shall be appended to the DCI format 1_2 until the payload size equals that of the DCI format 0_2.

Step 4C:

-	If the total number of different DCI sizes configured to monitor is more than 4 for the cell after applying the above steps, or if the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell after applying the above steps

-	If the number of information bits in the DCI format 0_1 prior to padding is less than the payload size of the DCI format 1_1 for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_1 until the payload size equals that of the DCI format 1_1.

-	If the number of information bits in the DCI format 1_1 prior to padding is less than the payload size of the DCI format 0_1 for scheduling the same serving cell, zeros shall be appended to the DCI format 1_1 until the payload size equals that of the DCI format 0_1.

Step 4D:

-	If DCI format 3_0 and/or DCI format 3_1 is monitored in a UE-specific search space and the size of the DCI format 3_0/3_1 does not equal that of any other DCI format configured to monitor for the cell, zeros shall be appended to the DCI format 3_0 and/or DCI format 3_1 until the payload size equal the smallest value among the sizes of other DCI formats configured to monitor for the cell which are larger than that of the DCI format 3_0/3_1 prior to padding.

The UE is not expected to handle a configuration that, after applying the above steps, results in

-	the total number of different DCI sizes configured to monitor is more than 4 for the cell; or

-	the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell; or

-	the size of DCI format 0_0 in a UE-specific search space is equal to DCI format 0_1 in another UE-specific search space; or

-	the size of DCI format 1_0 in a UE-specific search space is equal to DCI format 1_1 in another UE-specific search space; or

-	[the size of DCI format 0_0 in a UE-specific search space is equal to DCI format 0_2 in another UE-specific search space; or]

-	[the size of DCI format 1_0 in a UE-specific search space is equal to DCI format 1_2 in another UE-specific search space.]

#### < Unchanged parts are omitted >

TP for subclause 16.5 in TS38.213 on timing determination of PUCCH for SL A/N to gNB

#### < Unchanged parts are omitted >

#### 16.5	UE procedure for reporting HARQ-ACK on uplink

A UE can be provided PUCCH resources [12, TS 38.331] to report HARQ-ACK information to a serving gNB that the UE generates based on HARQ-ACK information that the UE obtains from PSFCH receptions, or from absence of PSFCH receptions.

For SL configured grant Type 1 or Type 2 PSSCH receptions by a UE within a time period provided by periodSlCG, the UE generates one HARQ-ACK information bit in response to the PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource, in a set of time resources.

For each PSFCH reception occasion, from a number of PSFCH reception occasions that the UE generates HARQ-ACK information to report to the gNB in a PUCCH or PUSCH transmission, the UE can be indicated by higher layers to perform one of the following and the UE constructs a HARQ-ACK codeword with HARQ-ACK information, when applicable.

-	generate HARQ-ACK information with same value as a value of HARQ-ACK information the UE determines from a PSFCH reception in the PSFCH reception occasion and, if the UE determines that a PSFCH is not received at the PSFCH reception occasion, generate NACK

-	generate ACK when the UE determines ACK from each PSFCH reception for the number of PSFCH reception occasions; otherwise, generate NACK if the UE determines absence of PSFCH reception or determines a NACK value from a PSFCH reception at a corresponding PSFCH reception occasion

-	generate ACK when the UE determines absence of PSFCH reception for each PSFCH reception occasion from the number of PSFCH reception occasions; otherwise, generate NACK

With reference to slots for PUCCH transmissions and for slot n which is the last slot in Uu that overlap(s) with the ending slot of a number of PSFCH reception occasions in sidelink, the UE provides the generated HARQ-ACK information in a PUCCH transmission within slot n+k, subject to the overlapping conditions in Clause 9.2.5, where k is a number of slots indicated by a PSFCH-to-HARQ_feedback timing indicator field, if present, in a DCI format indicating a slot for PUCCH transmission to report the HARQ-ACK information, or k is provided by sl-ACKToUL-ACK. k=0 corresponds to a last slot for a PUCCH transmission that would overlap with the last PSFCH reception occasion.

For a PSSCH reception by a UE that is scheduled by a DCI format, or for a SL configured grant Type 2 PSCCH reception activated by a DCI format, the DCI format can indicate to the UE that a PUCCH resource is not provided. For a SL configured grant Type 1 PSSCH reception, a PUCCH resource can be provided PUCCH-SL-Config. If a PUCCH resource is not provided, the UE does not transmit a PUCCH with generated HARQ-ACK information from PSFCH reception occasions.

#### < Unchanged parts are omitted >