3GPP TSG RAN WG1 Meeting #100	R1- 2000879

Athens, Greece, February 24th – 28th, 2020

Source:	Sharp

Title:	Remaining issues on synchronization mechanism for NR sidelink

Agenda Item:	7.2.4.3

Document for:	Discussion and Decision

Introduction

In this document, we share our views on a few remaining issues on synchronization mechanism for NR sidelink.

Discussion

DFN derivation from GNSS

In LTE V2X, when the UE selects GNSS as the synchronization reference source, the DFN and subframe used for SL communication is derived from the current UTC time as follows,

DFN= Floor (0.1*(Tcurrent –Tref–offsetDFN)) mod 1024 SubframeNumber= Floor (Tcurrent –Tref–offsetDFN) mod 10

In NR SL, the DFN derivation procedure in LTE V2X should be reused, with the addition of clarifying how the slot number within a subframe is derived, which in our understanding should be done as follows,

SlotNumber = Floor (Nslotsubframe,μ * (Tcurrent – Tref – offsetDFN)) mod Nslotsubframe,μ

The reuse of LTE V2X formulae has been captured in the latest running CR to TS 38.331, with derivation of slot number still missing. Therefore, RAN1 should agree on clarification of slot number derivation and ask RAN2 to complete this piece in TS 38.331.

Proposal 1: For DFN derivation from GNSS in NR SL,

- The formulae for DFN and SubframeNumber are reused from LTE V2X.

- Slot number is derived as follows,

  - SlotNumber = Floor (Nslotsubframe,μ * (Tcurrent – Tref – offsetDFN)) mod Nslotsubframe,μ

- Send an LS to request RAN2 to capture the above in TS 38.331.

Determination of in-coverage and out-of-coverage

In LTE SL, according to section 11.4, TS 36.304, a UE should determine “in-coverage” and “out-of-coverage” for sidelink operation as follows,

When UE is interested to perform sidelink communication or sidelink discovery announcement on non-serving frequency, it shall perform measurements on that frequency for cell selection and intra-frequency reselection purpose in accordance with TS 36.133 [10]. When UE is interested to perform V2X sidelink communication on non-serving frequency, it may perform measurements on that frequency or the frequencies which can provide inter-carrier V2X sidelink configuration for that frequency for cell selection and intra-frequency reselection purpose in accordance with TS 36.133 [10]. If the UE detects at least one cell on the frequency which UE is configured to perform sidelink operation on fulfilling the S criterion in accordance with clause 11.4.1, it shall consider itself to be in-coverage for sidelink operation on that frequency. If the UE cannot detect any cell on that frequency meeting the S criterion, it shall consider itself to be out-of-coverage for sidelink operation on that frequency.

It can be seen that for determination of in-coverage or out-of-coverage for sidelink operation, a UE has to perform cell measurements and cell selection, and for this purpose a DL carrier frequency has to be first derived from the SL carrier configuration.

In LTE SL, and for TDD, it is obvious that the same carrier frequency as the SL carrier should be used as the DL carrier frequency. For FDD, it is specified in section 9.3.2 of TS 36.331 that the DL carrier frequency is determined from the default TX-RX frequency separation.

In NR SL, one difference comparing to LTE SL is that, in case the SL carrier corresponds to a SUL carrier in a TDD cell, the DL carrier frequency of the cell cannot be derived from the SL carrier frequency. Furthermore, unlike in LTE where the synchronization signals can be located based on the carrier frequency (i.e. the centre frequency of the carrier), in NR, the SSB of a cell can be located anywhere in the carrier bandwidth (and cannot be derived from the centre frequency of the carrier).

Therefore, in our view, it is still unclear how the UE can determine the DL carrier frequency (to perform measurements) based on a pre-configured SL carrier frequency.

Proposal 2: RAN1 further discusses how a UE determines the DL carrier frequency (to perform measurements) based on a pre-configured SL carrier frequency.

Conclusion

In this contribution, we discuss some remaining issues on synchronization for NR sidelink, and make the following proposals:

Proposal 1: For DFN derivation from GNSS in NR SL,

- The formulae for DFN and SubframeNumber are reused from LTE V2X.

- Slot number is derived as follows,

  - SlotNumber = Floor (Nslotsubframe,μ * (Tcurrent – Tref – offsetDFN)) mod Nslotsubframe,μ

- Send an LS to request RAN2 to capture the above in TS 38.331.

Proposal 2: RAN1 further discusses how a UE determines the DL carrier frequency (to perform measurements) based on the pre-configured SL carrier frequency.

References

RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei, 3GPP RAN#83.