3GPP TSG RAN WG1 #100-e			R1-2000910

e-Meeting, February 24th – March 6th, 2020

Source:	NTT DOCOMO, INC.

Title:	Maintenance for Channel Structure for Two-Step RACH

Agenda Item:	7.2.1.1

Document for: 	Discussion and Decision

- Introduction

At the RAN1#99 meeting, the remaining issues related to channel structure for 2-step RACH were discussed and RAN1 made agreements [1]. In this contribution, we propose the corrections on Rel-16 specification regarding channel structure for 2-step RACH.

- Discussion

  - RACH configuration index

msgA-prach-ConfigurationIndexNew was introduced to RRC parameter in order to indicate new prach configuration index in Table 6.3.3.2-3, i.e., for unpaired spectrum and FR1, in TS 38.211, which is introduced in Rel-16 TEI. Rel-16 UE behaviour related to new PRACH configuration index in case of 4-step RACH or 2-step RACH is following.

- If UE is indicated new PRACH configuration index, i.e., prach-ConfigurationIndexNew for 4-step RACH or msgA-prach-ConfigurationIndexNew for 2-step RACH, UE transmits preamble in the time resources indicated by new PRACH configuration index, i.e., an index within the range from  256 to 262.

- Otherwise, UE transmits preamble in the time resources indicated by existing PRACH configuration index, i.e., an index within the range from 0 to 255 indicated by prach-ConfigurationIndex for 4-step RACH or msgA-prach-ConfigurationIndex for 2-step RACH.

However, according to the current Rel-16 specification, the description for 4-step RACH is correct, but on the other hand, the description for 2-step RACH looks unclear. Although a UE should use either msgA-prach-ConfigurationIndex or msgA-prach-ConfigurationIndexNew, the specification describes “given by msgA-prach-ConfigurationIndex and msgA-prach-ConfigurationIndexNew if configured”.

In addition, msgA-prach-ConfigurationIndexNew can be applied only to Table 6.3.3.2-3, i.e., for unpaired spectrum and FR1. However, the current specification describes that msgA-prach-ConfigurationIndexNew can be applied to other tables, i.e., Tables 6.3.3.2-2 and 6.3.3.2-4.

Furthermore, either prach-ConfigurationIndexNew or prach-ConfigurationIndex is applied for 2-step RACH if both msgA-prach-ConfigurationIndex and msgA-prach-ConfigurationIndexNew are not configured.

Above points should be clarified and corrected.

Proposal 1: Following text proposal is applied to section 6.3.3.2 in TS 38.211.

#### 6.3.3.2 Mapping to physical resources ~ Random access preambles can only be transmitted in the time resources obtained from Tables 6.3.3.2-2 to 6.3.3.2-4 and depends on FR1 or FR2 and the spectrum type as defined in [8, TS38.104]. The PRACH configuration index in Tables 6.3.3.2-2 to 6.3.3.2-4 is - for Table 6.3.3.2-3 - for type-2 random access procedure if both msgA-prach-ConfigurationIndex and msgA-prach-ConfigurationIndexNew are not configured or for a type-1 random access procedure, given by the higher-layer parameter prach-ConfigurationIndexNew if configured, otherwise by the higher-layer parameter prach-ConfigurationIndex, or by msgA-prach-ConfigurationIndex and msgA-prach-ConfigurationIndexNew if configured; and - otherwise, given by the higher-layer parameter msgA-prach-ConfigurationIndexNew if configured, otherwise by the higher-layer parameter msgA-prach-ConfigurationIndex; and - for Tables 6.3.3.2-2 and 6.3.3.2-4 given by the higher-layer parameter prach-ConfigurationIndex or by msgA-prach-ConfigurationIndex if configured for a type-2 random access procedure and msgA-prach-ConfigurationIndexNew if configured. ~

  - MsgA PUSCH configuration

The current Rel-16 specification describes that “If the active UL BWP is not the initial UL BWP and msgA-PUSCH-config is not provided for the active UL BWP, the UE uses the msgA-PUSCH-config provided for the initial active UL BWP.” The current specification does not use the term of “initial active UL BWP”, and it should be “initial UL BWP”.

Proposal 2: Following text proposal is applied to section 8.1A in TS 38.213.

#### 8.1A PUSCH for Type-2 random access procedure ~ A UE determines time resources and frequency resources for PUSCH occasions in an active UL BWP from msgA-PUSCH-config for the active UL BWP. If the active UL BWP is not the initial UL BWP and msgA-PUSCH-config is not provided for the active UL BWP, the UE uses the msgA-PUSCH-config provided for the initial active UL BWP. ~

  - Frequency hopping for MsgA PUSCH

Regarding frequency hopping, intra-slot frequency hopping can be applied to MsgA PUSCH. If intra-slot frequency hopping is applied to MsgA PUSCH, for the mapping between a preamble and PUSCH occasion, it would be unclear according to current Rel-16 specification whether a preamble is mapped on a set of the first hop and the second hop or on either the first hop or the second hop. Thus, the definition of a PUSCH occasion should be clarified, and a PUSCH occasion consists of the first hop and the second hop based on the concept of existing frequency hopping so that it becomes clear that a preamble is mapped on a set of the first hop and the second hop in case of intra-slot frequency hopping.

Proposal 3: Following text proposal is applied to section 8.1A in TS 38.213.

#### 8.1A PUSCH for Type-2 random access procedure ~ For a PUSCH transmission with frequency hopping in a slot, when indicated by msgA-intraSlotFrequencyHopping for the active UL BWP, the frequency offset for the second hop [6, TS 38.214] is determined as described in Clause 8.3, Table 8.3-1 using msgA-HoppingBits instead of . If guardPeriodMsgAPUSCH is provided, a first symbol of the PUSCH transmission after frequency hopping is separated by guardPeriodMsgAPUSCH symbols from a last symbol of the PUSCH transmission before frequency hopping; otherwise, there is no time separation of the PUSCH transmission before and after frequency hopping. A PUSCH transmission uses a same spatial filter as an associated PRACH transmission. A PUSCH occasion with frequency hopping consists of the first hop and the second hop. ~

- Conclusion

In this contribution, maintenance for channel structure of MsgA for 2-step RACH was discussed. Based on the discussion, the following proposals were made:

Proposal 1: Following text proposal is applied to section 6.3.3.2 in TS 38.211.

#### 6.3.3.2 Mapping to physical resources ~ Random access preambles can only be transmitted in the time resources obtained from Tables 6.3.3.2-2 to 6.3.3.2-4 and depends on FR1 or FR2 and the spectrum type as defined in [8, TS38.104]. The PRACH configuration index in Tables 6.3.3.2-2 to 6.3.3.2-4 is - for Table 6.3.3.2-3 - for type-2 random access procedure if both msgA-prach-ConfigurationIndex and msgA-prach-ConfigurationIndexNew are not configured or for a type-1 random access procedure, given by the higher-layer parameter prach-ConfigurationIndexNew if configured, otherwise by the higher-layer parameter prach-ConfigurationIndex, or by msgA-prach-ConfigurationIndex and msgA-prach-ConfigurationIndexNew if configured; and - otherwise, given by the higher-layer parameter msgA-prach-ConfigurationIndexNew if configured, otherwise by the higher-layer parameter msgA-prach-ConfigurationIndex; and - for Tables 6.3.3.2-2 and 6.3.3.2-4 given by the higher-layer parameter prach-ConfigurationIndex or by msgA-prach-ConfigurationIndex if configured for a type-2 random access procedure and msgA-prach-ConfigurationIndexNew if configured. ~

Proposal 2: Following text proposal is applied to section 8.1A in TS 38.213.

#### 8.1A PUSCH for Type-2 random access procedure ~ A UE determines time resources and frequency resources for PUSCH occasions in an active UL BWP from msgA-PUSCH-config for the active UL BWP. If the active UL BWP is not the initial UL BWP and msgA-PUSCH-config is not provided for the active UL BWP, the UE uses the msgA-PUSCH-config provided for the initial active UL BWP. ~

Proposal 3: Following text proposal is applied to section 8.1A in TS 38.213.

#### 8.1A PUSCH for Type-2 random access procedure ~ For a PUSCH transmission with frequency hopping in a slot, when indicated by msgA-intraSlotFrequencyHopping for the active UL BWP, the frequency offset for the second hop [6, TS 38.214] is determined as described in Clause 8.3, Table 8.3-1 using msgA-HoppingBits instead of . If guardPeriodMsgAPUSCH is provided, a first symbol of the PUSCH transmission after frequency hopping is separated by guardPeriodMsgAPUSCH symbols from a last symbol of the PUSCH transmission before frequency hopping; otherwise, there is no time separation of the PUSCH transmission before and after frequency hopping. A PUSCH transmission uses a same spatial filter as an associated PRACH transmission. A PUSCH occasion with frequency hopping consists of the first hop and the second hop. ~

References

- 3GPP RAN1#99, Chairman’s note, November 2019.