3GPP TSG RAN WG1 #100-e			R1-2000912

e-Meeting, February 24th – March 6th, 2020

Source:	NTT DOCOMO, INC.

Title:	Channel access procedures for NR-U

Agenda Item:	7.2.2.2.1

Document for: 	Discussion and Decision

- Introduction

In this contribution, remaining aspects of channel access procedures are discussed.

- CP extension value before RRC configuration

At the RAN1#99 meeting, following agreements have been made [1]:

Agreement: C2 &C3 (from prior agreement on CP extension) are UE specifically RRC configured - Note: this does not restrict the maximum TA that can be used in a cell - FFS: Agree the values of C2 and C3 in RAN1#99 Agreement: The supported range of values of both C2 and C3 (from prior agreement on CP extension) that can be configured by RRC is - 1, 2, …, 28 for 15 and 30 kHz SCS, and - 2, 3, …, 28 for 60 kHz SCS Note: Other agreements are not superseded by this agreement Agreement: At least for LBE operation: - For signaling of LBT type & CP extension for both Fallback DL assignment and Fallback UL Grant, the following table is used: LBT Type; CP extension. Cat1 16 µs; C2*symbol length – 16 us – TA. Cat2 25 µs; C3*symbol length – 25 us – TA. Cat2 25 µs; C1*symbol length – 25 us. Cat4; 0. - CAPC is not indicated explicitly: - For the UL grants - The UE assumes CAPC=4 was used by the gNB to acquire the CO, - For UE initiated COTs (Cat4 case) the UE may select the CAPC by itself. - Note: The mapping between priority classes and traffic classes follows the same mechanism as defined for UL CG transmissions. - Note: For PUCCH associated with DL assignments may use the highest priority CAPC when CAT4 LBT is used, as agreed earlier. - If the network indicates FBE operation, for an indication of LBT type of Cat 2 25 us or Cat4 the UE follows the mechanism whereby one 9 microsecond slot is measured within a 25 microsecond interval as in 37.213. Agreement: - For RAR, we use the same LBT type and CP extension tables and CAPC selection mechanism as used for the Fallback DCI UL grant. - This requires 2 bits to be signaled in RAR (the PDSCH) - If the UE multiplexes user plane data with PUSCH - The UE assumes CAPC=4 was used by the gNB to acquire the CO, - For UE initiated COTs (Cat4 case) the UE may select the CAPC by itself. - Note: The mapping between priority classes and traffic classes follows the same mechanism as defined for UL CG transmissions. - The frequency domain resource allocation field can be reduced to accommodate 2 bits in the RAR - Include this agreement in an LS to RAN2

As stated above, C2 and C3 are UE specifically RRC configured. However, the CP extension value including either C2 or C3 (corresponding to index 0 or 1 in Table 7.3.1.1.1-4 in TS 38.212) may be indicated by DCI format 0_0/0_1 or RAR before the RRC configuration, and UE doesn’t know which C2/C3 candidate (i.e., 1, 2, …, 28 for 15 and 30 kHz SCS, and 2, 3, …, 28 for 60 kHz SCS) should be used for the CP extension. The possible solutions would be:

- Option 1: UE is not expected to be indicated with the CP extension value including either C2 or C3 before RRC configuration

- Option 2: Default C2 and C3 values are defined

- Option 3: C2 and C3 before RRC configuration are determined by UE using TA value

Option 1 may limit scheduling flexibility due to limited available sets of LBT type and CP extension value. Option 2 may not be able to use appropriate C2/C3 values. For example, if default C2 and C3 are set to large values while TA value is small, the CP extension value becomes longer than 1 OFDM symbol. Even if TA value is set to be larger than necessary to avoid the CP extension value longer than 1 OFDM symbol, gNB cannot schedule one or multiple symbols due to the larger TA value. Therefore, C2 and C3 should be determined based on the TA value applied to the UE, i.e., Option 3 should be supported. More specifically, as CP extension value should not exceed symbol length, C2 and C3 should be set to the maximum integers which result in the CP extension value less than symbol length.

Proposal 1: C2 and C3 before RRC configuration are set to the maximum integers which satisfy following:

- C2*symbol length – 16 us – TA < symbol length

- C3*symbol length – 25 us – TA < symbol length

- Text proposal

Based on above proposal, following corrections would be necessary;

Proposal 2: Adapt following text proposal for TS 38.211 which reflects proposal 1 in this contribution.

== Start ==

### 5.3.1	OFDM baseband signal generation for all channels except PRACH and RIM-RS

The time-continuous signal  on antenna port p and subcarrier spacing configuration μ for OFDM symbol  in a subframe for any physical channel or signal except PRACH is defined by

sl(p,μ)t=sl(p,μ)ttstart,lμ≤t<tstart,lμ+Tsymb,lμ0otherwise
sl(p,μ)t=k=0Ngrid,xsize,μNscRB-1ak,l(p,μ)ej2πk+k0μ-Ngrid,xsize,μNscRB2Δft-NCP,lμTc-tstart,lμ
k0μ=Ngrid,xstart,μ+Ngrid,xsize,μ2NscRB-Ngrid,xstart,μ0+Ngrid,xsize,μ02NscRB2μ0-μ
Tsymb,lμ=Nuμ+NCP,lμTc

where t=0 at the start of the subframe,

and

-	 is given by clause 4.2;

-	 is the subcarrier spacing configuration;

-	μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList.

In case of cyclic extension of the first OFDM symbol l allocated for PUSCH transmission, the time-continuous signal sext(p,μ)t for the interval tstart,lμ-Text≤t<tstart,lμ preceding the first OFDM symbol for PUSCH is given by

sext(p,μ)t=sl(p,μ)t

where t<0 refers to the signal in the previous subframe and Text is given by

-	Table 5.3.1-1 with the index, C2 and C3 given by the higer-layer parameters CP-ExtensionC2-r16 and CP-ExtensionC3-r16, respectively, and TTA given by clause 4.3.1 for dynamically scheduled PUSCH transmissions if the UE is configured with CP-ExtensionC2-r16 and CP-ExtensionC3-r16;

-	Table 5.3.1-1 with the index, C2 and C3 being the maximum integers which satisfy C2Tsymb,lμ-16∙10-6-TTA<Tsymb,lμ and C3Tsymb,lμ-25∙10-6-TTA<Tsymb,lμ, respectively, and TTA given by clause 4.3.1 for dynamically scheduled PUSCH transmissions if the UE is not configured with CP-ExtensionC2-r16 or CP-ExtensionC3-r16;

-	the procedure in [6, TS 38.214] for a PUSCH transmission using configured grant.

The starting position of OFDM symbol  for subcarrier spacing configuration in a subframe is given by

Table 5.3.1-1: The cyclic extension Text.

| Index | μ=0 | μ=1 | μ=2 |
| --- | --- | --- | --- |
| 0 | - | - | - |
| 1 | Tsymb,lμ-25∙10-6 | Tsymb,lμ-25∙10-6 | 2Tsymb,lμ-25∙10-6 |
| 2 | C2Tsymb,lμ-16∙10-6-TTA | C2Tsymb,lμ-16∙10-6-TTA | C2Tsymb,lμ-16∙10-6-TTA |
| 3 | C3Tsymb,lμ-25∙10-6-TTA | C3Tsymb,lμ-25∙10-6-TTA | C3Tsymb,lμ-25∙10-6-TTA |

== End ==

- Conclusion

In this contribution, we discussed on the channel access procedure for NR-U. Based on the discussion, we made following proposals.

Proposal 1: C2 and C3 before RRC configuration are set to the maximum integers which satisfy following:

- C2*symbol length – 16 us – TA < symbol length

- C3*symbol length – 25 us – TA < symbol length

Proposal 2: Adapt text proposal in Section 3 for TS 38.211 which reflects proposal 1 in this contribution.

References

- 3GPP, RAN1#99, RAN1 Chairman’s Notes, November 2019.