3GPP TSG RAN WG1 #100                                                                                   R1-2000924

e-Meeting, February 24th – March 6th, 2020

Source:	NTT DOCOMO, INC.

Title:	PUCCH resource determination for HARQ-ACK for SPS PDSCH

Agenda Item:	7.2.5.7

Document for: 	Discussion and Decision

- Introduction

In this contribution, we discussed one issue related to the PUCCH resource determination for HARQ-ACK of SPS PDSCH for Rel.16 URLLC/IIoT with multiple SPS configurations.

- PUCCH resource determination for HARQ-ACK of SPS PDSCH

For a UE configured with multiple SPS PDSCH configurations, the PUCCH resource determination for HARQ-ACK bit(s) for SPS PDSCH without associated PDCCH is captured in TS 38.213 section 9.2.1 and 9.2.3 [1] as following:

## 9.2.1 PUCCH Resource Sets If the UE is provided more than one active SPS PDSCH configurations and transmits OUCI UCI information bits that include only HARQ-ACK information bits in response to one or more SPS PDSCH receptions, the UE is provided by SPS-PUCCH-AN-List a set of PUCCH resources and determines a PUCCH resource to be - a PUCCH resource with SPS-PUCCH-AN-ResourceID = 0 if OUCI≤2, or - a PUCCH resource with SPS-PUCCH-AN-ResourceID = 1, if provided, if 2<OUCI≤N1,SPS where N1,SPS is either provided by maxPayloadSize in SPS-PUCCH-AN-List for SPS-PUCCH-AN-ResourceID = 1 or is otherwise equal to 1706, or - a PUCCH resource with SPS-PUCCH-AN-ResourceID = 2, if provided, if N1,SPS<OUCI≤N2,SPS where N2,SPS is either provided by maxPayloadSize in SPS-PUCCH-AN-List for SPS-PUCCH-AN-ResourceID = 2 or is otherwise equal to 1706, or - a PUCCH resource with SPS-PUCCH-AN-ResourceID = 3, if provided, if N2,SPS<OUCI≤N3,SPS where N3,SPS is equal to 1706

## 9.2.3 UE procedure for reporting HARQ-ACK If a UE is provided a single active SPS PDSCH configuration and transmits HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH, a PUCCH resource for corresponding PUCCH transmission with HARQ-ACK information is provided by n1PUCCH-AN.

With above spec., for a UE configured with multiple SPS PDSCH configurations, whether PUCCH resource provided by SPS-PUCCH-AN-List or PUCCH resource provided by n1PUCCH-AN is used for HARQ-ACK of SPS PDSCH depends on the number of active SPS PDSCH configurations. However, the number of active SPS PDSCH configurations can be changed dynamically by SPS PDSCH release/activation. An example is given in Fig. 1. When a UE receives a SPS PDSCH of SPS configuration #2, it has two active SPS configurations (SPS configuration #1 and SPS configuration #2). While just before UE reports the HARQ-ACK for SPS PDSCH associated with SPS configuration #2, SPS configuration #1 is released. Therefore, there exists ambiguity between the gNB and UE regarding to the PUCCH resource e.g. provided by SPS-PUCCH-AN-List or by n1PUCCH-AN for HARQ-ACK of SPS PDSCH. Therefore, following TP is proposed:

Fig.1 PUCCH resource ambiguity when the number of active SPS configurations is dynamically changed

Proposal 1:

- Adopt following TP in TS 38.213 section 9.2.1 and 9.2.3

## 9.2.1 PUCCH Resource Sets If the UE is provided more than one configured SPS PDSCH configurations and transmits OUCI UCI information bits that include only HARQ-ACK information bits in response to one or more SPS PDSCH receptions, the UE is provided by SPS-PUCCH-AN-List a set of PUCCH resources and determines a PUCCH resource to be - a PUCCH resource with SPS-PUCCH-AN-ResourceID = 0 if OUCI≤2, or - a PUCCH resource with SPS-PUCCH-AN-ResourceID = 1, if provided, if 2<OUCI≤N1,SPS where N1,SPS is either provided by maxPayloadSize in SPS-PUCCH-AN-List for SPS-PUCCH-AN-ResourceID = 1 or is otherwise equal to 1706, or - a PUCCH resource with SPS-PUCCH-AN-ResourceID = 2, if provided, if N1,SPS<OUCI≤N2,SPS where N2,SPS is either provided by maxPayloadSize in SPS-PUCCH-AN-List for SPS-PUCCH-AN-ResourceID = 2 or is otherwise equal to 1706, or - a PUCCH resource with SPS-PUCCH-AN-ResourceID = 3, if provided, if N2,SPS<OUCI≤N3,SPS where N3,SPS is equal to 1706

## 9.2.3 UE procedure for reporting HARQ-ACK If a UE is provided a single configured SPS PDSCH configuration and transmits HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH, a PUCCH resource for corresponding PUCCH transmission with HARQ-ACK information is provided by n1PUCCH-AN.

- Conclusion

In this contribution, we discuss the one issue related to the PUCCH resource determination for HARQ-ACK of SPS PDSCH for Rel.16 URLLC/IIoT with multiple SPS configurations. Following is the proposed:

Proposal 1:

- Adopt following TP in TS 38.213 section 9.2.1 and 9.2.3

## 9.2.1 PUCCH Resource Sets If the UE is provided more than one configured SPS PDSCH configurations and transmits OUCI UCI information bits that include only HARQ-ACK information bits in response to one or more SPS PDSCH receptions, the UE is provided by SPS-PUCCH-AN-List a set of PUCCH resources and determines a PUCCH resource to be - a PUCCH resource with SPS-PUCCH-AN-ResourceID = 0 if OUCI≤2, or - a PUCCH resource with SPS-PUCCH-AN-ResourceID = 1, if provided, if 2<OUCI≤N1,SPS where N1,SPS is either provided by maxPayloadSize in SPS-PUCCH-AN-List for SPS-PUCCH-AN-ResourceID = 1 or is otherwise equal to 1706, or - a PUCCH resource with SPS-PUCCH-AN-ResourceID = 2, if provided, if N1,SPS<OUCI≤N2,SPS where N2,SPS is either provided by maxPayloadSize in SPS-PUCCH-AN-List for SPS-PUCCH-AN-ResourceID = 2 or is otherwise equal to 1706, or - a PUCCH resource with SPS-PUCCH-AN-ResourceID = 3, if provided, if N2,SPS<OUCI≤N3,SPS where N3,SPS is equal to 1706

## 9.2.3 UE procedure for reporting HARQ-ACK If a UE is provided a single configured SPS PDSCH configuration and transmits HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH, a PUCCH resource for corresponding PUCCH transmission with HARQ-ACK information is provided by n1PUCCH-AN.

References

- R1-1913645, ‘Introduction of Industrial IoT in TS 38.213’, Samsung

- 3GPP, RAN1 #99 meeting, chairman’s notes.