3GPP TSG RAN WG1 #100-e			R1-2000930

e-Meeting, February 24th – March 6th, 2020

Agenda item:	7.2.11

Source: 	AT&T, NTT DOCOMO, INC.

Title: 	RAN1 UE features list for Rel-16 NR

Document for:	Information

- Introduction

This is a document for email discussion/approval for Rel-16 NR UE features according to following chairman’s note at RAN1 #99 meeting. The updated Rel-16 NR UE features are provided by rapporteurs for each Rel-16 NR WI based on R1-1911751 and an outcome of the first phase email discussion on Rel-16 NR UE features after RAN1 #99 meeting. Companies are encouraged to check the updates and to provide feedbacks if any in the second phase email discussion on Rel-16 NR UE features which will start after RAN1#100-e meeting.

Email discussion/approval for Rel-16 NR UE features till next meeting – Hiroki/Xiaoyi

- NR_2step_RACH

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9. NR_2step_RACH | 9-1 | Basic channel structure and procedure of 2-step RACH | - RACH type selection based on a SSB-based RSRP threshold - Separate ROs with 4-step RO configuration - Same ROs and preamble partitioning with 4-step RO configuration - Single msgA PUSCH configuration in an UL BWP from UE perspective - Two msgA PUSCH configurations in an UL BWP from UE perspective - Validation of msgA PRACH and PUSCH - Mapping between preamble of MsgA PRACH and PUSCH resource units of MsgA PUSCH - msgA PUSCH scrambling - Multiple DMRS sequences for CP-OFDM based msgA PUSCH - Single DMRS sequence for DFT-s-OFDM based msgA PUSCH - msgA PRACH power control - msgA PUSCH power control - Monitoring of msgB including SuccessRAR and fallbackRAR, and PDCCH addressed to C-RNTI in response to a MsgA with C-RNTI - Support PUCCH transmission for HARQ-ACK feedback to a MsgB that contains successRAR and to a PDSCH transmission with the corresponding PDCCH addressed to C-RNTI in response to a MsgA with C-RNTI. The PUCCH is configured with the same numerology, TX spatial filter and UL BWP as the last msgA PUSCH transmission/retransmission - For MsgA PRACH support all preamble formats applicable to the band - For short sequence based preamble format in MsgA PRACH, support 15 kHz and 30 kHz in FR1 and 60 and 120 kHz in FR2. - For MsgA PUSCH, support 15 kHz, 30 kHz in FR1. Support 60 kHz and 120 kHz for FR2 - For MsgA PUSCH waveform, support both CP-OFDM and DFT-s-OFDM - Support DMRS ports with configuration type 1 - PUCCH power control for HARQ-ACK feedback to a MsgB - MSGA PRACH configuration - MSGA PRACH new configuration support (aligned with NR TEI 14-7) - Minimum TX gap between PRACH and PUSCH (for both TDD and FDD, FR1 and FR2, single CC and intra-band CA) - Minimum TX gap between last DL SSB reception symbol and PRACH (TDD. FR1 and FR2, single CC and intra-band CA) - MsgA PRACH and PUSCH in different slots - Minimum TX gap between the last symbol of msgB PDSCH and the first symbol of PUCCH carrying HARQ-ACK to msgB PDSCH - Support RV=0 for msgA PUSCH initial transmission, re-attempt and fall-back - 2-step RACH operation in RRC_IDLE/INACTIVE state - 2-step RACH operation in RRC_CONNECTED state |  | Yes | N/A | UE cannot initiate a 2-step RACH process, and thus would not be expected understand the 2-step RACH configurations | per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 9-2 | Supported 2 symbols DMRS for msgA PUSCH | Supported 2 symbols DMRS for msgA PUSCH (‘len2’) | 9-1, 2-18 | Yes | N/A | If UE does not support ‘len2’, and if msgA-maxLength is configured as ‘len2’, the UE cannot use 2-step RACH resources | per UE | No | Yes |  | Shall be aligned with 2-18 | Optional with capability signalling |
|  | 9-3 | Parallel MsgA and PRACH SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA | Parallel MsgA and PRACH SRS./PUCCH/PUSCH transmissions across CCs in inter-band CA with msgA in PCell/PScell | 9-1, 2-52, 4-1, 2-12, 6-6 | Yes | N/A | UE cannot transmit an MsgA and other UL transmissions in parallel across CCs in inter-band CA | per band combination | No | No |  |  | Optional with capability signalling |
|  | 9-4 | MsgA operation in a band combination including SUL | MsgA operations in a band combination including SUL | 9-1, 6-16 | Yes | N/A | UE does not support msgA operations in a band combination including SUL | per band combination | N/A | N/A |  |  | Optional with capability signalling |
|  | 9-5 | Support 60kHz for MsgA PUSCH in FR1 | Support 60kHz for MsgA PUSCH in FR1 | 9-1, 1-1 | Yes | N/A |  | per UE | No | Only applied for FR1 |  | Shall be aligned with 1-1 | Optional with capability signalling |
|  | 9-6 | Intra-slot frequency hopping for msgA PUSCH | Intra-slot frequency hopping for msgA PUSCH | 9-1 | Yes | N/A |  | per band | N/A | N/A |  |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | The basic feature 9-1 is well defined and it sets a good example on how to define features, though we understand this type of structure is not feasible for all WIDs. Otherwise no technical comments on the features listed under this WID. Type in feature 9-1, change “2-tep” to “2-step” |
| Samsung | 9-1) For clarity, clarify that component 18) applies for short sequence, i.e., “18) For short sequence based preamble format in msgA PRACH, ...” New) The following agreement would require separate FG for msgA-maxLength (rather than to include this in FG9-1) Agreement (RAN1#99): - Regarding the parameter msgA-maxLength - Keep the agreement for msgA-maxLength, if UE does not support ‘len2’, and if msgA-maxLength is configured as ‘len2’, the UE cannot use 2-step RACH resources. - Note: it is UE capability issue, no spec impact. |
| NTT DOCOMO | As FG9-1/2/3 are per band or per band combination reporting, FDD/TDD differentiation and FR1/FR2 differentiation would not be necessary. |
| CATT | For 9-1, 4 more components needs to be added: 1) PUCCH power control for HARQ-ACK feedback to a MsgB 2) MSGA PRACH configuration 3) MSGA PRACH new configuration support (aligned with NR TEI 14-7) 4) Timing advance adjustment for MSGA PUSCH For 9-2, we suggest to modify “Parallel MsgA and PRACH Msg1/SRS./PUCCH/PUSCH transmissions across CCs in inter-band CA” to “Parallel MsgA and PRACH Msg1/SRS./PUCCH/PUSCH transmissions across CCs in inter-band CA with MsgA in Pcell/Pscell” based on RAN1 agreement “ MsgA is not supported in Scell for CA/DC case” in RAN1#99 meeting. |
| MediaTek | - For FG9-1, we suggest to remove 60kHz from item #19. - Add feature FG9-4 as follows Index; Feature group; Components; Prerequisite feature groups; Per; Mandatory/Optional. 9-4; Support 60kHz for MsgA PUSCH; - Support 60kHz for MsgA PUSCH in FR1 - Support 60kHz for MsgA PUSCH in FR2; 9-1; Per BC; Optional with capability signalling. |
| Huawei, HiSilicon | - The column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2” is not applied for 2-step RACH, i.e. N/A, as MsgA on SCell is not supported. - On FG#9-1, - Item 2) is not needed since fallback can be reflected by item 1) and item 15), with item 15) modified to “monitoring of MsgB including SuccessRAR and Fallback RAR”. - Item 4) needs clarification to be limited to fully shared case. Subset sharing of ROs with 4-step configuration should be a separate UE capability - Item 6) should be a separate UE capability, as FH in Rel-15 - Item 7) can be split into two: one is the UE capability of single MsgA PUSCH configuration, and the other is up to two MsgA PUSCH configurations as a separate UE capability - Item 9) is not aligned with agreements wherein MsgA PRACH to PRU mapping is defined. Suggest to update it as mapping between preamble of MsgA PRACH and PUSCH resource units of MsgA PUSCH - Item 14) seems not needed, as it can be considered as part of item 15). - Item 15), we need to further discuss whether to define the limit of the total number MsgB to be monitored per slot, since MsgB could be successRAR dedicated to another UE or could carry RRC content with large payload, which leads to an increased decoding efforts compared to msg2. - On FG 9-2 PRACH Msg1 can be removed as there seems be no use case of parallel MsgA and Msg1 transmission. This is also in line with FG 4-26, i.e. there are no parallel two Msg1 transmissions. |
| Panasonic | - On 9-1 19), we think 60 kHz SCS in FR1 should be optional depending on the capability indication of scs-60kHz. It should be clarified that the SCS supported by PUSCH is also supported in Msg.A depending on FR1/2. - It looks no conclusion on extended CP. It should be clarified. If extended CP is supported, it is supported only when UE feature 0-5 is supported. |
| Ericsson | 9-1 is rather more detailed than we see needed. It would be sufficient with the following components: - msgA PRACH & PUSCH transmission - msgB reception If a detailed list is still used, we have the following comments: - Some capabilities might be merged, - e.g. 5) seems redundant if we have 9), as does 15) if we have 14). Regarding 19) it seems simpler to us to omit 60 kHz rather than make it optional. |
| Intel | For 9-1, 15), we suggest to update it as “Monitoring of msgB and PDCCH addressed to C RNTI in response to a MsgA with C-RNTI” to add procedure for RRC_CONNECTED mode. For 16), we suggest to update it as “HARQ-ACK feedback to a MsgB that contains successRAR and to a PDSCH transmission with the corresponding PDCCH addressed to C-RNTI in response to a MsgA with C-RNTI” to add procedure for RRC_CONNECTED mode |
| Apple | For sub-feature 9-1, similar as component 11), the component 20) can replace with “single DMRS sequence for DFT-s-OFDM based msgA PUSCH”. We would like to introduce additional two components of 9-1 as showing below 2-step RACH operation in RRC_IDLE/INACTIVE state 2-step RACH operation in RRC_CONNECTED state The reason is the UE behavior of msgB HARQ-ACK feedback and time domain resource allocation is different in different RRC state. |
| Qualcomm | Suggested changes on 9-1: - RACH type selection based on RSRP threshold - Fall back from 2-step RACH to 4-step RACH - Separate RO with 4-step RO configuration - Shared RO with 4-step RO configuration - msgA PUSCH occasions configuration - Intra-slot frequency hopping for msgA PUSCH - Up to two msgA PUSCH configurations in an UL BWP from UE perspective - Periodic validation of msgA PRACH and PUSCH occasion per SSB to RO association period - Periodic mapping between preamble and PUSCH occasion with DM-RS resource per SSB to RO association period - msgA PUSCH scrambling with ID set extension - Multiple DMRS sequences for msgA PUSCH - msgA PRACH power control - msgA PUSCH power control - msgB DCI format with CRC scrambled by msgB-RNTI and indication of 2 LSBs ofSFN - Monitoring of msgB - Support PUCCH transmission for HARQ-ACK feedback to a MsgB that contains successRAR, and the PUCCH is configured with the same numerology, TX spatial filter and UL BWP as the last msgA PUSCH transmission/retransmission - For MsgA PRACH support all preamble formats applicable to the band - For MsgA PRACH, support the same set of numerologies as for 4-step RACH in the same band - For MsgA PUSCH, support the same set of numerologies as for Msg3 in the same band - For MsgA PUSCH waveform, support both CP-OFDM and DFT-s-OFDM - Support DMRS ports with configuration type 1. - Minimum TX gap between PRACH and PUSCH (for both TDD and FDD, FR1 and FR2, single CC and intra-band CA) - Minimum TX gap between last DL SSB reception symbol and PRACH (TDD. FR1 and FR2, single CC and intra-band CA) - MsgA PRACH and PUSCH in different slots - Support 2-step RACH on PCell or PSCell after UL carrier selection - Minimum TX gap between the last symbol of msgB PDSCH and the first symbol of PUCCH carrying HARQ-ACK to msgB PDSCH - Support msgA-prach-ConfigurationIndexNew in FR1: INTEGER (256.. 262). - Support RV=0 for msgA PUSCH initial transmission, re-attempt and fall-back - msgB PDSCH multiplexing different types of RARs and BI, including 12-bit MAC CE for timing advance command, 3-bit PDSCH-to-HARQ FB timing indicator, 4-bit PUCCH resource indicator and new MAC subheader Add two feature groups: “2-step RACH on SCell in mTAG” and “2-step RACH on SCell in sTAG” |
| Spreadtrum Communications | For 9-1, Clarify the following components 1) a new rsrp-ThresholdSSB-2stepRach is used for RACH type selection from 4-step RACH and 2-step RACH, we have the following comments: - RACH type selection based on a new SSB-based RSRP threshold. 9) preamble and PRU mapping should be within an msgA association period, we have the following comments: Preamble of MsgA PRACH and PUSCH resource units of MsgA PUSCH mapping within an msgA association period |

- NR-unlicensed

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-1 | UE channel access procedures for UL transmission for dynamic channel access mode | 1. Type 1 channel access 2. Type 2A channel access 3. Type 2B channel access 4. Type 2C channel access 5. 20MHz LBT bandwidth 6. Contention window adjustment |  |  | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band with both DL and UL transmission support under dynamic channel access | Optional with capability signalling |
|  | 10-1a | UE DL only operation in unlicensed operation with dynamic channel access mode |  |  |  | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band with DL only operation | Optional with capability signalling |
|  | 10-2 | UE channel access procedures for UL transmission for semi-static channel access mode | 1. Type 2C channel access 2. Single sensing slot of 9us channel access 3. 20MHz LBT bandwidth |  |  | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band. Support of channel access mechanism for FBE operation, including fixed frame period, Cat 2 LBT, Cat 1 LBT | Optional with capability signalling |
|  | 10-3 | PRB interlace mapping for PUCCH and PUSCH | 1) PRB interlace mapping for PUCCH (EPF0, EPF1, EPF2, EPF3) 2) PRB interlace frequency domain resource allocation for PUSCH | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band. Support of PRB interlace PUCCH and PUSCH | Optional with capability signalling |
|  | 10-7 | LBT bandwidth size of 10MHz |  | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-8 | Type B PDSCH length | Length 3, 5, 6, 8, 11, 12, 13 FFS the capability are separate for each length or some groups are formed to signal the capability together |  | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A |  | Support of additional length (other than 2/4/7) for type B PDSCH | Optional with capability signalling |
|  | 10-9 | Search space set group switching | Two groups of search space sets | 10-1 or 10-2 | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A |  | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
|  | 10-10 | RSSI measurement | RSSI measurement | 10-1 or 10-2 | Yes | N/A |  | per band | N/A | N/A |  | RSSI measurement for channel occupancy. | Optional with capability signalling |
|  | 10-11 | SRS starting position at any OFDM symbol in a slot |  | 10-1 or 10-2 | Yes | N/A |  | Per band or per UE | N/A | N/A |  | Support transmitting SRS starting in all symbols (0,…,13) of a slot | Optional with capability signalling |
|  | 10-12 | OCC for PRB interlace mapping for PF2 and PF3 FFS if need this capability | 1. No OCC 2. OCC2 3. OCC4 | 10-3 | Yes | N/A |  | Per band | N/A | N/A |  | UE OCC capability for EPF2/EFP3 | Optional with capability signalling |
|  | 10-13 | CP extension for the first symbol of PUSCH/PUCCH FFS if need this capability |  | 10-1 Do we need this for 10-2? | Yes | N/A |  | Per band | N/A | N/A |  | Support CP extension of at most one OFDM symbol duration preceding the first symbol of a scheduled PUSCH/PUCCH transmission subject to UE processing time defined in Rel-15 | Optional with capability signalling |
|  | 10-13a | Extended CP range for CG-PUSCH FFS if need this capability |  | 10-1 Do we need this for 10-2? | Yes | N/A |  | Per band | N/A | N/A |  | How long a UE can generate the CP extension beyond 1 symbol for CG-PUSCH | Optional with capability signalling |
|  | 10-14 | Non-numerical PDSCH to HARQ-ACK timing | Non-numerical value for dl-DataToUL-ACK | 10-1 or 10-2 | Yes | N/A |  | Per band or per UE | N/A | N/A |  | If non-numerical K1 value is supported | Optional with capability signalling |
|  | 10-15 | Enhanced dynamic HARQ codebook | Support of bit fields signalling PDSCH HARQ group index and NFI in DCI 1_1 | 10-1 or 10-2 | Yes | N/A |  | Per band or per UE | N/A | N/A |  | Enhanced dynamic HARQ codebook supporting grouping of HARQ ACK and triggering the retransmission of HARQ ACK in each groups | Optional with capability signalling |
|  | 10-16 | One-shot HARQ ACK feedback | HARQ-ACK codebook containing all configured HARQ processes for all configured CCs | 10-1 or 10-2 | Yes | N/A |  | Per band or per UE | N/A | N/A |  | Upon triggering, UE reports A/N for all HARQ processes and all CCs in a PUCCH group. | Optional with capability signalling |
|  | 10-16a | One-shot HARQ ACK feedback trigger with empty DCI 1_1 | Support of one-shot HARQ ACK feedback with a DCI 1_1 without scheduling a PDSCH using a reserved FDRA value | 10-1 or 10-2 | Yes | N/a |  | Per band or per UE | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-17 | Multi-PUSCH UL grant | Support of scheduling more than one PUSCH with a single DCI 0_1 | 10-1 or 10-2 FFS if applicable to licensed? | Yes | N/A |  | Per band or per UE | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-18a | Configured grant with retransmission in CG resources with LBE | Configured grant with retransmission in CG resources with LBE UE transmits multiple PUSCHs in a configured grant period according to the RRC configuration for both type 1 and type 2 configured grant | 10-1 | Yes | N/A |  | Per band | N/A | N/A |  | Support configured grant with retransmission in configured grant resource | Optional with capability signalling |
|  | 10-18b | Configured grant with retransmission in CG resources with FBE FFS if 10-18a and 10-18b should be merged | Configured grant with retransmission in CG resources with FBE UE transmits multiple PUSCHs in a configured grant period according to the RRC configuration for both type 1 and type 2 configured grant | 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | Support configured grant with retransmission in configured grant resource | Optional with capability signalling |
|  | 10-19 | Number of LBT bandwidth FFS if this is needed | Number of ED measurements the UE is able to perform simultaneously [Whether the UE in WB operating mode can support nx20MHz measurement | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | This is the number of LBT bandwidth a UE can perform separate ED check on simultaneously | Optional with capability signalling |
|  | 10-20 | Support search space set association with multiple frequency domain locations | 1. Support coreset configuration with rb-Offset and replicability of coreset configuration where the coreset is confined within one LBT bandwidth 2. Support search space set configuration with freqMonitorLocations-r16 | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | This is also the capability to support coreset configuration with rb-Offset | Optional with capability signalling |
|  | 10-21 | Support using ED threshold for UL to DL COT sharing | 1. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for scheduled UL to share COT with gNB for DL 2. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for CG-PUSCH to share COT with gNB for DL 3. Indicate in CG-UCI the COT sharing information | 10-1 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-22 | No gap 2-step RACH msgA transmission | Support transmitting PRACH and PUSCH of msgA without gap in between FFS if RAN1 can disallow this in NR-U. | 10-1 or 10-2 and 9-1 | Yes | N/A |  | Per band | N/A | N/A |  | For licensed case, a minimum gap between PRACH and PUSCH of msgA is introduced, but is not applicable to NR-U | Optional with capability signalling |
|  | 10-23 | CGI reading based on off-sync raster SSB for ANR functionality |  | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | Support reading RMSI from SCell from an off-sync raster SSB for ANR | Optional with capability signalling |
|  | 10-24 | CG-UCI multiplexing with HARQ ACK | Support multiplexing CG-UCI with HARQ ACK | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-25 | Cat 4 LBT based configured UL transmission out of COT | Controlled by gNB and support Cat 4 LBT based configured UL transmission out of COT when COT-SI is configured | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-26 | CSI-RS based RLM outside of DRS windows | Support RLM measurements using CSI-RS resources that are outside of DRS transmission windows | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-27 | Wideband PRACH | Enhanced PRACH design for NR-U by adopting a single long ZC sequence, with ZC sequence = 1151 for 15kHz and ZC sequence = 571 for 30kHz | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-28 | Configured grant with Rel-16 enhanced | Support configuration of resources with cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, | 10-18a or 10-18b | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-29 | Read availableRB-Sets-r16 in DCI _0 | Suppor monitoring DCI 2_0 to read availableRB-Sets-r16 | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-30 | Read COT duration in DCI _0 | Suppor monitoring DCI 2_0 to read COT duration | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | - To clarify that all the features with type “per band” are applicable to unlicensed band operation only, change “per band” -> “per band (unlicensed band only)” [QC] We can wait for the end of feature discussion to capture this. - For 10-1, 10-2 and 10-3, we suggest that they are optional with capability signaling - For 10-7, we suggest component 1 and 2 are split into separate UE features (e.g. 10-7 for 20MHz, 10-7a for 10MHz) and the prerequisite feature groups for both 10-7 and 10-7a should be 10-1 or 10-2 [QC] I actually moved 20MHz to the 10-1 and 10-2 as basic functionality and keep 10MHz here. For 10-14, 10-15 and 10-16, to be consistent with other features, we suggest adding 10-1 or 10-2 as prerequisite feature groups. |
| OPPO | - [10-13] there is RAN1 agreement to restrict the CP extension no longer than 1 OFDM symbol. Why do we need 10-13 for capability of supporting different ranges? [QC] We actually initially brought in for CP extension for Configured Grant PUSCH where up to 4 symbols @ 60KHz is needed. Refined the language, and keep the FFS if this is needed - [10-14] The spec does not support non-numerical K1 and semi-static HARQ codebook simultaneously. To us, the capability of NNK1 is a capability based on pre-requisite feature: enhanced dynamic HARQ codebook or dynamic HARQ codebook or semi-static HARQ codebook + one shot HARQ codebook [QC] Seems that it is not explicitly ruled out to use NNK1 with semi-static codebook, though it is not very useful I agree. Feature-wise, seems to be simpler keep it as a separate feature - [10-16] It is not clear if one-shot HARQ-ACK feedback can be configured as standalone mode. To us, it is a sub-capability of enhanced dynamic HARQ codebook or dynamic HARQ codebook or semi-static HARQ codebook [QC] Agree, but it is an add-on feature. At least one of the other codebook will be used and this can work on top. - [10-22] this feature is not supported by RAN1 as there is no clear RAN1 agreement [QC] We proposed it. Actually this is to for UE not supporting continuous PRACH and PUSCH transmission. In 2-step RACH agenda item, the 2 symbol gap is only agreed for licensed case, and interlaced PUSCH case may still have no gap transmission. - Missing UE features: - PCI reading based on off-sync raster SSB [QC] Added but need further discussion if this should be included in ANR capability - CG-UCI multiplexing with UCI, CG can be supported even without this multiplexing feature - CG COT sharing with gNB [QC] Is this captured in 10-21? - Multiple active CG configurations [QC] Is this captured by multiple CG in URLLC? That should be per band anyway? - Configured UL transmission Rel. 16 new behavior - Supporting type 1 to type 2A channel access switching [QC] Do we need this as a capability? There is no discussion on what if a UC cannot do this, so I assumed this is by default supported? Supporting multiple RB-sets [QC] Do we need this? Since one RB set is about 20MHz, is this implied by UE cell bandwidth? |
| Nokia | - In general, we are missing the basic feature descriptions, with the corresponding components. For example, which components are required by a UE implementing stand-alone NR-U or CA-based NR-U (DL and/or UL)? [QC] Agree, but need more discussion on how to do this. - Features 10-4, 10-5, 10-6 should not be listed separately but part of a basic LBT capability. [QC] I do agree, so I captured a question if these should be removed. We can decide during the discussion phase. - Features 10-12 and 10-13 are not needed. [QC] We can discuss the FFS during the discussion phase - 10-19: It is unclear if this is needed in addition to general indication of supported BW by the UE in the band. [QC] Added an FFS - 10-20: This feature should be conditionally mandatory for BWs larger than 20MHz. In addition, coreset replication could be part of this feature or as a separate feature. [QC] Further discussion needed - 10-22: Dependency on 9-1 to be clarified. |
| Samsung | 10-18a/b) Delete FG10-18b and delete ‘with LBE’ in FG10-18a. Configured grant retransmission is not relevant to the LBT mode and there is no agreement related with this. So, it does not need to have a separate feature group according to the LBT mode. [QC] More discussion needed 10-20) add ‘replicability of CORESET configuration’ in addition to rb-Offset and CORESET confinement |
| NTT DOCOMO | We also think that the basic feature group for NR-U operation (at least for NR-U DL operation) seems missing. [QC] Need discussion on how to form feature groups Following new features should be part of basic FG for DL operation in unlicensed spectrum (i.e., mandatory for UE supporting DL in NR-U band). - Discovery burst monitoring (e.g., frame timing and QCL determination based on SSB with up to 10/20 candidate positions) - RSSI/CO measurement (FG10-10) - GC-PDCCH (DCI format 2_0 with COT duration and available RB set indication) monitoring Following new features should be part of basic FG for UL operation in unlicensed spectrum (i.e., mandatory for UE supporting UL in NR-U band). - Channel access mechanism (both LBE and FBE) - PRACH sequence with length 571 (for 15 kHz) or 1151 (for 30 kHz) - Interlaced PUCCH(ePF0/1/2/3) and PUSCH resource allocation For both DL/UL operation in unlicensed spectrum, CP extension capability (FG10-13) could also be a part of basic feature group. In addition, the following unclear parts should be resolved to appropriately define the UE feature list: - FG10-10 (RSSI measurement capability) includes CO measurement capability. - Whether PRACH sequence length 571/1151 are also supported for licensed band or unlicensed band only As pointed by vivo’s comment#1, whether “per band” means the feature can also be supported for licensed band or only for unlicensed band |
| ZTE | In general, we also think it is better to consolidate more technical components to form a more meaningful basic feature group for NR-U. [QC] Need discussion on how to form feature groups For 10-13 and 10-18b: It has not been discussed whether to support CP extension or CG retransmission in FBE mode in Rel-16 NR-U, so we are not sure if it is appropriate to capture them as the UE features. If companies cannot converge quickly, probably a better way is to continue the discussion in Rel-17, e.g. under URLLC WI or above 52.6GHz SI/WI. [QC] CG as a feature did not rule out FBE during the design. We believe it should not be rules out. Same thing for CP extension. For 10-22: According to the agreement of 2-steps RACH, “the minimize transmission gap between the end of MsgA PRACH and the beginning of MsgA PUSCH(guard time excluded) is no less than Ngap symbols. -This is not applied for NR-U”, it implies that no gap between MsgA RACH and MsgA PUSCH is possible for NR-U. But this has not been discussed in Rel-16 NR-U, which means it could be also considered as implementation issue. If this feature is captured, there might be different interpretations of “no gap between MsgA RACH and MsgA PUSCH”. E.g., one understanding is to configure MsgA RACH and MsgA PUSCH continuously in a same slot, or MsgA RACH is allocated at the end of the first slot while MsgA PUSCH is allocated at the start of the second slot. Actually it has been only agreed to support PRACH and PUSCH transmitted in different slots in Rel-16 2-step RACH. So capturing this feature may cause unnecessary confusion. We suggest to continue the discussion in Rel-17, e.g. under URLLC WI. [QC] Better if we can have some discussion in RAN1 on the gap. However if we don’t have further agreement on the gap, the no-gap transmission between PRACH and PUSCH (in different slots) will be allowed for Rel.16. We propose the capability here to support a mode the UE is doing the same thing as in licensed. Then no-gap PRACH and PUSCH is extra. |
| MediaTek | - Basic feature groups corresponding to different NR-U deployment scenarios should be defined. Therefore, we propose the following feature groups. Index; Feature group; Components; Prerequisite feature groups; Per; Mandatory/Optional. 10-1a; Basic feature group for downlink NR-U operation; - DRS-based RRM measurements; [Per BC]; Conditional mandatory. 10-1b; Basic feature group for uplink NR-U operation; - PRB-based interlaced resource allocation for PUSCH - Cat-1 LBT; [Per BC]; Conditional mandatory. 10-1c; Basic feature group for stand-alone (SA) and DC NR-U operations; - DRS-based RRM measurements - Cat 1 LBT - PRB-based interlaced resource allocation for PUSCH - PRB-based interlaced resource allocation for PUCCH formats 0/1 (ePF0 and ePF1) - SSB based RLM - Wideband PRACH: Enhanced PRACH design for NR-U by adopting a single long ZC sequence - ZC sequence = 1151 for 15kHz - ZC sequence = 571 for 30kHz - Validation rules for RACH occasions in FBE operation - RRACH occasions (ROs) in Idle Period are invalid. - RAR extension - Decoding of 2-bit SFN indication in DCI 1_0 - Non-numerical PDSCH to HARQ-ACK timing - Enhanced dynamic HARQ-ACK codebook; 10-1a and 10-1b; [Per BC]; Conditional mandatory. - Add the following features to the NR-U UE feature list. Index; Feature group; Components; Prerequisite feature groups; Per; Mandatory/Optional. 10-23; CGI reporting for NR-U cells; Support acquisition of relevant information from a neighbouring NR unlicensed cell in an unlicensed carrier by reading the SI of the neighbouring unlicensed cell and reporting the acquired information to the network; 10-1a or 10-1c; [Per BC]; Optional with capability signalling. 10-24; CSI-RS based RLM outside of DRS windows; Support RLM measurements using CSI-RS resources that are outside of DRS transmission windows; 10-1c; [Per BC]; Optional with capability signalling. 10-25; Wideband PRACH; Enhanced PRACH design for NR-U by adopting a single long ZC sequence - ZC sequence = 1151 for 15kHz - ZC sequence = 571 for 30kHz; 10-1b; [Per BC]; Optional with capability signalling. 10-26; CORESET configuration with RB offset; Frequency domain resource allocation with RB offset in CORESET configuration regardless of the bandwidth of the CORESET.; 10-1a or 10-1c; [Per BC]; Optional with capability signalling. 10-9a; Search space set group switching by explicit indication; Search space set group switching by monitoring a bit field in DCI format 2_0; 10-9; [Per BC]; Optional with capability signalling. 10-27; Configured grant with Rel-16 enhanced resource allocation; 10-1b; [Per BC]; Optional with capability signalling. - For FG10-8, remove Type B PDSCH lengths of 9 and 10 from the component column. - Feature 14-3 in TEI section has covered these two lengths. - For FG10-13, revise to the following: - Feature group: CP extension for scheduled PUSCH/PUCCH - Components: Support CP extension of at most one OFDM symbol duration preceding the first symbol of a scheduled PUSCH/PUCCH transmission subject to UE processing time defined in Rel-15 - For FG10-17, in the “Components” column, change to “Support of scheduling more than one PUSCH with one UL grant a single DCI 0_1” - For FG10-20, we suggest to modify as follows: - Revise feature group name as “Support monitoring locations in frequency domain provided by search space set configuration” - It is better to change the name of this feature to avoid confusion with the feature of CORESET configuration with RB offset, 10-26, that we propose in the above. - Revise component description as “Support number of monitoring locations in frequency domain provided by search space set configuration only when for new coreset configuration with rb-Offset and the coreset the associated CORESET is confined within one LBT bandwidth” - We don't see any RAN1 agreements saying that multiple monitoring locations in frequency domain can be only supported for a CORESET configured with rb_offset. Frequency domain monitoring occasions were agreed for CORESET confined within an LBT bandwidth, and it can be independently configured from the configuration of rb_offset. Therefore, we suggest the above changes to be aligned with RAN1 agreements. [QC] Do we need search space set with freqMonitorLocations and coreset with rb-offset in different feature? I think they can be common features with 2 components - For FG10-22, it should be clarified whether Rel-16 support such feature because there is no explicit RAN1 agreement. If companies think this feature is supported in Rel-16, we suggest to modify the feature as follows. - Revise feature group name as “No or shorter gap 2-step RACH msgA transmission” - Revise component description as “Support transmitting PRACH and PUSCH of msgA without gap in between or with a gap that shorter than 2 OFDM symbols with SCS=15kHz and 30kHz or 4 OFDM symbols with SCS=60kHz and 120kHz where SCS is the SCS of the msA PUSCH” - Change “Per band” to “[Per BC]” for every feature in the NR-U feature list. |
| LG | 10-1: The Note (Support of channel access mechanism for LBE operation, including Cat 4 LBT, Cat 2 LBT, Cat 1 LBT, contention window adjustment. UE supports either 16 us or 25 us or both 16 and 25 us for Cat-2 LBT) implies that, UE may support one of the followings. - Support of 16 us Cat-2 LBT, Not support of 25 us Cat-2 LBT - Support of 25 us Cat-2 LBT, Not support of 16 us Cat-2 LBT - Support both of 16 us and 25 us Cat-2 LBT However, since it was agreed that fallback DL/UL DCI and RAR grant can indicate one of {16 us Cat-1, 25 us Cat-2, Cat-4}, support of 25 us Cat-2 LBT needs to be mandated. Therefore, we suggest the Note saying that UE supports either 16 us or 25 us or both 16 and 25 us for Cat-2 LBT. 10-4/5/6: Based on the same logic for the case of 10-1 above, 10-5 (25us Cat 2 LBT support) and 10-6 (Cat 4 LBT support) can be removed but 10-4 (16us Cat 2 LBT support) can be kept as one of UE capabilities. 10-12: For this feature, the prerequisite feature group needs to be modified as 10-3 (not 10-1 or 10-2). 10-13: The necessity of this UE feature should be clarified. According to agreements, gNB should ensure extended CP length less than one OFDM symbol. In addition to this, do we still need UE capability for extended CP length? 10-16a: Before adding this feature (on top of more general feature 10-16), the motivation/necessity should be clarified first. 10-17: For this feature, it can be considered not to have any prerequisite feature group, similarly with other feature such as HARQ-ACK codebook. 10-18a/b: What is the intention to split CG capability depending on FBE or LBE? Is there any difference between CG in FBE and CG in LBE, from the perspective of CG operation? 10-22: From our understanding, there is no explicit agreement to support no gap between PRACH and msgA PUSCH under any agenda item. Without corresponding RAN1 agreement, can we make UE capability? - In general, some of UE features have FBE and/or LBE pre-requisite features. However, considering FBE or LBE is required for UL operation, do we need FBE and/or LBE pre-requisite features for DL-specific operation (e.g., 10-10 for RSSI measurement, 10-20 for search space set configuration)? |
| Huawei, HiSilicon | - On FG 10-1: - The terminologies should align with those defined in TS37.213, i.e. CAT4 LBT-> type1 DL/UL channel access procedure, CAT2 25us LBT-> type 2A DL/UL channel access procedure, CAT2 16us LBT -> type 2B DL/UL channel access procedure and CAT1 LBT -> type 2C DL/UL channel access procedure. - Additional capability, e.g. FG10-1a, should be defined, such as type A/B multiple channel access procedure when a PUSCH is scheduled across multiple LBT bandwidth, which depends on FG 10-1. - On FG 10-2, the terminology of CAT1 LBT and CAT2 LBT did not appear in the spec of TS37.213. In addition, there is no agreement in RAN1 to support UE initiated COT, thus the FFS should be removed. Considering FBE is just one of the potential channel access mechanisms, thus this feature group should be optional with capability signaling. - On FG 10-4 to 10-6, we agree with the author that they are redundant and can be removed from the table. - On FG 10-7, this feature group should depend on the channel access mode applied on the band 46. “Prerequisite feature groups” should be “10-1 or 10-2”. “Type” should be per band. - On FG 10-8, the PDSCH mapping type B with length 9 and 10 can apply to both licensed and unlicensed band according to the agreement in NRU and DSS TEI, We propose to have a separate feature group with “Type” as “Per UE”. For the rest of additional length, it was only discussed in NRU and the “Type” should be per band. - On FG 10-9, the search space group switching will only balance the UE complexity and efficiency of channel access when dynamic channel access procedure is used in band 46. The “Prerequisite feature groups” should be “10-1” and “Type” should be “per band”. - On FG 10-10, report of channel occupancy should be the component of this feature group. Suggest to change the name of feature group to RSSI-CO-Measurement according to RAN2 specification of TS38.331. It should be optional with capability signaling - On FG 10-12, the OCC configuration only applies to PUCCH mapping on PRB interlace. The “Prerequisite feature groups” should be “(10-1 or 10-2) and 10-3”. - On FG 10-13, CP extension is only useful to fill the gap after LBT and start of PUSCH. For semi static channel access, the length of channel sensing fixed and no need for CPE. The “Prerequisite feature groups” should be “10-1” - On FG 10-16a, there was no agreement in RAN1 that one shot feedback triggered by DCI 1_1 without scheduling PDSCH should be depend on the feature group 10-16. The dependence in the row of 10-16a should be removed. The description on 10-16 and 10-16a should be self-contained. - On FG 10-18a, “UE transmits multiple PUSCHs in a configured grant period according to the RRC configuration for both type 1 and type 2 configured grant” should be added as one of the components for FG 10-18a. In addition, another UE capability should be added for the capability to multiplex between CG-UCI and HARQ-ACK configured by higher layer signaling. - On FG 10-18b, all components should be same as those for LBE thus there is no need to define a separate capability, except how UE will determine the availability of configured resource within a fixed frame period, which needs more RAN1 discussion. If FG 10-18b is deleted, then “with LBE” should be deleted as well. - On FG 10-19, this capability can be derived implicitly from channel combination supported, whether a separate capability is needed can be discussed in RAN4. If the carrier bandwidth is larger than 20MHz, UE should be capable to perform LBT on all LBT bandwidths in the carrier. - On FG 10-20, the rb-offset should be applied to CORESET configuration. The name of feature group should be changed. - On FG 10-22, no agreement either in 2 step RACH or NRU to support this feature. The whole row should be FFS before it can be agreed. - The following feature groups are not included a) A feature group for monitor PDCCH on multiple monitoring locations. This feature should be separate from CORESET associate with rb offset. b) A feature group of initial access signal and procedures for shared spectrum should be included c) A feature group of RLM measurement based on discovery burst should be included d) A feature group of RMSI detection on SCell where SSB is not on the sync raster should be included e) A feature group for UE to detect DCI format 2_0 including available RB set indicator, remaining COT duration and explicit flag for search space set group switch should be included. |
| Panasonic | - Several minimum features related to unlicensed operation should be merged to one feature. It means to merge 10-1, 10-2, 10-3, 20MHz LBT bandwidth in 10-7, 10-20, 10-21. - 10-7 should be whether 10MHz LBT is supported or not. - 10-4, 10-5, 10-6 can be delated since they are already included in 10-1 or 10-2. |
| Ericsson | General comments: - There are a few features that we propose changing from “per-band” to “per UE.” These are features that if implemented at the UE do not need to be restricted to unlicensed bands since they offer enhancements that are generic in nature. This can also save signalling overhead in not having to indicate capability per band. - Proposal: Change “per-band” to “per UE” for the following features: 10-8, -9, -11, -14, -16, -17 - Features 10-1 and 10-2 defines components consisting of different UL channel access procedures. Unless the implementation of a specific feature inherently requires implementation of UL channel access procedure(s), then that feature should not list 10-1 or 10-2 as a pre-requisite. - Proposal: Remove 10-1 and 10-2 as a pre-requisite from the following features: 10-3, -10, -11, -12, -13, -17, -18a, -18b, -19, -20, -22 - In general, the “Components” column should be empty unless a feature consists of multiple components that need to be listed explicitly - Proposal: Move/merge the text from the “Components” column into the “Feature Group” column so it accurately describes the feature for the following feature groups: 10-9, -10, -14, -15, -16, -16a, -17, -18a, -18b, -20, -21, -22 - The text in the “Notes” column may become redundant and can be removed. - For example, for FG 10-16: The moved/merged text can be “One-shot HARQ ACK feedback: HARQ-ACK codebook contains A/N for all configured HARQ processes for all configured CCs” 10-1: - FG name should be changed to: “UE channel access procedures for UL transmission for dynamic channel access mode” - The wording “UL transmission” is important, for example, since for DL only deployments, there is no required UE capability on UL channel access procedures - The wording “channel access procedures” is used instead of “LBE” to align with terminology defined in 37.213 - The components should use terminology consistent with that in 37.213, i.e., the channel access procedures - Type 1 channel access procedure - Type 2A channel access procedure - Type 2B channel access procedure - Type 2C channel access procedure - This feature group should be mandatory (all components) since it is essential for UL operation in shared spectrum. 10-2: - FG name should be changed to: “UE channel access procedures for UL transmission for semi-static channel access mode” - The wording “UL transmission” is important, for example, since for DL only deployments, there is no required UE capability on UL channel access procedures - The wording “channel access procedures” is used instead of “FBE” to align with terminology defined in 37.213 - The components should use terminology consistent with that in 37.213, i.e., Sensing with single 9 us slot or no sensing - Procedure with sensing single 9 us slot - Procedure with no sensing - This feature should be optional with capability signalling, and if supported, both components are supported 10-3: - FG name should be changed to: “PRB interlace mapping for PUCCH and PUSCH” - Component descriptions should more accurately reflect specifications, e.g., Type 2 FDRA is used in 38.214 for PUSCH. - PRB-based interlace mapping for PUCCH Formats 0, 1, 2, and 3 - Type 2 frequency domain resource allocation for PUSCH - Interlaced mapping is important in order to achieve full transmit power, thus it should be mandatory at least for 23 dBm (Power class 3 UEs). It can be further discussed optionality for other power classes. If the feature is supported, then both components should be supported since RAN1 agreed that either both channels are transmitted with interlacing or both without interlacing. 10-4, 10-5, 10-6: - Propose to remove these FG since they are components of 10-1 10-7: - FG name should be changed to “10 MHz SCell” to reflect that this feature is about whether or not 10 MHz SCells are supported rather than about LBT bandwidth in general. RAN1 agreed that LBT bandwidth is always in integer multiples of 20 MHz. The only exception is for a 10 MHz SCell which is only supported for 2 specific ARFCNs in one geography region in the world (India). - Components not needed anymore after this change 10-8: - This feature is generic UE capability, and is thus defined as “Per UE” - It is beneficial (and simpler) that if any new Type B mapping lengths are supported, then all are supported 10-9: - This feature is beneficial as a generic UE power saving tool, and is thus defined as “Per UE” 10-11: - Feature group name should be changed to better reflect actual capability, i.e., “SRS starting at any OFDM symbol in a slot” - This feature is beneficial for positioning using legacy SRS, and is thus defined as “Per UE” 10-12: - Change feature group name to “OCC for PRB interlace mapping for PUCCH Format 2 and 3” - This feature is dependent on support of interlaced mapping for PUCCH (FG 10-3), and thus 10-3 should be a pre-requisite - If interlaced mapping for PUCCH is supported, this feature should be mandatory and all 3 components should be supported 10-13: - This capability should be changed to a binary capability, i.e., all CP extension durations should be supported or none, hence the word “maximum” and “range” should be removed from the feature group name/description. - Propose the following FG name: “CP extension prior to first symbol for PUSCH and PUCCH” - Since this is basic functionality in order to operate in unlicensed spectrum without gaps, it should be defined as mandatory 10-14 - Change the FG name to “Configuration of negative value to use for PDSCH to HARQ-ACK timing” to be consistent with specifications, i.e., a negative value rather than “non-numerical” value is defined in 38.331 - This feature is beneficial for disabling HARQ in low latency applications, and is thus defined as “Per UE” 10-16 - This feature is beneficial as a generic robustness enhancement tool, and is thus defined as “Per UE” 10-16a - This feature is based on the following Working assumption that is not yet captured in the specifications. - Working assumption: One value of the frequency domain resource assignment field indicates that this DCI does not schedule a PDSCH - Resolve conflict with the UE power saving WI - Propose to remove this FG for now and postpone the discussion until it is specified. 10-17 - This feature is beneficial generic tool to improve PDCCH capacity, and is thus defined as “Per UE” 10-18a - It is not clear why this should be tied to channel access mode, i.e., LBE. Further, it is not clear why differentiation is needed between LBE and FBE in this feature (10-18a) and the next one (10-18b) - Propose to remove “LBE” from the feature group name 10-18b - Propose to remove this feature group based on comment for 10-18a 10-19 - FG name should be changed to: “Intra-cell guard band(s) for UL carrier BW >20 MHz” - Feature group should be defined based on whether or not the UE can support intra-cell guard bands in the UL. Then, the number of LBT bandwidths is determined based on the carrier bandwidth(s) supported by the UE. - This feature should have the pre-requisite “Support for UL carrier bandwidth > 20 MHz” - We propose a new feature group for DL, 10-19a. “Intra-cell guard band(s) for DL carrier BW >20 MHz” - This feature should have the pre-requisite “Support for DL carrier bandwidth > 20 MHz” 10-21: - FG 10-1 removed as pre-requisite, since “Type 1” (a.k.a. CAT4) channel access procedure should be mentioned in the Feature Group column after moving/merging text from the “Components” column. Also, since 10-1 is mandatory, there may not be capability signaling of 10-1. New feature group 10-23: - CGI reporting for off-sync raster SS/PBCH block to support automatic neighbour relations (ANR) functionality Since ANR functionality is not needed in all network deployments, it can simplify UE implementations that may be targeted at specific deployment scenarios where ANR not used. |
| Intel | [10-2] UE is not considered as an initiating device for FBE in Rel-16. [10-4, 10-5, 10-6] Not needed since they are essential features for LBE or FBE. [10-7, 10-8] Add “10-1 or 10-2” in the column of “Prerequisite feature groups” [10-9] Add “10-1” in the column of “Prerequisite feature groups”, This is only for LBE [10-13] Not needed. CP extension has to be supported for all UEs since fall-back DCI already indicates CP extension length during initial access. [10-22] Add “9-1” in the column of “Prerequisite feature groups” in addition to “10-1 or 10-2” |
| Apple | [FG 10-1]: - We believe this should be either “optional” or “conditional mandatory” since some UEs may ONLY support DL transmissions on unlicensed band and UL transmission on licensed band for NR-U operation (i.e. scenario D).It can be defined as “mandatory conditional on the UE supports UL transmission on unlicensed band” - Terminologies alignment with TS 37.213. [FG 10-2] - Remove the FFS as there is no agreement to allow UE initiate COT for FBE operation. - Terminologies alignment with TS 37.213. - This should be “Optional” UE feature since two options exist, LBE and FBE. [FG 10-3] - Similar reasoning as for Feature 10-1, this feature should be “conditional mandatory” on condition that “UE supports UL transmission on unlicensed band” [FG 10-4] [FG 10-5] [ FG 10-6] - Not needed as they were covered in FG 10-1. [FG 10-7] - Remove the component 20MHz, given it is the default LBT bandwidth and basic component for NR-U UL transmission. Also, coupling of ‘20MHz’ and “10MHz” LBT bandwidth is less flexible from UE perspective, given the 10MHz LBT use cases are very limited but 20MHz is kind of basic component. - Prefer to be defined as “per band” - Add the “10-1 or 10-2” in the column of “Prerequisite feature groups” - Define as “Optional with capability signalling” [FG 10-8] - Type-B PDSCH lengths should exclude 9/10 that have been covered by capability in DSS TEI and applied for both licensed and unlicensed band. - Assuming the length-9/10 would be covered in DSS TEI, the “Type” for this feature should be “per band” as the relevant context was discussed and agreed in NR-U WI. [FG 10-9] - The feature should be ‘per band’ as it is mainly motivated by characteristic of unlicensed band to dynamic access channel with LBT and agreed in NR-U WI. [FG 10-10] - Supportive to add component of “CO” as part of RSSI reporting to align with RAN2 spec and define it as “optional with capability signaling” [FG 10-12] Use FG 10-3 as pre-requisite as it is a feature for interlaced PUCCH format 2/3 only. |
| Spreadtrum Communications | [FG 10-2] Remove the FFS, there is no agreement to support the UE as an initiating device [FG 10-4] The “Prerequisite feature group” should be 10-1, as there is no agreement to support 16us Cat 2 LBT in FBE mode. [FG 10-8] The lengths of the new PDSCH mapping type B should exclude the length 9 and 10 already covered by DSS TEI, and the remaining lengths are agreed only for NR-U, hence the Type should be “per band”. [FG 10-9] This feature is introduced to balance the power saving of the UE and the channel access efficiency of gNB. Therefore, the Prerequisite feature group should be 10-1 and the Type should be “per band”. [FG 10-12] Add 10-3 in the column of “Prerequisite feature group”. [FG 10-22] This feature should be FFS, because there is no agreement in NR-U to support this feature. |

- NR_L1enh_URLLC

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11. NR_L1enh_URLLC | 11-1 | Monitoring DCI format 1_2 and DCI format 0_2 | 1) Supports monitoring DCI format 1_2 for DL scheduling 2) Supports monitoring DCI format 0_2 for UL scheduling |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-1a | Monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space | 1) Supports monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space | 11-1 | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-2 | Rel-16 PDCCH monitoring capability | 1) Supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span for combination (X, Y, ) 2) If UE reports the support of more than one combination of C(X, Y) for a given SCS, and if multiple combinations of C(X, Y) are valid for the span pattern, the maximum value of C of the valid combinations is applied 3) Supports the limit M on the maximum number of monitored PDCCH candidates per PDCCH monitoring span for combination (X, Y, ) 4) If UE reports the support of more than one combination of M(X, Y) for a given SCS, and if multiple combinations of M(X, Y) are valid for the span pattern, the maximum value of M of the valid combinations is applied 5) Capability on the number of CCs with Rel-16 PDCCH monitoring capability on all the serving cells. | 3-5b | Yes | N/A |  |  | No |  |  | This capability is necessary for SCS 15 kHz and 30 kHz. A list of separate UE capabilities C(X, Y, ), M(X, Y, ) for processing capability #1; A list of separate UE capabilities C(X, Y, ), M(X, Y, ) for processing capability #2; For component 5), if UE supports carrier aggregation with more than [x] DL carriers, UE should report this capability. Value of x (can be < 4) is TBD. | Optional with capability signalling Candidate value set for (X, Y): {(7, 3), (4, 3), (2, 2)} The value of C for combination (7, 3) for 15 kHz and 30 kHz is 56 Candidate value for component 5): { x, x+1, …, 16} |
|  | 11-2b | Capability on the number of CCs with Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells | - Capability on the number of CCs with Rel-15 PDCCH monitoring capability - Capability on the number of CCs with Rel-16 PDCCH monitoring capability | 11-2 | Yes | N/A |  | Per UE | No |  |  | Capability on the number of CCs with Rel-15 PDCCH monitoring capability can be smaller than 4 CCs; Capability on the number of CCs with Rel-16 PDCCH monitoring capability can be smaller than 4 CCs; The summation of the minimum of the capability on the number of CCs with Rel-15 PDCCH monitoring capability and the minimum of the capability on the number of CCs with Rel-16 PDCCH monitoring capability is not larger than 4 | Optional with capability signalling |
|  | 11-3 | More than one PUCCH for HARQ-ACK transmission within a slot | 1) Supports sub-slot based HARQ-ACK feedback procedure. • A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot. • At least one sub-slot configuration for PUCCH can be UE specifically configured to a UE. • Supports a single configuration for PUCCH resource for all sub-slots in a slot. The starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot. Any sub-slot PUCCH resource is not across sub-slot boundaries. 2) Supported sub-slot configuration 3) Supported combinations of (A, B), where A is the minimum gap between sub-slots containing actual PUCCH transmissions measured from beginning to beginning of the sub-slots, including across slots, and B is the sub-slot duration, with both A and B in units of symbols |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 | Candidate value set for component 2): { 7-symbol*2, 2-symbol*7 and 7-symbol*2} Candidate value set for component 3): (A, B) = {(7, 7), (4, 2) and (7, 7), (2, 2) and (4, 2) and (7, 7)} | Optional with capability signalling |
|  | 11-4 | Up to two HARQ-ACK codebooks simultaneously constructed for supporting different service types for a UE | 1) Supports up to two HARQ-ACK codebooks with different priorities to be simultaneously constructed. 2) Supports separate PUCCH configuration for different HARQ-ACK codebooks 3) Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. 4) Supports a DCI format (from the formats 0_1/1_1/0_2/1_2) scheduling PDSCH with different HARQ-ACK priorities or PUSCH with different priorities when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured in USS per BWP 5) Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and ‘codeBlockGroupTransmission” for different HARQ-ACK codebooks. |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-4a | Monitoring a DCI format (from the formats 0_1/1_1/0_2/1_2) scheduling PDSCH with different HARQ-ACK priorities or PUSCH with different priorities when both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP |  | 11-1a, 11-4 | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-5 | PUSCH repetition type B | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots. • The number of the repetitions signalled by gNB represents the "nominal" number of repetitions. The actual number of repetitions can be larger than the nominal number. • The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first "nominal" repetition. • The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols • If a "nominal" repetition goes across the slot boundary or DL/UL switching point, this "nominal" repetition is splitted into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot. 2) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. • Jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. • The maximum TDRA table size is increased to 64. 3) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field. 4) PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 5) S and L are separately indicated (4-bit for S and 4-bit for L). L <= 14. 6) TBS is determined based on L indicated in TDRA table entry reusing Rel-15 mechanism. 7) Handling of interaction with DL/UL directions depending on whether dynamic SFI is configured or not 8) Supported maximum number of repetitions |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 | Candidate value for component 8): {1, 2, [3], 4, [6], 7, [8], 12, 16} | Optional with capability signalling |
|  | 11-6 | PUSCH repetition Type A | 1) PUSCH transmission with Rel-15 behavior with or without slot aggregation. • With slot aggregation, the number of repetitions can be either semi-statically configured (as in Rel-15) or dynamically indicated (as agreed for Rel-16). • When dynamically indicated, the number of repetitions is jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. | 2-12, 2-13, 2-14, 2-15 | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-7 | UL cancelation scheme | 1) Supports group common DCI (i.e. DCI format 2_4) for cancelation indication 2) UL cancelation for PUSCH • Cancellation is applied to each PUSCH repetition individually in case of PUSCH repetitions 3) UL cancelation for SRS symbols that overlap with the cancelled symbols |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-8 | Enhanced UL power control scheme | For DG-PUSCH, one bit (separately from SRI) in UL grant is used to indicate the open loop power control parameter set if SRI is present in the UL grant, and 1 or 2 bits is used to indicate the P0 value if SRI is not present in the UL grant |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-9 | Multiple active configured grant configurations for a BWP of a serving cell | 1) Supports up to 12 configured/active configured grant configurations in a BWP of a serving cell. • Separate RRC parameters for different configured grant configurations • Separate activation for different configured grant Type 2 configurations • Separate release for different configured grant Type 2 configurations |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-9a | Joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell | M<=4 bits indication in the Release DCI is used for indicating which CG configuration(s) is/are released, where the association between each state indicated by the indication and the CG configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple CG configurations to be released • In case of no higher layer configured state(s), separate release is used where the release corresponds to the CG configuration index indicated by the indication | 11-9 | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-10 | Type 2 configured grant release by DCI format 0_1 | Support of type 2 configured grant release by DCI format 0_1 |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 | A UE supporting this feature and 11-1 (DCI format 0_2/1_2) shall also support 11-11 (Type 2 configured grant release by DCI format 0_2). | Optional with capability signalling |
|  | 11-11 | Type 2 configured grant release by DCI format 0_2 | Support of type 2 configured grant release by DCI format 0_2 | 11-1 | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 | A UE supporting this feature shall also support 11-10 (Type 2 configured grant release by DCI format 0_1). | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | - On 11-1, the components should be revised to cover the activation/release of UL Type 2 CG or DL SPS, suggest the following revisions - Supports monitoring DCI format 1_2 for DL scheduling and DL SPS activation/release - Supports monitoring DCI format 0_2 for UL scheduling and UL Type 2 CG activation/release CHY> Per the comments from companies, it is possible support of DCI format 0_2/1_2 for Type 2/SPS release is another UE capability. For activation, it can belong to “scheduling”? Probably one way I can delete “for DL scheduling” and “for UL scheduling” in 11-1. Let's see if any other views first. - On 11-3, we suggest that sub-slot configuration of “2-symbol*7” and “7-symbol*2” be split into separate UE capabilities CHY> Several companies have similar thinking, update accordingly. - On 11-4 component 4), if DCI format 0_2/1_2 is used, the prerequisite feature should be 11-1. CHY> It seems we need more discussion on the structure of 11-4 first. - On 11-5, the component 4) “PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG).” should be dependent on whether UE support DCI format 0_2 (i.e. 11-1) CHY> It seems we need more discussion on this first. - On 11-6, we suggest to modify the component part (to focus on the delta compared to Rel-15 features) as the following: “1) PUSCH transmission with slot aggregation, the number of repetitions can be dynamically indicated (as agreed for Rel-16).” CHY> Let’s see if we need to change the structure first. - Additional feature 11-10 should be added for supporting Rel-16 UL Type 2 CG release by DCI format 0_1 CHY> Let’s discuss this first. |
| OPPO | For 11-4, PUCCH configurations related to HARQ-ACK feedback are configured for different HARQ-ACK codebooks separately ,in the other words, not all the PUCCH configurations are configured separately for different HARQ-ACK codebook, e.g. multi-CSI-PUCCH-ResourceList, schedulingRequestResourceToAddModList, schedulingRequestResourceToReleaseList CHY> As in the RRC parameter discussion, though a few parameter may be no need to configure separately, for simplicity we assume separate PUCCH configuration for different HARQ-ACK codebooks. For 11-5, 2) For dynamic indication of the nominal number of repetitions, Jointly coded with S and L (not SLIV) in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. CHY> Add to 11-6? 4) PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 separately CHY> It seems we need more discussion on this first. Missing feature: CHY> Seen in IIoT UE feature list. - Physical layer priority indication, at least including HARQ-ACK, PDSCH, PUSCH, configured grant, SPS and SR. - Intra-UE dropping - Multiple active SPS configurations for a BWP of a serving cell - Joint release in a DCI for two or more SPS configurations for a given BWP of a serving cell For PUSCH repetition type B, invalid symbol pattern can be configured |
| Nokia | - 11-1 and 11-1a: it should be clarified if 11-1 covers the feature without restriction, and what UE behavior can be assumed if 11-1a is not reported. CHY> In my understanding, if there is any restriction, the a separate UE capability can be set accordingly, e.g. monitoring DCI format 0_2/1_2 together with DCI format 0_1/1_1 is a separate UE capability, then it is clear that 11-1 doesn't include this case. As to the behaviour if 11-1a is not reported, let's wait for further agreement before doing any update here. - Consider combining 11-2a and 11-2b as they are not completely independent features but different aspects of the same feature. Also consider combining them with 11-2. CHY> Considering the comments from other companies, I merged 11-2a with 11-2, but leave 11-2b as separate capability. - 11-4a: feature definition is somewhat confusing due to very long name without any description of components. CHY> In Rel-15 we have some examples without any component also. The feature name itself is the component. But anyway I can put it in the column again. - 11-5 and 11-6 are inter-related and we should have a statement saying that UEs supporting 11-5 need to support 11-6 as well. CHY> We need some discussion as shown in the last row of this table first, which may have impact on the structure of 11-5 and 11-6, then we can update accordingly if needed. - In general we are missing a categorization of features that are essential components of URLLC operation and which should be supported by Rel-16 UEs supporting URLLC, which could be represented by one or more basic URLLC features, for example. CHY> We need some discussion on this first. |
| NTT DOCOMO | - Add 11-1b: Monitoring DCI format 0_1/1_1 and DCI format 0_2/1_2 in different search space set on a cell for scheduling the same cell CHY> Is it straightforward to support DCI format 0_1/1_1 and DCI format 0_2/1_2 in different search space set? Though probably we need some additional DCI size alignment to meet the budget, I feel seems there is no other difficulty to support this. - For 11-3, - the feature name is not accurate, it should be called sub-slot PUCCH for HARQ-ACK feedback. CHY> The name is corresponding to the objective in the WID, I feel it is ok? - separate the sub-slot configurations “2-symbol*7” and “7-symbol*2” to separate UE capabilities. CHY> Several companies have similar thinking, update accordingly. - For 11-4, - separate capabilities are needed for “support up to two HARQ-ACK codebooks with different priorities to be simultaneously constructed” - One is slot-based, and one is sub-slot-based. - Both are slot-based. - Both are sub-slot-based CHY> It seems we need more discussion on the structure of 11-4 first. - separate capabilities are needed for - Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH - Supports 2-level priority of HARQ-ACK for SPS PDSCH and SPS PDSCH release CHY> Any particular reason we need separate capability for dynamic PDSCH and SPS PDSCH? - for 11-5, - separate capabilities for dynamic grant and configured grant; separate the capabilities for the DCI format 0_1 and 0_2 - component 7) should be separate for when dynamic SFI is configured and semi-static rate-matching pattern is configured for DG PUSCH - the components on the supported number of repetitions is missing: {1, 2, [3], 4, [6], 7, [8], 12, 16} CHY> We need some discussion on this first. - for 11-6, the components on the supported number of repetitions for dynamic indication is missing: {1, 2, [3], 4, [6], 7, [8], 12, 16} - Feature for PUSCH repetition Type B frequency hopping modes are missing - for 11-7, UL cancellation for PUSCH and SRS should be separate capabilities CHY> Any particular reason? - for 11-9, additional UE capability is needed for ‘Multiple active configured grant configurations for a cell group’ CHY> Lets’s discuss this first. Adding release DCI formats (DCI format 0_1 and DCI format 0_2) for CG2 CHY> Update accordingly |
| CATT | For 11-2, the third component shall be bracketed, as there is no consensus to enhance the number of monitored PDCCH candidates. CHY> So far the key issue is what value for M, but the framework is there according to the agreements from last meeting, unless no consensus can be achieved on the value of M, probably we can remove this feature, For PUSCH repetition type B, it is proposed to add additional feature regarding frequency hopping. For example, two separate features for inter-repetition frequency hopping and inter-slot repetition hopping can be added. The prerequisite feature should be 11-5. CHY> We need some discussion on this first. |
| ZTE | Basic UE feature group for URLLC It lacks a meaningful basic feature group for URLLC. The basic feature group for URLLC should include basic features for each channels/signal, otherwise the reliability and latency requirements would not be able to be satisfied as a whole for supporting URLLC service. However, there is only one feature on support of the new DCI formats in the current 11-1 now, which is clearly not sufficient. So, we have following suggestions on this basic feature group. - FG 11-2 is included in the basic group. Rel-16 PDCCH monitoring capability is introduced due to support of new DCI formats. So, it is expected this UE should support the new monitoring capability as a basic feature for monitoring new DCI formats. - FG 11-3 as an enhancement to HARQ-ACK feedback is included in the basic group. This is to guarantee the latency of DL PDSCH transmission. In addition, FG 11-4 can be also considered to include in the basic feature or regarded as a subordinate feature. Because, slot based HARQ-ACK codebook is a Rel-15 mandatory feature, a UE additionally support of subslot based HARQ-ACK codebook means it has to also support up to two HARQ-ACK codebooks. - FG 11-5 and FG 11-6 as an enhancement to PUSCH repetition is included in the basic group. This is to guarantee the reliability and latency of PUSCH transmission. Note, a UE support PUSCH repetition type B, which means support of dynamic repetitions, should also be able to support PUSCH repetition type A with semi-static and dynamic repetitions. CHY> We need some discussion on this first. Other detailed comments In addition, we have some more detailed suggestions about some individual feature groups. Comment 1: For feature group 11-2, 11-2a and 11-2b, we don’t see the need of FDD/TDD and/or FR1/FR2 differentiation. Thus, “support mixture of FDD/TDD and/or FR1/FR2” can be put in the corresponding column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2”. CHY> It may depend on whether to support it for 60 kHz and 120 kHz. So far it is very likely will only be supported for 15 kHz and 30 kHz. Comment 2: For feature group 11-2, similar to bullet 2) for the maximum value of C, we also need similar descriptions for the value M. That is, add the following bullet. - If UE reports the support of more than one combination of C(X, Y) for a given SCS, and if multiple combinations of C(X, Y, u) are valid for the span pattern, the maximum value of M of the valid combinations is applied. CHY> updated accordingly. Comment 3: For bullet 1) of feature group 11-4, the UE needs to support all combinations of slot or subslot based HARQ-ACK codebook based on the agreements. So, we suggest adding ‘including one is slot-based and one is sub-slot-based, both are slot-based and both are sub-slot-based’ for the first bullet. For bullet 2) of feature group 11-4, according to agreements, the UE should support separate configuration of HARQ-ACK codebook type, i.e., ‘PDSCH-HARQ-ACK-Codebook’, UCI multiplexing on PUSCH for dynamic grant, i.e., ‘UCI-OnPUSCH’ and CBG configuration, i.e., ‘codeBlockGroupTransmission’. This should also be added into this UE feature. CHY> Update accordingly Comment 4: For bullet 2) of feature group 11-5, dynamic indication of the number of repetitions is also supported for type 2 CG PUSCH. So, suggest following changes: 2) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH or type 2 configured grant PUSCH. • Jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. • The maximum TDRA table size is increased to 64. CHY> We need to discuss whether to set different capabilities for dynamic grant and configured grant first based on the comment from other companies. Comment 5: We need to add a sub-FG for FG 11-7 to reflect the following UE capability. CHY> Let’s update once we have clear agreement Agreements: - Up to X BDs can be configured for UL CI - FFS per UL CI monitoring occasion or per span - The value of X is to be concluded during this week Note: UE is not expected to be configured with search space configuration for UL CI with AL and number of candidates exceeding X BDs |
| MediaTek | - FG11-3: The supported sub-slot configuration of “2-symbol*7” and “7-symbol*2” should be reported separately. CHY> Several companies have similar thinking, update accordingly. - FG11-3 & FG11-4: A capability of “the maximum number of PUCCHs for HARQ per slot” should be added. CHY> Several companies have similar thinking, update accordingly. - FG11-5: A capability of “the maximum number of PUSCH repetitions for the same TB in one slot” should be added. CHY> Update accordingly since several companies have similar view. - FG11-9: Clarify that the number of PUSCHs for different TBs in a slot is based on 5-12, 5-12a, 5-12b, 5-13d, 5-13e, 5-13f features from Rel-15. CHY> Let’s discuss this a little bit more first to see more views. |
| LG | 11-2: Due to absence of discussion, there is no rule on how to handle the case where UE reports the support of more than one combination of M(X, Y) for a given SCS and multiple combinations of M(X, Y) are valid for the span pattern, which needs to be further discussed. One straightforward way would be to have the similar rule as 2) as below highlighted in red. 1) Supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span for combination (X, Y, ) 2) If UE reports the support of more than one combination of C(X, Y) for a given SCS, and if multiple combinations of C(X, Y) are valid for the span pattern, the maximum value of C of the valid combinations is applied 3) Supports the limit M on the maximum number of monitored PDCCH candidates per PDCCH monitoring span for combination (X, Y, ) 4) If UE reports the support of more than one combination of M(X, Y) for a given SCS, and if multiple combinations of M(X, Y) are valid for the span pattern, the maximum value of M of the valid combinations is applied CHY> Updated accordingly. 11-5: In 4), it seems that only DG and type 2 CG support PUSCH repetition type B. However, type 1 CG can also support PUSCH repetition type B, which should be clarified. In addition, the actual repetition should also take into account invalid symbol pattern (please find the relevant agreement below), which is not reflected in the feature currently. It would be good to add this aspect. CHY> We need some discussion on this first. Agreement (RRC impact) For DG PUSCH with PUSCH repetition type B, if dynamic SFI is configured, introduce a first RRC parameter that indicates one pattern for invalid symbols for PUSCH transmission repetition type B applicable to both DCI format 0_1 and 0_2, and introduce a second RRC parameter for each of DCI format 0_1 and 0_2 to indicate the presence of an additional bit in the DCI to indicate whether the pattern applies or not. - If the first RRC parameter is not configured, semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols. - If the first RRC parameter is configured and the additional bit exists in a DCI, - Value ‘0’ means semi-static flexible symbols are used for PUSCH, and segmentation occurs only around semi-static DL symbols. - Value ‘1’ means that segmentation occurs around semi-static DL symbols and invalid symbols in the pattern, and the remaining symbols are used for PUSCH. - If the first RRC parameter is configured and the additional bit does not exist in a DCI, segmentation occurs around semi-static DL symbols and invalid symbols in the pattern, and the remaining symbols are used for PUSCH. - The first RRC parameter reuses the pattern definition of rateMatchPattern in time domain for PDSCH. |
| Huawei, HiSilicon | - On FG 11-4, separate capability should be defined for scheduling PDSCH with different HARQ-ACK priorities by DCI format 1_1 and DCI format 1_2, and separate capability should be defined for scheduling PUSCH with different priorities by DCI format 0_1 and DCI format 0_2. CHY> It seems we need more discussion on this before update. Please see my note in the row for capturing rapporteur notes for the next round discussion. - On FG 11-5, separate capability should be defined for PUSCH repetition type B scheduled by DCI format 0_1 and DCI format 0_2. CHY> We need some discussion on this first. - On FG 11-8, different UE capability should be defined for enhanced UL power control with DCI format 0_1 and DCI format 0_2 - On FG 11-3, different UE capability should be defined for the support of "7-symbol * 2” and "2-symbol * 7". Support of "2-symbol * 7" can imply the support of "7-symbol * 2”. CHY> Several companies have similar thinking, update accordingly. - On FG 11-5, splitting “nominal” repetition should be a separate UE capability. UE should report the capability on the number of repetitions supported in a slot. - On FG 11-9, UE should report the capability on the maximum number of simultaneous configured grant configurations CHY> Let’s hear more views first. |
| Panasonic | - On the relation between 11-4 and 11-4a, I think 11-4a is prerequisite of 11-4. (instead of 11-4 is prerequisite of 11-4a). CHY> It seems we need more discussion on 11-4 first, then we can update accordingly. - 11-7 should be rather "mandatory with capability signaling" in Rel.16, although we recognize Rel.16 would not have such feature basically. |
| Ericsson | First, add a UE capability report for d1 and d2 in the agreement below. Agreement When a high-priority UL transmission overlaps with a low-priority UL transmission in a slot, - The UE is expected to cancel the low-priority UL transmission starting from Tproc,2 +d1 after the end of PDCCH scheduling the high-priority transmission, where - Tproc,2 is correponding to UE processing time capability for the carrier. - Value d1 is the time duration corresponding to 0,1,2 symbols reported by UE capability - Note: d_2,1=0 is for cancellation - The minimum processing time of the high priority channel is extended by d2 symbols - Value d2 is the time duration corresponding to 0,1,2 symbols reported by UE capability 11-2 - Is it sufficient to have a note about “Separate UE capability signalling of the supported combination (X, Y, )”? The alternative way, as intended by RAN1 agreement, is to have: - a list of separate UE capabilities C(X,Y, ), M(X,Y, ), for capability #1, - a list of separate UE capabilities C(X,Y, ), M(X,Y, ), for capability #2 CHY> I think the original wording is ok, but ok to take the suggestion here. - For need of FR1/FR2 differentiation, it is a pending decision if SCS=60KHz and 120 kHZ are supported for Rel-16 monitoring. If they are not supported, then there exists FR1/FR2 differentiation - Same comment for 11-2a and 11-2b CHY> Agree, let’s update this later. 11-4 - The part about PUSCH with different priorities does not fit in this row. Delete “or a DCI format (from the formats 0_1/0_2) scheduling PUSCH with different priorities” CHY> It seems we need more discussion on this before update. Please see my note in the row for capturing rapporteur notes for the next round discussion. 11-4a - Clarify in components section that for HARQ-ACK and PUSCH, the different priorities are based on the Priority Indicator field in the relevant DCIs. - Propose to add a note that the following is already supported - a DCI format (from the formats 1_1/1_2) scheduling PDSCH with different HARQ-ACK priorities, or a DCI format (from the formats 0_1/0_2) scheduling PUSCH with different priorities, at least when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured in USS per BWP CHY> It seems we need more discussion on 11-4 and 1-4a first, then we can update accordingly if needed. 11-5 - No need to copy old RAN1 agreement texts. 38.214 contains definition of PUSCH repetition type B. CHY> use the agreements ok also? - No need to include the bullet about TBS determination since it is identical to Rel-15. CHY> It is part of repetition type B I feel. - Add a bullet about frequency hopping. For example, “Support frequency hopping, including inter-PUSCH-repetition frequency hopping, inter-slot frequency hopping.” CHY> We need to discuss this first since some company also think separate UE capability should be set for this. 11-6 - Add the following to point out the new features introduced in Rel-16 for repetition Type A: - “When dynamically indicated, the number of repetitions is jointly coded with SLIV in the TDRA table, by adding an additional column for the number of repetitions in the TDRA table. - The maximum allowed number of repetitions is increased to 16.” CHY> Updated accordingly. 11-7 - Clarify 2) and 3) as follows: 2) UL cancelation for of PUSCH, where cancellation is applied to each PUSCH repetition individually. 3) UL cancelation for of SRS symbols that overlap with the cancelled symbols. CHY> Updated accordingly. 11-9a - In general, UL configured grant feature description should mirror that of DL SPS. - For joint release of UL configured grant, there is no need to have a separate feature. It can be included in 11-9 as a component. This is similar to that of 12-2 for DL SPS. - Add “simultaneously active” to description. The same for 12-2 of DL SPS. CHY> Done |
| Intel | - For FG #11-2a: - Suggest to move the current text under “Note” to the “Components” column - For the “Note” column, suggest to add the following text: “If UE supports CA with more than [X] DL CCs, UE should report this capability”; value of X (can be < 4) is TBD CHY> Update accordingly. - Under the “Mandatory/Optional” column, the candidate value range should be added (e.g., {X, X+1, …, 16}) instead of “Optional with capability signaling” CHY> Update accordingly. - For FG #11-2b: - Similar changes as for FG #11-2a suggest in above bullets apply (for both Rel-15 and Rel-16 PDCCH monitoring capabilities) CHY> As to how to structure 11-2b, we need some discussion first as shown in the column of comments from rapporteur below. Let’s update this once we have a clear view on this. - For FG #11-4a: - FG #11-1, and NOT FG #11-1a should be a prerequisite for FG #11-4a (FG 11-4a needs the UE to be configured with monitoring for DCI formats 0_1/1_1 and 0_2/1_2 in a BWP, while FG #11-1a is about configuration of these format pairs in the same search space set) CHY> It seems we need more discussion on 11-4 and 1-4a first, then we can update accordingly if needed. - Need to add FG #11-1 as pre-requisite for all FG-components that involve DCI formats 0_2/1_2 |
| Apple | Feature 11-1: We suggest to introduce a component under 11-1: In Rel-15, multiple PDCCH monitoring capabilities 3-1,3-2, …,3-5, 3-5a, 3-5b are introduced; UE’s capability to support new DCI formats (0_2, 1_2) should be reported w.r.t. to the Rel-15 capabilities and Rel-16 capability. For example, a UE can report its capability to support 0_2/1_2 for Rel-15 PDCCH monitoring capability 3-1, 3-2, but not for 3-5a, 3-5b. CHY> In my understanding, if a UE reports to support monitoring DCI format 0_2/1_2,while it doesn’t report to support R15 PDCCH monitoring capability, then it means that it will support DCI format 0_2/1_2 with whatever it reports for Rel-15 capability, thus we don’t need to add a capability here for it. Features 11-2a/11-2b: Following the comment above, it is also necessary to clarify for Rel-15 monitoring capability, whether the monitoring of 0_2/1_2 is supported. CHY> See above. |
| Spreadtrum Communications | - On FG 11-1a, separate UE capability for DCI format 0_1 and 0_2 in a certain search space set, and DCI format 1_1 and 1_2 in a certain search space set. CHY> In my understanding, DCI format 0_1/1_1 is always configured together, that means in a certain search space if a UE needs to monitor 0_1 then it will monitor 1_1 also. Similar situation for DCI format 0_2 and DCI format 1_2. Then it seems we don't need to have separate UE capability for DCI format 0_1/ 0_2 and DCI format 1_1/1_2. Not sure if I miss anything here. Let’s see more views before making the update. - On FG 11-2, supported span patterns should be reported first. CHY> from the note column it is already clear that UE will report the supported combinations? In addition, once we have clear view on the capability on the number of CC, then I can see if better structure can be put here. - On FGs 11-2a, and 11-2b, value range should be given like [1,2…] CHY> See reply to Intel. - On FG 11-3, Different sub-slot configurations should be reported separately and a new capability should be defined for maximum PUCCH number in a slot CHY> Several companies have similar thinking, update accordingly. - On FG 11-4, one sub-slot-based HARQ-ACK codebook and two sub-slot-based HARQ-ACK codebook should be two separate UE capabilities CHY> It seems we need more discussion on this first. - On FG 11-5, a new capability should be defined for the number of PUSCH repetition type B transmitted in a slot CHY> Updated accordingly - On FG 11-7, separate capability should be defined for mini-slot PDCCH monitoring of CI CHY> Let’s discuss it a little bit more first, since only a few companies show the views, though I think it is reasonable to set separate capability. |
| Rapporteur | Based on the comments from companies, we may need to discuss this following points first. Most of them may have impact on the structure of the FGs depending on the outcome. We may not be able to discuss or get conclusion in the second round email discussion for all the points, but would be good if companies can provide their views on each of the points below in the second round email discusion, then I can try see if we have any good proposal or update for next F2F meeting: - For FG 11-2b: Whether to introduce separate UE capability for support of mixed Rel-16 PDCCH monitoring capability and Rel-15 PDCCH monitoring capability? If yes, whether to set separate capabilities for mixed Rel-16 capability with Rel-15 PDCCH monitoring capability FG 3-1, FG 3-2, FG 3-5b? Once we have consensus on this point, we can update FG 11-2b accordingly. - For FG 11-4/11-4a: - Whether to set separate UE capabilities for different combinations based on slot-based HARQ-ACK codebook and sub-slot based HARQ-ACK codebook, i.e. the following combinations: • One is slot-based and one is sub-slot-based. • Both are slot-based. • Both are sub-slot-based - Whether to set separate UE capabilities for scheduling PDSCH with different HARQ-ACK priorities or PUSCH with different priorities by DCI format 1_1/0_1 and DCI format 1_2/0_2? - For FG 11-5: - Whether to set separate UE capabilities for the total number of unicast PUSCHs for different TBs per slot per CC? - Whether to set separate UE capabilities for different UE processing time capability? - Whether to set separate UE capabilities for dynamic grant and configured grant? - Whether to set separate UE capabilities for the case that dynamic SFI is configured and semi-static rate-matching pattern is configured for DG PUSCH? - Whether to set separate UE capabilities for inter-repetition frequency hopping and inter-slot repetition hopping? - Whether to set separate UE capabilities for DCI format 0_1 and DCI format 0_2 for PUSCH repetition type B? - For FG 11-7: - Whether to set separate UE capabilities for UL CI monitoring periodicity < 1 slot? - Whether to set separate UE capabilities for UL CI on the same CC and on another CC? - For FG 11-9: - Whether to set separate UE capabilities for the number of configured grant configurations? - Whether the number of PUSCHs for different TBs in a slot is based on 5-12, 5-12a, 5-12b, 5-13d, 5-13e, 5-13f features from Rel-15? - Whether to have a set of basic UE features for eURLLC? |

- NR_IIOT

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12. NR_IIOT | 12-1 | UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer | Support intra-UE multiplexing/prioritization of UL overlapping channels/signals with two priority levels in physical layer (PHY) - Configuration of PHY priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH - Multiplexing/prioritization between UL channels/signals with the same PHY priority level - Prioritization between UL channels/signals with different PHY priority levels |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 | A UE supporting this feature shall also support the LCP restriction based on DCI priority indication ([lch-ToGrantPriorityRestriction-r16]) and intra-UE prioritization in MAC ([lch-PriorityBasedPrioritization-r16]). The relationship between this feature and the feature of up to two HARQ-ACK codebooks should be further discussed. | Optional with capability signaling |
|  | 12-2 | Multiple SPS configurations | Support of up to 8 configured/active SPS configurations in a BWP of a serving cell - Separate RRC parameters for different SPS configurations - Separate activation for different SPS configurations - Separate release for different SPS configurations - The related HARQ-ACK enhancements to support multiple SPS configurations | downlinkSPS | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 | It can be further discussed whether the number of SPS configurations a UE supports in a BWP/in a cell group/across all serving cells should be reported by the UE. | Optional with capability signaling |
|  | 12-2a | Joint release in a DCI for two or more SPS configurations for a given BWP of a serving cell | M<=4 bits indication in the Release DCI is used for indicating which SPS configuration(s) is/are released, where the association between each state indicated by the indication and the SPS configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple SPS configurations to be released • In case of no higher layer configured state(s), separate release is used where the release corresponds to the SPS configuration index indicated by the indication | 12-2 | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signaling |
|  | 12-3 | SPS release by DCI format 1_1 | Support of SPS release by DCI format 1_1 |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 | A UE supporting this feature and 11-1 (DCI format 0_2/1_2) shall also support 12-4 (SPS release by DCI format 1_2). | Optional with capability signaling |
|  | 12-4 | SPS release by DCI format 1_2 | Support of SPS release by DCI format 1_2 | 11-1 | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 | A UE supporting this feature shall also support 12-3 (SPS release by DCI format 1_1). | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | - On 12-2, we suggest to split out the joint release of SPS configuration as a separate feature (12-2a) with feature 12-2 as prerequisite. - Additional feature 11-10 should be added for supporting Rel-16 DL SPS release by DCI format 1_1 |
| Nokia | - 12-1 should be combined with 11-4 - For 12-2, the structure should be consistent with 11-9/11-9a. |
| NTT DOCOMO | 1. For 12-2, - additional UE capability is needed for ‘Multiple active SPS configurations for a cell group’ - separate capabilities for separate activation/release and joint release - adding additional feature for release DCI formats (DCI format 1_1 and 1_2) for SPS - 2. shorter periodicities for DL SPS feature is missing |
| Samsung | 12-2) Given FG 11-9 and FG 11-9a separately in eURLLC, take the similar formulation. Proposed update is as following: 1) delete “including separate activation/release and joint release of SPS configurations” in component #1, 2) add following sub-bullets under existing component#1: • Separate RRC parameters for different SPS configurations • Separate activation for different SPS configurations • Separate release for different SPS configurations 12-2a) add new FG 12-2a similar with FG19-9a as following: Feature group: Joint release in a DCI for two or more SPS configurations for a given BWP of a serving cell Component: M<=4 bits indication in the Release DCI is used for indicating which SPS configuration(s) is/are released, where the association between each state indicated by the indication and the SPS configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple SPS configurations to be released • In case of no higher layer configured state(s), separate release is used where the release corresponds to the SPS configuration index indicated by the indication |
| MediaTek | - FG12-2: The joint release of SPS configuration should be added as a separate feature (in a similar way as FG11-9a). - A feature on supporting shorter periodicities for DL SPS should be added. |
| LG | 12-1: In rel-16, RAN1 has specified only intra-UE prioritization (not multiplexing) of UL channels/signals with two priorities. Thus, we think that “multiplexing” should be removed as follows. Furthermore, 3) is a Rel-15 feature, which seems redundant so this can be removed. 12-1; UL intra-UE multiplexing/prioritization of channel/signals with two priority levels in physical layer; Support intra-UE multiplexing/prioritization of UL channels/signals with two priority levels in physical layer - Configuration of priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH - Prioritization between UL channels/signals with different priority levels - Multiplexing/prioritization between UL channels/signals with the same priority level. 12-2: The feature of multiple SPS configurations and joint release should be separately defined (same as type 2 CG), e.g., as below: 12-2; Multiple SPS configurations; - Support of up to 8 SPS configurations in a BWP of a serving cell, including separate activation/release and joint release of SPS configurations • Separate RRC parameters for different SPS configurations • Separate activation for different SPS configurations • Separate release for different SPS configurations - The related HARQ-ACK enhancements to support multiple SPS configurations. 12-2a; Joint release in a DCI for two or more configured SPS configurations for a given BWP of a serving cell; M<=4 bits indication in the Release DCI is used for indicating which SPS configuration(s) is/are released, where the association between each state indicated by the indication and the SPS configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple SPS configurations to be released • In case of no higher layer configured state(s), separate release is used where the release corresponds to the SPS configuration index indicated by the indication. |
| Huawei, HiSilicon | - On FG 12-2, UE should report the capability on the maximum number of simultaneous SPS configurations |
| Panasonic | - To support 12-1, the feature 11-4 and 11-4a are necessary. Therefore, 11-4 and 11-4a should be prerequisite of 12-1 |
| Ericsson | 12-1 - Change the feature group name to “Overlapping UL channels/signals with two priority levels in physical layer.” Also this aligns to description of Rel-15 UE feature: - “4-27: More than one group of overlapping channels for control multiplexing” - Add to the description to emphasize that collision is allowed of the channels/signals. This distinguishes from the situation where overlapping is avoided by gNB scheduler. - Move “Prioritization between …" step after “Multiplexing/prioritization between ..." step. 12-2 - Describe the RRC configuration, activation, release in the same manner as that of UL configured grant feature group 11-9. - Support of up to 8 simultaneously active SPS configurations in a BWP of a serving cell, including separate activation/release and joint release of SPS configurations - Separate RRC parameters for different SPS configurations - Separate activation for different SPS configurations - Separate release and joint release for different SPS configurations |
| Intel | - For FG #12-1: - For components 2 and 3, the description needs further clarity. Recommended changes below: - “2) Prioritization between UL channels/signals with different priority levels, involving two or more of: SR, HARQ-ACK, PUSCH” - For “3) Multiplexing/prioritization between UL channels/signals with the same priority level.”, suggest to add “Handling of overlaps involving multiple UL channels of high priority is TBD” in the “Note” column - For FG #12-2: - This should be split into FGs #12-2 and #12-2a to separate the feature of joint release, similar to the description for FGs # 11-9 and 11-9a |
| Qualcomm | - Add the following to 12-2 - The max number of SPS configurations in a BWP of a serving cell - The max number of SPS configurations across all serving cells - A new capability for joint release of multiple SPS configurations |
| Spreadtrum Communications | - On FG 12-1, the configuration of BFR PUCCH priority is missing - On FG 12-2, add a new capability for separate mini-slot SPS periodicity and a new capability for joint release of multiple SPS configurations |

- NR positioning

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13. NR DL positioning | 13-1 | DL PRS RSRP Measurement and Reporting | - Max number of positioning frequency layers supported by UE for DL PRS RSRP measurement report Values = {1,4}, other values are FFS - Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE for DL PRS RSRP measurement report. Values = {1,2} - Max number of DL PRS Resources per DL PRS Resource Set a UE can be configured for DL PRS RSRP measurement report Values are FFS Maximum number of DL PRS RSRP measurements on different PRS from the same TRP supported by the UE Values {1,2,3,4,5,6,7,8} - Max number of DL PRS Resources supported by UE for DL PRS RSRP report across all frequency layers, TRPs and DL PRS Resource Sets. - Values are FFS - Max number of TRPs across all positioning frequency layers per UE for DL PRS RSRP measurement report. Values are FFS - Max number of DL PRS Resources per positioning frequency layer for DL PRS RSRP measurement report. Values are FFS Support of inter-frequency DL PRS RSRP measurement report in RRC_CONNECTED state - Support of SSB from neighbor cell as QCL Type C source of a DL PRS resource - Support of SSB from neighbor cell as QCL Type D source of a DL PRS resource - Support of a DL PRS resource as QCL Type D source of another DL PRS resource |  |  | N/A |  | [ #1 – per UE (FFS FR1/FR2 differentiation) #2 – per UE (FFS FR1/FR2 differentiation) #3 – per UE (FFS FR1/FR2 differentiation) #4 – per UE (FFS FR1/FR2 differentiation) #5 – per UE (FFS FR1/FR2 differentiation) #6 – per UE (FFS FR1/FR2 differentiation) #7 – per UE #8 – per UE ] | No | TBD |  |  | Optional with capability signaling |
|  | 13-2 | DL PRS RSTD Measurement and Reporting | - Max number of positioning frequency layers supported by UE for DL PRS RSTD measurement report Values = {1,4}, other values are FFS - Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE for DL PRS RSTD measurement report. Values = {1,2} - Max number of DL PRS Resources per DL PRS Resource Set a UE can be configured for DL PRS RSTD measurement report. Values are FFS - Max number of DL PRS Resources supported by UE for DL PRS RSTD report across all frequency layers, TRPs and DL PRS Resource Sets. Values are FFS - Max number of TRPs across all positioning frequency layers per UE for DL PRS RSTD measurement report. Values are FFS - Max number of DL PRS Resources per positioning frequency layer for DL PRS RSTD measurement report. Values are FFS - Max number of DL PRS RSTD measurements per pair of TRPs. Values {1,2,3,4}. Note: This is a max number of DL PRS RSTD measurements per pair of TRPs with each measurement between a different pair of DL PRS resources or DL PRS resource sets. All the RSTD measurements in a single report should have a single reference timing. - Support of inter-frequency DL PRS RSTD measurement report in RRC_CONNECTED state - Support of DL PRS RSTD measurement quality metric - Support of SSB from neighbor cell as QCL Type C source of a DL PRS resource - Support of SSB from neighbor cell as QCL Type D source of a DL PRS resource - Support of a DL PRS resource as QCL Type D source of another DL PRS resource | 13-1 |  | N/A |  | [ #1 – per UE (FFS FR1/FR2 differentiation) #2 – per UE (FFS FR1/FR2 differentiation) #3 – per UE (FFS FR1/FR2 differentiation) #4 – per UE (FFS FR1/FR2 differentiation) #5 – per UE (FFS FR1/FR2 differentiation) #6 – per UE (FFS FR1/FR2 differentiation) #7 – per UE #8 – per UE ] | No | NA |  |  | Optional with capability signaling |
|  | 13-3 | DL SSB RRM measurements for NR Positioning | Prerequisite – support of SSB measurements for RRM (SS-RSRP, SS-RSRQ) based on Rel.15 for NR Positioning in Rel.16 Configuration and reporting of SSB RRM measurements (SS-RSRP, SS-RSRQ) based on Rel.15 for NR Positioning in Rel.16 |  |  | N/A |  |  | No | NA |  |  | Optional with capability signaling |
|  | 13-4 | CSI-RS RRM measurements for NR Positioning | Prerequisite – support CSI-RS for RRM (CSI-RSRP, CSI-RSRQ) based on Rel.15 for NR Positioning in Rel.16 Configuration and reporting of CSI-RS for RRM (CSI-RSRP, CSI-RSRQ) for NR Positioning in Rel.16 |  |  | N/A |  |  | No | NA |  |  |  |
|  | 13-5 | UE Based Positioning | TBD specific content in RAN2 WG | 13-2 |  | N/A |  | [per UE] | No | NA |  |  | Optional with capability signaling |
| 13a. NR UL positioning | 13a-1 | Transmission of SRS for NR positioning | - Max number of SRS Resource Sets for positioning supported by UE per BWP. Values for the UE capability include {1, 16}. Other values FFS - Max number of aperiodic SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Value {64} is agreed. - Max number of semi-persistent SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Value {64} is agreed. - Max number of periodic SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Value {64} is agreed. - Support of SSB for serving and neighbour cell DL pathloss estimation and SRS for positioning OLPC. - Support of DL PRS for serving and neighbour cell DL pathloss estimation and SRS for positioning OLPC. - Max number of pathloss estimates across all SRS resource sets for positioning. It is indicated from the following set of values {0, 4, 8, 16} - Spatial Relation of SRS Resources for positioning with DL PRS from serving and neighbour cells - Spatial Relation of SRS Resources for positioning with SSB from serving and neighbour cells - Spatial Relation of SRS Resources for positioning with other SRS resources for positioning - Max number of active spatial relations including both DL PRS and SSB. Values FFS |  |  | N/A |  | [ #1 – per UE #2 – per UE #3 – per UE #4 – per UE #5 – per UE #6 – per UE #7 – per UE ] | No | NA |  |  | Optional with capability signaling |
| 13b. NR DL and UL positioning | 13b-1 | UE Rx-Tx Time Difference Measurement and Reporting | - Support of UE Rx-Tx time difference measurement with serving cell - Support of UE Rx-Tx time difference measurement with neighbouring cells - Support of inter-frequency UE Rx-Tx time difference measurements - Max number of UE Rx – Tx time difference measurements per TRP DL PRS Resource Set/Resource - A UE can be configured to report multiple Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL PRS resource/resource set. The DL PRS resource/resource sets can be in different positioning frequency layers - Support of UE Rx-Tx time difference measurement quality metric | 13-2 13a-1 |  | N/A |  | [ #1 - per UE #2 - FFS #3 - per UE ] | No | NA |  |  | Optional with capability signaling |
| 13c. NR RAT Independent Positioning | 13c-1 | TBD in RAN2 WG | [TBD in RAN2 WG] |  |  | N/A |  |  | No | NA |  |  | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | - On feature 13-1, suggest the following revisions to the components part - Component 7 revised to : Support of inter-frequency DL PRS RSRP measurement and report in RRC_CONNECTED state [Added in updated version] - Component 8 should be deleted as there was no agreement regarding RSRP measurement quality [Removed in updated version] - On feature 13-2, suggest the following revisions to the components part - Component 7 revised to : Support of inter-frequency DL PRS RSTD measurement and report in RRC_CONNECTED state [Added in updated version] - Adding additional component “Max number of DL PRS RSTD measurements per pair of TRPs with each measurement between a different pair of DL PRS resources or DL PRS resource sets.” according to the following agreement [Added in updated version] Agreement: The prior agreement is updated as given below: UE can be configured to measure and report up to [M] DL PRS RSTD measurements per pair of TRPs with each measurement between a different pair of DL PRS resources or DL PRS resource sets, and the M measurements being performed on the same pair of TRPs subject to UE capability - All the RSTD measurements in a single report should have a single reference timing - Note: Each RSTD measurement is between DL PRS Resources corresponding to different TRP IDs. - M=[3] 4. - Additional UE features (13-X) for DL PRS processing should be added according to the following agreement Agreement: Duration of DL PRS symbols in units of ms a UE can process every T ms assuming 272 PRB allocation is a UE capability. [Agreement requires additional details in order to be captured. Propose to discuss further and capture once complete proposal is finalized. Companies are encouraged to bring proposals to complete this agreement.] - On feature 13b-1, suggest adding additional component as the following - Component 4): Support of UE Rx-Tx time difference measurement quality metric [Added in updated version] |
| OPPO | [13-2] - Missing component - Support to report X DL PRS RSTD measurements per pairs of TRPs, X={1, 2, 4} [Added in updated version with values {1,2,3,4} given that agreements is “up to M” = 4] [13a-1] - Missing component - Support of X additional pathloss estimates across all SRS resource sets for positioning, X={0, 4, 8, 16} [Added in updated version for [13a-1] (3) and [13a-1] (4)] |
| Nokia | - We are missing the features that define the positioning methods being supported, which would then define the components needed to support them. [There was no such agreement in RAN1. If RAN2 decides on signaling per positioning method some mapping from capability of measurements to positioning methods may need to be defined. 3GPP does not define position calculation method itself.] - There are quite a few FFS values indicated in the capabilities. Are those expected to be discussed in NR Positioning maintenance session, directly in UE capability session, or some other means? [Companies are encouraged to propose values. Final decision on harmonized numbers can be made in April meeting] - 13-1/13-2 compoenent 4: It could be defined as a separate component for clarity. [It can be discussed further if motivation is provided and other companies agree on proposal] - 13-1: “Support of DL PRS RSRP measurement quality metric” needs to be removed as this is not supported in Rel-16. [It is removed in updated version above] - 13-5: More clear definition is needed. [It is expected to be finalized by RAN2 WG according to the details of feature operation (that RAN2 defines in Rel-16). Description added by RAN1 is removed to avoid confusion] - 13a-1, component 4: it could be defined as a separate feature if needed [It can be discussed further if motivation is provided and other companies agree on proposal] - 13a-1, components 5, 6, and 7: further discuss if there is a need for FR1/FR2 differentiation [It can be discussed further and concluded in April meeting. Companies are encouraged to provide their view in e-mail discussion before April meeting] |
| CATT | For 13-1, “Support of DL PRS RSRP measurement quality metric” should be removed. There is no agreement (and also no need) to support the quality metric for DL PRS RSRP measurement. [It is removed in updated version above] For 13b-1, we need to add the “Support of UE Rx-Tx Time Difference Measurement quality metric” based on RAN1’s agreement (e.g., Agreement: Support the same set of resolutions for the quality metrics of all UE/gNB timing measurements (DL PRS RSTD, UE Rx-Tx time difference, UL RTOA, and gNB Rx-Tx time difference) [It is added in updated version above] For 13-3 and 13-4, Rel.15 DL SSB and CSI-RS RRM measurements are used to support Rel.16 E-CID NR Positioning. Given that DL SSB and CSI-RS RRM measurements are not new measurements, they can be removed from the feature list. [It is assumed that RAN2 will define signalling of these measurements in LPP. This is a new functionality. If other companies prefer to remove it can be discussed further and removed if there is a consensus] |
| ZTE | Comments on 13-1 and 13-2: We think the bullet 1-7 in 13-1 and 13-2 are common capabilities for DL PRS RSRP, DL RSTD and UE Rx-Tx. So it is better to list these capabilities for all DL positioning schemes. [There was no such agreement in RAN1. If RAN2 decides on signaling per positioning method some mapping from capability of measurements to positioning methods may need to be defined. 3GPP does not define position calculation method itself.] Comments on 13-3 and 13-4: For E-CID scheme, UE usually reports the measurement results it has, additional UE capabilities are unnecessary. So 13-3 and 13-4 may be unnecessary, especially for 13-3 since SSB based RRM is mandatory for UE. [It is assumed that RAN2 will define signalling of these measurements in LPP. This is a new functionality. If other companies prefer to remove it can be discussed further and removed if there is a consensus] Comments on 13a-1: What is the different UE behavior between bullet 5 and 6? We think these two bullets can be combined. Same for bullet 3 and 4. [SSB and DL PRS are different signals. DL PRS may not be configured in UL positioning scenarios. It makes sense to keep it separate to clearly see different functionalities enabled by NR.] Other comments: new UE capabilities may be needed as shown in the following agreements Agreement: Duration of DL PRS symbols in units of ms a UE can process every T ms assuming 272 PRB allocation is a UE capability [Agreement requires additional details in order to be captured. Propose to discuss further and capture once complete proposal is finalized. Companies are encouraged to bring proposals to complete this agreement.] Agreement: The prior agreement is updated as given below: UE can be configured to measure and report up to [M] DL PRS RSTD measurements per pair of TRPs with each measurement between a different pair of DL PRS resources or DL PRS resource sets, and the M measurements being performed on the same pair of TRPs subject to UE capability - All the RSTD measurements in a single report should have a single reference timing - Note: Each RSTD measurement is between DL PRS Resources corresponding to different TRP IDs. - M=[3] 4 [It is added to updated version] Agreement: The prior agreement is updated as given below: UE can be configured to measure and report up to N=8 (> 1) DL PRS RSRP measurements on different DL PRS resources from the same TRP - N=[3] FFS: N is a UE capability [It is added to updated version.] Agreement: - A UE does not expect to simultaneously maintain more than N distinct pathloss estimates across all SRS resource sets for positioning which are different from the up to four pathloss estimates per serving cell the UE maintains for PUSCH/PUCCH and other SRS transmissions, where N can be up to 16 per UE. The maximum value of N supported by a UE can be indicated as a UE capability and can take the values N = {0, 4, 8, 16} [It is added to updated version] |
| LG | 13-1: For 8), From our understanding, we have no explicit agreement on the support of DL PRS RSRP measurement quality metric, and its necessity is not clear. Agreement: - Support reporting separate metric(s) corresponding to quality of each of the following measurements - RSTD - UE Rx-Tx time difference - UL-RTOA - gNB Rx-Tx time difference - UL angle of arrival, including the azimuth of arrival (AoA) and the zenith of arrival (ZoA) - FFS: The details of these quality metrics [It is removed in updated version above] 13-3: We would like to suggest removing SS-RSRPB since there was a typo in the agreement of #98 RAN1 meeting, and we have another agreed to exclude SS-RSRPB as follows. Agreement: - Exclude SS-RSRPB from the list of UE RRM measurement for E-CID - Include this agreement in an LS to other RAN 2/4 WGs [It is removed in updated version above] 13b-1: According to the above agreement, we may need to add “Support of UE RX-TX time difference measurement quality metric”. [It is added to updated version above] |
| Huawei, HiSilicon | - On FG 13-1: Component-8 “Support of DL PRS RSRP measurement quality metric” should be removed, because there is no RSRP quality reporting at all. [It is removed in updated version above] - On FG 13-2: “Max # of DL PRS Resource Sets per TRP supported by UE” in component-2 should be updated to “Max # of DL PRS Resource Sets per TRP per frequency layer supported by UE” based on agreement from RAN1#99. [It is added in updated version above] - On FG 13a-1 - “per BWP” should be added to component-1 based on agreements from RAN1#99 [It is added in updated version above] - Component-3 and 4 have “neighbor cell” in the description while Component-5 and 6 don’t have it, can you explain the reason? [Serving and neighbour cells are added to components (3) to (6). Please check] - Component-8 should be modified as below, as reported for Rel-15 SRS capability, i.e. - Max # of periodic SRS resources for positioning supported by UE across all SRS resource sets per BWP - Max # of semi-persistent SRS resources for positioning supported by UE across all SRS resource sets per BWP - Max # of aperiodic SRS resources for positioning supported by UE across all SRS resource sets per BWP [It was not discussed/proposed under positioning AI, however it makes sense to be consistent w/ R15. Proposal is added to updated version. Companies are encouraged to comment on values] - On FG 13b-1 For multi-RTT positioning purpose, we do not see the need to differentiate the capability of UE Rx – Tx time difference measurement with respect the serving cell and the neighbouring cells, because we do not have it for RSTD or PRS-RSRP. Likewise, we think support of intra-frequency and inter-frequency can be UE capability. Suggest to the merge the component 1 and 2 as Support of intra-frequency UE Rx – Tx time difference measurements [It was not commented by other companies. Propose to continue check views from other companies] - On FG 13-1 & FG 13-2 A new component should be added based on the following agreement in RAN1#99 Agreement: Duration of DL PRS symbols in units of ms a UE can process every T ms assuming 272 PRB allocation is a UE capability Note that the maximum of RBs for a carrier is 264 for FR2, 272 should be changed to 264 at least for FR2. Suggest to add the following component (X,T): Duration of DL PRS symbols in X ms a UE can process every T ms assuming 272 PRB allocation and 30kHz SCS for FR1 and assuming 264 PRB allocation and 120kHz SCS for FR2 [Agreement requires additional details in order to be captured. Propose to discuss further and capture once complete proposal is finalized. Companies are encouraged to bring proposals to complete this agreement.] - On FG 13-2 A new component should be added based on the following agreement in RAN1#99 Agreement: The prior agreement is updated as given below: UE can be configured to measure and report up to [M] DL PRS RSTD measurements per pair of TRPs with each measurement between a different pair of DL PRS resources or DL PRS resource sets, and the M measurements being performed on the same pair of TRPs subject to UE capability - All the RSTD measurements in a single report should have a single reference timing - Note: Each RSTD measurement is between DL PRS Resources corresponding to different TRP IDs. - M=[3] 4 Suggest to add the following component Max number of RSTD measurements per TRP pair [It is added in updated version] - On FG 13a-1 A new component should be added based on the following agreement from RAN1#99 Agreement: A UE does not expect to simultaneously maintain more than N distinct pathloss estimates across all SRS resource sets for positioning which are different from the up to four pathloss estimates per serving cell the UE maintains for PUSCH/PUCCH and other SRS transmissions, where N can be up to 16 per UE. The maximum value of N supported by a UE can be indicated as a UE capability and can take the values N = {0, 4, 8, 16} Suggest to have Maximum number of distinct pathloss estimates across all SRS resource sets for positioning values = {0, 4, 8, 16} [It is added in updated version] - On FG 13b-1 Similar to RSTD, we suggest to add the following component based on agreement for RSTD Max number of UE Rx – Tx time difference measurements per TRP [It is added in updated version - to be consistent with DL PRS RSTD] - On FG 13-1 & FG 13-2: Firstly, when the SCS is 120 kHz and the number of PRBs is 272 (approximately 400MHz), the buffering and computing resources required by the UE will increase sharply. Furthermore, the UE needs to process PRS from multiple cells in a short time. It would need a higher UE capability to support such a large bandwidth for positioning. Therefore, we propose to add a new feature on the maximum bandwidth of PRS that UE can support, similar to what we have in Rel-15 as supportedBandwidthDL in FeatureSetDownlinkPerCC. Secondly, the current LPP specification allows assistance data delivery containing both LTE PRS and NR PRS, and measurement request requesting both measurements based on LTE PRS and NR PRS. Since processing LTE PRS and NR PRS simultaneously would inevitably increase the UE load, thus it would be better to define a UE capability regarding simultaneous LTE-NR PRS processing. Thirdly, in case PRS configuration includes both FR1 and FR2, another buffering capability considering mixed FR1 and FR2 operation should be introduced. The condition can reuse what we have for FR2 standalone, i.e. assuming 264 PRB allocation and 120kHz SCS. In summary, we propose to add the following three new components under FG 13-1 and FG 13-2. New-1: The maximum bandwidth of PRS that the UE can support similar to supportedBandwidthDL in FeatureSetDownlinkPerCC [It was not discussed/agreed by RAN1. Current understanding is that capabilities are discussed under assumption of any supported carrier/system bandwidth (including maximum bandwidth). Companies are encouraged to provide more views] New-2: Support of simultaneous processing of LTE PRS and NR PRS [It is also applicable for UE Rx-Tx time difference feature i.e. 13b-1. Given that it was not discussed by RAN1, companies are encouraged to provide more views] New-3: (X,T): Duration of DL PRS symbols in X ms a UE can process every T ms assuming 264 PRB allocation and 120kHz SCS mixed FR1 and FR2 operation [Propose to continue discussion. Companies are encouraged to provide views] - On FG 13a-1 Firstly, when multiple SRS resources are simultaneously transmitted on the same slot, the UE processing capability will be greatly challenged, since the processing resources are directly proportional to the number of SRS resources that need to be supported. For Rel-15, there is also such a UE capability regarding the number of periodic/SP/AP SRS resources in a slot, a similar capability should be introduced for SRS for positioning. Secondly, when multiple SRS resources are simultaneously transmitted on the same symbol, the UE processing capability will be greatly challenged, since processing resources are directly proportional to the number of SRS resources that need to be supported. Therefore, it is better to add a new capability on number of simultaneous transmission of SRS for positioning on a symbol. Finally, for the number of spatial relations, the Rel-15 UE capability is related to the serving cell. For NR positioning, the spatial relations can be for the neighbouring cells. Therefore, it is reasonable to introduce another UE capability of the active spatial relations toward neighbouring cells. Therefore, it would be better to add a new UE capability on the number of active spatial relations for the neighbouring cells/TRPs. In summary, we propose to add the following three new components under FG 13a-1. New-1: Max # of SRS resources for positioning supported by UE across all SRS resource sets per BWP per slot. Similar to component-8 and captured in Rel-15 SRS capability, it should be sorted by each resourceType, i.e., - Max # of periodic SRS resources for positioning supported by UE across all SRS resource sets per BWP per slot - Max # of semi-persistent SRS resources for positioning supported by UE across all SRS resource sets per BWP per slot - Max # of aperiodic SRS resources for positioning supported by UE across all SRS resource sets per BWP per slot [Max # of SRS resources for positioning per slot was not discussed by RAN1. It is not clear if it is assumed. Companies are encouraged to discuss further.] New-2: Max # of simultaneous transmission of SRS for positioning on a symbol (for single carrier operation and intra-band CA) [Max # of simultaneous transmission of SRS for positioning on a symbol was not discussed by RAN1. It is not clear if more than one resource was assumed. Companies are encouraged to discuss further.] New-3: Max # of active spatial relations for neighbouring cells/TRPs (FFS: separate for each source type or totally number irrespective of the source type) [Configuration of spatial relations for neighbouring cells/TRPs was agreed. Max # of active spatial relations for neighbouring cells/TRPs needs to be concluded] |
| Ericsson | 13-1: - Based on the following agreement Agreement The prior agreement is updated as given below: UE can be configured to measure and report up to N=8 (> 1) DL PRS RSRP measurements on different DL PRS resources from the same TRP. - Propose to add component: - Maximum number of DL PRS RSRP measurements on different PRS from the same TRP supported by the UE (up to 8) [Added in updated version] - Support of DL PRS RSRP measurement quality metric was not agreed and should be removed (only agreed to time-based measurement quality metric) [Removed in updated version] 13-2 - Based on the following agreement Agreement The prior agreement is updated as given below: UE can be configured to measure and report up to [M] DL PRS RSTD measurements per pair of TRPs with each measurement between a different pair of DL PRS resources or DL PRS resource sets, and the M measurements being performed on the same pair of TRPs subject to UE capability - All the RSTD measurements in a single report should have a single reference timing - Note: Each RSTD measurement is between DL PRS Resources corresponding to different TRP IDs. - M=[3] 4. - Propose to add component: - Max # of DL PRS RSTD measurements between different pairs of PRS resources or resource sets for a given TRP pair [Added in updated version] 13a-1 - Note that the name of the new SRS used for positioning is not fixed yet in RAN2, we have added brackets around the name for the time being. - Based on the following agreement: Agreement: - A UE does not expect to simultaneously maintain more than N distinct pathloss estimates across all SRS resource sets for positioning which are different from the up to four pathloss estimates per serving cell the UE maintains for PUSCH/PUCCH and other SRS transmissions, where N can be up to 16 per UE. - The maximum value of N supported by a UE can be indicated as a UE capability and can take the values N = {0, 4, 8, 16}. - Propose to add component: - Maximum number of neighbour and serving cell DL pathlosses maintained by the UE (0,4,8,16) [Added in updated version] |
| Rapporteur (Intel) | The following points need to be further discussed and addressed on the next stage of UE feature discussions: - Resolve FFS values for capability components. Companies are encouraged to provide their views for each feature/component. - Resolve FR1/FR2 differentiation and type of signaling i.e. per band, etc. Companies are encouraged to provide their views for each feature/component. - Two companies mentioned association of capabilities with positioning method. One company proposed details of grouping measurements under positioning methods. Considering that RAN1 discussed mapping of reference signals and measurements to positioning methods and send LS to RAN2 in R1-1909796, the current understating based on the cited agreement below is that discussion is still open and is subject to RAN2 Agreement: Inform RAN2 of the following: - It is RAN1 understanding that discussion on mapping of positioning techniques to reference signals and measurements is out of RAN1 scope and therefore RAN1 does not intend to define such mapping. - It is RAN1 understanding that RAN2 will define signalling that can support any RAT dependent positioning technique including hybrid RAT dependent positioning solutions - Discuss proposals to finalize the following agreement and how to capture it as a part of UE feature/capability: “Duration of DL PRS symbols in units of ms a UE can process every T ms assuming 272 PRB allocation is a UE capability” The following proposal based on views from single company can be used as a starting point for discussion: - Define DL PRS processing capability per measurement window when measurement gaps are configured that will include following components - Max number of positioning frequency layers that can be supported by the UE - Limit on max number of DL PRS resources N1 the UE is expected to measure across all frequency layers within a measurement window T1 msec - Limit on max number of symbols N2 containing PRS resources of maximum bandwidth the UE is expected to measure within a measurement window of T2 msec - UE reports a tuples {N1, T1}, {N2, T2} when measurement gaps are configured - Define DL PRS processing capability per measurement window when measurement gaps are not configured that will include following components - Max number of positioning frequency layers that can be supported by the UE - Limit on max number of DL PRS resources N3 the UE is expected to measure across all frequency layers within a measurement window T3 msec - Limit on max number of symbols N4 containing PRS resources of maximum bandwidth the UE is expected to measure within a measurement window of T4 msec - UE reports a tuples {N3, T3}, {N4, T4} when measurement gaps are configured - Discuss whether to define maximum bandwidth of DL PRS that the UE can support (by default max system/carrier bandwidth is assumed in capability discussion). - Discuss whether to define “Support of simultaneous processing of LTE PRS and NR PRS” - Discuss whether to define max number of simultaneous transmission of SRS for positioning on a symbol - Discuss whether to define max number of SRS resources for positioning per slot - Discuss whether to define capability for support of concurrent measurements (DL RSRP, DL RSTD, UE Rx-Tx Time Difference) / positioning methods (DL AoD, DL-TDOA, multi-RTT) - Discuss indication of concurrent configuration of list of measurements [positioning methods] in supported CA Band Combination in the BandCombinationList |

- NR TEI

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14. NR TEI | 14-1 | Multiple LTE-CRS rate matching patterns | - Maximum number of LTE-CRS rate matching patterns in total within a NR carrier | 5-28 (Rate-matching around LTE CRS) |  | N/A |  | Per band | No | No |  | For DSS [The number of the additional CRS rate matching patterns reported in Rel-16 is accounted in the total number of rate matching pattern reported by the UE for Rel-15 by using pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot and pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot] | Optional with capability signaling {2, 3, 4, 5, 6} |
|  | 14-2 | Multiple LTE-CRS overlapping rate matching patterns within a part of NR carrier overlapping with a LTE carrier | - Up to two LTE-CRS overlapping rate matching patterns within a part of NR carrier overlapping with a LTE carrier | 14-1 (Multiple LTE-CRS rate matching patterns), x-x (mTRP support) |  | N/A |  | [Per band] | No | No |  | For DSS [The number of the additional CRS rate matching patterns reported in Rel-16 is accounted in the total number of rate matching pattern reported by the UE for Rel-15 by using pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot and pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot] | Optional with capability signaling {supported} |
|  | 14-3 | PDSCH Type B mapping of length 9 and 10 OFDM symbols | - Indicates whether the UE supports PDSCH Type B scheduling of length 9 and 10 OFDM symbols | 5-6a (PDSCH mapping type B) |  | N/A |  | Per band | No | No |  | For DSS | [Mandatory without capability signailng] |
|  | 14-4 | One slot periodic TRS configuration for FR1 | - UE can be configured with one-slot periodic TRS configuration only when no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated | 2-51 (CSI-RS for tracking) |  | N/A |  | Per UE | No (TDD only) | No (FR1 only) |  | This capability may not be necessary if 2-51(CSI-RS for tracking) is modified. Following options can be considered. Opt.1: 1 slot periodic TRS is optional as agreed. There is no change on 2-51. Even if UE reports value 2 for MaxBurstLength of csi-RS-ForTracking, support of 1 slot periodic TRS in FR1 depends on this new UE capability signaling. Opt.2: 1 slot periodic TRS is optional as agreed. 2-51 is updated so that UE can report either “2” or “1 and 2” for MaxBurstLength (i.e., for Rel-16 UE, candidate value “1” is updated to “2” for FR1). New UE capability signaling may not be necessary. Opt.3: 1 slot periodic TRS is optional as agreed. UE reporting value 2 in FR1 means both 1 slot and 2 slots TRS should be supported. Rel-16 capability should only indicate whether 1 slot TRS can be configured in FR1. ] | Optional with capability signalling {supported} |
|  | 14-5 | SRS Tx switch with allowing downgrading configuration | 1) Support SRS Tx port switch 2) Report whether the uplink Tx switching impact to downlink receiving in a band 3) Report whether the UL Tx is switched together with UL Tx in another band [Define affected DL and UL bands by using txSwitchImpactToRx and txSwitchWithAnotherBand for the new (downgraded) entries] | 2-53 (SRS resource) [2-55] |  | N/A |  | [Per band combination] [FSPC] | No | No |  | Agreement: •Rel-16 UE capability design for SRS antenna switching in conjunction with the existing Rel-15 UE capability should allow UE to indicate support of one of the following combinations o{t1r1, t1r2} o{t1r1, t1r2, t1r4} o{t1r1, t1r2, t2r2, t2r4} o{t1r1, t2r2} o{t1r1, t2r2, t4r4} o{t1r1, t1r2, t2r2, t1r4, t2r4} oNote: Detailed signaling design is up to RAN2 | Optional with capability signalling Component^1: Candidate value set: { o{t1r1, t1r2} o{t1r1, t1r2, t1r4} o{t1r1, t1r2, t2r2, t2r4} o{t1r1, t2r2} o{t1r1, t2r2, t4r4} o{t1r1, t1r2, t2r2, t1r4, t2r4} } Component-2: Candidate value set: {yes, no} Component-3: Candidate value set: {yes, no} |
|  | [14-6] | Half-duplex UE behaviour in TDD CA | - Support for directional collision handling between reference and other cell(s) for half-duplex operation in CA with same SCS - Support for directional collision handling between reference and other cell(s) for half-duplex operation in CA with different SCS |  |  | N/A |  | [Per band combination] [Per UE] | No (TDD only) | No |  | UEs that do not indicate this capability should still be able to operate half-duplex TDD CA if network ensures same transmission direction across all the serving cells | [Mandatory with capability signaling for intra-band CA band and for inter-band CA in band combination without RAN4 FG 2-5 capability] [Optional with capability signaling] |
|  | [14-7] | New RACH configuration for FR1 TDD | - new RACH configuration entries with subframe number 2 and/or 7 for RACH periodicity longer than 10 ms |  |  | N/A |  | N/A | No | [Applicable only to FR1] |  | Agreement: •A new UE capability is not introduced for this TEI, i.e., it is a mandatory UE feature for Rel-16. | Mandatory without capability signalling |
|  | [14-8] | New capability for beamSwitchTiming values of 224 and 336 | - 48 is used as the beam switching threshold for UEs reporting 224 or 336 - When using sym224 and sym336, beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission in a CSI-RS resource configured with repetition ‘ON’ to apply TCI indication in CSI-RS triggering DCI. |  |  |  |  |  |  |  |  | Agreements: ・48 is used as the beam switching threshold for UEs reporting 224 or 336 When using the higher values of the feature (sym224 and sym336), beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission in a CSI-RS resource configured with repetition ‘ON’ to apply TCI indication in CSI-RS triggering DCI. |  |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | Since Rel-15 UE may also report beamSwitchTiming value of 224 or 336, a new UE capability for Rel-16 TEI feature on Aperiodic CSI-RS Triggering for UE reporting beamSwitchTiming values of 224 and 336 is needed to differentiate UE and gNB behavior. One possible solution is to introduce following UE capability in 38.331. beamSwitchTiming–R16 SEQUENCE { scs-60kHz ENUMERATED { sym224, sym336} OPTIONAL, scs-120kHz ENUMERATED { sym224, sym336} OPTIONAL } OPTIONAL, |
| Nokia | - 14-3: component 2 should be renamed as component 1 |
| ZTE | Comments on 14-1 and 14-2: The previous relevant agreements are as follows. The agreement only indicates that Z is UE capability, which is 14-1. We fail to identify clear agreements to indicate the configuration of overlapping CRS patterns is a UE capability. Hence we suggest to remove 14-2 or at least put it in brackets for now, until the need is justified after discussion. Agreements: - For DSS TEI - Maximum number of LTE-CRS non-overlapping rate matching patterns within a NR carrier is X - X is three - Maximum number of LTE-CRS overlapping rate matching patterns within a part of NR carrier overlapping with a LTE carrier is Y - Y is two - Maximum number of LTE-CRS rate matching patterns in total within a NR carrier is Z - Z is six - UE capability for Z={[1,] 2, 3, 4, 5, 6} is defined Comments on 14-4: Between Opt.1 (introduce new capability signaling) and Opt.2 (update 2-51), we prefer Opt. 2, since it can avoid the confusion of joint interpretation on two features caused by Opt. 1. But for Opt.2, rather than having 3 candidate values, we think a simpler solution is to change the candidate value “1” to “2”. That is, the candidate values of maxBurstLength in 2-51 are updated to “2” and “1 and 2”, and clarify that “1 and 2” is mandatory for FR2. |
| Huawei, HiSilicon | - On FG 14-6, for the input of “mandatory/optional”, we’d like to leave it open for discussion at this stage. - On FG 14-7: “No” should be changed to “Applicable only to FR1” for the column of “Need of FR1/FR2 differentiation” according to the agreements. |
| Ericsson | 14-3 - Propose to change the components description to - Indicates whether the UE supports PDSCH Type B scheduling of length 9 and 10 OFDM symbols. - Feature group should be mandatory |
| CMCC | 14-4 Either Opt.1 or Opt.2 is OK. For Opt.2, ZTE’s suggestion is also fine. |
| Intel | - Propose to add a clarification note for 14-1 and 14-2 that the number of the additional CRS rate matching patterns reported in Rel-16 is accounted in the total number of rate matching pattern reported by the UE for Rel-15 by using pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot and pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot - Regarding options provided for FG 14-4. We can’t modify the already existing UE capability for Rel-15. In Rel-15 UE is required to report value 2 in FR1, which means both 1 slot or 2 slots TRS should be supported. Rel-16 capability should only indicate whether 1 slot TRS can be configured in FR1. New UE capability should be supported per UE in Rel-16. - Regarding FG 14-5. Given that Rel-16 UE reports downgraded configuration, we propose to also include additional component 14-5a which defines affected DL and UL bands by using txSwitchImpactToRx and txSwitchWithAnotherBand for the new (downgraded) entries not reported in Rel-15. In addition we propose to report xTyR capability with the same granularity as in Rel-15, i.e. change ‘per band’ to ‘BC’ as in Rel-15. |
| Rapporteur (NTT DOCOMO) | Basically all comments are reflected in the updated feature list. - For FG14-4, Opt.1, Opt.2 (with modification suggested by ZTE) and Opt.3 (interpretation suggested by Intel) are listed in the note for further discussion. |

- 5G_V2X_NRSL

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15. 5G_V2X_NRSL | 15-1 | Receiving NR sidelink[by preconfiguration] | 1) UE can receive NR PSCCH/PSSCH. Up to [A] sidelink processes are supported. 2) UE can receive [X] PSCCH in a slot. 3) UE can decode [Y] RBs per slot (FFS: counting both PSCCH and PSSCH). 4) UE supports reception based on the normal 64QAM MCS table [and 256QAM MCS table]. 5) UE supports PT-RS reception in FR2. 6) The UE can receive [Z] total number of soft channel bits in a slot. 7) UE supports 2 receive antennas. [8) 30 kHz SCS is supported for FR1.] | None | FFS | No | \ | Per band | No | N.A. | N.A. | FFS: This is the basic FG for sidelink Component-2 candidate value set: {value1, value2, …} Component-3 candidate value set: {value1, value2, …} | Optional with capability signaling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
|  | [15-1a] | FFS: Receiving NR sidelink configured by NR Uu | 1) UE can receive NR PSCCH/PSSCH. Up to [A] sidelink processes are supported. 2) UE can receive [X] PSCCH in a slot. 3) UE can decode [Y] RBs per slot (FFS: counting both PSCCH and PSSCH). 4) UE supports reception based on the normal 64QAM MCS table [and 256QAM MCS table]. 5) UE supports PT-RS reception in FR2. 6) The UE can receive [Z] total number of soft channel bits in a slot. 7) UE supports 2 receive antennas. [8) 30 kHz SCS is supported for FR1.] | 15-1 |  |  |  |  |  |  |  | [Component-2 and Component-3 are the same as in FG15-1 and are not reported separately] | Optional with capability signalling |
|  | 15-2 | Transmitting NR sidelink mode 1 scheduled by NR Uu | 1) UE can transmit PSCCH/PSSCH using dynamic scheduling or configured grant type 1 and 2 in NR sidelink mode 1 scheduled by NR Uu. Up to [8] configured grants can be configured for a UE. 2) UE supports transmission based on the normal 64QAM MCS table [and 256QAM MCS table]. 3) UE supports PT-RS transmission in FR2. 4) UE can monitor DCI format 3_0 for NR sidelink dynamic scheduling and configured grant type 2. [5) UE supports rank 1 PSSCH transmissions.] [6) 30 kHz SCS is supported for FR1.] |  | Yes | No |  | Per band | No | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. FFS: This is the basic FG for sidelink. | Optional with capability signalling FFS: For UE supports NR sidelink [in licensed spectrum], UE must indicate this FG is supported. |
|  | 15-3 | Transmitting NR sidelink mode 2 configured by [NR Uu or] preconfiguration | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by [NR Uu or] preconfiguration. Up to [B] sidelink processes are supported. 2) UE supports transmission based on the normal 64QAM MCS table and 256QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform sensing and resource allocation operations. [5) UE supports rank 1 PSSCH transmissions.] [6) 30 kHz SCS is supported for FR1.] | 15-1 | FFS | No |  | Per band | No | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. FFS: This is the basic FG for sidelink. | Optional with capability signalling FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
|  | 15-3a | FFS: Transmitting NR sidelink mode 2 configured by NR Uu | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by NR Uu. Up to [B] sidelink processes are supported. 2) UE supports transmission based on the normal 64QAM MCS table and 256QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform sensing and resource allocation operations. [5) UE supports rank 1 PSSCH transmissions.] [6) 30 kHz SCS is supported for FR1.] | 15-1, 15-2 |  |  |  |  |  |  |  | Note: Random selection in the exceptional pool is supported. | Optional with capability signalling |
|  | 15-4 | S-SSB for NR sidelink [based on pre-configured parameters for GNSS based synchronisation in case of P0, P1, P2 and P6 priority sources] | 1) UE can receive S-SSB in NR sidelink if it supports 15-1. 2) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3. [3) UE supports GNSS based synchronisation based on P0, P1, P2 and P6 priority order of synchronisation sources] | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | No | N.A. | N.A. | FFS: This is the basic FG for sidelink. | Optional with capability signalling FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
|  | 15-4a | FFS: Separated FG(s) for other synchronization priorities. |  |  |  |  |  |  |  |  |  |  |  |
|  | 15-5 | Sidelink congestion control | 1) UE can report CBR measurement to gNB. 2) UE can adjust its radio parameters based on CBR measurement and CRlimit. |  | Yes | No |  | Per band | No | N.A. | N.A. |  | Optional with capability signalling |
|  | 15-6 | Short-term time-scale TDM for in-device coexistence | 1) Support prioritization between LTE sidelink transmission/reception and NR sidelink transmission/reception | At least one of 15-1, 15-2, 15-3 |  | No |  | Per band | No | N.A. | N.A. |  | Optional with capability signalling |
|  | 15-7 | Transmitting LTE sidelink mode 3 scheduled by NR Uu | 1) UE can be scheduled over NR Uu for LTE sidelink mode 3 transmission.. 2) UE reports a value ‘X’ for the minimum value it supports for the additional time indicated in the NR DCI scheduling LTE sidelink mode 3. 3) UE can monitor DCI format 3_1 for LTE sidelink SPS grant. |  | Yes | No |  | Per band | No | N.A. | N.A. | Component-2 candidate value set: {0.75 ms, 1 ms, [1.25 ms], [1.5 ms], 2 ms, 4 ms, 5 ms, 8 ms, 10 ms, 20 ms} | Optional with capability signalling |
|  |  |  | 1) |  |  |  |  |  |  |  |  |  |  |
|  | 15-9 | Transmitting LTE sidelink mode 4 configured by NR Uu or preconfiguration | 1) UE can be configured over NR Uu or preconfiguratino for LTE sidelink mode 4 operation.. |  | Yes | No |  | Per band | No | N.A. | N.A. |  | Optional with capability signalling |
|  | 15-10 | 256QAM sidelink transmission | 1) UE can transmit PSSCH with 256QAM in NR sidelink | At least one of 15-2, 15-3 | Yes | Yes | UE supports QPSK, 16QAM, and 64 QAM for transmission only. | Per band | No | N.A. | N.A. | Reception of 256QAM is FFS. | Optional with capability signalling |
|  | 15-11 | PSFCH format 0 | 1) UE can transmit and receive NR PSFCH format 0 2) UE can receive [N] PSFCH(s) in a slot. 3) UE can transmit [M] PSFCH(s) in a slot. 4) UE can report sidelink HARQ-ACK to gNB via PUCCH when it is operating in NR sidelink mode 1. | At least one of 15-1, 15-2, 15-3 | Yes | Yes |  | Per band | No | N.A. | N.A. | FFS: This is the basic FG for sidelink. | Optional with capability signalling FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
|  | 15-12 | Low-spectral efficiency 64QAM MCS table | 1) UE can transmit or receive PSSCH with low-spectral efficiency 64QAM MCS table. | At least one of 15-1, 15-2, 15-3 | Yes | Yes | UE supports normal 64QAM MCS table and 256QAM MCS table only. | Per band | No | N.A. | N.A. |  | Optional with capability signalling |
|  | 15-14 | Sidelink CSI report | 1) UE can transmit and receive sidelink CSI-RS with 1 or 2 antenna port(s). 2) UE supports RI and CQI feedback on sidelink. | 15-1 and at least one of 15-2 and 15-3 |  | Yes |  | Per band | No | N.A. | N.A. |  | Optional with capability signalling. |
|  | 15-15 | eNB type synchronization source for NR sidelink | 1) UE can transmit or receive NR sidelink based on the synchronization to an eNB. | At least one of 15-1, 15-2, 15-3 | Yes | Yes |  | Per band | No | N.A. | N.A. |  | Optional with capability signalling. |
|  | 15-16 | Simultaneous transmission of uplink and sidelink | 1) UE supports simultaneous transmission of NR uplink and NR sidelink (on different carriers) in all bands for which the UE indicated simultaneous sidelink and uplink support in a band combination. | At least one of 15-2 and 15-3 | Yes | No |  | Per band combination | No | N.A. | N.A. |  | Optional with capability signalling. |
|  | 15-17 | 4Rx antenna | 1) UE supports 4 receive antennas | 15-1 |  | No |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-18 | Support of Rank 2 transmission | 1) UE additionally supports Rank 2 transmission | At least one of 15-2 and 15-3 |  | Yes |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-19 | Support of Rank 2 reception | 1) UE additionally supports Rank 2 reception | 15-1 |  | Yes |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-20 | FFS: 60kHz SCS for FR1 | [1) UE additionally supports 60KHz SCS for FR1 for PSCCH/PSSCH/PSFCH/S-SSB] | FFS |  | No |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-21 | FFS: 15kHz SCS for FR1 | [1) UE additionally supports 15KHz SCS for FR1 for PSCCH/PSSCH/PSFCH/S-SSB] | FFS |  | No |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-22 | FFS: Support of SL slot less than 14 consecutive symbols | [1) UE additionally support transmission/reception of SL slot less than 14 consecutive symbols] | FFS |  | No |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-23 | FFS: Support of open loop SL power control and RSRP report | [1) Support open loop power control and RSRP report in case of unicast] | FFS |  | Yes |  | Per band | N/A | N/A |  |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| OPPO | - It should be clarified in 15-1/2/3 that the support of any of these FGs and the 256QAM MCS table does not imply that the UE also supports 256QAM transmission and reception. An indication for 256QAM transmission / reception will be separately provided. If reception of 256QAM is made mandatory later, then this clarification is not needed for 15-1. => (X) consequence of 15-10 already explains this. - Similar to 15-6, UE capability signaling for “long-term timescale FDM for in-device coexistence” may be also needed. => (X) long-term coordination is an implementation issue and no impact on UE capability. - Similar to 15-7 (for cross-RAT control), should we also have UE capability signaling to indicate its support for NR sidelink mode 1 (Type 1 CG) and mode 2 resource pool configuration by LTE Uu? => (X) Captured in the LTE feature list. - For 15-11 on PSFCH, it should be included as part of UE capability. [ ] brackets should be removed. => (O) - For 15-11, at least one of 15-1/2/3 should be prerequisite, otherwise UE cannot transmit/receive PSFCH. => (O) - For 15-12, at least one of 15-1/2/3 should be prerequisite. => (O) - For 15-13 on PT-RS, it is expected that if a UE supports NR sidelink operation in a FR2 band then it needs to support PT-RS transmission and reception. And normally it would be sufficient to make this FG to be mandatory in FR2. But since there is a need of FR1/FR2 differentiation, capability signaling for this FG seems to be still necessary. Due to this FG is not needed in FR1, it should be noted that UE’s capability signaling for this FG in FR1 can be ignored by the receiver (i.e. gNB and another UE). => (O) Some suggested editorial changes as in the above. |
| Nokia | - Similar list has been added to LTE discussions, to be clarified which specifications are capturing the UE features in the end and the expected process. => This will be in 38.306 and the other for LTE feature list will be in 36.306. - Many features are defined as “per band” but in fact they should be “per UE” with the understanding they are supported only in bands where V2X is supported. => The UE will indicate in which band it supports SL via “per band” signaling. At least two bands are supported in Rel-16, n38 and n47. - 15-1, component 4: the UE needs to support reception based on the normal 64QAM MCS table and 256QAM MCS table, and not only the tables themselves. => (O) - 15-8 should be a component of 15-7 => (O) - 15-12: the feature name has a negative connotation, it should be changed to something like “high-reliability 64QAM MCS table” => (X) This wording was used in the agreement and the RRC parameter name in 38.213 is 'qam64LowSE.' - 15-13 needs to be part of the basic V2X functionality when operating in FR2. => (O) |
| Samsung | 15-1/2/3) Remove “and 256QAM MCS table.” in FG 15-1/2/3. Based on the agreement below, supporting 256QAM in TX perspective is by UE capability (i.e., optional). In addition, there is another FG 15-10 to indicate whether a UE supports 256QAM transmission or not. For RX perspective, it will be discussed further. => (X) This means UE supports the MCS table which includes 256 QAM and whether a specific MCS index is supported for the actual TX/RX is subject to other features such as 15-10. This topic may be related to the number of MCS tables configured per resource pool as a UE not supporting 256 QAM should be able to transmit 64 QAM, for example, using the 256 QAM table when the resource pool is configured only with the 256 QAM table. Agreements (RAN1#98bis): - 256QAM is supported for SL. - Support of 256QAM by a UE is FFS between mandatory vs. based on UE capability from the Rx perspective - Support of 256QAM is based on UE capability from the Tx perspective - 64QAM is mandatory. 15-2) For component 1), change 'and' to 'or': if 'and' is used, then it misleads that UE simultaneously transmits dynamic scheduling, configured grant type 1 and 2. => (O) 15-3,9) For component 1), change 'and' to 'or': There will be mode 2 either configured by gNB or preconfiguration, not both. => (O) Update feature group name (i.e., third column) by adding 'or preconfiguration': mode 2 should be operated in in-coverage (configured by NR Uu) or in out-of-coverage (preconfiguration) 15-11) Add “2) UE can receive [N] PSFCH(s) in a slot”. To reflect the UE’s capability of processing multiple received PSFCH(s). => (O) 15-11) Add “3) UE can transmit [M] PSFCH(s) in a slot”. To reflect the UE’s capability of processing/transmitting multiple PSFCH(s) simultaneously. => (O) |
| NTT DOCOMO | The following features should be ‘mandatory’ for UE supports NR sidelink, i.e. FG 15-1 includes them. - SL TX in mode 1 and mode 2 (FG 15-2, FG 15-3): fundamental function for NR sidelink - SSB TX/RX (FG 15-4): fundamental function for NR sidelink - PSFCH TX/RX (FG 15-11): necessary function to satisfy NR sidelink requirements => Propose to have FFS In FG 15-1, 1) should be updated as below since RX UE does not differentiate between mode 1 and mode 2. 1) UE can receive PSCCH/PSSCH transmitted using NR sidelink mode 1 and 2. => (O) The following functions should be introduced either in FG 15-1 or as new FG. - SL HARQ feedback on UL: we believe that FG 15-1 can include this function since this is a basic function of mode 1.=> Captured in FG 15-11 as additional component. - Unicast/Groupcast/Broadcast: FG 15-1 includes cast-type as well, i.e. all cast-types are mandatory in NR sidelink UE. => (X) Not clear if this is the feature relevant to PHY |
| CATT | For 15-1, only receiving NR sidelink is defined as basic capability. But for NR sidelink to operate properly, both transmitting and receving shall be part of the basic capability. Moreover, as NR PSCCH and NR PSSCH is not simply FDM multiplexed, counting #RBs of PSCCH and PSSCH is not feasible. The 3rd bullet shall be revised. => Propose to have FFS For 15-11, HARQ-ACK feedback is a basic functionality of groupcast and unicast, so PSFCH shall be part of basic capability. In addition, only ‘PSFCH format 0’ is defined and it would be better to spell it explicity in UE feature definition. => Propose to have FFS |
| ZTE | Add one capability on CSI-RS transmission, i.e., support NR sidelink transmission using CSI-RS with 1 or 2 antenna port(s). This feature is supported Per UE, and intended to report to gNB. Optional with capability signaling. Add one capability on CSI-RS reception, i.e., support NR sidelink reception using CSI-RS with 1 or 2 antenna port(s). This feature is supported Per UE, and intended to report to Tx peer UE and/or gNB. Optional with capability signaling. Add one capability on CSI feedback, i.e., support RI and CQI feedback on sidelink for unicast. This feature is supported Per UE, and intended to report to Tx peer UE. Optional with capability signaling. The prerequisite feature group for this capability is “capability on CSI-RS reception”. => One FG for sidelink CSI report will be enough as it is configured via PC5-RRC which means two UEs in the unicast are already capable of sidelink TX and RX. |
| Futurewei | The basic FG should include the following: 15-1: Receiving NR sidelink mode 1 and 2. 15-2: Transmitting NR sidelink mode 1 scheduled by NR Uu 15-3: Transmitting NR sidelink mode 2 configured by NR Uu 15-5: Sidelink congestion control 15-11: PSFCH In addition, mandatory support for 15-4: S-SSB should be considered: having all sidelink UEs capable of receiving and transmitting the S-SSB would increase the chances of having all sidelink UEs using the same synchronization reference, thus would improve reliability. More importantly, transmission and reception of S-SSB are essential for UEs operation in out-of-coverage scenarios. => Propose to have FFS. For congestion control and S-SSB, it is proposed to follow the LTE feature list where the two were optional. We also suggest to include and additional feature for sensing that would be helpful for public safety UEs => (X) No specification support was agreed to handle mode 2 with no sensing except the exceptional pool case. 15-14. Sensing. Indicates if the UE is capable of full sensing or no sensing |
| MediaTek | - FG15-1: 2nd SCI can be added for support. => (X) 2nd SCI is a part of PSSCH. - FG15-2/15-3: Support of transmission based on “256QAM MCS table” can be removed from 2), which has been indicated separately in 15-10. (X) This means UE supports the MCS table which includes 256 QAM and whether a specific MCS index is supported for the actual TX/RX is subject to other features such as 15-10. - FG15-4: “mode1 and mode 2” can be removed since S-SSB transmission is not relevant to mode 1 and mode 2. => (O) - FG15-7/15-8: these two FGs can be merged as one. => (O) - FG15-11: adding “2) UE can forward SL HARQ feedback carried in PSFCH to BS for NR sidelink mode 1”. => (O) - FG15-12: the “Prerequisite feature groups” can be “at least one of 15-1,15-2 and 15-3”. => (O) - FG15-13: removed. Additionally, it can be added into 15-1 as “ 5) PT-RS reception in FR2.”, and 15-2 as “3) 5) PT-RS transmission in FR2.” and 15-3 as “5) PT-RS transmission in FR2.”. => (O) - FG15-14 is added as “UE can support of CSI measurement and reporting” => (O) - FG15-15 is added as “UE can support the other DMRS patterns than those used for 14 SL symbol operation” => Further discussion seems necessary, at least considering PSSCH in a slot configured with PSFCH. |
| LG | It is suggested to add the number of simultaneous PSFCH transmissions to the component box of 15-11, so that all UEs supporting PSFCH has the same capability for it. An example text is “2) UE supports up to [X: FFS] PSFCH transmissions in the same slot.” |
| Huawei, HiSilicon | FG 15-1: Component 5) should be added based on the UE capability defined in LTE-V2X: => (O) 5) The UE can receive [Z] total number of soft channel bits in a slot. This kind of parameter is defined in LTE-V2X, because it sets the requirement for the UE buffer implementation. If the buffered total data from all candidate UEs is larger than the supported soft channel bits, the UE can drop some part of the received data. Component 6) should be added to limit the maximum number of ACK/NACK PSFCHs that need to be received in a slot, for a similar reason as the limitation on PSCCH receptions: => This is addressed in a new component added to 15-11. 6) The UE can receive [W] total number of ACK/NACK PSFCHs in a slot. FG 15-2: n38 licensed band has been specified by RAN4 as a dedicated sidelink carrier. In order to fully control the sidelink licensed spectrum and perform better QoS assurance for sidelink communication, Mode 1 is the only choice for sidelink communication operating on licensed band such as n38. Suggest to modify the “Note” item as below: => Propose to have FFS This is the basic FG for sidelink using licensed spectrum Also, suggest to modify the “Mandatory/Optional” item as below Optional with capability signalling. For UE supports NR sidelink in licensed spectrum, UE indicates this FG is supported. FG 15-11: Support of unicast and groupcast are key differentiating features for NR-V2X. It is important that they are not deployed in a weak form at first by UEs that may remain in the field for many years. PSFCH should be a mandatory feature. => propose to have FFS FG 15-12: There does not seem to be any necessity to condition low-spectral efficiency 64-QAM support on also having 256-QAM. The “and 256 QAM MCS table” words should be deleted from the consequence column. . => (X) This means UE supports the MCS table which includes 256 QAM and whether a specific MCS index is supported for the actual TX/RX is subject to other features such as 15-10. This topic may be related to the number of MCS tables configured per resource pool as a UE not supporting 256 QAM should be able to transmit 64 QAM, for example, using the 256 QAM table when the resource pool is configured only with the 256 QAM table. Add new FG 15-14: eNB type synchronization source for NR SL => (O) Use of eNB as a sync source for the NR sidelink should be optional per-band. NR UEs can be implemented without being required to include LTE synchronization capability to reduce the cost of the UE. The other columns of this are common with the other V2X FGs, with no pre-requisite. We also have the following agreement as in RAN1#94bis: Agreements on sidelink synchronization references - At least GNSS, gNB, NR UE, and eNB are supported as the synchronization source for NR V2X. - eNB as a synchronization source for NR V2X UEs supporting LTE Uu/PC5 or Uu only (no change to the eNB behaviour) - Whether a source is supported is for further NR V2X UE capability consideration |
| Panasonic | - Several minimum features related to sidelink operation of mode 2 should be merged to one basic feature. => Propose to discuss further - 15-3 should be contained in 15-1. 15-3 should be updated from "mode 2 configured by NR Uu" to "mode 2 configured by NR Uu". (Pre-)configuration is not required to be always signalled by NR Uu. => Propose to discuss further - PSFCH is not required to be transmitted/received but the resource reserved for PSFCH should be basic feature. => This is the UE operation on receiving the resource pool configuration, so no additional feature is necessary. - The feature indicator related to simultaneous operation between Uu and sidelink would be required as per band combination. => Does this propose to change “per band” to “per band combination?” |
| Ericsson | 15-2 and 15-5: - The gNB must know if the feature is supported. => (O) - and 15-9: - The ‘components’ columns in this two feature groups are written as if this were LTE features. In our opinion, this should be rewritten to: => (O) - UE can be scheduled over NR Uu for LTE sidelink mode 3 transmission. - UE can be configured over NR Uu for LTE sidelink mode 4 operation. 15-10: - The gNB must know if the feature is supported for Mode 1. => (O) 15-11: - We do not think that having PSFCH as a separate feature group is necessary. In our view, everything related to HARQ feedback should be captured as a part of either 15-2 or 15-3. => Propose to have FFS 15-12: - It should also be optional with capability signaling. => (O) 15-13: - PT-RS is only supported for FR2. => (O) Some additional questions: - What is the motivation behind putting YES for differentiation between FR1 and FR2 for all the feature groups? => Now changed to N.A. with the understanding that “per band” indication can already differentiate FR1 and FR2. - How about transmission/reception of CSI-RS? It is not part of any feature group. In our view, it should be a separate UE feature: => (O) - Pre-requisite: 15-1 and one of 15-2 or 15-3. - Signaling between UEs: Yes - Per band - No TDD/FDD differentiation - Optional with capability signalling. |
| Intel | - Propose to add a feature for OLPC on sidelink (sidelink RSRP reporting) => unclear if separation is needed. - Propose to add a feature for TX parameter adaptation (MCS range, #of subchannels, # of ReTx) based on TX UE absolute speed => unclear if separation is needed. - Propose to split feature of SSB transmission and reception on separate capability for transmission and reception (UEs supporting Mode-1 only do not need to support SSB reception SSB) => The components are now separated by TX and RX according to the UE support for general SL TX and RX. Unclear if separation is needed. - Propose to confirm feature for PSFCH transmission and reception => The component is updated. - Propose to add a capability for support of simultaneous transmission of N PSFCH, where N is from 1 to X, and a particular number is a capability. May be a binary choice: support > 1 or not => Further discussion is necessary, but at this moment, it seems reasonable to have a single capability on the number of PSFCH TX. - Propose to add a capability for support of simultaneous reception of M PSFCH receptions per slot, where M is from 1 to X, and a particular number is a capability. May be a binary choice: support > 1 or not => Further discussion is necessary, but at this moment, it seems reasonable to have a single capability on the number of PSFCH RX. - Propose to add a feature of simultaneous transmission on UL and SL carrier (power sharing) => FG 15-16 is added similarly to “commSimultaneousTx-r12” in LTE. - Propose to add a capability on number of multiple SPS configurations for UEs supporting Mode-1 communication => See the updated component in 15-2, which is based on RAN2 working assumption. - Propose to add a capability on number of semi-persistent processes for UEs supporting Mode-2 communication => See the updated component in 15-3. - Propose to add a capability on number of RX HARQ processes for Mode-1 and Mode-2 communication => See the updated component in 15-1. - Propose to add a capability on inter-RAT communication/processing time for resolution of in-device coexistence conflicts => No agreement was made on this. - Propose to add a capability on number of simultaneous resource selection processes for UEs supporting Mode-2 communication => See the updated component in 15-3. - Propose to add a capability for PSCCH/PSSCH preparation time for UEs supporting Mode-1 communication if multiple PSCCH/PSSCH preparation times are agreed => can be discussed once agreed. |
| Apple | Feature 15-1, we suggest to change the first component to “UE can receive PSCCH/PSSCH transmitted using NR sidelink mode 1 and 2, configured by NR Uu and preconfiguration” => See the updated text Feature 15-2, we suggest adding a component of “UE can monitor DCI format 3_0 for NR sidelink dynamic scheduling and configured grant type 2” => (O) Feature 15-3, we suggest adding a component of “UE can perform sensing and resource allocation operations” => (O) Feature 15-7, we suggest adding a component of “UE can monitor DCI format 3_1 for LTE sidelink SPS grant” => (O) Feature 15-8, besides the list of supported values of X, spare values are reserved for future deployments. => Not sure if this feature list needs to mention the reserved fields as they will not be indicated by the UE in Rel-16. Feature 15-11, the prerequisite feature groups are at least one of 15-1, 15-2, 15-3. => (O) Feature 15-12, the prerequisite feature groups are at least one of 15-1, 15-2, 15-3. Also, it is preferred to change the component to “UE can transmit and receive PSSCH with low-spectral efficiency 64QAM MCS table” => (O) |
| Rapporteur (LG Electronics) | - One of the main issues would be whether to mandate several features as the basic feature of the sidelink. I observed that view are not so aligned among the companies on this, and even when RAN1 agrees to the basic feature, I assume it does not necessarily mean that there should be only a single basic FG. I think multiple FGs can be indicated as the basic FGs and mandated for every SL supporting UE (so we don’t need to try to merge several FGs). - I put FFS for most of the newly raised proposals including the optionality for the multiple SCS, separation of synchronization features, optionality for a slot shorter than 14 symbols, etc. - I think LTE feature list needs to be worked on after reaching the principle-wise consensus. Some companies commented that only cross-RAT related features need to be there, but my view is somewhat different. If eNB needs to perform proper scheduling considering both Uu and sidelink, eNB needs to know somewhat detailed UE feature. As an example, eNB needs to know whether a UE support PSFCH TX if it wants to avoid overlapping scheduling of UL TX and PSFCH TX. |

- NR_eMIMO

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16. NR_eMIMO | 16-1 | Multi-beam operation | - The maximum number of L1-SINR based beam measurement and reporting based on ZP IMR and/or NZP IMR (FFS whether to be a separate UE feature, e.g. 16-1a) - FFS: Support of group-based reporting for L1-SINR - Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH (FFS whether to be a separate UE feature, e.g. 16-1b) - Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS (FFS whether to be a separate UE feature, e.g. 16-1c) - The maximum number of PUCCH resources per BWP for simultaneous spatial relation update (FFS whether to be a separate UE feature, e.g. 16-1c) - Support of default spatial relation pathloss reference RS for dedicated-PUCCH/SRS and PUSCH scheduled by DCI format 0_0 - [Support of / The maximum number of] spatial relation update for AP-SRS via MAC CE (FFS whether to be a separate UE feature, e.g. 16-1c) - The maximum number of configured pathloss reference RS update for PUSCH/SRS/PUCCH via MAC CE (FFS whether to be a separate UE feature, e.g. 16-1c) - FFS: The maximum number of activated pathloss reference RS update for PUSCH/SRS/PUCCH [across CCs, within a slot across all CCs] (FFS whether to be a separate UE feature, e.g. 16-1c) - The maximum number of SCells for SCell beam failure recovery (FFS whether to be a separate UE feature, e.g. 16-1d) - FFS: Support of PUCCH-BFR (FFS whether to be a separate UE feature, e.g. 16-1d) - FFS: The maximum number of CSI-RS and/or SSB resources for new beam identification of SCell BFR [across all CCs, within a slot across all CCs] (FFS whether to be a separate UE feature, e.g. 16-1d) - FFS: Densigy of CSI-RS for SCell BFR | TBD for #1 2-1, 2-4 for #2 2-53, 2-59 for #3-#6 8-2 for #7 2-31 for #8 |  | N/A |  | TBD [Per band for #1, #2, #5, #6 Per UE for , #7, #8 Per BC for #3, #4] | N | N for #1, #2, #7, #8 Y for #3 - #6 |  |  | Optional |
|  | 16-2 | Multi-TRP operation | Multi-DCI-based: (FFS whether to be a separate UE feature, e.g. 16-2a) - The maximum number of CORESETs configured per “PDCCH-Config” - The maximum number of CORESETs configured per CORESETPoolIndex (if configured) per “PDCCH-Config” - The value of R=[1,2] for BD/CCE - Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex - The maximum number of PUCCH transmissions within a slot - Support of fully/partially time/frequency overlapped PDSCH reception (PDSCHs overlapping types in time and frequency domain) and out of HARQ process. - Support of two PDSCH scrambling sequences per serving cell - Support of default QCL assumption per CORESETPoolIndex - The maximum number of activated TCI states - FFS: The maximum number of MIMO layers of scheduled PDSCHs - Support of separate HARQ-ACK and/or joint HARQ-ACK Single-DCI-based: (FFS whether to be a separate UE feature, e.g. 16-2b) - Whether supporting two PTRS ports - Support of new DMRS entries - Support of default QCL assumption with two TCI states - FFS: Support of URLLC scheme 1a - Support of FDMSchemeA - Support of FDMSchemeB - Support of TDMSchemeA - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - For FDMSchemeB, whether the UE can support CW soft combining - Supported maximum TBS size for TDMSchemeA - Supported maximum TBS size according to RepNumR16 in PDSCH-TimeDomainResourceAllocation - FFS: TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping) | TBD |  | N/A |  | TBC [Per band] | N | TBD |  |  | TBD |
|  | 16-3 | eType-II codebook | Regular eType-II (FFS whether to be a separate UE feature, e.g. 16-3a) - FFS: {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II - 8 parameter combinations (FFS: Value of L per the number of antenna ports) - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) - Rank 1 to 4 - CBSR - Rank restriction - FFS: UCI omission - FFS: The maximum number of configured aperiodic CSI Report Settings - FFS: Support of mixed codebook types Port selection eType-II (FFS whether to be a separate UE feature, e.g. 16-3b) - FFS: {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II - 6 parameter combinations (combos with L=6 don’t apply) (FFS: Value of L per the number of antenna ports) - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) - Rank 1 to 4 - Rank restriction - FFS: UCI omission - FFS: The maximum number of configured aperiodic CSI Report Settings - FFS: Support of mixed codebook types | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-4 | Low PAPR DMRS for DL | Low PAPR DMRS for PDSCH | TBD |  | N/A |  | Per UE | N | N |  |  | [Optional] |
|  | 16-5 | Power-efficient UL transmission | UL full power transmission (mode [0], 1, 2) - Supported UL full power transmission mode ([mode 0], mode 1, mode 2, [mode1AndMode2]) - FFS: New UL codebook set(s) per supported mode - The maximum number of SRS resources in set with different number of ports [for usage set to ‘codebook’] - FFS: Number of ports per SRS resource - FFS: Maximum number of different spatial relationship info for all SRS resources for usage set to ‘codebook’ in a resource set - TPMI group which delivers full power Note: UE indicating mode 2 shall support full power transmission for 1 antenna port | [2-13, 2-14] for #1-#3 |  | N/A |  | Per UE | N | N |  |  | TBD |
|  | 16-6 | Low PAPR DMRS for UL | - For PUSCH without transform precoding - For PUSCH with transform precoding and with pi/2 BPSK modulation - For PUCCH format 3 and/or PUCCH format 4 with transform precoding and with pi/2 BPSK modulation | TBD for #1-#3 |  | N/A |  | Per UE | N | N |  |  | TBD |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Vivo | - On feature 16-1, suggest following revisions to the components part - Component 1: Support of L1-SINR based beam selection (Done) - Support ZP IMR configuration - Support NZP IMR configuration - Component 2: Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH - Band-combination list that supports this operation (Done, signalling per BC) - Max number of different QCL-D RS resources in the active TCI states per band combination; - Component 3: Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - Band-combination list that supports this operation (Done, signalling per BC) - Max number of different RS resources in the active spatial relations states per band combination; - Component 4: Simultaneous spatial relation update for multiple PUCCH resources (Done) - max number of PUCCH resource groups per BWP) - Component 7: Pathloss reference RS update for PUSCH/SRS via MAC CE (Done) - max number of configured pathloss reference RS for PUSCH/SRS/PUCCH - Component 8: SCell beam failure recovery - max # SCells configured for BFR - Support of PUCCH-BFR configuration (Done) - In addition, component 1 and 8 can be split out as separate UE features, e.g. 16-1a, 16-1b (suggested as FFS) - On feature 16-2, suggest to split multi-DCI based and single-DCI based into separate UE features, i.e. 16-2a and 16-2b. In addition, suggest following revisions to the component part - For multi-DCI based (16-2a), adding following components - Increased number of PUCCH transmission within a slot; (Done) - Out of order PDSCH/PUCSH, including fully/partially time/frequency overlapped PDSCH; (Done) - Enhanced PDSCH scrambling; (Done) - For single-DCI based (16-2b), adding following components - Support of new DMRS entries; (Done) - Support of scheme 2a/2b/3/4; (Done) - Feature 16-3 can be split into two separate UE features, i.e. 16-3a Regular eType-II , 16-3b Port selection eType-II (suggested as FFS) - Feature 16-4 should be optional (suggested as FFS) - On feature 16-6, suggest the following additions to component 3 - Component 3: PUCCH with transform precoding and with pi/2 BPSK modulation (Done) - PUCCH format3 with pi/2 BPSK modulation - PUCCH format4 with pi/2 BPSK modulation |
| OPPO | [16-1] - Component 1: “Support of L1-SINR based beam selection” -> “Support of L1-SINR based beam measurement and reporting” (Done) - Component 2: The following should be added to the component (Done, signaling per BC) a. Maximum number of lists of CCs, b. Maximum number of CCs in one list - Component 3: Similar to Component 2, the following should be added (Done, signaling per BC) a. Maximum number of lists of CCs b. Maximum number of CCs in one list - Component 4: the following should be added (Done) - Maximum number of PUCCH resource groups - Component 5: “Default spatial relation for dedicated PUCCH/SRS” -> “Default spatial relation and pathloss reference RS for dedicated PUCCH/SRS” (Done) [16-2] - Missing components for Multi-DCI based - Support of multiple DCI based M-TRP transmission (receive multiple PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain) (Done) - Default QCL assumption with multiple PDCCHs (Done) - Out of order support of PDCCH to PDSCH with multiple PDCCHs (Done) - Out of order support of PDCCH to PUSCH with multiple PDCCHs (Done) - Out of order support of PDSCH to HARQ-ACK with multiple PDCCHs (Done) - Missing components for Single-DCI based - Support multiple TCI states in a DCI (Done) - Default QCL assumption with two TCI states (Done) - Supported schemes for URLLC (Done) - FFS: maximum value of URLLCRepNum for scheme 4 (Done) - FFS: TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping) (Done) [16-5] - Component 1: should be removed since the new codebook subset is defined without UE capability sigaling (suggested as FFS) - Component 2: “Multiple SRS resources in set with different number of ports” -> “Maximum number of SRS resources in set with different number of ports” (Done) - Component 3: “UE indication of TPMI(s) which delivers full power” -> “TPMI group which delivers full power” (Done) - Missing component Transmission mode, and the candidate values are {mode 0, mode 1, mode 2, [mode1AndMode2]} (Done) |
| Nokia | - 16-2: The following components should be listed here: (Done) - Multi-DCI based multi-TRP: Default beam assumption by supporting that the DM-RS ports of PDSCH are QCL-ed with the RS(s) with respect to the QCL parameter(s) used for PDCCH of the lowest CORESET index among CORESETs configured with the same value of CORESETPoolIndex. - Single DCI based multi-TRP (all schemes): Default beam assumption by using the TCI-states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states which are activated for PDSCH. - 16-2: In general, single-DCI and multi-DCI should be seen as components of multi-TRP related features, which would be the schemes themselves. In the current organization it is not clear if the UEs are expected to support all mTRP schemes that can be operated with single- or multi-DCI. Moreover, the current organization is not covering some of the schemes, in particular NC-JT and URLLC scheme 2a. (Done) - 16-3: At least for port selection eType-II the maximum rank should be determined based on the Rel-15 capabilities, which are dependent on the frequency band. (suggested as FFS) - 16-4: Replace “power efficient DL transmission” with Low PAPR DMRS for PDSCH. In the component description use the actual terminology used in the specification texts. (Done) - 16-5: “new UL codebook set(s)” is not very descriptive and it will become ambiguous in future releases. To be revised with more precise definition. (Please provide specific suggestions) - 16-6: Component 1 should be mandatory and a separate feature from components 2 and 3. (Marked as TBD) |
| Samsung | 16-2) suggest to make two separate feature groups; one for multi-DCI and the other for single-DCI (Done) 16-2) as per following agreement, add a new component ‘Whether supporting default QCL behaviour per CORESETPoolIndex’ under multi-DCI (Done) Agreement (RAN1#99) For multi-DCI based multi-TRP/panel transmission, if CORESETPoolIndex is configured, - If the time offset between the reception of the PDCCH and the corresponding PDSCH is less than a threshold, UE could assume that the DM-RS ports of PDSCH are QCL-ed with the RS(s) with respect to the QCL parameter(s) used for PDCCH of the lowest CORESET index among CORESETs configured with the same value of CORESETPoolIndex, - in the respective latest slot in which one or more CORESETs associated with each of CORESETPoolIndex within the active BWP of the serving cell are monitored by the UE - The support of this feature is indicated by UE capability - If the UE does not support the above feature, Rel-15 behavior is reused regardless of CORESETPoolIndex. 16-2) As per following agreement, add a new component ‘Whether supporting URLLC scheme 2a and/or 2b’ under single DCI (Done) Agreement (RAN1#97) For M-TRP based URLLC, support both 2a and 2b - Scheme 2a and 2b have separate UE capabilities.. 16-2) Similar approach as above is applicable for URLLC scheme 1a/3/4, i.e., add new components under single DCI - ‘Whether supporting URLLC scheme 1a’ and ‘Whether supporting URLLC scheme 3 and/or 4’ (Done) 16-2) add a new component ‘Whether supporting new antenna port indication tables’ under single DCI (Done) 16-2) As per following agreement, add a new component ‘Whether supporting two default TCI states’ under single DCI (Done) Agreement (RAN1#99) For single-DCI based Multi-TRP/panel transmission with at least one configured TCI states for the serving cell of scheduled PDSCH containing 'QCL-TypeD', - If the offset between the reception of the PDCCH and the corresponding PDSCH is less than timeDurationForQCL and after the reception of activation command of TCI states for UE specific PDSCH, the UE may assume that DMRS ports of PDSCH follows QCL parameters indicated by default TCI state(s) as following: - Use the TCI-states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states which are activated for PDSCH. - If all the TCI codepoints are mapped to a single TCI state, then Rel-15 behavior is followed The support of this feature is part of UE capability.. 16-3) Delete ‘CBRS’ under Port selection eType-II. For eType-II port selection, we only have rank restriction -- we don’t have CBSR on beams (Done) 16-4) Set to ‘optional’ (Suggested as FFS) |
| NTT DOCOMO | We suggest to modify as following: 16-1: 1. Support of L1-SINR based beam selection reporting (Done) 5: Default spatial relation/pathloss reference RS for dedicated-PUCCH/SRS and PUSCH scheduled by DCI format 0_0 (Done) 16-2: Multi-DCI-based: (Done) 1: The maximum number of CORESETs configured per “PDCCH-Config” 2: The maximum number of CORESETs configured per TRP (i.e. same value of HigherLayerIndexPerCORESET (if configured) per “PDCCH-Config”) 4: Whether the UE shall rate match around configured CRS patterns which is associated with the same HigherLayerIndexPerCORESET (if configured) We suggest to add following: 16-2: Multi-DCI-based: - Support of default TCI-state for multi DCI (Done) - The total number of activated TCI states that a UE supports (Done) - The total number of MIMO layers of scheduled PDSCHs (Suggested as FFS) - Time/freq. overlapped type(s) of scheduled PDSCH (fully/partially/non-overlapped) (Done) Single-DCI-based: - Support of default TCI-state for single DCI (Done) - Supported URLLC scheme type(s) of (1a/2a/2b/3/4) (Done) |
| CATT | - 16-1: 3 and 4 can be combined, similar to 2 where PDSCH/PDCCH are combined. (Please provide specific suggestions) - 16-2: should be optional. In addition, the following agreements should be captured for 16-2: (Marked as TBD) Agreement For single-DCI based Multi-TRP/panel transmission with at least one configured TCI states for the serving cell of scheduled PDSCH containing 'QCL-TypeD', - If the offset between the reception of the PDCCH and the corresponding PDSCH is less than timeDurationForQCL and after the reception of activation command of TCI states for UE specific PDSCH, the UE may assume that DMRS ports of PDSCH follows QCL parameters indicated by default TCI state(s) as following:  Use the TCI-states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states which are activated for PDSCH. - If all the TCI codepoints are mapped to a single TCI state, then Rel-15 behavior is followed The support of this feature is part of UE capability. Agreement For M-TRP based URLLC, support both 2a and 2b - Scheme 2a and 2b have separate UE capabilities. Regarding M-DCI part, according to the latest version of spec., instead of “HigherLayerIndexPerCORESET”, the terminology “CORESETPoolIndex” should be used. (Done) 16-4: this should be “TBD” now instead of “mandatory”. There is no RAN1 agreement that this feature should be mandatory. (Suggested as FFS) |
| ZTE | 16-2: Multi-TRP We think separate UE capabilities should be supported for multi-DCI and single DCI based. (Suggested as FFS) In addition, some agreed UE capabilities are missed as follows (Done) Agreement For multi-DCI based multi-TRP/panel transmission, if CORESETPoolIndex is configured, - If the time offset between the reception of the PDCCH and the corresponding PDSCH is less than a threshold, UE could assume that the DM-RS ports of PDSCH are QCL-ed with the RS(s) with respect to the QCL parameter(s) used for PDCCH of the lowest CORESET index among CORESETs configured with the same value of CORESETPoolIndex, - in the respective latest slot in which one or more CORESETs associated with each of CORESETPoolIndex within the active BWP of the serving cell are monitored by the UE - The support of this feature is indicated by UE capability - If the UE does not support the above feature, Rel-15 behavior is reused regardless of CORESETPoolIndex Agreement For single-DCI based Multi-TRP/panel transmission with at least one configured TCI states for the serving cell of scheduled PDSCH containing 'QCL-TypeD', - If the offset between the reception of the PDCCH and the corresponding PDSCH is less than timeDurationForQCL and after the reception of activation command of TCI states for UE specific PDSCH, the UE may assume that DMRS ports of PDSCH follows QCL parameters indicated by default TCI state(s) as following: - Use the TCI-states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states which are activated for PDSCH. - If all the TCI codepoints are mapped to a single TCI state, then Rel-15 behavior is followed The support of this feature is part of UE capability. Agreement Send LS to RAN2 as a response to R1-1911803. The LS is endorsed in R1-1913446. Question 1. Does the total number of CORESETs per cell need to be increased from current 12 corresponding to 3 CORESETs per BWP? RAN1 Answer1: The maximum total number of configured CORESETs per cell (across BWPs) is 16 Question 2. Does RAN1 think the current operation is sufficient for mPDCCH mTRP operation? RAN1 Answer 2: RAN1 would like to support 8 activated TCI states per TRP, i.e. per CORESETPoolIndex. The total number of activated TCI states that a UE supports is subject to UE capability. Further detailed design is up to RAN2. Agreement For M-TRP based URLLC, support both 2a and 2b - Scheme 2a and 2b have separate UE capabilities. - For scheme 2b, - Additional UE capability is specified to inform the gNB whether the UE can support CW soft combining - Support up to two-layer transmission - In the case of one layer, up to two CBs per CW - In the case of two layers, one CB per CW - FFS: Support of multi-DCI based FDM scheme with repetition (to be concluded in RAN1#98) - FFS: Support of independent MCS selection for each TRP 16-3: eType-II codebook Comments on 16-3-2 “2 PMI FD resolutions (R=1 and 2)” should be changed to “Number of PMI sub-bands” based on the following relevant agreement in RAN1#99. (Done) Agreement On the # of PMI subbands: - Mandatory support for R=1 - Optional support for N3>19 FFS: whether it is mandatory or optional for R=2 and N3<=19 - To be discussed during the Rel-16 UE capabilitity discussions It is to be decided with the following alternatives Alt 1: R=2 and N3<=19 is mandatory if eType-II codebook is supported Alt 2: R=2 and N3<=19 is optional if eType-II codebook is supported From our perspective, a large portion of the performance gain of Rel-16 Type II comes from the utilization of R=2. Further, restricting N3<= 19 can keep the UE complexity/HW resource consumption same as Rel-15. Hence we support Alt 1. (Suggested as FFS) Comments on 16-3-5 “UCI omission” should be removed. In Rel-15, we don’t have signaling to indicate whether UCI omission is supported. We don’t see the need in Rel-16 either. Further, the UE behavior if UCI omission is not supported is never defined. (Suggested as FFS) Other comments in 16-3 The following agreement is made on RAN1#99, which requires update of a Rel-15 UE feature. We think it can also be treated here. (Please provide specific suggestions) Agreement In Rel-16, for CSI report other than beam report: - Increase the maximum number of configured aperiodic CSI Report Settings a UE can report per BWP (RRC parameter maxNumberAperiodicCSI-PerBWP-ForCSI-Report) up to 8, subject to UE capability - The maximum number of configured aperiodic CSI Report Settings a UE supports is part of UE capability - Note: This increases the value range of the associated UE capability signaling from 1...4 to 1...8 - Note: FFS for FR2 whether UE can report a value larger than 4 16-5 Power-efficient UL transmission Comments on 16-5 (Suggested as FFS) Firstly, Mode 0 should be removed due to the reason that there is no clear definition in RAN1, and instead we can add one more component of supporting full power transmission, as in RRC framework for full power transmission. Secondly, if the UE supports the Mode1, new UL codebook set(s) should be supported accordingly. Consequently, the 16-5-1 “New UL codebook set(s)” should be replaced by support Full power Mode 1 or Full power Mode2. Thirdly, the maximum number of SRS resources in Mode 2 should be UE capability according to the agreement in RAN1#98 (copied below). Consequently, we have the following suggestions on this feature group. UL full power transmission - Support full power transmission - Support full power Mode 1 or Mode 2 - For full power mode 2, - The support of multiple SRS resources in set with different number of ports - Maximum number of SRS resources - UE indication of TPMI(s) which delivers full power. Agreement in RAN1#98 - For 4 TX UEs, a maximum of 4 SRS resources are supported in Mode 2 for usage set to ‘codebook’ in a set - Depending on UE capability, either up to 2 or 4 SRS resources are supported - For 2 TX UEs, a maximum of 4 SRS resources are supported in Mode 2 for usage set to ‘codebook’ in a set - Depending on UE capability, either up to 2 or 4 SRS resources are supported - For mode 2 UEs, up to 2 different spatial relation info can be configured for all SRS resources with usage set to ‘codebook’ Note: it does not mean to support simultaneous transmission of multiple SRS resources usage is set to ‘codebook’ |
| MediaTek | - For FG16-1 multi-beam operation, - This feature should be split into the following features because a UE may not support all, e.g. Scell BFR is needed for CA only (Suggested as FFS) - FG16-1a: L1-SINR based beam selection - FG16-1b: DL BM enhancement - FG16-1c: UL BM enhancement - FG16-1d: Scell BFR - Item #1: UE capability signaling should be able to support the following reports that are similar to those in FG2-24, instead of just saying support or not: (It is preferred to revisit this proposal after the first round of revision) - Max number of SSB/NZP-CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured for CMR of L1-SINR within a slot shall not exceed - Max number NZP-CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured for IMR of L1-SINR within a slot shall not exceed - Max number CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured for IMR of L1-SINR within a slot shall not exceed - Max number of NZP-CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured for CMR of L1-SINR shall not exceed - Max number of NZP-CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured for IMR of L1-SINR shall not exceed - Max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured for IMR of L1-SINR shall not exceed - Max number of aperiodic NZP-CSI-RS resources across all CCs configured for CMR of L1-SINR shall not exceed - Max number of aperiodic NZP-CSI-RS/CSI-IM resources across all CCs configured for IMR of L1-SINR shall not exceed Note: A candidate value ‘0’ can reflect NZP-IMR/ZP-IMR is supported or not for L1-SINR. - Item #6: UE capability signaling should support the report of “maxmum number of configured spatial relation for AP-SRS” (Suggested as FFS) - Item #7: UE capability signaling should support the report of “maximum number of configured path-loss RS” (Done) - For FG16-2 multi-TRP operation for eMBB/URLLC - This feature should be split into the following features because a UE may not support both (Suggested as FFS) - FG16-2a: Single-DCI based multi-TRP operation - FG16-2b: Multi-DCI based multi-TRP operation - Add new item with UE capability report for “out-of-order for HARQs responsive to multiple PDSCHs across TRPs” in multi-DCI based multi-TRP operation (Done) - Add new item with UE capability report for “Maximum number of cells supporting single-DCI based multi-TRP operation” in single-DCI based multi-TRP operation (Need further discussions) - Add new item with UE capability report for “Maximum number of cells supporting multi-DCI based multi-TRP operation” in multi-DCI based multi-TRP operation (Do we need other parameter in addition to 16-2-3?) - Add new item with UE capability report for “The number of PDSCHs using different TCI states within a slot” in both (Need further discussions) - Add new item with UE capability report for “Two indicated QCL-typeD TCI states for PDSCH reception” in both (Need further discussions) - For FG16-3 MU-CSI - Item #3: UE capability signaling should support the report of “Support of Rank 3 to 4” (It is captured by 16-3-3) - Add new item with UE capability report for ‘the maximum number of configured aperiodic CSI Report Settings’ (Suggested as FFS) |
| Huawei, HiSilicon | On FG 16-1 “multi-beam operation”: (It is suggested to revisit the candidate values of each component after revising component list first) - Regarding component #1 “Support of L1-SINR based beam selection”, add the following sub-components and corresponding candidate values - Component 1a) Supported type of interference measurement resource {“Dedicated”, “Non-dedicated”, “Both”} - Component 1b) Supported type of dedicated IMR {“NZP CSI-RS only”, “CSI-IM only”, “NZP CSI-RS only and CSI-IM only”, “NZP CSI-RS only and CSI-IM only and Mixed NZP CSI-RS + CSI-IM”} - Component 1c) Max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR for L1-SINR {2, 4, 8, 16, 32, 48, 64} - Component 1d) Max number of NZP CSI-RS and CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as IMR for L1-SINR {2, 4, 8, 16, 32, 48, 64} - Component 1e) The total max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) to measure RSRP and L1-SINR (CMR + IMR), activated pathloss RSs and different SSB/CSI-RS resources for UE to monitor PDCCH quality for PCell and SCell across all CCs within a slot. candidate value set is {0, 2, 4, 8, 16, 32, 64} - Component 1f) Maximum number of L1-SINR report setting per BWP - Regarding component #2 “Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH” candidate values: {support PDCCH only, support PDSCH only, support both} Add sub-Component 2a) Max number of lists of CCs configured by RRC {1, 2} (Suggested as per BC signalling) - Regarding component #3 “Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS” {candidate values: support AP-SRS only, support SP-SRS only, support both} Add sub-Component 3a) Max number of lists of CCs configured by RRC {1, 2} (Suggested as per BC signalling) - Regarding component #4 “Simultaneous spatial relation update for multiple PUCCH resources” Add sub-Component 4a)Max number of PUCCH resource groups per BWP {1, 2, 3, 4} (Done) - Regarding component #7 “Pathloss reference RS update for PUSCH/SRS via MAC CE” Candidate values：{support PUSCH only, Support SRS only, Support both} Add the following sub-components and corresponding candidate values - Component 7a) Maximum number of configured pathloss RSs {4, 8, 16, 64} (Done) - Component 7b) Maximum number of activated pathloss RSs across CCs {4, 8, 16, 32} (Suggested as FFS) - Component 7c) Maximum number of activated pathloss RSs within a slot across all CCs {4, 8, 16, 32} (Suggested as FFS) - Regarding component #8 “SCell beam failure recovery” Add the following sub-components and corresponding candidate values - Component 8a) Maximum number of SCell(s) for which the UE can be configured to perform BFR {1..31} (Done) - Component 8b) Maximum number of different CSI-RS and/or SSB resources across all SCell(s) for new beam identification 64 * {0.5, 1, …, 31} (Suggested as FFS) - Component 8c) Maximum number of different SSB/CSI-RS resources within a slot across all CCs (not limited to SCells) for UE to monitor PDCCH quality {2, 4, 8} (Suggested as FFS) On FG 16-2 “Multi-TRP operation for eMBB/URLLC”: (It is suggested to revisit the candidate values of each component after revising component list first) - For Multi-DCI-based component #1 “The maximum number of CORESET configured per “PDCCH-Config” candidate values {2, 3, 4, 5} - For Multi-DCI-based component #2 “The maximum number of CORESET configured per TRP” candidate values {1,2,3} - If UE supporting Multi-DCI-based Multi-TRP operation, at least one candidate value of each components #1 and #2 shall be reported by the UE. (Notes) - For Multi-DCI-based Multi-TRP operation, following components and corresponding candidate values are suggested: - Support enhanced scheduling/HARQ order Optional (Done) - Support the DMRS ports of PDSCH to be QCLed with the RS(s) with respect to the QCL parameter(s) used for PDCCH of the lowest CORESET index among CORESETs configured with the same value of CORESETPoolIndex, when the time offset between the reception of the PDCCH and the corresponding PDSCH is less than a threshold Optional (Marked as TBD) - For Single-DCI-based Multi-TRP operation, following components are added: - The maximal value of “RepNumR16” {8, 16}. Optional. When this component is not reported, enhanced slot-level repetition scheme is not supported. (Done) - Support transmission scheme(s) with enhanced reliability 3 bit. Optional. The bits from the leftmost to the rightmost correspond to schemes {“ FDMSchemeA”, “FDMSchemeB”, “TDMSchemeA” }. Value 1 in the bitmap indicates that the corresponding scheme is supported, otherwise it is not supported. (Done) - Supports default TCI state(s) if at least one configured TCI states for the serving cell of scheduled PDSCH contain “QCL-TypeD” and the offset between the reception of the PDCCH and the corresponding PDSCH is less than timeDurationForQCL and after the reception of activation command of TCI states for UE specific PDSCH. optional (Done) On FG 16-3 “eType-II codebook”: (It is suggested to revisit the candidate values of each component after revising component list first) - For component 2 for Regular eType-II and component 7 for Port selection eType-II, we suggest to replace it as following due to higher flexibility of balance between UE complexity and gNB implementation: - A list of supported combinations across all CCs simultaneously whereas each combination is {S, P, X, P_total}, where S is max # of PMI subbands, P is max # of Tx ports in one resource, X is max # of resources and P_total is total # of Tx ports candidate values S: {19:2:37}, P: {4, 8, 12, 16, 24, 32}, X:{1:1:64}, P_total:{4:1:256} - {S, P, X, P_total} is per band per BC and other components for eType-II codebook is per band. (Suggested as FFS) - For component 5 for Regular eType-II, we suggest to remove “UCI omission” from now unless RAN1 spec can clarify further UE behavior. (Suggested as FFS) On FG 16-5 “Power-efficient UL transmission”: (It is suggested to revisit the candidate values of each component after revising component list first) The name of Component-1 should be replaced to “Full power transmission mode(s)”, due to the fact that Transmission mode-1 is only with a unique new codebook set, so “new codebook sets” is redundant. (Suggested as FFS) |
| Ericsson | 16-2 - Remove eMBB/URLLC from feature group names (Done) - Subfeature names should be aligned with RRC spec names (Done) - Add a top capability on whether UE support 2 different values of CORESETPoolIndex since this is the indication that UE support multi-DCI based schemes to start with. (Please review the updated component list) - For component 2, it is sufficient to say number of configured CORESETs with the same value of CORESETPoolIndex instead of TRP. Candidate value of 3 is agreed and other values can be discussed further. (Done) - Add the capabilities agreed in Reno, related to default beam (Done) - Add a new component which is the capability related to answer 2 in reply LS to RAN2 in RAN1#99. Corresponding agreement is “RAN1 would like to support 8 activated TCI states per TRP, i.e. per CORESETPoolIndex. The total number of activated TCI states that a UE supports is subject to UE capability. Further detailed design is up to RAN2.” (Done) - The above comments lead to the following component descriptions: - Multi-DCI-based: - Whether UE supports 2 different values of CORESETPoolIndex - The maximum number of CORESET configured per “PDCCH-Config” - The maximum number of CORESETs configured with the same value of CORESETPoolIndex (if configured) per “PDCCH-Config” - The value of R for BD/CCE - When receiving two PDSCH simultaneously, whether the UE support rate matching of each PDSCH only around the configured CRS patterns which is associated with the corresponding CORESETPoolIndex - Whether UE supports default beam, related to lowest CORESET with the same CORESETPoolIndex - Total number of activated TCI states that the UE supports - Single-DCI-based: - Whether UE support receiving two PTRS ports for one PDSCH - For reception according to FDMSchemeB, whether the UE support CW soft combining - The supported maximum TBS size for the UE for reception according to TDMSchemeA or when at least one entry in pdsch-TimeDomainAllocationList containing RepNumR16 in PDSCH-TimeDomainResourceAllocation, - Whether UE support default beam, related to lowest TCI codepoint with two TCI states - Signaling values per component as numbered above: (It is suggested to revisit the candidate values of each component after revising component list first) - Component-1:_ {[not supported], supported} - Component-2:_5 - Component-3: 3, [other values TBD] - Component-4: [1] - Component-5,6,8,9,11,: {[not supported], supported} - Component 7: [16] - Component-10: TBD 16-4: - Change FG name to “Low PAPR DMRS for DL” (Done) 16-5: - Elaborate the components with supported values as follows: (Please check the revised components per inputs from other companies) - New UL codebook set(s): indicate separate support for - 2 Tx: {not supported or not supported}, or - 4 Tx non-coherent: {not supported or supported}, or - 4 Tx partial coherent: {not supported or supported} - Multiple SRS resources in set with different number of ports: indicate separate support for different number of ports among resources when the UE is configured with the listed number of SRS resources: - 2 Tx: {not supported or either 1 or 2 resources}, or - 4 Tx non-coherent: {not supported or either 1, 2, or 4 resources}, or - 4 Tx partial coherent: {not supported or either 1, 2, or 4 resources} - UE indication of TPMI(s) which delivers full power: indicate separate support for - 2 Tx: {not supported, or either or both of TPMI=0 and TPMI=1 are supported}, or - 4 Tx non-coherent: {not supported or one of Group 0 – Group 3}, or - 4 Tx partial coherent: {not supported or one of Group 0 – Group 6} 16-6: - Change FG name to “Low PAPR DMRS for DL” (For UL, I guess?) - Change component descriptions for better clarity: (Done) - For PUSCH without transform precoding - For PUSCH with transform precoding and with pi/2 BPSK modulation (and 8-PSK modulation for the special case of a sequence of length 6) - For PUCCH format 3 and 4 with transform precoding and with pi/2 BPSK modulation - Signaling values per component: - Component-1,2,3:_ {[not supported], supported} |
| Convida Wireless | On FG 16-1 “multi-beam operation”: - Regarding component #8 “SCell beam failure recovery”: The agreed capability is the number of SCells for which the UE performs BFR, rather than the number of SCells with BFR configured. The corresponding agreement from RAN1#97 is pasted below: ----- Agreement A UE can be configured to perform BFR for any configured SCells - The maximum number of SCells for which the UE performs BFR is a UE capability ----- Hence, the component could be worded as: Component 8a) Maximum number of SCell(s) for which the UE performs BFR → {1..31} (Done) It can be noted that there are already three parameters in the Rel-15 specification that are applicable to SCell BFR: maxNumberCSI-RS-BFD INTEGER (1..64) OPTIONAL, maxNumberSSB-BFD INTEGER (1..64) OPTIONAL, maxNumberCSI-RS-SSB-CBD INTEGER (1..256) OPTIONAL, Definitions for parameters; Per; Mandatory; FDD TDD DIFF; FR1 FR2 DIFF. maxNumberCSI-RS-BFD Indicates maximal number of CSI-RS resources across all CCs, and across MCG and SCG in case of NR-DC, for UE to monitor PDCCH quality. In this release, the maximum value supported by the UE is upto 16. It is mandatory with capability signalling for FR2 and optional for FR1.; Band; CY; No; No. maxNumberSSB-BFD Defines maximal number of different SSBs across all CCs, and across MCG and SCG in case of NR-DC, for UE to monitor PDCCH quality. In this release, the maximum value supported by the UE is upto 16. It is mandatory with capability signalling for FR2 and optional for FR1.; Band; CY; No; No. maxNumberCSI-RS-SSB-CBD Defines maximal number of different CSI-RS [and/or SSB] resources across all CCs, and across MCG and SCG in case of NR-DC, for new beam identifications. In this release, the maximum value supported by the UE is upto 128. It is mandatory with capability signalling for FR2 and optional for FR1. The UE is mandated to report at least 32 for FR2.; Band; CY (Conditional mandatory); No; No. Additional components to limit the number of BFD-RS and RS for new beam identification doesn’t seem necessary. Note that these parameters refer to the RS the UE performs BFD/NBI on, i.e. only on active BWPs. The number of configured RS for BFR/NBI can be higher, since that number would include RS on deactivated SCells and inactive BWPs. (Please check the revised component list) |
| Intel | - Regarding 16-1 (1) - We propose to include sub-components with capability signalling: - Support of L1-SINR for SSB - Support of L1-SINR for NZP CSI-RS for both CMR and IMR - Support of L1-SINR for NZP for CMR and CSI-IM for IMR - Support of L1-SINR for NZP for CMR and NZP for CRM - For L1-SINR we propose to include UE capability for group-based reporting (Suggested as FFS) - For L1-SINR we propose to include the number of supported L1-SINR reports, similar to maxNumberNonGroupBeamReporting for L1-RSRP (Done) - For L1-SINR we propose to include reference signal configuration for L1-SINR - Option 1: Define new set of the reference signal configuration that can be configured for L1-SINR. Reference signal configuration for L1-RSRP reported in Rel-15 as part of beamManagementSSB-CSI-RS is shared with L1-SINR for concurrent configuration - Option 2: Define two new sets of the reference signal configuration that can be configured for L1-SINR and concurrently for L1-SINR / L1-RSRP - Define UE capability for periodic, sp, and aperiodic CSI report for L1-SINR - Regarding 16-1 (2, 3, 4) - Current capability is per UE and doesn’t allow indication of the band combination that can support simultaneous update of the TCI and spatial relations. Propose to further discuss granularity of the reporting so that the band combination supporting this feature can be indicate by the UE. (Marked as TBD) - Regarding 16-2 (multi-TRP) Multi-DCI (multiple PDSCH reception in the same slot and BWP) and single DCI (two TCI state per TCI codepoint) should be separate FG (Suggested as FFS) - Propose to include additional components for multi-DCI - #6 Out of order operation support (a) PDCCH to PDSCH, (b) PDCCH to PUSCH (c) PDSCH to HARQ-ACK (Done) - #7 HARQ operation support (a) separate HARQ-ACK (b) joint HARQ-ACK (Done) - #8 2 default PDSCH beam operation (Done) - For component 3. If pdcch-BlindDetectionCA is reported, reporting R=2 implies UE supports both R=1 and R=2. If pdcch-BlindDetectionCA is not reported, reporting R=2 implies UE supports only R=2. UE may support both R=1 and R=2 also if pdcch-BlindDetectionCA is not reported, add signalling of R=[1,2] (Done) - The range of the values for components 1 and 2 are {3,4,5}, UE should support at least 3 - Propose to include additional components for single DCI - Support of two TCI state per TCI codepoint (mandatory) (Done, mandatory/optional is marked as TBD) - Support of schemes (a) 2a, (b) 2b, (c) 3 (Done) - Support of additional DM-RS codepoint (mandatory) (Done, mandatory/optional is marked as TBD) - 2 default PDSCH beam operation (Done) - Granularity of capability is per band (Suggested as FFS) - Regarding 16-3 (eType II codebook) - The design of UE capability signalling for the eType-II codebook should be similar to other codebooks defined in codebookParameters In Rel-15 (Please check the revised component list. Also please provide specific suggestions, if any) - Include FFS on UE capability for mixed codebooks types (Done) - Regarding 16-5 (full power UL) - We propose the following structure: (Please check the revised component list) - Mode {0, 1, 2} - For Mode 2 capable UE, reporting of TPMI subset - For Mode 2 capable UE, reporting of the number of SRS resources with the same and different number of antenna ports Add note that UE indicating mode 2 shall support full power transmission for 1 antenna port (Done) |
| LGE | 16-1: For component1, it may be better to change ‘beam selection’ to ‘beam reporting’. (Done) 16-2: For both M-DCI and S-DCI, one component needs to be added respectively, i.e. ‘whether the UE supports two different default TCIs or not’ from the agreements in #99. This component infers whether the UE is able to maintain two Rx beams from two Rx panels simultaneously in FR2. (Done) 16-4 & 16-6: As both 16-4 and 16-6 are relevant to low PAPR RS, it may be better to change the feature tile of 16-4 as ‘low PAPR DL DMRS reception’ and that of 16-6 as ‘low PAPR UL DMRS transmission’. For 16-4, in addition, was there any agreement to mandate this feature? If not, we prefer to change it ‘TBD’ for now. (Done) |
| Apple | FG 16-1 Multi-beam operation: we suggest splitting this FG into 4 FGs with elements as follows - FG 16-1a: UL/DL beam selection (Generally done, please check the updated component list) - Support of simultaneous TCI update for PDSCH for multiple CCs - Support of simultaneous TCI update for CORESET for multiple CCs - Support of simultaneous spatial relation update for AP/SP-SRS for multiple CCs - Support of MAC-CE to update spatial relation for a group of PUCCH - Support of default PUCCH spatial relation (enableDefaultBeamPlForPUCCH) - Support of default SRS spatial relation (enableDefaultBeamPlForSRS) - Support of default spatial relation for PUSCH scheduled by DCI 0_0 (enableDefaultBeamPlForPUSCH0_0) - Band lists that shares the same QCL-typeD property - FG 16-1b: L1-SINR measurement and report (Generally done, please check the updated component list) - Maximum number of CMRs for L1-SINR measurement per slot per BWP - Maximum number of CMRs + IMRs for L1-SINR measurement per slot per BWP - Discuss separately ZP CSI-RS and NZP CSI-RS as IMR - Support of ZP-IMR based L1-SINR measurement - Support of NZP-IMR based L1-SINR measurement - FG 16-1c: SCell BFR - Maximum number of SCells configured for BFR/BFD (Done) - Density of CSI-RS for BFD (Done) - Maximum number of CBD RS per BWP (Please check the updated component list) - Maximum number of CBD RS + CMRs for L1-RSRP measurement per BWP (Please check the updated component list) - FG 16-1d: pathloss RS update (Done) - Support of pathloss RS update by MAC CE for SRS (enablePLRSupdateForPUSCHSRS) - Support of pathloss RS update by MAC CE for PUCCH (enablePLRSupdateForPUSCHSRS) FG 16-2 Multi-TRP operation for eMBB/URLLC: - FG 16-2a: multi-DCI based multi-TRP - We suggest to add the following (Done) - Out of Order (OOO) PDSCH (This was agreed to be an optional feature) - Out of Order (OOO) PUSCH (This was agreed to be an optional feature) - Out of Order (OOO) HARQ-ACK (This was agreed to be an optional feature) - Support of default PDSCH QCL assumption when time offset of less than a threshold. (This was agreed to be an optional feature) - Support of partially-overlapping PDSCH: even with DMRS configuration and TCI restriction, PDSCH can still partially-overlapped in frequency, time or both which creates many cases that UE needs to support in case UE need to support partially-overlapping PDSCH. - Discuss joint and separate PUCCH HARQ-ACK feedback, for separate (TDM) discuss - two TDMed long PUCCHs within a slot - TDMed short PUCCH and long PUCCH within a slot - TDMed short PUCCH and short PUCCH within a slot - FG 16-2b: single-DCI based single-TRP (Done) - We suggest to add the following - Supported URLLC scheme - SDM Scheme 1a: lacks name in specification - One TCI codepoint maps to 2 TCI states - Antenna ports (0,2,3) FDM Scheme 1a: FDMSchemeA - One TCI codepoint maps to 2 TCI states - StartingSymbolOffsetK - FDM Scheme 1b: FDMSchemeB - One TCI codepoint maps to 2 TCI states - RepNumR16 in PDSCH-TimeDomainResourceAllocation - TDM Scheme 2a: TDMSchemeA - One TCI codepoint maps to 2 TCI states - TDM Scheme 2b: lacks name in specification - One TCI codepoint maps to 2 TCI states - RVSeqOffset - Discuss RepTCIMapping among {CycMapping SeqMappin} - Support of default PDSCH QCL assumption when time offset of less than a threshold. (This was agreed to be an optional feature) FG 16-3 eType-II codebook: - FG 16-3a: Regular eType-II - 2 PMI FD resolutions (R=1 and 2) (Done) - Current status is that R=1 is mandatory conditioned on UE supports typeII-r16. (R=2 and N3>19) is optional. (R=2 and N3<=19) is for FFS. So it might be better to split in to three cases (1) R=1 (2) R=2 N3<=19 (3) R=2 N3>19 and merge them later after the agreement of which case is mandatory and which case is optional - CBSR and rank restriction (Done) - We are not sure about the meaning of rank restriction. However, we suggest to split this into two (1) Whether UE supports CBSR (2) If UE supports CBSR, where UE supports 4 values of restriction, i.e. amplitudeSubsetRestriction as in 38.214 - FG 16-3b: Port selection eType-II - Similar comment as typeII-r16 regarding the (1) 2 PMI FD resolutions (R=1 and 2) (2) CBSR and rank restriction (Done) - Additionally we propose to add the following - maxNumberAperiodicCSI-PerBWP-ForCSI-Report - It was agreed to increase this to 1…8 but we need to discuss whether the increase is needed for FR2 which is a FFS point in the agreement (Done) - Rel-15 CSI-RS processing capability per codebook should be extended, i.e. FG 2-36, 2-40, 2-41, 2-43 - Triplet {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for (1) Rel-16 Type II (2) Rel-16 Type II Port Selection (Suggested as FFS) - Further discuss per band/per BC issue etc. (Done) - Further discussion “Lx”, even though it was agreed that L=2/4 is mandatory and L=6 is optional, we still need to discussion “Lx” corresponding to 4,8,12,16,24 and 32 ports. (Done) FG 16-5: Power-efficient UL transmission. we suggest combining the first and second elements as follows - Supported full power mode with candidate value of {mode [0], mode 1, mode 2} (Done) - For mode 1 - Support new codebook subset for non-coherent and partial-coherent UE - For mode 2 - Multiple SRS resources in set with different number of ports, further discuss the following - Maximum of SRS resources for usage set to ‘codebook’ in a resource set (current specification allows up to 4) - Number of ports per SRS resource - Maximum number of different spatial relationship info for all SRS resources for usage set to ‘codebook’ in a resource set (current specification allows up to 2) UE indication of TPMI(s) group index which delivers full power |
| Spreadtrum Communications | 16-1: Considering the following agreement, suggest to add new component ‘The total number of RSs for new beam identification and layer 1 RSRP measurement’ (Done) Agreement Support the configuration of up to 64 candidate beams for BFR by RRC signalling, without introducing additional MAC CE signalling for down-selecting a subset of beams. - The total number of RSs for new beam identification and layer 1 RSRP measurement are part of UE capability signaling This applies per BWP.. 16-2: Considering the following agreement, to make it more clear, suggest to revise component 4 in Multi-DCI-based as ‘Whether the UE shall rate match around configured CRS patterns which is associated with HigherLayerIndexPerCORESET (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same higher layer index’ (Done) Agreement(RAN1#98b) For multi-DCI based multi-TRP/panel transmission, the UE shall rate match around: - Configured CRS patterns which optionally associated with a higher layer signaling index per CORESET (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same higher layer index. - This is a UE optional feature with separate UE capability signalling - If UE does not support this feature, the default UE behaviour is the following: - For multi-DCI based multi-TRP/panel transmission, the UE shall rate match PDSCH around configured CRS patterns from multiple TRPs FFS: Whether/How to handle DMRS shifting if CRS patterns are configured.. 16-2: Considering the following agreement, suggest to add new component ‘PDSCHs overlapping types in time and frequency domain’ for Multi-DCI-based: (Done) Agreement(RAN1#99) - If a UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet for the active BWP of a serving cell, the UE may expect to receive multiple PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain subject to UE capability - Note: This allows a UE to be not configured with either joint HARQ ACK feedback or separate HARQ ACK feedback - For the CORESET without CORESETPoolIndex, the UE may assume that the CORESET is assigned with CORESETPoolIndex as 0. 16-2: Considering the following agreement, suggest to add new component ‘The total number of activated TCI states to support’ for Multi-DCI-based. (Done) Agreement(RAN1#99) Send LS to RAN2 as a response to R1-1911803. The LS is endorsed in R1-19134463463. Question 1. Does the total number of CORESETs per cell need to be increased from current 12 corresponding to 3 CORESETs per BWP? RAN1 Answer1: The maximum total number of configured CORESETs per cell (across BWPs) is 16 Question 2. Does RAN1 think the current operation is sufficient for mPDCCH mTRP operation? RAN1 Answer 2: RAN1 would like to support 8 activated TCI states per TRP, i.e. per CORESETPoolIndex. The total number of activated TCI states that a UE supports is subject to UE capability. Further detailed design is up to RAN2. Question 3. RAN2 would like to ask RAN1 for confirmation that the understanding in RAN2 agreements is correct. RAN1 Answer 3: RAN1 would like to confirm that the understanding in RAN2 agreements is correct. 16-2: Considering the following agreement, suggest to add new component ‘Default beams for PDSCHs’ for Multi-DCI-based: (Done) Agreement(RAN1#99) For multi-DCI based multi-TRP/panel transmission, if CORESETPoolIndex is configured, - If the time offset between the reception of the PDCCH and the corresponding PDSCH is less than a threshold, UE could assume that the DM-RS ports of PDSCH are QCL-ed with the RS(s) with respect to the QCL parameter(s) used for PDCCH of the lowest CORESET index among CORESETs configured with the same value of CORESETPoolIndex, - in the respective latest slot in which one or more CORESETs associated with each of CORESETPoolIndex within the active BWP of the serving cell are monitored by the UE - The support of this feature is indicated by UE capability - If the UE does not support the above feature, Rel-15 behavior is reused regardless of CORESETPoolIndex. 16-2: Considering the following agreement, suggest to add new component ‘the total number of MIMO layers of scheduled PDSCHs if resource allocation of PDSCHs are overlapped’ for multiple-DCI-based: (Suggested as FFS) Agreement(RAN1#AH1901) For multi-DCI based multi-TRP/panel transmission, the total number of CWs in scheduled PDSCHs, each of which is scheduled by one PDCCH, is up to X and also the total number of MIMO layers of scheduled PDSCHs is up to reported UE MIMO capability, if resource allocation of PDSCHs are overlapped. - X=2 - FFS: X=3. 16-2: Considering the following agreement, suggest to add new component ‘Whether to support scheme 2a and/or 2b’ for Single-DCI-based: (Done) Agreement(RAN1#97) For M-TRP based URLLC, support both 2a and 2b - Scheme 2a and 2b have separate UE capabilities.. 16-2: Considering the following agreement, suggest to add new component ‘maximum value of URLLCRepNum for scheme 4’ for Single-DCI-based: (Done) Agreement(RAN1#99) For single-DCI based M-TRP URLLC scheme 4, the candidate values of URLLCRepNum is up to 16 - FFS: UE capability for maximum value of URLLCRepNum (including whether UE capability is needed). 16-2: Considering the following agreement, suggest to add new component ‘default TCI states for PDSCHs’ for Single-DCI-based: (Done) Agreement (RAN1#99) For single-DCI based Multi-TRP/panel transmission with at least one configured TCI states for the serving cell of scheduled PDSCH containing 'QCL-TypeD', - If the offset between the reception of the PDCCH and the corresponding PDSCH is less than timeDurationForQCL and after the reception of activation command of TCI states for UE specific PDSCH, the UE may assume that DMRS ports of PDSCH follows QCL parameters indicated by default TCI state(s) as following: - Use the TCI-states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states which are activated for PDSCH. - If all the TCI codepoints are mapped to a single TCI state, then Rel-15 behavior is followed The support of this feature is part of UE capability.. 16-3: UE omission should not be a UE capability. (Suggested as FFS) 16-4: We suggest to change “mandatory” to “TBD” because there is no RAN1 agreement that this feature should be mandatory. (Suggested as FFS) 16-5: Suggest to delete component 1: New UL codebook set(s), for the reason that there is no related agreement. (Suggested as FFS) 16-5: Considering the following agreement, suggest to revise component 2 as ‘maximum of SRS resources in a set for usage set to ‘codebook’’. (Suggested as FFS) Agreement(RAN1#98) - For 4 TX UEs, a maximum of 4 SRS resources are supported in Mode 2 for usage set to ‘codebook’ in a set - Depending on UE capability, either up to 2 or 4 SRS resources are supported - For 2 TX UEs, a maximum of 4 SRS resources are supported in Mode 2 for usage set to ‘codebook’ in a set - Depending on UE capability, either up to 2 or 4 SRS resources are supported - For mode 2 UEs, up to 2 different spatial relation info can be configured for all SRS resources with usage set to ‘codebook’ Note: it does not mean to support simultaneous transmission of multiple SRS resources usage is set to ‘codebook’. |

- NR_CLI_RIM

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17. NR_CLI_RIM | 17-1 | CLI-RSSI measurement | 1) Support CLI-RSSI measurement 2) Up to 64 of measurement resource for CLI-RSSI measurement |  |  | N/A |  | Per UE |  | Yes |  | UE operates CLI-RSSI measurement. | Optional with capability signalling |
|  | 17-2 | SRS-RSRP measurement | 1) Support SRS-RSRP measurement 2) Up to 32 of SRS resources to be monitored by a UE |  |  | N/A |  | Per UE |  | Yes |  | UE operates SRS-RSRP measurement. | Optional with capability signalling |
|  | 17-3 | Simultaneous reception of PDSCH and CLI-RSSI measurement resource | Support simultaneousRxPDSCHCLIRSSIMeasResource | 17-1 |  | N/A |  | Per UE |  | Yes |  | UE can assume that PDSCH is FDMed with CLI-RSSI measurement resource(s) | Optional with capability signalling |
|  | 17-4 | Simultaneous reception of PDSCH and SRS-RSRP measurement resource | Support simultaneousRxPDSCHSRSRSRPMeasResource | 17-2 |  | N/A |  | Per UE |  | Yes |  | UE can assume that PDSCH is FDMed with SRS RSRP measurement resource(s). | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | On 17-1, we suggest to split CLI-RSSI and SRS-RSRP as separate UE features. |
| Nokia | - 17-2, 17-3: actually this is about reception of PDSCH and measurement of CLI-RSSI. The definition of simultaneous will need to be clarified at some point, e.g. is TDM rate-matching assumed in any case with the measurement being performed? |
| Huawei, HiSilicon | 1. FR1/FR2 differentiation should be “Yes” in general for all CLI capabilities to align with the LS R2-1916348. Note that FG 17-1/2/3 on CLI are already reflected in endorsed CR R2-1915716. |
| Ericsson | 17-1 - RAN2 has already agreed to have separate capabilities for CLI RSSI and SRS RSRP (R2-19115716) with FR1/FR2 differentiation. 17-3 - This FG has already been agreed in RAN 2 (R2-19115716) with FR1/FR2 differentiation. |
| Intel | Suggest to split 17-1 into two features: CLI-RSSI measurement, SRS-RSRP measurement. UE can report its capability on supporting each of the features. |
| LG Electronics | To vivo, Huawei, HiSilicon, Ericsson, Intel Thanks for providing good comments and suggestions. - Following the comments, I modify CLI measurement into separate UE features for CLI-RSSI and SRS-RSRP measurement. - Following the agreements, I include the maximum number of resource for each UE features in the table. In addition, in order to reply the LS from RAN2 [R1-2000155], we may have additional technical discussion “whether the UE needs to be able to indicate the maximum number of CLI measurement resources it supports (separately for SRS-RSRP and CLI-RSSI) including feedback on whether to introduce/reuse any capability(ies)”. Depending on the result of discussion, we may include additional UE feature for the maximum number of CLI measurement resource. To Nokia, Thanks for providing good comments If my understanding is correct, a UE with simultaneous reception capability may operate FDM rate-matching or TDM rate-matching depending on gNB configuration for PDSCH rate matching. On the other hand, if UE dose not support simultaneous reception, UE can operate only TDM rate-matching and gNB should configure TDM rate-matching pattern to the UE. |

- MR-DC/CA enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18. MR-DC/CA enhancement | 18-1 | Basic UL power sharing for DC | Semi-static power sharing mode 2 between MCG and SCG cells of same FR for NR dual connectivity. |  | Yes | N/A | Intra-frequency range DC is not supported by the UE | Per band combination | No | Yes |  | Absence means intra-FR DC is not supported. | Optional with capability signalling |
|  | 18-1a | Semi-static UL power sharing mode Y for DC | Semi-static power sharing mode 1 between MCG and SCG cells of same FR for NR dual connectivity. | 18-1 |  |  |  |  | No | Yes |  |  | Optional with capability signalling |
|  | 18-1b | Dynamic UL power sharing for DC | Dynamic power sharing between MCG and SCG cells of same FR for NR dual connectivity. | 18-1 |  |  |  |  | No | Yes |  |  | Optional with capability signalling |
|  | 18-2 | Single UL TX operation for TDD PCell EN-DC | TDM restriction to LTE TDD PCell in EN-DC for single UL Tx operation 1) TDD UL/DL configuration#2, #4, #5 configured as DL-reference UL/DL configuration 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ subframe offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern | EN-DC |  | N/A |  | Per band combination | Applicable to TDD-TDD EN-DC only | Applicable to FR1 only |  | Extension of the R15 capability singleUL-Transmission to TDD PCell | Optional with capability signalling |
|  | 18-2a | Enhanced single UL TX operation for FDD PCell EN-DC | TDM restriction to LTE FDD PCell in EN-DC for single UL Tx operation 1) TDD UL/DL configuration#2, #4, #5 configured as DL-reference UL/DL configuration 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ sub-frame offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern | EN-DC |  | N/A |  | Per band combination | Applicable to in FDD-LTE TDD-NR EN-DC | Applicable to FR1 only |  | Enhancement to the R15 capability singleUL-Transmission. | Optional with capability signalling |
|  | 18-3 | Dual Tx transmission for EN-DC with FDD PCell(TDM pattern for dual Tx UE) | TDM restriction to LTE FDD PCell in EN-DC for dual UL Tx operation 1) TDD UL/DL configuration#2, #4, #5 configured as DL-reference UL/DL configuration 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ subframe offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern | EN-DC |  | N/A |  | Per band combination | Applicable in EN-DC with FDD-LTE only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to a 2Tx UE: Indicates that the UE supports restricting the FDD-LTE uplink transmission occasions with a TDM pattern. | Optional with capability signalling |
|  | 18-4 | SCell dormancy within active time | Support for SCell dormancy indication sent within the active time on PCell with DCI format 1_0/1_1 |  |  | N/A |  | Per UE | No | No |  | Support L1-based mechanism to indicate dormancy/non-dormancy status for group of Scells | Optional with capability signalling |
|  | 18-4a | SCell dormancy outside active time | Support for SCell dormancy indication sent outside the active time on PCell with DCI format 2_6 | [19-1] |  |  |  | Per UE | No | No |  | Support L1-based mechanism to indicate dormancy/non-dormancy status for group of Scells | Optional with capability signalling |
|  | 18-5 | Cross-carrier scheduling with different SCS | 1) The UE supports cross carrier scheduling for the different numerologies with carrier indicator field (CIF) in DL carrier aggregation where numerologies for the scheduling cell and scheduled cell are different [2) Processing up to X unicast DCI scheduling (DL and UL) per scheduled CC ] |  |  | N/A |  | Per band combination | No | No |  | crossCarrierScheduling-OtherSCS 1) {Scheduling cell of lower SCS and scheduled cell of higher SCS, Scheduling cell of higher SCS and scheduled cell of lower SCS, both} [2) X={TBD}] Note: this capability exists in Rel-15 38.306, but the functionality was not completed in Rel-15. | Optional with capability signalling |
|  | 18-6 | Cross-carrier A-CSI RS triggering with different SCS | Cross-carrier A-CSI RS triggering with different SCS |  |  | N/A |  | Per band combination | No | No |  | 1) {PDCCH of lower SCS and A-CSI RS cell of higher SCS, PDCCH cell of higher SCS and A-CSI-RS of lower SCS, both} . | Optional with capability signalling |
|  | 18-7 | CA with non-aligned frame boundaries | CA with non-aligned frame boundaries for inter-band CA |  |  | N/A |  | Per band combination | No | No |  | Defines whether the UE supports carrier aggregation operation where the frame boundaries of the PCell and the SCell are not aligned, while the slot boundaries are. | Optional with capability signalling |
|  | 18-8 | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group |  |  | N/A |  | Per UE | No | No |  | Support HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group. Rel-15 had this per cell group | [Optional with capability signalling or conditioned to support for multiple PUCCH groups] |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | On 18-4 component, we suggest the following revisions - SCell dormancy with L1 indication within DRX active time |
| Nokia | - 18-1: the note saying that “Absence means DC not supported” is too strong, as it is not taking into account that R15 already supports DC for FR1+FR2. - 18-4: Remove “with L1 indication” from the feature name as it creates confusion on what is actually being defined here. Further discuss if the capability should be defined “per UE” or “per band combination”. |
| Samsung | 18-4) This should be further broken down (either separate FG or separate component) into supporting this feature with (a) DCI format 2_6 (in conjunction with support of UE power saving – correspondingly to update FG 19-1 by removing existing component (4)), (b) DCI format 1_1 |
| NTT DOCOMO | FG18-5 can be deleted since there is FG6-10a (crossCarrierScheduling-OtherSCS) from Rel-15. |
| ZTE | We are fine with the general structure of R16 UE feature for MR-DC. Some minor updates from our perspective are as below. 1. For 18-3 (TDM restriction to an FD-LTE cell in EN-DC), the Note is not clear. We propose the following update Extension of the R15 capability tdm-Pattern to a 2Tx UE: Indicates that the UE supports restricting the FD-LTE uplink transmission occasions with a TDM pattern. 2. For 18-4 (SCell dormancy with L1 indication), the Type shall be updated from ‘Per band combination’ to ‘Per UE’. For the case when DCI format 1_1 is used only for dormancy indication without scheduling data, it can be used for indicating the dormancy of all the SCells. 18-4 is similar to R15 dynamicSFI to some extent, i.e., both of them are used to indicate information for one or more cells. In R15, dynamicSFI is a per-UE capability. From this perspective, it is more appropriate to make 18-4 a UE level capability. 3. The following agreements (irrelevant parts are omitted) agreed in RAN1#98 meeting has not been included. Thus, for both 18-2 and 18-3, “HARQ-offset” can be added in the “Components” column. - Support HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell - Note: from UE perspective, it is expected that HARQ-offset value doesn’t violate the DL/UL configuration (in SIB1). - For type 1 UE, the feature is optional. FFS for type 2 UE. |
| MediaTek | - For FG18-1, we propose that semi-static-mode1 as basic feature for Rel-16 MR-DC/CA and candidate values should be revised as {semi-static-mode2, dynamic without lookahead, dynamic with lookahead}. In note: UE supporting Rel-16 MR-DC/CA must support semi-static-mode1 and indicate whether it can further support other UL power sharing schemesat least 1 UL power sharing scheme. - For FG18-2 and FG18-3, suggest to revise the descriptions as: 18-2; TDM restriction to 1Tx operation for LTE PCell in TDD intra-band EN-DC for 1-Tx operation; 1) TDM restriction to 1Tx operation for LTE TDD PCell in TDD intra-band EN-DC for 1-Tx operation 2) TDM restriction to LTE FDD PCell in EN-DC for 1-Tx operation; Per band combination; Applicable to in intra.band TDD-TDD EN-DC/NE-DC UE only; Applicable to FR1 only; Extension of the R15 capability singleUL-Transmission to intra-band TDD for EN-DC LTE PCell. 18-3; TDM restriction to an FD-LTE Pcell in EN-DC for dual-Tx operation (TDM pattern for 2Tx UE); 1) TDM restriction to an FD-LTE TDD Pcell in intra-band EN-DC for dual-Tx operation 2) TDM restriction to LTE FDD PCell in EN-DC for dual-Tx operation; 18-2; Per band combination; Applicable in EN-DC/NE-DC with FD-LTE UE only; Applicable to FR1 only; Extension of the R15 capability tdm-Pattern to a 2Txdual-Tx UE: Indicates that the UE supports restricting the TD-LTE uplink transmission occasions with a TDM pattern.. - For FG18-2, there are new behaviors specified for both LTE TDD/FDD PCell, so the feature group is not limited to LTE TDD PCell. - For FG18-3, according to Rel-16 RAN1 agreement, for daul Tx UE, the used case is applied to both LTE TDD/FDD PCell - For FG18-4, i.e., SCell dormancy, according to RAN1 agreements, DCI format 2_6, i.e., wake-up signal, can be used to indicate SCell dormancy when UE is not in DRX Active Time. And non-fallback DCI formats, e.g., DCI format 0_1 and 1_1, can be used to indicate SCell dormancy when UE is in DRX Active Time. Therefore, it is more reasonable to split FG 18-4 into FG 18-4a and FG-18-4b for SCell dormancy indication outside and inside DRX Active Time, respectively. - FG18-4a: SCell dormancy with DCI format 2_6 indication - 1) UE SCell dormancy/non-dormancy indication via DCI format 2_6 - FG18-4b: SCell dormancy with DCI format 0_1 & 1_1 indication - 1) UE SCell dormancy/non-dormancy indication via DCI format 0_1/1_1 with PUSCH/PDSCH scheduling - 2) UE SCell dormancy/non-dormancy indication via DCI format 1_1 without PDSCH scheduling - For FG 18-6, similar UE capability report structure as FG 18-5 should be applied as in Rel-16 RAN1 agreement. - For FG 18-7, it should enable UE to report its supported minimum grid {0.5ms, 0.25ms, 0.125ms} for the slot offset, instead of whether it can support the feature or not. - We propose to add the following two new feature groups: 18-9; Semi-statically configured LTE UL transmissions in all UL subframes not limited to the DL-reference configuration; Support of semi-statically configured LTE UL transmissions in all UL subframes not limited to the DL-reference configuration; 18-2; Per band combination; Applicable in EN-DC UE only; Applicable to FR1 only; RAN1 to further discuss whether this is a separate feature from 18-2; Optional with capability signalling. 18-10; HARQ-offset for EN-DC UE configured with DL-reference configuration with LTE TDD PCell; Support of HARQ-offset for EN-DC UE configured with DL-reference configuration with LTE TDD PCell; 18-2; Per band combination; Applicable in EN-DC UE with TD-LTE PCell only; Applicable to FR1 only; For type 1 UE (capable of DPS), the feature is optional. FFS for type 2 UE (not capable of DPS) RAN1 to further discuss whether this is a separate feature from 18-2; Optional with capability signalling. - For FG 18-9, it is agreed in RAN1#99 to support semi-statically configured LTE UL transmissions in all UL subframes for SUO EN-DC subject to UE capability. - For FG 18-10, it is agreed in RAN1#98 to support HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell subject to UE capability. |
| Huawei, HiSilicon | - On FG 18-2 - It is better to change “1 Tx” to “Single UL Tx” to align with FG 8-2 in Rel-15. - “intra-band” should be removed in all the places for this FG 18-2. Firstly, it is not aligned with the scope of the WID nor the current spec. Secondly, it may result in that “Single UL operation for TDD inter-band ENDC” is mandatory for a UE supporting TDD inter-band EN-DC - “EN-DC” can be put for the column “Prerequisite feature groups” as in Rel-15. - “Yes” can be used for the column “Need of FR1/FR2 differentiation” following FG 6-13 and 8-2 in Rel-15. - The component for this FG 18-2 is too general, instead the following two items following the agreements can be used similar as FG 6-13 in Rel-15 - item 2) one DL-reference UL/DL configuration defined for LTE-TDD-PCell, Only LTE TDD Pattern 2, 4, 5 can be used as DL-reference. The UL/DL configuration indicated in SIB1 for LTE TDD PCell cannot be 0 or 6 - item 3) HARQ subframe offset (Optional feature with Rel-15 FG 8-1 as prerequisite feature) with value range {0, 1, 2, 5, 6} for the offset - On FG 18-3 - Seems some typo, e.g. “FD” should be “FDD” and “cell” should be “PCell” . - “2 Tx” should be “dual Tx” following the terminology in the WID. - The components for FG 18-3 should be modified as below following FG 6-13 and 8-2 in Rel-15. - “EN-DC” can be put for the column “Prerequisite feature groups” as in Rel-15. - The following 18-x should be added, which is extension of Rel-15 6-13 according to the agreements. - On FG 18-5 - According to the agreements, separate UE capabilities for lower SCS scheduling higher SCS and higher SCS scheduling lower SCS should be set. - crossCarrierScheduling-OtherSCS should indicate whether the UE supports cross carrier scheduling per SCS combination (Scheduling carrier SCS, Scheduled carrier SCS) for each band combination. For example, some UEs support 30kHz SCS PDCCH in band 1 scheduling 60kHz SCS PDSCH in band 2 but not support 15kHz SCS PDCCH in band 1 scheduling 60kHz SCS PDSCH in band 2 - According to the agreements in RAN1#97, it should add one more UE capability for using M DCIs in one slot for PDCCH to schedule M respective PDSCH (or PUSCH) transmissions in N slots for lower-SCS scheduling higher-SCS, where M<=N (for slot aggregation, M<N), once further agreement on which option to choose is achieved. - For FG 18-5, no need of FDD/TDD differentiation. - On FG 18-6 - According to the agreements in RAN1#98bis, separate UE capabilities for lower-SCS scheduling higher-SCS and higher SCS scheduling lower SCS should be set. - Similar to FG 18-5, it should indicate whether the UE supports triggering of aperiodic CSI-RS where the CSI-RS is on a carrier with one sub-carrier spacing and the triggering PDCCH is on another carrier per SCS combination (PDCCH SCS, A-CSI-RS SCS) for each band combination - On FG 18-7 - Based on the WID scope (RP-192336), this feature is restricted for inter-band CA. Thus, this should be clarified in Note column. - On FG 18-8 - For flexibility, it is more proper to have separate UE capabilities for HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration. - The type for this FG 18-8 should be “per UE”. |
| Ericsson | 18-1: - Propose to split this into three feature groups - 18-1: UL power sharing for NR-DC (mode2-semi-static) - Component description - Semi-static uplink power sharing using mode 2 between MCG and SCG cells of same FR when UE is configured with NR-DCUL - No Need of FR1/FR2 differentiation - Note can be removed after split - Mandatory if UE indicates support for NR-DC with cells of MCG and SCG in same FR - 18-1a : UL power sharing for NR-DC (mode1-semi-static) - Component description - Semi-static uplink power sharing using mode 1 (configured by NW using RRC parameter NR-DC-PC-mode) between MCG and SCG cells of same FR when UE is configured with NR-DC - FR1/FR2 differentiation - Note - Supported only for synchronous NR-DC case - Optional - 18-1b : UL power sharing for NR-DC (dynamic) - Component description - Dynamic power sharing between MCG and SCG cells of same FR when UE is configured with NR-DC FR1/FR2 differentiation - FR1/FR2 differentiation - Optional - No need of FDD/TDD differentiation for the three FGs 18-2: - Update the text for clarity and alignment with agreements - Name - 1Tx operation for EN-DC with TDD Pcell - Component description - 1Tx operation for TDD intra-band EN-DC with TDD Pcell for UEs configured with TDD Pcell and tdm-patternConfig-r16 - Remove note - Add new feature (18-2a) to indicate support for Rel16 single-Tx operation with FDD Pcell (i.e., support for the enhancements added in Rel16) - Name - Enhanced 1Tx operation for EN-DC with FDD Pcell - Component description - Enhanced behaviour agreed in Rel16 for UEs configured with FDD Pcell and tdm-patternConfig-r16 - Optional 18-4: - Split into two feature groups - 18-4: Scell Dormancy with L1 indication inside Active time - SCell dormancy with L1 indication sent in active time on Pcell - 18:4a: Scell Dormancy with L1 indication outside Active time - SCell dormancy with L1 indication sent using DCI 2-6 - Prerequisite feature groups: [19-1] /connected mode wakeup signal for DRX adaptation - Optional 18-5 - Propose to add another feature 18-5a : Cross-carrier scheduling with different SCS and increased number of DCIs per span for case of scheduling cell with lower SCS and scheduled cell of higher SCS - Component description - For scheduling cell of lower SCS and scheduled cell of higher SCS, for the set of monitoring occasions within a span of scheduling CC, - Processing up to X unicast DCI scheduling (DL and UL) per scheduled CC - where X is based on pair of (scheduling CC SCS, scheduled CC SCS): - X = 4 for (15 kHz, 120 kHz), (15 kHz, 60 kHz), (30 kHz, 120 kHz), - X = 2 for (15 kHz, 30 kHz), (30 kHz, 60 kHz), (60 kHz, 120 kHz), - Note : This applies also to the case where there is a single span in the slot for the scheduling CC. - In case UE supports 3-5b, the limits apply for each span for FDD scheduling cell and TDD scheduling cell. - Prerequisite feature groups: 18-5, indicating support of Scheduling cell of lower SCS and scheduled cell of higher SCS - Optional |
| Intel | 18-2: it needs further clarification whether 1TX operation for TDD EN-DC applies to both intra-band and inter-band Suggest adding a feature for the new behavior for 1 TX UE for FDD EN-DC, since the behavior is different from Rel-15. |
| Apple | - 18-2 1Tx operation for TDD intra-band EN-DC It’s not necessary to limit the feature to intra-band EN-DC, also the title can be updated as “1Tx operation for EN-DC with TDD PCell”. Components can include: TDD UL/DL configuration#2, #4, #5 configured as DL-reference UL/DL configuration PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) HARQ-offset LTE UL transmissions scheduled/triggered by DCI in any UL subframes with the TDM pattern configuration for type 1 UE - 18-x 1Tx operation for EN-DC with FDD PCell For TDM pattern configuration, the following agreement is made in RAN1#99 meeting, Agreements: Clarify that “tdm-PatternConfig-r16” as agreed in RAN1 #98bis is applied in both single Tx case (with LTE TDD PCell or LTE FDD PCell) and dual-Tx case. So, TDM pattern is applied to three use cases. New row is needed for sub-feature Rel.16 single UL FDD Pcell EN-DC, i.e., 1Tx operation for EN-DC with FDD PCell. The components include DL-reference UL/DL configuration is TDD UL/DL configuration#2, #4, #5 configured or same Rel.15 EN-DC with FDD PCell? PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) HARQ-offset LTE UL transmissions scheduled/triggered by DCI in any UL subframes with the TDM pattern configuration for type 1 UE, UL scheduling timing is not changed - 18-3 TDM restriction to an FD-LTE cell in EN-DC (TDM pattern for 2Tx UE) The title could be updated as dual Tx transmission for EN-DC with FDD PCell. Components include, DL-reference UL/DL configuration is TDD UL/DL configuration#2, #4, #5 configured or same Rel.15 EN-DC with FDD PCell? PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) HARQ-offset LTE UL transmissions scheduled/triggered by DCI in any UL subframes with the TDM pattern configuration for type 1 UE, UL scheduling timing is not changed - All UL subframes can be used if TDM pattern is configured According to RAN1#99 meeting discussion and agreements, Agreements: For a UE configured with DL-reference DL/UL configuration in Rel-16 (including single Tx with LTE TDD PCell or LTE FDD PCell, and dual Tx cases): - For type 2 UE (i.e., UE without dynamic power sharing capability), any LTE UL transmissions should take place only in UL subframes designated for HARQ-ACK feedback. - For type 1 UE (i.e., UE with dynamic power sharing capability), - Confirm that any LTE UL transmissions scheduled/triggered by DCI can take place in UL subframes not designated for HARQ-ACK feedback. - FFS UE is not expected to transmit semi-statically configured LTE UL transmissions in the UL subframes other than those designated as UL in the DL-reference configuration if such transmission collide with NR UL transmissions. Agreements For the FFS part in the agreement above, - semi-statically configured LTE UL transmissions are allowed in all UL subframes. - Note: In case of collision, LTE transmission is prioritized - Note: this configuration is subject to UE capability Thus, it’s the UE capability that semi-statically configured LTE UL transmissions are allowed in all UL subframes New row can be added like, Semi-statically configured LTE UL transmission in all UL subframes with the TDM pattern configuration for type 1 UE - NE-DC consideration For sub-feature 18-2, 18-3 and Rel1.6 FDD PCell EN-DC, these features can also apply to NE-DC case, any standard impacts can study further. |
| Spreadtrum Communications | - On FG 18-4, separate capability for Scell Dormancy inside of active time or outside of active time. And for inside of active time, separate capability for with PDSCH scheduling PDCCH or without PDSCH scheduling PDCCH - On FG 18-5, separate capability for higher SCS PDCCH schedules lower SCS PDSCH, or lower SCS PDCCH schedule higher SCS PDSCH - On FG 18-6, separate capability for higher SCS PDCCH triggers lower SCS A-CSI-RS, or lower SCS PDCCH triggers higher SCS A-CSI-RS. |

- UE Power Saving

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19.UE Power Saving | 19-1 | DRX Adaptation | - PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDuration timer at the DRX ON by DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected outside Active time - Configured P-CSI report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON |  |  |  |  | N/A |  | [Per UE] | No | No |  | Minimum time gap before the start of the DRX ONis determined based on UE capability | Optional with capability signalling |
|  | 19-1a | DRX Adaptation | - Indication of UE dormancy-like behaviour by DCI format 2_6 | 18-4 |  |  |  | N/A |  | [Per UE] | No | No |  |  | Optional with capability signalling |
|  | 19-2 | Cross Slot Scheduling | - Dynamic indication of minimum scheduling offset by DCI format 0_1 and 1_1 - o minimumSchedulingOffset configuration for PDSCH and aperiodic CSI-RS triggering offset - minimumSchedulingOffset configuration for PUSCH |  |  |  |  | N/A |  | [Per UE] | No | No |  |  | Optional with capability signalling |
|  | 19-3 | Maximum MIMO Layer Adaptation | - Maximum number of MIMO layer configuration per DL BWP |  |  |  |  | N/A |  | [Per BC | No | No |  | This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication | Optional with capability signalling |
|  | 19-4a | UE assistance information | - Preferred k0/k2 - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots |  |  |  |  | N/A |  | [Per UE] | No | No |  |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | For 19-1, we suggest the following revisions - Component (1) is revised to the following and the note can be deleted. - PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI before DRX ON duration of long DRX - The minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing - Component (2) is revised to: DCI format 2_6 - Component (4) should be a separate UE feature (e.g. 19-1a) as UE supporting WUS may not support Scell dormancy behavior. And the description of feature 19-1a should be revised to: Indication of UE Scell dormancy-like behaviour by DCI format 2_6 outside active time - Adding component (5): UE wakeup or not by configuration when DCI format 2_6 outside active time is not detected - Adding component (6): UE CSI/L1-RSRP/SRS behavior impacted by DCI format 2_6 For 19-3, we suggest the following revision - Component 1) is revised to : Maximum number of MIMO layer configuration per DL BWP |
| OPPO | - For 19-4, it shall also include preferred k0/k2. - For 19-3, since in different frequency band, the supported Maximum MIMO Layer may be different, it shall be a per-band feature type. Why DRX adaptation is a Mandatory without capability signaling, it shall be optional as other features. |
| Nokia | - 19-1, component 1: Replace PS_offset with “minimum gap” - 19-1, component 2: DCI format 2_6 instead of 3_0 - 19-1, component 4: dormancy-like behavior should be a separate feature dependent on 19-1 and 18-4. - 19-1: add two components: 5) ‘PS_Periodic_L1-RSRP_TransmitOrNot’ for L1-RSRP reporting when onDurationTimer is not started; 6) ‘PS-Periodic_CSI_TransmitOrNot’ for CSI reporting when OnDurationTimer is not started - 19-3: it is unclear if there is a need to separately indicate this capability as there is no change in UE behavior from gNB point of view. |
| Samsung | 19-1) Suggest to set as ‘optional with capability signaling’ 19-1) Update ‘DCI format 3’ to ‘DCI format 2_6’ 19-1) Remove component (4) and update FG 18-4 in MR-DC/CA. Component (4) would be independent functionality from DRX adaptation |
| NTT DOCOMO | We suggest to modify as following: 19-1: (1) PS_offset for the detection of DCI format 32_6 with CRC scrambling by PS-RNTI before DRX ON duration (2) DCI format 3_02_6 (3) Indication of UE wakeup or not by DCI format 2_6 (4) Indication of UE dormancy-like behavior by DCI format 2_6 Note: PS_offset is determined based on UE capabilityThe minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing |
| ZTE | Regarding 19-1 (1) According to the Agreements in RAN1#99, ‘the minimum time gap ’ between the end of WUS PDCCH and the start of DRX-on is a UE capability, not ‘PS_offset ’. Agreements:(RAN1#99) The minimum time gap between the end of the slot of last DCI format 3_0 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to [3]ms - FFS impact of dormancy/non-dormancy transition (2) The L1-based Scell dormancy indication is optional in MR-DC. It should be the same for the dormancy indication by DCI format 2_6.Therefore, we suggest the 19-1, DRX Adaptation feature, is “Optional with capability signalling”, not “mandatory without capability signalling”. Regarding 19-4 and 19-5 In general, the 19-4 and 19-5 are RAN2 related feature, it is more proper to be discussed by RAN2. In addition, minimum K0/K2 is missed in 19-4. Agreements:RAN2#99 Minimum K0/K2 value is signaled as UE assistance. Value of infinity can be configured for the prohibit timer. |
| MediaTek | - For FG 19-1, we propose the following revision: - Item #1: PS_offset for the detection of DCI format 32_6 with CRC scrambling by PS-RNTI on PCell with CA or SpCell with DC before DRX ON duration of next long DRX cycle - DCI format 2_6 is used for wakeup indication in specifications, therefore, DCI format 3 should be revised to DCI format 2_6. - DCI format 2_6 can only be configured in primary cells, i.e., PCell with CA or SpCell with DC. - DCI format 2_6 is only applicable to the next long DRX cycle. - Item #2: Monitoring DCI format 32_6 - Item #4: Indication of UE dormancy-like behavior - We suggest to delete this component because SCell dormancy indication exists in DCI format 2_6 only when SCell dormancy is supported. This component is the same as in FG 18-4a. - Add new item “UE reports minimum time gap per SCS” - In RAN1 #99, it was agreed “UE capability report on the minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON”. This should be included in FG 19-1. - For FG 19-2, we propose the following revision: - Item #1: Minimum applicable scheduling offset indication in Enhancement of DCI format 0_1 and 1_1 - Item #2: DCI Indication of minimum scheduling offset - There is no need to have separate components for DCI supporting cross-slot scheduling adaptation. - For FG 19-3, we propose the following revision: - Item #1: Support of mMaximum MIMO layer configuration per DL BWP - For FG 19-4 and 19-5: Add “Details are up to RAN2” in the note column. - These two features should be RAN2 features |
| LG | 19-1 (1) and “Note”: In our understanding, a UE monitors DCI format 2_6 on a first full duration within a monitoring window. The monitoring window is defined by “PS_offset” and “time gap”. (The PS_offset and time gap indicate starting and ending point of the monitoring window, respectively.) In addition, the “PS_offset” is configured by a gNB, and the “time gap” is a UE capability based on subcarrier spacing. The related agreement is as in agreement below. So, “time gap” should be included in 19-1 with a note of capability signaling or “time gap” should be added as an another FG, which should be mandatory signaling for DRX adaptation Agreements: (RAN1#99) The minimum time gap between the end of the slot of last DCI format 3_0 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to [3]ms - FFS impact of dormancy/non-dormancy transition 19-1 (2): According to TS28.212, DCI format 3_0 is used for sidelink (for scheduling of NR PSCCH and NR PSSCH). The DCI format for power saving is 2_6. So, in (2), editorial correction is needed as “(2) DCI format 3_02_6” 19-4: RAN1 hasn’t agreed any of those UE assistance information but RAN2 has agreed some UE assistance information (not sure about all of them). Therefore, it may be better to discuss this FG in RAN2. |
| Huawei, HiSilicon | - On Feature group 19-1: - The name of the DCI format was renamed to DCI format 2_6 during the CR draft/review. So, the name of DCI format in UE feature list should be updated to DCI format 2_6 in component (1) and (2). - There is another feature group 18-4 to define the UE capability to support the L1 indicated dormancy adaptation. The component (4) overlaps with 18-4. Component (4) should be deleted in 19-1. - As agreed in the agreement “Agreements: The minimum time gap between the end of the slot of last DCI format 3_0 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing.”, the minimum gap needs to be reported as UE capability. Therefore, another component should be added: Report of the minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON. - It is clearer to change the note “PS_offset is determined based on UE capability” to “PS_offset is determined based on UE capability on the minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON”. - This FG 19-1 should be “Optional with capability signalling” also. - On FG 19-2: Cross slot scheduling is actually supported in Rel-15. The feature group 19-2 actually refers to the feature to support the DCI triggered adaptation on minimum K0/K2. To avoid the confusion, it is suggested to change the name of the feature group to ‘dynamic adaptation on minimum scheduling offset’. The current name is misleading. - On FG 19-3: The UE capability of supported maximum MIMO layers for PDSCH has been defined in Rel-15. The feature group 19-3 is not defining the supported maximum MIMO layers per DL BWP. It should be the capability to support the configuration of maximum MIMO layers per BWP. Therefore, it is suggested to refine the description of the component to ‘support of per-BWP configured maximum MIMO layers for a DL BWP’. |
| Panasonic | - Maximum MIMO layer adaptation should be per BC. - UE assistance information should be up to RAN2 discussion. |
| Ericsson | 19-1: - Change name to “Connected mode wakeup signal for DRX Adaptation” - Components should be - DCI format 2-6 with CRC scrambled by PS-RNTI before DRX ON duration - Indication of UE wakeup or not - Indication of whether UE report L1-RSRP when drx_OnDurationTimer does not start at the DRX ON - Indication of whether UE report Periodic CSI report when drx_OnDurationTimer does not start at the DRX ON - Minimum gap between WUS end and On duration - Note - For minimum time gap, UE reports one value per SCS out of two values (TBD) per SCS - Optional 19-2: - Update the name to “Minimum scheduling offset for Cross Slot Scheduling” to avoid confusion with Rel-15 cross-slot scheduling and components - Components should be - Minimum applicable scheduling offset indicator in DCI format 0_1 and 1_1 - minimumSchedulingOffset configuration for PDSCH and aperiodic CSI-RS triggering offset - minimumSchedulingOffset configuration for PUSCH 19-3: - Change name to “Per-BWP Maximum MIMO Layer Adaptation for DL” - Modify component to “1) Maximum MIMO layer configuration per DL BWP” - Add note: - This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication 19-4: - This is a RAN2 feature and can be deleted - Propose to add a new feature group 19-4a for UE suggested minimum K0/K2 values - Name - UE assistance information for minimum scheduling offset for cross-slot scheduling - Component description - UE suggestion of values for minimum scheduling offset for DL and for UL - Note - UE suggested value for K0 and suggested value for K2 from the following range. - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots - Optional 19-5: - This is a RAN2 feature and can be deleted |
| Intel | 19-1 : DRX adaptation should be optional with capability signaling Component (4) should be a separate UE feature 19-3: Maximum number of MIMO layers per DL BWP |
| Apple | In 19-1, change DCI format 3 to DCI format 2_6 to make it consistent with 38.212, 38.213, 38.214. In 19-1 (3), it is better to use terminology in line with spec: change “Indication of UE wakeup or not” to “Indication whether to start drx-OnDurationTimer or not for next DRX On duration" In 19-1 (4), the indication of UE dormancy like behavior is for different feature. Although the indication could be carried in the same DCI 2_6, they are different feature. Thus, it should be moved to 18-4 SCell dormancy with L1 indication. In 19-2 (1), the description is not clear what it exactly means. It should be rather the “support of minimum applicable value of K0/K2 in TDRA table”. In 19-2 (2), It is better to be more descriptive like “L1 indication for minimum applicable value for K0/K2 in DCI0_1 and 1_1”. In 19-4 (5), change “Release Request” to “Preferred RRC state” In 19-4, add “(6) Preferred minimum applicable value for K0/K2 |
| Spreadtrum Communications | 19-1: Components - Monitoring Type3-PDCCH CSS, including PS_offset before DRX ON duration of long DRX cycle, and monitoring on a first full duration - Acquire wakeup indication in DCI format 2-6 - (move to 2) - (consider in MR-CA/DC) - The minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing - UE wakeup or not by configuration when DCI format 2_6 outside active time is not detected - UE CSI/L1-RSRP/SRS behavior impacted by DCI format 2_6 - Fall back to R15 DRX behavior in the next DRX ON, if Type3-PDCCH monitoring occasion is not valid 19-2： Feature group: Restriction on Minimum Scheduling Offset Components: (1) Enhancement of DCI format 0_1 and 1_1 (2) DCI Indication of minimum scheduling offset 19-3: Components: 1) Support of Maximum number of MIMO layer configuration per DL BWP 19-4: - This is RAN2 related feature, and thus it is more proper to be discussed by RAN2. In addition, preferred minimum K0/K2 should be added to 19-4. 19-5: This is RAN2 related feature and thus it is more proper to be discussed by RAN2. |

- NR_IAB

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20. NR_IAB | 20-1 | Inter-IAB-node discovery and measurements: SSB transmission Configuration | Support up to 4 STCs configured for an IAB node DU per cell per frequency location, including IAB-specific SSB transmission periodicities |  |  |  | Yes | N/A | Separate configuration of SSB transmissions for access UE and IAB node measurements is not possible | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |
|  | 20-2 | Inter-IAB-node discovery and measurements: SSB reception configuration | Support up to 4 SMTCs configured for an IAB node MT per frequency location, including IAB-specific SMTC window periodicities |  |  |  | Yes | N/A | Separate configuration of SMTC windows for Inter-IAB node discovery and measurement is not possible | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |
|  | 20-3 | Extension of RACH occasions and periodicities for backhaul RACH resources | Support RACH configuration separately from the RACH configuration for UE access, including new IAB-specific offset and scaling factors |  |  |  | Yes | N/A | Separate configuration of RACH transmissions/receptions for access UEs and IAB nodes is not possible | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |
|  | 20-4 | IAB node non-TDM multiplexing capability | TDM not required between IAB-MT and IAB-DU functions |  |  |  | Yes | N/A | Only TDM-based resource configurations can be supported | [per {MT CC, DU cell} pair] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling |
|  | 20-5 | UL-Flexible-DL slot formats | Support semi-static and dynamic configuration/indication of UL-Flexible-DL slot formats for IAB-DU and IAB-MT resources |  |  |  | Yes | N/A | Only Rel-15 slot formats can be configured for backhaul links | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling |
|  | 20-6 | Dynamic indication of soft resource availability | Support DCI Format [4]_0 based indication of soft resource availability to an IAB node |  |  |  | Yes | N/A | Explicit indication of soft resource availability is not supported | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |
|  | 20-7 | Case 1 OTA timing alignment | Support IAB-DU DL timing alignment with parent node based on NTA+NTA,offset⋅Tc/2+Tdelta |  |  |  | Yes | N/A | Case-1 OTA timing alignment is not supported | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | Regarding 20-4, we suggest to further distinguish non-TDM multiplexing schemes by separate capabilities, i.e., capability for MT-TX/DU-TX; MT-TX/DU-RX; MT-RX/DU-TX; MT-RX/DU-RX. Which reflects the agreement made at the RAN1#99 meeting as following. Agreements: The indication of the multiplexing capability for the case of no-TDM between IAB MT and IAB DU is additionally provided with respect to each transmission-direction combination (per MT CC/DU cell pair) - MT-TX/DU-TX - MT-TX/DU-RX - MT-RX/DU-TX - MT-RX/DU-RX |
| Nokia | In general, capabilities related to DU operation do not belong in this table or in RAN1 specs. It is questionable whether the MT-related features need to be captured here either, so further discussion is needed, possibly including RAN2 and RAN3. |
| NTT DOCOMO | For FG20-4, since it is per {MT CC, DU cell} pair (i.e., similar to band combination), FDD/TDD differentiation and FR1/FR2 differentiation are not necessary. |
| ZTE | Add one capability, i.e., child IAB node can be made aware of the number of guard symbols that the parent IAB node will provide. [per IAB node],Optional with capability signalling. Add one capability, i.e., parent IAB node can be made aware of the number of symbols that the child IAB node would like the parent IAB node not to use. [per IAB node],Optional with capability signalling. For feature UL-Flexible-DL slot formats (20-5), it is better to list MT and DU capability separately. For feature UL-Flexible-DL slot formats (20-5), it is better to list the dynamic indication separately from semi-static configuration. Note that RAN1 agreed dynamic indication of U-F-D format for IAB-MT only (ref. RAN1 #98bis). There seems no agreement to include IAB –DU as well. The agreement made in RAN1 #98 to support U-F-D format in both DU and MT is just for RRC signaling with details up to RAN2, which may also mean the feature capability for the semi-static configuration should be decided in RAN2. At last, we are not sure whether the IAB DU FG should be considered in the “UE feature list” and included in the spec targeting UE features. But if yes, every instance of “IAB-DU” is better to be “IAB-DU/donor-DU”. |
| LGE | 20-5: Dynamic configuration/indication of UL-Flexible-DL slot formats is applied only for IAB MT. Thus, it should be changed accordingly. 20-6: For AI indication, DCI format 2_5 is used in the specification. Thus, it should be better to change ‘4_0’ to ‘2_5’. 20-7: Based on the agreement in NR Ad-hoc 1901 captured below, it seem case-1 OTA timing is mandatory feature. Agreements: An IAB node should set its DL TX timing ahead of its DL Rx timing by TA/2 + T_delta - T_delta is signalled from the parent node, where the value is intended to account for factors such the offset between parent DL Tx and UL Rx, if any due to factors such as Tx to Rx switching time, HW impairments, etc. - TA is the timing gap between UL Tx timing and DL Rx timing, which is derived based on existing Rel-15 mechanism - FFS (not necessarily an exhaustive list): - value range and granularity of Tdelta - need for aperiodic/periodic updates of Tdelta - other timing impairment factors for adjusting IAB node timing to be included in Tdelta - timing alignment when the IAB node has multiple parents Note: once the design of the above FFS points is in a good shape, an LS to RAN4 may be necessary to solicit their input |
| Huawei, HiSilicon | - It seems there is no basic features for IAB from the above feature groups, it is probably ok. But we hope it is common understanding that an IAB node may need some basic functionalities to support backhaul link operation. Features which are IAB-node specific but not listed above may be considered as mandatory, such as single STC for IAB-node DU and 2 SMTC for IAB-node MT for inter-IAB-node discovery and measurement, Semi-static H/S/NA resource configuration for IAB-node DU, Initial cell selection, an IAB-node MT may assume 160ms SS/PBCH periodicity, etc. - For FG 20-1: To support inter-IAB-node discovery and measurement, single STC can be the baseline for IAB node DU hence we proposed to add a note “For device supports IAB backhaul, it must support single STC” to the column of Note. - For FG 20-2: To support inter-IAB-node discovery and measurement, 2 SMTC similar to a access UE can be the baseline for IAB node MT hence we proposed to add a note “For device supports IAB backhaul, it must support 2 SMTCs” to the column of Note. - For FG 20-4: the description of the feature should be updated according to the agreement in RAN1 #99: Multiplexing capability between IAB MT and IAB DU with respect to each transmission-direction combination: {MT-TX/DU-TX, MT-TX/DU-RX, MT-RX/DU-TX, MT-RX/DU-RX}. - For FG 20-6: DCI format [4_0 should be changed to DCI format 2_5 to align with 38.212. - Regarding whether the FG is Mandatory or Optional, feature FG 20-1/2/3/6/7 should be “Optional with capability signaling”. The sentence “For device supports IAB backhaul, it must report this FG is supported” can be removed, some detailed analysis are listed below - FG 20-1/2: An IAB node does not need to support multiple STC/SMTC for discovery and measurement, .e.g. CSI-RS based measurement can be used. - FG 20-3: The support of separate RACH configurations from the configuration for access UEs does not need to be mandatory for IAB node since the IAB node MT can still access the network via the RACH configurations for the access UE. - FG 20-6: Semi-static resource configuration is the baseline for IAB node and the support of the dynamic indication of soft resource availability can be viewed as an performance enhancement. - FG 20-7: An IAB node DU does not need to rely on Case 1 OTA timing to set its DL Tx timing, e.g. GNSS can be used. |
| Ericsson | 20-1: - This is not an MT related feature and should be removed. 20-2: - We agree with the proposed “Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported”. 20-3: - This FG should be mandatory for the MT, but no capability signalling is needed since the feature is required supported when RA is performed. 20-4: - This is an IAB feature, not necessarily an MT feature. According to R1-1911724, the signalling related to this FG will be F1AP signalling between CU and IAB-node or Donor DU. We understand the feature is actually the signalling ability related to “IAB node non-TDM multiplexing capability”. This signalling ability should be mandatory. 20-5: - We agree with the proposed “Optional with capability signalling”. 20-6: - The FG should be “Optional with capability signalling.”. It should not be mandatory for IAB-nodes (IAB-MT); it is a performance related feature that reflects implementation related options. 20-7: - We agree with the proposed “Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported”. Additionally, there are some UE features that are mandatory in Rel-15, but do not need to be supported by Rel-16 IAB-nodes. These needs further discussions. |
| AT&T | 20-5: We think this is most useful when supported by both the DU and MT and should be a single feature. However, the description should be updated as noted by other companies: “Support semi-static configuration/indication of UL-Flexible-DL slot formats for IAB-DU and IAB-MT resources and dynamic configuration/indication of UL-Flexible-DL slot formats for IAB-MT resources” Although typically there are no DU-related features, IAB is a new feature and DU behaviour is captured in RAN1 specifications (i.e. 38.213). Also, these features are important for enabling inter-operability testing of IAB nodes. As a result, we do not agree with removing DU-related features from the RAN1 list. We do agree that how the signalling is designed can be left to RAN2/3. Related to the comments from Huawei and Ericsson, there does seem to be a need to define basic features for IAB nodes. Taking the Rel-15 UE features as a starting point is reasonable and further discussion is needed on whether those features are now mandatory or optional for IAB node MTs. |

- Mobility Enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21. Mobility Enhancement | 21-1 | DAPS HO | RAN1 assumes RAN2 will define a basic DAPS HO capability for a given [band and band combination] 1) RAN1 assumes that this Indicates support of intra-frequency and/or inter-frequency DAPS-HO for a given [band and band combination]. 2) Indicates support of simultaneously DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO. 3) Indicates support of PDCCH blind decoding capability in the first MCG and second MCGz. Note 1: RAN1 is assuming RAN2 will define separate UE capabilities on the support of - simultaneous uplink transmission - different TAG - different SCS} to/for source and target cells - support sync/async} cases as indicated in previous LS to RAN2 (R1-1913395). Therefore above are not captured in RAN1 feature list. Note 2: It should be understood that 21-1 could be merged with RAN2’s basic feature set description for DAPS HO. |  |  |  | Yes | N/A | The network cannot configure UE with DAPS HO for intra-frequency HO scenarios | [Per BC] (Note: requires RAN2 confirmation) | No | Yes | N/A |  | [Optional with capability signalling] |
|  | 21-2 | UE power sharing for DAPS HO | Indicates support of dynamic UL power sharing during DAPS-HO operation. |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The UE is only able to perform semi-static power allocation for source and target cell , or to drop the transmission to the source. | [Per BC] | No | Yes | N/A |  | Optional with capability signalling |
|  | 21-3 | PDCCH blind detection for MCG1 | Indicates PDCCH blind decoding capability supported in the first MCG (intended for target cell in HO) when in DAPS-HO. Value range should be {[2], …, 16}. |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The network may not able to configure UE with DAPS HO, as parameters required for PDCCH monitoring is essential for defining UE behaviour | [Per BC] | No | Yes | N/A | This capability (although optional) parameter value must be conveyed to the network along with 21-4 in order to correctly define UE behaviour | Optional with capability signalling {values: 2, 3, … 16} |
|  | 21-4 | PDCCH blind detection for MCG2 | Indicates PDCCH blind decoding capability supported in the second MCG (intended for source cell in HO) when in DAPS-HO. Value range should be {[2], …, 16}. |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The network may not able to configure UE with DAPS HO, as parameters required for PDCCH monitoring is essential for defining UE behaviour | [Per BC] | No | Yes | N/A | This capability (although optional) parameter value must be conveyed to the network along with 21-3 in order to correctly define UE behaviour | Optional with capability signalling {values: 2, 3, … 16} |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | - The features should be reorganized to give a better idea of what is supported by a UE supporting intra-frequency DAPS-HO and inter-frequency DAPS-HO. One possibility is to have one capability for “intra-frequency DAPS HO” including all 21-1 to 21-4 as basic components. Then we can have a capability on inter-frequency support, which would also include a limitation on the number of frequencies supported. |
| Samsung | 21-2) Suggest to set as ‘optional with capability signaling’. Current spec (38.213 section 15) allows UE not to provide ‘UplinkPowerSharingDAPS-HO’. New) As per RAN1 reply LS in R1-1913581, the following is assumed for UE capability for DAPS HO in addition to above FGs. It should be included in the UE capability for DAPS HO (either RAN1 or RAN2 list): - sync/async DAPS HO; - support for multiple TAG (i.e. different TAG in source and target cells) - ability to simultaneously transmit with both source and target PCells based on RF chain capability (i.e. whether dual UL is supported during the DAPS HO or not); - Handover involving different SCS (as agreed in RAN4) |
| ZTE | Comment 1 Change the basic feature group to include both intra-f and inter-f DAPS HO, and also combine the capability for PDCCH blind detection, i.e., 21-3 and 21-4 into the basic feature group. Fundamentally, all these features are essential for support of DAPS. That is, 21-1; Intra-frequency Support ofDAPS HO; 1) Indicates support of intra-frequency DAPS-HO for a given [band and band combination]. 2) Indicates support of inter-frequency DAPS-HO for a given [band and band combination]. 3) Indicates PDCCH blind decoding capability supported in the first MCG (intended for target cell in HO) when in DAPS-HO. Value range should be {[2], …, 16}. 4) Indicates PDCCH blind decoding capability supported in the second MCG (intended for source cell in HO) when in DAPS-HO. Value range should be {[2], …, 16}.. Comment 2 For 21-2, if UE doesn’t support UE power sharing for DAPS HO, it means that UE doesn’t support simultaneous transmission between source cell and target cell, i.e., in case of collision, transmission in source cell is dropped. Thus, we propose to update the “Consequence if the feature is not supported by the UE” of 21-2 as below. The UE doesn’t support simultaneous transmission between source cell and target cell, i.e., in case of collision, transmission in source cell is dropped. |
| Huawei, HiSilicon | - On FG 21-1: As a friendly reminder, we suggest adding a note to FG 21-1 saying that RAN1 is assuming RAN2 will define separate UE capabilities on the support of {simultaneous uplink transmission, different TAG, different SCS} to/for source and target cells, as well as the support of {intra/inter-band, intra/inter-frequency, sync/async} cases, as indicated in previous LS to RAN2 (R1-1913395). - In RAN1#99, it was agreed that without additional capability signaling, support of DAPS HO implies support of simultaneous reception of DL signals/channels in overlapping OFDM symbols. We suggest capturing this implied capability in UE feature list and then TS38.306. For example, it can be captured in the column of ‘Note’ at least for FG 21-1, probably also other feature groups added based on the first comment. Agreement: - In RAN1 understanding, if UE indicates that it supports DAPS HO for a specific feature set, it implies (without additional capability signaling) that UE supports simultaneous reception of DL signals/channels in overlapping OFDM symbols for DAPS HO. - Additional restrictions may apply, e.g. BWP, BD/CCE limit for PDCCH monitoring - Additional capability signaling is required to indicate the intra-frequency DAPS HO for a specific feature set - FFS: Whether indicated capabilities are per FS or per FSPC - Note: as per RAN2 LS R1-1911924, UE may additionally indicate that UE supports simultaneous transmission of UL signals/channels in overlapping OFDM symbols. - FFS: Whether, a UE that indicates DAPS-HO capability is capable of simultaneously receiving PDCCH/PDSCH from source and target cells when the resources for PDCCH/PDSCH from the two cells overlap in frequency and time. - Regarding mandatory/optional, as DAPS HO is designed to meet IMT2020 requirement of 0ms data interruption during handover, it would be beneficial to mandate UE to support intra-frequency synchronous DAPS HO for selected scenarios (e.g., bands where 4 Rx are mandatory). To this end, a note can be added to feature 21-1 that ‘UE shall report supporting this feature for band n7/n38/n41/n77/n78/n79’. Note that we are proposing this for intra-frequency synchronous cases only. - On FG21-2: The current formation of UE feature 21-2 is a bit unclear, e.g., multiple options listed in feature description but no candidate value is given for reporting. The statement in ‘consequence if not supported’ implies all the UEs should support semi-static power sharing, while some UEs may additionally support dynamic power sharing, aligned with the agreements from RAN1#99. With this understanding, we suggest to modify/clarify feature description as ‘Indicates support of dynamic UL power sharing during DAPS-HO operation, i.e. semi-static or dynamic.’ and mark it as ‘optional with capability signalling’. |
| Ericsson | 21-2 - Consequence if the feature is not supported by the UE - Add “, or to drop the transmission to the source” to the end of the sentence since according to RAN1 agreement the UE should be able to always drop the transmission to the source, - Should be optional since the case where the UE does not provide the capability is already described in the specification. |
| Apple | One left issue in RAN1#99 meeting is showing below, - FFS: Whether, a UE that indicates DAPS-HO capability is capable of simultaneously receiving PDCCH/PDSCH from source and target cells when the resources for PDCCH/PDSCH from the two cells overlap in frequency and time. Similar to UE capability 21-1, UE needs an additional capability to indicate simultaneous DL reception from both source cell and target cell if the PDCCH/PDSCH overlaps in time and frequency, such as for inter-frequency intra-band HO. |
| Rapporteur (intel) | # 21-1 changes: change the description to a basic feature set for DAPS HO. It should be noted that RAN2 might already define such basic feature set, in such case it should be understood that 21-1 would be merged with RAN2’s basic feature set description for DAPS HO. Incorporated comments from Huawei, Nokia, Samsung, and ZTE to better describe the basic feature set for DAPS HO, which includes simultaneous DL reception, and PDCCH monitoring capability for fist and second MCG. # It should be noted that Huawei does suggest to make the feature mandatory for select bands. Currently FL does not know a good method to capture this in the RAN1 feature list as band dependent mandatory/optional signaling category does not exist. FL has highlighted this in yellow to mark for further discussions. # 21-2 changes: added or to drop the transmission to source as commented by Ericsson and Huawei. Changed to optional with capability signaling as commented by Samsung. ZTE’s comments to change the consequence to UE does not support simultaneous UL transmission may not be entirely correct. 21-2 was intended to describe the semi-static vs dynamic and not between whether power sharing is feasible or not feasible. The underlying assumption for 21-2 is that power sharing (semi-statically or dynamically) is possible. |

- Potential change/update on existing UE features for Rel-16 UE

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8. UL TPC | 8-1 | Dynamic power sharing for LTE-NR DC | When total transmission power exceeds Pcmax, UE scales NR transmission power. |  | EN-DC |  | No | N/A |  | Per UE | N/A | N/A |  |  | Mandatory with capability signalling set to 1 |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Sprint | DPS is supposed to be mandatory. We agreed to have capability signaling for Rel-15 devices due to implementation issues that some vendors had, however now we would like to see 'Mandatory with capability bit set to 1' for subfeature 8.1 in Rel-16. 8. UL TPC; 8-1; Dynamic power sharing for LTE-NR DC; When total transmission power exceeds Pcmax, UE scales NR transmission power.; EN-DC; No; N/A; Per UE; N/A; N/A; Mandatory with capability signalling set to 1. |
| Nokia | We fully support the proposal from Sprint. Indeed it has been the understanding from RAN Plenary discussions in Rel-15 time frame that this capability would become truly mandatory for Rel-16 UEs. |
| ZTE | Comments on 14-4: Between Opt.1 (introduce new capability signaling) and Opt.2 (update 2-51), we prefer Opt. 2, since it can avoid the confusion of joint interpretation on two features caused by Opt. 1. But for Opt.2, rather than having 3 candidate values, we think a simpler solution is to change the candidate value “1” to “2”. That is, the candidate values of maxBurstLength in 2-51 are updated to “2” and “1 and 2”, and clarify that “1 and 2” is mandatory for FR2. Other comments in 16-3 The following agreement is made on RAN1#99, which requires update of a Rel-15 UE feature. We think it can also be treated here. Agreement In Rel-16, for CSI report other than beam report:  Increase the maximum number of configured aperiodic CSI Report Settings a UE can report per BWP (RRC parameter maxNumberAperiodicCSI-PerBWP-ForCSI-Report) up to 8, subject to UE capability o The maximum number of configured aperiodic CSI Report Settings a UE supports is part of UE capability  Note: This increases the value range of the associated UE capability signaling from 1...4 to 1...8  Note: FFS for FR2 whether UE can report a value larger than 4 |
| Huawei, HiSilicon | - Fine with Sprint’s proposal. - Support of simultaneous CBG operations and multiple PDSCHs per slot - There is conclusion in Rel-15 that the scenario described in the draft CR (R1-1907505) can happen but can be handled via implementation in which case the UE performance may not be optimal. To ensure the NR data rate performance while maximizing the respective benefits of CBG based operations and multiple PDSCHs per slot, an explicit UE capability of support simultaneous operation is proposed - It should be, if supported, an optional feature with UE capability signalling, and with need of FRX differentiation while without need of XDD differentiation |
| Ericsson | We are fine with the Sprint proposal. |
| AT&T | DPS capability should be mandated as 1 |

Appendix: Layer-1 UE feature list in TR38.822 v1.1.0 [9]

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0. Waveform, modulation, subcarrier spacings, and CP | 0-1 | CP-OFDM waveform for DL and UL | 1) CP-OFDM for DL 2) CP -OFDM for UL |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 0-2 | DFT-S-OFDM waveform for UL | Transform precoding for single-layer PUSCH |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 0-3 | DL modulation scheme | 1) QPSK modulation 2) 16QAM modulation 3) 64QAM modulation for FR1 |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 0-4 | UL modulation scheme | 1) QPSK modulation 2) 16QAM modulation |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 0-5 | Extended CP | Extended CP | 1-1 in Table 4.3-1 | extendedCP | BandNR | n/a | n/a |  | Optional with capability signalling |
| 1. Initial access and mobility | 1-1 | Basic initial access channels and procedures | 1) RACH preamble format 2) SS block based RRM measurement 3) Broadcast SIB reception including RMSI/OSI and paging |  | n/a | n/a | No | No | Broadcast SIB reception including RMSI/OSI and paging are components of basic initial access channels and procedures for NR standalone and NE-DC | Mandatory without capability signalling |
|  | 1-2 | SS block based SINR measurement (SS-SINR) | SS-SINR measurement | 1-1 | ss-SINR-Meas | MeasAndMobParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 1-3 | SS block based RLM | SS block based RLM | 1-1 | ssb-RLM | MeasAndMobParametersCommon | No | No |  | Mandatory with capability signalling which shall be set to ‘1’ |
|  | 1-4 | CSI-RS based RRM measurement with associated SS-block | 1) CSI-RSRP measurement 2) CSI-RSRQ measurement | 1-1, CSI-RS | csi-RSRP-AndRSRQ-MeasWithSSB | MeasAndMobParametersFRX-Diff | No | Yes | This does not discourage RAN4 to complete their work. There is expectation that RAN4 will complete the corresponding RRM measurement | Optional with capability signalling |
|  | 1-5 | CSI-RS based RRM measurement without associated SS-block | 1) CSI-RSRP measurement 2) CSI-RSRQ measurement 3) There is SS-block in the target frequency on which the RRM measurement is performed | 1-1, CSI-RS | csi-RSRP-AndRSRQ-MeasWithoutSSB | MeasAndMobParametersFRX-Diff | No | Yes | This does not discourage RAN4 to complete their work. There is expectation that RAN4 will complete the corresponding RRM measurement | Optional with capability signalling |
|  | 1-6 | CSI-RS based RS-SINR measurement | CSI-SINR measurements | 1-1, 1-4 | csi-SINR-Meas | MeasAndMobParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 1-7 | CSI-RS based RLM | CSI-RS based RLM | 1-1, CSI-RS | csi-RS-RLM | MeasAndMobParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 1-8 | RLM based on a mix of SS block and CSI-RS signals within active BWP | RLM based on a mix of SS block and CSI-RS signals within active BWP | 1-3 and 1-7 | ssb-AndCSI-RS-RLM | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |
|  | 1-9 | CSI-RS based contention free RA for HO | CSI-RS based contention free RA for HO | 1-1, CSI-RS, 1-4 or 1-5 | csi-RS-CFRA-ForHO | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 1-10 | Support of SCell without SS/PBCH block | Support SCell without SS/PBCH block | 1-1 | scellWithoutSSB | FeatureSetDownlink | n/a | n/a | Component 1) Whether or not UE is able to use SS/PBCH block from other Cells for time/frequency synchronization of SCell without SS/PBCH block | Mandatory with capability signalling for intra-band CA This feature is not supported for inter band CA |
|  | 1-11 | Support of CSI-RS RRM measurement for SCell without SS/PBCH block |  | 1-10 | csi-RS-MeasSCellWithoutSSB | FeatureSetDownlink | n/a | n/a |  | Optional with capability signalling |
|  | 1-12 | E-UTRA RS-SINR measurement |  |  | rs-SINR-MeasEUTRA | EUTRA-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 1-13 | Maximal number of CSI-RS resources for RRM and RS-SINR measurement across all measurement frequencies per slot |  | 1-4 or 1-5 or 1-6 | maxNumberCSI-RS-RRM-RS-SINR | MeasAndMobParametersCommon | No | No | If UE supports any of 1-5, 1-5a, and 1-6, UE shall report this capability 1-13 | Candidate value set: {4,8,16,32,64, 96} |
|  | 1-14 | Maximal number of CSI-RS resources within a slot per PCell/PSCell for CSI-RS based RLM |  | 1-7 or 1-8 | maxNumberResource-CSI-RS-RLM | MeasAndMobParametersFRX-Diff | No | Yes | If UE supports any of 1-7 and 1-8, UE shall report this capability 1-14 | Candidate value set: {2,4, 6, 8} |
| 2. MIMO | 2-1 | Basic PDSCH reception | 1) Data RE mapping 2) Single layer transmission 3) Support one TCI state |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 2-2 | PDSCH beam switching | 1) Time duration (definition follows section 5.1.5 in TS 38.214), Xi, to determine and apply spatial QCL information for corresponding PDSCH reception. Time duration is defined counting from end of last symbol of PDCCH to beginning of the first symbol of PDSCH. Xi is the number of OFDM symbols, i is the index of SCS, l=1,2, corresponding to 60,120 kHz SCS. | 2-1 | timeDurationForQCL | FeatureSetDownlink | No | Applicable only to FR2 |  | Mandatory with capability signalling for FR2 Candidate value set for X1 is {7, 14, 28}, Candidate value set for X2, {14, 28} |
|  | 2-3 | PDSCH MIMO layers | Supported maximal number of MIMO layers | 2-1 | maxNumberMIMO-LayersPDSCH | FeatureSetDownlinkPerCC | n/a | n/a |  | For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. Some relaxations to this requirement may be applicable in the future (including in Rel-15). Mandatory in all cases means mandatory with capability signalling. It is not expected that there is a signalling change (i.e. signalling remains to be defined as {1, 2, 4, 8} in every band and every band combination, including FR1 and FR2 in all cases. |
|  | 2-4 | TCI states for PDSCH | 1) Support number of active TCI states per BWP per CC, including control and data 2) Maximum number of configured TCI states per CC for PDSCH | 2-1 | tci-StatePDSCH { 1. maxNumberActiveTCI-PerBWP 2. maxNumberConfiguredTCIstatesPerCC } | MIMO-ParametersPerBand | n/a | n/a | UE is required to track only the active TCI states For component 1 of FG2-4, if a UE reports X active TCI state(s), it is not expected that more than X active QCL type D assumption(s) for any PDSCH and any CORESETs for a given BWP of a serving cell become active for the UE. | Mandatory with capability signalling Component-1: Candidate value set: {1, 2, 4, 8} Component-2: candidate value set: {4, 8, 16, 32, 64, 128} UE is mandated to signal 64 for FR2. For FR1, UE is mandated to report at least the max number of allowed SSB in the band. |
|  | 2-4a | Additional active TCI state for PDCCH | Support one additional active TCI state for control in addition to the supported number of active TCI states for PDSCH | 2-1 | additionalActiveTCI-StatePDCCH | MIMO-ParametersPerBand | n/a | n/a | Only applicable if Component-1 of 2-4 is set to 1 | Mandatory with capability signalling |
|  | 2-5 | Basic downlink DMRS for scheduling type A | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port. | 2-1 | n/a | n/a | n/a | n/a | conditioned to whether PDSCH scheduling type A is supported | Mandatory without capability signalling (condition to scheduling capability) |
|  | 2-6 | Basic downlink DMRS for scheduling type B | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol |  | n/a | n/a | n/a | n/a | conditioned to whether PDSCH scheduling type B is supported | Mandatory without capability signalling (condition to scheduling capability) |
|  | 2-6a | Support 1+2 DMRS (downlink) | Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port | 2-5 | oneFL-DMRS-TwoAdditionalDMRS-DL | FeatureSetDownlink-v1540 | No | Yes |  | Mandatory with capability signalling |
|  | 2-6b | Support alternative additional DMRS location | Support alternative additional DMRS position for co-existence with LTE CRS | 2-5 and 5-28 | additionalDMRS-DL-Alt | FeatureSetDownlink-v1540 | No | n/a | This FG applies to FR1 only and 15kHz SCS. This applies to one additional DMRS case only | Optional with capability signalling |
|  | 2-7 | Supported 2 symbols front-loaded DMRS (downlink) | Support 2 symbols FL-DMRS | 2-5 | twoFL-DMRS (MSB) | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-8 | Supported 2 symbols front-loaded +2 symbols additional DMRS (downlink) | Support 2-symbol FL DMRS + one additional 2-symbols DMRS | 2-5 | twoFL-DMRS-TwoAdditionalDMRS-DL | FeatureSetDownlink-v1540 | No | Yes |  | Optional with capability signalling |
|  | 2-9 | Support 1+3 DMRS symbols(downlink) | Support 1 symbol FL DMRS and 3 additional DMRS symbols | 2-5 | oneFL-DMRS-ThreeAdditionalDMRS-DL | FeatureSetDownlink-v1540 | No | Yes |  | Optional with capability signalling |
|  | 2-10 | Support DMRS type (downlink) | Support DMRS {type 1, both type 1 and type 2} |  | supportedDMRS-TypeDL | Phy-ParametersFRX-Diff | No | Yes |  | Type 1 is mandatory with capability signalling. Type 2 is optional with capability signalling |
|  | 2-11 | Downlink dynamic PRB bundling (downlink) | Support dynamic PRB bundling indication via DCI | 2-1 | dynamicPRB-BundlingDL | Phy-ParametersCommon | No | No | Support of semi-static PRB bundling is mandatory | Optional with capability signalling |
|  | 2-12 | Basic PUSCH transmission | Data RE mapping Single layer (single Tx) transmission Single port, single resource SRS transmission (SRS set use is configured as for codebook) |  | n/a | n/a | n/a | n/a | Support of SRS set usage configured as for codebook does not imply UE support of codebook based PUSCH MIMO transmission. | Mandatory without capability signalling |
|  | 2-13 | PUSCH codebook coherency subset | Supported codebook coherency subset type | 2-12 | pusch-TransCoherence | MIMO-ParametersPerBand | n/a | n/a |  | Optional with UE capability Candidate value set: {non-coherent, partial/non-coherent, full/partial/non-coherent} |
|  | 2-14 | Codebook based PUSCH MIMO transmission | 1) Supported codebook based PUSCH MIMO with maximal number of supported layers 2) Supported max number of SRS resource per set (SRS set use is configured as for codebook). | 2-13 | mimo-CB-PUSCH { 1. maxNumberMIMO-LayersCB-PUSCH 2. maxNumberSRS-ResourcePerSet } | FeatureSetUplinkPerCC | n/a | n/a | For SUL, uplink MIMO is not supported. | Optional with UE capability Component-1: Candidate value: {no-codebook based MIMO, 1, 2, 4} Component-2 Candidate value: {1, 2} |
|  | 2-15 | non-codebook based PUSCH transmission | 1) Maximal number of supported layers (non-codebook transmission scheme) 2) Supported max number of SRS resource per set (SRS set use is configured as for non-codebook transmission). 3) Maximum number of simultaneous transmitted SRS resources at one symbol | 2-12 | 1. maxNumberMIMO-LayersNonCB-PUSCH mimo-NonCB-PUSCH { 2. maxNumberSRS-ResourcePerSet 3. maxNumberSimultaneousSRS-ResourceTx } | 1. FeatureSetUplinkPerCC 2, 3. FeatureSetUplinkPerCC-v1540 | n/a | n/a | For SUL, uplink MIMO is not supported | Optional with UE capability Component-1 candidate values: {1, 2, 4} Component-2 Candidate value: {1,2,3,4} Component-3 Candidate value: {1,2,3,4} |
|  | 2-15a | Association between CSI-RS and SRS | 1) Support association between NZP-CSI-RS and SRS resource set via RRC parameter “SRSresoureset” 2) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. | 2-15 | srs-AssocCSI-RS SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF { 2.1. maxNumberTxPortsPerResource 2.2. maxNumberResourcesPerBand 2.3. totalNumberTxPortsPerBand } | MIMO-ParametersPerBand | n/a | n/a |  | Optional with capability signalling Component-2: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports is: {from 2 to 256} |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 2.2. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 2.3. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-15b | CSI-RS processing framework for SRS | 1) Maximum number of periodic SRS resources associated with CSI-RS per BWP 2) Maximum number of aperiodic SRS resources associated with CSI-RS per BWP 3) Maximum number of semi-persistent SRS resources associated with CSI-RS per BWP 4) UE can process Y SRS resources associated with CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS. 5) UE can process X SRS resources associated with CSI-RS resources simultaneously across all CCs. Includes P/SP/A SRS. | 2-15a | csi-RS-ProcFrameworkForSRS { 1. maxNumberPeriodicSRS-AssocCSI-RS-PerBWP 2. maxNumberAperiodicSRS-AssocCSI-RS-PerBWP 3. maxNumberSP-SRS-AssocCSI-RS-PerBWP 4. simultaneousSRS-AssocCSI-RS-PerCC } | MIMO-ParametersPerBand Phy-ParametersFRX-Diff (for FR1 + FR2 band combination) | n/a | n/a | Other MIMO capabilities than component 5 may further restrict (reduce) the number of SRS associated with CSI-RS that the UE has to simultaneously derive. | Optional with capability signalling Component-1 candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-3 candidate values: {0, 1, 2, 3, 4} Component-4 candidate values: {from 1 to 8} Component-5: candidate values: {from 5 to 32} |
|  |  |  |  |  | 5. simultaneousSRS-AssocCSI-RS-AllCC | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-16 | Basic uplink DMRS (uplink) for scheduling type A | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols |  | n/a | n/a | n/a | n/a | Conditioned to whether PUSCH scheduling type A is supported | Mandatory without capability signalling |
|  | 2-16a | Basic uplink DMRS for scheduling type B | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol |  | n/a | n/a | n/a | n/a | conditioned to whether PUSCH scheduling type B is supported | Mandatory without capability signalling |
|  | 2-16b | Support 1+2 DMRS (uplink) | Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port | 2-16a and 2-16 | oneFL-DMRS-TwoAdditionalDMRS-UL | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 2-17 | Support DMRS type (uplink) | Support DMRS {type 1, both type 1 and type 2} | 2-16 | supportedDMRS-TypeUL | Phy-ParametersFRX-Diff | No | Yes |  | Support both type 1 and type 2 are mandatory with capability signalling |
|  | 2-18 | Supported 2 symbols front-loaded DMRS (uplink) | Support 2 symbols FL-DMRS | 2-16 | twoFL-DMRS (LSB) | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 2-18a | Supported 2 symbols front-loaded +2 symbols additional DMRS (uplink) | Support 2-symbol FL DMRS + one additional 2-symbols DMRS | 2-16 | twoFL-DMRS-TwoAdditionalDMRS-UL | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 2-19 | Support 1+3 uplink DMRS symbols(uplink) | Support 1 symbol FL DMRS and 3 additional DMRS symbols | 2-16 | oneFL-DMRS-ThreeAdditionalDMRS-UL | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-20 | Beam correspondence | Support Beam correspondence |  | beamCorrespondenceWithoutUL-BeamSweeping | MIMO-ParametersPerBand | No | Applicable only to FR2 | Beam correspondence means each Tx port can be beamformed in a desirable direction but does not imply setting phase across ports. | Mandatory with capability signalling - UE that fulfils the beam correspondence requirement without the uplink beam sweeping shall set the bit to 1 - UE that fulfils the beam correspondence requirement with the uplink beam sweeping shall set the bit to 0 |
|  | 2-21 | Periodic beam report | 1) Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot 2) Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot |  | periodicBeamReport | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability signalling for both FR1 and FR2 |
|  | 2-22 | Aperiodic beam report | Support aperiodic report on PUSCH |  | aperiodicBeamReport | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability signalling for both FR1 and FR2 |
|  | 2-23 | Semi-persistent beam report on PUCCH | 1) Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot (or piggybacked on a PUSCH) 2) Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot (or piggybacked on a PUSCH) |  | sp-BeamReportPUCCH | MIMO-ParametersPerBand | n/a | Yes |  | Optional with capability signalling |
|  | 2-23a | Semi-persistent beam report on PUSCH | Support semi-persistent report on PUSCH |  | sp-BeamReportPUSCH | MIMO-ParametersPerBand | n/a | Yes |  | Optional with capability signalling |
|  | 2-24 | SSB/CSI-RS for beam measurement | 1) The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot shall not exceed MB_1 2) The max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP shall not exceed MC_1 3) The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_2 4) Supported density of CSI-RS 5) The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-RSRP shall not exceed MD_1 | 2-21, 2-22 or 2-23, 2-23a | beamManagementSSB-CSI-RS { 1. maxNumberSSB-CSI-RS-ResourceOneTx 2. maxNumberCSI-RS-Resource 3. maxNumberCSI-RS-ResourceTwoTx 4. supportedCSI-RS-Density 5. maxNumberAperiodicCSI-RS-Resource } | MIMO-ParametersPerBand | No | Yes |  | Mandatory with capability signalling Component-1, candidate value set for MB_1 is {0, 8, 16, 32, 64} On FR2, UE is mandated to signal MB_1 >=8 On FR1, MB_1 >=8 is supported mandatory with capability signalling. Component-2, candidate value set for MC_1 is {0, 4, 8, 16, 32, 64} For FR1, UE is mandated to report at least 8. Component-3, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-4: candidate value set: {“not supported”, “1 only”, “3 only”, “both 1 and 3”} On FR2, UE is mandated to signal either “3 only” or “both 1 and 3” On FR1, either “3 only” or “both 1 and 3” is mandatory with UE capability signalling. Component-5, candidate value set for MD_2 is {0, 1, 4, 8, 16, 32, 64} For both FR1 and FR2, UE is mandated to report at least 4 |
|  | 2-25 | Beam reporting timing | The number of symbols, Xi, between the last symbol of SSB/CSI-RS and the first symbol of the transmission channel containing beam report is at least RBi, where i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. | 2-24 | beamReportTiming | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability Candidate value sets: X1 is {2, 4, 8} X2 is {4, 8, 14, 28} X3 is {8,14, 28} X4 is{14,28, 56} |
|  | 2-26 | Receiving beam selection using CSI-RS resource repetition "ON" | 1. Support Rx beam switching procedure using CSI-RS resource repetition "ON" 2. Recommended CSI-RS resource repetition number per resource set, |  | maxNumberRxBeam | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with UE capability at least for FR2 Componet-2: candidate value set {2, 3, 4, 5, 6, 7, 8} |
|  | 2-27 | Beam switching | Maximum number of Tx + Rx beam changes a UE can conduct during a slot across the whole band CC B_(B_Total,). This number is defined as per SCS | 2-24 | maxNumberRxTxBeamSwitchDL | MIMO-ParametersPerBand | n/a | Applicable only to FR2 | It is assumed that spec enable the possibility to restrict the same beam across intra-band CCs | Optional with capability signalling Candidate value set: {4, 7, 14} |
|  | 2-28 | A-CSI-RS beam switching timing | Minimum time between the DCI triggering of AP-CSI-RS and aperiodic CSI-RS transmission shall be at least KBi symbols. (Symbols measured from last symbol containing the indication to first symbol of CSI-RS), where i is the index of SCS, l=1,2 corresponding to 60,120 kHz SCS. |  | beamSwitchTiming | MIMO-ParametersPerBand | n/a | Applicable only to FR2 |  | Optional with capability signalling Candidate values: {14, 28, 48, 224, 336} |
|  | 2-29 | Non-group based beam reporting | Support of non-group based RSRP reporting with N_max RSRP values reported |  | maxNumberNonGroupBeamReporting | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability for both FR1 and FR2 candidate value set is {1, 2, 4} |
|  | 2-29a | Group based beam reporting | Support of beam group RSRP reporting for group of 2 beams |  | groupBeamReporting | MIMO-ParametersPerBand | n/a | n/a |  | Optional with capability signalling |
|  | 2-30 | Uplink beam management | 1) Support of SRS based beam management 2) Supported max number of SRS resource per set (SRS set use is configured as for beam management). 3) Supported max number of SRS resource sets (SRS set use is configured as for beam management). |  | uplinkBeamManagement { 2. maxNumberSRS-ResourcePerSet-BM 3. maxNumberSRS-ResourceSet } | MIMO-ParametersPerBand | n/a | n/a | Component-3 also impose additional constraint on the maximum number of SRS resource sets per supported time domain behaviour (periodic/semi-persistent/aperiodic) as {1,1,1,2,2,2,4,4} corresponding to reported values {from 1 to 8} | Optional with capability signalling - Capability signalling shall be set to 1 if 2-20 is set to 0 - For the UE meeting the minimum peak EIRP and spherical coverage requirements without the uplink beam sweeping, this feature is optional Component-2, candidate value set is {2, 4, 8, 16} Component-3, candidate value set is {from 1 to 8} |
|  | 2-31 | Beam failure recovery | 1) Maximal number of CSI-RS resources across all CCs for UE to monitor PDCCH quality 2) Maximal number of different SSBs across all CCs for UE to monitor PDCCH quality 3) Maximal number of different CSI-RS and/or SSB resources across all CCs for new beam identifications. | 1-7 for CSI-RS based BFD/BFR | 1. maxNumberCSI-RS-BFD 2. maxNumberSSB-BFD 3. maxNumberCSI-RS-SSB-CBD | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability signalling for FR2 Optional with capability signalling for FR1 Component-1 candidate value set: {from 1 to 16} Component-2 candidate: {from 1 to 16} Component-3: Candidate value set is: {from 1 to 128} UE is mandated to support at least 32 for FR2 |
|  | 2-32 | Basic CSI feedback | 1) Type I single panel codebook based PMI (further discuss which mode or both to be supported as mandatory) 2) 2Tx codebook for FR1 and FR2 3) 4Tx codebook for FR1 4) 8Tx codebook for FR1 when configured as wideband CSI report 5) p-CSI on PUCCH formats over 1 – 2 OFDM symbols once per slot (or piggybacked on a PUSCH) 6) p-CSI report on PUCCH formats over 4 – 14 OFDM symbols once per slot (or piggybacked on a PUSCH) 7) a-CSI on PUSCH (at least Z value >= 14 symbols, detail processing time to be discussed separately) further check a-CSI on p-CSI-RS and/or SP-CSI-RS from component-7 |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 2-32a | Semi-persistent CSI report on PUCCH | 1) Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot (or piggybacked on a PUSCH) s 2) Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot (or piggybacked on a PUSCH) |  | sp-CSI-ReportPUCCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 2-32b | Semi-persistent CSI report on PUSCH | Support semi-persistent CSI report on PUSCH |  | sp-CSI-ReportPUSCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 2-32c | New CQI table | CQI table with target BLER of 10^-5 |  | cqi-TableAlt | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-33 | CSI-RS and CSI-IM reception for CSI feedback | 1) Supported max # of configured NZP-CSI-RS resources per CC, 2) Supported max # of ports across all configured NZP-CSI-RS resources per CC 3) Supported max # of configured CSI-IM resources per CC 4) Supported max # simultaneous NZP-CSI-RS resources in active BWPs across all CCs 5) Supported max # simultaneous NZP-CSI-RS resources per CC 6) Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7) Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources per CC | 2-32 | csi-RS-IM-ReceptionForFeedback { 1. maxConfigNumberNZP-CSI-RS-PerCC 2. maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC 3. maxConfigNumberCSI-IM-PerCC 5. maxNumberSimultaneousNZP-CSI-RS-PerCC 7. totalNumberPortsSimultaneousNZP-CSI-RS-PerCC } | MIMO-ParametersPerBand Phy-ParametersFRX-Diff (for FR1 + FR2 band combination) | n/a | n/a | All the candidate values are the range of capability signalling which doesn’t determine whether UE is mandatory to support all the signalling values. | Mandatory with capability signalling Component-1 candidate values: {from 1 to 32} Component-2 candidate values: {2, 4, 8, 12, 16, 24, 32, 40, 48 … ,256} Component-3: candidate values: {1,2,4,8,16,32} Component-4: candidate values {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} (includes all even numbers between 16 and 64) Component-5: candidate values {1, 2, 3 … 32} Component-6: candidate values {8, 16, 24, …, 248, 256} Component-7: candidate values {8, 16, 24, … 128 } |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 4. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 6. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-33a | Supported PDSCH RE-mapping patterns | 1) Supported max # of RE mapping patterns, each pattern can be described as a resource (including NZP/ZP CSI-RS and CRS, CORESET and SSB and bitmap configured in 5-26/27) Note: patterns are counted as per symbol per CC 2) Supported max # of RE mapping patterns, each pattern can be described as a resource (including NZP/ZP CSI-RS and CRS, CORESET and SSB and bitmap configured in 5-26/27/27a) Note: patterns are counted as per slot per CC |  | 1. pdsch-RE-MappingFR1-PerSymbol 2. pdsch-RE-MappingFR1-PerSlot | Phy-ParametersFR1 | No | Yes |  | Mandatory with capability signalling candidate values: {10, 20} for FR1 {6, 20} for FR2 Compponent-2 candidate values: {from 16: 16: 256} for FR1 {16: 16: 256} for FR2 |
|  |  |  |  |  | 1. pdsch-RE-MappingFR2-PerSymbol 2. pdsch-RE-MappingFR2-PerSlot | Phy-ParametersFR2 |  |  |  |  |
|  | 2-33b | SP CSI-RS | Support SP CSI-RS | 2-1 | sp-CSI-RS | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 2-33c | SP CSI-IM | Support SP CSI-IM | 2-1 | sp-CSI-IM | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-34 | NZP-CSI-RS based interference measurement | Support NZP-CSI-RS based interference measurement | 2-33 | nzp-CSI-RS-IntefMgmt | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 2-35 | CSI report framework | 1) Maximum number of periodic CSI report setting per BWP for CSI report 2) Maximum number of periodic CSI report setting per BWP for beam report 3) Maximum number of aperiodic CSI report setting per BWP for CSI report 4) Maximum number of aperiodic CSI report setting per BWP for beam report 5) Maximum number of configured aperiodic CSI triggering states in CSI-AperiodicTriggerStateList per CC, 6) Maximum number of semi-persistent CSI report setting per BWP for CSI report 7) Maximum number of semi-persistent CSI report setting per BWP for beam report 8) UE can process Y CSI report(s) simultaneously in a CC. CSI reports can be P/SP/A CSI and any latency class and codebook type. 9) UE can process X CSI report(s) simultaneously across all CCs. CSI reports can be P/SP/A CSI and any latency class and codebook type. | 2-32 | csi-ReportFramework { 1. maxNumberPeriodicCSI-PerBWP-ForCSI-Report 2. maxNumberAperiodicCSI-PerBWP-ForCSI-Report 3. maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report 4. maxNumberPeriodicCSI-PerBWP-ForBeamReport 5. maxNumberAperiodicCSI-PerBWP-ForBeamReport 6. maxNumberAperiodicCSI-triggeringStatePerCC 7. maxNumberSemiPersistentCSI-PerBWP-ForBeamReport 8. simultaneousCSI-ReportsPerCC } | MIMO-ParametersPerBand Phy-ParametersFRX-Diff (for FR1 + FR2 band combination) | n/a | n/a | Other MIMO capabilities than component 5 may further restrict (reduce) the number of simultaneously CSI report that UE is required to update The CSI report in component 4 and 5 includes the beam report and CSI report Each component is independent CSI report setting are counted in the CC indicated by the parameter carrier in CSI-ResourceConfig. | Mandatory with capability signaling Component-1 candidate values: {1, 2, 3, 4} Component-1a candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-2a candidate values {1, 2, 3, 4} Component-2b candidate values {3, 7, 15, 31, 63, 128} Component-3 candidate values: {0, 1, 2, 3, 4} Component-3a candidate values: {0, 1, 2, 3, 4} Component-4 candidate values: {from 1 to 8} Component-5: candidate values: {from 5 to 32} |
|  |  |  |  |  | 9. simultaneousCSI-ReportsAllCC | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-36 | Type I single panel codebook | 1) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2) Supported Codebook Mode(s) 3) Max # of CSI-RS resource in a resource set | 2-35 | 1. supportedCSI-RS-ResourceList SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF { 1.1. maxNumberTxPortsPerResource 1.2. maxNumberResourcesPerBand 1.3. totalNumberTxPortsPerBand } 2. modes 3. maxNumberCSI-RS-PerResourceSet | CodebookParameters | No | n/a | Simultaneously doesn’t mean in the same slot For the purpose of component-1 calculation: CSI-RS resources and CSI-RS ports within one CSI-RS resource are counted N times if the CSI-RS resource is referred by N report settings. | Mandatory with capability signalling Component-1: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2 candidate values: {“Mode-1 only”, “Mode-1 and Mode-2”}. Component-3 Candidate values set: {1:8} |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 1.2. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 1.3. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-37 | Support Semi-open loop CSI | Support Semi-open loop CSI report | 2-35 | semiOpenLoopCSI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-38 | CSI report without PMI | Support CSI report without PMI | 2-35 | csi-ReportWithoutPMI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-39a | CSI report without CQI | Support CSI report without CQI | 2-35 | csi-ReportWithoutCQI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-40 | Type I multi-panel codebook | 1) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2) Supported Codebook Mode(s): 3) Supported number of panels, Ng 4) Max # of CSI-RS resource in a resource set | 2-35 | 1. supportedCSI-RS-ResourceList SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF { 1.1. maxNumberTxPortsPerResource 1.2. maxNumberResourcesPerBand 1.3. totalNumberTxPortsPerBand } 2. modes 3. nrofPanels 4. maxNumberCSI-RS-PerResourceSet | CodebookParameters | n/a | n/a | Simultaneously doesn’t mean in the same slot For the purpose of component-1 calculation: CSI-RS resources and CSI-RS ports within one CSI-RS resource are counted N times if the CSI-RS resource is referred by N report settings. | Optional with capability signalling Component-1: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {8, 16, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2 candidate values: {Mode-1, Mode-2, both} Component-3: Candidate value: {2,4} Component-4: candidate value set is {1:8} |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 1.2. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 1.3. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-41 | Type II codebook | 1) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2) Parameter “Lx” (number of beams) in codebook generation, where x is index of Tx ports, corresponding to 4,8,12,16,24 and 32 ports. 3) Support amplitude scaling type 4) Support amplitude subset restriction level | 2-35 | 1. supportedCSI-RS-ResourceList SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF { 1.1. maxNumberTxPortsPerResource 1.2. maxNumberResourcesPerBand 1.3. totalNumberTxPortsPerBand } 2. parameterLx 3. amplitudeScalingType 4. amplitudeSubsetRestriction | CodebookParameters | n/a | n/a | Simultaneously doesn’t mean in the same slot For the purpose of component-1 calculation: CSI-RS resources and CSI-RS ports within one CSI-RS resource are counted N times if the CSI-RS resource is referred by N report settings. | Optional with capability signalling Component-1: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2, candidate values {2,3,4} Component-3, candidate values set: {wideband, wideband/subband} Component-4, candidate value set: {“no amplitude subset restriction”, “support amplitude subset restriction”} |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 1.2. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 1.3. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-42 | Support Type II SP-CSI feedback on long PUCCH | Support type II SP-CSI feedback part-1 on PUCCH formats over 4 – 14 OFDM symbols once per slot | 2-41 | type2-SP-CSI-Feedback-LongPUCCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 2-43 | Type II codebook with port selection | 1) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2) Parameter “Lx” (number of selected ports) in codebook generation, where x is index of Tx ports, corresponding to 4,8,12,16,24 and 32 ports. 3) Support amplitude scaling type |  | 1. supportedCSI-RS-ResourceList SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF { 1.1. maxNumberTxPortsPerResource 1.2. maxNumberResourcesPerBand 1.3. totalNumberTxPortsPerBand } 2. parameterLx 3. amplitudeScalingType | CodebookParameters | n/a | n/a | Simultaneously doesn’t mean in the same slot For the purpose of component-1 calculation: CSI-RS resources and CSI-RS ports within one CSI-RS resource are counted N times if the CSI-RS resource is referred by N report settings. | Optional with capability signalling Component-1: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2, candidate values set for “Lx” is {2,3,4} Component-3, candidate values set: {wideband, wideband/subband} Component-4: candidate value set is {1:8} |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 1.2. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 1.3. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-44 | Basic DL PTRS | Support 1 port of DL PTRS |  | onePortsPTRS (MSB) | Phy-ParametersFRX-Diff | n/a | Yes |  | Mandatory with capability signalling for FR2 Optional with capability signalling for FR1 |
|  | 2-46 | Downlink PTRS density recommendation | Preferred threshold sets, TSi for determine PTRS density, candidate value range is the same as that of downlink PTRS RRC configuration. i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. | 2-44 | ptrs-DensityRecommendationSetDL { 1. frequencyDensity1 2. frequencyDensity2 3. timeDensity1 4. timeDensity2 5. timeDensity3 } | MIMO-ParametersPerBand | n/a | n/a | For each TSi, it composes of two values each selected from {1..276} for frequency density, and three values each selected from {0..29} for time density | Optional with capability signalling |
|  | 2-47 | Basic UL PTRS | Support 1 port of UL PTRS |  | onePortsPTRS (LSB) | Phy-ParametersFRX-Diff | n/a | Yes |  | Mandatory with capability signalling for FR2 Optional with capability signalling for FR1 |
|  | 2-48 | Uplink PTRS | Supported 2 ports of PTRS | 2-47 | twoPortsPTRS-UL | MIMO-ParametersPerBand | n/a | n/a |  | Optional with capability signalling |
|  | 2-49 | Uplink PTRS density recommendation | Preferred threshold sets, TSi, for determine PTRS density, candidate value range is the same as that of uplink PTRS RRC configuration. i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. | 2-47 | ptrs-DensityRecommendationSetUL { 1. frequencyDensity1 2. frequencyDensity2 3. timeDensity1 4. timeDensity2 5. timeDensity3 6. sampleDensity1 7. sampleDensity2 8. sampleDensity3 9. sampleDensity4 10. sampleDensity5 } | MIMO-ParametersPerBand | n/a | n/a | For each TSi, it composes of two values each selected from {1..276} for frequency density, and three values each selected from {0..29} for time density, and five values each selected from {1..276} for sample density | Optional with capability signalling |
|  | 2-50 | Basic TRS | 1) Support of TRS (mandatory) 2) All the periodicity are supported. 3) Support TRS bandwidth configuration as both “BWP” and “min(52, BWP)” |  | n/a | n/a | n/a | n/a | TRS bandwidth configuration does not imply UE processing bandwidth | Mandatory without capability signalling |
|  | 2-51 | TRS (CSI-RS for tracking) | 1) TRS burst length (X), 2) Max # of TRS resource sets (per CC) UE is able to track simultaneously 3) Max # of TRS resource sets configured to UE per CC 4) Max # of TRS resource sets configured to UE across CCs | 2-50 | csi-RS-ForTracking { 1. maxBurstLength 2. maxSimultaneousResourceSetsPerCC 3. maxConfiguredResourceSetsPerCC 4. maxConfiguredResourceSetsAllCC } | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability signalling Component-1: candidate values {1, ”both 1 and 2”}. UE is mandated to report “both 1 and 2” Component-2: Candidate value set: {1 to 8} Component-3: Candidate value set: {1 to 64} UE is mandated to report at least 8 for FR1 and 16 for FR2. Component-4: Candidate value set: {1 to 256} UE is mandated to report at least 16 for FR1 and 32 for FR2. |
|  | 2-51a | Aperiodic TRS | DCI triggering Aperiodic TRS associated with periodic TRS | 2-50 | aperiodicTRS | MIMO-ParametersPerBand | n/a | Yes |  | Optional with capability signalling |
|  | 2-52 | Basic SRS | 1) Support 1 port SRS transmission 2) Support periodic/aperiodic SRS transmission 3) Support SRS Frequency intra/inter-slot hopping within BWP 4) At least one SRS resource per CC for aperiodic and periodic separately |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 2-53 | SRS resources | 1) Maximum number of aperiodic SRS resources (configured to UE) per BWP 2) Maximum number of aperiodic SRS resources (configured to UE) per BWP per slot 3) Maximum number of periodic SRS resources (configured to UE) per BWP 4) Maximum number of periodic SRS resources (configured to UE) per BWP per slot 5) Maximum number of semi-persistent SRS resources (configured to UE) per BWP 6) Maximum number of semi-persistent SRS resources (configured to UE) per BWP per slot 7) Maximum number of SRS port per resource | 2-52 | supportedSRS-Resources { 1. maxNumberAperiodicSRS-PerBWP 2. maxNumberAperiodicSRS-PerBWP-PerSlot 3. maxNumberPeriodicSRS-PerBWP 4. maxNumberPeriodicSRS-PerBWP-PerSlot 5. maxNumberSemiPersitentSRS-PerBWP 6. maxNumberSP-SRS-PerBWP-PerSlot 7. maxNumberSRS-Ports-PerResource } | FeatureSetUplink | n/a | n/a |  | Mandatory with capability signalling Component-1: candidate value: {from 1, 2, 4, 8, 16} Component-2 candidate value: {1,2,3,4,5,6} Component-3: candidate value: {from 1, 2, 4, 8, 16} Component-4 candidate value: {1,2,3,4,5, 6} Component-5: candidate value: {from 1, 2, 4, 8, 16} } Component-6 candidate value: {1, 2,3,4,5, 6} Component-7 candidate values: {1, 2, 4} Support SP-SRS is mandatory with capability |
|  | 2-55 | SRS Tx switch | 1) Support SRS Tx port switch, 2) Report whether the uplink TX switching impact to downlink receiving in a band, 3) Report whether the UL Tx is switched together with UL Tx in another band | 2-53 | srs-TxSwitch { 1. supportedSRS-TxPortSwitch 2. txSwitchImpactToRx 3. txSwitchWithAnotherBand } | BandParameters-v1540 | n/a | n/a | Component-2 is per band pair per band combination Component-3 is per band pair per band combination The band pair in Component-2 and Component-3 can be an LTE band and an NR band 2T4R is 2 pairs of antennas “R” refers to a subset/set of receive antennas for PDSCH; “T” refers to the SRS antennas used for DL CSI acquisition | Mandatory with capability signalling Component-1 is a list of TRx pairs, candidates are {“Not supported”, “1T2R”, “1T4R”, “2T4R”, “1T4R/2T4R”, “1T=1R”, “2T=2R”, “4T=4R”} Component-2: Candidate value set: {yes, no} Component-3: Candidate value set: {yes, no} |
|  | 2-56 | SRS carrier switch | Report inter-cell switching time capability | 2-53 | srs-CarrierSwitch { srs-SwitchingTimesListNR, or srs-SwitchingTimesListEUTRA } | BandParameters-v1540 | No | n/a | RAN4 reply LS, R1-1805817, includes candidate value sets | Optional with capability signalling |
|  | 2-58 | For SRS for CB PUSCH and antenna switching on FR1, zero slot offset for aperiodic SRS transmission | For SRS for CB PUSCH and antenna switching on FR1, support of zero slot offset between aperiodic SRS triggering and transmission | 2-53 | zeroSlotOffsetAperiodicSRS | FeatureSetUplink-v1540 | n/a | n/a |  | Optional with capability signalling |
|  | 2-59 | Configured spatial relations | Maximum number of configured spatial relations per CC for PUCCH and SRS |  | spatialRelations { 2-59. maxNumberConfiguredSpatialRelations 2-60. maxNumberActiveSpatialRelations 2-61. additionalActiveSpatialRelationPUCCH 2-62. maxNumberDL-RS-QCL-TypeD } | MIMO-ParametersPerBand | n/a | Only applicable to FR2 |  | Candidate value set: {4, 8, 16, 32, 64, 96} UE is mandated to report 16 or higher values. |
|  | 2-60 | Active spatial relations | Maximum total number of {unique DL RS (except for aperiodic NZP CSI-RS) and SRS without spatial relation configured, and, TCI states available for DCI triggering of aperiodic NZP CSI-RS}, for indicating spatial domain transmit filter for PUCCH and SRS for PUSCH, per BWP per CC | 2-59 |  |  | n/a | Only applicable to FR2 | “Unique” means RS identity. An SSB and a CSI-RS are always counted as different. Two CSI-RSs are different if they have different CSI-RS resource IDs. | Mandatory with capability signalling Candidate value set: {1, 2, 4, 8, 14} |
|  | 2-61 | Additional active spatial relation for PUCCH | Support one additional active spatial relation for PUCCH | 2-60 |  |  | n/a | n/a | Only applicable if 2-60 is set to 1. | Mandatory with capability signalling |
|  | 2-62 | Max number of downlink RS resources used for QCL type-D in the active TCI states and active spatial relation info | Max number of downlink RS resources in the active TCI states and active spatial relation info per CC | 2-4, 2-4a and 2-60 |  |  | n/a | n/a | Reference relationship follows 2-4/2-60 | Optional with capability signalling Candidate value set: {1,2,4,8, 14} |
| 3. DL control channel and procedure | 3-1 | Basic DL control channel | 1) One configured CORESET per BWP per cell in addition to CORESET0 - CORESET resource allocation of 6RB bit-map and duration of 1 – 3 OFDM symbols for FR1 - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSSs, CORESET resource allocation of 6RB bit-map and duration 1-3 OFDM symbols for FR2 - For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 1-2 OFDM symbols for FR2 - REG-bundle sizes of 2/3 RBs or 6 RBs - Interleaved and non-interleaved CCE-to-REG mapping - Precoder-granularity of REG-bundle size - PDCCH DMRS scrambling determination - TCI state(s) for a CORESET configuration 2) CSS and UE-SS configurations for unicast PDCCH transmission per BWP per cell - PDCCH aggregation levels 1, 2, 4, 8, 16 - UP to 3 search space sets in a slot for a scheduled SCell per BWP This search space limit is before applying all dropping rules. - For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot 3) Monitoring DCI formats 0_0, 1_0, 0_1, 1_1 4) Number of PDCCH blind decodes per slot with a given SCS follows Case 1-1 table 5) Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot per scheduled CC for FDD 6) Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot per scheduled CC for TDD |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 3-1’ | For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 3 OFDM symbols for FR2 | For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 3 OFDM symbols for FR2 |  | type1-3-CSS | FeatureSetDownlink | n/a | n/a |  | Mandatory with capability signalling |
|  | 3-2 | PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot | For a given UE, all search space configurations are within the same span of 3 consecutive OFDM symbols in the slot |  | pdcchMonitoringSingleOccasion | Phy-ParametersFR1 | No | Applicable only to FR1 |  | Optional with capability signalling |
|  | 3-3 | More than one CORESET configurations per BWP in addition to CORESET0 | More than one CORESET configurations per BWP in addition to CORESET0 |  | multipleCORESET | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 |
|  | 3-4 | More than one TCI state configurations per CORESET | More than one TCI state configurations per CORESET |  | multipleTCI | BandNR | n/a | n/a | UE is only required to track one active TCI state per CORESET UE is required to support minimum between 64 and number of configured TCI states in 2-4, component 2). | Mandatory with capability signaling which shall be set to ‘1’ |
|  | 3-5 | For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 | For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 |  | pdcch-MonitoringAnyOccasions { 3-5. withoutDCI-Gap 3-5a. withDCI-Gap } | FeatureSetDownlink | n/a | n/a |  | Optional with capability signalling |
|  | 3-5a | For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a DCI gap | For type 1 CSS with dedicated RRC configuration, type 3 CSS and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2, with minimum time separation (including the cross-slot boundary case) between two DL unicast DCIs, between two UL unicast DCIs, or between a DL and an UL unicast DCI in different monitoring occasions where at least one of them is not the monitoring occasions of FG-3-1, for a same UE as - 2OFDM symbols for 15kHz - 4OFDM symbols for 30kHz - 7OFDM symbols for 60kHz with NCP - 11OFDM symbols for 120kHz Up to one unicast DL DCI and up to one unicast UL DCI in a monitoring occasion except for the monitoring occasions of FG 3-1. In addition for TDD the minimum separation between the first two UL unicast DCIs within the first 3 OFDM symbols of a slot can be zero OFDM symbols. |  |  |  | n/a | n/a |  | Optional with capability signalling |
|  | 3-5b | All PDCCH monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a span gap | PDCCH monitoring occasions of FG-3-1, plus additional PDCCH monitoring occasion(s) can be any OFDM symbol(s) of a slot for Case 2, and for any two PDCCH monitoring occasions belonging to different spans, where at least one of them is not the monitoring occasions of FG-3-1, in same or different search spaces, there is a minimum time separation of X OFDM symbols (including the cross-slot boundary case) between the start of two spans, where each span is of length up to Y consecutive OFDM symbols of a slot. Spans do not overlap. Every span is contained in a single slot. The same span pattern repeats in every slot. The separation between consecutive spans within and across slots may be unequal but the same (X, Y) limit must be satisfied by all spans. Every monitoring occasion is fully contained in one span. In order to determine a suitable span pattern, first a bitmap b(l), 0<=l<=13 is generated, where b(l)=1 if symbol l of any slot is part of a monitoring occasion, b(l)=0 otherwise. The first span in the span pattern begins at the smallest l for which b(l)=1. The next span in the span pattern begins at the smallest l not included in the previous span(s) for which b(l)=1. The span duration is max{maximum value of all CORESET durations, minimum value of Y in the UE reported candidate value} except possibly the last span in a slot which can be of shorter duration. A particular PDCCH monitoring configuration meets the UE capability limitation if the span arrangement satisfies the gap separation for at least one (X, Y) in the UE reported candidate value set in every slot, including cross slot boundary. For the set of monitoring occasions which are within the same span: • Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for FDD • Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD • Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD The number of different start symbol indices of spans for all PDCCH monitoring occasions per slot, including PDCCH monitoring occasions of FG-3-1, is no more than floor(14/X) (X is minimum among values reported by UE). The number of different start symbol indices of PDCCH monitoring occasions per slot including PDCCH monitoring occasions of FG-3-1, is no more than 7. The number of different start symbol indices of PDCCH monitoring occasions per half-slot including PDCCH monitoring occasions of FG-3-1 is no more than 4 in SCell. |  | pdcch-MonitoringAnyOccasionsWithSpanGap (X, Y): set1 = (7, 3); set2 = (4, 3) and (7, 3); set3 = (2, 2) and (4, 3) and (7, 3). | FeatureSetDownlink-v1540 | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling Candidate value set for (X, Y): {(7, 3), (4, 3) and (7, 3), (2, 2) and (4, 3) and (7, 3)} |
|  | 3-6 | Dynamic SFI monitoring | Adjust periodic and semi-persistent signal reception and transmission in response to detected dynamic UL/DL configuration |  | dynamicSFI | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Optional with capability signalling |
|  | 3-7 | Precoder-granularity of CORESET size | Precoder-granularity of CORESET size |  | precoderGranularityCORESET | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 3-8 | Up to 10 search spaces in a SCell | Up to 10 search spaces in a slot in an SCell per BWP |  | maxNumberSearchSpaces | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
| 4. UL control channel and procedure | 4-1 | Basic UL control channel | 1) PUCCH format 0 over 1 OFDM symbols once per slot 2) PUCCH format 0 over 2 OFDM symbols once per slot with frequency hopping as “enabled” 3) PUCCH format 1 over 4 – 14 OFDM symbols once per slot with intra-slot frequency hopping as “enabled” 5) One SR configuration per PUCCH group 6) HARQ-ACK transmission once per slot with its resource/timing determined by using the DCI 7) SR/HARQ multiplexing once per slot using a PUCCH when SR/HARQ-ACK are supposed to be sent by overlapping PUCCH resources with the same starting symbols in a slot 8) HARQ-ACK piggyback on PUSCH with/without aperiodic CSI once per slot when the starting OFDM symbol of the PUSCH is the same as the starting OFDM symbols of the PUCCH resource that HARQ-ACK would have been transmitted on 9) Semi-static beta-offset configuration for HARQ-ACK 10) Single group of overlapping PUCCH/PUCCH and overlapping PUCCH/PUSCH s per slot per PUCCH cell group for control multiplexing |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 4-2 | 2 PUCCH of format 0 or 2 in consecutive symbols | 1) 2 PUCCH format 0/2 in different symbols and once per slot for HARQ-ACK, 2) 2 PUCCH format 0 in different symbols and once per slot for SR 3) 2 PUCCH format 2 in different symbols and once per slot for CSI over two consecutive OFDM symbols |  | twoPUCCH-F0-2-ConsecSymbols | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Optional with capability signalling |
|  | 4-3 | PUCCH format 2 over 1 – 2 OFDM symbols once per slot with frequency hopping as “enabled” | PUCCH format 2 over 1 – 2 OFDM symbols once per slot with frequency hopping as “enabled” |  | pucch-F2-WithFH | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling which shall be set to ‘1’ |
|  | 4-4 | PUCCH format 3 over 4 – 14 OFDM symbols once per slot with frequency hopping as “enabled” | PUCCH format 3 over 4 – 14 OFDM symbols once per slot with frequency hopping as “enabled” |  | pucch-F3-WithFH | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling which shall be set to ‘1’ |
|  | 4-5 | PUCCH format 4 over 4 – 14 OFDM symbols once per slot with frequency hopping as “enabled” | PUCCH format 4 over 4 – 14 OFDM symbols once per slot with frequency hopping as “enabled” |  | pucch-F4-WithFH | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 4-6 | Non-frequency hopping for PUCCH formats 0 and 2 with frequency hopping as “disabled” | Non-frequency hopping for PUCCH formats 0 and 2 with frequency hopping as “disabled” |  | freqHoppingPUCCH-F0-2 | Phy-ParametersFRX-Diff | No | Yes | The value indicated by this field is “notSupported”. | Mandatory with capability signalling |
|  | 4-7 | Non-frequency hopping for PUCCH format 1, 3, and 4 with frequency hopping as “disabled” | Non-frequency hopping for PUCCH format 1, 3, and 4 with frequency hopping as “disabled” |  | freqHoppingPUCCH-F1-3-4 | Phy-ParametersFRX-Diff | No | Yes | The value indicated by this field is “notSupported”. | Mandatory with capability signalling |
|  | 4-10 | Dynamic HARQ-ACK codebook | Dynamic HARQ-ACK codebook |  | dynamicHARQ-ACK-Codebook | Phy-ParametersCommon | No | No |  | Mandatory with capability signaling which shall be set to ‘1’ |
|  | 4-11 | Semi-static HARQ-ACK codebook | Semi-static HARQ-ACK codebook |  | semiStaticHARQ-ACK-Codebook | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 4-12 | HARQ-ACK spatial bundling for PUCCH or PUSCH per PUCCH group | HARQ-ACK spatial bundling for PUCCH or PUSCH per PUCCH group |  | spatialBundlingHARQ-ACK | Phy-ParametersCommon | No | No | Applicable to UE supporting more than 4 layers | Mandatory with capability signalling |
|  | 4-13 | More than one SR configurations per PUCCH group | More than one SR configurations per PUCCH group |  | multipleSR-Configurations | MAC-ParametersXDD-Diff | Yes | No |  | Optional with capability signalling |
|  | 4-19 | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with the same starting symbol on the PUCCH resources in a slot | SR/HARQ-ACK/CSI multiplexing once per slot, where overlapping PUCCH resources have the same starting symbols on the PUCCH resources in a slot while precluding the case of SR/HARQ-ACK by overlapping PUCCH resources with the same starting symbols on the PUCCH resources in a slot |  | sameSymbol in mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot | Phy-ParametersFRX-Diff | No | Yes | If FG4-28 is not included or not supported, HARQ-ACK/CSI piggyback on PUSCH once per slot when the starting OFDM symbol of the PUSCH is the same as the starting OFDM symbols of the PUCCH resource(s) that would have been transmitted on If FG4-28 is included and supported, HARQ-ACK/CSI piggyback on PUSCH once per slot for which case the starting OFDM symbol of the PUSCH is the different from the starting OFDM symbols of the PUCCH resource(s) that would have been transmitted on | Mandatory with capability signalling |
|  | 4-19a | SR/HARQ-ACK multiplexing once per slot using a PUCCH (or HARQ-ACK piggybacked on a PUSCH) when SR/HARQ-ACK are supposed to be sent with different starting symbols in a slot | Overlapping PUCCH resources have different starting symbols in a slot | 4-19 | mux-SR-HARQ-ACK-PUCCH | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 4-19b | SR/HARQ-ACK/CSI multiplexing more than once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with the same or different starting symbol in a slot | Overlapping PUCCH resources have same or different starting symbols in a slot | 4-19c | mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 4-19c | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with different starting symbols in a slot | Overlapping PUCCH resources have different starting symbols in a slot | 4-19a | diffSymbol in mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 4-20 | UCI code-block segmentation | UCI code-block segmentation |  | uci-CodeBlockSegmentation | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 4-21 | Dynamic beta-offset configuration and indication for HARQ-ACK and/or CSI | Dynamic beta-offset configuration and indication for HARQ-ACK and/or CSI |  | dynamicBetaOffsetInd-HARQ-ACK-CSI | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 4-22 | 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3, or 4 in the same slot | 1 PUCCH format 0 or 2and 1 PUCCH format 1, 3, and 4 in the same slot |  | onePUCCH-LongAndShortFormat | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 4-22a | 2 PUCCH transmissions in the same slot which are not covered by 4-22 and 4-2 | 2 PUCCH transmissions in the same slot which are not covered by 4-22 and 4-2 |  | twoPUCCH-AnyOthersInSlot | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 4-23 | Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8 |  | pucch-Repetition-F1-3-4 | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 4-24 | PUCCH-spatialrelationinfo indication by a MAC CE per PUCCH resource | PUCCH-spatialrelationinfo indication by a MAC CE per PUCCH resource |  | pucch-SpatialRelInfoMAC-CE | BandNR | n/a | n/a |  | Mandatory with capability signalling for FR2 Optional with capability signalling for FR1 |
|  | 4-25 | Parallel SRS and PUCCH/PUSCH transmission across CCs in inter-band CA | Parallel SRS and PUCCH/PUSCH transmission across CCs in inter-band CA | 2-52, 4-1, 2-12, 6-6 | parallelTxSRS-PUCCH-PUSCH | CA-ParametersNR | n/a | n/a | This feature is supported only in inter-band CA. | Optional with capability signalling |
|  | 4-26 | Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA | Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA | 1-1, 2-52, 4-1, 2-12, 6-6 | parallelTxPRACH-SRS-PUCCH-PUSCH | CA-ParametersNR | n/a | n/a | This feature is supported only in inter-band CA. | Optional with capability signalling |
|  | 4-27 | More than one group of overlapping channels for control multiplexing | More than one group of overlapping PUCCHs and PUSCHs per slot per PUCCH cell group for control multiplexing |  | mux-MultipleGroupCtrlCH-Overlap | Phy-ParametersFRX-Diff | No | Yes | If a UE does not indicate supporting any of 4-2, 4-22, and 4-22a, the UE is not expected to be scheduled with more than one group of overlapping PUCCHs without PUSCH in each of the groups | Optional with capability signalling |
|  | 4-28 | HARQ-ACK multiplexing on PUSCH with different PUCCH/PUSCH starting OFDM symbols | HARQ-ACK piggyback on a PUSCH with/without aperiodic CSI once per slot when the starting OFDM symbol of the PUSCH is different from the starting OFDM symbols of the PUCCH resource that HARQ-ACK would have been transmitted on | 4-1 | mux-HARQ-ACK-PUSCH-DiffSymbol | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
| 5. Scheduling/HARQ operation | 5-1 | Basic scheduling/HARQ operation | 1) Frequency-domain resource allocation - RA Type 0 only and Type 1 only for PDSCH without interleaving - RA Type 1 for PUSCH without interleaving 2) Time-domain resource allocation - 1-14 OFDM symbols for PUSCH once per slot - One unicast PDSCH per slot - Starting symbol, and duration are determined by using the DCI - PDSCH mapping type A with 7-14 OFDM symbols - PUSCH mapping type A and type B - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, PDSCH mapping type A with {4-14} OFDM symbols and type B with {2, 4, 7} OFDM symbols 3) TBS determination 4) Nominal UE processing time for N1 and N2 (Capability #1) 5) HARQ process operation with configurable number of DL HARQ processes of up to 16 6) Cell specific RRC configured UL/DL assignment for TDD 7) Dynamic UL/DL determination based on L1 scheduling DCI with/without cell specific RRC configured UL/DL assignment 8) Intra-slot frequency-hopping for PUSCH scheduled by Type 1 CSS before RRC connection 9) In TDD support at most one switch point per slot for actual DL/UL transmission(s) 10) DL scheduling slot offset K0=0 11) DL scheduling slot offset K0=1 for type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS 12) UL scheduling slot offset K2<=12 For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, interleaving for VRB-to-PRB mapping for PDSCH |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 5-1a | UE specific RRC configure UL/DL assignment | Dynamic UL/DL determination based on L1 scheduling DCI with cell-specific and UE specific RRC configured UL/DL assignment |  | ue-SpecificUL-DL-Assignment | FeatureSetDownlink | n/a | n/a |  | Optional with capability signalling |
|  | 5-1b | More than one DL/UL switch point in a slot | In TDD support more than one switch points in a slot for actual DL/UL transmission(s) |  | tdd-MultiDL-UL-SwitchPerSlot | Phy-ParametersFRX-Diff | TDD only | Yes |  | Optional with capability signalling |
|  | 5-2 | RA Type 0 for PUSCH | RA Type 0 for PUSCH |  | ra-Type0-PUSCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-3 | Dynamic switching between RA Type 0 and RA Type 1 for PDSCH | Dynamic switching between RA Type 0 and RA Type 1 for PDSCH |  | dynamicSwitchRA-Type0-1-PDSCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-4 | Dynamic switching between RA Type 0 and RA Type 1 for PUSCH | Dynamic switching between RA Type 0 and RA Type 1 for PUSCH | 5-2 | dynamicSwitchRA-Type0-1-PUSCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-5a | UE PDSCH processing capability #2 | UE can report values ‘X’ and ‘Fallback’, and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 1 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled, otherwise - If Fallback = ‘SC’, UE supports Capability #2 processing time on lowest cell index among the configured carriers in the band where the value is reported - If Fallback = ‘Cap1-only’, UE supports only Capability #1, in the band where the value is reported 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz |  | pdsch-ProcessingType2 | FeatureSetDownlink-v1540 | n/a | Applicable to FR1 only | This capability is necessary for each SCS (15kHz, 30kHz, 60kHz) More than one set of per SCS per band reports can be signaled for a given band combination | ptional with capability signaling Candidate values for Component 1: X in {1, ..., 16}, Fallback {‘SC’,’Cap1-only’} |
|  | 5-5b | UE PDSCH processing capability #2 with scheduling limitation for 30kHz-SCS | Capability #2 supported only if 1 carrier configured in the band (independent of #carriers configured in other bands) 2) Max PDSCH BW of 136 PRBs on the configured serving cell which processingType2Enabled is configured and set to enabled 3) N1 based on Table 5.3-2 of TS 38.214 for 30 kHz SCS 4) UE reports the number of unicast PDSCH per slot for different TBs |  | pdsch-ProcessingType2-Limited | FeatureSetDownlink-v1540 | n/a | Applicable to FR1 only | This capability is applicable to 30kHz-SCS only | Optional with capability signaling Component 4) the value ranges {1, 2, 4, 7} |
|  | 5-5c | UE PUSCH processing capability #2 | UE can report values ‘X’ and ‘Fallback’, and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 1 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled, otherwise - If Fallback = ‘SC’, UE supports Capability #2 processing time on lowest cell index among the configured carriers in the band where the value is reported - If Fallback = ‘Cap1-only’, UE supports only Capability #1, in the band where the value is reported 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz |  | pusch-ProcessingType2 | FeatureSetUplink-v1540 | n/a | Applicable to FR1 only | This capability is necessary for each SCS (15kHz, 30kHz, 60kHz) More than one set of per SCS per band reports can be signaled for a given band combination | Optional with capability signaling Candidate values for Component 1: X in {1, …, 16}, Fallback {‘SC’,’Cap1-only’} |
|  | 5-6 | PDSCH mapping type A with less than 7 OFDM symbols | or type 1 CSS with dedicated RRC configuration, for type 3 CSS and UE-SS, PDSCH mapping type A with less than 7 OFDM symbols |  | pdsch-MappingTypeA | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling which shall be set to “1” |
|  | 5-6a | PDSCH mapping type B | PDSCH mapping type B |  | pdsch-MappingTypeB | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 5-7 | Interleaving for VRB-to-PRB mapping for PDSCH | Interleaving for VRB-to-PRB mapping for PDSCH |  | interleavingVRB-ToPRB-PDSCH | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 5-9 | Intra-slot frequency-hopping for PUSCH except for PUSCH scheduled by Type 1 CSS before RRC connection | Intra-slot frequency-hopping for PUSCH except for PUSCH scheduled by Type 1 CSS before RRC connection |  | intraSlotFreqHopping-PUSCH | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 5-10 | Inter-slot frequency hopping for PUSCH | Inter-slot frequency hopping for PUSCH |  | interSlotFreqHopping-PUSCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-11 | Up to 2 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 2 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 1) PDSCH(s) for Msg. 4 is included |  | pdsch-ProcessingType1-DifferentTB-PerSlot | FeatureSetDownlink | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-11a | Up to 7 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 7 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 1) PDSCH(s) for Msg. 4 is included |  |  |  | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-11b | Up to 4 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 4 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 1) PDSCH(s) for Msg. 4 is included |  |  |  | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-12 | Up to 2 PUSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 2 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1 |  | pusch-ProcessingType1-DifferentTB-PerSlot | FeatureSetUplink | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-12a | Up to 7 PUSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 7 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1 |  |  |  | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-12b | Up to 4 PUSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 4 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1 |  |  |  | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-13 | Up to 2 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 2 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 2 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5a | pdsch-ProcessingType2 | FeatureSetDownlink | n/a | n/a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-13a | Up to 7 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 7 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 7 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5a |  |  | n/a | n//a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-13c | Up to 4 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 4 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 4 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5a |  |  | n/a | n/a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-13d | Up to 2 PUSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 2 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 2 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5c | pusch-ProcessingType2 | FeatureSetUplink | n/a | n/a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-13e | Up to 7 PUSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 7 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 7 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5c |  |  | n/a | n/a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-13f | Up to 4 PUSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 4 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 4 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5c |  |  | n/a | n/a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-14 | Type 1 configured PUSCH repetitions over multiple slots | K = 2, 4, 8 times repetitions with RV sequences |  | type1-PUSCH-RepetitionMultiSlots | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-16 | Type 2 configured PUSCH repetitions over multiple slots | K = 2, 4, 8 times repetitions with RV sequences |  | type2-PUSCH-RepetitionMultiSlots | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-17 | PUSCH repetitions over multiple slots | K = 2, 4, 8 times repetitions |  | pusch-RepetitionMultiSlots | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 5-17a | PDSCH repetitions over multiple slots | K = 2, 4, 8 times repetitions |  | pdsch-RepetitionMultiSlots | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-18 | DL SPS | DL SPS |  | downlinkSPS | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-19 | Type 1 Configured UL grant | K = 1 |  | configuredUL-GrantType1 | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-20 | Type 2 Configured UL grant | K = 1 |  | configuredUL-GrantType2 | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-21 | Pre-emption indication for DL | Pre-emption indication for DL |  | pre-EmptIndication-DL | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-22 | CBG-based re-transmission for DL using CBGTI | CBG-based re-transmission for DL using CBGTI |  | cbg-TransIndication-DL | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-23 | CBGFI for CBG-based re-transmission for DL | CBGFI for CBG-based re-transmission for DL | 5-22 | cbg-FlushIndication-DL | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-24 | Dynamic HARQ-ACK codebook using sub-codebooks for CBG-based re-transmission for DL | Dynamic HARQ-ACK codebook using sub-codebooks for CBG-based re-transmission for DL |  | dynamicHARQ-ACK-CodeB-CBG-Retx-DL | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-25 | CBG-based re-transmission for UL using CBGTI | CBG-based re-transmission for UL using CBGTI |  | cbg-TransIndication-UL | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-26 | Semi-static rate-matching resource set configuration for DL | 1) Bitmap 1/2/3 2) controlResourceSet |  | rateMatchingResrcSetSemi-Static | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 5-27 | Dynamic rate-matching resource set configuration for DL | Bitmap 1/2/3 |  | rateMatchingResrcSetDynamic | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-27a | Dynamic rate-matching control resource set for DL | Dynamic rate-matching control resource set for DL |  | rateMatchingCtrlResrcSetDynamic | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 5-28 | Rate-matching around LTE CRS | Rate-matching around LTE CRS |  | rateMatchingLTE-CRS | BandNR | n/a | n/a |  | Mandatory with capability signalling |
|  | 5-29 | LBRM for PUSCH | Limited buffer rate matching in UL |  | pusch-LBRM | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 5-30 | DL scheduling slot offset greater than zero for PDSCH mapping type A | Support of DL scheduling slot offset (K0) greater than zero for PDSCH mapping type A |  | dl-SchedulingOffset-PDSCH-TypeA | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Mandatory with capability signalling |
|  | 5-30a | DL scheduling slot offset greater than zero for PDSCH mapping type B | Support of DL scheduling slot offset (K0) greater than zero for PDSCH mapping type B |  | dl-SchedulingOffset-PDSCH-TypeB | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Mandatory with capability signalling |
|  | 5-31 | UL scheduling slot offset greater than 12 | Support of UL scheduling slot offset (K2) greater than 12 |  | ul-SchedulingOffset | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Mandatory with capability signalling |
|  | 5-32 | Separation of two unicast PDSCHs with a gap | For any two consecutive slots n and n+1, if there are more than 1 unicast PDSCH in either slot, the minimum time separation between starting time of any two unicast PDSCHs within the duration of these slots is 4 OFDM symbol for 30kHz and 7 OFDM symbol for 60kHz | 5-11, 5-11b, 5-13, or 5-13c | pdsch-SeparationWithGap | FeatureSetDownlink-v1540 | No | No | This feature only applies to SCS 30kHz and 60kHz | Optional with capability signalling |
|  | 5-33 | Separation of two unicast PUSCHs with a gap | For any two consecutive slots n and n+1, if there are more than 1 unicast PUSCH in either slot, the minimum time separation between starting time of any two unicast PUSCHs within the duration of these slots is 2OFDM symbols for 15kHz, 4 OFDM symbols for 30kHz and 7 OFDM symbols for 60kHz | 5-12, 5-12b, 5-13d, or 5-13f | pusch-SeparationWithGap | FeatureSetUplink-v1540 | No | No | This feature only applies to SCS 15kHz, 30kHz and 60kHz | Optional with capability signalling |
|  | 5-34 | New 64QAM MCS table for PDSCH | New 64QAM MCS table for PDSCH |  | dl-64QAM-MCS-TableAlt | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 5-34a | New 64QAM MCS table for PUSCH | New 64QAM MCS tables for PUSCH with and without transform precoding respectively |  | ul-64QAM-MCS-TableAlt | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 5-34b | Dynamic indication of MCS table with MCS-C-RNTI for PDSCH | Dynamic indication of MCS table using MCS-C-RNTI for PDSCH | 5-34 | dl-MCS-TableAlt-DynamicIndication | FeatureSetDownlink-v1540 | n/a | n/a |  | Optional with capability signalling |
|  | 5-34c | Dynamic indication of MCS tables with MCS-C-RNTI for PUSCH | Dynamic indication of MCS tables using MCS-C-RNTI for PUSCH | 5-34a | ul-MCS-TableAlt-DynamicIndication | FeatureSetUplink-v1540 | n/a | n/a |  | Optional with capability signalling |
| 6. CA/DC, BWP, SUL | 6-1 | Basic BWP operation with restriction | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell |  | n/a | n/a | n/a | n/a | This feature should be mandatory without capability signalling for at least BWPs which is the same as the set of specified channel BW UE-specific RRC configured DL/UL BWP can have the same or different numerology from the initial active DL/UL BWP | Mandatory without capability signalling |
|  | 6-1a | BWP operation without restriction on BW of BWP(s) | BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell | 6-1, 6-2, 6-3, or 6-4 | bwp-WithoutRestriction | BandNR | n/a | n/a | 6-1a is applicable to 6-1, 6-2, 6-3, or 6-4. | Optional with capability signalling |
|  | 6-2 | Type A BWP adaptation with same numerology | 1) Up to 2 UE-specific RRC configured DL BWPs per carrier 2) Up to 2 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell | 6-1 | upto2 in bwp-SameNumerology | BandNR | n/a | n/a |  | Optional with capability signalling |
|  | 6-3 | Type B BWP adaptation with same numerology | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell | 6-1 | upto4 in bwp-SameNumerology | BandNR | n/a | n/a |  | Optional with capability signalling |
|  | 6-4 | BWP adaptation with different numerologies | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) More than one numerologies for the UE-specific RRC configured BWPs per carrier 5) Same numerology between DL and UL per cell except for SUL at a given time 6) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell | 6-1 | upto4 in bwp-DiffNumerology | BandNR | n/a | n/a |  | Optional with capability signalling |
|  | 6-5 | Basic DL NR-NR CA operation | 1) Up to16 DL carriers 2) Same numerology across carrier for data/control channel at a given time |  | supportedBandCombinationList | RF-Parameters | n/a | n/a | This is conditioned on the support of DL CA band combination(s). | Optional with capability signalling |
|  | 6-5a | PDCCH blind detection capability for CA | 1) More than 4 DL CCs 2) Reporting value is one of integer from 4 to 16 | 6-5 | pdcch-BlindDetectionCA | Phy-ParametersFRX-Diff | No | Yes | If UE supports CA with more than 4 DL CCs, UE should report this capability | {4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16} |
|  | 6-6 | Basic UL NR-NR CA operation | 1) Up to16 UL carriers 2) Same numerology across carrier for data/control channel at a given time 3) One PUCCH group 4) Single TAG | 6-5 | supportedBandCombinationList | RF-Parameters | n/a | n/a | This is conditioned on the support of UL CA band combination(s). The terminology ‘carrier’ in the components in this FG does not refer to ‘SUL’. | Optional with capability signalling |
|  | 6-7 | Two NR PUCCH group with same numerology | 1) For NR CA UE, same numerology across NR carriers for data/control channel at a given time 2) For EN-DC UE, same numerology across NR carriers for data/control channel at a given time, wherein an NR PUCCH group is configured in FR1 and another NR PUCCH group is configured in FR2 | 6-5, 6-6 | twoPUCCH-Group | FeatureSetUplink | n/a | n/a |  | Optional with capability signalling |
|  | 6-8 | Different numerology across NR PUCCH groups | For both NR CA UE and EN-DC UE, different numerology between two NR PUCCH groups for data/control channel at a given time | 6-5, 6-7 | diffNumerologyAcrossPUCCH-Group | CA-ParametersNR | n/a | n/a |  | Optional with capability signalling |
|  | 6-9 | Different numerologies across NR carriers within the same NR PUCCH group, with PUCCH on a carrier of smaller SCS | 1) For both NR CA UE, EN-DC/NE-DC UE and NR-DC UEs, same numerology between DL and UL per carrier for data/control channel at a given time 2) For both NR CA UE and EN-DC/NE-DC UE with one NR PUCCH group, different numerologies across NR carriers within the same NR PUCCH groups up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is sent on the carrier with smaller SCS for data/control channel at a given time 3-1) For NR CA UE with two NR PUCCH groups, different numerologies across NR carriers up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is sent on the carrier with smaller SCS for data/control channel at a given time 3-2) For EN-DC/NE-DC UE with two NR PUCCH groups, different numerologies across NR carriers up to two different numerologies within an NR PUCCH group in FR1 wherein NR PUCCH is sent on the carrier with smaller SCS, and same numerology across NR carriers within another NR PUCCH group in FR2 for data/control channel at a given time 4) For NR DC UE, different numerologies across NR carriers within the same NR PUCCH group in MCG (in FR1) and up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is sent on the carrier with smaller SCS for data/control channel at a given time; and same numerology across NR carriers in SCG (in FR2). | 6-5 | diffNumerologyWithinPUCCH-GroupSmallerSCS | CA-ParametersNR | n/a | n/a | The terminologies ‘UL’ and ‘carrier’ in this FG do not refer to ‘SUL’. NR PUCCH is sent on a carrier with SCS not larger than SCS of any DL carriers corresponding to the NR PUCCH group. The case with PUCCH on UL carrier with different numerologies within SCG is not supported for NR-DC. | Optional with capability signalling |
|  | 6-9a | Different numerologies across NR carriers within the same NR PUCCH group, with PUCCH on a carrier of larger SCS | 1) For both NR CA UE, EN-DC/NE-DC UE and NR DC UEs, same numerology between DL and UL per carrier for data/control channel at a given time 2) For both NR CA UE and EN-DC/NE-DC UE with one NR PUCCH group, different numerologies across NR carriers within the same NR PUCCH groups up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is on the carrier with larger SCS for data/control channel at a given time 3-1) For NR CA UE with two NR PUCCH groups, different numerologies across NR carriers up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is sent on the carrier with larger SCS for data/control channel at a given time 3-2) For EN-DC/NE-DC UE with two NR PUCCH groups, different numerologies across NR carriers up to two different numerologies within an NR PUCCH group in FR1 wherein NR PUCCH is sent on the carrier with larger SCS, and same numerology across NR carriers within another NR PUCCH group in FR2 for data/control channel at a given time 4) For NR DC UE, different numerologies across NR carriers within the same NR PUCCH group in MCG (in FR1) and up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is sent on the carrier with larger SCS for data/control channel at a given time; and same numerology across NR carriers in SCG (in FR2). | 6-5 | diffNumerologyWithinPUCCH-GroupLargerSCS | CA-ParametersNR-v1560 | n/a | n/a | The terminologies ‘UL’ and ‘carrier’ in this FG do not refer to ‘SUL’. NR PUCCH is sent on a carrier with SCS not smaller than SCS of any DL carriers corresponding to the NR PUCCH group. The case with PUCCH on UL carrier with different numerologies within SCG is not supported for NR-DC. | Optional with capability signalling |
|  | 6-10 | Cross carrier scheduling for the same numerology | Cross carrier scheduling for the same numerology with CIF where numerologies for scheduling cell and scheduled cell are same | 6-5, 6-6 | crossCarrierScheduling-SameSCS | BandNR | n/a | n/a |  |  |
|  | 6-10a | Cross carrier scheduling for different numerologies | Cross carrier scheduling for the different numerologies with CIF where numerologies for scheduling cell and scheduled cell are different | 6-10 | crossCarrierScheduling-OtherSCS | FeatureSetDownlink FeatureSetUplink | n/a | n/a | This is not supported in Rel-15 |  |
|  | 6-11 | Number of supported TAGs | Need of multiple capability question about the resolution here |  | supportedNumberTAG | CA-ParametersNR | n/a | n/a | This feature group is applied to NR-NR CA and EN-DC. For EN-DC, the feature group indicates number of TAGs only for NR CG. The number of TAGs for the LTE MCG is signalled by existing LTE TAG capability signalling | {1, 2, 3, 4} |
|  | 6-12 | Support 2 simultaneous UL transmissions for problematic cases | Support 2 simultaneous UL transmissions for problematic cases |  | singleUL-Transmission | MRDC-Parameters | n/a | n/a | This is a UE feature for LTE for a LTE/NR dual connectivity UE | Optional with capability signalling |
|  | 6-13 | Case 1 Single Tx UL LTE-NR DC | 1) Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) HARQ subframe offset |  | tdm-Pattern NOTE: This capability bit also indicates support of the feature 8-2, i.e. Operation A with single UL Tx case 1. | MRDC-Parameters | Yes | Yes | This is a UE feature for LTE for a LTE/NR dual connectivity UE | Mandatory with capability signalling conditional on the UE not supporting simultaneous dual-Tx operation in the band combination; optional if the UE supports simultaneous dual-Tx operation in the band combination |
|  | 6-16 | Supplemental uplink | 1) RACH, PUSCH, PUCCH, SRS operations in a band combination including SUL 2) Supplemental uplink with same numerology between SUL and non SUL carriers | 6-15 | supportedBandCombinationList | RF-Parameters | n/a | n/a | This is conditioned on the support of SUL band combination(s). | Optional with capability signalling |
|  | 6-17 | Supplemental uplink with different numerologies between SUL and non SUL carriers | Different numerologies between SUL and non SUL | 6-16 | supportedBandCombinationList | RF-Parameters | n/a | n/a | This is conditioned on the support of SUL band combination(s). | Mandatory with capability signalling |
|  | 6-18 | Supplemental uplink with dynamic switch | DCI based selection of PUSCH carrier | 6-16 | dynamicSwitchSUL | FeatureSetUplink | n/a | n/a | his is conditioned on the support of SUL band combination(s). | Optional with capability signalling |
|  | 6-19 | Simultaneous transmission of SRS on an SUL/non-SUL carrier and PUSCH/PUCCH/SRS on the other UL carrier in the same cell | Simultaneous transmission of SRS on an SUL/non-SUL carrier and PUSCH/PUCCH/SRS on the other UL carrier in the same cell | 6-16 | simultaneousTxSUL-NonSUL | FeatureSetUplink | n/a | n/a |  | Optional with capability signalling |
|  | 6-21 | DL search space sharing for CA | DL search space sharing for CA | 6-10 or 6-10a | searchSpaceSharingCA-DL | FeatureSetDownlink | n/a | n/a |  | Optional with capability signalling |
|  | 6-22 | UL search space sharing for CA | UL search space sharing for CA | 6-10 or 6-10a | searchSpaceSharingCA-UL | FeatureSetUplink | n/a | n/a |  | Optional with capability signalling |
|  | 6-23 | Incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band EN-DC, intra-band CA, and FDM based ULSUP | Incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band EN-DC, intra-band CA, and FDM based ULSUP |  | pa-PhaseDiscontinuityImpacts | FeatureSetUplink-v1540 | n/a | n/a | See LS (R1-1809992) | Optional with capability signalling |
|  | 6-24 | Applying the same UL timing between NR and LTE | Applying the same UL timing between NR and LTE for dynamic power sharing capable UE operating in intra-band contiguous synchronous EN-DC | 8-1 | ul-TimingAlignmentEUTRA-NR | MRDC-Parameters | n/a | n/a | UEs that set this bit to 0 should be able to operate with a timing difference up to applicable MTTD requirements when operating in a synchronous intra-band contiguous EN-DC network. | Optional with capability signalling |
|  | 6-25 | Support of synchronous NR-NR DC operation only wherein MCG is only in FR1 and SCG is only in FR2 | Support of synchronous NR-NR DC operation only wherein MCG is only in FR1 and SCG is only in FR2 |  | ca-ParametersNRDC | BandCombination-v1560 | n/a | n/a | This is conditioned on the support of DC band combination(s). UE reports a set of supported band partitionings corresponding to MCG in FR1 and to SCG in FR2. | Optional with capability signalling |
|  | 6-25a | PDCCH blind detection capability for MCG and for SCG in synchronous NR-NR DC | RRC parameters pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE for optional new UE capability signalling that informs the maximum values for pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG, respectively | 6-5, 6-25 | pdcch-BlindDetectionMCG-UE pdcch-BlindDetectionSCG-UE | Phy-ParametersFRX-Diff | No | Yes | pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE are per UE capability signalling. The value range of pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE is - [1, …, pdcch-BlindDetectionCA-1] and pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= pdcch-BlindDetectionCA if the UE reports pdcch-BlindDetectionCA, and - [1, 2, 3] and pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= the maximum number of DL serving cells over CGs that UE can support if the UE does not report pdcch-BlindDetectionCA. If the UE does not report pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE, - pdcch-BlindDetectionCA for NR-CA is re-used as the UE capability signalling for NR-DC to determine BD/CCE limit across serving cells over CGs if the UE reports pdcch-BlindDetectionCA, and - the number of configured DL serving cells over CGs is used to determine BD/CCE limit across serving cells over CGs if the UE does not report pdcch-BlindDetectionCA. If the UE reports pdcch-BlindDetectionMCG-UE or pdcch-BlindDetectionSCG-UE, both of them are reported (i.e., not either of them). | Optional with capability signalling |
| 7. Channel coding | 7-1 | Channel coding | 1) LDPC encoding and associated functions for data on DL and UL 2) Polar encoding and associated functions for PBCH, DCI, and UCI 3) Coding for very small blocks |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
| 8. UL TPC | 8-1 | Dynamic power sharing for LTE-NR DC | When total transmission power exceeds Pcmax, UE scales NR transmission power. | EN-DC | dynamicPowerSharing | MRDC-Parameters | n/a | n/a | RP-172833 | Mandatory with capability signalling |
|  | 8-2 | Operation A with single UL Tx case 1 | Operation A with single UL Tx case 1 | EN-DC | tdm-Pattern NOTE: This capability bit also indicates support of the feature 6-13, i.e. Case 1 Single Tx UL LTE-NR DC. | MRDC-Parameters | Yes | Yes | RP-172833 | Mandatory with capability signalling conditioned that UE does not support dynamic power sharing, i.e., UE indicate “0” as non-support for 8-1, optional for UEs supporting dynamic power sharing |
|  | 8-3 | Basic power control operation | 1) Accumulated power control mode for closed loop 2) 1 TPC command loop for PUSCH, PUCCH respectively 3) One or multiple DL RS configured for pathloss estimation 4) One or multiple p0-alpha values configured for open loop PC 5) PUSCH power control 6) PUCCH power control 7) PRACH power control 8) SRS power control 9) PHR |  | n/a | n/a | No | No |  | Mandatory without capability signalling |
|  | 8-4 | TPC-PUSCH-RNTI | Specific group DCI message for TPC commands for PUSCH |  | tpc-PUSCH-RNTI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 8-5 | TPC-PUCCH-RNTI | Specific group DCI message for TPC commands for PUCCH |  | tpc-PUCCH-RNTI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 8-6 | TPC-SRS-RNTI | Specific group DCI message for TPC commands for SRS |  | tpc-SRS-RNTI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 8-7 | Absolute TPC command mode | Absolute TPC command mode |  | absoluteTPC-Command | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 8-8 | UL power control with 2 PUSCH closed loops | Two different TPC loops |  | twoDifferentTPC-Loop-PUSCH | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Mandatory with capability signalling |
|  | 8-9 | UL power control with 2 PUCCH closed loops | Two different TPC loops |  | twoDifferentTPC-Loop-PUCCH | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Mandatory with capability signalling |