3GPP TSG RAN WG1 #100-e	R1-2000973

February 24th – March 6th, 2020

Agenda item:	7.2.6.1

Source: 	Qualcomm Incorporated

Title: 	Remaining issues on CSI enhancement for MU-MIMO support

Document for:	Discussion/Decision

# Introduction

In this contribution, we raise the issues of confusing UE behavior in CSI measurement and propose solutions to resolve these confusing issues.

# Wideband PMI for Enhanced Type II and Enhanced Type II port-selection

In Rel-15, Type II and Type II port-selection codebooks only support subband-frequency granularity, and it is an error case if the UE is configured with Type II CSI report with wideband-frequency granularity (As specified in table 6.3.2.1.2-4/5 of [1], Type II CSI is reported if the UE is configured with higher layer parameter pmi-FormatIndicator=subbandPMI). However, it is yet to clear that whether Enhanced Type II and Enhanced Type II port-selection codebooks support wideband granularity in Rel-16.

In our view, the motivation of Rel-16 Type II is to achieve overhead reduction compared to Rel-15 Type II CSI, as the payload of Rel-15 Type II scales nearly with the number of subbands. Hence, it is nature for Rel-16 Type II to support only subband PMI. Besides, Type II CSI provides high-resolution CSI report, wideband CSI can be obtained via Type I CSI. Based on the discussion, we observe and propose

Proposal 1: Rel-16 Enhanced Type II and Enhanced Type II port-selection codebook only support subband PMI, i.e., pmi-FormatIndicator=subbandPMI.

The corresponding text proposal is as follows.

---------------------------------------- Start of text proposal to Section 6.3.2.1.2 in TS 38.212 ---------------------------------------

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

Table 6.3.2.1.2-5A: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType=typeII-r16 or typeII-PortSelection-r16

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Tables 6.3.2.1.2-1A/2A, if reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-1A/2A: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ and (KNZ.TOT2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,(KNZ,TOT/2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυ-KNZ,TOT/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on function Pril,i,f defined in clause 5.2.3 of TS38.214, if pmi-FormatIndicator=subbandPMI and if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-1A/2A: KNZ,TOT/2×3 lowest priority bits of i2,4,l:l=1,…,υ, KNZ,TOT/2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ,TOT/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on function Pril,i,f defined in clause 5.2.3 of TS38.214, if pmi-FormatIndicator=subbandPMI and if reported |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

-------------------------------------------------------------- End of text proposal --------------------------------------------------------------

# Size of intermediate set when N3>19

When the total number of PMI subbands is greater than 19, i.e., N3>19, a two-step FD bases selection was agreed in RAN1 #97 [2]. The first step is to select an intermediate set which is common to all layers, the size of the intermediate set is agreed as follows:

- For N3>19, IntS is window-based and fully parameterized with Minitial, indicating that the intermediate set consists of FD bases mod(Minitial + n, N3), n=0,1, …,

  - The value  where  is higher-layer configured from two possible values

In RAN1 #98, it was further agreed that α is fixed to 2, but the M value above is yet to be defined. There might be two understandings:

- Alt1: the M value above is fixed to the value in low-rank (i.e., M1 and M2)

- Alt2: the M value above depends on the reported rank (i.e., Mν).

The current spec [1,3] follow Alt2, but it may result in a smaller window size for high rank than the low rank.

- For instance, for rank-2, we have M2=10 and N3'=20 for rank-2, it means that total 20 FD bases selected across layers should fit in a window with size N3'=20. However, for rank-4, we may have M4=5 and N3'=10, there are still total 20 FD bases selected across layers, but window size N3'=10 seems too small.

In our view, considering that the total number of FD bases are comparable for low rank and high rank, and the FD bases selection may vary across different layers, so Alt1 is preferred as it yields a constant window size for both low and high rank.

Another benefit of rank-common intermediate set size is lower implementation complexity. With Alt2, the UE has to recalculate the intermediate set to perform FD bases selection for RI={3,4}, such operation is unnecessary and may lead to performance loss compared to Alt1 as the size of intermediate set resulted by Alt2 is smaller than the size of intermediate set resulted by Alt1.

Based on the discussion, we observe and propose.

Proposal 2: When N3>19, the size of the intermediate set is give by N3'=2M1 for RI={1,2,3,4}, where M1 is the number of FD bases selected for RI={1,2}.

The corresponding text proposal is as follows

---------------------------------------- Start of text proposal to Section 5.2.2.2.5 in TS 38.214 ---------------------------------------

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

Mυ=pυN3R vectors, y0,lf,y1,lf,…,yN3-1,lfT, f=0,1,…,Mυ-1, are identified by Minitial (for N3>19) and n3,l l=1,…, υ where

Minitial∈-2M1υ+1,-2M1υ+2,…,0

n3,l=n3,l(0),…,n3,l(Mυ-1)

n3,lf∈0,1,…,N3-1

which are indicated by means of the indices i1,5 (for N3>19) and i1,6,l l=1,…, υ, where

i1,5∈0,1,…,2M1υ-1

i1,6,l∈0,1,…,N3-1Mυ-1-1N3≤190,1,…,2M1υ-1Mυ-1-1N3>19.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

For all values of N3, n3,l(0)=0 for l=1,…,υ. The nonzero elements of n3,l, identified by  n3,l(1), …, n3,lMυ-1, are found from i1,6,l (l=1,…,υ), for N3≤19, and from i1,6,l (l=1,…,υ) and Minitial, for N3>19, using  Cx,y as defined in 5.2.2.2.3 and the algorithm:

s0=0

for f=1,…,Mυ-1

Find the largest x*∈Mυ-1-f,…,N3-1-f in Table 5.2.2.2.5-4 such that

i1,6,l -sf-1≥Cx*,Mυ-f

ef=Cx*,Mυ-f

sf=sf-1+ef

if N3≤19

n3,l(f)=N3-1-x*

else

nl(f)=2M1υ-1-x*

if nl(f)≤Minitial+2M1υ-1

n3,l(f)=nl(f)

else

n3,l(f)=nl(f)+(N3-2M1υ)

end if

end if

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

When n3,l and Minitial are known, i1,5 and i1,6,l (l=1,…,υ) are found as follows:

-	If  N3≤19, i1,5=0 and is not reported. i1,6,l=f=1Mυ-1CN3-1-n3,lf,Mυ-f, where Cx,y is given in Table 5.2.2.2.5-4 and where the indices f=1,…,Mυ-1 are assigned such that n3,l(f) increases as f increases.

-	If  N3>19, Minitial is indicated by i1,5, which is reported and given by

i1,5=MinitialMinitial=0 Minitial+2M1υMinitial<0

Only the nonzero indices n3,l(f)∈IntS, where IntS={Minitial+i mod N3, i=0,1,…,2M1υ-1}, are reported, where the indices f=1,…,Mυ-1 are assigned such that n3,l(f) increases as f increases. Let

nl(f)=n3,lf n3,lf≤Minitial+2M1υ-1 n3,lf-(N3-2Mυ1)n3,lf>Minitial+N3- 1,

then i1,6,l=f=1Mυ-1C2Mυ1-1-nlf,Mυ-f, where Cx,y is given in Table 5.2.2.2.5-4.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

-------------------------------------------------------------- End of text proposal --------------------------------------------------------------

---------------------------------------- Start of text proposal to Section 6.3.2.1.2 in TS 38.212 ------------------------------------------

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

Table 6.3.2.1.2-1A: PMI of codebookType= typeII-r16

|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i2,3,1 | i2,3,2 | i2,3,3 | i2,3,4 | i1,5 | i1,6,1 | i1,6,2 | i1,6,3 | i1,6,4 | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ | {i1,7,l}l=1,…,υ |
| Rank=1 N3>19 | 4 | N/A | N/A | N/A | log2(2M1) | log22M1-1M1-1 | N/A | N/A | N/A | 3(KNZ,TOT-1) | 4(KNZ,TOT-1) | 2LM1 |
| Rank=2 N3>19 | 4 | 4 | N/A | N/A | log2(2M12) | log22M12-1M2-1 | log22M12-1M2-1 | N/A | N/A | 3(KNZ,TOT-2) | 4(KNZ,TOT-2) | 4LM2 |
| Rank=3 N3>19 | 4 | 4 | 4 | N/A | log2(2M13) | log22M13-1M3-1 | log22M13-1M3-1 | log22M13-1M3-1 | N/A | 3(KNZ,TOT-3) | 4(KNZ,TOT-3) | 6LM3 |
| Rank=4 N3>19 | 4 | 4 | 4 | 4 | log2(2M14) | log22M14-1M4-1 | log22M14-1M4-1 | log22M14-1M4-1 | log22M14-1M4-1 | 3(KNZ,TOT-4) | 4(KNZ,TOT-4) | 8LM4 |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

Table 6.3.2.1.2-2A: PMI of codebookType= typeII-PortSelection-r16

|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i2,3,1 | i2,3,2 | i2,3,3 | i2,3,4 | i1,5 | i1,6,1 | i1,6,2 | i1,6,3 | i1,6,4 | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ | {i1,7,l}l=1,…,υ |
| Rank=1 N3>19 | 4 | N/A | N/A | N/A | log2(2M1) | log22M1-1M1-1 | N/A | N/A | N/A | 3(KNZ,TOT-1) | 4(KNZ,TOT-1) | 2LM1 |
| Rank=2 N3>19 | 4 | 4 | N/A | N/A | log2(2M12) | log22M12-1M2-1 | log22M12-1M2-1 | N/A | N/A | 3(KNZ,TOT-2) | 4(KNZ,TOT-2) | 4LM2 |
| Rank=3 N3>19 | 4 | 4 | 4 | N/A | log2(2M13) | log22M13-1M3-1 | log22M13-1M3-1 | log22M31-1M3-1 | N/A | 3(KNZ,TOT-3) | 4(KNZ,TOT-3) | 6LM3 |
| Rank=4 N3>19 | 4 | 4 | 4 | 4 | log2(2M14) | log22M14-1M4-1 | log22M14-1M4-1 | log22M14-1M4-1 | log22M14-1M4-1 | 3(KNZ,TOT-4) | 4(KNZ,TOT-4) | 8LM4 |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

-------------------------------------------------------------- End of text proposal --------------------------------------------------------------

# Conclusion

In this contribution, we discuss the remaining UE capability issues related to Type II CSI enhancement. Based on the discussion, we propose

Proposal 1: Rel-16 Enhanced Type II and Enhanced Type II port-selection codebook only support subband PMI, i.e., pmi-FormatIndicator=subbandPMI.

Proposal 2: When N3>19, the size of the intermediate set is give by N3'=2M1 for RI={1,2,3,4}, where M1 is the number of FD bases selected for RI={1,2}.

We propose adopting text proposals in Sections 2 and 3.

# References

- TS38.212 v16.0.0

- Chairman’s notes RAN1#97 Final

- TS38.214 v16.0.0