3GPP TSG-RAN WG1 #100	R1-2000980

e-Meeting, February 24th – March 6th, 2020

Agenda item:	7.2.10.1

Source: 	Qualcomm Incorporated

Title: 	Remaining issues on uplink power control for NR-DC

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss remaining issues on uplink power control for NR-DC.

# T_offset for dynamic power-sharing

At the RAN1#99bis meeting, introduction of T_offset was agreed for dynamic power-sharing [1]. However, the exact value of the T_offset is not yet addressed.

Agreements: - For NR-DC dynamic power sharing, to compute the transmit power for SCG UL transmission starting at time T0, - UE checks for PDCCH(s) received before time T0-T_offset that trigger an overlapping MCG UL transmission, and - If such PDCCH(s) are detected, UE sets it’s transmit power in SCG (pwr_SCG) such that pwr_SCG <= min{PSCG, Ptotal – MCG tx power} where ‘MCG tx power’ is the actual transmission power of MCG - Otherwise, pwr_SCG <= Ptotal; - UE does not expect to be scheduled by PDCCH(s) received on MCG after T0-[T_offset] that trigger(s) MCG UL transmission(s) that overlaps with the SCG transmission. - (working assumption) No new RRC signaling is introduced for T_offset: - Alt.1: T_offset <= T_proc,2 - Alt.2: T_offset <= 2*T_proc,2 - Alt.3: T_offset reasonbly larger than Alt 1. & Alt 2 but <=4ms - To be addressed in the CR stage

For a SCG transmission(s) starting at time T0, the UE needs to process following during the T_offset from the timing T0 – T_offset:

- Compute the sum power of MCG transmission(s) that overlaps with the SCG transmission(s) starting at time T0 and then move forward to the step 2

- Allocate power(s) among SCG transmission(s) starting at time T0 similar to UL-CA using the available power for the SCG transmission(s) identified in the step 1

Fig.1	Dynamic power-sharing with look-ahead.

T_offset should be long enough to process the cascaded step 1 and step 2 (i.e., step 2 cannot be executed until step 1 is completed). In principle, it is necessary to guarantee the minimum UE processing time for the minimum UE processing time for the SCG transmission(s) starting at time T0 and for any of the MCG transmission(s) overlapping with the SCG transmission starting at time T0.

As a nature of dual connectivity, at each MCG scheduling opportunity, MgNB does not know how exactly SgNB schedules uplink transmissions on SCG cells to the UE. Because of this, what MgNB can assume is the worst case of all the possible minimum UE processing time for SCG, meaning that MgNB cannot schedule MCG uplink transmissions overlapping a SCG transmission before the maximum possible UE processing time among all the possible UE processing time for a SCG transmission starting at time T0. As a simple example, if the SCG transmission starting at time T0 could be either a PUSCH without A-CSI or a PUSCH with A-CSI, then the MCG DCI scheduling MCG uplink transmission overlapping with the SCG transmission starting at time T0 cannot be after T0-maxTproc,2, Tproc,CSI=T0-Tproc,CSI . Therefore, requiring UE to be able to process dynamic power-sharing with T_offset shorter than the maximum possible UE processing time for a SCG transmission does not provide an insentive.

MgNB can fully understand exact minimum UE processing time for MCG uplink transmission(s). However, actual UE processing time specified so far for a CG depends on various dynamic factors such as DCI indication and UCI multiplexing. For example, for Tproc,2=maxN2+d2,12048+144∙κ∙2-μ∙Tc, d2,2,  d2,1 is 0 or 1 depending on whether the first symbol consists of DMRS only, d2,2 is 0 or BWP switching delay defined in TS38.133 depending on whether the DCI indicates BWP switching. When a UE has UCI feedback on the transmission occasion starting at time T0, the minimum processing time is Tproc,CSI, Tproc,releasemux, Tproc,2mux, or Tproc,CSImux. Making T_offset be variable based on these dynamic factors requires UE to adjust processing timeline (the step 1 described earlier) dynamically. Same as for SCG cells, T_offset should not be shorter than the maximum possible UE processing time among all the possible UE processing time for the MCG transmission that overlaps with the SCG transmission starting at time T0.

In summary, T_offset should be not smaller than the maxTproc, MCGmax, Tproc, SCGmax, where Tproc, MCGmax denotes the maximum possible minimum UE processing time among all the possible UE processing time for a MCG transmission that overlaps with the SCG transmission starting at time T0, and Tproc, SCGmax denotes the maximum possible minimum UE processing time among all the possible UE processing time for a SCG transmission starting at time T0.

With the above, it is understood that UE processing capability 2 is not applicable to derive T_offset; in any case when a UE is configured with UE processing capability 2, the maximum possible minimum UE processing time corresponds to UE processing capability 1.

Proposal 1: T_offset = maxTproc, MCGmax,  Tproc, SCGmax, where:

- Tproc, MCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and/or Tproc,CSImux as specified in TS38.213 and TS38.214 based on the configurations for the MCG.

- Tproc, SCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and/or Tproc,CSImux as specified in TS38.213 and TS38.214 based on the configurations for the SCG.

# Handling of the capability for look-ahead operation

With the sufficiently duration for T_offset proposed in Section 2, a UE should be able to determine transmission power for each transmission occasion dynamically. Besides, by the MCG scheduling restriction from T_offset prior to the SCG transmission, a UE is not required to handle possible MCG transmission starting from a middle of a SCG transmission occasion. Taking these into account, now there is less motivation to introduce the separate optional UE capability to indicate the support of ’look-ahead’ operation.

Proposal 2: Merge the capability for look-ahead opration to the capability for dynamic power-sharing.

# Handling of other than a DCI scheduling uplink transmission

For dynamic power-sharing, current spec describes such that the UE determines max transmission power on the SCG as minPSCG,PTotalNR-DC- PMCGactual, if the UE determines transmissions on the MCG with the transmission power PMCGactual. Then, the UE does not expect to have transmissions on the MCG that are scheduled by DCI after the T0 – T_offset. However, this does not address following:

- Semi-statically configured uplink transmissions, e.g., P-/SP-CSI, P-/SP-SRS, CG-PUSCH

These transmissions are not scheduled by DCI. However, the transmissions according to higher layer configurations should be taken into account for determining the value of PMCGactual.

- Group-TPC command by DCI format 2_2, or 2_3 for MCG transmission

These DCI formats do not schedule uplink transmissions but would impact on the value of PMCGactual. Therefore, these should be restricted to be before T0 – T_offset.

- Uplink transmission cancellation by DCI format 2_4 for MCG transmission

This DCI format does not schedule uplink transmissions but would impact on the value of PMCGactual. However, the cancellation does not increase the MCG transmission power. Therfore, it is beneficial not to prohibit reception of DCI format 2_4 that cancells MCG transmission after T0 – T_offset, with allowing UE not to take into account the DCI 2_4 for PMCGactual determination if it is after T0 – T_offset.

- Slot format indication by DCI format 2_0 for MCG transmission

This DCI format does not schedule uplink transmissions but would impact on the value of PMCGactual, since it may cancel or trigger semi-static uplink transmissions. Therefore, the DCI format 2_0 should be restricted to be before T0 – T_offset.

- Uplink transmission skip for MCG dynamic grant PUSCH/configured grant PUSCH due to 5.4.3.1.3 of TS38.321

For configured grant PUSCH transmission occasion, the UE may or may not transmit the PUSCH. For dynamic grant PUSCH, the UE may skip the transmission if some conditions are met. These are specified in 5.4.3.1.3 of TS38.321. Since the transmission skip is a UE internal determination, it is not appropriate to specify the timeline. Although the skip might happen after T0 – T_offset, the UE should be allowed to keep the determined transmission power at T0 – T_offset even if the transmission is skipped according to 5.4.3.1.3 of TS38.321.

Proposal 3: For a MCG sum power determination that overlaps with a SCG transmission starting at time T0, following should be clear.

- Semi-statically configured uplink transmissions are taken into account for PMCGactual determination.

- DCI formats that would impact on PMCGactual, other than DCI format 2_4, are only before T0 – T_offset.

- UE is not required to take into account DCI format 2_4 and UL transmission skip according to 5.4.3.1.3 of TS38.321 after T0 – T_offset for PMCGactual determination.

# Clarification on synchronous and asynchronous NR-DC operations

RAN1 has agreed that semi-static power-sharing mode 1 is applicable to both synchronous and asynchronous NR-DC operations, while mode 2 is applicable only to synchronous NR-DC operation. Considering that dynamic power-sharing is more advanced feature than the semi-static power-shaing mode 2, it should also only be synchronous NR-DC operation.

Proposal 4: Dynamic power-sharing is supported only for synchronous NR-DC operation.

# Clarifications on PMCG and PSCG setting for semi-static power-sharing

For semi-static power-shaing, it should be clarified that a UE is not required to handle the configurations of PMCG + PSCG higher than the total avaliable UE transmission power.

Proposal 5: For semi-static power-sharing, PMCG and PSCG are configured such that  PMCG+PSCG≤PTotalNR-DC where PMCG and PSCG are the linear values of  PMCG or PSCG, respectively, and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation.

# Text proposal

In this section, text proposal for TS38.213 Section 7.6.2 to address proposals 1 – 5 is provided.

### 7.6.2	NR-DC

If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5.

If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2 and with an inter-CG power sharing mode by NR-DC-PC-mode for FR1 and/or by NR-DC-PC-mode for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range.

The UE expects to be provided NR-DC-PC-mode = Semi-static-mode2 or dynamic only for synchronous NR-DC operation [10, TS 38.133].

If a UE is provided NR-DC-PC-mode = Semi-static-mode1, the UE determines a transmission power for the MCG or for the SCG as described in Clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively. The UE does not expect PMCG+PSCG>PTotalNR-DC where PMCG and PSCG are the linear values of  PMCG or PSCG, respectively, and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 as defined in [8-3, TS 38.101-3]

If a UE is provided NR-DC-PC-mode = Semi-static-mode2

-	if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission in slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG in slot i2 as described in Clauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power

-	otherwise, the UE determines a transmission power for the MCG or the SCG, as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively

-	The UE does not expect PMCG+PSCG>PTotalNR-DC.

The UE expects to be provided NR-DC-PC-mode = Semi-static-mode2 only for synchronous NR-DC operation [10, TS 38.133].

i1i2PMCGactuali1+PSCGactuali2>PTotalNR-DCi2i2PMCGactuali1+PSCGactuali2≤PTotalNR-DCi2PMCGactuali1PSCGactuali2i1i2PMCGPSCGPTotalNR-DC

i1i2i1i2PMCGPSCG

If a UE

-	is provided NR-DC-PC-mode = Dynamic, and

-	indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, and

-	overlap with the transmission occasion on the SCG

the UE determines a maximum transmission power on the SCG at the beginning of the transmission occasion on the SCG as

-	minPSCG,PTotalNR-DC- PMCGactual, PMCGactualwhere PMCGactual is the transmission power for the MCG transmission that overlaps with the transmission occasion on the SCG, determined based on

-	the higher layers, and/or

-	the DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG

PTotalNR-DC

The UE does not expect to receive DCI formats other than DCI format 2_4 in PDCCH receptions on the MCG

-	with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG, that

-	may change the value of the PMCGactual for the MCG transmission that overlaps with the transmission occasion on the SCG

where Toffset is given by

-	maxTproc, MCGmax,  Tproc, SCGmax, where Tproc,MCGmax and Tproc, SCGmax are the maximum of any possible timeline conditions from a PDCCH reception to the start of the corresponding uplink transmission specified in 5.4 and 6.4 of [TS 38.214] and in 9.2.5 of [TS 38.213] assuming the UE processing capability 1 over any of the serving cells in the MCG and SCG, respectively.

# Conclusion

In this contribution, we discussed remaining issues for NR-DC power-control, and proposed the following.

Proposal 1: T_offset = maxTproc, MCGmax,  Tproc, SCGmax, where:

- Tproc, MCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and/or Tproc,CSImux as specified in TS38.213 and TS38.214 based on the configurations for the MCG.

- Tproc, SCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and/or Tproc,CSImux as specified in TS38.213 and TS38.214 based on the configurations for the SCG.

Proposal 2: Merge the capability for look-ahead opration to the capability for dynamic power-sharing.

Proposal 3: For a MCG sum power determination that overlaps with a SCG transmission starting at time T0, following should be clear.

- Semi-statically configured uplink transmissions are taken into account for PMCGactual determination.

- DCI formats that would impact on PMCGactual, other than DCI format 2_4, are only before T0 – T_offset.

- UE is not required to take into account DCI format 2_4 and UL transmission skip according to 5.4.3.1.3 of TS38.321 after T0 – T_offset for PMCGactual determination.

Proposal 4: Dynamic power-sharing is supported only for synchronous NR-DC operation.

Proposal 5: For semi-static power-sharing, PMCG and PSCG are configured such that  PMCG+PSCG≤PTotalNR-DC where PMCG and PSCG are the linear values of  PMCG or PSCG, respectively, and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation.

The corresponding text proposal is available in Section 7.

# References

[1] 3GPP RAN1#99bis Chairman’s note.