3GPP TSG RAN WG1 Meeting #100	R1-2000988

e-Meeting, February 24th – March 6th, 2020

Agenda item:		7.2.6.2

Source:		Nokia, Nokia Shanghai Bell

Title:	Maintenance of Rel-16 Multi-TRP operation

Document for:		Discussion and Decision

# 1	Introduction

In this contribution, we summarize our input to the maintenance discussions on multi-TRP operation supported in Rel-16.

# 2	Discussion

Suggested changes on the CRs on 38.214, 38.213, and 38.212 are highlighted as follows,

### Correction 1: Selection of URLLC schemes

In Rel-16, multi-TRP techniques for URLLC supports different schemes. Exact schemes to be supported in Rel-16 are configured and/or indicated based on the RRC configuration and parameters used in the DCI. RAN1 #99 agreed on the following to further clarification of the supported schemes.

Agreement

Following TCI state and joint schemes are supported

|  | TCI states | CDM groups | URLLCRepNum | URLLCSchemeEnabler | UE Behavior |
| --- | --- | --- | --- | --- | --- |
| 0 (in spec draft) | 1 | >=1 | Not applicable | Not applicable | Rel 15 |
| A (one scheme) | 1 | 1 | Condition 1 | Configured or not configured | "Scheme 4" with repetition from the same TRP Limitations agreed for Scheme 4 apply |
| A’ (one scheme) | 1 | >=1 | Condition 2 | Not configured | Rel 15 |
| B (in spec draft) | 2 | 1 | Condition 1 | Not configured | Scheme 4 |
| C (in spec draft) | 2 | 2 | Condition 2 | Not configured | 1a/NCJT |
| E (in spec draft) | 2 | 2 | Condition 4 | Not configured | 1a/NCJT |
| F (in spec draft) | 2 | 1 | Condition 4 | Configured | Scheme 2a/2b/3 |
| D’’ (one scheme) | 2 | 2 | Condition 4 | Configured | 1a/NCJT |
| G’ (one scheme) | 1 | >=1 | Condition 2 | Configured | Rel 15 |
| G (one scheme) | 1 | >=1 | Condition 4 | Configured | Rel 15 |

Note:

- Condition 1: indicates at least one entry in pdsch-TimeDomainAllocationList containing URLLCRepNum (>1) in TDRA by DCI

- Condition 2: indicates one entry in pdsch-TimeDomainAllocationList having no URLLCRepNum by DCI, but at least one entry having URLLCRepNum

- Condition 4: None of entry in TDRA contains URLLCRepNum

However, there are certain questions on dynamic switching between Schemes 2a/2b/3 and Scheme 4 which was raised during the CR review stages, and further clarifications may be useful to understand the options.

- Yellow: F mentioned in the Table indicated ‘in spec draft”. However, that is not fully correct as there was nothing about condition 4 in the spec draft (CR after RAN1 #98).  We think it is reasonable to support switching between Schemes 2a/2b/3 and Scheme 4 and in some company views it is already supported in the spec draft. From our point of view, it is reasonable to support dynamic switching between these schemes and it would be good to clarify this further in the specs.

Proposal 1: Adopt the following text proposals to clarify the URLLC schemes in 38.214.

- Allowing dynamic switching between Scheme 2a/2b/3 and Scheme 4 (Red).

- Moving the Scheme 4 description prior to the Scheme 2a/2b/3 (Green).

TP to TS 38.214, Sec. 5.1: ## <Unchanged text is omitted> When a UE is configured by the higher layer parameter PDSCH-config that indicates at least one entry in pdsch-TimeDomainAllocationList containing RepNumR16 in PDSCH-TimeDomainResourceAllocation, the UE may expect to be indicated with one or two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' together with the DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNum16 in PDSCH-TimeDomainResourceAllocation and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”. - When two TCI states are indicated in a DCI with ‘Transmission Configuration Indication’ field, the UE may expect to receive multiple slot level PDSCH transmission occasions of the same TB with two TCI states used across multiple PDSCH transmission occasions as defined in Subclause 5.1.2.1. - When one TCI state is indicated in a DCI with ‘Transmission Configuration Indication’ field, the UE may expect to receive multiple slot level PDSCH transmission occasions of the same TB with one TCI state used across multiple PDSCH transmission occasions as defined in Subclause 5.1.2.1. When a UE is not indicated with a DCI that DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation, When a and it UE is configured by higher layer parameter RepSchemeEnabler set to one of ‘FDMSchemeA’, ‘FDMSchemeB’, ‘TDMSchemeA’, if the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, - When two TCI states are indicated in a DCI and the UE is set to ‘FDMSchemeA’, the UE shall receive a single PDSCH transmission occasion of the TB with each TCI state associated to a non-overlapping frequency domain resource allocation as described in Subclause 5.1.2.3. - When two TCI states are indicated in a DCI and the UE is set to ‘FDMSchemeB’, the UE shall receive two PDSCH transmission occasions of the same TB with each TCI state associated to a PDSCH transmission occasion which has non-overlapping frequency domain resource allocation with respect to the other PDSCH transmission occasion as described in Subclause 5.1.2.3. - When two TCI states are indicated in a DCI and the UE is set to ‘TDMSchemeA’, the UE shall receive two PDSCH transmission occasions of the same TB with each TCI state associated to a PDSCH transmission occasion which has non-overlapping time domain resource allocation with respect to the other PDSCH transmission occasion and both PDSCH transmission occasions shall be received within a given slot as described in Subclause 5.1.2.1. When a UE is configured by the higher layer parameter PDSCH-config that indicates at least one entry in pdsch-TimeDomainAllocationList containing RepNumR16 in PDSCH-TimeDomainResourceAllocation, the UE may expect to be indicated with one or two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' together with the DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNum16 in PDSCH-TimeDomainResourceAllocation and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”. - When two TCI states are indicated in a DCI with ‘Transmission Configuration Indication’ field, the UE may expect to receive multiple slot level PDSCH transmission occasions of the same TB with two TCI states used across multiple PDSCH transmission occasions as defined in Subclause 5.1.2.1. - When one TCI state is indicated in a DCI with ‘Transmission Configuration Indication’ field, the UE may expect to receive multiple slot level PDSCH transmission occasions of the same TB with one TCI state used across multiple PDSCH transmission occasions as defined in Subclause 5.1.2.1. When a UE is not indicated with a DCI that DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation, and it is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within two CDM group in the DCI field “Antenna Port(s)”, the UE may expect to receive a single PDSCH where the association between the DM-RS ports and the TCI states are as defined in Subclause 5.1.6.2. When a UE is not indicated with a DCI that DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation, and it is indicated with one TCI states in a codepoint of the DCI field 'Transmission Configuration Indication', the UE procedure for receiving the PDSCH upon detection of a PDCCH follows Subclause 5.1. ## <Unchanged text is omitted>

TP to TS 38.214, Sec. 5.1.2.1: ## <Unchanged text is omitted> When a UE is not indicated with a DCI that DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation, and it When a UE is configured by the higher layer parameter RepSchemeEnabler set to ‘TDMSchemeA’ and indicated DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, the number of PDSCH transmission occasions is derived by the number of TCI states indicated by the DCI field 'Transmission Configuration Indication' of the scheduling DCI. <Unchanged text is omitted>

TP to TS 38.214, Sec. 5.1.2.3: ## <Unchanged text is omitted> For a UE is not indicated with a DCI that DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation, and it For a UE configured by the higher layer parameter RepSchemeEnabler set to ‘FDMSchemeA’ or ‘FDMSchemeB’, and when the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, ## <Unchanged text is omitted> For a UE is not indicated with a DCI that DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation, and it For a UE configured by the higher layer parameter RepSchemeEnabler set to ‘FDMSchemeB’, and when the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, each PDSCH transmission occasion shall follow the Subclause 7.3.1 of [4, TS 38.211] with the mapping to resource elements determined by the assigned PRBs for corresponding TCI state of the PDSCH transmission occasion, and the UE shall only expect at most two code blocks per PDSCH transmission occasion when a single transmission layer is scheduled and a single code block per PDSCH transmission occasion when two transmission layers are scheduled. For two PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n=0, 1 are applied to the first and second TCI state, respectively. ## <Unchanged text is omitted>

TP to TS 38.214, Sec. 5.1.3.1: ## <Unchanged text is omitted> For a UE is not indicated with a DCI that DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation, and it For a UE configured with FDMSchemeB, and when the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, the determined modulation order of PDSCH transmission occasion associated with the first TCI state is applied to the PDSCH transmission occasion associated with the second TCI state. ## <Unchanged text is omitted>

TP to TS 38.214, Sec. 5.1.3.2: ## <Unchanged text is omitted> For a UE is not indicated with a DCI that DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation, and it For a UE configured with FDMSchemeB and indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, the TBS determination follows the steps 1-4 with the following modification in step 1: a UE determines the total number of REs allocated for PDSCH ()NRE) by NRE= NRE'* nPRB, where nPRB is the total number of allocated PRBs corresponding to the first TCI state. and the determined TBS of PDSCH transmission occasion associated with the first TCI state is also applied to the PDSCH transmission occasion associated with the second TCI state. ## <Unchanged text is omitted>

TP to TS 38.214, Sec. 5.1.5: When a UE is not indicated with a DCI that DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation, and it When a UE configured by the higher layer parameter RepSchemeEnabler set to ‘FDMSchemeA’ or ‘FDMSchemeB’, and the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, the UE shall receive a single PT-RS port which is associated with the lowest indexed DM-RS antenna port among the DM-RS antenna ports assigned for the PDSCH, a PT-RS frequency density is determined by the number of PRBs associated to each TCI state, and a PT-RS resource element mapping is associated to the allocated PRBs for each TCI state. ## <Unchanged text is omitted>

### Correction 2: Separate HARQ-ACK feedback

In Rel-16, multi-DCI based multi-TRP transmission can have both joint and separate feedback modes. If the RRC parameter ACKNACKFeedbackMode is not configured, the separate HARQ-ACK feedback should be still possible when the indicated PUCCH resources for two TRPs are different slots. However, this case is not captured in 38.213.

Proposal 2: Adopt the following text proposals to capture default scenario for separate HARQ-ACK feedback.

TP to TS 38.213, Sec. 9: < Unchanged text is omitted> If a UE - is not provided CORESETPoolIndex or is provided CORESETPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and - is provided CORESETPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and - is provided ACKNACKFeedbackMode = SeparateFeedback or ACKNACKFeedbackMode is not configured. the UE shall separately apply the procedures described in Subclauses 9.1 and 9.2.3 for reporting HARQ-ACK information associated with the first CORESETs on active DL BWP of the serving cells and for reporting HARQ-ACK information associated with the second CORESETs on active DL BWP of the serving cells. HARQ-ACK information reporting is associated with a CORESET through a reception of a PDCCH with a DCI format triggering the reporting of the HARQ-ACK information by the UE. ## <Unchanged text is omitted>

TP to TS 38.213, Sec. 9.2.5: < Unchanged text is omitted> A UE that - is not provided CORESETPoolIndex or is provided CORESETPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and - is provided CORESETPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and - is provided ACKNACKFeedbackMode = SeparateFeedback or ACKNACKFeedbackMode is not configured. does not expect a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the first CORESETs to overlap with a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the second CORESETs. <Unchanged text is omitted>

### Correction 3: RRC configuration of R

In Rel-16, the following agreements were made for a UE supporting multi-TRP transmission,

Agreement

The gNB can configure UE to use Rel-15 BD/CCE limits regardless of UE capability signalling on BD/CCE limit enhancement

- FFS: Whether RRC signalling is needed

Agreement

If a UE can support and report R>1 for M-DCI based M-TRP/panel transmission,

- The value of r for a downlink cell configured with M-DCI based M-TRP is determined as

  - If UE reports pdcch-BlindDetectionCA, the value of r to be applied is optionally configured by RRC, either r=1 or reported value r=R

    - Note that when network configures r=1, it does not imply that UE has to support more CCs beyond the UE reported capability

  - If UE does not report pdcch-BlindDetectionCA or the value of r is not configured by RRC, r=R.

- UE indicates pdcch-BlindDetectionCA when it is possible to configure A+B DL cells to the UE with A>= 0 DL serving cells without multi-DCI based multi-TRP and B >=0 DL serving cells with multi-DCI based multi-TRP such that A+R∙B>4, whereas R is reported by UE capability signaling.

- If the UE does not report pdcch-BlindDetectionCA, the UE does not expect to be configured with DL cells to the UE such that A+ R∙B>4 with A>= 0 DL serving cells without multi-DCI based multi-TRP and B >=0 DL serving cells with multi-DCI based multi-TRP, whereas R is reported by UE capability signaling.

- The value range of R is [1, 2], and is indicated through UE capability signalling.

- Note that this agreement does not preclude a UE from reporting multiple R values and corresponding A and B pairs depending on UE capability

- Note that how to capture above into the spec can be up to the editor.

RAN1 further introduced a RRC parameter BDFactorR such that gNB can configure the UE to use Rel-15 BD/CCE limits. However, this is not captured correctly in 38.213 specification text.

Proposal 3: Adopt the following text proposals to capture/clarify BDFactorR.

TP to TS 38.213, Sec. 10: < Unchanged text is omitted> If a UE can support - a first set of Ncells,0DL serving cells where the UE is either not provided CORESETPoolIndex or is provided CORESETPoolIndex with a single value for all CORESETs on all DL BWPs of each serving cell from the first set of serving cells, and - a second set of Ncells,1DL serving cells where the UE is provided CORESETPoolIndex with a value 0 for a first CORESET and with a value 1 for a second CORESET on any DL BWP of each serving cell from the second set of serving cells the UE determines, for the purpose of reporting pdcch-BlindDetectionCA, a number of serving cells as Ncells,0DL+R⋅Ncells,1DL where R is either a value reported by the UE or R=1 if the UE does not report a value of R or the UE is configured by the higher layer parameter BDFacotorR. <Unchanged text is omitted>

TP to TS 38.213, Sec. 10.1: < Unchanged text is omitted> If a UE - does not report pdcch-BlindDetectionCA or is not provided BDFactorR, γ=R - reports pdcch-BlindDetectionCA, if the UE can be indicated is configured by BDFactorR, either γ=1, otherwise or γ=R If a UE is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells with DL BWPs having SCS configuration where μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ≤Ncellscap, the UE is not required to monitor, on the active DL BWP of the scheduling cell, <Unchanged text is omitted>

### Correction 4: Condition to apply antenna port tables

In Rel-16, for single DCI based multi-TRP operation, new antenna port mapping tables are introduced but there is no description on the condition to apply either of tables.

Proposal 4: Adopt the following text proposals for clarification of the condition to apply antenna port mapping tables.

TP to TS 38.212, Sec. 7.3.1.2.2: < Unchanged text is omitted> - Antenna port(s) – 4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4 and Tables 7.3.1.2.2-1A/2A/3A/4A, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4 or Tables 7.3.1.2.2-1A/2A/3A/4A. When a UE supports two TCI states by the MAC-CE, as described in clause [6.1.3.X] of [10, TS 38.321], UE shall use Tables 7.3.1.2.2-1A/2A/3A/4A, otherwise it shall use Tables 7.3.1.2.2-1/2/3/4. If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB, the bitwidth of this field equals , where is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA and is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB. A number of zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of and . <Unchanged text is omitted>

### Correction 5: miscellaneous

Incorrect text level updated.

Proposal 5: Adopt the following correction.

TP to TS 38.214, Sec. 5.1.2.1 Resource allocation in time domain: < Unchanged text is omitted> When a UE configured by the higher layer parameter PDSCH-config that indicates at least one entry in pdsch-TimeDomainAllocationList contain RepNumR16 in PDSCH-TimeDomainResourceAllocation, - If two TCI states are indicated by the DCI field ‘Transmission Configuration Indication’ together with the DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, the same SLIV is applied for all PDSCH transmission occasions, the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Clause 5.1.2.1. When the value indicated by RepNumR16 in PDSCH-TimeDomainResourceAllocation equals to two, the second TCI state is applied to the second PDSCH transmission occasion. When the value indicated by RepNumR16 in PDSCH-TimeDomainResourceAllocation is larger than two, the UE may be further configured to enable CycMapping or SeqMapping in RepTCIMapping. - When CycMapping is enabled, the first and second TCI states are applied to the first and second PDSCH transmission occasions, respectively, and the same TCI mapping pattern continues to the remaining PDSCH transmission occasions. - When SeqMapping is enabled, first TCI state is applied to the first and second PDSCH transmissions, and the second TCI state is applied to the third and fourth PDSCH transmissions, and the same TCI mapping pattern continues to the remaining PDSCH transmission occasions. - The UE may expect that each PDSCH transmission occasion is limited to two transmission layers. For all PDSCH transmission occasions associated with the first TCI state, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n is counted only considering PDSCH transmission occasions associated with the first TCI state. The redundancy version for PDSCH transmission occasions associated with the second TCI state is derived according to Table 5.1.2.1-3, where additional shifting operation for each redundancy version rvs is configured by higher layer parameter RVSeqOffset and n is counted only considering PDSCH transmission occasions associated with the second TCI state. The UE may expect that each PDSCH transmission occasion is limited to two transmission layers. For all PDSCH transmission occasions associated with the first TCI state, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n is counted only considering PDSCH transmission occasions associated with the first TCI state. The redundancy version for PDSCH transmission occasions associated with the second TCI state is derived according to Table 5.1.2.1-3, where additional shifting operation for each redundancy version rvs is configured by higher layer parameter RVSeqOffset and n is counted only considering PDSCH transmission occasions associated with the second TCI state. <Unchanged text is omitted>

# 3	Conclusions

In this contribution, we provide corrections on the Multi-TRP CR on 38.212, 38.213, and 38.214, and our proposals are as follows,

# References

[1] 		TS 38.214, v16.0.0

[2] 		TS 38.213, v16.0.0