3GPP TSG-RAN WG1 Meeting #100-e	R1-2001009

e-Meeting, February 24th- March 6th, 2020

Agenda Item:	7.2.4.2.2

Source:	Ericsson

Title:	Resource allocation Mode 2 for NR SL

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, the following open issues related to the resource allocation mode 2 AI in NR SL Rel-16 are discussed:

| Mode 2 Resource Allocation | C1 | Details of T1, T2, T3 in the sensing and resource (re-)selection window |
| --- | --- | --- |
|  | C2 | Details on resource selection from the identified candidate resources within the selection window for blind and HARQ feedback-based (re)transmission |
|  | C3 | Remaining details of pre-emption |
|  | C4 | Remaining details of re-evaluation of Step 1 and Step 2 |

# 2	Sensing and selection windows

The current specifications define the sensing window in terms of the parameters T0 and Tproc,0. While T0 is configurable, the specification does not define Tproc,0. In our view, Tproc,0 is the time required for decoding all transmissions in a slot. Table 1, obtained from TS 38.214, summarizes the processing times for PDSCH processing for the case that is most related to the physical layer structure of sidelink (i.e., with DMRS delay and simplest capabilities).

Table 1. PDSCH processing times

| SCS | Processing time |
| --- | --- |
| 15 kHz | 13 symbols (0.92 ms) |
| 30 kHz | 13 symbols (0.46 ms) |
| 60 kHz | 20 symbols (0.36 ms) |
| 120 kHz | 24 symbols (0.21 ms) |

Given that the Rel-16 SL design is essentially slot-centric, we propose to specify Tproc,0 in terms of slots. Changes to the specification may be introduced without problems in a later release. Based on this, we propose to have Tproc,0 equal to 1 slot for 15 and 30 kHz and 2 slots for 60 and 120 kHz.

Tproc,0 is defined as 1 slot for 15 kHz and 30 kHz and 2 slots for 60 kHz and 120 kHz.

Similarly, the selection window is defined in terms of Tproc,1 and T2min. After long discussions in Rel-15, for LTE it was decided to support (pre-)configurable values of T2min per PPPP. We propose to avoid repeating the discussion and do the same here.

T2min is independently (pre-)configurable for each SCI priority value.

Regarding Tproc,1, our view is that it is the time required for preparing a PSCCH/PSSCH transmission. Table 2, obtained from TS 38.214, summarizes the preparation times for PUSCH (i.e., with simplest capabilities).

Table 2. PUSCH preparation times

| SCS | Preparation time |
| --- | --- |
| 15 kHz | 10 symbols (0.71 ms) |
| 30 kHz | 12 symbols (0.43 ms) |
| 60 kHz | 23 symbols (0.41 ms) |
| 120 kHz | 36 symbols (0.32 ms) |

Given that the Rel-16 SL design is essentially slot-centric, we propose to specify Tproc,1 in terms of slots. Changes to the specification may be introduced without problems in a later release. Noting that the selection window is defined as [n+T1,n+T2] (rather than (n+T1,n+T2)), we propose to take the values in Table 2 plus one slot, resulting in Tproc,1 equal to 2 slots for 15 and 30 kHz and 2 slots for 60 and 120 kHz.

Tproc,1 is 2 slots for 15 kHz and 30 kHz, 3 slots for 60 kHz, and 4 slots for 120 kHz.

Finally, re-evaluation of Steps 1 and 2 is defined in terms of T3. In our view, this is conceptually the same as Tproc,0. However, in the way the agreement on re-evaluation states that “The re-evaluation of the (re-)selection procedure for a resource reservation signalled in a moment ‘m’ is not required to be triggered at moment > ‘m – T3’ (i.e. resource reselection processing time needs to be ensured)”, meaning that re-evaluation at t =m – T3 is possible. Thus, it is necessary to add an additional slot, i.e., T3 = Tproc,0 + 1.

T3 = Tproc,0 + 1, measured in slots.

# 3	Reservations

Per agreement from the e-mail discussion after RAN1#98bis, the specification allows for a single SCI to reserve resource for up to two future transmissions (in addition to the current transmission), i.e., Nmax = 3.

Agreements: - When reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB is disabled, NMAX is 3 - SCI signaling is designed to allow to indicate 1 or 2 or 3 resources at least of the same number of sub-channels with full flexibility in time and frequency position in a window W of a resource pool - FFS: if full flexibility is limited in some cases - Value 2 or 3 is (pre-)configured per resource pool - FFS size of window W Agreements: - When reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB is enabled, select in RAN1#99 from the following: - Option. 1-a. A period > W is additionally signaled in SCI and the same reservation is applied with respect to resources indicated within NMAX within window W at subsequent periods - FFS number of subsequent reservation periods - FFS NMAX is always same regardless if a period > W is additionally signaled or not for SCI size perspective. - Option. 1-b. A time gap > W is additionally signaled in SCI and the same reservation is applied with respect to resources indicated within NMAX within window W at resources indicated by the time gap - FFS NMAX is always same regardless if a time gap > W is additionally signaled or not for SCI size perspective. - Option. 2. There is no additional field (NDI and HARQ ID are used at the moment of SCI reception) to distinguish reservation for another TB), and at least one of NMAX resources can be signaled beyond window W

The specification does not place any restriction on how to use the reservation capability. Without restrictions, a UE can reserve resources at a higher rate than it uses them. While this is no problem for blind HARQ retransmissions, it may result in severe underutilization of the resources for feedback-based HARQ retransmissions. An example of this undesirable behaviour is illustrated in Figure 1.

Figure 1. Transmission#1 of a TB reserves resources for transmission#2 and transmission#3. Since a NACK is received transmission#2 takes place, reserving resources for transmission#4 and transmission#5. A further NACK is received so transmission#3 takes place, reserving resources for transmission#6 and transmission#7. After this, an ACK is received. Thus, the resources reserved for transmissions #4, #5, #6, and #7 (in red) are not used but are still treated as reserved by a receiver decoding the corresponding SCIs.

In our view, the specification must prevent that for a given TB a UE has more than two outstanding reservations at a time. This is illustrated in Figure 2.

Figure 2. Transmission#1 of a TB reserves resources for transmission#2 and transmission#3. Since a NACK is received transmission#2 takes place, reserving resources for transmission#3 (again) and transmission#4. A further NACK is received so transmission#3 takes place, reserving resources for transmission#4 and transmission#5. After this, an ACK is received. Thus, the resources reserved for transmissions #4 and #5 (in red) are not used but are still treated as reserved by a receiver decoding the corresponding SCIs.

The current specification allows a UE to reserve two resources with each SCI, without any restriction on the total number of reserved resources. This may lead to server resource underutilization in the case of early termination (e.g., due to ACK being received).

In mode 2, for transmission of a TB the UE can have at most two outstanding reservations at a time.

For reservations for different TBs, RAN1 agreed to support the following periodicities: 0, [1:99], 100, 200, 300, 400, 500, 600, 700, 800, 900, 1000 ms, out of which up to 16 could be (pre-)configured and one of them would be selected by the UE. In their implementation of the agreement, RAN2 has considered [1:99] to be a set of values pending discussion. It is therefore necessary to clarify that this is indeed the set of values.

For reservation of an initial transmission of a TB by an SCI associated with a different TB, the following periodicities are supported: 1, 2, 3, 4, …, 97, 98, 99.

# 4	Re-evaluation of Steps 1 and 2

The following agreement from RAN1#98bis allows for re-evaluation of Steps 1 and 2.

Agreements: - Resource (re-)selection procedure supports re-evaluation of Step 1 and Step 2 before transmission of SCI with reservation - The re-evaluation of the (re-)selection procedure for a resource reservation signalled in a moment ‘m’ is not required to be triggered at moment > ‘m – T3’ (i.e. resource reselection processing time needs to be ensured) - FFS condition to change resource(s) from previous iteration to resource(s) from current iteration - FFS relationship of T1 and T3, if any - FFS whether to handle it differently for blind and feedback-based retransmission resources

Besides the value for T3, which is discussed in Section 2, it is necessary to discuss the conditions for changing resources. Without any restriction, a UE would be allowed to make reservations of resources, announce them using SCI but change the behaviour later. This would result in poor resource utilization. In our view, it is enough that the UE cannot change decision for resources for which a reservation has been announced with a corresponding SCI. Other than this, the behavior can be left up to UE implementation.

In re-evaluation of Step 1 and Step 2, the UE can change resource(s) from previous iteration as long as it has not sent a corresponding reservation in SCI.

# 5	Pre-emption

In the current specifications, there are some unclear or missing parts in the procedure for pre-emption. The existing specification is based on the following agreements from RAN1#98bis:

Agreements: - Support a resource pre-emption mechanism for Mode-2 - A UE triggers reselection of already signaled resource(s) as a resource reservation in case of overlap with resource(s) of a higher priority reservation from a different UE and, SL-RSRP measurement associated with the resource reserved by that different UE is larger than an associated SL-RSRP threshold - Only the overlapped resource(s) is/are reselected - FFS - the timeline for reselection - other details - FFS whether or not to support other potential UE behaviour (e.g, power boosting/reduction) - This mechanism can be enabled or disabled, per resource pool - FFS details

Although it is implicit in the agreement, we think it would be good to clarify that the priorities refer to SCI priorities. The latest draft of the MAC running CR compares the received SCI priority with the priority of the logical channel to be transmitted.

In the evaluation of the condition for determining reselection due to pre-emption, SCI priorities are used.

The process of pre-emption is typically as follows:

- UE2, which has a low-priority message, selects resource X for future transmission and sends an SCI with a reservation.

- UE1, which has a high-priority message, after performing sensing, selects resource X for future transmission as well and sends an SCI with a reservation

- Reception of the SCI transmitted by UE1 triggers pre-emption for UE2.

- UE2 selects a new resource for its transmission based on its sensing information.

An example of the first three steps is depicted in Figure 3.

Figure 3. Example of resource reselection mechanism in NR Mode-2.

Two aspects about the process must be discussed: timeline and the details of how the pre-empted UE selects new resources.

Regarding timeline, it is necessary that the pre-emption message is signaled sufficiently in advance so that the pre-emption signaling could be received and processed by the pre-empted UE, to avoid collisions. In our view, this is the processing time Tproc,0 discussed above (see Table 1).

A UE with a reservation for transmission in slot n does not expect a pre-emptying SCI to arrive outside the sensing window [n – T0, n – Tproc,0).

Regarding the possibility of selecting new resources, we propose to leave the decision up to UE implementation. In some cases, the UE may have an additional reservation (i.e., due to Nmax = 3). In other cases, the UE may have already received a positive HARQ-ACK. Note also that in some cases, the PDB is exhausted, the resource selection window is empty, and hence the UE cannot select a new resource. In any case, if the UE decides to select new resources, then it follows the already agreed procedures (i.e., Step 1 and Step 2). Note that in some cases

It is up to UE implementation to select new resources after being pre-empted.

It is important to emphasize the pre-emption creates a disruption to the system and, as such, its impact should be minimized. This can be done by prioritizing pre-emption of those resources which are likely to be unused. For example, resources reserved for HARQ-based retransmissions should be prioritized for pre-emption by the high priority UE as they are not used in the case of ACK.

RSRP thresholds are (pre-)configured for each TX-RX priority pair for initial transmission and retransmission.

In addition, we believe that it is worth clarifying the part of the agreement that states that “Only the overlapped resource(s) is/are reselected”. Given that transmissions of a TB with varying bandwidth are not supported, this agreement can only be interpreted if the overlap is considered in time. That is, if the reservation by UE1 in a slot is pre-empted by a reservation by UE2 for the same slot, UE1 drops the reservation for that slot independently of whether the overlap is partial or total.

In the agreement about pre-emption, overlap corresponds to two reservations (pre-empted and pre-empting) in the same time slot with partial or total frequency overlap.

# 6	Conclusion

In the previous sections we made the following observations:

Observation 1	The current specification allows a UE to reserve two resources with each SCI, without any restriction on the total number of reserved resources. This may lead to server resource underutilization in the case of early termination (e.g., due to ACK being reeived).

Based on the discussion in the previous sections we propose the following:

Proposal 1	Tproc,0 is defined as 1 slot for 15 kHz and 30 kHz and 2 slots for 60 kHz and 120 kHz.

Proposal 2	T2min is independently (pre-)configurable for each SCI priority value.

Proposal 3	Tproc,1 is 2 slots for 15 kHz and 30 kHz, 3 slots for 60 kHz, and 4 slots for 120 kHz.

Proposal 4	T3 = Tproc,0 + 1, measured in slots.

Proposal 5	In mode 2, for transmission of a TB the UE can have at most two outstanding reservations at a time.

Proposal 6	For reservation of an initial transmission of a TB by an SCI associated with a different TB, the following periodicities are supported: 1, 2, 3, 4, …, 97, 98, 99.

Proposal 7	In re-evaluation of Step 1 and Step 2, the UE can change resource(s) from previous iteration as long as it has not sent a corresponding reservation in SCI.

Proposal 8	In the evaluation of the condition for determining reselection due to pre-emption, SCI priorities are used.

Proposal 9	A UE with a reservation for transmission in slot n does not expect a pre-emptying SCI to arrive outside the sensing window [n – T0, n – Tproc,0).

Proposal 10	It is up to UE implementation to select new resources after being pre-empted.

Proposal 11	RSRP thresholds are (pre-)configured for each TX-RX priority pair for initial transmission and retransmission.

Proposal 12	In the agreement about pre-emption, overlap corresponds to two reservations (pre-empted and pre-empting) in the same time slot with partial or total frequency overlap.