3GPP TSG-RAN WG1 Meeting #100-e	R1-2001012

e-Meeting, February 24th- March 6th, 2020

Agenda Item:	7.2.4.5

Source:	Ericsson

Title:	Physical layer procedures for NR sidelink

Document for:	Discussion, Decision

# 1	Introduction

This paper addresses remaining issues for physical layer procedures of NR SL Rel-16 including the following.

Details of “when to prioritize which transmission” in case of simultaneous transmission of UL and SL across difference carriers

Details on PSFCH candidate resource determination

Whether or not to introduce restriction on the size of group in groupcast HARQ feedback option 2

Details of calculating TX-RX distance for groupcast HARQ feedback option 1

Details of transmission power of PSCCH

How to derive reference PSSCH DMRS power for SL pathloss estimation

Details of sidelink CSI measurement

Contents of 2nd SCI format, including down-selection of the following options: - Option 1: The same 2nd stage SCI format is used for groupcast HARQ feedback option 1 and option 2. - SCI indicator to indicate between groupcast Option 1 and groupcast Option 2 is in the 2nd-stage SCI. - Option 2: Different 2nd stage SCI formats are used in groupcast HARQ feedback option 1 and option 2. - 1st stage SCI indicates which format is used.

# 2		Remaining issues on PSFCH resource determination

The following was agreed in RAN1#99 regarding the PSFCH resource determination with the yellow highlighted part still to be discussed.

Agreements in RAN1#99: - In determining PSFCH candidate resources for a PSFCH format from the starting sub-channel index and the slot index used for the corresponding PSSCH for actual transmission, - Notation - S: the number of sub-channels in a slot - N: the number of PSSCH slots 0 with a single PSFCH slot - NF: the number of PRBs in the set (pre-)configured for the actual PSFCH resources. - Within the set of PRBs (pre-)configured for the actual PSFCH resources, the first Z PRBs are associated with the first sub-channel in the first slot associated with the PSFCH slot, the second Z PRBs are associated with the first sub-channel in the second slot associated with the PSFCH slot, and so on. - FFS when NF is not a multiple of S*N - For a PSSCH, the candidate PSFCH resource is the set of PRBs associated with - Option 1: the starting sub-channel and slot used for that PSSCH. - Option 2: the sub-channel(s) and slot used for that PSSCH - PSFCH TX/TX conflict case 2 and 3 will be discussed separately.

Working assumption in RAN1#99: - For the PSFCH candidate resource set with Z PRBs and Y cyclic shift pairs in each PRB, - Each PSFCH resource is indexed in the manner of frequency first and cyclic shift second. - FFS the order of cyclic shift indexing in a PRB. - PSFCH resource with the index ((K+M) mod (Z*Y)) is used for PSFCH transmission of a RX UE. - K is the L1 source ID of the associated PSCCH/PSSCH. - M is 0 for unicast and groupcast feedback option 1 and M is the member ID of the RX UE for groupcast feedback option 2. - FFS whether to have the following restriction. - Groupcast HARQ feedback option 2 is not used if X > Z*Y (Y denotes the number of PSFCH in a PRB). - Note: RAN1 assumes that the member ID M is an integer between 0 and X-1.

In our view, for a PSSCH the set of PRBs associated with the candidate PSFCH resource should be associated with the starting subchannel and slot used for that PSSCH. This is because we do not see any advantage of mapping the PSFCH candidate resource set according to the number of subchannels used by the associated PSSCH. Instead, such mapping will lead to PSFCH collision if same subchannels are used for different PSSCH. Furthermore, the size of candidate resource set (i.e. Z PRBs) should remain fixed regardless of the PSSCH resources.

Adopt option 1 for PSFCH candidate resource set mapping i.e. the candidate PSFCH resource is the set of PRBs associated with the starting subchannel and slot used for the PSSCH.

As a consequence of having the fixed (pre-)configured size of candidate resource set, it is straightforward to introduce a restriction at the higher layers if the group size is large enough to maintain a unique PSFCH resource for each group member. Therefore, we propose the following:

Higher layers restrict the use of groupcast HARQ feedback option 2 if X > Z*Y. Details up to RAN2.

In RAN1#99 there was an unsettled FFS on PSFCH resource determination, as follows:

Agreements: - CDM between PSFCH transmissions from different UEs in the same PRB is supported as follows: - Cyclic shift can be selected based on - the L1 source ID of TX UE for unicast and groupcast option 1. - the L1 source ID of TX UE and the member ID of RX UE in groupcast option 2. - FFS whether or not to use additional parameter(s) for the selection (e.g., using PSCCH DM-RS, etc) to conclude this week - Base sequence is - (Pre-)configured per resource pool

Note that while the working assumption cited in the beginning of this section has incorporated the agreement that the cyclic shift (CS) of the PSFCH can be selected based on the L1 source ID (and on the member ID for groupcast option 2), it may still happen that two PSFCHs collide in the same PSFCH resource. Since a pre-condition of such a collision is that the corresponding PSCCHs were sent in the same subchannel but were successfully decoded at the respective receivers, we believe it is necessary to associate a PSFCH resource with a parameter of the corresponding PSCCH. For example, the PSFCH resource can be selected based on the formula ((K+M+C) mod (Z*Y)) where C is a parameter of the associated PSCCH DMRS

Support using a parameter of PSCCH in determining the corresponding PSFCH resource, e.g., PSFCH resource is determined by ((K+M+C) mod (Z*Y)) where C is a parameter of the PSCCH.

# 3	TX-RX distance for groupcast option 1

RAN1#98bis made a working assumption on the distance-based HARQ feedback for Groupcast Option 1 as follows:

Working assumption in RAN1#98bis: - For TX-RX distance-based HARQ feedback for groupcast Option 1, - Zone is (pre-)configured with respect to geographical area, and Zone ID associated with TE UE’s location is indicated by SCI. - Details FFS - Note: this does not intend to impact the discussion on the zone based resource allocation.

RAN1#99 made another related working assumption:

Working assumption in RAN1#99: - At least 4 bits are used in 2nd stage SCI to indicate communication range requirement. - Candidates at least include - {50, 80, 180, 200, 350, 400, 500, 700, 1000} meters

However, details of how the Tx-Rx distance is determined are still missing. In our view the distance can be determined based on the location of the centers of corresponding zones.

For TX-RX distance-based HARQ feedback for groupcast Option 1, the TX-RX distance is calculated as the distance between the centers of the zones that the TX and the RX belong to.

The number of bits for signaling zone ID depends on the zone configuration and it is up to RAN2.

# 4	Transmit power for PSCCH

Based on the RAN4 reply on the need for transient time for multiplexing options of PSCCH and PSSCH, a transient period is required whenever there is a change in the total transmit power or in the RB configuration. Since PSCCH power boosting may imply changing the total transmit power, a power boosting mechanism for PSCCH should not be supported.

Power boosting for PSCCH is not supported.

# 5	How to derive reference PSSCH DMRS power for SL pathloss estimation

RAN1 agreed that the Rx UE will report Layer-3 filtered RSRP measurement to the Tx UE for pathloss estimation. It is therefore important for the Tx UE to associate each RSRP report with the corresponding Tx power (of the DMRS), otherwise the Tx UE may use the wrong reference DMRS power in estimating the pathloss. Hence, we believe it is necessary to allow the Tx UE to send a new RSRP measurement request to the Rx UE whenever there is a significant change of the Tx power. Therefore, we propose the following.

In case of changes in the power of the RS used for RSRP measurement, the Tx UE triggers a new request for RSRP report. Details of signaling up to RAN2.

# 6		Prioritization of transmissions

In RAN1#98bis, the following was agreed:

Working assumption: - For the power limited case in supporting simultaneous sidelink and uplink transmissions (SL carrier is different from UL carrier), - If sidelink transmission is prioritized over uplink transmission, the UE shall adjust the uplink transmission power before the start of the transmission such that its total transmission power does not exceed on any overlapped portion. In this case, calculation of the adjustment to the uplink transmission power is not specified. - If uplink transmission is prioritized over sidelink transmission, the UE shall adjust the sidelink transmission power before the start of the transmission such that its total transmission power does not exceed on any overlapped portion. In this case, calculation of the adjustment to the sidelink transmission power is not specified. - Total sidelink transmit power is the same in the symbols used for actual PSCCH/PSSCH transmissions in a slot in case of simultaneous transmission of sidelink and uplink - PSCCH/PSSCH transmissions can be dropped in some symbols when there are uplink transmissions with higher priority and the UE cannot keep the same sidelink transmission power in the symbols. - Selection of the dropped symbols is up to UE implementation where the dropped symbols should include the overlapping symbols. - If the simultaneous transmission of sidelink and uplink is beyond the UE capability, the one not prioritized can be dropped. - FFS: when to prioritize which transmission - FFS: how to address UE processing time - FFS: whether there is a case of dropping some symbols of uplink transmissions - Whether/how to address RF transient period is up to RAN4.

In this section we address the issues left FFS in the above agreement.

On the issue of prioritization, the following procedure was used in LTE (see TS 36.213, Section 14):

If a UE uplink transmission of a serving cell overlaps in time domain with a sidelink transmission for sidelink transmission mode 3 or 4 of the same serving cell and the value in "Priority" field of the corresponding SCI is smaller than the high layer parameter thresSL-TxPrioritization, then the UE shall drop the uplink transmission. Else, if a UE uplink transmission of a serving cell overlaps in time domain with sidelink transmission for sidelink transmission mode 3 or 4 of the same serving cell, then the UE shall drop the sidelink transmission.

For normal UL traffic, we believe that the same signaling and procedure can be reused for NR. However, UL URLLC traffic must be treated in a different way. In our view, it should always be prioritized over SL. The LTE procedures and signaling are reused for defining the relative priorities of SL and non-URLLC UL transmissions. URLLC UL traffic is always prioritized over SL.

The LTE procedures and signaling are reused for defining the relative priorities of SL and non-URLLC UL transmissions. URLLC UL traffic is always prioritized over SL.

For reference, we also copy the following related agreement made in RAN2 #107-bis:

Agreements on prioritization: 1: A separate LCH priority thresholds is configured for both NR-UL and NR-SL. 2: For between SL-data and UL-data/SRB, the SL transmission is prioritized if the highest priority value of UL LCH(s) with available data is larger than the UL priority threshold and the highest priority value of SL LCH(s) with available data is lower than the SL priority threshold. Otherwise the UL transmission is prioritized.

# 7	2nd-stage SCI format

In RAN1#99, following options were listed for 2nd stage SCI formats.

- Option 1: The same 2nd stage SCI format is used for groupcast HARQ feedback option 1 and option 2. - SCI indicator to indicate between groupcast Option 1 and groupcast Option 2 is in the 2nd-stage SCI. - Option 2: Different 2nd stage SCI formats are used in groupcast HARQ feedback option 1 and option 2. - 1st stage SCI indicates which format is used.

We prefer Option 2 due to large difference in the 2nd stage SCI sizes for HARQ feedback option 1 and option 2. This results in exploiting the benefit of 2 stage SCI.

Different 2nd-stage SCI formats is used for groupcast HARQ feedback option 1 and option 2 with indication of which format in the first stage.

# 8	Details on CSI measurement

As agreed durign SI/WI, non-subband-based RI and CQI reports will be supported for sidelink unicast. In NR Uu transmissions, typically one RI value and the associated PMI and/or CQI are reported, where RI represents the maximum possible transmission rank of the measured channel. However, this may not be suitable for V2X applications which have diverse service requirements in terms of data rate and reliability. More specifically, some NR V2X use cases may target high data rate while others target high reliability. On the other hand, single unicast connection established between the transmitter UE and the receiver UE may carry different V2X services (using different SLRB). In addition, different unicast connections can be established (as per SA2 outcome), however the link condition between the UEs is the same. Accordingly, to satisfy the diverse requirements, some services are interested in multi-layer transmissions while other services are interested in single-layer transmissions. However, when the receiver reports CSI parameters, it is typically not aware of the transmitter’s interest, e.g., the transmission requirement. In this case, it is beneficial to report multiple CQI values associated with different RI values respectively, which gives the transmitter the flexibility to select more proper transmission parameters based on its own needs.

One sidelink CSI report may include multiple CQIs associated with different RIs respectively.

Since it has been agreed to support up to two antenna ports, the rank of a PSSCH transmission can only be either 1 or 2. Hence, 1 bit is enough for RI. Moreover, for one CQI report, within a given CQI table, 4 bits are enough as in NR Uu. In this way, SL CSI report size is 5 bits when reporting one RI and its associated CQI. SL CSI report size is 9 bits when report two CQIs associated with rank-1 and rank-2 respectively.

SL CSI report size is up to 9 bits for NR Rel-16.

# 9	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt option 1 for PSFCH candidate resource set mapping i.e. the candidate PSFCH resource is the set of PRBs associated with the starting subchannel and slot used for the PSSCH.

Proposal 2	Higher layers restrict the use of groupcast HARQ feedback option 2 if X > Z*Y. Details up to RAN2.

Proposal 3	Support using a parameter of PSCCH in determining the corresponding PSFCH resource, e.g., PSFCH resource is determined by ((K+M+C) mod (Z*Y)) where C is a parameter of the PSCCH.

Proposal 4	For TX-RX distance-based HARQ feedback for groupcast Option 1, the TX-RX distance is calculated as the distance between the centers of the zones that the TX and the RX belong to.

Proposal 5	The number of bits for signaling zone ID depends on the zone configuration and it is up to RAN2.

Proposal 6	Power boosting for PSCCH is not supported.

Proposal 7	In case of changes in the power of the RS used for RSRP measurement, the Tx UE triggers a new request for RSRP report. Details of signaling up to RAN2.

Proposal 8	The LTE procedures and signaling are reused for defining the relative priorities of SL and non-URLLC UL transmissions. URLLC UL traffic is always prioritized over SL.

Proposal 9	Different 2nd-stage SCI formats is used for groupcast HARQ feedback option 1 and option 2 with indication of which format in the first stage.

Proposal 10	One sidelink CSI report may include multiple CQIs associated with different RIs respectively.

Proposal 11	SL CSI report size is up to 9 bits for NR Rel-16.

# References

R1-2001007, "Physical layer structure for NR sidelink", RAN1-100e, Feb. 2020.