3GPP TSG-RAN WG1 Meeting #100-e	R1-2001013

e-Meeting, February 24th- March 6th, 2020

Agenda Item:	7.2.4.6

Source:	Ericsson

Title:	QoS management for NR sidelink

Document for:	Discussion, Decision

# 1	Introduction

This contribution addresses the remaining issues related to QoS management for NR SL Rel-16.

# 2	Discussion

RAN1#99 and a follow-up email discussion made the following agreements on CR evaluation.

Agreements: The CBR measurement time window size is 100 ms and 100 slots by (pre-)configuration. CR window size is { 1000 ms, 1000 slots } by (pre)-configuration Agreements: (from [99-NR-12]) - The future segment of the CR evaluation window reuses the same behaviour as in the LTE V2X sidelink. - FFS whether additional constraints on UE’s choice of values for a and b are needed.

Regarding the FFS, recall that for LTE V2X the following definition of CR is used:

Definition; Channel occupancy ratio (CR) evaluated at subframe n is defined as the total number of sub-channels used for its transmissions in subframes [n-a, n-1] and granted in subframes [n, n+b] divided by the total number of configured sub-channels in the transmission pool over [n-a, n+b].. NOTE 1: a is a positive integer and b is 0 or a positive integer; a and b are determined by UE implementation with a+b+1 = 1000, a >= 500, and n+b should not exceed the last transmission opportunity of the grant for the current transmission.

In our view NR V2X needs to handle the aperiodic traffic effectively because that type of traffic has an important role for eV2X use cases. Therefore, the CR definition should put more emphasis on the resource consumption in the past (than in the future). This can be done by increasing the value of parameter a in the above definition. Hence, we propose the following:

For CR definition, reuse the definition of LTE V2X with “subframe” replaced by “slot” and the following constraints apply: a+b+1 = 1000, a >=950.

Another remaining issue, which was raised during the email discussion, is how often the CR is evaluated, namely only for the initial transmission or for every (re)transmission of a packet. As a starting point, in LTE V2X the following rule applies:

- CBR is measured and CR is evaluated for each (re)transmission - For a (re)transmission in subframe n+4, the CR is evaluated in subframe n. - For a (re)transmission in subframe n+4, the CBR measured in subfrme n is used.

In our view, the same procedure of LTE V2X can be reused in NR V2X.

CR is evaluated for each (re)transmission.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	For CR definition, reuse the definition of LTE V2X with “subframe” replaced by “slot” and the following constraints apply: a+b+1 = 1000, a >=950.

Proposal 2	CR is evaluated for each (re)transmission.