3GPP TSG RAN WG1 Meeting #100-e	  R1-2001044

February 24-28, 2020

Agenda Item:	7.2.5.1

Source:	Huawei, HiSilicon

Title:	Feature lead summary #1 on PDCCH enhancements

Document for:	Discussion and Decision

Introduction

In the RAN1 #99 meeting, the WID [1] on Physical Layer Enhancements for NR URLLC was concluded. In the RAN1#100-e meeting, remaining open issues and corrections will be discussed.

This document summarizes the key issues discussed under agenda item 7.2.5.1 based on the views in [2][3][4][5][6][7][8][9][10][11][12][13][14][15][16][17][18][19][20], and aims to identify a set of critical issues for RAN1#100-e email discussion.

Summary of issues raised for PDCCH enhancements

This section summarize the issues raised by companies on PDCCH enhancements, among which a set of issues can be identified for RAN1#100-e email discussions per the guidance from Chairman.

Issues raised on DCI format design

| Issue # | Description | Source | Recommended handling |
| --- | --- | --- | --- |
| A-1 | DCI size alignment in TS 38.212 - If both formats 0_1/1_1 and 0_2/1_2 are configured to monitor in USS, their size should be made distinguishable - Other extension due to the introduction of DCI format 0_2/1_2 Note: Details seen in section 3 | Ericsson [2] Huawei/HiSi [21] Vivo [3] CATT [8] Panasonic [9] LG [11] Intel [12] China Unicom [13] Spreadtrum [16] WILUS [18] Qualcomm [19] | Included in email discussion #A, to discuss the proposal #1 in section 3 Reason: - Open issues need to be solved; - Proposed by Top 1 number of companies; - Diverse views from companies on the potential TP |
| A-2 | Resource allocation type 1 with granularity larger than 1 RB for DCI format 0_2/1_2 - RIV for downlink resource allocation type 1 in TS 38.214 is still missing - RIV for uplink resource allocation type 1 in TS 38.214 is still missing - Number of bits for frequency domain resource assignment for resource allocation type 1 in TS 38.212 is still in bracket Note: Details seen in section 3 | Ericsson [2] Nokia [5] OPPO [7] CATT [8] NTT DCM [17] WILUS [18] | Included in email discussion #A, to discuss the proposal #2 in section 3 Reason: - Open issues need to be solved; - Proposed by top 2 number of companies; - Diverse views from companies on the potential TP |
| A-3 | Priority indication via DCI format 0_1/1_1 and 0_2/1_2 - How to determine the priority of transmissions scheduled by the DCI formats 0_1/1_1/0_2/1_2 for a UE that doesn’t support a DCI format scheduling PDSCH with different HARQ-ACK priorities or PUSCH with different priorities when both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to monitor? Note: Details seen in section 3 | Intel [12] Qualcomm [19] | Included in email discussion #A, to discuss the proposal #3 in section 3 Reason: - May have RRC impact depending on the solutions |
| A-4 | Whether to introduce independent RRC parameters AntennaPorts-FieldPresence-ForDCIFormat0_2 for DCI format 0_2 and AntennaPorts-FieldPresence-ForDCIFormat1_2 for DCI format 1_2 for Antenna ports for DCI format 0_2 and 1_2? - More flexibility to control the presence of the field Note: Details seen in section 3 | Nokia [5] Intel [12] | Included in email discussion #A, to discuss the proposal #4 in section 3 Reason: May have impact on RRC parameter, though it belongs to optimization |
| A-5 | Clarification and discussion on RRC parameter CSI-AperiodicTriggerStateList-ForDCIFormat0_2 and CSI-SemiPersistentOnPUSCH-TriggerStateList-ForDCIFormat0_2. - Whether to introduce CSI-SemiPersistentOnPUSCH-TriggerStateList-ForDCIFormat0_2? - Whether to introduce a separate Aperiodic CSI Trigger State Subselection MAC CE will be required for the new DCI format 0_2？ Note: Details seen in section 3 | Nokia [5] | Included in email discussion #A, to discuss the proposal #5 in section 3 Reason: Impact on RRC parameter |
| A-6 | Determination of DCI field sizes for the case of two HARQ-ACK codebooks - Confirm the working assumption (Vivo, CATT) - Extend to the case that both R-15 and Rel-16 DCI formats can be used to schedule same priority traffic (Samsung) - Change “When two HARQ-ACK codebooks are configured for the same serving cell” to “If higher layer parameter PriorityIndicator-ForDCIFormat0_1 is configured” in TS 38.212 (Nokia) | Vivo [3] Nokia [5] CATT [8] Samsung [10] | No discussion in RAN1#100-e Reason: - Time limit for email discussion |
| A-7 | Changing the candidate RV values from {0, 3} to {0, 2} in case of 1 bit for Redundancy version for DCI format 0_2 | Ericsson [2] | No discussion in RAN1#100-e Reason: - Time limit for email discussion - Belongs to optimization |
| A-8 | Miscellaneous corrections - Correction to DMRS reception procedure for PDSCH scheduled by DCI formats 1_2 - Missing DMRS transmission procedure for PUSCH scheduled by DCI format 0_2 (38.214, Sec. 6.1.4.2 & 6.2.2) - Missing PTRS reception procedure for PDSCH scheduled by DCI formats 1_2 (38.214, Sec. 5.1.6.2 & 5.1.6.3) - Missing PTRS reception procedure for PDSCH scheduled by DCI formats 1_2 (38.214, Sec. 5.1.6.2 & 5.1.6.3) - Applicable parameters for SPS PDSCH due to separate RRC parameters for DCI format 1_2 (38.214 - Sec. 5.1) - Correction to PDSCH TDRA for DCI formats 1_2 (38.214, Sec. 5.1.2.1) - Remove CBG PUSCH additions to DCI format 0_1 (38.212, Sec. 10.1) - Correction to FH field size definition for DCI format 0_2 (38.212, Sec. 7.3.1.1.3) - Remove bracket for the description for UL-SCH indicator field for DCI format 0_2 in 38.212 - Remove underline from new RRC parameter in UE PDCCH procedures in 38.213 - Correct typo in Modulation order for PUSCH in 38.214 - Clarifications on some configurable fields, e.g. carrier indicator, FDRA, DAI, etc. - Clarification on the bit width for TCI bit field with configured tci-PresentInDCI-ForDCIFormat1_2 in a DCI format 1_2 indicating BWP switch | Nokia [5] OPPO [7] Sharp [15] ASUSTeK [20] | Postpone the discussion to next meeting (if time permit we can try some, e.g. the first 4 issues), some of the issues need to be resolved. Reason: - Time limit for email discussion - Corrections from one company thus lower priority this meeting. |

Based on the above summary in the above table, the following proposals are recommended for companies to check for the first phase email discussion, i.e. to identify essential issues for RAN1#100-e email discussion.

Proposal # 2-1: Set email discussion #A to discuss the following issues on DCI format:

- Issue A-1: DCI size alignment with proposal #1 in section 3 as the starting point.

- Issue A-2: Resource allocation type 1 with granularity larger than 1 RB for DCI format 0_2/1_2, with proposal #2 in section 3 as the starting point

- Issue A-3: Priority indication via DCI format 0_1/1_1 and 0_2/1_2,  with proposal #3 in section 3 as the starting point

- Issue A-4:Whether to introduce independent RRC parameters AntennaPorts-FieldPresence-ForDCIFormat0_2 for DCI format 0_2 and AntennaPorts-FieldPresence-ForDCIFormat1_2 for DCI format 1_2 for Antenna ports for DCI format 0_2 and 1_2, with proposal #4 in section 3 as the starting point

- Issue A-5: Clarification and discussion on RRC parameter CSI-AperiodicTriggerStateList-ForDCIFormat0_2 and CSI-SemiPersistentOnPUSCH-TriggerStateList-ForDCIFormat0_2,  with proposal #5 in section 3 as the starting point

Companies are encouraged to provide views here.

| Company | View |
| --- | --- |

Issues raised on enhanced PDCCH monitoring capability

| Issue # | Description | Source | Recommended handling |
| --- | --- | --- | --- |
| B-1 | The per-CC limit on the maximum number of non-overlapping CCEs per monitoring span for (2, 2) and (4, 3) Note: Details seen in section 3 | Ericsson [2] Huawei/HiSi [21] Vivo [3] Nokia [5] MTK [6] OPPO [7] CATT [8] Samsung [10] Intel [12] China Unicom [13] Apple [14] Spreadtrum [16] Qualcomm [19] NTT DCM [17] | Included in email discussion #B, to discuss the proposal #1 in section 4 Reason: - Open issues need to be solved; - Proposed by Top 1 number of companies; - Diverse views from companies |
| B-2 | The per-CC limit on the maximum number of monitored PDCCH candidates per monitoring span | Ericsson [2] Huawei/HiSi [21] Vivo [3] Nokia [5] MTK [6] OPPO [7] Samsung [10] Intel [12] Apple [14] Spreadtrum [16] Qualcomm [19] NTT DCM [17] | Included in email discussion #B, to discuss the proposal #2 in section 4 Reason: - Open issues need to be solved; - Proposed by Top 2 number of companies; - Diverse views from companies |
| B-3 | Capability on the number of CCs with Rel-16 monitoring capability | Ericsson [2] Huawei/HiSi [21] Nokia [5] MTK [6] CATT [8] Samsung [10] Spreadtrum [16] Qualcomm [19] | Included in email discussion #B, to discuss the proposal #3 and proposal #4 in section 4 Reason: - Open issues need to be solved; - Proposed by Top 3 number of companies; |
| C-1 | PDCCH candidate overbooking and dropping | Ericsson [2] Huawei/HiSi [21] Vivo [3] ZTE [4] Panasonic [9] LG [11] CATT [8] China Unicom [13] Apple [14] Spreadtrum [16] Qualcomm [19] | Included in email discussion #C, to discuss the proposal #5 in section 4 Reason: - Open issues need to be solved; - Proposed by Top 2 number of companies; |
| C-2 | How to scale the monitoring capability if the number of CCs configured is larger than the reported capability? | Ericsson [2] ZTE [4] Nokia [5] Samsung [10] Intel [12] ASUSTeK [20] WILUS [18] | Included in email discussion #C, to discuss the proposal #6 and proposal #7 in section 4 Reason: - Open issues need to be solved; - Proposed by a large number of companies; |
| C-3 | Correction on span definition | Huawei [21] Sharp [14] | Included in email discussion #C, to discuss the proposal #8 and proposal #9 in section 4 Reason: - Essential issues need to be solved; |

In addition to the issues list in the above table, a few other issues are discussed in some contributions also, however due to the time limit probably we are not able to discuss in the meeting. Based on the summary in the above table, the following proposals are recommended for companies to check for the first phase email discussion, i.e. to identify essential issues for RAN1#100-e email discussion.

Proposal # 2-2: Set email discussion #B to discuss the following issues on enhanced PDCCH monitoring capability:

- Issue B-1: The per-CC limit on the maximum number of non-overlapping CCEs per monitoring span for (2, 2) and (4, 3), with proposal #1 in section 4 as the starting point.

- Issue B-2: The per-CC limit on the maximum number of monitored PDCCH candidates per monitoring span, with proposal #2 in section 4 as the starting point.

- Issue B-3: Capability on the number of CCs with Rel-16 monitoring capability, with proposal #3 and proposal #4 in section 4 as the starting point.

Companies are encouraged to provide views here.

| Company | View |
| --- | --- |

Proposal # 2-3: Set email discussion #C to discuss the following issues on enhanced PDCCH monitoring capability:

- Issue C-1: PDCCH candidate overbooking and dropping, with proposal #5 in section 4 as the starting point.

- Issue C-2: Scaling the monitoring capability if the number of CCs configured is larger than the reported capability, with proposal #6 and proposal #7 in section 4 as the starting point.

- Issue C-3: Correction on span definition, with proposal #8 and proposal #9 in section 4 as the starting point.

Companies are encouraged to provide views here.

| Company | View |
| --- | --- |

DCI format scheduling Rel-16 URLLC

Based on the contributions from companies, the following issues related to DCI format design are discussed. Companies are encouraged to check and provide your inputs.

Issue A-1: Further extension of DCI size alignment due to the introduction of DCI format 0_2/1_2?

In the current TS 38.212, DCI size alignment is extended due to the introduction of DCI format 0_2/1_2 with a few places in bracket due to lack of explicit agreements. In addition, one remaining open issue is how to differentiate DCI format 0_1/1_1 and DCI format 0_2/1_2 if both are configured to monitor in USS. Based on the views from companies, it seems most companies think that padding can be used to make the size distinguishable. Accordingly, different TPs are proposed in the contributions. Based on all the proposed TPs, it is possible we can start with the following TP:

Proposal #1: Adopt the following Text proposal for section 7.3.1.0 in TS 38.212:

TP for Sec. 7.3.1.0 of 38.212 v16.0.0 <Unchanged text omitted> Step 0: - Determine DCI format 0_0 monitored in a common search space according to clause 7.3.1.1.1 where is the size of the initial UL bandwidth part. - Determine DCI format 1_0 monitored in a common search space according to clause 7.3.1.2.1 where is given by - the size of CORESET 0 if CORESET 0 is configured for the cell; and - the size of initial DL bandwidth part if CORESET 0 is not configured for the cell. - If DCI format 0_0 is monitored in common search space and if the number of information bits in the DCI format 0_0 prior to padding is less than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_0 until the payload size equals that of the DCI format 1_0. - If DCI format 0_0 is monitored in common search space and if the number of information bits in the DCI format 0_0 prior to truncation is larger than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, the bitwidth of the frequency domain resource assignment field in the DCI format 0_0 is reduced by truncating the first few most significant bits such that the size of DCI format 0_0 equals the size of the DCI format 1_0. Step 1: - Determine DCI format 0_0 monitored in a UE-specific search space according to clause 7.3.1.1.1 where is the size of the active UL bandwidth part. - Determine DCI format 1_0 monitored in a UE-specific search space according to clause 7.3.1.2.1 where is the size of the active DL bandwidth part. - For a UE configured with supplementaryUplink in ServingCellConfig in a cell, if PUSCH is configured to be transmitted on both the SUL and the non-SUL of the cell and if the number of information bits in DCI format 0_0 in UE-specific search space for the SUL is not equal to the number of information bits in DCI format 0_0 in UE-specific search space for the non-SUL, a number of zero padding bits are generated for the smaller DCI format 0_0 until the payload size equals that of the larger DCI format 0_0. - If DCI format 0_0 is monitored in UE-specific search space and if the number of information bits in the DCI format 0_0 prior to padding is less than the payload size of the DCI format 1_0 monitored in UE-specific search space for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_0 until the payload size equals that of the DCI format 1_0. - If DCI format 1_0 is monitored in UE-specific search space and if the number of information bits in the DCI format 1_0 prior to padding is less than the payload size of the DCI format 0_0 monitored in UE-specific search space for scheduling the same serving cell, zeros shall be appended to the DCI format 1_0 until the payload size equals that of the DCI format 0_0 Step 2: - Determine DCI format 0_1 monitored in a UE-specific search space according to clause 7.3.1.1.2. - Determine DCI format 1_1 monitored in a UE-specific search space according to clause 7.3.1.2.2. - For a UE configured with supplementaryUplink in ServingCellConfig in a cell, if PUSCH is configured to be transmitted on both the SUL and the non-SUL of the cell and if the number of information bits in format 0_1 for the SUL is not equal to the number of information bits in format 0_1 for the non-SUL, zeros shall be appended to smaller format 0_1 until the payload size equals that of the larger format 0_1. - If the size of DCI format 0_1 monitored in a UE-specific search space equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 0_1. - If the size of DCI format 1_1 monitored in a UE-specific search space equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 1_1. Step 2A: - Determine DCI format 0_2 monitored in a UE-specific search space according to clause 7.3.1.1.3. - Determine DCI format 1_2 monitored in a UE-specific search space according to clause 7.3.1.2.3. - For a UE configured with supplementaryUplink in ServingCellConfig in a cell, if PUSCH is configured to be transmitted on both the SUL and the non-SUL of the cell and if the number of information bits in format 0_2 for the SUL is not equal to the number of information bits in format 0_2 for the non-SUL, zeros shall be appended to smaller format 0_2 until the payload size equals that of the larger format 0_2. - If the size of DCI format 0_2 monitored in a UE-specific search space equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 0_2. - If the size of DCI format 1_2 monitored in a UE-specific search space equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 1_2. Step 2B: - If the size of DCI format 0_1 monitored in a UE-specific search space equals that of a DCI format 0_2 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 0_1. - If the size of DCI format 0_1 equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 0_1 - If the size of DCI format 1_1 monitored in a UE-specific search space equals that of a DCI format 1_2 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 1_1. - If the size of DCI format 1_1 equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 1_1. Step 3: - If both of the following conditions are fulfilled the size alignment procedure is complete - the total number of different DCI sizes configured to monitor is no more than 4 for the cell - the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell Step 4: - Otherwise Step 4A: - Remove the padding bit (if any) introduced in step 2 and step 2A and step 2B above. - Determine DCI format 1_0 monitored in a UE-specific search space according to clause 7.3.1.2.1 where is given by - the size of CORESET 0 if CORESET 0 is configured for the cell; and - the size of initial DL bandwidth part if CORESET 0 is not configured for the cell. - Determine DCI format 0_0 monitored in a UE-specific search space according to clause 7.3.1.1.1 where is the size of the initial UL bandwidth part. - If the number of information bits in the DCI format 0_0 monitored in a UE-specific search space prior to padding is less than the payload size of the DCI format 1_0 monitored in UE-specific search space for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_0 monitored in a UE-specific search space until the payload size equals that of the DCI format 1_0 monitored in a UE-specific search space. - If the number of information bits in the DCI format 0_0 monitored in a UE-specific search space prior to truncation is larger than the payload size of the DCI format 1_0 monitored in UE-specific search space for scheduling the same serving cell, the bitwidth of the frequency domain resource assignment field in the DCI format 0_0 is reduced by truncating the first few most significant bits such that the size of DCI format 0_0 monitored in a UE-specific search space equals the size of the DCI format 1_0 monitored in a UE-specific search space. Step 4B: - If the total number of different DCI sizes configured to monitor is more than 4 for the cell after applying the above steps, or if the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell after applying the above steps - If the number of information bits in the DCI format 0_2 prior to padding is less than the payload size of the DCI format 1_2 for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_2 until the payload size equals that of the DCI format 1_2. - If the number of information bits in the DCI format 1_2 prior to padding is less than the payload size of the DCI format 0_2 for scheduling the same serving cell, zeros shall be appended to the DCI format 1_2 until the payload size equals that of the DCI format 0_2. Step 4C: - If the total number of different DCI sizes configured to monitor is more than 4 for the cell after applying the above steps, or if the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell after applying the above steps - If the number of information bits in the DCI format 0_1 prior to padding is less than the payload size of the DCI format 1_1 for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_1 until the payload size equals that of the DCI format 1_1. - If the number of information bits in the DCI format 1_1 prior to padding is less than the payload size of the DCI format 0_1 for scheduling the same serving cell, zeros shall be appended to the DCI format 1_1 until the payload size equals that of the DCI format 0_1. Step 4D: - If the size of DCI format 0_1 monitored in a UE-specific search space equals that of a DCI format 0_2 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 0_1. - If the size of DCI format 0_1 equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 0_1 - If the size of DCI format 1_1 monitored in a UE-specific search space equals that of a DCI format 1_2 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 1_1. - If the size of DCI format 1_1 equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 1_1. The UE is not expected to handle a configuration that, after applying the above steps, results in - the total number of different DCI sizes configured to monitor is more than 4 for the cell; or - the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell; or - the size of DCI format 0_0 in a UE-specific search space is equal to DCI format 0_1 in another UE-specific search space; or - the size of DCI format 1_0 in a UE-specific search space is equal to DCI format 1_1 in another UE-specific search space; or - the size of DCI format 0_0 in a UE-specific search space is equal to DCI format 0_2 in another UE-specific search space; or - the size of DCI format 1_0 in a UE-specific search space is equal to DCI format 1_2 in another UE-specific search space; or - the size of DCI format 0_2 in a UE-specific search space is equal to DCI format 0_1 in another UE-specific search space; or - the size of DCI format 1_2 in a UE-specific search space is equal to DCI format 1_1 in another UE-specific search space.

Companies are encouraged to check the above text proposal and provide your comments here.

| Company | View |
| --- | --- |

Issue A-2: Resource allocation type 1 with granularity larger than 1 RB for DCI format 0_2/1_2

Regarding resource allocation type 1, description for downlink resource allocation type 1 and for uplink resource allocation type 1 is still missing in TS 38.214, and the number of bits for frequency domain resource assignment for resource allocation type 1 in TS 38.212 is still in bracket. Several companies provide TPs for these aspects.

One issue raised by CATT and WILUS is whether/how to align common PRB grid when grouping PRBs using the configured scheduling granularity, i.e. RBs are grouped into RBGs with considering common PRB grid, which will have impact on the formula to determine the number of bits for “Frequency domain resource assignment” for resource allocation type 1 in TS 38.212, and also have impact on how to do resource allocation assignment in TS 38.214. It seems there are some benefit to do this, like to support better multiplexing with type 0 resource allocation.

Based on the inputs in the contributions, there are three main proposals on the text proposals:

- Option 1: Adopt the following text proposal 1 for TS38.214.

  - Support: CATT, WULUS

Text proposal 1 ======================The Start of Text Proposal of TS38.214======================= ##### 5.1.2.2.2 Downlink resource allocation type 1 In downlink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved or interleaved virtual resource blocks within the active bandwidth part of size PRBs except for the case when DCI format 1_0 is decoded in any common search space in which case the size of CORESET 0 shall be used if CORESET 0 is configured for the cell and the size of initial DL bandwidth part shall be used if CORESET 0 is not configured for the cell. A downlink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by if then else where 1 and shall not exceed . When the DCI size for DCI format 1_0 in USS is derived from the size of DCI format 1_0 in CSS but applied to an active BWP with size of , a downlink type 1 resource block assignment field consists of a resource indication value (RIV) corresponding to a starting resource block and a length in terms of virtually contiguously allocated resource blocks , where is given by - the size of CORESET 0 if CORESET 0 is configured for the cell; - the size of initial DL bandwidth part if CORESET 0 is not configured for the cell. The resource indication value is defined by: if then else where, and where shall not exceed . If , K is the maximum value from set {1, 2, 4, 8} which satisfies ; otherwise K = 1. When the scheduling grant is received with DCI format 1_2, a downlink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting resource block group RBGstart=0,1,…,NRBG-1 and a length in terms of virtually contiguously allocated resource block groups LRBGs=0,1,…,NRBG-1, where the resource block groups are defined in 5.1.2.2.1 with P configured by ResourceAllocationType1-granularity-ForDCIFormat1_2. The resource indication value is defined by replacing LRBs with LRBGs, replacing NBWPsize with NRBG, and replacing RBstart with RBGstart. ===========================Unchanged part is omitted=========================== ##### 6.1.2.2.2 Uplink resource allocation type 1 In uplink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved virtual resource blocks within the active bandwidth part of size PRBs except for the case when DCI format 0_0 is decoded in any common search space in which case the size of the initial UL bandwidth part shall be used. An uplink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by if then else where 1 and shall not exceed. When the DCI size for DCI format 0_0 in USS is derived from the initial UL BWP with size but applied to another active BWP with size of , an uplink type 1 resource block assignment field consists of a resource indication value (RIV) corresponding to a starting resource block and a length in terms of virtually contiguously allocated resource blocks . The resource indication value is defined by if then else where, and where shall not exceed . If , K is the maximum value from set {1, 2, 4, 8} which satisfies ; otherwise K = 1. When the scheduling grant is received with DCI format 0_2, an uplink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting resource block group RBGstart=0,1,…,NRBG-1 and a length in terms of virtually contiguously allocated resource block groups LRBGs=0,1,…,NRBG-1, where the resource block groups are defined in 6.1.2.2.1 with P configured by ResourceAllocationType1-granularity-ForDCIFormat0_2. The resource indication value is defined by replacing LRBs with LRBGs, replacing NBWPsize with NRBG, and replacing RBstart with RBGstart. ===========================Unchanged part is omitted=========================== =======================The END of Text Proposal of TS38.214=====================

- Option 2: Adopt the following text proposal for the DL/UL resource allocation Type 1 for DCI format 1_2 to Sec. 5.1.2.2 and for DCI format 0_2 to Sec. 6.1.2.2 of TS 38.214 with the additions to be amended to the end of this section in red.

  - Support: Nokia

TP to TS 38.214, Sec. 5.1.2.2 - Downlink resource allocation type 1 description completion ##### 5.1.2.2.2 Downlink resource allocation type 1 ## <Unchanged text is omitted> When the scheduling grant is received with DCI format 1_2, a downlink type 1 resource allocation field consists of [TBD]. a resource indication value (RIV) corresponding to a nominal starting virtual resource block and a nominal length in terms of contiguously allocated resource blocks , where K is given by ResourceAllocationType1-granularity-ForDCIFormat1_2 if configured; otherwise K = 1. The resource indication value is defined by if then else where, and , and where 1 and shall not exceed . The actual starting virtual resource block of the downlink type 1 resource allocation given by the resource indication value is defined by if and ; otherwise . The actual length allocation in terms of contiguously allocated resource blocks of the downlink type 1 resource allocation given by the resource indication value is defined by if then elseif and then else . [If the resource allocation indicates the corresponding PDSCH is mapped to RB index , the UE shall assume the PDSCH is also mapped to PRB indexes to .] TP to TS 38.214, Sec. 6.1.2.2 - Uplink resource allocation type 1 description completion ##### 6.1.2.2.2 Uplink resource allocation type 1 ## <Unchanged text is omitted> When the scheduling grant is received with DCI format 0_2, an uplink type 1 resource allocation field consists of [TBD]. a resource indication value (RIV) corresponding to a nominal starting virtual resource block and a nominal length in terms of contiguously allocated resource blocks , where K is given by ResourceAllocationType1-granularity-ForDCIFormat0_2 if configured; otherwise K = 1. The resource indication value is defined by if then else where, and , and where 1 and shall not exceed . The actual starting virtual resource block of the uplink type 1 resource allocation given by the resource indication value is defined by if and ; otherwise . The actual length allocation in terms of contiguously allocated resource blocks of the uplink type 1 resource allocation given by the resource indication value is defined by if then elseif and then else . [If the resource allocation indicates the corresponding PUSCH is mapped to RB index , the UE shall assume the PUSCH is also mapped to PRB indexes to .]

- Option 3: Adopt the following text proposal for the DL/UL resource allocation Type 1 for DCI format 1_2 to Sec. 5.1.2.2 and for DCI format 0_2 to Sec. 6.1.2.2 of TS 38.214 with the additions to be amended to the end of this section in red.

  - Support: Ericsson, NTT DCM

TP for 38.214, Section 5.1.2.2.2 ---------------------------- Start of proposed TP for 38.214 -------------------------------------------- When the scheduling grant is received with DCI format 1_2, a downlink type 1 resource allocation field consists of [TBD]. a resource indication value (RIV) corresponding to a starting virtual resource block RBstart=0,K,2⋅K,…,(NBWPsize/K-1)⋅K and a length in terms of contiguously allocated resource blocks LRBs=K,2⋅K,…,NBWPsize/K⋅K, where K is given by ResourceAllocationType1-granularity-ForDCIFormat1_2 if configured; otherwise K = 1. The resource indication value is defined by if (L'RBs-1)≤N'BWPsize/2 then RIV=N'BWPsize(L'RBs-1)+RB'start else RIV=N'BWPsize(N'BWPsize-L'RBs+1)+(N'BWPsize-1-RB'start) where L'RBs=LRBsK, RB'start=RBstartK and N'BWPsize=NBWPsizeK, RB'start=RBstartK, and where L'RBs≥1 and shall not exceed N'BWPsize-RB'start. [If the resource allocation indicates the corresponding PDSCH is mapped to mapped onto an RB index NBWPsize/K⋅K-1, the UE shall assume the PDSCH is also mapped to that the PDSCH is also extended to include PRB indexes NBWPsize/K⋅K to NBWPsize.] ----------------------------------------------End of proposed TP ----------------------------------------------------

TP for 38.214, Section 6.1.2.2.2 ---------------------------- Start of proposed TP for 38.214 -------------------------------------------- When the scheduling grant is received with DCI format 0_2, an uplink type 1 resource allocation field consists of [TBD]. a resource indication value (RIV) corresponding to a starting virtual resource block RBstart=0,K,2⋅K,…,(NBWPsize/K-1)⋅K and a length in terms of contiguously allocated resource blocks LRBs=K,2⋅K,…,NBWPsize/K⋅K, where K is given by ResourceAllocationType1-granularity-ForDCIFormat0_2 if configured; otherwise K = 1. The resource indication value is defined by if (L'RBs-1)≤N'BWPsize/2 then RIV=N'BWPsize(L'RBs-1)+RB'start else RIV=N'BWPsize(N'BWPsize-L'RBs+1)+(N'BWPsize-1-RB'start) where L'RBs=LRBsK, RB'start=RBstartK and N'BWPsize=NBWPsizeK, RB'start=RBstartK, and where L'RBs≥1 and shall not exceed N'BWPsize-RB'start. [If the resource allocation indicates the corresponding PUSCH is mapped to mapped onto an RB index NBWPsize/K⋅K-1, the UE shall assume the PUSCH is also mapped to that the PUSCH is also extended to include PRB indexes NBWPsize/K⋅K to NBWPsize.] ----------------------------------------------End of proposed TP ----------------------------------------------------

Proposal #2: Further discuss the text proposals for resource allocation type 1 for TS 38.214, taking the above three options as the starting point.

Companies are encouraged to provide your preference and explain your reasons.

| Company | View |
| --- | --- |

As to the text proposal for TS 38.212 for this issue, once the text proposal for 38.214 is achieved, it should be straightforward to achieve.

Issue A-3: Priority indication via DCI format 0_1/1_1 and 0_2/1_2

The following agreements was made in RAN1#99:

Agreement:

When both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP, a DCI format (from the formats 0_1/1_1/0_2/1_2) can be used to schedule PDSCH with different HARQ-ACK priorities or PUSCH with different priorities.

- This feature is UE optional

Both Intel and Qualcomm discuss how to determine the priority of transmissions scheduled by the DCI formats 0_1/1_1/0_2/1_2 for a UE that is not capable of supporting this feature. The following two options were proposed:

- Option 1: Introduce new RRC parameters PriorityForDCIFormat0_2 and PriorityForDCIFormat1_2 to indicate the priority level of the PUSCH and PUCCH scheduled by DCI format 0_2 and 1_2.

  - Support: Qualcomm

- Option 2: DCI formats 0_1/1_1 may only schedule PUSCH or HARQ-ACK transmission with priority index 0, while DCI formats 0_2/1_2 may still schedule PUSCH or HARQ-ACK transmission associated with either priority index 0 or 1.

  - Support: Intel

- Option 3: DCI formats 0_1/1_1 only schedule PUSCH or HARQ-ACK transmission with priority index 0, while DCI formats 0_2/1_2 only schedule PUSCH or HARQ-ACK transmission associated with priority 1.

  - Support:

Proposal #3: Further discuss how to determine the priority of transmissions scheduled by the DCI formats 0_1/1_1/0_2/1_2 for a UE that doesn’t support a DCI format scheduling PDSCH with different HARQ-ACK priorities or PUSCH with different priorities, when both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to monitor.

Companies are encouraged to provide your preference and explain your reasons.

| Company | View |
| --- | --- |

Issue A-4: Conditional or direct configurability of the AP field sizes for DCI format 0_2 and 1_2?

In the current TS 38.212, the following were captured to reflect the agreements from RAN1#99:

- 7.3.1.1.3 Format 0_2: Antenna ports – number of bits determined by the following: - 0 bit if both higher layer parameter dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and higher layer parameter dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 are not configured; - 7.3.1.2.3 Format 1_2 Antenna port(s) – 0, 4, 5, or 6 bits - 0 bit if both higher layer parameter dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and higher layer parameter dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 are not configured;

Based on the current agreements, AP field size can be 0 only if separate DMRS parameters are not configured for DCI formats 0_2 and 1_2. Nokia propose to introduce a separate RRC parameter such as ‘Antenna_ports_field_presence- ForDCIFormat0_2 / Antenna_ports_field_presence- ForDCIFormat1_2’ to control the presence of the field would provide more flexibility. Intel has similar view. The main benefits shown in the Nokia contribution are as below.

- It will allow to independently configure the size of the antenna port(s) fields and the PTRS transmission for higher speed scenarios for DCI formats 0_2 and 1_2. With the current conditional linkage, it will not be possible to have PTRS transmission configured and the antenna port(s) field size set to 0 bit at the same time.

- It will allow to operate URLLC with the configured DM-RS using the first antenna port of the respective tables. This will allow configuring different DM-RS sequence initialization for UL and DL independently. Moreover, MU-MIMO multiplexing e.g. useful especially in UL direction will not be possible when operating with a single DM-RS port.

It seems there is some benefits to support this. However, more inputs are needed before making any decision. Companies are encouraged to show your views.

Proposal #4: Further discuss whether to introduce independent RRC parameters AntennaPorts-FieldPresence-ForDCIFormat0_2 for DCI format 0_2 and AntennaPorts-FieldPresence-ForDCIFormat1_2 for DCI format 1_2, enabling to set the field size of the Antenna port(s) fields to 0.

Companies are encouraged to provide your views here.

| Company | View |
| --- | --- |

Issue A-5: Whether to introduce CSI-SemiPersistentOnPUSCH-TriggerStateList-ForDCIFormat0_2?

In the endorsed RRC parameter sheet, the RRC parameter CSI-SemiPersistentOnPUSCH-TriggerStateList-ForDCIFormat0_2 is still in bracket. Nokia discuss this issue as below:

Nokia R1-2000432 Looking now at the A-CSI trigger state list, there is a similar behavior supported using MAC CE activation of a subset of A-CSI trigger states, when checking 38.214, 38.321 & 38.331 here. The Rel-15 A-CSI trigger state list can have up to 128 entries, but the CSI request field is limited to 6 bits which can at maximum trigger 63 A-CSI states (as state ‘0’ indicates no CSI triggering). A MAC CE (described in 38.321, Sec. 6.1.3.13) is then used to indicate to the UE by a bitmap which of the trigger states actually are used (i.e. the ones with ‘1’ are used) – from 38.214 Sec. 5.1.2.1: When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to trigger states to the codepoints of the CSI request field in DCI. is configured by the higher layer parameter reportTriggerSize where .. Therefore, even though the gNB would configure a smaller CSI triggering size for DCI format 0_2, it can through the MAC CE select any subset from the larger list (e.g. used with DCI format 0_2). From this perspective, there is no need for independent configuration of the CSI-AperiodicTriggerStateList-ForDCIFormat0_2 and anyhow a separate MAC CE for DCI format 0_2 is required for DCI format 0_2 (just due to the fact if having the list size >63 entries). But as discussed, there a is at least a need for separate MAC CE due to the potentially different size of the CSI request field. RAN1 should inform RAN2 that a separate MAC CE of 6.1.3.13 denoted with “Aperiodic CSI Trigger State Subselection MAC CE”. So, summarizing here there is no need for neither of the two lists to be separately configured for DCI format 0_2 and considering the expected RRC overhead, especially the A-CSI trigger list IE contains plenty of RRC parameters which gets rather large. Therefore, we think that RAN1 may remove both RRC parameters from the RRC parameter list or alternatively (which is not our preference) to support both new lists as if separate lists due to some reason, then it should equally apply to A-CSI and SP-CSI triggering. Moreover, there is at a need for a separate MAC CE “Aperiodic CSI Trigger State Subselection MAC CE” for DCI format 0_2 due to the potentially different size of the CSI request field (RAN1 to inform RAN2). Proposal 2.2A: Inform RAN2, that a separate Aperiodic CSI Trigger State Subselection MAC CE will be required for the new DCI format 0_2. Proposal 2.2B: As based on the analysis provided, there is no need for separate configuration of the A-CSI trigger list and SP-CSI trigger list for DCI format 0_2 even though supporting a separate reportTriggerSize, the RRC parameters CSI-AperiodicTriggerStateList-ForDCIFormat0_2 and [CSI-SemiPersistentOnPUSCH-TriggerStateList-ForDCIFormat0_2] from the Rel-16 RRC parameter lists.

Since CSI-AperiodicTriggerStateList-ForDCIFormat0_2 was already agreed, and it can provide more flexibility for the configuration if same RRC entries are used for different DCI formats, probably no need to remove it again. As to whether to introduce separate MAC CE, it seems RAN2 is discussing this issue also, in the case we can leave it for RAN2 to decide. If we keep the RRC parameter CSI-AperiodicTriggerStateList-ForDCIFormat0_2, then it seems ok to introduce the RRC parameter CSI-SemiPersistentOnPUSCH-TriggerStateList-ForDCIFormat0_2 also.  However, companies are encouraged to provide their views on this on the following two options:

- Option 1: Remove the bracket for the RRC parameter CSI-SemiPersistentOnPUSCH-TriggerStateList-ForDCIFormat0_2.

  - Support:

- Option 2: Remove the RRC parameters CSI-AperiodicTriggerStateList-ForDCIFormat0_2 and [CSI-SemiPersistentOnPUSCH-TriggerStateList-ForDCIFormat0_2] from the Rel-16 RRC parameter lists.

  - Support: Nokia

Proposal #5: Further discuss the following two options:

- Option 1: Remove the bracket for the RRC parameter CSI-SemiPersistentOnPUSCH-TriggerStateList-ForDCIFormat0_2.

  - Support:

- Option 2: Remove the RRC parameters CSI-AperiodicTriggerStateList-ForDCIFormat0_2 and [CSI-SemiPersistentOnPUSCH-TriggerStateList-ForDCIFormat0_2] from the Rel-16 RRC parameter lists.

  - Support:

Companies are encouraged to provide your preference and explain your reasons.

| Company | View |
| --- | --- |

Issue A-6: Determination of DCI field sizes for the case of two HARQ-ACK codebooks

In the RAN1#99 meeting, the following agreement was made:

Working assumption:

When the UE is configured with two HARQ-ACK codebooks at least for the case when only one of the two DCI formats (1_1 and 1_2 for DL, 0_1 and 0_2 for UL), configured to support two HARQ-ACK codebooks, is configured to be monitored by the UE, the bit width of the following fields is the maximum of the bit widths for the two configurations corresponding to the two HARQ-ACK codebooks. The necessary number of most significant zero bits can be added to a field to achieve the alignment.

- PDSCH-to-HARQ_feedback timing indicator

- Beta offset indicator

Firstly, Vivo, CATT and Samsung propose to confirm the above working assumption. Secondly, Samsung proposes to extend it to the case that both R-15 and Rel-16 DCI formats can be used to schedule same priority traffic. In addition, Nokia proposes to change “When two HARQ-ACK codebooks are configured for the same serving cell” to “If higher layer parameter PriorityIndicator-ForDCIFormat0_1 is configured” in TS 38.212.

Issue A-7: Whether to change the candidate RV values from {0, 3} to {0, 2} in case of 1 bit for Redundancy version for DCI format 0_2?

Ericsson R1-2000230 For DCI format 1_2 scheduling PDSCH, if only one bit is signalled, the redundancy version to be applied is either 0 or 3. This is a reasonable choice for PDSCH since both RV 0 and 3 are self-decodable for high code rate, and error cases exist where the gNB cannot tell whether the UE received the first transmission and stored the corresponding soft values or not. This is not the case for PUSCH. If the UE does not transmit the PUSCH correctly due to a missed grant, it is possible for the gNB to detect this, e.g. by looking at the noise level estimate based on DMRS. In this case the gNB can schedule the retransmission using RV 0 (basically treating it as the first transmission), which gives better performance than using RV 3 for a first transmission. On the other hand, if the first PUSCH transmission is transmitted correctly, but not decoded at the gNB due to a noisy transmission, the gNB would like to schedule the retransmission using RV 2, and soft combine with the first transmission. This gives better performance than using RV 3, as can be seen in [2] where Figure 1 appears. For this case, LDPC base graph (BG) #1 is used for information block size of K=1056 bits, and two consecutive transmissions are soft combined before decoding. As can be observed from Figure 1, for medium to high code rates above 2/3 (=0.67), the difference between using RV 3 and RV 2 for the second transmission is more than 1.5 dB over an AWGN channel. Figure 1: Required SNR for decoding after two transmissions for different RV orders for BG1. K is the TBS including CRC bits. Dynamically scheduled PUSCH is a case where there is no ambiguity about whether transmission occurred, or which instance of a transmission occurred. Self-decodability is not important for an individual retransmission. Hence the gNB should be able to schedule for best performance, i.e. it should be able to signal RV 2.

More views are needed before making any decision on this issue. Companies are encouraged to provide their views on this.

| Company | View |
| --- | --- |

Miscellaneous corrections

Issue A-8-1: Correction to DMRS reception procedure for PDSCH scheduled by DCI formats 1_2 (38.214, Sec. 5.1.3.2 & 5.1.6.2)

Nokia (R1-2000432) bought up this issues, and propose to adopt the following TP for PDSCH DMRS reception with DCI format 1_2 to Sec. 5.1.3.2 and 5.1.6.2 of TS 38.214 with changes marked in red, to address this issue by implementing the following logic:

- If dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 are not configured, the DMRS reception procedure for PDSCH scheduled by DCI format 1_2 follows the fallback DCI operation (i.e. DCI format 1_0)

- Otherwise (i.e. if configured), the intention of the current clause would apply here.

TP to TS 38.214, DM-RS reception procedure affecting Sec. 5.1.3.2 and 5.1.6.2: #### 5.1.3.2 Transport block size determination In case the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled, then one of the two transport blocks is disabled by DCI format 1_1 if IMCS = 26 and if rvid = 1 for the corresponding transport block. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword. For the PDSCH assigned by a PDCCH with DCI format 1_0, format 1_1 or format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or SI-RNTI, if Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤27, or a table other than Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤28, the UE shall, except if the transport block is disabled in DCI format 1_1, first determine the TBS as specified below: 1) The UE shall first determine the number of REs (NRE) NRE)within the slot. - A UE first determines the number of REs allocated for PDSCH within a PRB () by , where is the number of subcarriers in a physical resource block, Nsymbslot is the number of symbols of the PDSCH allocation within the slot, NDMRSPRB is the number of REs for DM-RS per PRB in the scheduled duration including the overhead of the DM-RS CDM groups without data, as indicated by DCI format 1_1 or format 1_2 or as described for format 1_0 and format 1_2 in Clause 5.1.6.2, and NohPRBis the overhead configured by higher layer parameter xOverhead in PDSCH-ServingCellConfig. If the xOverhead in PDSCH-ServingCellconfig is not configured (a value from 0, 6, 12, or 18), the is set to 0. If the PDSCH is scheduled by PDCCH with a CRC scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI or P-RNTI, is assumed to be 0. - A UE determines the total number of REs allocated for PDSCH ()NRE) by NRE= NRE'* nPRB, where nPRB is the total number of allocated PRBs for the UE. #### <Unchanged text is omitted> #### 5.1.6.2 DM-RS reception procedure If both higher layer parameter dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and higher layer parameter dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 are not configured, the DM-RS reception procedures for PDSCH scheduled by PDCCH with DCI format 1_0 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2; Otherwise, Tthe DM-RS reception procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clausesection equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 instead of dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB. When receiving PDSCH scheduled by DCI format 1_0 or receiving PDSCH before dedicated higher layer configuration of any of the parameters dmrs-AdditionalPosition, maxLength and dmrs-Type, the UE shall assume that the PDSCH is not present in any symbol carrying DM-RS except for PDSCH with allocation duration of 2 symbols with PDSCH mapping type B (described in clause 7.4.1.1.2 of [4, TS 38.211]), and a single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 1000 is transmitted, and that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE and in addition ## <Unchanged text is omitted>

Issue A-8-2: Correction to DMRS transmission procedure for PUSCH scheduled by DCI format 0_2 (38.214, Sec. 6.1.4.2 & 6.2.2)

Nokia (R1-2000432) bought up this issues, and propose to adopt the following text proposal for PUSCH DMRS transmission with DCI format 0_2 to Sec. 6.1.4.2 and 6.2.2 of TS 38.214 with changes /additions marked in red, to address this issue by implementing the following logic:

- If dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat1_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat1_2 are not configured, the DMRS transmission procedure for PUSCH scheduled by DCI format 0_2 follows the fallback DCI operation (i.e. DCI format 0_0)

- Otherwise (i.e. if configured), the procedures of PUSCH scheduled by DCI format 0_1 apply also for PUSCH scheduled by DCI format 0_2 by using the separately configured DMRS parameters.

TP to TS 38.214, Sec. 6.1.4.2 & 6.2.2: UE DM-RS transmission procedure description for DCI format 0_2 #### 6.1.4.2 Transport block size determination For a PUSCH scheduled by RAR UL grant or for a PUSCH scheduled by fallbackRAR UL grant or for a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or for a PUSCH scheduled by a DCI format 0_1 or DCI format 0_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, SP-CSI-RNTI, or for a PUSCH transmission with configured grant, or for a MsgA PUSCH transmission, if - and transform precoding is disabled and Table 5.1.3.1-2 is used, or - and transform precoding is disabled and a table other than Table 5.1.3.1-2 is used, or - and transform precoding is enabled, the UE shall first determine the TBS as specified below: The UE shall first determine the number of REs (NRE) within the slot: - A UE first determines the number of REs allocated for PUSCH within a PRB by - , where is the number of subcarriers in the frequency domain in a physical resource block, Nsymbslotis the number of symbols L of the PUSCH allocation according to Clause 6.1.2.1 for scheduled PUSCH of Clause 6.1.2.3 for configured PUSCH, is the number of REs for DM-RS per PRB in the allocated duration including the overhead of the DM-RS CDM groups without data, as described for PUSCH with a configured grant in Clause 6.1.2.3 or as indicated by DCI format 0_1 or DCI format 0_2 or as described for DCI format 0_0 and DCI format 0_2 in Clause 6.2.2, and is the overhead configured by higher layer parameter xOverhead in PUSCH-ServingCellConfig. If the is not configured (a value from 6, 12, or 18), the is assumed to be 0. For Msg3 transmission the is always set to 0. <Unchanged text is omitted> ### 6.2.2 UE DM-RS transmission procedure If both higher layer parameter dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and higher layer parameter dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 are not configured, the DM-RS transmission procedures for PUSCH scheduled by PDCCH with DCI format 0_0 described in this clause equally apply to PUSCH scheduled by PDCCH with DCI format 0_2; Otherwise, the DM-RS transmission procedures for PUSCH scheduled by PDCCH with DCI format 0_1 described in this clause equally apply to PUSCH scheduled by PDCCH with DCI format 0_2, by applying the parameters of dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 instead of dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB. When transmitted PUSCH is neither scheduled by DCI format 0_1 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, nor corresponding to a configured grant, nor being a PUSCH for Type-2 random access procedure,. the UE shall use single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 0 and the remaining REs not used for DM-RS in the symbols are not used for any PUSCH transmission except for PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, additional DM-RS can be transmitted according to the scheduling type and the PUSCH duration as specified in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled, and If frequency hopping is disabled: - The UE shall assume dmrs-AdditionalPosition equals to 'pos2' and up to two additional DM-RS can be transmitted according to PUSCH duration, or If frequency hopping is enabled: - The UE shall assume dmrs-AdditionalPosition equals to 'pos1' and up to one additional DM-RS can be transmitted according to PUSCH duration. ## <Unchanged text is omitted>

Issue A-8-3: Missing PTRS transmission procedure for PUSCH scheduled by DCI format 0_1 & 0_2 (38.214 - Sec. 6.2.3 & 6.2.3.1, 38.212 – Sec. 7.3.1.1.2 & 7.3.1.1.3)

RAN1 had agreed to support independent PTRS configuration which is available due to the independent DMRS configurations for DCI format 0_2 and Rel-15 containing the RRC parameter phaseTrackingRS in DMRS-UplinkConfig.

Agreements:

Support new RRC configuration for “PTRS-DMRS association” in DCI format 0_2

Currently, the effect on PTRS is currently not at all addressed in Sec. 6.2.3 of TS 38.214. Nokia proposes to adopt the following TP for PDSCH PTRS transmission with DCI format 0_2 to Sec. 6.2.3 & 6.2.3.1 of TS 38.214 with changes marked in red:

TP to TS 38.214, 6.2.3 and 6.2.3.1: UE PTRS transmission procedure description for DCI format 0_2 ### 6.2.3 UE PT-RS transmission procedure The procedures on PT-RS transmission described in this clause as well as clauses 6.2.3.1 and 6.2.3.2 apply to a UE PUSCH transmission scheduled by DCI format 0_2 if the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 or dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 is configured, to PUSCH transmissions scheduled by DCI format 0_0 or format 0_1 if the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB is configured and PUSCH transmissions corresponding to a configured grant if the higher layer parameter phaseTrackingRS in cg-DMRS-Configuration is configured. If a UE is not configured with the higher layer parameter phaseTrackingRS in the respective DMRS-UplinkConfig, the UE shall not transmit PT-RS. The PTRS is only present on PUSCH scheduled by PDCCH with CRC scrambled by MCS-C-RNTI, C-RNTI, CS-RNTI, SP-CSI-RNTI and on PUSCH corresponding to a configured grant. For PUSCH repetition Type B, the PT-RS transmission procedure is applied for each actual repetition separately based on the allocation duration of the actual repetition. #### 6.2.3.1 UE PT-RS transmission procedure when transform precoding is not enabled ## <Unchanged text is omitted> For codebook or non-codebook based UL transmission, the association between UL PT-RS port(s) and DM-RS port(s) is signalled by PTRS-DMRS association field in DCI format 0_1 and DCI format 0_2. For a PUSCH corresponding to a configured grant Type 1 transmission, the UE may assume the association between UL PT-RS port(s) and DM-RS port(s) defined by value 0 in Table 7.3.1.1.2-25 or value "00" in Table 7.3.1.1.1.2-26 described in Clause 7.3.1 of [5, TS38.212]. For PUSCH scheduled by DCI format 0_0 or by activation DCI format 0_0, the UL PT-RS port is associated to DM-RS port 0. For non-codebook based UL transmission, the actual number of UL PT-RS port(s) to transmit is determined based on SRI(s) in DCI format 0_1 and DCI format 0_2 or higher layer parameter sri-ResourceIndicator in rrc-ConfiguredUplinkGrant. A UE is configured with the PT-RS port index for each configured SRS resource by the higher layer parameter ptrs-PortIndex configured by SRS-Config if the UE is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig. If the PT-RS port index associated with different SRIs are the same, the corresponding UL DM-RS ports are associated to the one UL PT-RS port. For partial-coherent and non-coherent codebook based UL transmission, the actual number of UL PT-RS port(s) is determined based on TPMI and/or number of layers which are indicated by Precoding information and number of layers field in DCI format 0_1 and DCI format 0_2 or configured by higher layer parameter precodingAndNnumberOfLayers: - if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n2', the actual UL PT-RS port(s) and the associated transmission layer(s) are derived from indicated TPMI as: - PUSCH antenna port 1000 and 1002 in indicated TPMI share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI share PT-RS port 1. - UL PT-RS port 0 is associated with the UL layer [x] of layers which are transmitted with PUSCH antenna port 1000 and PUSCH antenna port 1002 in indicated TPMI, and UL PT-RS port 1 is associated with the UL layer [y] of layers which are transmitted with PUSCH antenna port 1001 and PUSCH antenna port 1003 in indicated TPMI, where [x] and/or [y] are given by DCI parameter PTRS-DMRS association as shown in DCI format 0_1 and DCI format 0_2 described in Clause 7.3.1 of [5, TS38.212]. ## <Unchanged text is omitted>

Similarly, Nokia proposes to adopt the following text proposal on the PTRS-DMRS association field for DCI format 0_1 and 0_2 to Sec. 7.3.1.1.2 & 7.3.1.1.3 of TS 38.212 with changes marked in red:

TP to TS 38.212, 7.3.1.1.2 and 7.3.1.1.3: Correction to DCI field size determination for PTRS-DMRS association ##### 7.3.1.1.2 Format 0_1 ## <Unchanged text is omitted> - PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig in dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB is not configured and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1; - 2 bits otherwise, where Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) for transmission of one PT-RS port and two PT-RS ports respectively, and the DMRS ports are indicated by the Antenna ports field. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part. ## <Unchanged text is omitted> ##### 7.3.1.1.3 Format 0_2 ## <Unchanged text is omitted> - PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig in dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 or dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 is not configured and transform precoder is disabled, or if transform precoder is enabled, or if maxRank-ForDCIFormat0_2=1; - 2 bits otherwise, where Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) for transmission of one PT-RS port and two PT-RS ports respectively, and the DMRS ports are indicated by the Antenna ports field. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part. ## <Unchanged text is omitted>

Some other issues are raised in Nokia R1-2000431, Sharp R1-200081, OPPO R1 R1-2000479 and ASUSTeK R1-2000995. Due to the time limit, probably we are not able to discuss it this meeting. Companies are encouraged to check the issues there.

Enhanced PDCCH monitoring capability

This section summarize the issues on enhanced PDCCH monitoring capability.

The maximum number of non-overlapped CCEs per slot for channel estimation

Issue B-1: The per-CC limit on the maximum number of non-overlapping CCEs per monitoring span for (2, 2) and (4, 3)

No agreement has been achieved so far on the number of non-overlapping CCEs for span patterns (2, 2) and (4, 3) for the SCS 15 kHz and 30 kHz. This issue has been extensively discussed during the work item but companies have not been able to converge.

Currently, the maximum numbers of non-overlapping CCEs are captured as follows in the specification.

Table 10.1-3A: Maximum number CPDCCHmax,X,Y,μ of non-overlapped CCEs in a span of a span pattern (X, Y) for a DL BWP with SCS configuration μ∈0, 1 for a single serving cell Maximum number CPDCCHmax,X,Y,μ of non-overlapped CCEs per span pattern X,Y and per serving cell. μ; (2, 2); (4, 3); (7, 3). 0; C01; C02; 56. 1; C11; C12; 56.

The views from companies are summarized as below:

| Company | 15 kHz SCS |  | 30 kHz SCS |  |
| --- | --- | --- | --- | --- |
|  | (2, 2) | (4, 3) | (2, 2) | (4, 3) |
| Ericsson [2] | 32 | 48 | 32 | 48 |
| vivo [3] | 24 | 48 | 24 | 48 |
| Nokia [5] | 32 | 48 | 32 | 48 |
| MTK [6] | 24 | 48 | 24 | 48 |
| OPPO [7] | - | - | 16 | 32 |
| CATT [8] | 24 | 32 | 24 | 32 |
| Samsung [10] | 48 | 56 | 48 | 56 |
| Intel [12] | [16-20] | [36-42] | [16-20] | [36-40] |
| China Unicom [13] | 16 | 36 | 16 | 36 |
| Apple [14] | 18 | 32 | 18 | 32 |
| Spreadtrum [16] | 16 | 32 | 16 | 32 |
| NTT DCM [17] | 32 | 56 | 32 | 32 |
| Qualcomm [19] | 16 | 36 | 16 | 36 |
| Huawei/HiSi [21] | 28 | 36 | 28 | 36 |

The issues was discussed extensively and unfortunately no consensus was achieved. Based on the views in the contributions, it seems same situation is still there. On the one hand, UE vendors cannot provide a higher number due to UE complexity, on the other hand gNB vendors prefer a larger value and prefer less UE capabilities. In RAN1#99 meeting, we were discussing to get the following proposal as working assumption as shown in R1-1913541, however no consensus was achieved due to the strong concern from a few companies.  Another one possible way is to take multiple values for each combination and let UE report it as UE capability, though it seems some companies don’t like way either. Let’s take these two options as the starting point for discussion, companies are encouraged to provide your preference.

Proposal #1: For limit C on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span,

- Option 1：

  - The value of C for combination (4, 3) for 15 kHz and 30 kHz is 32.

  - The value of C for combination (2, 2) for 15 kHz and 30 kHz is 18.

- Option 2：

  - For the value of C for combination (4, 3) for 15 kHz and 30 kHz, UE can report one of following values as a UE capability:

    - 32

    - 48

  - For the value of C for combination (2, 2) for 15 kHz and 30 kHz, UE can report one of following values as a UE capability:

    - 16

    - 32

Companies are encouraged to provide your views. If you have strong objection on the framework (i.e. one value or multiple values as UE capability) of either option 1 or option 2, please indicate here also.

| Company | View |
| --- | --- |

Issue B-2: The per-CC limit on the maximum number of monitored PDCCH candidates per monitoring span

No agreement has been achieved on the number of blind decodes for (2, 2), (4, 3) and (7, 3) for the SCS 15 kHz and 30 kHz, nor on the number of supported carriers. Although an enhancement of the number of BDs is not explicitly in the scope of the WID [1], these issues have been extensively discussed during the work item but companies have not been able to converge.

The maximum numbers of monitored PDCCH candidates per span are captured as follows in the specification:

Table 10.1-2A: Maximum number MPDCCHmax,X,Y,μ of monitored PDCCH candidates in a span of a span pattern (X, Y) for a DL BWP with SCS configuration μ∈0, 1 for a single serving cell Maximum number MPDCCHmax,X,Y,μ of monitored PDCCH candidates per span pattern X,Y and per serving cell. μ; (2, 2); (4, 3); (7, 3). 0; M01; M02; M03. 1; M11; M12; M13.

The views from companies are summarized as below:

| Company | 15 kHz SCS |  |  | 30 kHz SCS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | (2,2) | (4,3) | (7,3) | (2,2) | (4,3) | (7,3) |
| Ericsson [2] | 28 | 32 | 44 | 24 | 28 | 36 |
| Vivo [3] | 18 | 36 | 44 | 14 | 28 | 36 |
| Nokia [5] | 18 | 36 | 44 | 14 | 28 | 36 |
| MTK [6] | 14 | 30 | 44 | 12 | 24 | 36 |
| OPPO [7] | - | - | - | 6 | 12 | 18 |
| Samsung [10] | 12 | 28 | 44 | 10 | 24 | 36 |
| Intel [12] | 10-14 | 20-28 | 36-44 | 8-12 | 16-24 | 28-36 |
| Apple [14] | 12 | 30 | 44 | 10 | 24 | 36 |
| Spreadtrum [16] | 6 | 11 | 22 | 5 | 9 | 18 |
| NTT DCM [17] | 12 | 30 | 44 | 10 | 24 | 36 |
| Qualcomm [19] | 12 | 28 | 44 | 10 | 24 | 36 |
| HW/HiSi [21] | 8 | 18 | 28 | 8 | 18 | 28 |

Unfortunately the views are very diverse. Similar as the capability for CCE, the following proposal are given as the starting point for discussion. Note that the values got the most support are given in option 1 in the following proposal. For option 2, really difficult for what value to pick, some medium values are chosen here.

Proposal #2: For limit M on the maximum number of monitored PDCCH candidates per monitoring span,

- Option 1：

  - For 15 kHz,

    - The value of M03 for combination (7, 3) is 44

    - The value of M02 for combination (4, 3) is 28

    - The value of M01 for combination (2, 2) is 12

  - For 30 kHz,

    - The value of M13 for combination (7, 3) is 36

    - The value of M12 for combination (4, 3) is 24

    - The value of M11 for combination (2, 2) is 10

- Option 2：

  - For 15 kHz,

    - UE can report one of following values for combination (7, 3) as a UE capability:

      - 44

      - 28

    - UE can report one of following values for combination (4, 3) as a UE capability:

      - 30

      - 18

    - UE can report one of following values for combination (2, 3) as a UE capability:

      - 18

      - 10

Companies are encouraged to provide your views. If you have strong objection on the framework (i.e. one value or multiple values as UE capability) of either option 1 or option 2, please indicate here also.

| Company | View |
| --- | --- |

From some companies’ contributions, e.g. [5], [21], it has further been observed that to simply divide a slot limit equally by all spans does not necessarily allow for high scheduling flexibility in the spans where it is mostly needed. If asymmetric limits would be allowed, the need for BDs per span could possibly be reduced. This could also be one option to converge on numbers that are agreeable for everyone.

Issue B-3: Capability on the number of CCs with Rel-16 monitoring capability

In the RAN1#99 meeting, the following agreement was made:

Agreement from RAN1#99 UE reports its PDCCH monitoring capability for the following cases: - Case 1: Capability on the number of CCs with Rel-15 monitoring capability only - This capability already exists in Rel-15 - Case 2: Capability on the number of CCs with Rel-16 monitoring capability only - pdcch-BlindDetectionCA-R16 can be smaller than 4 - Case 3: Capability on the number of CCs with Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability - pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability - Each of pdcch-BlindDetectionCA-R16 and pdcch-BlindDetectionCA-R15 can be smaller than 4 - (The minimum of pdcch-BlindDetectionCA-R15 + The minimum of pdcch-BlindDetectionCA-R16) is not larger than 4 - FFS (the minimum of pdcch-BlindDetectionCA-R15 + the minimum of pdcch-BlindDetectionCA-R16) can be smaller than 4 pdcch-BlindDetectionCA-R15 and pdcch-BlindDetectionCA-R16 for the above three cases can be reported separately

The view from companies are summarized as below:

| Company | Supported #CCs for mixed Rel-16/Rel-15 monitoring | Feature lead comment |
| --- | --- | --- |
| Ericsson [2] | 4 | Comment #1: From feature lead perspective, it means that UE can support the combination of (#Rel-16, #Rel-15) as below: (4, 0) (3, 1) (2, 2) (1, 3) (0, 4) |
| Nokia | 4 | Similar as above comment #1 |
| MTK [6] | <4 (1 Rel 16 + 2 Rel-15) |  |
| CATT [8] | 4 | Similar as above comment #1 |
| Samsung [10] | 4 | May support this value |
| Spreadtrum [16] | < 4 |  |
| Qualcomm [19] | <4 (1 Rel + 2 Rel-15) |  |
| HW/HiSi [21] | 4 | Similar as above comment #1 |

In addition, some companies also provide their view on pdcch-BlindDetectionCA-R16 for case 2, e.g. 2. It is observed that the number of CC to be supported may be related to the value of M also. For example, it is possible that if the value of M is smaller, then more CC with Rel-16 capability can be supported, while a larger value of M may result in smaller number of CC to be supported.

Proposal #3: Further study the following two options for the number of CCs with Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells

- Option 1：Minimum of pdcch-BlindDetectionCA-R15 + Minimum of pdcch-BlindDetectionCA-R1 = 4

- Option 2：The minimum value of pdcch-BlindDetectionCA-R15 is 2 and the minimum value of pdcch-BlindDetectionCA-R16 is 1

Companies are encouraged to provide your views and your preference.

| Company | View |
| --- | --- |

Proposal #4: Further study the following two options for the number of CCs with Rel-16 monitoring capability and Rel-16 monitoring capability only

- Option 1：Minimum of pdcch-BlindDetectionCA-R16 = 4

- Option 2：Minimum of pdcch-BlindDetectionCA-R16 = 2

Companies are encouraged to provide your views and your preference.

| Company | View |
| --- | --- |

Issue C-1: PDCCH candidate overbooking and dropping

To support PDCCH candidate overbooking and dropping is a complex function for the UE. In Rel-15 it is performed on a slot level basis, if it is performed for every span in Rel-16, then this can increase the complexity substantially, e.g. [19].

Therefore an agreement was reached in RAN1#99, where it is FFS if overbooking and dropping is performed in every span.

| Agreement from RAN1#99 PDCCH monitoring can be configured based on either Rel-15 capability (i.e. per slot based capability) or Rel-16 capability (i.e. per span based capability) on a serving cell - gNB configures which capability is used - For Rel-16 PDCCH monitoring capability, - The limit C on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span is the same across different spans within a slot, each span can cover CSS and/or USS - PDCCH dropping is performed in a span if needed - PDCCH overbooking and PDCCH dropping are only allowed on PCell and PSCell - FFS PDCCH overbooking and PDCCH dropping are not performed in all spans in a slot - PDCCH overbooking and PDCCH dropping are only performed in the span with CSS present - The maximum number of monitored PDCCH candidates per span is - M1 per span for (2, 2) - M2 per span for (4, 3) - M3 per span for (7, 3) - Note: - The total number of monitored PDCCH candidates is not smaller than the limit per slot in Rel-15 - The value of M1, M2 and M3 can be different and SCS dependent Note: PDCCH overbooking and PDCCH dropping are not performed per slot |  |
| --- | --- |

The views are summarized as below:

| Source | Option A Overbooking/ Dropping in every span | Option B Overbooking /Dropping only in Spans with CSS | Additional information |
| --- | --- | --- | --- |
| Ericsson [2] | X |  |  |
| vivo [3] |  | X |  |
| ZTE [4] | X |  |  |
| CATT [8] |  | X |  |
| Panasonic [9] |  | X |  |
| LG [11] |  | [X] | LG proposes to support overbooking/dropping on a sub-sets of span in the slot, e.g. the first “k” spans, but not necessarily on the spans that contain CSS. |
| China Unicom [13] |  | X |  |
| Apple [14] |  | X |  |
| Spreadtrum [15] |  | X |  |
| Qualcomm [19] |  | [X] | Dropping is not performed in all spans. Only in a fix span, e.g. always the first span. |
| HW/HiSi [21] |  | [X] | The main concern is that dropping and overbooking is not done in all spans of a slot. To limit to CSS seems a good choice. But group common CSS that can be configured in every span should then be excluded as well. |
| Total | 2 | 9 |  |

A majority number of companies wants to supports PDCCH overbooking/dropping in some spans. Probably we can take the following proposal as the starting point for email discussion.

Proposal #5: PDCCH overbooking and PDCCH dropping are only allowed on a subset of spans within a slot.

- Option 1：Only performed in the span(s) with CSS present;

  - FFS whether type-3 CSS is included or not

- Option 2：Only performed in the first X span(s) within a slot;

  - FFS the value of X, e.g. 1

Companies are encouraged to provide your views on the above proposal.

| Company | View |
| --- | --- |

Issue C-2: How to scale the monitoring capability if the number of CCs configured is larger than the reported capability?

It still remains open how to handle the CA case when the reported capability is less than the actual configured number of CCs. Ericsson, ZTE, Nokia, Samsung, Intel, WILUS and ASUSTeK provide their views. Note that both Samsung and Ericsson mentioned the impact from multi-TRP, however whether and how to support the combination of multi-TRP may need some separate discussion. Here let’s focus on single TRP case first. Based on the inputs from companies, the following proposals can be considered as the starting point for the email discussion:

Proposal #6: If a UE is configured with serving cells with Rel-16 PDCCH monitoring capability with a span pattern (X, Y) and SCS configuration µ, the maximum number of non-overlapping CCEs per span a UE monitors for these serving cells are given by

, where

  - is the number serving cells configured with Rel-16 PDCCH monitoring capability with SCS configuration j. If a UE is configured with multiple carriers with a mix of Rel-15 and Rel-16 PDCCH monitoring capability,  is replaced by .

Companies are encouraged to provide your views on the above proposals.

| Company | View |
| --- | --- |

Proposal #7: If a UE is configured with serving cells with Rel-16 PDCCH monitoring capability with a span pattern (X, Y) and SCS configuration µ, the maximum number of monitored PDCCH candidates per span a UE monitors for these serving cells are given by

, where

  - is the number serving cells configured with Rel-16 PDCCH monitoring capability with SCS configuration j. If a UE is configured with multiple carriers with a mix of Rel-15 and Rel-16 PDCCH monitoring capability,  is replaced by .

Companies are encouraged to provide your views on the above proposals.

| Company | View |
| --- | --- |

Please note that for the case of Rel-16 and Rel-15 capability on different serving cells, based on the above proposals, the scaling can be done within the cells configured with Rel-15 capability and Rel-16 capability separately.

Issue C-3: Correction on span definition

In the current specification, span pattern is defined as below:

TS 38.213 V16.0.0 (2019-12) # 10 UE procedure for receiving control information A span pattern is a time configuration within a slot and per slot for PDCCH monitoring on an active DL BWP of a serving cell. A span pattern is defined by a pair of numbers of symbols X,Y with X≥Y. X is a minimum number of consecutive symbols between first symbols of two PDCCH monitoring occasions in two respective consecutive span patterns and defines a span, and Y is a number of consecutive symbols for PDCCH monitoring occasions within the X symbols, starting from the first symbol of the X symbols, and defines a span gap.

Huawei/HiSilicon [21] and Sharp [15] discuss some corrections for span definition.

Contribution R1-2000881, Sharp According to the above description of the span pattern and (X, Y), it seems that the span pattern in the 38.213 is equivalent to the UE reported combination (X, Y). Agreements as below reached in RAN1#97 has shown that UE feature 3-5b should be taken into considered as a starting point when discussing the PDCCH monitoring span. The span pattern and (X, Y) has been described above in TS 38.213 V16.0.0 [4] against the understanding of the span pattern and (X, Y) upon which the relevant agreements has been reached. According to the 3-5b, a span pattern should be deterministic according to the multiple UE reported combinations (X, Y) and CORESET/search space set configuration. It thus should be better to decouple the span pattern from the UE reported combination (X, Y). The description in UE feature 3-5b helps to illustrate the span pattern and the combination (X, Y) and should be captured for describing the span pattern and the (X, Y) in the TS38.213. Moreover, the above description related to (X, Y) as like X defines a span and the Y defines a span gap seems to be not align the original intention of the combination (X, Y). The description for pdcch-MonitoringAnyOccasionsWithSpanGap in TS 38.306 can be used to refine the description of the combination (X, Y). Proposal 2: Adopt the TP2 to correct the description for span pattern and UE reported combination (X, Y) in section 10 and 10.1 in the TS 38.213. An agreement as below reached in RAN1#98 resolved which maximum number CPDCCHmax,X,Y,μof non-overlapped CCEs to be applied when there are multiple valid combinations (X, Y) for the span pattern. Whether the combination is valid or not lies in whether the minimum time separation during the span pattern satisfies the (X, Y). The current TS 38.213 description ‘If the UE indicates a capability to monitor PDCCH according to multiple span patterns and a configuration of search space sets to the UE results to a separation of two PDCCH monitoring occasions in two respective consecutive span patterns that is equal to or larger than the value of X for two or more of the multiple span patterns, the UE is expected to monitor PDCCH according to the span pattern associated with the largest maximum number of CPDCCHmax,X,Y,μ.’, especially for the yellow part, cannot completely reflect the agreement when the separation between two consecutive spans for a span pattern may be unequal. Therefore, the following TP3 is to clarify the agreement below. Agreements in RAN1#98:: If UE reports the support of more than one combination of C(X, Y) for a given SCS, and if multiple combinations of C(X, Y) are valid for the span pattern, the maximum value of C of the valid combinations is applied. - A combination C(X, Y) is valid if the span pattern satisfies X and Y of the given combination in every slot, including cross slot boundary FFS the impact from empty span(s) on the span pattern. Proposal 3: Adopt the TP3 to reflect the agreement related to how to determine a valid combination (X, Y).

Companies are encouraged to check the TPs in R1-1000881.

Proposal #8: Further study TP2 in R1-2000881 to correct the description for span pattern and UE reported combination (X, Y) in section 10 and 10.1 in the TS 38.213.

Proposal #9: Further study TP3 in R1-2000881 to reflect the agreement related to how to determine a valid combination (X, Y) in section 10.1 in the TS 38.213.

In addition, switching between Rel-15 and Rel-16 PDCCH monitoring configurations is also discussed in R1-2000735 (Intel). Companies are encouraged to check and probably we can discuss it next meeting.

- References

- RP-191584, “Revised WID: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC)”, RAN#84, Newport Beach, CA, June 3-6, 2019.

- R1-2000230	Remaining Issue of PDCCH Enhancements for NR URLLC	Ericsson

- R1-2000325	PDCCH enhancements for URLLC	vivo

- R1-2000355	Remaining issues on PDCCH enhancements for NR URLLC	ZTE

- R1-2000432	Maintenance of Rel-16 URLLC PDCCH enhancements	Nokia, Nokia Shanghai Bell

- R1-2000451	Remaining issues on PDCCH enhancements	MediaTek Inc.

- R1-2000479	PDCCH enhancements for URLLC	OPPO

- R1-2000527	Remaining issues on PDCCH enhancements	CATT

- R1-2000587	PDCCH enhancements for NR URLLC	Panasonic Corporation

- R1-2000625	Remaining issues for PDCCH enhancements	Samsung

- R1-2000670	Remaining issues of PDCCH enhancements for NR URLLC	LG Electronics

- R1-2000735	Remaining aspects on Rel-16 PDCCH enhancements for URLLC	Intel Corporation

- R1-2000855	Remaining Details on PDCCH Enhancements	Apple

- R1-2000881	Remaining issues on PDCCH enhancements for NR URLLC	Sharp

- R1-2000896	Discussion on remaining issues of PDCCH enhancements for URLLC	Spreadtrum Communications

- R1-2000919	Remaining issues for PDCCH enhancements for Rel.16 URLLC	NTT DOCOMO, INC.

- R1-2000935	Remaining issues on PDCCH enhancement for NR URLLC	WILUS Inc.

- R1-2000968	Remaining issues on PDCCH Enhancements for URLLC	Qualcomm Incorporated

- R1-2000995	Remaining issue for TCI field	ASUSTeK

- R1-2001023	Corrections on PDCCH enhancement for URLLC	Huawei, HiSilicon