3GPP TSG RAN WG1 Meeting #100	R1-2001074

e-Meeting, February 24th – March 6th, 2020

| Source: | Nokia, Nokia Shanghai Bell |
| --- | --- |
| Title: | Discussion of need for correction for inconsistent shadow fading parameters in NTN rural scenario |
| Document for: | Decision |
| Agenda Item: | 7.1.6 |

# 1 Introduction

Channel model for non-terrestrial networks evaluations is specified in TR 38.811 chapter 6, where clause 6.6.2 defines path loss and shadow fading models, and clause 6.7.2 covers the details of frequency selective fading.  A number of channel model parameter tables (Tables 6.7.2-1a to 6.7.2-8b) are provided in clause 6.7.2 for various environment scenarios. Most of the parameters in those tables are related to fast fading, but shadow fading (SF) standard deviation is listed there for the purpose of parameter cross-correlation and correlation distance specifications. The values of SF parameter, however, should be consistent with the SF values in clause 6.7.2, where the shadow fading model is normatively specified.

In the latest revision of TR 38.811 [1], we found that SF parameters for rural scenario in Tables 6.7.2-7a, 6.7.2-7b, 6.7.2-8a, 6.7.2-8b are inconsistent with the SF parameter values given in clause 6.6.2 (or Table 6.6.2-3). To illustrate this problem, Table 6.6.2-3 and part of Table 6.7.2-7a in [1] are attached below with the conflicting SF values highlighted.

The rural scenario model is the baseline channel model for system level calibration in [2]. The rural environment is also a dominant signal propagation scenario for NTN deployment. We consider it imperative to fix the inconsistency before the R-17 NTN work item starts to avoid unnecessary misunderstanding and incorrect simulation results.

# 2 Proposal

A corresponding CR draft is provided in [4]. The SF values in all channel model parameter tables (Tables 6.7.2-1a to 6.7.2-8b) are replaced by a reference to a corresponding table in clause 6.6.2 where normative SF parameter values are specified, following the same manner the SF parameter is given in the NR channel model TR 38.901 [3].

# 3 Conclusion

Inconsistency of shadow fading parameters in NTN rural channel model scenario has been identified. We recommend RAN1 endorse the Rel-15 CR [4] for TR 38.811.

# References

- 3GPP TR 38.811 v15.2.0: "Study on New Radio (NR) to support non-terrestrial networks (Release 15)"

- 3GPP TR 38.821 v16.0.0: "Solutions for NR to support non-terrestrial networks (Release 16)"

- 3GPP TR 38.901 v16.1.0: “Study on channel model for frequencies from 0.5 to 100 GHz (Release 16)”

- R1-20abcde, “Correction for inconsistent shadow fading parameters in NTN rural scenario”, Nokia, Nokia Shanghai Bell