3GPP TSG RAN WG1 Meeting #100-e	R1-2001106

February 24 – March 6, 2020

Agenda Item:	7.2.5.7

Source:	Huawei, HiSilicon

Title:	Corrections on other aspects for URLLC/IIOT enhancements

Document for:	Discussion and Decision

# Introduction

During the RAN1 Rel-16 discussion, some enhancements have been introduced for DL SPS, which mainly include multiple SPS PDSCH configurations and the HARQ-ACK codebook enhancement to support these multiple SPS PDSCH configurations. This contribution mainly discusses the corrections on DL SPS enhancements.

# Corrections on DL SPS enhancements

The following two agreements have been achieved in the RAN1 #99 meeting. They define the HARQ-ACK bit order for SPS PDSCH reception without a corresponding PDCCH. This order could be used for the case where only HARQ-ACK feedback for one or more SPS PDSCH receptions without a corresponding PDCCH shall be reported and for the case where HARQ-ACK feedback for one or more SPS PDSCH receptions without a corresponding PDCCH is multiplexed with HARQ-ACK feedback for dynamic scheduled PDSCH and/or for SPS PDSCH release.

Agreement If the UE is configured with more than one SPS PDSCH configurations, for cases where only HARQ-ACK feedback for one or more SPS PDSCH receptions without a corresponding PDCCH shall be reported (i.e. no HARQ-ACK feedback for dynamic scheduled PDSCH and/or for SPS PDSCH release - HARQ-ACK bit order for SPS PDSCH reception without a corresponding PDCCH is determined - In ascending order of DL slot per {SPS configuration index, serving cell index}, and then in ascending order of SPS configuration index per {serving cell index}, and then in ascending order of serving cell index Agreement If the UE is configured with more than one SPS PDSCH configurations, and for type-2 HARQ-ACK codebook construction, - HARQ-ACK bit order for SPS PDSCH release with a separate/joint release DCI is derived by reusing rel-15 mechanism (i.e., based on DAI and K1 indicated in the release DCI) - HARQ-ACK bit order for SPS PDSCH with associated PDCCH is derived by reusing rel-15 mechanism (i.e., based on DAI and K1 indicated in the activation DCI) - For cases where HARQ-ACK feedback for one or more SPS PDSCH receptions without a corresponding PDCCH is multiplexed with HARQ-ACK feedback for dynamic scheduled PDSCH and/or for SPS PDSCH release, - HARQ-ACK for one or more SPS PDSCH receptions without a corresponding PDCCH is appended after HARQ-ACK bits for dynamic scheduled PDSCHs and/or for SPS PDSCH release - In ascending order of DL slot per {SPS configuration index, serving cell index}, and then in ascending order of SPS configuration index per {serving cell index}, and then in ascending order of serving cell index

In 38.213, we can find that this HARQ-ACK bit order is used for SPS PDSCH receptions but not for SPS PDSCH receptions without a corresponding PDCCH, which means the first SPS PDSCH reception with activation PDCCH is also included. Actually the first SPS PDSCH reception with activation PDCCH should be treated as one dynamic PDSCH reception. So we propose the following text proposal.

--------------------------------------------Start of text proposal--------------------------------------------------------

### 9.1.2	Type-1 HARQ-ACK codebook determination

This clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

A UE reports HARQ-ACK information for a corresponding PDSCH reception or SPS PDSCH release only in a HARQ-ACK codebook that the UE transmits in a slot indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format 1_0 or DCI format 1_1. The UE reports NACK value(s) for HARQ-ACK information bit(s) in a HARQ-ACK codebook that the UE transmits in a slot not indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format 1_0 or DCI format 1_1.

If the UE is provided pdsch-AggregationFactor,  is a value of pdsch-AggregationFactor; otherwise, . The UE reports HARQ-ACK information for a PDSCH reception from slot  to slot  only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot , where  is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format. If the UE reports HARQ-ACK information for the PDSCH reception in a slot other than slot , the UE sets a value for each corresponding HARQ-ACK information bit to NACK.

If a UE reports HARQ-ACK information in a PUCCH only for

-	a SPS PDSCH release indicated by DCI format 1_0 with counter DAI field value of 1, or

-	a PDSCH reception scheduled by DCI format 1_0 with counter DAI field value of 1 on the PCell, or

-	SPS PDSCH receptions

within the  occasions for candidate PDSCH receptions as determined in Clause 9.1.2.1, the UE determines a HARQ-ACK codebook only for the SPS PDSCH release or only for the PDSCH reception or only for the SPS PDSCH receptions without a corresponding PDCCH according to corresponding  occasion(s) on respective serving cell(s), where the value of counter DAI in DCI format 1_0 is according to Table 9.1.3-1 and HARQ-ACK information bits in response to SPS PDSCH receptions are ordered according to the following pseudo-code; otherwise, the procedures in Clause 9.1.2.1 and Clause 9.1.2.2 for a HARQ-ACK codebook determination apply.

Set NcellsDL to the number of serving cells configured to the UE

Set NcSPS to the number of SPS PDSCH configuration configured to the UE for serving cell c

Set NcDL to the number of DL slots for SPS PDSCH reception without a corresponding PDCCH on serving cell c with HARQ-ACK information multiplexed on the PUCCH

*** Unchanged text is omitted ***

#### 9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

*** Unchanged text is omitted ***

If multiple SPS PDSCH receptions without a corresponding PDCCH are activated for a UE and the UE multiplexes corresponding HARQ-ACK information in the PUCCH in slot n, the UE generates the HARQ-ACK information as described in Clause 9.1.2 and appends it to the  HARQ-ACK information bits.

---------------------------------------------End of text proposal--------------------------------------------------------

# Conclusions

Based on the above discussion, the following proposal is given:

Proposal: Take the following TP for TS 38.213 to reflect the agreements.

--------------------------------------------Start of text proposal--------------------------------------------------------

### 9.1.2	Type-1 HARQ-ACK codebook determination

This clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

A UE reports HARQ-ACK information for a corresponding PDSCH reception or SPS PDSCH release only in a HARQ-ACK codebook that the UE transmits in a slot indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format 1_0 or DCI format 1_1. The UE reports NACK value(s) for HARQ-ACK information bit(s) in a HARQ-ACK codebook that the UE transmits in a slot not indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format 1_0 or DCI format 1_1.

If the UE is provided pdsch-AggregationFactor,  is a value of pdsch-AggregationFactor; otherwise, . The UE reports HARQ-ACK information for a PDSCH reception from slot  to slot  only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot , where  is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format. If the UE reports HARQ-ACK information for the PDSCH reception in a slot other than slot , the UE sets a value for each corresponding HARQ-ACK information bit to NACK.

If a UE reports HARQ-ACK information in a PUCCH only for

-	a SPS PDSCH release indicated by DCI format 1_0 with counter DAI field value of 1, or

-	a PDSCH reception scheduled by DCI format 1_0 with counter DAI field value of 1 on the PCell, or

-	SPS PDSCH receptions

within the  occasions for candidate PDSCH receptions as determined in Clause 9.1.2.1, the UE determines a HARQ-ACK codebook only for the SPS PDSCH release or only for the PDSCH reception or only for the SPS PDSCH receptions without a corresponding PDCCH according to corresponding  occasion(s) on respective serving cell(s), where the value of counter DAI in DCI format 1_0 is according to Table 9.1.3-1 and HARQ-ACK information bits in response to SPS PDSCH receptions are ordered according to the following pseudo-code; otherwise, the procedures in Clause 9.1.2.1 and Clause 9.1.2.2 for a HARQ-ACK codebook determination apply.

Set NcellsDL to the number of serving cells configured to the UE

Set NcSPS to the number of SPS PDSCH configuration configured to the UE for serving cell c

Set NcDL to the number of DL slots for SPS PDSCH reception without a corresponding PDCCH on serving cell c with HARQ-ACK information multiplexed on the PUCCH

*** Unchanged text is omitted ***

#### 9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

*** Unchanged text is omitted ***

If multiple SPS PDSCH receptions without a corresponding PDCCH are activated for a UE and the UE multiplexes corresponding HARQ-ACK information in the PUCCH in slot n, the UE generates the HARQ-ACK information as described in Clause 9.1.2 and appends it to the  HARQ-ACK information bits.

---------------------------------------------End of text proposal--------------------------------------------------------

# References

- 3GPP TS 38.213 V16.0.0 (2019-12)