3GPP TSG RAN WG1#100-e			R1-2001129

e-Meeting, 24th February – 6th March, 2020

Agenda item:	7.2.6.1

Source: 	Samsung

Title: 	Feature lead summary for MU-MIMO CSI

Document for:	Discussion and Decision

# Introduction

The FL summary of the proposals in the submitted contributions () for Rel.16 NR_eMIMO MU-CSI maintenance is given below and categorized under the following sections:

- High priority (essential)

- Editorial

- Low priority (non-essential)

In addition, there is one contribution in this agenda item that does not pertain to MU-MIMO CSI [11] and therefore is not addressed in this summary.

Summary

## High priority (essential)

The following issues pertain to some ambiguity in the current description of the specs and may have some significant impact on spec completeness and/or UE implementation. Some of these issues, however, are still subject to further assessment.

Table 1 High-priority (essential)

| Issue | Description/Proposal | Companies |
| --- | --- | --- |
| H.1: SB size is undefined when NPRB < 24 | Alt1. NSB = 1 if csi-ReportingBand is not present (when NPRB < 24) | Huawei/HiSi |
|  | Alt2. UE expects csi-ReportingBand to be configured - Note: This may imply that when the UE is configured with eType-II CSI, the UE is not expected to be configured with NPRB < 24 (i.e. NPRB is at least 24) | Apple |
| H.2: Per-layer bitwidths of {i1,7,l}, {i2,4,l} and {i2,5,l} (bitmap size) is missing in 38.212, leading to ambiguity in per layer bitwidth of these parameters | #### 6.3.2.1.2 CSI TP for TS 38.212 after Table 6.3.2.1.2-1A: Note: the bitwidth for {i1,7,l}l=1,…,v, {i2,4,l}l=1,…,v and {i2,5,l}l=1,…,v shown in Table 6.3.2.1.2-1A is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = v, respectively, and the corresponding per layer bitwidths are 2LMυ, 3KNZ, l-1, and 4KNZ, l-1, where KNZ, l as defined in Subclause 5.2.2.2.5 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ, TOT=l=1υKNZ, l. TP for TS 38.212 after Table 6.3.2.1.2-2A: Note: the bitwidth for {i1,7,l}l=1,…,v, {i2,4,l}l=1,…,v and {i2,5,l}l=1,…,v shown in Table 6.3.2.1.2-2A is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = v, respectively, and the corresponding per layer bitwidths are 2LMυ, 3KNZ, l-1, and 4KNZ, l-1, where KNZ, l as defined in Subclause 5.2.2.2.6 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ, TOT=l=1υKNZ, l. | Samsung |
| H.3: Current range of x* value can lead to negative value of nl(f)when N3>19 hence some correction is proposed | #### TP for TS 38.214: #### 5.2.2.2.5 Enhanced Type II Codebook For all values of N3, n3,l(0)=0 for l=1,…,υ. The nonzero elements of n3,l, identified by n3,l(1), …, n3,lMυ-1, are found from i1,6,l (l=1,…,υ), for N3≤19, and from i1,6,l (l=1,…,υ) and Minitial, for N3>19, using Cx,y as defined in 5.2.2.2.3 and the algorithm: s0=0 if N3≤19 T = N3 else T = 2Mυ end if for f=1,…,Mυ-1 Find the largest x*∈Mυ-1-f,…,N3T-1-f in Table 5.2.2.2.5-4 such that i1,6,l -sf-1≥Cx*,Mυ-f ef=Cx*,Mυ-f sf=sf-1+ef if N3≤19 n3,l(f)=N3-1-x* else nl(f)=2Mυ-1-x* if nl(f)≤Minitial+2Mυ-1 n3,l(f)=nl(f) else n3,l(f)=nl(f)+(N3-2Mυ) end if end if | LGE |
| H.4: ‘PMI format indicator’ (wideband vs. subband) for eType-II CSI (including port selection) Issue: While eType-II includes frequency selective PMI, it is neither wideband nor subband due to DFT-based compression. It is a single reported PMI indicating a recommended precoder for each frequency unit | Alt1. For eType-II CSI and eType-II port selection CSI, UE shall expect pmi-FormatIndicator to be configured as ‘widebandPMI’ | ZTE |
|  | Alt2. For eType-II CSI and eType-II port selection CSI, UE shall expect pmi-FormatIndicator to be configured as ‘subbandPMI’ - Motivation is to emphasize that eType-II only supports frequency-selective PMI reporting | Apple, Qualcomm |
|  | Alt3. For eType-II CSI and eType-II port selection CSI, the parameter pmi-FormatIndicator is not used/configured - It is unclear why pmi-FormatIndicator is a relevant parameter for eType-II since the reporting is performed in a compressed domain | [FL] |

For the above issues, the following initial proposals are made.

Initial proposals:

- Issue H.1: Unclear UE behavior when the UE is configured with eType-II and NPRB < 24 should be resolved. RAN1 needs to select a solution.

- Issue H.2: Assess if per-layer bitwidth ambiguity of {i1,7,l}, {i2,4,l} and {i2,5,l} will lead to ambiguity in UE procedure (including UCI multiplexing and bit processing). If so, discuss if the TP from Samsung needs to be refined.

- Issue H.3: Assess if negative value of nl(f)when N3>19 is indeed an issue. If so, discuss if the proposed scheme (TP included) from LGE needs to be refined.

- Issue H.4: Since, strictly speaking, eType-II PMI reporting is defined in transformed domain, discuss the relevance of pmi-FormatIndicator and decide how to resolve the ambiguity.

## Editorial

The following issues pertain to relative simple editorial corrections which are valid and not expected to be contentious. Some textual refinement may be fitting and can be discussed.

| Issue # | Description/Proposal | Companies |
| --- | --- | --- |
| E.1 Typographical fix | ##### 5.2.2.2.6 Enhanced Type II Port Selection Codebook *** Unchanged text is omitted *** The codebooks for 1-4 layers are given in Table 5.2.2.2.6-2, where vm is a PCSI-RS/2 -element column vector containing a value of 1 in element m mod PCSI-RS/2 and zeros elsewhere (where the first element is element 0), and the quantities φl,i,f and yt yt,l are defined as in clause 5.2.2.2.5. ### 5.2.3 CSI reporting using PUSCH *** Unchanged text is omitted *** When CSI reporting on PUSCH comprises two parts, the UE may omit a portion of the Part 2 CSI. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where is the number of CSI reports configured to be carried on the PUSCH. Priority 0 is the highest priority and priority is the lowest priority and the CSI report n corresponds to the CSI report with the nth smallest Prii,CSI(y,k,c,s) value among the CSI reports as defined in Clause 5.2.5. The subbands for a given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in increasing order with the lowest subband of csi-ReportingBand as subband 0. When omitting Part 2 CSI information for a particular priority level, the UE shall omit all of the information at that priority level. - For Enhanced Type II reports, for a given CSI report n, each reported element of indices i2,4,l i2,5,l and i1,7,l, indexed by l,i and f, is associated with a priority value Pril,i,f=2⋅L⋅υ⋅πf+υ⋅i+l, with π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1) with l=1,2,…,υ, i=0,1,…,2L-1, and f=0,1,…,Mυ-1, and where n3,l(f) is defined in Clause 5.2.2.2.5. The element with the highest priority has the lowest associated value Pril,i,f. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-2, 5.2.3-1, where | CATT |
| E.2 Addressing issues with the description “The remaining 2L⋅Mν⋅ν-KNZ indicators “ and “The KNZ-υ indicators …” and confusion on the definition of the modulus of a negative value | ##### 5.2.2.2.5 Enhanced Type II Codebook For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031}, and UE configured with higher layer parameter codebookType set to ' typeII-r16' --- Unchanged text omitted --------- Let fl*∈0,1,…,Mυ-1 be the index of i2,4,l and il*∈0,1,…,2L-1 be the index of kl,fl*(2) which identify the strongest coefficient of layer l, i.e., the element kl,il*,fl*(2) of i2,4,l, for i=0,1,…,2L-1 and f=0,1,…, Mυ-1 l=1,…,υ. The codebook indices of n3,lkmi are remapped with respect to n3,l(fl*) as n3,l(f)=n3,l(f)-n3,l(fl*)mod N3kmi*=0, such that n3,l(fl*)=0, after remapping. The index f is remapped with respect to fl* as f=f-fl*mod Mυ, such that the index of the strongest coefficient is fl*=0 (l=1,…,υ), after remapping, where (-¦a¦)mod N=N∙(¦a¦N)-¦a¦ . The indices of i2,4,l, i2,5,l and i1,7,l indicate amplitude coefficients, phase coefficients and bitmap after remapping. --- Unchanged text omitted --------- The amplitude and phase coefficient indicators are reported as follows: - kl,il*L(1)=15, kl,il*,0(2)=7, kl,il*,0(3)=1 and cl,il*,0=0 l=1,…,υ. The indicators kl,il*L(1), kl,il*,0(2) and cl,il*,0 are not reported for l=1,…,υ. - The indicator kl,il*L+1 mod 2(1) is reported for l=1,…,υ. - The KNZ-υ indicators kl,i,f(2) for which i≠il*, f≠0 and kl,i,f(3)=1 are reported. - The KNZ-υ indicators cl,i,f for which i≠il*, f≠0 and kl,i,f(3)=1 are reported. - The remaining 2L⋅Mν⋅ν-KNZ indicators kl,i,f(2)=0 are not reported and the corresponding kl,i,f(3)=0 are reported. - The remaining 2L⋅Mν⋅ν-KNZ indicators cl,i,f=0 are not reported and the corresponding kl,i,f(3)=0 are reported. - The KNZ-υ indicators kl,i,f(3)=1 for which i≠il*, f≠0 are reported, the corresponding kl,i,f(2) and cl,i,f are also reported. - The remaining 2L⋅Mν⋅ν-KNZ indicators kl,i,f(3)=0 are reported, but the corresponding kl,i,f(2) and cl,i,f are not reported. ----------------------------End of proposed TP for 38.214 ------------------------------ | Ericsson, OPPO |

For the above issues, the following initial proposals are made.

Initial proposals: For the above editorial TPs (E.1 and E.2), agree in principle and discuss any potential refinement.

## Non-essential

The following issues pertain to non-essential proposal (see below for FL assessment) and therefore will not be discussed during the eMeeting.

| Issue # | Proposal(s) | Companies |
| --- | --- | --- |
| N.1 | At least one orthogonal spatial beam group with no less than L beams available after CBSR FL assessment: It is unclear if this cannot be handled by gNB implementation | Huawei/HiSi |
| N.2 | - R = 2 and N3<=19 is an optional - for FR2, the maximum number of configured aperiodic CSI reporting settings = 4 (same as R15) FL assessment: These are UE capability issues | vivo |
| N.3 | Cx,y defined in Table 5.2.2.2.5-4 (of TS38.214) should be applied to the algorithm for calculating selected FD basis index. FL assessment: It is unclear if this addresses any lack of clarity in the current version of the spec | CATT |
| N.4 | When N3>19, the size of the intermediate set is give by N3'=2M1 for RI={1,2,3,4}, where M1 is the number of FD bases selected for RI={1,2}. FL assessment: Assuming if this proposal indeed offers some benefit, it is optimization at best and non-essential | Qualcomm |
| N.5 | Replace notation KNZ,TOT in TS 38.212 [2] with notation KNZ in TS 38.214 FL assessment: It is unclear if this addresses any lack of clarity in the current version of the spec | Fraunhofer/HHI |

# References

- R1-2000205, Huawei, HiSilicon, Remaining issues on MU-CSI in R16

- R1-2000238, ZTE, Maintenance of CSI enhancement for MU-MIMO support

- R1-2000332, vivo, Discussion on remaining issues on MU-CSI

- R1-2000456, OPPO, Text proposals for CSI enhancements for MU-MIMO support

- R1-2000534, CATT, Remaining issues on CSI enhancement for MU-MIMO

- R1-2000632, Samsung, Correction on CSI part 2 payload in 38.212

- R1-2000678, LG Electronics, Text proposals on CSI Enhancement for MU-MIMO Support

- R1-2000858, Apple, Remaining issues for Rel-16 Type II CSI enhancement

- R1-2000939, Ericsson, Remaining issues for MU-CSI

- R1-2000973, Qualcomm, Remaining issues on CSI enhancement for MU-MIMO support

- R1-2000987, Nokia, Nokia Shanghai Bell, Maintenance on Rel.16 CSI enhancements

- R1-2001119, Fraunhofer IIS, Fraunhofer HHI, Text proposals for Rel.16 Type-II enhancements