3GPP TSG-RAN WG1 Meeting #100-e	R1-2001208

Online, February 24th – March 6th, 2020

Agenda Item:	7.2.10.1

Source:	Apple

Title:	Feature lead summary #2 on UL Power Control for NN-DC

Document for:	Discussion

# 1	Introduction

This document contains the feature lead summary of issues based on the contributions submitted under AI 7.2.10.1 related to uplink power control for NR-NR DC topic under Rel-16 LTE_NR_DC_CA_enh WI.

# 2	UL Power Control for NN-DC maintenance issues and priority

To organize the email discussion, the issues have been grouped into maximum three threads according to the chairman’s guidance.

- Section 2.1: Functional corrections to existing specifications.

- Section 2.2: Editorial correction or capturing missed agreements.

- Section 2.3: Relevant features but already handled by other AIs.

## 2.1 Issues with high priority (Essential)

### Issue 1: Applicable scenario for Dynamic Power Sharing (DPS) scheme

One issue raised in [3][10] is applicable scenario for dynamic power sharing scheme. Especially, it is claimed in [3] that asynchronous DC is supportable with an architecture that has distinct hardware blocks belonging to the two cell groups. Since for DPS, one CG has to refer to the transmission status of another CG. It therefore proposed in[3][10] to limit the dynamic power sharing scheme to synchronous DC scenario only.

Table 1: Applicable scenario for Dynamic Power Sharing

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt.1: Only applicable for synchronous scenario |  | MTK, Qualcomm, Apple |
| At.2: No limitation, (i.e. both synchronous and asynchronous scenarios) |  | Ericsson |

Companies are encouraged to provide more inputs on this issue to conclude it:

| Companies | Views |
| --- | --- |
| MTK | Support Alt. 1. |
| Nokia/NSB | Alt. 2 |
| ZTE | Alt.2 For EN-DC and NE-DC, dynamic power sharing can be applied to both synchronous and asynchronous scenarios. Not sure why we have to impose the restrictions only for NR-DC. Some companies may argue that DPS requires tight coordination between MCG and SCG in both UE side and network side. For asynchronous NR-DC scenario, MCG and SCG are not co-located and the tight coordination between MCG and SCG are not guaranteed. However, in this case, the DPS performance can be guaranteed by appropriate semi-static configuration, e.g., configuring a larger K1/K2 values in MCG. In summary, we fail to see the necessity to restrict that DPS is only applicable for asynchronous scenario. |

### Issue 2: T_offset determination for DPS

In the RAN1 #99 meeting, the following was agreed for DPS:

Agreements: - For NR-DC dynamic power sharing, to compute the transmit power for SCG UL transmission starting at time T0, - UE checks for PDCCH(s) received before time T0-T_offset that trigger an overlapping MCG UL transmission, and - If such PDCCH(s) are detected, UE sets it’s transmit power in SCG (pwr_SCG) such that pwr_SCG <= min{PSCG, Ptotal – MCG tx power} where ‘MCG tx power’ is the actual transmission power of MCG - Otherwise, pwr_SCG <= Ptotal; - UE does not expect to be scheduled by PDCCH(s) received on MCG after T0-[T_offset] that trigger(s) MCG UL transmission(s) that overlaps with the SCG transmission. - (working assumption) No new RRC signaling is introduced for T_offset: - Alt.1: T_offset <= T_proc,2 - Alt.2: T_offset <= 2*T_proc,2 - Alt.3: T_offset reasonably larger than Alt 1. & Alt 2 but <=4ms - To be addressed in the CR stage

It remains open regarding the value of Toffset. The determination of Toffset. value essentially is a tradeoff between processing time requirement for power adjustment at the UE and gain of DPS e.g. in terms of MCG latency. Companies views on this issue is summarized in the following Table:

Table 2: Summary of companies views on Toffset

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt.1: Toffset=max⁡(Tproc,MCGmax,Tproc,SCGmax ) - where Tproc, MCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and/or Tproc,CSImux as specified in TS38.213 and TS38.214 based on the configurations for the MCG. - Tproc, SCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and/or Tproc,CSImux as specified in TS38.213 and TS38.214 based on the configurations for the SCG. |  | Qualcomm, Apple, vivo, MTK, [OPPO], [Intel] |
| Alt.2: Toffset=Tproc,2mux+δ , - where δ is determined by PUSCH timing capability and SCS combinations for MCG and SCG, see details in [5] |  | Samsung |
| Alt.3: - At least for sync NR-DC case, Toffset = N2+4sym - For async NR-DC case, Toffset= 0.5ms+ N2+4sym - Note: For above two cases, N2 is as defined in sub clause 6.4 of 38.214, and numerology of the CC with largest SCS configured on the MCG should be used. |  | Ericsson |

Companies are encouraged to provide more inputs on this issue to make progress:

| Companies | Views |
| --- | --- |
| MTK | Support Alt.1. For “any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and/or Tproc,CSImux as specified in TS38.213 and TS38.214”, numerology of the CC with largest SCS configured on the MCG should be used (This is just for clarification, since it may be implicitly implied by “any of the possible values”). |
| ZTE | We support Toffset=Tproc,SCGmax. If needed, an additional delta can be added on top of that, i.e., Toffset=Tproc,SCGmax+ δ. Not sure whether this is identical to Alt.2, if yes, then we are also fine with Alt.2. It is understandable that Toffset should be the larger than any possible processing time in SCG. However, from our perspective, the Toffset doesn’t need to factor in the Tproc,MCGmax. The analysis is as below. If Tproc,MCGmax< Tproc,SCGmax, the Toffset is determined by the processing time in SCG and of course, the processing time in MCG should not be factored in, i.e., Toffset=Tproc,SCGmax. If Tproc,MCGmax> Tproc,SCGmax, we can set Toffset=Tproc,SCGmax. With a larger processing time in MCG, network can guarantee that the UL transmissions in MCG overlapping with the SCG transmission won’t be scheduled after To-Toffset. |

### Issue 3: Look-ahead Operation for UL transmissions without scheduling DCI

For dynamic UL grant, the corresponding PDCCH is compared with the cut-off time to determine whether this dynamic UL grant shall be considered into power allocation. While for some cases as listed below based on contributions [2][5][10], no individual DCI is available. How to handle this case for DPS needs to be discussed.

- Case 1: CG-PUSCH

- Case 2: Periodic PUCCH (e.g. SR, P-/SP-CSI) and P-/SP-SRS

  - For this case, there is no DCI associated with periodic PUCCH e.g. for SR and SP-SRS.

- Case 3: Group-TPC command by DCI format 2_2, or 2_3 or 2_0 for MCG transmission

  - These Group-TPC command DCI formats do not schedule uplink transmissions but would impact on the value of PMCGactual. Therefore, how to handle it needs to be discussed.  DCI format 2_0 does not schedule uplink transmissions but would impact on the value of PMCGactual, since it may cancel or trigger semi-static uplink transmissions.

- Case 4: Uplink transmission skip for MCG dynamic grant PUSCH/configured grant PUSCH due to 5.4.3.1.3 of TS38.321 or by DCI format 2_4

  - This DCI format 2-4 does not schedule uplink transmissions but would impact on the value of PMCGactual. Similarly, for configured grant PUSCH transmission occasion, the UE may or may not transmit the PUSCH. For dynamic grant PUSCH, the UE may skip the transmission if some conditions are met. These are specified in 5.4.3.1.3 of TS38.321.

Companies views on these identified cases are summarized in Tables 4/5/6/7 below:

Table 4: Summary of companies views on Case 1 (i.e. CG-PUSCH)

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt.1: CG-PUSCH transmissions with a first uplink symbol that is earlier by more than (Toffset+Tproc,2) from the first symbol of the transmission occasion on the SCG (See Details in R1-2000361) |  | ZTE |
| Alt.2: Reuse the T_offset value defined for Grant-based PUSCH |  | Samsung |
| Alt.3.determining the value of PMCGactual in accordance to the higher layers configurations |  | Qualcomm |

Table 5: Summary of companies views on Case 2 (e.g. Periodic PUCCH or SRS)

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt.1: determining the value of PMCGactual in accordance to the higher layers configuration |  | Qualcomm, ZTE |

Table 6: Summary of companies views on Case 3 (e.g. Group-command DCI formats)

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt.1: Restrict these DCI format 2_0/2_2/2-3 to be before T0 – T_offset. |  | Qualcomm |

Table 7: Summary of companies views on Case 4

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt.1: UE is not required to take into account DCI format 2_4 and UL transmission skip according to 5.4.3.1.3 of TS38.321 after T0 – T_offset for PMCGactual determination. |  | Qualcomm |

Companies are encouraged to provide inputs on these cases to conclude them (e.g. in format of Company A: on case 1.., on case 2, …):

| Companies | Views |
| --- | --- |
| MTK | On case 1, to our understanding, QC’s proposal [10] is “Semi-statically configured uplink transmissions are taken into account for PMCGactual determination” and it is equivalent to “UE assumes that UL transmission always exists in the configured periodic time-frequency domain resource [2]”. If this is equivalent to Alt. 3, then we support Alt. 3. On case 2, we support Alt. 1. To our understanding, Alt. 1 is equivalent to “UE assumes that UL transmission always exists in the configured periodic time-frequency domain resource”. On case 3, we support Alt. 1. On case 4, we support Alt. 1. |
| Nokia/NSB | We support one generalized rule on UE operation related with look-ahead. So all DCI received within/before look-ahead window should be consider for SCG power control, or should be ignored. We also prefer to define one common T_offset value than to define case specific values. For Case 1, we are on the same page with MTK. |
| ZTE | For Case1 Sorry to say, we used the old version of wording in our tDoc, i.e., Alt.1 here is not our latest wording. We’d like to update the Alt.1 as below. The philosophy is to reuse the timeline for R15 PHR for configured grant and the wording is borrowed from 38.213 Section 7.7. Configured PUSCH transmissions whose first uplink symbol minus T'proc,2=Tproc,2 are earlier by more than Toffset from the first symbol of the transmission occasion on the SCG should be considred, where Tproc,2 is determined according to [TS 38.214] assuming d2,1 = 1, d2,2=0, and with µDL corresponding to the subcarrier spacing of the active downlink BWP of the scheduling cell for a configured grant. For Case3 & For Case4 DCI 2_0, 2_4 and UL skipping Regarding DCI 2_0, DCI 2_4 and UL skipping, all of them could be used for uplink cancellation. One unified rule applied for all of them is preferable. It is not reasonable to simply restrict that DCI 2_0 is not allowed to be transmitted after T0 – T_offset because DCI 2_0 may or may not cancel any uplink transmissions overlapping with the SCG uplink transmission. Thus, we propose the following. For DPS purpose, UE is not required to take into account DCI format 2_0, 2_4 and UL transmission skip according to 5.4.3.1.3 of TS38.321 after T0 – T_offset for PMCGactual determination. Restricting DCI format 2_0 to be before T0 – T_offset is not needed. DCI 2_2 and 2_3 It seems impossible to restrict the DCI 2_2 and 2_3 are not allowed to be transmitted after T0 – T_offset. If there are continuous UL transmissions in SCG and each UL transmission in SCG has a duration T0 – T_offset. This may end up with no transmission occasion for DCI 2_2 and 2_3 in MCG. Thus, we propose the following. For DPS purpose, UE is not required to take into account DCI format 2_2 and 2_3 after T0 – T_offset for PMCGactual determination for the current UL transmission. |

### Issue 4: DPS without Look-ahead Operation

Clarifications on how to adjust SCG transmission power for DPS without Look-ahead was brought up in several contributions [2][3][6], which can be categorized as below:

Table 8: Summary of TPs for Dynamic Power Sharing without Look-ahead Operation

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt.1: TP in R1-2000361 |  | ZTE |
| Alt.2: TP in R1-2001067 |  | Nokia |
| Alt.3: Reuse the behavior defined for Rel-15 CA, i.e. symbol-by-symbol power determination |  |  |
| Alt.4: Make decision after the value of T_offset is decided |  | Qualcomm |

As proposed in [12], LS to RAN2 regarding the configurable power control modes maybe necessary after making conclusion on the support of DPS without look-ahead.

Companies are encouraged to provide more inputs on this issue to conclude it:

| Companies | Views |
| --- | --- |
| MTK | We support Alt.2: TP in R1-2001067, which determines overlap of MCG and SCG on slot level. However, R1-2001067 did not specify when should UE determine whether there is a overlap. For “without-lookahead“, we support: “UE determines whether there is a overlap when UE receives the corresponding scheduling DCI“. We are also OK with Alt. 4. |
| ZTE | The wording “the UE reduces transmission power in any portion of slot i2 of the SCG so that PMCGactuali1+PSCGactuali2≤PTotalNR-DC in all portions of slot i2” in Alt.2 implies that Alt.2 is also trying to determine the power symbol-by-symbol. The key difference of Alt.1, Alt.2/Alt.3 is that the power scaling is performed on a transmission occasion basis for Alt1 and on a symbol basis for Alt2/Alt3. Before jumping into the detailed TP, maybe we have to first agree on which is the common understanding. |

## 2.2 Maintenance or Editorial issues

### Issue 5: Capture agreements for Semi-static Power Sharing (SPS)

For semi-static power control mode, it was proposed in [4][9][10] to capture the following agreement into specification:

- PMCG+ PSCG≤PTotalNN- DC.

For example, the following TP proposed in [9] can be considered to address this issue:

---------------------------- start TP1 to sub clause 7.6.2 of 38.213v16.0.0 --------------------------------- ……. If a UE is provided NR-DC-PC-mode = Semi-static-mode1 or NR-DC-PC-mode = Semi-static-mode2, the UE does not expect PMCG and PSCG to be configured such that PMCG+PSCG>PTotalNR-DC If a UE is provided NR-DC-PC-mode = Semi-static-mode1, the UE determines a transmission power for the MCG or for the SCG as described in Clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively. ……..

Companies views on this identified clarification can be provided in the table below:

| Companies | Views |
| --- | --- |
| MTK | We agree on this TP. |

### Issue 6: Clarification on specification context of SPS

One clarification on semi-static power control mode (i.e. issue 6-1) was raised in [4] and it was proposed to clarify the following:

Proposal 1: In TS 38.213, for NR-DC-PC-mode = Semi-static-mode2, clarify the UE behavior in the part of otherwise is for the transmission in slot i2 .

Companies views on this identified clarification can be provided in the table below:

| Companies | Views |
| --- | --- |
| MTK | To our understanding, the interpretation of Option 1 (the UE determines a transmission power for the MCG or the SCG in slot i2 without considering PMCG or PSCG, respectively) in [4] is which intended when RAN1 agreed to support semi-static-mode 2 (Alt.1-2) in RAN1-98bis. |
| ZTE | We agree that it is for the transmission in slot i2. But it seems the current spec is clear since the description only mentions UL transmission in slot i2. |

One more clarification on semi-static-mode 2 (i.e. issue 6-2) was brought up in [2] to collect common understanding. As shown in FIG.1 below (copied from [2]), the last part of slot i1 in MCG is UL symbols configured by tdd-UL-DL-ConfigurationCommon. PUSCH 1 is transmitted in SCG CC1 with 30 KHz SCS and PUSCH 2 is transmitted in SCG CC2 with 15 KHz. The first slot of SCG CC1 is regarded as slot i2. When calculating the Tx power for PUSCH 1, it is not clear whether PUSCH 2 shall be considered as the ‘ongoing transmission in slot i2’ since only part of PUSCH 2 transmission is located within slot i2. The maximum transmission power for PUSCH 1 varies from different interpretation:

- Interpretation 1: PUSCH 2 is NOT considered as the ‘ongoing transmission in slot i2’, then PTotal is regarded as the maximum transmission power for PUSCH 1 since PUSCH 1 is not overlapped with the UL symbols in slot i1 of MCG.

- Interpretation 2: PUSCH 2 is considered as the ‘ongoing transmission in slot i2’, the PSCG is regarded as the maximum transmission power for PUSCH 1 since the ongoing PUSCH 2 transmission overlaps with the UL symbols in i1 of MCG.

Figure 1 Interpretation of ‘ongoing transmission in slot i2’.

Companies views on this identified clarification can be provided in the table below:

| Companies | Views |
| --- | --- |
| MTK | We think Interpretation 2 makes more sense. For Interpretation 1, PUSCH 1 would have maximum transmission power PTotal while PUSCH 2 would have maximum transmission power PSCG. This seems strange to us. |
| ZTE | Interpretation#2 is our understanding. Note that the issue doesn’t exist in NE-DC semi-static power sharing (as shown below) because the numerology of each cell in LTE is always 15KHz. Furthermore, the 15KHz slot is always the longest slot, then there is no such kind of ambiguity. If at least one symbol of slot of the MCG that is indicated as uplink or flexible by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated overlaps with subframe of the SCG - for subframe , the UE determines a transmission power for the SCG as described in [13, TS 36.213] using as the maximum transmission power otherwise - the UE determines a transmission power for the SCG as described in [13, TS 36.213] without considering as the maximum transmission power If Interpretation#2 is the common understanding, two ways can be considered. 1. Update "ongoing transmission in slot i2" to "ongoing transmission overlapping with slot i2" in this part of the spec. 2. Clarify that the slot i2 is based on the minimum numerology (i.e., longest slot) configured for SCG or MCG. |

## 2.3 Issue with low priority

The issues in this list is covered by other AIs e.g. UE features and can be deferred to future meeting discussions. It is thus given lowest priority in this e-meeting.

| Issue # | Description | Tdoc |
| --- | --- | --- |
| 7 | Split UE Feature 18-1 into two sub-features for semi-static power sharing | R1-2000440 |
| 8 | Merge the capability for look-ahead Operation to the capability for dynamic power-sharing. | R1-2000980 |

Companies views on these issues, if any, can be provided in the table below:

| Companies | Views |
| --- | --- |
| MTK | We support both descriptions of Issue 7 & Issue 8. |

# 3	Summary of preparation phase email discussion

The following was recommended by FL based on the discussions.

- Email Discussion thread <100e-Prep-NR_DC_CA_ULPC-1> - Issue 1 and Issue 3 - Email Discussion thread <100e-Prep-NR_DC_CA_ULPC-2> - Issue 2 and Issue 4, considering the dependency between them (e.g. Alt.4 for issue-4), Issue 3 - Email Discussion thread <100e-Prep-NR_DC_CA_ULPC-3> - Issue 5 and 6 in section 2.2.

No further comments received on it and seems to be agreeable for next phase.

# Reference

| [1] R1-2000346 | Remaining issues on uplink power control for NR-NR DC | vivo |
| --- | --- | --- |
| [2] R1-2000361 | Remaining Issues on NR-DC Power Control | ZTE |
| [3] R1-2000440 | On UL Power Control for NN-DC | MediaTek Inc. |
| [4] R1-2000561 | Text proposals for UL Power Sharing for NR-DC | OPPO |
| [5] R1-2000644 | Power Control for NR-DC | Samsung |
| [6] R1-2000721 | Remaining issues on uplink power control for NN-DC | Intel Corporation |
| [7] R1-2000845 | On remaining issues of UL Power Control for NN-DC | Apple |
| [8] R1-2000846 | FL summary on UL Power Control for NN-DC | Apple |
| [9] R1-2000888 | Remaining issues for NR-DC UL Power Control | Ericsson |
| [10] R1-2000980 | Remaining issues on uplink power control for NR-DC | Qualcomm Incorporated |
| [11] R1-2001039 | UL power control for NR-NR dual connectivity | Huawei, HiSilicon |
| [12] R1-2001067 | Maintenance of rel-16 DC power control | Nokia, Nokia Shanghai Bell |