3GPP TSG-RAN WG1 Meeting #100-e	R1-2001241

Online, February 24th – March 6th, 2020

Agenda Item:	7.2.6.5

Source:	Ericsson

Title:	Output of email thread [100e-NR-eMIMO-lowPAPR-02] - TP#3 to 38.211

Document for:	Decision

# 1	Text Proposal

-------- start of the TP for TS 38.211-------

7.4.1    Reference signals

7.4.1.1 Demodulation reference signals for PDSCH

7.4.1.1.1              Sequence generation

The UE shall assume the sequence  is defined by

.

where the pseudo-random sequence  is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with

cinit=217Nsymbslotns,fμ+l+12NIDnSCIDλ+1+217λ2+2NIDnSCIDλ+nSCIDλmod 231

cinit=217Nsymbslotns,fμ+l+12NIDnSCIDλ+1+217λ2+2NIDnSCIDλ+nSCIDλmod 231

where  is the OFDM symbol number within the slot, ns,fμ  is the slot number within a frame, and

-     NID0,NID1∈0,1,…,65535 are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-DownlinkConfig IE if provided and the PDSCH is scheduled by PDCCH using DCI format 1_1 or 1_2 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI

-     NID0∈0,1,…,65535 is given by the higher-layer parameter scramblingID0 in the DMRS-DownlinkConfig IE if provided and the PDSCH is scheduled by PDCCH using DCI format 1_0 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI;

-     NIDnSCIDλ=NIDcell NIDnSCID=NIDcell otherwise;

-     nSCIDλ    nSCIDλ and λ are is given by

- if the higher-layer parameter DMRSdownlink-r16 in the DMRS-DownlinkConfig IE is provided

nSCIDλ=nSCIDλ=0 or λ=21-nSCIDλ=1

nSCIDλ=nSCIDλ=0 or λ=21-nSCIDλ=1

λ=λ

where λ is the CDM group defined in clause 7.4.1.1.2 if the higher-layer parameter DMRSdownlink-r16 in the DMRS-DownlinkConfig IE is provided, otherwise by

- otherwise

nSCIDλ=nSCID

nSCIDλ=nSCID

λ=0

-     λ is the CDM group defined in clause 7.4.1.1.2.

The quantity nSCID∈0, 1 is given by the DM-RS sequence initialization field, if present, in the DCI associated with the PDSCH transmission if DCI format 1_1 or 1_2 in [4, TS 38.212] is used, otherwise nSCID=0.

-------- end of the TP for TS 38.211 -------