3GPP TSG-RAN WG1 Meeting #100-e	R1-2001302

Online, February 24th – March 6th, 2020

Agenda Item:	7.2.10.1

Source:	Apple

Title:		Outcome of email thread [100e-NR-LTE_NR_DC_CA_enh-ULPC-02]

Document for:	Discussion

# 1	Introduction

This document discusses issue 2 and issue 4 in [13] that are prioritized for the e-meeting during the preparation phase.

[100e-NR-LTE_NR_DC_CA_enh-ULPC-02] E-mail discussion/approval on the issue 2 and issue 4 in R1-2000846 by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3 – Hong (Apple)

Companies are asked to provide their comments using the tables below the proposal for each issue. Initial input is requested by 2/26. Feel free to respond multiple times and to respond to other companies comments by adding more rows or even revising your earlier comment. Feel free to highlight the key comments in the body of an email for more dynamic discussion by email, but don’t attach the file to the email. Please also indicate if you agree with the proposal and proposed TP. If you would like to propose a revision of a TP, please copy-paste the TP in the table and make your revisions in the table.

The feature leader proposal will be provided in the later version after collecting companies inputs.

# 2	UL Power Control for NN-DC maintenance issues and priority

### Issue 2: T_offset determination for DPS

In the RAN1 #99 meeting, the following was agreed for DPS:

Agreements: - For NR-DC dynamic power sharing, to compute the transmit power for SCG UL transmission starting at time T0, - UE checks for PDCCH(s) received before time T0-T_offset that trigger an overlapping MCG UL transmission, and - If such PDCCH(s) are detected, UE sets it’s transmit power in SCG (pwr_SCG) such that pwr_SCG <= min{PSCG, Ptotal – MCG tx power} where ‘MCG tx power’ is the actual transmission power of MCG - Otherwise, pwr_SCG <= Ptotal; - UE does not expect to be scheduled by PDCCH(s) received on MCG after T0-[T_offset] that trigger(s) MCG UL transmission(s) that overlaps with the SCG transmission. - (working assumption) No new RRC signaling is introduced for T_offset: - Alt.1: T_offset <= T_proc,2 - Alt.2: T_offset <= 2*T_proc,2 - Alt.3: T_offset reasonably larger than Alt 1. & Alt 2 but <=4ms - To be addressed in the CR stage

It remains open regarding the value of Toffset. The determination of Toffset. value essentially is a tradeoff between processing time requirement for power adjustment at the UE and gain of DPS e.g. in terms of MCG latency. Companies views on this issue is summarized in the following Table:

Table 2: Summary of companies views on Toffset

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt.1: Toffset=max⁡(Tproc,MCGmax,Tproc,SCGmax ) - where Tproc, MCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and/or Tproc,CSImux as specified in TS38.213 and TS38.214 based on the configurations for the MCG. - Tproc, SCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and/or Tproc,CSImux as specified in TS38.213 and TS38.214 based on the configurations for the SCG. |  | Qualcomm, Apple, vivo, MTK, [OPPO], [Intel] |
| Alt.2: Toffset=Tproc,2mux+δ , - where δ is determined by PUSCH timing capability and SCS combinations for MCG and SCG, see details in [5] |  | Samsung |
| Alt.3: - At least for sync NR-DC case, Toffset = N2+4sym - For async NR-DC case, Toffset= 0.5ms+ N2+4sym - Note: For above two cases, N2 is as defined in sub clause 6.4 of 38.214, and numerology of the CC with largest SCS configured on the MCG should be used. |  | Ericsson |

Companies are encouraged to provide more inputs on this issue to make progress:

| Companies | Views |
| --- | --- |
| MTK | Support Alt.1. For “any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and/or Tproc,CSImux as specified in TS38.213 and TS38.214”, numerology of the CC with largest SCS configured on the MCG should be used (This is just for clarification, since it may be implicitly implied by “any of the possible values”). |
| ZTE | We support Toffset=Tproc,SCGmax. If needed, an additional delta can be added on top of that, i.e., Toffset=Tproc,SCGmax+ δ. Not sure whether this is identical to Alt.2, if yes, then we are also fine with Alt.2. It is understandable that Toffset should be the larger than any possible processing time in SCG. However, from our perspective, the Toffset doesn’t need to factor in the Tproc,MCGmax. The analysis is as below. If Tproc,MCGmax< Tproc,SCGmax, the Toffset is determined by the processing time in SCG and of course, the processing time in MCG should not be factored in, i.e., Toffset=Tproc,SCGmax. If Tproc,MCGmax> Tproc,SCGmax, we can set Toffset=Tproc,SCGmax. With a larger processing time in MCG, network can guarantee that the UL transmissions in MCG overlapping with the SCG transmission won’t be scheduled after To-Toffset. |

### Issue 4: DPS without Look-ahead Operation

Clarifications on how to adjust SCG transmission power for DPS without Look-ahead was brought up in several contributions [2][3][6], which can be categorized as below:

Table 8: Summary of TPs for Dynamic Power Sharing without Look-ahead Operation

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt.1: TP in R1-2000361 |  | ZTE |
| Alt.2: TP in R1-2001067 |  | Nokia |
| Alt.3: Reuse the behavior defined for Rel-15 CA, i.e. symbol-by-symbol power determination |  |  |
| Alt.4: Make decision after the value of T_offset is decided |  | Qualcomm |

As proposed in [12], LS to RAN2 regarding the configurable power control modes maybe necessary after making conclusion on the support of DPS without look-ahead.

Companies are encouraged to provide more inputs on this issue to conclude it:

| Companies | Views |
| --- | --- |
| MTK | We support Alt.2: TP in R1-2001067, which determines overlap of MCG and SCG on slot level. However, R1-2001067 did not specify when should UE determine whether there is a overlap. For “without-lookahead“, we support: “UE determines whether there is a overlap when UE receives the corresponding scheduling DCI“. We are also OK with Alt. 4. |
| ZTE | The wording “the UE reduces transmission power in any portion of slot i2 of the SCG so that PMCGactuali1+PSCGactuali2≤PTotalNR-DC in all portions of slot i2” in Alt.2 implies that Alt.2 is also trying to determine the power symbol-by-symbol. The key difference of Alt.1, Alt.2/Alt.3 is that the power scaling is performed on a transmission occasion basis for Alt1 and on a symbol basis for Alt2/Alt3. Before jumping into the detailed TP, maybe we have to first agree on which is the common understanding. |

# 3	Feature Leader Proposal

Based on the discussions and majority views to address the issue 2 and 4 identified above,  the following was proposed by Feature leader as working assumption:

Update the previous agreement as follows: Agreements: · For NR-DC dynamic power sharing, to compute the transmit power for SCG UL transmission starting at time T0, - UE checks for PDCCH(s) received before time T0-T_offset that trigger an overlapping MCG UL transmission, and - If such PDCCH(s) are detected, UE sets it’s transmit power in SCG (pwr_SCG) such that pwr_SCG <= min{PSCG, Ptotal – MCG tx power} where ‘MCG tx power’ is the actual transmission power of MCG - Otherwise, pwr_SCG <= Ptotal; - UE does not expect to be scheduled by PDCCH(s) received on MCG after T0-[T_offset] that trigger(s) MCG UL transmission(s) that overlaps with the SCG transmission. - (working assumption) No new RRC signaling is introduced for T_offset: - Alt.1: T_offset <= T_proc,2 Tproc,MCGmax+δ where: - Tproc,MCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and/or Tproc,CSImux as specified in TS38.213 and TS38.214 based on the configurations for the MCG. - FFS on δ value - This is the “DPS without look-ahead”. - Alt.2: T_offset <= 2*T_proc, Tproc,MCGmax+δ where: - Tproc,MCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, and/or Tproc,2mux, as specified in TS38.213 and TS38.214 based on the configurations for the MCG. - FFS on δ value - This is the “DPS with look-ahead”. - Alt.3: T_offset reasonbly larger than Alt 1. & Alt 2 but <=4ms - To be addressed in the CR stage - A UE reports the UE capability of Alt.1 and/or Alt.2. - Details up to UE feature list discussion

# Reference

| [1] R1-2000346 | Remaining issues on uplink power control for NR-NR DC | vivo |
| --- | --- | --- |
| [2] R1-2000361 | Remaining Issues on NR-DC Power Control | ZTE |
| [3] R1-2000440 | On UL Power Control for NN-DC | MediaTek Inc. |
| [4] R1-2000561 | Text proposals for UL Power Sharing for NR-DC | OPPO |
| [5] R1-2000644 | Power Control for NR-DC | Samsung |
| [6] R1-2000721 | Remaining issues on uplink power control for NN-DC | Intel Corporation |
| [7] R1-2000845 | On remaining issues of UL Power Control for NN-DC | Apple |
| [8] R1-2000846 | FL summary on UL Power Control for NN-DC | Apple |
| [9] R1-2000888 | Remaining issues for NR-DC UL Power Control | Ericsson |
| [10] R1-2000980 | Remaining issues on uplink power control for NR-DC | Qualcomm Incorporated |
| [11] R1-2001039 | UL power control for NR-NR dual connectivity | Huawei, HiSilicon |
| [12] R1-2001067 | Maintenance of rel-16 DC power control | Nokia, Nokia Shanghai Bell |

[13] R1-2001208		FL summary#2 on UL Power Control for NN-DC		Apple