3GPP TSG RAN WG1#100-e			R1-2001349

e-Meeting, 24th February – 6th March, 2020

Agenda item:	7.2.6.2

Source: 	OPPO

Title: 	Text Proposal 2 for [100e-NR-eMIMO-multiTRP-02]

Document for:	Discussion and Decision

# Text Proposal

The following TP is proposed for TS38.213-g00.

-------- start of the TP------- #### 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel < Unchanged parts are omitted > Set to the number of serving cells configured by higher layers for the UE; If for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, and is provided ACKNACKFeedbackMode = JointFeedback, the serving cell is counted as two times, each of which corresponds to the first and second CORESETs respectively Set to the number of PDCCH monitoring occasion(s) while Set – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell; If a serving cell is counted as two times, lower index corresponds to the first CORESETs while < Unchanged parts are omitted > If a UE - is provided PDSCH-CodeBlockGroupTransmission for serving cells; and - is not provided PDSCH-CodeBlockGroupTransmission, for serving cells where the UE determines the according to the previous pseudo-code with the following modifications - is used for the determination of a first HARQ-ACK sub-codebook for SPS PDSCH release, SPS PDSCH reception, and for TB-based PDSCH receptions on the serving cells and on the serving cells - is replaced by for the determination of a second HARQ-ACK sub-codebook corresponding to the serving cells for CBG-based PDSCH receptions, and - if for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, and is provided ACKNACKFeedbackMode = JointFeedback, the serving cell is counted as two times, each of which corresponds to the first and second CORESETs respectively, and < Unchanged parts are omitted > -------- end of the TP-------