3GPP TSG RAN WG1 #100bis			                                   					       												        R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.11.4

Source:	Moderator (AT&T)

Title:	Summary of Email Approval [100e-b-NR-UEFeatures-Remaining] — 5G V2X with NR Sidelink Aspects

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email approval [100e-b-NR-UEFeatures-Remaining] following RAN1 #100bis-e (5G V2X with NR sidelink aspects only). According to the Chairman’s Notes:

[100e-b-NR-UEFeatures-Remaining] Email discussion/approval of remaining issues (especially the one identified as low priority items in FL’s summaries) starting no earlier than 4/30 till next meeting – Hiroki (DCM)/Ralf (ATT)

The following was discussed and agreed after RAN1 #100bis-e within the scope of [100e-b-NR-UEFeatures-Remaining] (5G V2X with NR sidelink aspects only).

# Summary of Email Approval [100e-b-NR-UEFeatures-Remaining] — 5G V2X with NR Sidelink Aspects

RAN1 discussed all FGs for 5G V2X during RAN1 #100bis-e [3], however, due to lack of time, only FGs 15-1, 15-3, 15-4, 15-10, 15-18, 15-19, 15-23 were agreed as baseline, working assumption or otherwise [4]. That leaves FGs 15-2, 15-5, 15-6, 15-7, 15-9, 15-11, 15-12, 15-14, 15-1, 15-16, 15-22 without agreement (see yellow highlighting in the Appendix). In addition to [3], all NR V2X FGs were also discussed by conference call on April 30, 2020. At the end of the conference call, there were no additional comments to edit any of the FGs for NR V2X in [2]. Even the yellow highlighted FGs that lack a formal agreement should thus be very stable, stable enough to be agreed as baseline similar to the agreements for FGs 15-1, 15-3, and 15-4 during RAN1 #100bis-e.

The purpose of this email discussion summary document is to collect views on the yellow highlighted FGs that lack a formal agreement, to reach consensus on a baseline for all of them, which can then be agreed as part of the email discussion/approval [100e-b-NR-UEFeatures-Remaining]. RAN1 #101-e can then be used to finalize FFS points in the agreed baselines for NR V2X FGs with the intention to at least conclude on all aspects with ASN.1 impact.

The FGs below already capture feedback from all companies that participated in [3][4]. Additional comments can be provided in the tables here. In a second round, based on that company feedback, [100e-b-NR-UEFeatures-Remaining] would strive to reach consensus on a baseline for each FG as basis for RAN1 #101-e. Note that a large number of FFS points per row is acceptable and expected, as the main purpose of this discussion is to agree that the FGs highlighted in yellow will be introduced. As such, the proposed procedure for yellow FGs 15-2, 15-5, 15-6, 15-7, 15-9, 15-11, 15-12, 15-14, 15-1, 15-16, 15-22 is similar to what was done for FGs 15-1, 15-3, and 15-4 during RAN1 #100bis-e.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-2 | Transmitting NR sidelink mode 1 scheduled by NR Uu | 1) UE can transmit PSCCH/PSSCH using dynamic scheduling or configured grant type 1 and 2 in NR sidelink mode 1 scheduled by NR Uu. Up to [8] configured grants can be configured for a UE. Up to [C] sidelink processes are supported. 2) UE supports transmission based on the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can monitor DCI format 3_0 for NR sidelink dynamic scheduling and configured grant type 2. 6) UE can transmit using the subcarrier spacing [and CP length] it reports. FFS: 7) CP length 8) Supports 14-symbol SL slot with [all/some] DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 4}, {9, 3} for slots w/wo PFSCH [9) Support downlink pathloss based open loop power control] [10) SL HARQ feedback on Uu] |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. FFS: This is the basic FG for sidelink in licensed spectrum where gNB is operating on or managing that spectrum and optional FG otherwise Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} FFS: whether to mandate an SCS. | Optional with capability signalling FFS: For UE supports NR sidelink in licensed spectrum where gNB is defined, UE must indicate this FG is supported. Candidate values for C are {value1, value2 …} |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE/Sanechips | Agree that downlink pathloss based open loop power control should be here as component 9 and component 11 in 15-3 respectively , thus DLPC should be deleted from 15-23 |
| CATT | Since the SL HARQ feedback forward to gNB is only applied in mode 1. Agree to include component 10 in 15-2, and remove the component 4 in 15-11. |
| Huawei, HiSilicon | We agree with general structure of FG 15-2, but we have some further comments. Comments on the components: - For component 1), as in 15-1, ‘Up to [C] sidelink processes are supported’ should be ‘Up to [C] sidelink HARQ processes are supported’. However, clarity around the wording of this component in 15-1 and 15-2, in terms of broadcast and groupcast is still needed. - For component 2), remove “MCS table” as we should not split support transmission of a modulation scheme from support of using its associated MCS table.. - For component 6), we support “[and CP length]”. ECP can only applied to 60kHz SCS, so there is no need to specify a component separately. Component 7) should be deleted. - For component 8), we prefer mandating all DMRS patterns for 14-symbol SL slot. The component should be revised to “Supports 14-symbol SL slot with all DMRS patterns corresponding to #PSSCH symbols = 12, 9 for slots w/wo PFSCH”. In this regard, the DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 2}, {12, 3}, {12, 4}, {9, 2}, {9, 3} for slots w/wo PFSCH should be supported. Mandating all DMRS patterns for x-symbol SL slot helps to prevent over-dimensioning the resource pool w.r.t. TimePatternPsschDmrs configurations. - For component 9), agree. The bracketing should be removed since the licensed carrier would be used. Pathloss based power control is the main mechanism to reduce the interference to the licensed band. - For component 10), agree. The bracketing should be removed. We are also okay if this component is kept in FG 15-11 thanks to the “Note” method. Comments on the “Note” column: - For the first FFS about basic FG: We believe this FG should be basic FG for sidelink in licensed spectrum. - For the second FFS about SCS, disagree. Mode 1 transmission does not need a mandatory SCS. - For component-6, we should add a further note like: ‘the supported SCS should be the same as the Uu link in licensed carrier’, otherwise, UE has to support multiple different SCS BWP which hasn’t been supported until Rel-16. Comments on the “Mandatory/Optional” column: - For the FFS about basic FG, agree. - “Candidate values for C are {value1, value2 …}” should be moved to the “Note” column after it is settled. |
| Panasonic | We agree component 9 of DL pathloss here. We agree component 10 of SL HARQ feedback on Uu here. On SCS, whether the network deployment of Uu and sidelink could be different and IODT availability can be different. Therefore, except one SCS (30 kHz in FR1 and 60 kHz in FR2?), the remaining SCS should be separately indicated to show the test is finished or not. |
| Qualcomm | Component 6: The required SCS should follow Uu UL required/optional SCS in that band. In a band without Uu UL, 30 kHz can be default in FR1 Component 8: all DMRS patterns for 12-symbol and 9-symbol PSSCH (14 and 11 SL symbols) should be supported. This is important to avoid fragmenting the system. Component 9: We prefer to keep DL-based OLPC as part of the OLPC feature. Component 10: We prefer it in FG 15-11 We don’t think that FG 15-2 should be required in a licensed band. It is still possible to have Mode 2-only operation in a licensed band where the network only configured the UEs. |

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-5 | Sidelink congestion control | 1) UE can report CBR measurement to gNB [when operating in Mode 1] [and mode 2] (FFS: delete component 1) 2) UE can adjust its radio parameters based on CBR measurement and CRlimit. 3) UE can process CBR and CR within the time it indicates | 15-1 and at least one of 15-2 and 15-3 | Yes | FFS |  | Per band | N.A. | N.A. | N.A. | FFS: This is the basic FG for NR sidelink FFS: details of components (1) Component-3 candidate value set {Congestion process time 1, Congestion process time 2} where Congestion process time 1: 2, 2, 4, 8 slots for 15, 30, 60, 120 kHz subcarrier spacing. Congestion process time 2: 2, 4, 8, 16 slots for 15, 30, 60, 120 kHz subcarrier spacing | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE/Sanechips | Component 2) should be under mode 2. 2) UE can adjust its radio parameters based on CBR measurement and CRlimit in mode 2. |
| CATT | First, the sidelink congestion control should be a basic FG, the reason is that sidelink congestion control is a resource pool specific configuration, if congestion control is enabled for a resource pool, and UEs supporting congestion control and UEs not supporting congestion control share this pool, it is unfair to the UEs supporting congestion control. Second, regarding to component 1, since it is only related to Uu connection, we don't think it is tightly bundled with sidelink congestion control. from our perspective, it would be better to remove it from 15-5. And optional manner could be similar as that in 15-1/15-3, that means component 1 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 |
| Huawei, HiSilicon | Comments on the components: - For component 1), we support this component. Alternatively, the content “[when operating in Mode 1] [and mode 2] (FFS: delete component 1)” can be removed. CBR measurement is important for mode 1 as well as mode 2. Even for OoC mode 2, CBR is still needed. E.g. in 36.331 for LTE, there is signaling for OoC as SL-CBR-PreconfigTxConfigList-r14. The current 38.331 also include pre-configuration signaling ‘SL-CBR-CommonTxConfigList-r16’. Comments on the “signalling exchange” column: - We think there is no need to exchange this capability. Comments on the “Note” column: - The first FFS should be confirmed in the positive. We believe this FG should be basic FG for sidelink. |
| Panasonic | Component 1) and 3) are under mode 2 Tx. On component 2), depending on upper layer specification, when congested, to reduce the amount of Tx traffic or to keep the traffic but radio parameter usage can be different, and its choice may be up to regional deployment decision. Therefore, this 15-5 is kept only for component 2). Then, this is not required to be indicated to gNB. |
| Qualcomm | Component 1: should be kept. If the brackets around Mode 2 are removed, then it should be tied to the note on supporting gNB configuration. Components 2 and 3: we support keeping those components. Reporting the capability to gNB depends on which components are kept. There is no need to report to other UEs. Components 2 and 3 should be required for UEs implementing sidelink. |

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-6 | Short-term time-scale TDM for in-device coexistence | - Support prioritization between LTE sidelink transmission/reception and NR sidelink transmission/reception | At least one of 15-1, 15-2, 15-3 | No | No |  | Per band | N.A. | N.A. | N.A. | FFS whether a set of candidate values need to be defined for the time required for the inter-RAT conflict resolution | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE/Sanechips | Agreed |
| CATT | It is better for UE to report the processing time, otherwise it would be difficult to test the UE’s behaviour. |
| Huawei, HiSilicon | Comments on the “Note” column: - The FFS part of the note column should be removed: FFS whether a set of candidate values need to be defined for the time required for the inter-RAT conflict resolution. It’s up to UE implementation if inter-RAT conflict happened as the following agreed. - We note comments from other companies that the working assumption deals only with the case of a tie in priorities: this is not the case, as can be seen below. We note also the comments from other companies that the scheduler needs candidate values derived from those used in NR→LTE cross-RAT control. This is not the case, because cross-RAT control includes transfer of DCI information from a Uu interface and between two RAT modules. In-device prioritization does not have the same timeline, and there are few cases where the priorities can be known in advance. This is why the agreements below leave it up to implementation. #### RAN1#96bis Working assumption: - For Tx/Tx overlap, - If packet priorities of both LTE and NR sidelink transmissions are known to both RATs prior to time of transmission subject to processing time restriction, then the packet with a higher relative priority is transmitted - In case the priorities of LTE and NR SL transmissions are the same, then it is up to UE implementation as to which transmission is chosen (e.g., taking into account congestion, etc.) - If packet priorities of both LTE and NR sidelink transmissions are not known to both RATs prior to time of transmission subject to processing time restriction, then it is up to UE implementation to manage Tx/Tx overlaps (e.g., LTE transmissions are always prioritized, etc.) - RAN1 does not assume any impact to LTE physical layer specifications #### RAN1#97 Agreements: - For Tx/Tx overlap, - Confirm the working assumption made in RAN1#96bis - UE capability is defined for short-term time-scale TDM for in-device coexistence - For Rx/Rx overlap, - Up to UE implementation to manage receptions of LTE and NR sidelinks. #### RAN1#98 Agreements: Unless packet priorities of both LTE and NR sidelink are known to both RATs prior to time of collision (subject to processing time restriction), then - It is up to UE implementation to handle LTE Tx/NR Rx overlap. - It is up to UE implementation to handle NR Tx and LTE Rx overlap. |
| Panasonic | Agreed. |
| Qualcomm | Agree with the feature. |

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-7 | Transmitting LTE sidelink mode 3 scheduled by NR Uu | 1) UE can be scheduled over NR Uu for LTE sidelink mode 3 transmission.. 2) UE reports a value ‘X’ for the minimum value it supports for the additional time indicated in the NR DCI scheduling LTE sidelink mode 3. - 3) UE can monitor DCI format 3_1 for LTE sidelink SPS grant. |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Component-2 candidate value set: {0ms, 0.25ms, 0.5ms, 0.625ms, 0.75ms, 1ms, 1.25ms, 1.5ms,1.75ms, 2ms, 2.5ms, 3ms, 4ms, 5ms, 6ms, 8ms, 10ms, 20 ms } | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE/Sanechips | Agreed |
| CATT | Agreed |
| Huawei, HiSilicon | Support this FG. Numbering of the components to be tidied up. |
| Panasonic | Agreed |
| Qualcomm | Agree with the feature. |

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-9 | Transmitting LTE sidelink mode 4 configured by NR Uu | 1) UE can be configured over NR Uu for LTE sidelink mode 4 operation |  | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE/Sanechips | Agreed |
| CATT | Agreed |
| Huawei, HiSilicon | Support this FG. |
| Panasonic | Agreed |
| Qualcomm | Agree with the feature. |

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-11 | PSFCH format 0 | 1) UE can transmit and receive NR PSFCH format 0 2) UE can receive [N] PSFCH(s) resources in a slot. 3) UE can transmit [M] PSFCH(s) resources in a slot. [4) UE can report sidelink HARQ-ACK to gNB via PUCCH and PUSCH when it is operating in NR sidelink mode 1.] [FFS: move to 15-2] | At least one of 15-1, 15-2, 15-3 | FFS | FFS |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 4 is not required to be supported in a band indicated with the PC5 interface in 38.101-1 Table 5.2E-1 | Optional with capability signalling FFS: For UE supports NR sidelink, UE must indicate this FG is supported. ALT 1) Candidate values for N are {5, [10,] 15, [20,] 25, [30,] 35, [40,] 45, 50 } ALT 2) Candidate values for N are {32, 64} Candidate values for M are {1, 4, [5,] 8, 16} |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE/Sanechips | Agreed |
| CATT | Move component 4 to 15-2 |
| Huawei, HiSilicon | Comments on the components: - For component 4), we prefer moving this component to FG 15-2 since it can only be applied to mode 1 and it is related to transmission on uplink, while component 1), 2) and 3) are related to sidelink only. Yet, we are also okay if this component is kept in FG 15-11 thanks to the “Note” method. Comments on the “Mandatory/Optional” column: - For the candidate values for N, support Alt. 2 {32, 64}. For M, support {1, 4, 8, 16}. As a first step, it needs to be clarified for Alt. 1 why so many different values can be justified given the divergence this could cause among UE deployments. These candidate values should be move to the “Note” column after they are settled. |
| Panasonic | Agreed. "Need for the gNB to know if the feature is supported" is Yes. |
| Qualcomm | This feature is a basic feature and should be supported by all UEs supporting NR sidelink. Given that, there is no need to report the feature to other UEs or to gNB. We prefer to keep Component 4 here. If it is moved to FG 15-2, we prefer to also incorporate the other components into FG 15-1, 15-2, and 15-3. |

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-12 | Low-spectral efficiency 64QAM MCS table | 1) UE can transmit or receive PSSCH with low-spectral efficiency 64QAM MCS table. | At least one of 15-1, 15-2, 15-3 | Yes | Yes | UE supports normal 64QAM MCS table and 256QAM MCS table only. | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE/Sanechips | Agreed |
| CATT | Agreed |
| Huawei, HiSilicon | Support this FG. |
| Panasonic | Agreed |
| Qualcomm | Agree with the feature Need to update “consequence if not supported” to remove mention of 256-QAM table. |

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-14 | Sidelink CSI report | 1) UE can transmit and receive sidelink CSI-RS with 1 or 2 antenna port(s). 2) UE supports RI and CQI feedback on sidelink. | 15-1 and at least one of 15-2 and 15-3 | No | FFS |  | Per band | N.A. | N.A. | N.A. | FFS: This is the basic FG for NR sidelink | Optional with capability signalling. |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE/Sanechips | This FG should be per UE given the report is dependent on the CSI-RS transmission configuration PC5-RRC CSIsiReporting or the 2nd SCI configuration |
| CATT | This FG is unnecessary to be a basic FG. |
| Huawei, HiSilicon | Comments on the “Need for the gNB to know if the feature is supported” column: - It should be “No” since SL CSI should be mandatory for NR sidelink. Comments on the “Note” column: - We support confirming the FFS in the positive. We think SL CSI is one of the key design to improve the sidelink reliability and efficiency compared to LTE V2X. Unicast in Rel-16 needs to be ensured of a sufficient level of basic support, even if a particular feature within it is not applicable to groupcast/broadcast. Also, note that for NR UE, the “2-32 Basic CSI feedback” feature is mandatory without capability signalling. We should reuse the relevant aspect of NR Uu design to enhance the performance of NR sidelink. As a result, there is no need to exchange the capability between NR UEs. |
| Panasonic | Agreed. |
| Qualcomm | Component 1: we should remove 2 antenna ports or move into a separate feature because transmission with two ports is optional and reception with two ports is still being discussed (we think it should be optional) This feature should be reported to other UEs. We don’t think there is need to make this a required feature for sidelink. |

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-15 | eNB type synchronization source for NR sidelink | 1) UE can transmit or receive NR sidelink based on the synchronization to an eNB. 2) If UE supports 15-4, UE additionally supports eNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports 15-4, UE additionally supports eNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE/Sanechips | We prefer this deleted or merged into 15-4 with corresponding description modifications to 15-4 |
| CATT | Agreed |
| Huawei, HiSilicon | Support this FG. |
| Panasonic | Agreed. |
| Qualcomm | Agree with the feature |

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-16 | Simultaneous transmission of uplink and sidelink | 1) UE supports simultaneous transmission of NR uplink and NR sidelink (on different carriers) in a band combination for which the UE indicated simultaneous sidelink and uplink support in a band combination. | At least one of 15-2 and 15-3 | Yes | No |  | Per band combination | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE/Sanechips | Agreed |
| CATT | Agreed |
| Huawei, HiSilicon | Support this FG. |
| Panasonic | Agreed |
| Qualcomm | Agree with the feature |

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-22 | Support of SL slot less than 14 consecutive symbols | - UE additionally supports transmission/reception of SL slot configured with 7, 8, 9, 10, 11, 12, 13 consecutive symbols [and the corresponding DMRS patterns it reports.] - [UE supports [some/all] applicable DMRS patterns for the number of consecutive Sl symbols it reports] | At least one of 15-1, 15-2, 15-3 | Yes | No | UE supports SL only in a SL slot configured with 14 consecutive symbols. | Per band | N.A. | N/A | N.A. | FFS: This is the basic FG for NR sidelink The component-1 candidate value set can be DRMS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {{12,2},{12,1}, {11,4},{11,3},{11,2}, {10,4},{10,3},{10,2}, {9,2},{8,3},{8,2},{7,2},{6,2}, {5,2}} | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE/Sanechips | The #PSSCH symbol in the note seems to represent the actual PSSCH symbol number excluding AGC symbol and gap symbol per mandate symbol DMRS patterns {{12,4}{9,3}} in 15-2. In that sense {12,2},{12,1} should not be here and{9,3} is missing. |
| CATT | From our perspective, it should be optional. Otherwise, it is unnecessary to introduce a separate FG, it should be a component in 15-1/15-2/15-3. |
| Huawei, HiSilicon | First, the FG name would be more precise as “Support of fewer than 14 consecutive sidelink symbols in a slot”, i.e. the slot length is not changing, only the SL allocation within it. Comments on the components: - For component 1) and 2), we prefer deleting “[and the corresponding DMRS patterns it reports.]” Component 2) can be “UE supports all applicable DMRS patterns for the number of consecutive SL symbols it reports”. Mandating all DMRS patterns for x-symbol SL slot helps to prevent over-dimensioning the resource pool w.r.t. TimePatternPsschDmrs configurations.. Comments on the “Note” column: - We prefer this FG to be optional. - Propose deleting the “The component-1 candidate value set…” since we think all the applicable DMRS patterns for the number of consecutive SL symbols it reports should be supported. |
| Panasonic | Agreed. |
| Qualcomm | Component 1: The UE should support all DMRS patterns for a supported number of sidelink symbols. Component 2: Same comment on supporting all DMRS patterns for a given SL length We think that this is an optional feature and shouldn’t be in the basic feature set. |

# Conclusions

…

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-1 | Receiving NR sidelink | 1) UE can receive NR PSCCH/PSSCH. Up to [A] sidelink HARQ processes are supported. 2) UE can receive [X] PSCCH in a slot. 3) UE can decode [Y] RBs per slot (FFS: counting both PSCCH and PSSCH). 4) UE supports reception based on the normal 64QAM MCS table [and 256QAM MCS table in FR1]. 5) UE supports PT-RS reception in FR2. FFS: 6) The UE can receive [Z] total number of soft channel bits in a slot. [7) minimum two receive antennas] 8) UE can receive using the subcarrier spacing [and CP length] [defined for a given band in R15 in RAN4] [as configured for NR UL] FFS: 9) CP length 10) Supports 14-symbol SL slot with [all/some] DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 4}, {9, 3} for slots w/wo PFSCH [11) UE can receive PSSCH with 256QAM in NR sidelink] 12) [for NR SL by preconfiguration] UE can receive using 30 kHz subcarrier spacing in FR1, FFS FR2 | None | FFS | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 8 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 FFS: details for component (10) Note: Component 12 is required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Component-2 candidate value set: {value1, value2, …} FFS: whether to report different value for each SCS indicated in component-8 Component-3 candidate value set: {value1, value2, …} FFS: whether to report different value for each SCS indicated in component-8 FFS: Component-6 candidate value set: {value1, value2, …} [Component-8 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-8 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}}] [Component-12 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-8 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}}] Candidate values for A are {value1, value2 …} | Optional with capability signaling. For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-2 | Transmitting NR sidelink mode 1 scheduled by NR Uu | 1) UE can transmit PSCCH/PSSCH using dynamic scheduling or configured grant type 1 and 2 in NR sidelink mode 1 scheduled by NR Uu. Up to [8] configured grants can be configured for a UE. Up to [C] sidelink HARQ processes are supported. 2) UE supports transmission based on the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can monitor DCI format 3_0 for NR sidelink dynamic scheduling and configured grant type 2. 6) UE can transmit using the subcarrier spacing [and CP length] it reports. FFS: 7) CP length 8) Supports 14-symbol SL slot with [all/some] DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 4}, {9, 3} for slots w/wo PFSCH [9) Support downlink pathloss based open loop power control] [10) SL HARQ feedback on Uu] |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. FFS: This is the basic FG for sidelink in licensed spectrum where gNB is operating on or managing that spectrum and optional FG otherwise Candidate values for C are {value1, value2 …} Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} FFS: whether to mandate an SCS. [Note: For component (6) the supported SCS should be the same as the Uu link in licensed carrier] | Optional with capability signalling FFS: For UE supports NR sidelink in licensed spectrum where gNB is defined, UE must indicate this FG is supported. Candidate values for C are {value1, value2 …} |
| 15-3 | Transmitting NR sidelink mode 2 | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by NR Uu or preconfiguration. Up to [B] sidelink processes are supported. 2) UE supports transmission based on the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. [4) UE can perform sensing and resource allocation operations.] [5) UE supports rank 1 PSSCH transmissions.] 6) UE can transmit using the subcarrier spacing [and CP length] it reports for FG 15-1. FFS: 7) CP length 8) Supports 14-symbol SL slot with [all/some] DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 4}, {9, 3} for slots w/wo PFSCH 9) default SCS with pre-configuration: 30 kHz with normal CP: [operator managed] same as Rel. 15 Uu 10) UE can transmit using 30 kHz [and normal CP] subcarrier spacing in FR1, FFS FR2 [11) DL pathloss based open loop power control when mode 2 is configured by NR Uu] | 15-1 | FFS | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 This is the basic FG for sidelink [in ITS spectrum where gNB is not defined and optional FG for licensed spectrum where gNB is defined] [Note: Component 5 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1] Note: Component 6 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 10 is required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 FFS: all details for component (11) | Optional with capability signalling For UE supports NR sidelink, [for UE supports NR sidelink in ITS spectrum where gNB is not defined, UE must indicate this FG is supported,] UE must indicate this FG is supported. Candidate values for B are {value1, value2 …} |
| 15-4 | Synchronization sources for NR sidelink | 1) UE can receive S-SSB in NR sidelink if it supports 15-1. 2) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3. 3) UE supports GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 4) UE can transmit or receive NR sidelink based on the synchronization to an gNB 5) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 6) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 4 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 5 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 6 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 | Optional with capability signalling For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-5 | Sidelink congestion control | 1) UE can report CBR measurement to gNB [when operating in Mode 1] [and mode 2] (FFS: delete component 1) 2) UE can adjust its radio parameters based on CBR measurement and CRlimit. [in mode 2]. 3) UE can process CBR and CR within the time it indicates [in mode 2] | 15-1 and at least one of 15-2 and 15-3 | Yes | FFS |  | Per band | N.A. | N.A. | N.A. | FFS: This is the basic FG for NR sidelink FFS: details of components (1) [Note: component 1 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1] Component-3 candidate value set {Congestion process time 1, Congestion process time 2} where Congestion process time 1: 2, 2, 4, 8 slots for 15, 30, 60, 120 kHz subcarrier spacing. Congestion process time 2: 2, 4, 8, 16 slots for 15, 30, 60, 120 kHz subcarrier spacing | Optional with capability signalling |
| 15-6 | Short-term time-scale TDM for in-device coexistence | Support prioritization between LTE sidelink transmission/reception and NR sidelink transmission/reception | At least one of 15-1, 15-2, 15-3 | No | No |  | Per band | N.A. | N.A. | N.A. | FFS whether a set of candidate values need to be defined for the time required for the inter-RAT conflict resolution | Optional with capability signalling |
| 15-7 | Transmitting LTE sidelink mode 3 scheduled by NR Uu | 1) UE can be scheduled over NR Uu for LTE sidelink mode 3 transmission.. 2) UE reports a value ‘X’ for the minimum value it supports for the additional time indicated in the NR DCI scheduling LTE sidelink mode 3. 3) UE can monitor DCI format 3_1 for LTE sidelink SPS grant. |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Component-2 candidate value set: {0ms, 0.25ms, 0.5ms, 0.625ms, 0.75ms, 1ms, 1.25ms, 1.5ms,1.75ms, 2ms, 2.5ms, 3ms, 4ms, 5ms, 6ms, 8ms, 10ms, 20 ms } | Optional with capability signalling |
| 15-9 | Transmitting LTE sidelink mode 4 configured by NR Uu | 1) UE can be configured over NR Uu for LTE sidelink mode 4 operation |  | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 15-10 | 256QAM sidelink transmission | 1) UE can transmit PSSCH with 256QAM in NR sidelink | At least one of 15-2, 15-3 | Yes | Yes | UE supports QPSK, 16QAM, and 64 QAM for transmission only. | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 15-11 | PSFCH format 0 | 1) UE can transmit and receive NR PSFCH format 0 2) UE can receive [N] PSFCH(s) resources in a slot. 3) UE can transmit [M] PSFCH(s) resources in a slot. [4) UE can report sidelink HARQ-ACK to gNB via PUCCH and PUSCH when it is operating in NR sidelink mode 1.] [FFS: move to 15-2] | At least one of 15-1, 15-2, 15-3 | FFS | FFS |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 4 is not required to be supported in a band indicated with the PC5 interface in 38.101-1 Table 5.2E-1 | Optional with capability signalling FFS: For UE supports NR sidelink, UE must indicate this FG is supported. ALT 1) Candidate values for N are {5, [10,] 15, [20,] 25, [30,] 35, [40,] 45, 50 } ALT 2) Candidate values for N are {32, 64} Candidate values for M are {1, 4, [5,] 8, 16} |
| 15-12 | Low-spectral efficiency 64QAM MCS table | 1) UE can transmit or receive PSSCH with low-spectral efficiency 64QAM MCS table. | At least one of 15-1, 15-2, 15-3 | Yes | Yes | UE supports normal 64QAM MCS table and 256QAM MCS table only. | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 15-14 | Sidelink CSI report | 1) UE can transmit and receive sidelink CSI-RS with 1 [or 2 antenna] port(s). 2) UE supports RI and CQI feedback on sidelink. | 15-1 and at least one of 15-2 and 15-3 | No | FFS |  | [Per band] | N.A. | N.A. | N.A. | FFS: This is the basic FG for NR sidelink | Optional with capability signalling. |
| 15-15 | eNB type synchronization source for NR sidelink | 1) UE can transmit or receive NR sidelink based on the synchronization to an eNB. 2) If UE supports 15-4, UE additionally supports eNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports 15-4, UE additionally supports eNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 15-16 | Simultaneous transmission of uplink and sidelink | 1) UE supports simultaneous transmission of NR uplink and NR sidelink (on different carriers) in a band combination for which the UE indicated simultaneous sidelink and uplink support in a band combination. | At least one of 15-2 and 15-3 | Yes | No |  | Per band combination | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 15-18 | Support of rank 2 transmission | 1) UE additionally supports rank 2 PSSCH transmission | [At least one of 15-2 and 15-3] | FFS | FFS | UE supports rank 1 PSSCH transmission only. | Per band | N.A. | N.A. | N.A. | This FG is a WA | Optional with capability signalling |
| 15-19 | Support of rank 2 reception | 1) UE additionally supports rank 2 PSSCH reception | [15-1] | FFS | FFS | UE supports rank 1 PSSCH reception only. | Per band | N.A. | N.A. | N.A. | This FG is a WA. FFS: This is the basic FG for NR sidelink | [Optional with capability signalling] |
| 15-22 | Support of SL slot less than 14 consecutive symbols Support of fewer than 14 consecutive sidelink symbols in a slot | 1) UE additionally supports transmission/reception of SL slot configured with 7, 8, 9, 10, 11, 12, 13 consecutive symbols [and the corresponding DMRS patterns it reports.] [2) UE supports [some/all] applicable DMRS patterns for the number of consecutive Sl symbols it reports] | At least one of 15-1, 15-2, 15-3 | Yes | No | UE supports SL only in a SL slot configured with 14 consecutive symbols. | Per band | N.A. | N/A | N.A. | FFS: This is the basic FG for NR sidelink [The component-1 candidate value set can be DRMS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {[{12,2}, {12,1},] {11,4}, {11,3}, {11,2}, {10,4}, {10,3}, {10,2}, {9,2}, [{9,3},] {8,3}, {8,2}, {7,2}, {6,2}, {5,2}}] | Optional with capability signalling |
| 15-23 | Support of open loop SL power control and RSRP report | 1) Support sidelink pathloss based open loop power control and RSRP report in case of unicast [2) downlink pathloss based OLPC] | FFS | FFS | Yes |  | Per band | N.A. | N/A | N.A. | Working assumption: This FG is a basic UE FG [at least] for UEs supporting mode 1 FFS: all details for component (2) FFS: whether this is a basic FG also for UEs not supporting mode 1 | Optional with capability signalling |
| 15-24 | [Support of multiple synchronization references] | [1) UE can support sidelink reception using up to A synchronziaion references in a carrier/BWP] | At least one of 15-1, 15-2, 15-3 | Yes | No | UE supports only a single synchronization reference in a carrier/BWP. | Per band | N.A. | N.A. | N.A. | Component-1 candidate value set: {1, 2, 3, 4} Note: RAN1 is still discussing whether this FG is needed | Optional with capability signalling |

# References

- R1-2003072, LS on Rel-16 RAN1 UE features lists for NR, RAN1

- R1-2003073, RAN1 UE features list for Rel-16 NR after RAN1#100bis-E, AT&T/NTT DOCOMO, INC.

- R1-2003093, Summary of Email Approval [100b-e-NR-UEFeatures-V2X-04], Moderator (AT&T)

- R1-2003078, Chairman's Notes of AI 7.2.11.4, Ad-Hoc Chair (AT&T)

# Appendix

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-1 | Receiving NR sidelink | 1) UE can receive NR PSCCH/PSSCH. Up to [A] sidelink HARQ processes are supported. 2) UE can receive [X] PSCCH in a slot. 3) UE can decode [Y] RBs per slot (FFS: counting both PSCCH and PSSCH). 4) UE supports reception based on the normal 64QAM MCS table [and 256QAM MCS table in FR1]. 5) UE supports PT-RS reception in FR2. FFS: 6) The UE can receive [Z] total number of soft channel bits in a slot. [7) minimum two receive antennas] 8) UE can receive using the subcarrier spacing [and CP length] [defined for a given band in R15 in RAN4] [as configured for NR UL] FFS: 9) CP length 10) Supports 14-symbol SL slot with [all/some] DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 4}, {9, 3} for slots w/wo PFSCH [11) UE can receive PSSCH with 256QAM in NR sidelink] 12) [for NR SL by preconfiguration] UE can receive using 30 kHz subcarrier spacing in FR1, FFS FR2 | None | FFS | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 8 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 FFS: details for component (10) Note: Component 12 is required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Component-2 candidate value set: {value1, value2, …} FFS: whether to report different value for each SCS indicated in component-8 Component-3 candidate value set: {value1, value2, …} FFS: whether to report different value for each SCS indicated in component-8 FFS: Component-6 candidate value set: {value1, value2, …} [Component-8 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-8 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}}] [Component-12 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-8 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}}] Candidate values for A are {value1, value2 …} | Optional with capability signaling. For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-2 | Transmitting NR sidelink mode 1 scheduled by NR Uu | 1) UE can transmit PSCCH/PSSCH using dynamic scheduling or configured grant type 1 and 2 in NR sidelink mode 1 scheduled by NR Uu. Up to [8] configured grants can be configured for a UE. Up to [C] sidelink processes are supported. 2) UE supports transmission based on the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can monitor DCI format 3_0 for NR sidelink dynamic scheduling and configured grant type 2. 6) UE can transmit using the subcarrier spacing [and CP length] it reports. FFS: 7) CP length 8) Supports 14-symbol SL slot with [all/some] DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 4}, {9, 3} for slots w/wo PFSCH [9) Support downlink pathloss based open loop power control] [10) SL HARQ feedback on Uu] |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. FFS: This is the basic FG for sidelink in licensed spectrum where gNB is operating on or managing that spectrum and optional FG otherwise Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} FFS: whether to mandate an SCS. | Optional with capability signalling FFS: For UE supports NR sidelink in licensed spectrum where gNB is defined, UE must indicate this FG is supported. Candidate values for C are {value1, value2 …} |
| 15-3 | Transmitting NR sidelink mode 2 | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by NR Uu or preconfiguration. Up to [B] sidelink processes are supported. 2) UE supports transmission based on the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. [4) UE can perform sensing and resource allocation operations.] [5) UE supports rank 1 PSSCH transmissions.] 6) UE can transmit using the subcarrier spacing [and CP length] it reports for FG 15-1. FFS: 7) CP length 8) Supports 14-symbol SL slot with [all/some] DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 4}, {9, 3} for slots w/wo PFSCH 9) default SCS with pre-configuration: 30 kHz with normal CP: [operator managed] same as Rel. 15 Uu 10) UE can transmit using 30 kHz [and normal CP] subcarrier spacing in FR1, FFS FR2 [11) DL pathloss based open loop power control when mode 2 is configured by NR Uu] | 15-1 | FFS | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 This is the basic FG for sidelink [in ITS spectrum where gNB is not defined and optional FG for licensed spectrum where gNB is defined] [Note: Component 5 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1] Note: Component 6 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 10 is required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 FFS: all details for component (11) | Optional with capability signalling For UE supports NR sidelink, [for UE supports NR sidelink in ITS spectrum where gNB is not defined, UE must indicate this FG is supported,] UE must indicate this FG is supported. Candidate values for B are {value1, value2 …} |
| 15-4 | Synchronization sources for NR sidelink | 1) UE can receive S-SSB in NR sidelink if it supports 15-1. 2) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3. 3) UE supports GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 4) UE can transmit or receive NR sidelink based on the synchronization to an gNB 5) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 6) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 4 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 5 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 6 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 | Optional with capability signalling For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-5 | Sidelink congestion control | 1) UE can report CBR measurement to gNB [when operating in Mode 1] [and mode 2] (FFS: delete component 1) 2) UE can adjust its radio parameters based on CBR measurement and CRlimit. 3) UE can process CBR and CR within the time it indicates | 15-1 and at least one of 15-2 and 15-3 | Yes | FFS |  | Per band | N.A. | N.A. | N.A. | FFS: This is the basic FG for NR sidelink FFS: details of components (1) Component-3 candidate value set {Congestion process time 1, Congestion process time 2} where Congestion process time 1: 2, 2, 4, 8 slots for 15, 30, 60, 120 kHz subcarrier spacing. Congestion process time 2: 2, 4, 8, 16 slots for 15, 30, 60, 120 kHz subcarrier spacing | Optional with capability signalling |
| 15-6 | Short-term time-scale TDM for in-device coexistence | - Support prioritization between LTE sidelink transmission/reception and NR sidelink transmission/reception | At least one of 15-1, 15-2, 15-3 | No | No |  | Per band | N.A. | N.A. | N.A. | FFS whether a set of candidate values need to be defined for the time required for the inter-RAT conflict resolution | Optional with capability signalling |
| 15-7 | Transmitting LTE sidelink mode 3 scheduled by NR Uu | 1) UE can be scheduled over NR Uu for LTE sidelink mode 3 transmission.. 2) UE reports a value ‘X’ for the minimum value it supports for the additional time indicated in the NR DCI scheduling LTE sidelink mode 3. - 3) UE can monitor DCI format 3_1 for LTE sidelink SPS grant. |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Component-2 candidate value set: {0ms, 0.25ms, 0.5ms, 0.625ms, 0.75ms, 1ms, 1.25ms, 1.5ms,1.75ms, 2ms, 2.5ms, 3ms, 4ms, 5ms, 6ms, 8ms, 10ms, 20 ms } | Optional with capability signalling |
| 15-9 | Transmitting LTE sidelink mode 4 configured by NR Uu | 1) UE can be configured over NR Uu for LTE sidelink mode 4 operation |  | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 15-10 | 256QAM sidelink transmission | 1) UE can transmit PSSCH with 256QAM in NR sidelink | At least one of 15-2, 15-3 | Yes | Yes | UE supports QPSK, 16QAM, and 64 QAM for transmission only. | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 15-11 | PSFCH format 0 | 1) UE can transmit and receive NR PSFCH format 0 2) UE can receive [N] PSFCH(s) resources in a slot. 3) UE can transmit [M] PSFCH(s) resources in a slot. [4) UE can report sidelink HARQ-ACK to gNB via PUCCH and PUSCH when it is operating in NR sidelink mode 1.] [FFS: move to 15-2] | At least one of 15-1, 15-2, 15-3 | FFS | FFS |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Note: Component 4 is not required to be supported in a band indicated with the PC5 interface in 38.101-1 Table 5.2E-1 | Optional with capability signalling FFS: For UE supports NR sidelink, UE must indicate this FG is supported. ALT 1) Candidate values for N are {5, [10,] 15, [20,] 25, [30,] 35, [40,] 45, 50 } ALT 2) Candidate values for N are {32, 64} Candidate values for M are {1, 4, [5,] 8, 16} |
| 15-12 | Low-spectral efficiency 64QAM MCS table | 1) UE can transmit or receive PSSCH with low-spectral efficiency 64QAM MCS table. | At least one of 15-1, 15-2, 15-3 | Yes | Yes | UE supports normal 64QAM MCS table and 256QAM MCS table only. | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 15-14 | Sidelink CSI report | 1) UE can transmit and receive sidelink CSI-RS with 1 or 2 antenna port(s). 2) UE supports RI and CQI feedback on sidelink. | 15-1 and at least one of 15-2 and 15-3 | No | FFS |  | Per band | N.A. | N.A. | N.A. | FFS: This is the basic FG for NR sidelink | Optional with capability signalling. |
| 15-15 | eNB type synchronization source for NR sidelink | 1) UE can transmit or receive NR sidelink based on the synchronization to an eNB. 2) If UE supports 15-4, UE additionally supports eNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports 15-4, UE additionally supports eNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 15-16 | Simultaneous transmission of uplink and sidelink | 1) UE supports simultaneous transmission of NR uplink and NR sidelink (on different carriers) in a band combination for which the UE indicated simultaneous sidelink and uplink support in a band combination. | At least one of 15-2 and 15-3 | Yes | No |  | Per band combination | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 15-18 | Support of rank 2 transmission | 1) UE additionally supports rank 2 PSSCH transmission | [At least one of 15-2 and 15-3] | FFS | FFS | UE supports rank 1 PSSCH transmission only. | Per band | N.A. | N.A. | N.A. | This FG is a WA | Optional with capability signalling |
| 15-19 | Support of rank 2 reception | 1) UE additionally supports rank 2 PSSCH reception | [15-1] | FFS | FFS | UE supports rank 1 PSSCH reception only. | Per band | N.A. | N.A. | N.A. | This FG is a WA. FFS: This is the basic FG for NR sidelink | [Optional with capability signalling] |
| 15-22 | Support of SL slot less than 14 consecutive symbols | - UE additionally supports transmission/reception of SL slot configured with 7, 8, 9, 10, 11, 12, 13 consecutive symbols [and the corresponding DMRS patterns it reports.] - [UE supports [some/all] applicable DMRS patterns for the number of consecutive Sl symbols it reports] | At least one of 15-1, 15-2, 15-3 | Yes | No | UE supports SL only in a SL slot configured with 14 consecutive symbols. | Per band | N.A. | N/A | N.A. | FFS: This is the basic FG for NR sidelink The component-1 candidate value set can be DRMS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {{12,2},{12,1}, {11,4},{11,3},{11,2}, {10,4},{10,3},{10,2}, {9,2},{8,3},{8,2},{7,2},{6,2}, {5,2}} | Optional with capability signalling |
| 15-23 | Support of open loop SL power control and RSRP report | - Support sidelink pathloss based open loop power control and RSRP report in case of unicast - [downlink pathloss based OLPC] | FFS | FFS | Yes |  | Per band | N.A. | N/A | N.A. | Working assumption: This FG is a basic UE FG [at least] for UEs supporting mode 1 FFS: all details for component (2) FFS: whether this is a basic FG also for UEs not supporting mode 1 | Optional with capability signalling |
| 15-24 | [Support of multiple synchronization references] | [1) UE can support sidelink reception using up to A synchronziaion references in a carrier/BWP] | At least one of 15-1, 15-2, 15-3 | Yes | No | UE supports only a single synchronization reference in a carrier/BWP. | Per band | N.A. | N.A. | N.A. | Component-1 candidate value set: {1, 2, 3, 4} Note: RAN1 is still discussing whether this FG is needed | Optional with capability signalling |