3GPP TSG RAN WG1 #100bis			                                   					       												        R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.11.6

Source:	Moderator (AT&T)

Title:	Summary of Email Approval [100e-b-NR-UEFeatures-Remaining] — MIMO Enhancement Aspects

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email approval [100e-b-NR-UEFeatures-Remaining] following RAN1 #100bis-e (MIMO enhancement aspects only). According to the Chairman’s Notes:

[100e-b-NR-UEFeatures-Remaining] Email discussion/approval of remaining issues (especially the one identified as low priority items in FL’s summaries) starting no earlier than 4/30 till next meeting – Hiroki (DCM)/Ralf (ATT)

The following was discussed and agreed after RAN1 #100bis-e within the scope of [100e-b-NR-UEFeatures-Remaining] (MIMO enhancement aspects only).

# Summary of Email Approval [100e-b-NR-UEFeatures-Remaining] — MIMO Enhancement Aspects

RAN1 discussed most FGs for NR MIMO enhancements during RAN1 #100bis-e [4][5][6][7][8], however, due to lack of time, only FGs 16-1b, 16-1b-2, 16-1b-3, 16-1g, 16-2a-5, 16-2b-0, 16-2b-1, 16-2b-2, 16-2b-3, 16-2b-4, 16-2b-5, 16-3a, 16-3a-1, 16-3a-2, 16-3a-3, 16-3a-4, 16-3b, 16-3b-1, 16-3b-2, 16-5a, 16-6a, 16-6c, 16-7, 16-8 were agreed as baseline, working assumption, starting point for further discussion, or otherwise [3]. That leaves FGs 16-1a-1, 16-1a-2, 16-1a-3, 16-1c, 16-1d, 16-1e, 16-1f, 16-4, 16-5b, 16-6b without agreement (see yellow highlighting in the Appendix copied from [1][2]).

The purpose of this email discussion summary document is to collect views on the yellow highlighted FGs that lack a formal agreement, to reach consensus on a baseline for all of them, which can then be agreed as part of the email discussion/approval [100e-b-NR-UEFeatures-Remaining]. RAN1 #101-e can then be used to finalize FFS points in the agreed baselines with the intention to at least conclude on all aspects with ASN.1 impact during RAN1 #101-e.

Comments can be provided in the tables below. In a second round, based on that company feedback, [100e-b-NR-UEFeatures-Remaining] would strive to reach consensus on a baseline for each FG as basis for RAN1 #101-e. Note that a large number of FFS points per row is acceptable and expected, as the main purpose of this discussion is to agree that the FGs highlighted in yellow will be introduced. Furthermore, if a FG turns out to be too controversial, the moderator may decide to discuss a baseline for said FG during RAN1 #101-e instead. The intention is to agree on baselines without too much discussion using squared brackets and FFS points. The proposed procedure for yellow FGs 16-1a-1, 16-1a-2, 16-1a-3, 16-1c, 16-1d, 16-1e, 16-1f, 16-4, 16-5b, 16-6b is thus similar to what was done for FGs 16-1b, 16-1b-2, 16-1b-3, 16-1g, 16-2a-5, 16-2b-0, 16-2b-1, 16-2b-2, 16-2b-3, 16-2b-4, 16-2b-5, 16-3a, 16-3a-1, 16-3a-2, 16-3a-3, 16-3a-4, 16-3b, 16-3b-1, 16-3b-2, 16-5a, 16-6a, 16-6c, 16-7, 16-8 during RAN1 #100bis-e.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16-1a-1 | SSB/CSI-RS for L1-SINR measurement | - The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-1 - The max number of NZP-IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR (including CMR and IMR) within a slot shall not exceed MB_1-3 - The max number of SSB/CSI-RS (1Tx)/CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR (including CMR and IMR) shall not exceed MC_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-1 - The max number of NZP IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MC_1-3 - The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_2 - Supported density of CSI-RS (CMR) - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 | 2-21, 2-22 or 2-23, 2-23a |  |  |  | Per band |  |  |  |  | Optional with capability signalling Component-1 to 4, candidate value set for MB_1 and MB_1-x is {0, 8, 16, 32, 64} Component-5 to 8, candidate value set for MC_1 and MC_1-x is {0, 4, 8, 16, 32, 64} Component-9, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-10: candidate value set: {"not supported", "1 only", "3 only", "both 1 and 3"} Component-11, candidate value set for MD_2 is {0, 1, 4, 8, 16, 32, 64} |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LG Electronics | Fine with having this FG. Several modification seems needed. - The type of this FG needs to be ‘per UE’ not ‘per band’ as this FG is defined across all CCs. - Component 4 and 8: Given that 16-1g defines the total limit of RS resources for multiple purposes, it is not clear to us on the necessity of component 4 and 8. We suggest putting FFS on these components for further discussion in RAN1#101-e. - Component 5: Following modification seems necessary to avoid any confusion (CSI-IM cannot be configured as CMR and there has been no 1Tx/2Tx differentiation for this type of component in Rel-15)  The max number of SSB/CSI-RS (1Tx)/CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR (including CMR and IMR) shall not exceed MC_1 |
| ZTE | - Firstly, the motivation of distinguishing 1-Tx CSI-RS and 2-Tx CSI-RS is unclear, considering that we also have the component “10. Supported density of CSI-RS (CMR)”. - Secondly, we only need to consider the maximum number of either CMR or IMR, separately, or consider the maximum number of CMR+IMR, e.g., 4/5 vs1/2/3. We don’t need both of them. Consequently, we have the following updates for components. Proposed changes for components: - The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-1 - The max number of NZP-IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR (including CMR and IMR) within a slot shall not exceed MB_1-3 - The max number of SSB/CSI-RS (1Tx)/CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR (including CMR and IMR) shall not exceed MC_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-1 - The max number of NZP IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR (including CMR and IMR) shall not exceed MC_1-3 - The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_2 - Supported density of CSI-RS (CMR) - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 |
| Intel | We are not sure whether so many components are needed. We therefore propose to have other structure: 1. Type of L1-SINR measurements with candidate values TBD, e.g. {L1-SINR based on dedicated IMR, L1-SINR without dedicated IMR, etc.} 2. The number of configured resources for CMR and IMR across CCs - The number of configured SS/PBCH resources - The number of configured NZP CSI-RS resources for CMR - The number of configured NZP CSI-RS resources for IMR - The number of configured CSI-IM resource for IMR 3. The number of configured resources for CMR within a slot - The number of configured SS/PBCH resources - The number of configured NZP CSI-RS resources for CMR - The number of configured NZP CSI-RS / CSI-IM resources for IMR |

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16-1a-2 | Non-group based L1-SINR reporting | - Support of non-group based L1-SINR reporting with N_max L1-SINR values reported | 16-1a-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling Candidate value set is {1, 2, 4} |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LG Electronics | Fine with current FG |
| ZTE | 16-1a-2 is independent from 16-1a-1, or 16-1a-1 depends on 16-1a-2. Then, 16-1a-2 should be considered as mandatory with capability signaling, like L1-RSRP reporting. |
| Intel | OK. Set default value as 1 for UE supporting 16-1a-1. |

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16-1a-3 | Group based L1-SINR reporting | - Support of group based L1-SINR reporting | 16-1a-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LG Electronics | Fine with current FG |
| ZTE | Support. |
| Intel | OK |

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16-1c | Default spatial relation | Support of default spatial relation and pathloss reference RS for dedicated-PUCCH/SRS and PUSCH scheduled by DCI format 0_0 | 2-53, 2-59 |  | N/A |  | TBD [Per band] | N | Y |  |  | TBD |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LG Electronics | Fine with this FG. One more prerequisite seems needed, i.e. component 1 of 2-20 is set to ‘1’ because this feature was agreed for the UEs supporting beam correspondence. If this is controversial, we suggest putting this in square brackets for now in order to discuss this in RAN1#101-e. |
| ZTE | Support with the following modification. “DCI format 0_0” should be removed considering the on-going discussion for PUSCH scheduled by other DCI format, e.g., 0_1. Proposed changes for components: Support of default spatial relation and pathloss reference RS for dedicated-PUCCH/SRS and PUSCH scheduled by DCI format 0_0 |
| Intel | OK. The feature should be per UE and only applicable to FR2. It should be also optional with capability signaling. |

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16-1d | MAC CE spatial relation update for AP-SRS | [Support of / The maximum number of] spatial relation update for AP-SRS via MAC CE | 2-53, 2-59 |  | N/A |  | TBD [Per UE] | N | N |  |  | TBD |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LG Electronics | Fine with the FG. |
| ZTE | Support with the following modification. Proposed changes for components: [Support of / The maximum number of] spatial relation update for AP-SRS via MAC CE |
| Intel | We prefer to define this FG as “Support of spatial relation update for AP-SRS via MAC CE”. The feature should be per UE and only applicable to FR2. It should be also optional with capability signaling. |

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16-1e | Pathloss reference RS activation via MAC CE | - The maximum number of configured pathloss reference RSs for PUSCH/SRS by RRC for MAC-CE based pathloss reference RS update - FFS: The maximum number of activated pathloss reference RS update for PUSCH/SRS/PUCCH [across CCs / within a slot across all CCs / per CC] - FFS: Number of measurement samples N to apply newly activated pathloss reference RS | 8-2, 8-3 |  | N/A |  | TBD [Per UE] | N | N |  |  | TBD |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LG Electronics | Fine with the FG but we believe that component 2 and 3 should be deleted. - Component 2 is not needed because spec already defines a fixed number, which is 4, as the maximum number of activated PL RSs since Rel-15. - Component 3 needs to be removed because it was agreed not to handle this issue in RAN1 during RAN1#100bis-e meeting. |
| ZTE | Support with the following modification. In our views, Rel-15 has specified that up to 4 pathloss RS per CC is supported, which is sufficient. No more discussion is needed for the following two FFS parts. Proposed changes for components: - The maximum number of configured pathloss reference RSs for PUSCH/SRS by RRC for MAC-CE based pathloss reference RS update - FFS: The maximum number of activated pathloss reference RS update for PUSCH/SRS/PUCCH [across CCs / within a slot across all CCs / per CC] - FFS: Number of measurement samples N to apply newly activated pathloss reference RS |
| Intel | No need for the components 2 and 3. The feature should be per UE and optional with capability signaling |

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16-1f | SCell beam failure recovery | - The maximum number of SCells for SCell beam failure recovery - FFS: Support of PUCCH-BFR - FFS: The maximum number of CSI-RS and/or SSB resources for new beam identification of SCell BFR [across all CCs / within a slot across all CCs / per CC] - FFS: Density of CSI-RS for new beam identification for SCell BFR | 2-31 |  | N/A |  | TBD | N |  |  |  | TBD |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LG Electronics | Fine with the FG but we believe that at least component 2 and 4 should be deleted. - Component 2: As explained in R1-2002161, component 2 should be removed considering further spec impacts. In addition, from UE perspective, PUCCH-BFR is same as normal PUCCH-SR so no additional complexity exists except for the prioritization between LLR and SR. UCI/channel prioritization rules always exist but without any UE capability defined so it does not make sense to create this UE capability. - Component 4: As 2-31 (Rel-15 BFR) does not define this component, this component should be removed in order to be consistent with Rel-15. Note that there is no difference between Rel-15 BFR and Rel-16 BFR with regard to new beam identification at the UE side, i.e. measuring L1-RSRP of the candidate RS resources and comparing that with the threshold. - Component 3: If component 1 of 16-1g includes all RSRP related measurements, this component may not be needed anymore but we are fine to discuss this further in RAN1#101-e. |
| ZTE | Support with the following modification for components. - The support of PUCCH-BFR shall be mandatory with capability signaling for FR2 and optional for FR1 (as we did for PCell-BFR), in order to guarantee the effectiveness/low-latency of SCell-BFR. - Regarding the last FFS, we only need to reuse Rel-15 UE capability. Proposed changes for components: - The maximum number of SCells for SCell beam failure recovery - FFS: Support of PUCCH-BFR - FFS: The maximum number of CSI-RS and/or SSB resources for new beam identification of SCell BFR [across all CCs / within a slot across all CCs / per CC] - FFS: Density of CSI-RS for new beam identification for SCell BFR |
| Intel | For feature 2, FFS can be removed; For feature 3, FFS can be removed, and the resource indication should be within a slot per CC. We do not see the need for feature 4. For this FG, FR1/FR2 can be decoupled and the FG should be Optional with capability signaling. |

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16-4 | Low PAPR DMRS for DL | Low PAPR DMRS for PDSCH | TBD |  | N/A |  | Per UE | N | N |  |  | [Optional] |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LG Electronics | We are fine with current FG. And, we think that this FG should be optional. |
| ZTE | Agree this FG. This feature should be optional with capability signaling. |
| Intel | FG should be optional with capability signaling |

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16-5b | UL full power transmission mode 1 | - Supported UL full power transmission mode 1 - Number of Tx to support mode 1: {2Tx, 4Tx, 2Tx_4Tx} - FFS: New UL codebook set(s) per supported Tx | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | N | N |  |  | TBD |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LG Electronics | Fine with the FG but we think that the new UL codebook set is clear for Mode 1 operation. Thus, component 3 should be removed. |
| ZTE | Support with the following modification for components. - It is because that we do not see a strong motivation for indicating the number of Tx ports to support mode1, which is not dynamically indicated like mode 2. Also we fail to see the technical motivation that a UE only can support some partial additional TPMIs (e.g., for 4Tx), but not support other additional TPMIs (e.g., for 2Tx). It does not make sense that the extension of codebook is only applied to a specific number of Tx. The last FFS seems to be redundant due to the same reason above. In summary, all the new UL codebook sets should be supported if mode 1 is supported. Proposed changes for components: - Supported UL full power transmission mode 1 - Number of Tx to support mode 1: {2Tx, 4Tx, 2Tx_4Tx} - FFS: New UL codebook set(s) per supported Tx |
| Intel | Component 2 is not necessary. In Rel-15, we already have the UE capability indicating the number of SRS antenna ports. Component 3 is redundant. There is no need to introduce UE capability on this for Mode 1 UE. |

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16-6b | Low PAPR DMRS for PUCCH | For PUCCH format 3 and/or PUCCH format 4 with transform precoding and with pi/2 BPSK modulation | TBD |  | N/A |  | FFS: Per band | N | N |  |  | FFS: Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LG Electronics | We are fine with current FG. And, we think that this FG should be optional. |
| ZTE | Agree this FG. This feature should be optional with capability signaling, per band. One minor update for the components description is: For PUCCH format 3 and/or PUCCH format 4 with transform precoding and with pi/2 BPSK modulation |
| Intel | Component should be revised to “For PUCCH format 3 and PUCCH format 4, if capable, with transform precoding and with pi/2 BPSK modulation” to ensure that there is no ambiguity about support of FG for both PF3 and PF4 when the UE is capable of supporting PF4. The FG should be “Per Band” and “Optional with capability signaling” |

…

# Conclusions

…

# References

- R1-2003072, LS on Rel-16 RAN1 UE features lists for NR, RAN1

- R1-2003073, RAN1 UE features list for Rel-16 NR after RAN1#100bis-E, AT&T/NTT DOCOMO, INC.

- R1-2003079, Chairman's Notes of AI 7.2.11.6, Ad-Hoc Chair (AT&T)

- R1-2003094, Summary of Email Approval [100b-e-NR-UEFeatures-eMIMO-01], Moderator (AT&T)

- R1-2003095, Summary of Email Approval [100b-e-NR-UEFeatures-eMIMO-02], Moderator (AT&T)

- R1-2003096, Summary of Email Approval [100b-e-NR-UEFeatures-eMIMO-03], Moderator (AT&T)

- R1-2003097, Summary of Email Approval [100b-e-NR-UEFeatures-eMIMO-04], Moderator (AT&T)

- R1-2003098, Summary of Email Approval [100b-e-NR-UEFeatures-eMIMO-05], Moderator (AT&T)

# Appendix

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16-1a-1 | SSB/CSI-RS for L1-SINR measurement | - The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-1 - The max number of NZP-IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR (including CMR and IMR) within a slot shall not exceed MB_1-3 - The max number of SSB/CSI-RS (1Tx)/CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR (including CMR and IMR) shall not exceed MC_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-1 - The max number of NZP IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MC_1-3 - The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_2 - Supported density of CSI-RS (CMR) - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 | 2-21, 2-22 or 2-23, 2-23a |  |  |  | Per band |  |  |  |  | Optional with capability signalling Component-1 to 4, candidate value set for MB_1 and MB_1-x is {0, 8, 16, 32, 64} Component-5 to 8, candidate value set for MC_1 and MC_1-x is {0, 4, 8, 16, 32, 64} Component-9, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-10: candidate value set: {"not supported", "1 only", "3 only", "both 1 and 3"} Component-11, candidate value set for MD_2 is {0, 1, 4, 8, 16, 32, 64} |
|  | 16-1a-2 | Non-group based L1-SINR reporting | - Support of non-group based L1-SINR reporting with N_max L1-SINR values reported | 16-1a-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling Candidate value set is {1, 2, 4} |
|  | 16-1a-3 | Group based L1-SINR reporting | - Support of group based L1-SINR reporting | 16-1a-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
|  | 16-1b | TCI state activation and spatial relation update | - Support of Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH - FFS: details on whether/how to indicate band pairs which can share the same DL TCI state | Component 1: 2-1, 2-4 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
|  | 16-1b-2 | Spatial relation update across multiple CCs | - Support of Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - FFS: details on whether/how to indicate band pairs which can share the same UL spatial relation info | Component 1: 2-59, 2-60 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
|  | 16-1b-3 | Spatial relation update for PUCCH group | - Support of PUCCH resource groups per BWP for simultaneous spatial relation update | 2-53, 2-59, 4-24 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
|  | 16-1c | Default spatial relation | Support of default spatial relation and pathloss reference RS for dedicated-PUCCH/SRS and PUSCH scheduled by DCI format 0_0 | 2-53, 2-59 |  | N/A |  | TBD [Per band] | N | Y |  |  | TBD |
|  | 16-1d | MAC CE spatial relation update for AP-SRS | [Support of / The maximum number of] spatial relation update for AP-SRS via MAC CE | 2-53, 2-59 |  | N/A |  | TBD [Per UE] | N | N |  |  | TBD |
|  | 16-1e | Pathloss reference RS activation via MAC CE | - The maximum number of configured pathloss reference RSs for PUSCH/SRS by RRC for MAC-CE based pathloss reference RS update - FFS: The maximum number of activated pathloss reference RS update for PUSCH/SRS/PUCCH [across CCs / within a slot across all CCs / per CC] - FFS: Number of measurement samples N to apply newly activated pathloss reference RS | 8-2, 8-3 |  | N/A |  | TBD [Per UE] | N | N |  |  | TBD |
|  | 16-1f | SCell beam failure recovery | - The maximum number of SCells for SCell beam failure recovery - FFS: Support of PUCCH-BFR - FFS: The maximum number of CSI-RS and/or SSB resources for new beam identification of SCell BFR [across all CCs / within a slot across all CCs / per CC] - FFS: Density of CSI-RS for new beam identification for SCell BFR | 2-31 |  | N/A |  | TBD | N |  |  |  | TBD |
|  | 16-1g | Resources for beam management, [pathloss measurement, BFD, and BFR] | - The maximum number of SSB/CSI-RS/[CSI-IM] resources [within a slot] across all CCs for any of L1-RSRP measurement, L1-SINR measurement, [pathloss measurement, BFD, and new beam identification] - FFS: The maximum number of SSB/CSI-RS resources within a slot across all CCs for pathloss measurement - FFS: The maximum number of SSB/CSI-RS resources within a slot across all CCs for BFD - FFS: The maximum number of SSB/CSI-RS resources across all CCs for new beam identification |  |  | N/A |  | TBD | N |  |  |  | TBD |
|  | 16-2a | Multi-DCI based multi-TRP | - The maximum number of CORESETs configured per “PDCCH-Config” - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” - The value of R=[1,2] for BD/CCE - [Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency] [for FR1] FFS whether default value of R is needed | TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
|  | 16-2a-0 | Overlapping PDSCHs in time and non-overlapping in frequency | - Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency [merge with 16-2a? Merge with 16-2a-8?] [FR2 only?] |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-1 | Simultaneous reception of Multiple PDSCHs overlapping in frequency | - Support fully overlapping PDSCHs at frequency and partially/fully overlapping at time - Support partially overlapping PDSCHs in frequency and partially/fully overlapping in time - The maximum number of MIMO layers of each scheduled PDSCH [if PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain] - The maximal number of PDSCH scrambling sequences per serving cell FFS whether default values of component 3/4 to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-2 | Out-of-order operation for DL | 1. Support out-of-order operation for PDCCH to PDSCH 2. Support out-of-order operation for PDSCH to HARQ-ACK |  |  |  |  |  |  |  |  | Note: this FG is a WA in RAN1 |  |
|  | 16-2a-3 | Out-of-order operation for UL | 1. Support out-of-order operation for PDCCH to PUSCH |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-4 | HARQ-ACK for multi-DCI based multi-TRP - separate | - Support of separate HARQ-ACK - [Support of two TDMed long PUCCHs in a slot] |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-4a | HARQ-ACK for multi-DCI based multi-TRP - joint | - Support of joint HARQ-ACK |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-5 | Separate CRS rate matching | Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-6 | Default QCL enhancement for multi-DCI based multi-TRP | Support of default QCL assumption per CORESETPoolIndex |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-7 | Maximum number of activated TCI states | - Support of maximum per CORESETPoolIndex - Support of total maximum FFS whether default values to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-8 | Simultaneous reception with different Type-D | whether UE supports receiving time-overlapping [PDSCHs]/PDCCHs with different Type-D FFS whether default values to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-9 | [Simultaneous reception across CCs with Multi-DCI] | [The maximum number of CCs supporting multi-DCI based multi-TRP simultaneously] |  |  |  |  |  |  |  |  | Note: If the type of 16-2a is agreed to be FSPC or Fs this FG will be removed |  |
|  | 16-2a-10 | Value of BD factor | R=[(1,2] for BD/CCE |  |  |  |  |  |  |  |  |  |  |
|  | 16-2b-0 | Two default beams for single-DCI based multi-TRP | Support of default QCL assumption with two TCI states | 16-2b |  | N/A |  | Per band | TDD only | FR2 only |  |  | TBD |
|  | 16-2b-1 | Single-DCI based SDM scheme | - FFS: Support of DCI indication of of 2 TCI states by a codepoint and DMRS ports within two CDM groups - Support of two PTRS ports - FFS Support of DMRS entry {0, 2, 3} | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  |  | TBD |
|  | 16-2b-2 | Single-DCI based FDMSchemeA | Support of FDMSchemeA | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  |  | TBD |
|  | 16-2b-3 | Single-DCI based FDMSchemeB | - Support of FDMSchemeB - For FDMSchemeB, whether the UE can support CW soft combining | 16-2b, TBD |  | N/A |  | [per FSPC] | N | TBD |  |  | TBD |
|  | 16-2b-4 | Single-DCI based TDMSchemeA | - Support of TDMSchemeA - Supported maximum TBS size for TDMSchemeA | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  | Component 2 candidate values {10 CBs, TBD} | TBD |
|  | 16-2b-5 | Single-DCI based inter-slot TDM | - Support of MAC CE to activate two TCI states for a TCI codepoint - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size according to RepNumR16 in PDSCH-TimeDomainResourceAllocation - FFS: TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping) | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  | Component 1 candidate values: {8,16} Component 2 candidate values {10 CBs, TBD} | TBD |
|  | 16-3a | Regular eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=1 - Support of parameter combinations 1-6 - Support of rank 1,2 - FFS: CBSR with hard amplitude restriction | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3a-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=2 |  |  |  |  |  |  |  |  |  |  |
|  | 16-3a-2 | Support of parameter combinations 7-8 | Support of parameter combinations 7-8 |  |  |  |  |  |  |  |  |  |  |
|  | 16-3a-3 | Support of rank 3,4 | Support of rank 3,4 |  |  |  |  |  |  |  |  |  |  |
|  | 16-3a-4 | CBSR | ALT 1) CBSR with soft amplitude restriction (capture consequence if not supported  hard amplitude restriction is supported) ALT 2) CBSR |  |  |  |  |  |  |  |  |  |  |
|  | 16-3b | Port selection eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=1 - 6 parameter combinations (combos with L=6 don’t apply) - Support of rank 1,2 | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3b-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=2 |  |  |  |  |  |  |  |  |  |  |
|  | 16-3b-2 | Support of rank 3,4 | Support of rank 3,4 |  |  |  |  |  |  |  |  |  |  |
|  | 16-4 | Low PAPR DMRS for DL | Low PAPR DMRS for PDSCH | TBD |  | N/A |  | Per UE | N | N |  |  | [Optional] |
|  | 16-5a | UL full power transmission [mode 0] | - Supported UL full power transmission [mode 0] | 2-13, 2-14 | Y | N/A | Y | FFS: Per FS or Per band or Per band per BC | N/A | N/A |  |  | TBD |
|  | 16-5b | UL full power transmission mode 1 | - Supported UL full power transmission mode 1 - Number of Tx to support mode 1: {2Tx, 4Tx, 2Tx_4Tx} - FFS: New UL codebook set(s) per supported Tx | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | N | N |  |  | TBD |
|  | 16-5c | [UL full power transmission mode 2] | - [Supported UL full power transmission mode 2 - Number of Tx to support mode 2: {2Tx, 4Tx, 2Tx_4Tx} - The maximum number of SRS resources in set with different number of ports [for usage set to ‘codebook’]. FFS on details for supported number of Tx. - FFS: Number of ports per SRS resource - FFS: Maximum number of different spatial relation info for all SRS resources for usage set to ‘codebook’ in a resource set - TPMI group which delivers full power. FFS on details for supported number of Tx. Note: UE indicating mode 2 shall support full power transmission for 1 antenna port] | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | N | N |  |  | TBD |
|  | 16-6a | Low PAPR DMRS for PUSCH without transform precoding | - For PUSCH without transform precoding | TBD | Y | N/A | Y | FFS: Per band | N/A | N/A |  |  | FFS: Optional with capability signalling |
|  | 16-6b | Low PAPR DMRS for PUCCH | For PUCCH format 3 and/or PUCCH format 4 with transform precoding and with pi/2 BPSK modulation | TBD |  | N/A |  | FFS: Per band | N | N |  |  | FFS: Optional with capability signalling |
|  | 16-6c | Low PAPR DMRS for PUSCH with transform precoding and with pi/2 BPSK | For PUSCH with transform precoding and with pi/2 BPSK modulation | TBD | Y | N/A | Y | FFS: Per band | N/A | N/A |  |  | FFS: Optional with capability signalling |
|  | 16-7 | Extension of the maximum number of configured aperiodic CSI report settings | Extension of the maximum number of configured aperiodic CSI report settings for all codebook types |  |  |  |  | [Per band] |  |  |  |  |  |
|  | 16-8 | Active CSI-RS resources and ports for mixed codebook types in any slot | FFS: the detailed component design |  |  |  |  |  |  |  |  |  |  |