3GPP TSG RAN WG1 #100bis			                                   					       												        R1-2001869

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.11.7

Source:	Moderator (AT&T)

Title:	Summary on UE features for UE power savings

Document for:	Discussion/Decision

# Introduction

This document was drafted by the moderator of the agenda item under the direction of the RAN1 Chairman following the below guidance whose purpose it serves:

- April 13-17: preparation phase - April 13th – 14th: FLs to prepare summary - April 15th – 17th: FLs to lead the discussion identifying the set of email threads - Note: PLEASE KEEP THE EMAIL DISCUSSION SCOPE PER EMAIL THREAD REASONABLE! - Too much scope will force Chairman/Vice Chairman to step in to do the necessary cut down using the best judgement à if so, no complain please.

All Sections except Section 3 were exclusively prepared by the moderator of the agenda item. Specifically, Section 2 is the moderator’s summary of contributions submitted to RAN1 #100bis-e in this agenda item according to the Chairman’s guidance. During the preparation phase, companies were given the opportunity to revise their views in the moderator’s summary in Section 2 using revision marks as shown below, if any. Section 3 was jointly drafted by the moderator and contributing companies during the preparation phase of RAN1 #100bis-e whereby companies present their views on the moderator’s proposals according to the Chairman’s guidance above in the respective tables. After conclusion of the preparation phase, the moderator submitted the final document as input to RAN1 #100bis-e with recommendations captured in Section 4.

The following will be removed from the final document, however, in the meantime, please take note of this guidance of the RAN1 MCC technical officer:

W.r.t the naming convention, the following suggestion […] may be helpful to keep the previous company’s name (only the most recent one) in the filename, so that we can easily tell which previous version this is based on, and may solve the issue when there are crossing emails. e.g. something like the following: 5_Incoming_Liaison_Statements/Summary-1_v1-LG 5_Incoming_Liaison_Statements/Summary-1_v2-LG-CATT 5_Incoming_Liaison_Statements/Summary-1_v2-LG-vivo 5_Incoming_Liaison_Statements/Summary-1_v3-CATT-HWHiSi

# Summary on UE features for UE power savings

The following table represents the version of the NR UE feature list for UE power savings agreed by RAN1 as baseline for RAN1 #100bis-e Error: Reference source not found.

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19.UE Power Saving | 19-1 | DRX Adaptation | - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDuration timer at the DRX ON by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON |  |  |  |  | N/A |  | Per UE | No | No |  | The minimum time gap between the end of the slot of last DCI format 3_0 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap | Optional with capability signalling |
|  | 19-2 | Cross Slot Scheduling | - Dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1 minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset - minimumSchedulingOffset K2 configuration for PUSCH |  |  |  |  | N/A |  | Per UE | No | No |  |  | Optional with capability signalling |
|  | 19-3 | Maximum MIMO Layer Adaptation | - Support of maximum number of MIMO layer configuration per DL BWP |  |  |  |  | N/A |  | [Per BC] | No | No |  | This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication | Optional with capability signalling |
|  | 19-4a | UE assistance information | - Preferred k0/k2 [for same carrier scheduling] - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots |  |  |  |  | N/A |  | Per UE | No | No |  | The minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted | Optional with capability signalling |

The following table is the moderator’s summary of contributions submitted to RAN1 #100bis-e in this agenda item.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE [2] | 19-1: 19-1; DRX Adaptation; - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDuration timer for the next long DRX cycle by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI measurement and report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start for the next long DRX cycle - Configured L1-RSRP measurement and report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start for the next long DRX cycle; N/A; Per UE; YES; No; The minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and beginning of the slot where the UE would start the drx-onDurationTimer is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap; Optional with capability signalling. ZTE notes that DRX ON denotes a time duration, and the wording of “drx_OnDurationTimer does not start at the DRX ON” is confusing. ZTE further argues, the periodic CSI and L1-RSRP measurement are not impacted by DCI format 2_6 as well. They propose to update the components of feature group 19-1 as follows: (1) Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration (2) Indication of UE whether or not to start drx_OnDuration timer for the next long DRX cycle by detection of DCI format 2_6 (3) Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time (4) Configured P-CSI measurement and report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start for the next long DRX cycle (5) Configured L1-RSRP measurement and report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start for the next long DRX cycle ZTE notes the DRX feature needs FDD/TDD differentiation in 38.822. Since the feature group 19-1 is based on DRX configuration, it also needs to be differentiated between TDD and FDD 19-3: 19-3; Maximum MIMO Layer Adaptation; - Support of maximum number of MIMO layer configuration per DL BWP; N/A; Per UE; No; No; This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication; Optional with capability signalling. This feature group indicates UE whether or not supports per DL BWP maximum number of MIMO layer configuration. ZTE believes it is enough to have the type of this feature group Per UE, instead of Per Band Combination. 19-4: 19-4a; UE assistance information; 1)Support of reporting preferred minimum K0/K2 via UE assistance information; N/A; Per UE; No; No; This capability is indicated only if UE supports the UE capability of dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB. The minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted The minimum applicable value candidates of K0/K2 are {1, 2, 4, 6} for BWP with 15kHz/ 30kHz SCS and {2, 4, 8, 12} for BWP with 60kHz/ 120kHz SCS.; Optional with capability signalling. According to the agreements in RAN2 #108 meeting, minimum K0/K2 value is signalled as UE assistance. To better reflect the agreements, ZTE suggests to update the component of feature group 19-4a with “Support of reporting preferred minimum K0/K2 via UE assistance information” ZTE notes there are two UE feature groups related to K0 in 38.822. If UE does not support these two feature groups, ZTE believes it make no sense for UE to report the preferred minimum K0/K2 value. ZTE thus proposes that the preferred minimum K0/K2 is reported only if UE supports dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB |
| OPPO [3] | OPPO emphasizes that for the description of each component in a feature group, they should be more informative to state what the capability is about from UE perspective should be in a more uniform manner to let people easy to understand. OPPO recommends to put more directive and aligned description of each component of feature groups. 19-1: 19-1; DRX Adaptation; - Detection of DCI format 2_6 with CRC scrambling by PS-RNTI after PS-offset before DRX ON and before minimum time gap before DRX ON duration - Indication of UE can start drx_OnDuration timer or not at the DRX ON by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON; N/A; Per UE; No; No; The minimum time gap between the end of the slot of last DCI format 3_0 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap; Optional with capability signalling. OPPO prefers that the component (1) should be the (capability for) “detect format 2-6 during the PS-offset and before the minimum gap” 19-2: 19-2; Cross Slot Scheduling; - UE detection of Dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1 - Configured minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset - Configured minimumSchedulingOffset K2 configuration for PUSCH; N/A; Per UE; No; No; Optional with capability signalling. OPPO believes for cross-slot scheduling, a further feature group should be considered. In the last summary, the capability is only about if the UE can be configured with minimum ks values. However, in OPPO’s view it is unclear the purpose for the indication of cross-slot, although it is well known for power saving. Specifically, OPPO argues for cross-slot scheduling, there are 2 motivations for introducing the explicit description. For the UE capability, OPPO continues it would be more informative to indicate that by the minimum k indication, the UE can go to more power efficient state. Also, according to OPPO it was well discussed in Power saving study that UE can also save power if configured only TDRA tables with all larger values, even without a minimum k been configured. OPPO recommends to introduce 19-2a to indicate that UE can save power by assuming all possible data scheduling has k0/k2 > 0 New 19-x anxd 19-y: As the motivation of SCell dormancy is basically for power saving, OPPO recommends to include18-4 and 18-4a as one of feature group belong to 19 |
| MediaTek Inc. [4] | 19-1: 19-1; DRX Adaptation; - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDurationTimer for the next long DRX cycle by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON; N/A; Per UE; No; No; The minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap; Optional with capability signalling. To clearly describe the function of wakeup indication provided by DCI format 2_6, MediaTek suggests to revise Component 2 as follows: Component 2: Indication of UE whether or not to start drx_OnDurationTtimer at for the next long DRX cycleON by detection of DCI format 2_6 19-3: 19-3; Maximum MIMO Layer Adaptation; - Support of maximum number of MIMO layer configuration per DL BWP; N/A; Per UE; No; Yes; This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication; Optional with capability signalling. MediaTek doesn’t see the need to have ‘per BC’ capability report, and considers ‘per UE’ sufficient. In addition, they propose to have FR1/FR2 differentiation to allow UE flexibility. They argue since the UE implementations on RF modules and the UE power consumptions are very different in FR1 and FR2, it is beneficial that the capability report can be different in FR1/FR2 |
| Intel Corporation [5] | 19-1: Intel believes DRX adaptation feature 19-1 for supporting WUS functionality should be per CC group or per BC, not per UE. They argue that at least for MR-DC, separate configuration is possible in MCG and SCG. In Intel’s view, in component 3 “Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time ” “at all monitoring occasions” is not needed since not detected outside active time includes the case DCI format 2_6 in not detected in any monitoring occasions before DRX ON |
| CATT [6] | 19-1: CATT proposes the DRX adaptation feature is per UE 19-2: CATT proposes the cross-slot scheduling feature is per UE 19-3: CATT proposes the maximum MIMO layer adaptation should be per band combination 19-4: CATT proposes the UE assistance information is per UE |
| Ericsson [7] | 19-1: Ericsson notes a minor typo in note that should be fixed (3_0 -> 2_6) Ericsson’s preference is to have per-UE indication, while allowing indicating differentiation between FRx/xDD 19-2: To reduce capability signalling overhead, Ericsson prefers to avoid having cross-slot scheduling support indicated at feature set level, since the offset indicator in the DCI is purely a baseband function, and there is no need to have very granular support indication, and UE can anyways choose to not take advantage of micro-sleep opportunities provided by the bit in the DCI. Ericsson’s preference is to have per-UE indication, while allowing indicating differentiation between FRx/xDD 19-3 : Ericsson prefers that the capability is indicated per UE and not per BC. They argue the RRC parameter for per-BWP max-MIMO layers indicates the PDSCH scheduling restriction that UE can expect for that BWP, a form of NW assistance for the UE to save power. In their view it is then up to the UE implementation how to use it for power savings and signaling per UE is enough, whereas signaling per BC would also lead to a larger signaling overhead. Ericsson thinks differentiation based on frequency range/xDD is OK 19-4a: UE preferred K0/K2 for Cross-carrier scheduling is FFS in RAN1 - Ericsson thinks this should be reflected in the feature list |
| Nokia, Nokia Shanghai Bell [8] | 19-3: Nokia, NSB argues since the capability 19-3 informs whether UE supports the new BWP specific parameter, maxMIMO-Layers-r16, and does not require other UE specific behaviour e.g. in terms of TBSLBRM determination, there does not seem to be any band specific dependency. They propose the type definition should be per UE. |
| Qualcomm Incorporated [9] | FG 19-1: (1) Configured ps-Offset for the detection oflocating monitoring occasions of DCI format 2_6 with CRC scrambled by PS-RNTI and minimum time gap before DRX ON durationprior to the slot where drx-onDurationTimer would start (4) Configured P-CSI report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON during the time duration indicated by drx-onDurationTimer outside Active Time (5) Configured periodic L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON during the time duration indicated by drx-onDurationTimer outside Active Time |
| Huawei, HiSilicon [10] | 19-1: For component (1), Huawei, HiSilicon notes it is not mentioned that minimum time gap is reported by the UE and propose to add ‘reported’ before the minimum time gap For the Note column, Huawei, HiSilcon propose to correct ‘DCI format 3_0’ to ‘DCI format 2_6’ Huawei, HiSilicon proposes to add a new component: “When the UE does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next DRX cycle, the UE shall start the drx-onDurationTimer for the next DRX cycle. ” 19-2: Huawei, HiSilicon proposes to add a new component: ”Joint indication of the scheduling offset restriction for minimumSchedulingOffset K0 and minimumSchedulingOffset K2 by DCI format 0_1 and 1_1.” |
| Vivo [11] | 19-1: Vivo argues the UE capability of minimum gap for each band can be different considering different realization and it can be caused by different RF paths to different bands. To solve this issue, Vivo thinks the UE reported capability for (1) of 19-1 shall be per band. For the other components, Vivo thinks it can be per UE 19-3: Vivo argues this feature is designed to save UE power by reducing baseband processing, reducing RF path and other implementation oriented approach. Furthermore, they note maximum MIMO layer configuration is band-specific configuration. considering potential different RF implementations across different bands, Vivo proposes to confirm 19-3 is based on the granularity of per-band |

# Issues for discussion during the preparation phase

## Issues for discussion during the preparation phase not captured via a table

### Include 18-4 and 18-4a as feature groups belonging to UE power savings

Possible Proposal: Include 18-4 and 18-4a as feature groups belonging to UE power savings

- Note: SCell dormancy is for power saving

| Company | Comments/Questions/Suggestions |
| --- | --- |

## Issues for discussion during the preparation phase captured via a table

The following table summarizes all proposals for FG 19-1 from Section 2 as revisions on top of the agreed baseline in [1]. Companies provide their inputs in the second table below in order to allow the moderator to make a recommendation on which of the proposed changes should be discussed during RAN1 #100bis-e.

| 19-1 | DRX Adaptation | - Configured PS_offset for the detection of locating monitoring occasions of DCI format 2_6 with CRC scrambling by PS-RNTI and reported minimum time gap before DRX ON duration prior to the slot where drx-onDurationTimer would start [FFS: replace component (1) with “Detection of DCI format 2_6 with CRC scrambling by PS-RNTI after PS-offset before DRX ON and before minimum time gap before DRX ON duration”] - Indication of UE [Alt. 1: “whether or not to can” or Alt. 2: “whether or not to”] start drx_OnDurationTtimer [Alt. 1: “or not at for the next long DRX cycle ON” or Alt. 2: “for the next long DRX cycle”] by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI measurement and report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON during the time duration indicated by drx-onDurationTimer outside Active Time for the next long DRX cycle - Configured periodic L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON during the time duration indicated by drx-onDurationTimer outside Active Time for the next long DRX cycle - When the UE does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next DRX cycle, the UE shall start the drx-onDurationTimer for the next DRX cycle |  |  | N/A |  | Per UE or Per BC or Per Band at least for component (1) | No or Yes | No or Yes |  | The minimum time gap between the end of the slot of last DCI format 3_0 2_6 monitoring occasion and the start of the DRX ON beginning of the slot where the UE would start the drx-onDurationTimer is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

The following table summarizes all proposals for FG 19-2 from Section 2 as revisions on top of the agreed baseline in [1]. Companies provide their inputs in the second table below in order to allow the moderator to make a recommendation on which of the proposed changes should be discussed during RAN1 #100bis-e.

| 19-2 | Cross Slot Scheduling | - UE detection of Dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1 - Configured minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset - Configured minimumSchedulingOffset K2 configuration for PUSCH - Joint indication of the scheduling offset restriction for minimumSchedulingOffset K0 and minimumSchedulingOffset K2 by DCI format 0_1 and 1_1 |  |  | N/A |  | Per UE | No or Yes | No or Yes |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

The following table summarizes all proposals for FG 19-3 from Section 2 as revisions on top of the agreed baseline in [1]. Companies provide their inputs in the second table below in order to allow the moderator to make a recommendation on which of the proposed changes should be discussed during RAN1 #100bis-e.

| 19-3 | Maximum MIMO Layer Adaptation | - Support of maximum number of MIMO layer configuration per DL BWP |  |  | N/A |  | [Per BC] or Per UE | No or Yes | No or Yes |  | This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

The following table summarizes all proposals for FG 19-4a from Section 2 as revisions on top of the agreed baseline in [1]. Companies provide their inputs in the second table below in order to allow the moderator to make a recommendation on which of the proposed changes should be discussed during RAN1 #100bis-e.

| 19-4a | FFS: UE assistance information | - Preferred k0/k2 [for same carrier scheduling] - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots [FFS: replace component (1) with “Support of reporting preferred minimum K0/K2 via UE assistance information”] |  |  | N/A |  | Per UE or Per Band | No | No |  | This capability is indicated only if UE supports the UE capability of dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB. The minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted The minimum applicable value candidates of K0/K2 are {1, 2, 4, 6} for BWP with 15kHz/ 30kHz SCS and {2, 4, 8, 12} for BWP with 60kHz/ 120kHz SCS. UE preferred K0/K2 for Cross-carrier scheduling is FFS in RAN1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Conclusion

[To be completed towards the end of the preparation phase]

# References

- R1-2001484, RAN1 UE features list for Rel-16 NR after RAN1#100-E, AT&T & NTT DOCOMO

- R1-2001587, Discussion on UE feature for UE  power saving, ZTE

- R1-2001772, Discussion on Power Saving related UE features, OPPO

- R1-2001830, Views on Rel-16 UE features for NR UE power savings, MediaTek Inc.

- R1-2002021, On UE features for Rel-16 UE Power Saving, Intel Corporation

- R1-2002072, Discussion of UE features for NR UE power savings, CATT

- R1-2002425, Discussion on UE features for UEPS, Ericsson

- R1-2002481, On UE features for UE Power Savings, Nokia & Nokia Shanghai Bell

- R1-2002568, Discussion on power saving UE features, Qualcomm Incorporated

- R1-2002593, Rel-16 UE features for UE power saving, Huawei & HiSilicon

- R1-2002625, Discussion on Rel-16 UE power saving UE features, vivo