3GPP TSG-RAN WG1 Meeting #100bis-e	R1-2002036

e-Meeting, 20th – 30th April, 2020

Agenda Item:	7.2.2.1.3

Source:	Moderator (Ericsson)

Title:	Feature lead summary for Maintenance of UL Signals and Channels

Document for:	Discussion, Decision

# 1	Introduction

This document contains a high level summary of the contributions made under the “UL Signals and Channels” sub-agenda item for 7.2.2 Rel-16 Maintenance of NR-based Access to Unlicensed Spectrum. According to the Chairman’s guidance, 3 email threads have been assigned to this agenda item. This summary contains a proposal for a grouping of issues into the 3 threads for discussion this meeting. To keep the scope reasonable for each thread, it is proposed to defer the other issues to the next meeting.

Once the group agrees on the topics to be discussed, updated summaries containing proposals and TPs will be circulated for each of the 3 threads.

# 2	Issues to Address This Meeting

This section contains a proposed grouping of issues for the 3 threads. The threads are grouped as follows:

- Thread A

  - 1 critical issue that will require some discussion

  - 1 editorial issue – easy to agree

- Thread B

  - 1 critical issue that will require some discussion

  - 1 editorial issue – easy to agree

- Thread C

  - 2 critical issues, although they are both simple and should be easy to agree

  - 1 editorial issue – easy to agree

## 2.1	Email Thread A

| Issue | Description | Tdoc References | Class |
| --- | --- | --- | --- |
| 1 | FDRA field for DCI 0_0 for UL resource allocation Type 2: - DCI 0_0 in a CSS: Agree on rule for RB set allocation for PUSCH - DCI 0_0 in a USS: Agree on whether or not FDRA field includes Y bits for RB set allocation + rule for RB set allocation for PUSCH (if Y bits not included) or value of Y (if Y bits included) TPs needed to 38.212 §7.3.1.1.1 and 38.214 §6.1.2.2.3 | R1-2002321: P1,P2 R1-2002030: P1,P2 R1-2001875: P1-P3 R1-2001533: P1 R1-2001934: P1-P4 R1-2001973: P2-P4 R1-2002433: P1 R1-2001758: P1 R1-2002116: P1 R1-2002382: P1-P3 R1-2002276: P1-P2 R1-2001704: P1-P2 R1-2001651: P1-P2 | Critical |
| 2 | Clarify that minimum number of resource blocks within an interlace contained in a BWP is 10 (Interlaced transmission not supported for 10 MHz SCell) Simple TP needed to 38.211 §4.4.4.6 | R1-2002030: P6 R1-2001533: P2 R1-2001986: §2.2 | Editorial |

For Issue 1 above, companies that did not provide their preferred alternative in their contributions are invited to provide their preferred alternative in the tables below. This is useful input to next week’s discussion.

Issue 1-1: Alternatives for RB set allocation for PUSCH scheduled by DCI 0_0 in CSS:

- Alt-1: PUSCH allocated to the RB set of the active UL BWP that intersects the RB set of the active DL BWP in which DCI 0_0 is received

- Alt-2: PUSCH allocated to RB set 0 of the active UL BWP

- Alt-3: PUSCH allocated to all RB sets of the active UL BWP

- Alt-4a/b: PUSCH allocated to RB set(s) according to the following logic:

  - Alt-4a (ref: [4]):

    - If the active UL BWP does not include all of the RBs of the initial UL BWP or the active UL BWP has different SCS than the initial UL BWP, then

      - RB set 0 of the active UL BWP

    - Otherwise

      - RB set of the initial UL BWP

  - Alt-4b (ref: [18]):

    - If the active UL BWP includes all of the RBs of the initial UL BWP and the SCS/CP of the active UL BWP is the same as that of the initial UL BWP or the initial UL BWP is active

      - the initial UL BWP

    - Otherwise

      - All RB sets of the active UL BWP

| Company | View/Position |
| --- | --- |
| Apple | Alt-1 |
| Ericsson | Alt-1 |
| Fujitsu | Alt-1 |
| LGE | Alt-1 |
| DOCOMO | Alt-1 |
| OPPO | Alt-4a |
| Samsung | Alt-3 |
| Sharp | Alt-4b |
| Spreadtrum | Alt-2 |
| ZTE | Alt-1 |
| vivo | Alt-2 |
| Lenovo | Alt-2 |
| Qualcomm | Alt-2 |
| Nokia, NSB | Alt-1 |

Issue 1-2: Alternatives for FDRA field of DCI 0_0 in a USS:

- Alt-1: FDRA field of DCI 0_1 in a USS contains X bits only

  - Alt-1a: PUSCH allocated to the RB set of the active UL BWP that intersects the RB set of the active DL BWP in which DCI 0_0 is received

  - Alt-1b: PUSCH allocated to RB set 0 of the active UL BWP

- Alt-2: FDRA field of DCI 0_1 in a USS contains X + Y bits

  - Alt-2a: Y is variable and given by size of active UL BWP

  - Alt-2b: Y is fixed at [4] bits

| Company | View/Position |
| --- | --- |
| Apple | Alt-2a |
| Ericsson | Alt-1a |
| Fujitsu | Alt-1a |
| Huawei | Alt-2 |
| LGE | FFS between Alt-1a and Alt-2a |
| Lenovo | Alt-1b |
| DOCOMO | Alt-1a |
| OPPO | Alt-2 |
| Samsung | Alt-2b |
| Sharp | Alt-2 |
| Spreadtrum | Alt-1b |
| ZTE | Alt-2 |
| vivo | Alt-2a |
| Qualcomm | Alt-1b |
| Nokia, NSB | Alt-1a |

Company views on grouping of Email Thread A:

| Company | View/Position |
| --- | --- |
| Sharp | For issue 1-1, Sharp’s view [18] in Alt-4b is to schedule PUSCH in the initial UL BWP. Our proposal is not the RB-set in the initial UL BWP. That is, RB-set based PUSCH scheduling is not used for DCI format 0_0 monitored in CSS since there may be a case where the active UL BWP (e.g., the initial UL BWP = 20 MHz) includes only a subset of one RB-set corresponding to the UL carrier (e.g. 80 MHz) if wideband operation AI (7.2.2.2.5) agrees one RB-set corresponding to the carrier bandwidth for a case of “no intra-cell guard band” configuration. |
| Lenovo, Motorola Mobility | - For both Issue 1-1 and Issue 1-2, we prefer a unified solution, i.e., only X bits in DCI format 0-0 in either CSS or USS. - We are not pretty sure about the meaning of “intersect”, does it mean the DL BWP should have overlap (partially or fully) in frequency domain with UL BWP? |
| LGE | Regarding to issue 1-1, the reference BWP to determine the size of X bit in FDRA field of DCI format 0_0 needs to be clarified as below. - For DCI format 0_0 transmitted in CSS, X bit size of FDRA field in the DCI format 0_0 is determined based on the SCS of the initial UL BWP as in legacy Rel-15. - For DCI format 0_0 transmitted in USS, X bit size of FDRA field in the DCI format 0_0 is determined based on the SCS of the active UL BWP as in legacy Rel-15. |

## 2.2	Email Thread B

| Issue | Description | Tdoc References | Class |
| --- | --- | --- | --- |
| 3 | FDRA field in RAR UL grant for operation with shared spectrum channel access - Capture UE interpreation of FDRA field when interlacing configured - Truncation/expansion rule when interlacing not configured - Agree on rule for RB set allocation for PUSCH scheduled by RAR UL grant TP needed to 38.213 §8.3 | R1-2002030: P3,P4 R1-2001875: P4 R1-2001533: P3 R1-2001934: P5 R1-2001758: P1 R1-2002382: P4-P5 R1-2001651: P3 R1-2002383: P2 R1-2001706: P8 | Critical |
| 4 | Align procedure text in 38.213 §6.2.1 with SRS resource configuration in 38.331 capturing that SRS resource can start at any OFDM symbol in a slot. TP needed to 38.214 §6.2.1 | R1-2002075: P1 R1-2002030: P5 R1-2001986: §2.1 R1-2002365: TP1 R1-2002529: P4 R1-2001704: P4 | Editorial |

Company views on grouping of Email Thread B:

| Company | View/Position |
| --- | --- |
| Sharp | For issue 3, we provided FDRA interpretation for operation with shared spectrum channel access, with a TP. Would you allow me to add reference for it? |
| ZTE | For issue 3, our views are provided in the contribution submitted to IAP. I would like to add a reference for that. |

## 2.3	Email Thread C

| Issue | Description | Tdoc References | Class |
| --- | --- | --- | --- |
| 5 | Clarification on the conditions for using PUCCH Format 0,1,2,3,4 for UCI transmission Simple TP needed to 38.213 §9.2.2 | R1-2002192: I1 | Critical (simple) |
| 6 | Correction for multiplexing of coded UCI bits to PUCCH for PUCCH Format 3 considering spreading factor NSFPUCCH,3 TP needed to 38.212 §6.3.1.6 | R1-2001651: P4 | Critical (simple) |
| 7 | Alignment of RRC parameters between 38.331 and RAN1 specs. RAN2 has consolidated the number of parameters configuring interlaced transmission. The 2 new parameters are: - useInterlacePUCCH-PUSCH within the BWP-UplinkCommon IE - useInterlacePUCCH-PUSCH within the BWP-UplinkDedicated IE which replace the 4 old parameters: - useInterlacePUCCH-Common - useInterlacePUSCH-Common - useInterlacePUCCH-Dedicated - useInterlacePUSCH-Dedicated TPs needed to 38.211, 212, 213, 214 in multiple sections. | R1-2002030: P7 R1-2001704: P3 | Editorial |

Company views on grouping of Email Thread C:

| Company | View/Position |
| --- | --- |
| ZTE | We think Issue#5 is an editorial issue rather than critical. And at this moment we are not sure if the change is necessary. |

# 3	Issues to Address Next Meeting

## 3.1	PUSCH Related

| Issue | Description | Tdoc References | Class |
| --- | --- | --- | --- |
| 8 | Whether or not DCI 0_2 supports indication of interlace allocation | R1-2002382: P6 | Discussion |

## 3.2	PUCCH Related

| Issue | Description | Tdoc References | Class |
| --- | --- | --- | --- |
| 9 | Align description of PUCCH resource in 38.213 §9.2.1 with PUCCH-Resouce-r16 IE in 38.331. TP needed to 38.213 §9.2.1 | R1-2001903: P1 | Editorial |
| 10 | Rule for PUCCH allocation for a carrier without intra-cell guardbands (single RB set) when interlaced PUCCH is configured, e.g., lowest 10 RBs of the first, and if configured, second interlace TP needed to 38.213 §9.2.1 | R1-2002382: P8 | Discussion |

## 3.3	SRS Related

| Issue | Description | Tdoc References | Class |
| --- | --- | --- | --- |
| 11 | Support CP extension for SRS (in addition to PUCCH/PUSCH, at least for aperiodic SRS triggered by DCI 0_1/1_1 | R1-2002246: P1 R1-2002192: I2 | Discussion (in AI 7.2.2.2.1?) |

## 3.4	Interlace Configuration Related

| Issue | Description | Tdoc References | Class |
| --- | --- | --- | --- |
| 12 | For UL resource allocation Type 2, X bits provides the interlace allocation; however, 38.214 Section 6.1.2.2.3 does not explicitly state that X = 6/5 for 15/30 kHz SCS Simple TP needed to 38.214 §6.1.2.2.3 | R1-2001973: P1 | Editorial |

## 3.5	Other

| Issue | Description | Tdoc References | Class |
| --- | --- | --- | --- |
| 13 | Clarify that for operation with shared spectrum channel access, unpaired spectrum is always assumed, e.g., in 38.213 §12 | R1-2002246: P3 | Discussion |
| 14 | Discuss whether or not SFI that is indicated beyond the indicated end-of-COT is considered as valid for the purposes of CUL transmissions enabled by enableConfigureUL-r16. | R1-2001758: P3 | Discussion |

Company views on issues to address next meeting:

| Company | View/Position |
| --- | --- |
| Sharp | We are fine with the issue 10 to deprioritize at this meeting. On the other hand, I think this issue is “Discussion” rather than “Editorial”. This is not just a correction based on the previous agreement. |

# References

- R1-2001533	Maintainance on uplink signals and channels	Huawei, HiSilicon

- R1-2001651	Remaining issues on physical UL channel design in unlicensed spectrum	vivo

- R1-2001704	Remaining issues on the UL channels for NR-U	ZTE, Sanechips

- R1-2001758	Discussion on the remaining issues of UL signals and channels	OPPO

- R1-2001875	Remaining issues on UL signals and channels for NR-U	Fujitsu

- R1-2001903	Remaining issues on UL signals and channels for NR-U	MediaTek Inc.

- R1-2001934	Remaining issues of UL signals and channels for NR-U	LG Electronics

- R1-2001973	Remaining issues for UL signals and channels for NR-U	Lenovo, Motorola Mobility

- R1-2001986	UL signals and channels for NR-unlicensed	Intel Corporation

- R1-2002030	UL signals and channels	Ericsson

- R1-2002075	TP for SRS configuration	CATT

- R1-2002116	UL signals and channels for NR-U	Samsung

- R1-2002192	Remaining Issues on UL Signals and Channels for NR-U	Nokia, Nokia Shanghai Bell

- R1-2002246	UL signals and channels	ETRI

- R1-2002276	Remaining issues in UL signals and channels	Spreadtrum Communications

- R1-2002321	Remaining issues of UL signals and channels	Apple

- R1-2002365	TPs on uplink signals in NRU	NEC

- R1-2002382	Remaining issues on UL signals/channels for NR-U	Sharp

- R1-2002433	Remaining issues on UL signals and channels for NR-U	NTT DOCOMO, INC.

- R1-2002529	TP for UL signals and channels for NR-U	Qualcomm Incorporated