3GPP TSG RAN WG1 #100bis	R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Source:	moderator (vivo)

Title:	Feature lead summary NRU-CG-02

Agenda Item:	7.2.2.2.4

Document for:	Discussion and Decision

Introduction

Following email thread is assigned:

[100b-e-NR-unlic-NRU-CG-02] Email discussion/approval on following issues by 4/24; if necessary, followed by endorsing the corresponding TPs by 4/29 – Rakesh (Vivo)

- PUSCH repetition transmission related issues for NRU configured grant

- RV determination for CG repetition

Discussion on topic#2

Issue 2: The PUSCH repetition transmission related issues for NRU configured grant

### 2.2.1 TP1

-------------------------------------------------------------------------------------

===================TP for 38.214 6.1.2.3.1================

6.1.2.3.1	Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant

*** Unchanged text is omitted ***

For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when the UE is configured with repK > 1, the UE shall repeat the TB across the repK consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. For operation with shared spectrum channel access, where the UE is provided with higher layer parameters cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16  and repK>1, the UE shall perform the transmission of the first repetition in the earliest transmission occasion for which the related channel procedure described in 37.213 is successful. A Type 1 or Type 2 PUSCH transmission with a configured grant in a slot is omitted according to the conditions in Subclause 11.1 of [6, TS38.213].

*** Unchanged text is omitted ***

-----------------------------------------------------------------------------------

### 2.2.2 TP2

-----------------------------------------------------------------------------------

For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1, the UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots-r16 and or cg-nrofPUSCH-InSlot-r16, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration if the repK earliest consecutive transmission occasion candidates are within the same configuration period. If the repK earliest consecutive transmission occasion candidates are not within the same configuration period, the UE repeats the TB at least in the consecutive transmission occasion candidates within the same configuration period. A Type 1 or Type 2 PUSCH transmission with a configured grant in a slot is omitted according to the conditions in Subclause 11.1 of [6, TS38.213].

-----------------------------------------------------------------------------------

### 2.2.3 TP3

-----------------------------------------------------------------------------------

The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. For operation without shared spectrum channel access, Iif the parameter repK-RV is not provided in the configuredGrantConfig, the redundancy version for uplink transmissions with a configured grant shall be set to 0. Otherwise, for the nth transmission occasion among K repetitions, n=1, 2, …, K, it is associated with (mod(n-1,4)+1)th value in the configured RV sequence. If a configured grant configuration is configured with Configuredgrantconfig-StartingfromRV0 set to ‘off’, the initial transmission of a transport block may only start at the first transmission occasion of the K repetitions. Otherwise, the initial transmission of a transport block may start at

-	the first transmission occasion of the K repetitions if the configured RV sequence is {0,2,3,1},

-	any of the transmission occasions of the K repetitions that are associated with RV=0 if the configured RV sequence is {0,3,0,3},

-	any of the transmission occasions of the K repetitions if the configured RV sequence is {0,0,0,0}, except the last transmission occasion when K≥8.

For operation with shared spectrum channel access, the initial transmission of a transport block may start at any transmission occasion of the K repetitions with RV=0. If the parameter repK-RV is not provided in the configuredGrantConfig, the redundancy version for uplink transmission with a configured grant is determined by the UE. Otherwise, for the nth transmission among at most K repetitions, n=1, 2, …, K, it is associated with (mod(n-1,4)+1)th value in the configured RV sequence.

-----------------------------------------------------------------------------------

TP1, TP2, TP3 are discussing similar issues, thus they are discussed together. Please provide your views/comments in the table below

| Company/organization | comments |
| --- | --- |
| ZTE | We think TP1 is sufficient and we support it. We do not see the need of TP2. TP3 should be discussed together with Issue 12. And we prefer to adopt the TP in section 2.2 rather than this one. |

-----------------------------------------------------------------------------------

Proposal:

- UE terminates the repetitions if an explicit feedback indicating ACK in the DFI is received for the HARQ process not only for CG-PUSCH but also for DG-PUSCH.

-----------------------------------------------------------------------------------

Please provide your views/comments in the table below

| Company/organization | comments |
| --- | --- |
| ZTE | The proposal itself is not that clear. By reading the contribution [2435], we understanding the intention is to terminate the slot aggregation for DG-PUSCH if the DFI includes an ACK indication for that HARQ process. As DFI is introduced mainly for CG-PUSCH, We are not sure if this optimization for DG-PUSCH is necessary. |

Issue 12: RV determination for CG repetition (Editorial/clarification)

### 2.12.1 TP1

TP for 38.214

-----------------------------------------------------------------------------------------------------

6.1.2.3.1	Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant

The procedures described in this clause apply to PUSCH transmissions of PUSCH repetition Type A with a Type 1 or Type 2 configured grant.

The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If the parameter repK-RV is not provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, the redundancy version for uplink transmissions with a configured grant shall be set to 0. If cg-RetransmissionTimer is provided, the redundancy version for uplink transmission with a configured grant is determined by the UE. If the parameter repK-RV is provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided. Otherwise, for the nth transmission occasion among K repetitions, n=1, 2, …, K, it is associated with (mod(n-1,4)+1)th value in the configured RV sequence. If a configured grant configuration is configured with Configuredgrantconfig-StartingfromRV0 set to 'off', the initial transmission of a transport block may only start at the first transmission occasion of the K repetitions. Otherwise, the initial transmission of a transport block may start at

-----------------------------------------------------------------------------------------------------

Please provide your views/comments in the table below

| Company/organization | comments |
| --- | --- |
| ZTE | We think the three TPs are with the same meaning, and we are fine to adopt either of them. For TP1, there is an extra full stop which should be removed, i.e. “is not provided.” |

### 2.12.2 TP2

-----------------------------------------------------------------------------------------------------

The procedures described in this clause apply to PUSCH transmissions of PUSCH repetition Type A with a Type 1 or Type 2 configured grant.

If cg-RetransmissionTimer is provided, the redundancy version for uplink transmission with a configured grant is determined by the UE.

The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If the parameter repK-RV is not provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, the redundancy version for uplink transmissions with a configured grant shall be set to 0. Otherwise If the parameter repK-RV is provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, for the nth transmission occasion among K repetitions, n=1, 2, …, K, it is associated with (mod(n-1,4)+1)th value in the configured RV sequence. If a configured grant configuration is configured with Configuredgrantconfig-StartingfromRV0 set to ‘off’, the initial transmission of a transport block may only start at the first transmission occasion of the K repetitions. Otherwise, the initial transmission of a transport block may start at

-----------------------------------------------------------------------------------------------------

Please provide your views/comments in the table below

| Company/organization | comments |
| --- | --- |
| ZTE | We think the three TPs are with the same meaning, and we are fine to adopt either of them. |

### 2.12.3 TP3

-------------------------------------------------------------------------------------------------------

The procedures described in this clause apply to PUSCH transmissions of PUSCH repetition Type A with a Type 1 or Type 2 configured grant.

If cg-RetransmissionTimer is provided, the redundancy version for uplink transmission with a configured grant is determined by the UE. Otherwise, the higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions when the cg-RetransmissionTimer is not configured, for the nth transmission occasion among K repetitions, n=1, 2, …, K, it is associated with (mod(n-1,4)+1)th value in the configured RV sequence. If the parameter repK-RV is not provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, the redundancy version for uplink transmissions with a configured grant shall be set to 0. If a configured grant configuration is configured with Configuredgrantconfig-StartingfromRV0 set to 'off', the initial transmission of a transport block may only start at the first transmission occasion of the K repetitions. Otherwise, the initial transmission of a transport block may start at

----------------------------------------------------------------------------------------------------------

Please provide your views/comments in the table below

| Company/organization | comments |
| --- | --- |
| ZTE | We think the three TPs are with the same meaning, and we are fine to adopt either of them. |

# References

[1] R1-2002737, Feature lead summary#2 on NRU configured grant enhancement, RAN1#100b-e

[2] R1-2002745, Summary of prep email discussion on NRU-CG, RAN1#100b-e