3GPP TSG RAN WG1 #100bis	R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Source:	moderator (vivo)

Title:	Feature lead summary on ULFPTx-01

Agenda Item:	7.2.6.4

Document for:	Discussion and Decision

Introduction

Following email thread is assigned for discussion:

[100b-e-NR-eMIMO-ULFPTx-01] Email discussion on Issue #2 in R1-2002746: For 4Tx and partial-coherent UE in Mode 2, when the SRS resource set is configured with 2 ports SRS and 4 ports SRS, and the codebook subset associated with 2 port SRS resource. By 4/24 and corresponding TP (if any) by 4/30 – Rakesh (vivo)

Discussion on issue 2[1]

Issue 2: For 4Tx and partial-coherent UE in Mode 2, when the SRS resource set is configured with 2 ports SRS and 4 ports SRS, and the codebook subset associated with 2 port SRS resource:

- Alt1: introduce a new UE capability, UE indicates whether it is fullyAndPartialAndNonCoherent or nonCoherent

- Alt2: the codebook subset is nonCoherent

- Alt3: the codebook subset is fullyAndPartialAndNonCoherent

Please provide your views/comments in the table below

| Company/organization | comments |
| --- | --- |

Observation:

Proposal:

References

[1] R1-2002746, Summary of prep email discussion on ULFPTx, RAN1#100b-e