3GPP TSG RAN WG1 Meeting #101                                          	                    R1-2005101

e-Meeting, May 25th – June 5th, 2020

Source:	CATT

Title:	              Test proposal from email discussion thread #03 for AI 7.2.4.3 sidelink synchronization mechanism

Agenda Item:	7.2.4.3

Document for:	Discussion and Decision

- Introduction

[101-e--NR-5G_V2X_NRSL-SYNC-03]

Email discussion regarding

- Issue 12: Collision between S-SSB and DL slots

- Issue 14: (Pre-)configuration of SL BWP

- Issue 18: S-SSB RSRP measurement

Till 5/28. If consensus can be reached, any follow-up TP by 6/3 – Teng (CATT)

- TP 1

----------------------------------------------------Start of draft TP for 38.213----------------------------------------------------

16.1   Synchronization procedures

For reception of a S-SS/PSBCH block, a UE assumes a frequency location corresponding to the subcarrier with index 66 in the S-SS/PSBCH block [4, TS 38.211], is provided by absoluteFrequencySSB-SL. The UE assumes that a S-PSS symbol, a S-SSS symbol, and a PSBCH symbol have a same transmission power. The UE assumes a same numerology of the S-SS/PSBCH as for a SL BWP of the S-SS/PSBCH block reception, and that a bandwidth of the S-SS/PSBCH is within a bandwidth of the SL BWP. The UE assumes the subcarrier with index 0 in the S-SS/PSBCH block is aligned with a subcarrier with index 0 in a RB of the SL BWP.

--------------------------------------------------< Unchanged parts are omitted >-------------------------------------------------

-----------------------------------------------------End of draft TP for 38.213-----------------------------------------------------

- TP 2

----------------------------------------------------Start of draft TP for 38.215----------------------------------------------------

5.1.22   PSBCH reference signal received power (PSBCH-RSRP)

| Definition | PSBCH Reference Signal Received Power (PSBCH-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry demodulation reference signals associated with physical sidelink broadcast channel (PSBCH). For PSBCH-RSRP sidelink secondary synchronization signals in addition to demodulation reference signals for PSBCH may be used. PSBCH-RSRP using sidelink secondary synchronization signals shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals. For frequency range 1, the reference point for the PSBCH RSRP shall be the antenna connector of the UE. For frequency range 2, PSBCH-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported PSBCH-RSRP value shall not be lower than the corresponding PSBCH-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED inter-frequency |

NOTE 1:   The number of resource elements within the considered measurement frequency bandwidth and within the measurement period that are used by the UE to determine PSBCH-RSRP is left up to the UE implementation with the limitation that corresponding measurement accuracy requirements have to be fulfilled.

NOTE 2:   The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP.

NOTE 3:   It is up to UE implementation to use PSBCH DMRS only or both S-SSS and PSBCH DMRS for PSBCH-RSRP.

-----------------------------------------------------End of draft TP for 38.215-----------------------------------------------------

- Appendix: Corresponding Agreements of Email Thread#03

Agreements:

- Adopt the following TP (38.213, Section 16.1)

<Unchanged parts omitted>

For reception of a S-SS/PSBCH block, a UE assumes a frequency location corresponding to the subcarrier with index 66 in the S-SS/PSBCH block [4, TS 38.211], is provided by absoluteFrequencySSB-SL. The UE assumes that a S-PSS symbol, a S-SSS symbol, and a PSBCH symbol have a same transmission power. The UE assumes a same numerology of the S-SS/PSBCH as for a SL BWP of the S-SS/PSBCH block reception, and that a bandwidth of the S-SS/PSBCH is within a bandwidth of the SL BWP. The UE assumes the subcarrier with index 0 in the S-SS/PSBCH block is aligned with a subcarrier with index 0 in a RB of the SL BWP.

<Unchanged parts omitted>

Agreements:

- PSBCH-DMRS is used for S-SSB RSRP measurement.

  - S-SSS may be used for S-SSB RSRP measurement additionally, in which case,  it is up to UE implementation to use PSBCH DMRS only or S-SSS + PSBCH DMRS.

Agreements:

- Adopt the following TP

----------------------------------------------------Start of draft TP for 38.215----------------------------------------------------

5.1.22   PSBCH reference signal received power (PSBCH-RSRP)

| Definition | PSBCH Reference Signal Received Power (PSBCH-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry demodulation reference signals associated with physical sidelink broadcast channel (PSBCH). For PSBCH-RSRP sidelink secondary synchronization signals in addition to demodulation reference signals for PSBCH may be used. PSBCH-RSRP using sidelink secondary synchronization signals shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals. For frequency range 1, the reference point for the PSBCH RSRP shall be the antenna connector of the UE. For frequency range 2, PSBCH-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported PSBCH-RSRP value shall not be lower than the corresponding PSBCH-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED inter-frequency |

NOTE 1:   The number of resource elements within the considered measurement frequency bandwidth and within the measurement period that are used by the UE to determine PSBCH-RSRP is left up to the UE implementation with the limitation that corresponding measurement accuracy requirements have to be fulfilled.

NOTE 2:   The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP.

NOTE 3:   It is up to UE implementation to use PSBCH DMRS only or both S-SSS and PSBCH DMRS for PSBCH-RSRP.

-----------------------------------------------------End of draft TP for 38.215-----------------------------------------------------