3GPP TSG RAN WG1 #93 			R1-180xxxx

Busan, Korea, 21st– 25th May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.202 | CR | DRAFT | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.202 capturing RAN1#92bis and RAN1#93 meeting agreements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-4 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clarification on some terminology and update based on some agreements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated text in subclause 6 to clarify PUCCH groups and cell groups Updated text in subclause 6.1 for new supported transmissions combinations Updated text in subclause 6.2 to reflect IDLE mode, and corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Subclause 6, 6.1, 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 6	Simultaneous transmission and reception of physical channels and physical signals

This clause describes the requirements from the UE to send and receive multiple physical channels and physical signals simultaneously depending on the capabilities and service requirements. The following notation is used between both the uplink and downlink subclauses below.

-	p is the number of uplink carriers configured for the UE on which physical channels can be transmitted

-	p' is the number of uplink carriers configured for the UE on which SRS can be transmitted

-	q is the number of downlink carriers configured for the UE

-	j is the number of NR cell groups configured for the UE. In cases where NR-NR DC is not configured, j=1, while for cases where NR-NR DC is configured, j=2.

-	k is the number of PUCCH groups configured for the UE. In cases where NR-NR DC is not configured, k=1 or 2, while for the cases where NR-NR DC is configured, k=2 with one PUCCH group per NR cell group.

## 6.1	Uplink

The tables 6.1-1 and 6.1-2 describe the possible combinations of physical channels and SRS that can be sent in simultaneously in the uplink by one UE. Table 6.1-1 introduces notation for a "Transmission Type" which represents a physical channel or sounding reference signal, and any associated transport channel. Table 6.1-2 describes the combinations of these "Transmission Types" which are supported by the UE depending on capabilities, and enumerates how many of each can be transmitted simultaneously.

Table 6.1-1: Uplink "Transmission Types"

| "Transmission Type" | Physical Channel or SRS | Associated Transport Channel | Comment |
| --- | --- | --- | --- |
| A | PRACH | RACH | Note 1 |
| B | PUCCH | N/A |  |
| C | PUSCH | UL-SCH | Note 2 |
| D | SRS | N/A |  |
| Note 1: RACH corresponds to contention based. Note 2: UCI on PUSCH without UL-SCH is possible. |  |  |  |

Table 6.1-2: Uplink "Transmission Type" combinations

| UE capability | Supported Combinations | Comment |
| --- | --- | --- |
|  | j x A |  |
|  | k x B |  |
|  | p x C | Note 1 |
|  | p' x D | Note 2 |
|  | A + B | Note 3 |
|  | A + C | Note 3 |
|  | A + D | Note 3 |
|  | B + D | Note 4 |
|  | C + D | Note 4 |
| Note 1: In the case there is one SUL carrier, then p-1 would be supported. Note 2: UE may be configured with p' but may also have capability to simultaneously sound less than this number. Note 3: Simultaneous PRACH with PUCCH (or PUSCH or SRS) is supported only in the case of inter-band CA, with , , , and ’ depending on the configuration. Note 4: Simultaneous SRS with PUCCH (or PUSCH) is supported only in the case of inter-band CA, with , , and ’ depending on the configuration. |  |  |

## 6.2	Downlink

The tables 6.2-1, 6.2-2 describe the possible combinations of physical channels that can be received simultaneously in the downlink by one UE. Table 6.2-1 introduces notation for a "Reception Type" which represents a physical channel and any associated transport channel. Table 6.2-2 describes the combinations of these "Reception Types" which are supported by the UE depending on capabilities, and enumerates how many of each can be received simultaneously. The UE shall be able to receive all TBs according to the indication on PDCCH. Any subset of the combinations specified in table 6.2-2 is also supported.

Table 6.2-1: Downlink "Reception Types"

| "Reception Type" | Physical Channel(s) | Monitored RNTI | Associated Transport Channel | Comment |
| --- | --- | --- | --- | --- |
| A | PBCH | N/A | BCH |  |
| B | PDCCH+PDSCH | SI-RNTI | DL-SCH | Note 1 |
| C0 | PDCCH | P-RNTI | N/A | Note 2 |
| C1 | PDCCH+PDSCH | P-RNTI | PCH | Note 1 |
| D0 | PDCCH+PDSCH | RA-RNTI or Temporary C-RNTI | DL-SCH | Note 1 |
| D1 | PDCCH+PDSCH | C-RNTI, CS-RNTI | DL-SCH |  |
| E | PDCCH | C-RNTI | N/A | Note 3 |
| F | PDCCH | C-RNTI, CS-RNTI | UL-SCH |  |
| G | PDCCH | SFI-RNTI | N/A |  |
| H | PDCCH | INT-RNTI | N/A |  |
| J0 | PDCCH | TPC-PUSCH-RNTI | N/A |  |
| J1 | PDCCH | TPC-PUCCH-RNTI | N/A |  |
| J2 | PDCCH | TPC-SRS-RNTI | N/A |  |
| K | PDCCH | SP-CSI-RNTI | N/A |  |
| Note 1: These are received from PCell only. Note 2: In some cases UE is only required to monitor the short message within the DCI for P-RNTI. Note 3: This corresponds to PDCCH-ordered PRACH. |  |  |  |  |

Table 6.2-2: Downlink "Reception Type" combinations

| UE capability | Supported Combinations |  |  | Comment |
| --- | --- | --- | --- | --- |
|  | PCell | PSCell | SCell |  |
| 1. RRC_IDLE |  |  |  |  |
|  | A + (B and/or C1 and/or D0) |  |  | Note 1 |
| 2. RRC_INACTIVE |  |  |  |  |
|  | A + B + C1 + D0 |  |  |  |
| 3. RRC_CONNECTED |  |  |  |  |
|  | A + C0 + (B and/or (D0 or D1)) + E + F + G + H + J0 + J1 + J2 + K | A + C0 + (B and/or (D0 or D1)) + E + F + G + H + J0 + J1 + J2 + K | D1 + F + G + H + J0 + J1 + J2 + K | Note 2 |
| Note 1: UE is not required to decode more than two PDSCH simultaneously, and decoding prioritization when more than two are received is up to UE implementation. Note 2: UE is not required to decode SI-RNTI PDSCH simultaneously with C-RNTI PDSCH, unless in FR1. |  |  |  |  |