3GPP TSG RAN WG1 #101	DRAFT R1-2004640

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.10.4

Source:	Moderator (Nokia)

Title:	FL summary on aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH

Document for:	Discussion and Decision

# 1	Introduction

This contribution is the RAN1#101 meeting discussion summary for the AI7.2.10.4 on aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH.

# 2	Discussion on identified issues

| Issue # | Description | Source |
| --- | --- | --- |
| #1 | For cross-carrier A-CSI-RS triggering with different SCS, RRC parameter [enableDefaultBeamForCCS] is used to enable the default QCL assumption. | Vivo |
| #2 | Proposal 2 (vivo): Capture the default QCL assumption for cross-carrier A-CSI-RS triggering with same SCS when the scheduling offset is smaller than the threshold and no CORESET is configured on A-CSI-RS carrier. Proposal 2 (Qualcomm): In case of same numerology A-CSI RS triggering, when the offset between A-CSI RS and triggering DCI is less than beamSwitchTiming, capture the default QCL agreement in specification. - If no CORESET configured on the carrier for receiving the A-CSI RS, UE receives the A-CSI RS by applying the QCL parameters of the activated PDSCH TCI state with lowest ID. | Vivo, Qualcomm |
| #4 | RRC parameter name alignment - CORESET ID controlResourceSetID - minimumSchedulingOffset minimumSchedulingOffsetK0-r16 | Oppo |
| #5 | Correct the indentation level for three bullets for default QCL assumption determination. | Huawei, Qualcomm |
| #6 | Introduction of a missing RRC parameter in three places - aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16 | Ericsson |
| #7 | A case of missing +d in one timeDurationForQCL+ d∙2μPDSCH/2μPDCCH Note: this was not in the original FL summary as it got dropped between the lines. I am proposing to look at this anyway, as it got lost due to FL mistake | Oppo |

## 2.1	Issue #1

- For cross-carrier A-CSI-RS triggering with different SCS, RRC parameter [enableDefaultBeamForCCS] is used to enable the default QCL assumption.

TS 38.214 5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies < Unchanged parts are omitted > Beam switch timing: - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values of {224, 336} and where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero - if one of the associated trigger states has the higher layer parameter qcl-Type set to 'QCL-TypeD', - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS; - else, - if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored. - else if the UE is configured with [enableDefaultBeamForCCS], when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received. < Unchanged parts are omitted >

FL proposal: Adopt the text proposal above to TS 38214 subclause 5.2.1.5.1a

| Company | Comment |
| --- | --- |
| Qualcomm | We are fine with this proposal. |
| Ericsson | Support |
| MTK | We agree on this proposal. |
| vivo | Support |
| OPPO | Support |
| Lenovo/MOT | support |

May 28 status: Unanimous support, adopt the proposal

## 2.2	Issue #2

- Capture the default QCL assumption for cross-carrier A-CSI-RS triggering with same SCS when the scheduling offset is smaller than the threshold and no CORESET is configured on A-CSI-RS carrier.

- RRC parameter [enableDefaultBeamForCCS] is used to enable the default QCL assumption for cross-carrier A-CSI-RS triggering with same SCS.

| Vivo [1] | Qualcomm [5] |
| --- | --- |
| 5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology < Unchanged parts are omitted > - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48 when the reported value of beamSwitchTiming is one of the values of {224, 336}. - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS; |  |
| - else if the UE is configured with [enableDefaultBeamForCCS], - if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored; - else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received. - else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored. | - else if the active BWP of the serving cell for receiving the aperiodic CSI-RS has configured ControlResourceSet, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored; - else if the UE is configured with [enableDefaultBeamForCCS] and when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received. |

FL proposal: Agree on the intent, and develop a TP to TS 38214 subclause 5.2.1.5.1

| Company | Comment |
| --- | --- |
| Qualcomm | We are fine with both proposals. But the right side one is more compact by avoiding duplicate description for common behaviour between Rel-15 and Rel-16. Besides, the use of the flag [enableDefaultBeamForCCS] is consistent with proposal for Issue #1. Therefore, we prefer to adopt the right side proposal. FYI, the related RAN1 agreement is attached below for reference. Agreements For cross carrier aperiodic CSI-RS with triggering time offset smaller the threshold, with A-CSI RS contains QCL-typeD information: - If there is any other DL signal in the same symbols as the CSI-RS in the scheduled cell, the UE applies the QCL assumption of the other DL signal on the aperiodic CSI-RS. - The definition of “other DL signal” is same as Rel-15 - Otherwise, - if CORESET is configured in the A CSI-RS carrier, the UE follows existing spec text. I.e., A CSI-RS follows QCL assumption of the lowest ID CORESET in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored. - If CORESET is not configured in the A-CSI-RS carrier, apply the QCL assumption in the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the serving cell with the A-CSI-RS transmission |
| Ericsson | Agree to the FL proposal. Note that we write CORESET and not ControlResourceSet in RAN1 specs. We do write ControlResourceSetId when we refer to the RRC field name, though. |
| MTK | We are fine with both proposals. But we slightly prefer the proposal from QC [5] since it seems to match the RAN1 agreement better from our view. |
| Vivo | Support the FL proposal. |
| OPPO | Support to have a TP. |
| Lenovo/MOT | Support FL’s proposal. |

May 28 status: Unanimous support, adopt the proposal to develop a TP based on the proposals above

June 2nd: Updated proposal [Ericsson] based on the one in [5], the updated part yellow-highlighted

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48 when the reported value of beamSwitchTiming is one of the values of {224, 336}.

-	if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS;

-	else if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored;

-	else if the UE is configured with [enableDefaultBeamForCCS] and when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received.

## 2.3	Void

## 2.4	Issue #4

##### 5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies When the triggering PDCCH and the triggered aperiodic CSI-RS are of different numerologies, the ehaviour defined in 5.2.1.5.1 for the case where the numerologies are the same applies with the following exceptions: Beam switch timing: (omitted part) - else, - if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored. (omitted part) Aperiodic CSI-RS timing: - When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset, including the case that the UE is not configured with minimumSchedulingOffsetK0-r16 for any DL or UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to ‘QCL-TypeD’ in the corresponding TCI states.. The CSI-RS triggering offset has the values of {0, 1,…,31} slots when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 16, 24} when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where (omitted part)

FL proposal: Agree on the TP above to TS 38214 subclause 5.2.1.5.1a fixing two RRC parameter names

| Company | Comment |
| --- | --- |
| Qualcomm | We are fine with the proposal. |
| Ericsson | Support |
| MTK | We agree on this proposal. |
| Vivo | Fine |
| OPPO | Support |
| Lenovo/MOT | Support |

May 28 status: Unanimous support, adopt the proposal

## 2.5	Issue #5

5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies When the triggering PDCCH and the triggered aperiodic CSI-RS are of different numerologies, the ehaviour defined in 5.2.1.5.1 for the case where the numerologies are the same applies with the following exceptions: Beam switch timing: - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values of {224, 336} and where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero - if one of the associated trigger states has the higher layer parameter qcl-Type set to ‘QCL-TypeD’, - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS; - else, - if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored. - else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols, when the reported value is one of the values of {14,28,48}, or is equal to or greater than 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values of {224, 336}, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI. <Unchanged parts omitted>

FL proposal: Agree on the TP above correcting the indentation on the three bullets to TS 38214 subclause 5.2.1.5.1a

| Company | Comment |
| --- | --- |
| Qualcomm | The proposal is needed so that spec follows RAN1 agreement. The related RAN1 agreement is attached below for reference. Agreements For cross carrier aperiodic CSI-RS with triggering time offset smaller the threshold, with A-CSI RS contains QCL-typeD information: - If there is any other DL signal in the same symbols as the CSI-RS in the scheduled cell, the UE applies the QCL assumption of the other DL signal on the aperiodic CSI-RS. - The definition of “other DL signal” is same as Rel-15 - Otherwise, - if CORESET is configured in the A CSI-RS carrier, the UE follows existing spec text. I.e., A CSI-RS follows QCL assumption of the lowest ID CORESET in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored. - If CORESET is not configured in the A-CSI-RS carrier, apply the QCL assumption in the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the serving cell with the A-CSI-RS transmission |
| Ericsson | Support |
| MTK | We agree on this proposal |
| vivo | OK |
| OPPO | Support |
| Lenovo/MOT | Support |

May 28 status: Unanimous support, adopt the proposal

## 2.6	Issue #6

Introduction of a missing RRC parameter in three places

- aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16

Shown below with the cyan highlight on top of the April-agreed CR revision marks

##### 5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology <omitted text> When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 16, 24} slots. If the UE is not configured with [minimumSchedulingOffset] for any DL or UL BWP and if all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states , the CSI-RS triggering offset is fixed to zero. The aperiodic triggering offset of the CSI-IM follows offset of the associated NZP CSI-RS for channel measurement. <omitted text> ##### 5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies <omitted text> Aperiodic CSI-RS timing: - When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16, including the case that the UE is not configured with [minimumSchedulingOffset] for any DL or UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states.. The CSI-RS triggering offset has the values of {0, 1,…,31} slots when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 16, 24} when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where - n is the slot containing the triggering DCI, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16, - μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively, <omitted text>

FL proposal: Agree on the TP above adding or aperiodicTriggeringOffsetExt-r16 in three different places in TS 38.214 subclauses 5.2.1.5.1 and 5.2.1.5.1a

| Company | Comment |
| --- | --- |
| Qualcomm | We agree new parameters for Rel-16 should be captured in spec, but it is unclear why this parameter should be discussed in current cross-carrier A-CSIRS agenda. It would be necessary if the proponent can clarify the background about the proposal. In particular, could you please provide the RAN1 agreement information related to aperiodicTriggeringOffsetExt-r16 and was the discussion for this parameter made in current agenda? If our understanding is correct, the paper for Issue #6 proposal has referred to R1-2003138 for this new RRC parameter. However, we foud R1-2003138 did not mention aperiodicTriggeringOffsetExt-r16. |
| Ericsson | The new RRC parameter was introduced in this topic (see RRC parameter list R1-2003190, row 12 in NRDCCA tab) and hence the alignment proposal is made here. The extended values are also in 3138, but since the parameter belongs to this WI, the alignment is proposed here. |
| MTK | We are fine with the proposal |
| vivo | OK |
| Qualcomm 2nd | Thanks Ericsson for providing the background information about agreement for the TP. The spreadsheet mentioned the following in row 2 in NRDDCA. RAN1#98bis New slot offset values - Extend the X(≥0) values for cross-carrier A-CSI RS triggering when the PDCCH and A-CSI RS have different SCS RAN1:99: Range of X: {0,…,31} It looks the RAN1 #99 agreement was not correctly captured. For this, the related RAN1 #99 agreement and RAN1 #98bis agreement are copied below. The agreements show that - The extended value range of triggering offset applies to cross-carrier CSI-RS triggering with different numerology case - The extended value ragne applies to smaller SCS PDCCH triggering larger SCS A-CSIRS case For that, we think coverage of TP should be reduced to µPDCCH < µCSI-RS. Agreements: When µPDCCH < µCSI-RS, X{0, 1, …, 31} Agreements: To support aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH: Definition of the slot index where A-CSI RS is transmitted - n12CSI-RS/2PDCCH +Xn12μCSI-RS2μPDCCH+X, where the n1 is the PDCCH carrier slot with the DCI - Note: The case when the frame boundaries of the two carriers are not aligned may require additional compensation when determining the actual slot number New slot offset values - Extend the X(≥0) values for cross-carrier A-CSI RS triggering when the PDCCH and A-CSI RS have different SCS - FFS X Minimum A-CSI RS triggering offset for cross-carrier triggering of A-CSI RS when the PDCCH SCS and the A-CSI RS SCS are not the same - Minimum delay from the end of the triggering PDCCH and the start of the CSI-RS in the CSI-RS carrier’s slots is defined as m - m = 4, 4, 8, [12] symbols for PDCCH SCS = 15, 30, 60, 120 kHz, respectively as defined for cross-carrier scheduling of PDSCH with different PDCCH and PDSCH SCS. - [Working assumption] When µPDCCH < µCSI-RS the minimum delay is quantized to the beginning of the next A-CSI RS carrier slot - FFS impact, if any, due to beam switching timing |
| OPPO | Support to add the parameter. |
| Lenovo/MOT | Support FL’s proposal. |
| Ericsson2 | For 5.2.1.5.1a, we understand your comment and is the following update look OK (in red)? <start TP for endorsed editor CR R1-2003138> ##### 5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies <omitted text> Aperiodic CSI-RS timing: - When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16 when µPDCCH < µCSIRS. The CSI-RS triggering offset has the values of {0, 1,…,31} slots when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with CA-slot-offset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where - n is the slot containing the triggering DCI, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16 when µPDCCH < µCSIRS, <end TP> As explained in R1-2004204, intention of the TP was to also reflect RRC parameter name update for 5.2.1.5.1 and 5.2.1.5.1a (with µPDCCH > µCSIRS) covering below agreements, and this was missing from Editor CR in R1-2003138. We did not fully follow your comment about RRC parameter, are you saying instead of existing RRC parameters, a new RRC parameter is needed for this agreement? Agreements: - The minimum scheduling offset restriction is not applied when PDSCH transmission is scheduled with MsgB-RNTI. - When the minimum scheduling offset restriction is applied the UE is not expected to be scheduled with a DCI in slot n to transmit a PUSCH scheduled with SP-CSI-RNTI with K2 smaller than the applicable minimum scheduling offset restriction K2min. - For an active BWP with scheduling offset restriction(s) configured and when UE detects an invalid TDRA entry violating current applied K0min/K2min from DCI format 1_0/0_0, one of the following is decided in RAN1 #101-e meeting: - Alt 1: Additional RAN1 specification is defined for handling such error case - Solution to be converged from companies’ proposals to RAN1 #101-e - Alt 2: No additional RAN1 specification is defined for handling such error case - Aperiodic CSI-RS triggering offset value range is extended from {0, 1, 2, 3, 4, 16, 24} to {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} - (Working assumption) The minimum scheduling offset restriction is not applied when PDSCH transmission is scheduled with C-RNTI or MCS-C-RNTI in the search space set provided by recoverySearchSpacedId when monitoring PDCCH as described in Section 6 [38.213]. |

May 28 status: The intent of the RRC parameter is agreeable, but there maybe additional issues that may need to be resolved when developing the TP

June 2nd: Updated proposal [Ericsson] based on the one in [5], the updated part yellow-highlighted while the original update on top of the endorsed 3138 is shown in Cyan highlight

June 2nd update #2, removed the change to 5.2.1.5.1

##### 5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies <omitted text> Aperiodic CSI-RS timing: - When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16, including the case that the UE is not configured with [minimumSchedulingOffset] for any DL or UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states.. The CSI-RS triggering offset has the values of {0, 1,…,31} slots when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 16, 24} when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where - n is the slot containing the triggering DCI, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16 when µPDCCH < µCSIRS, - μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively, <omitted text>

## 2.7	Issue #7

The following was not present in the feature lead summary:

- a case of missing timeDurationForQCL+ d∙2μPDSCH/2μPDCCH

##### 5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies When the triggering PDCCH and the triggered aperiodic CSI-RS are of different numerologies, the behavior defined in 5.2.1.5.1 for the case where the numerologies are the same applies with the following exceptions: Beam switch timing: - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values of {224, 336} and where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero - if one of the associated trigger states has the higher layer parameter qcl-Type set to 'QCL-TypeD', - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL+ d∙2μPDSCH/2μPDCCH, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS; (omitted part)

FL proposal: Agree on the TP above to TS 38214 subclause 5.2.1.5.1a

| Company | Comment |
| --- | --- |
| Qualcomm | We agree with the intension of the TP, but it is better to clarify a few points first. We noticed the following text in TS 38.214 where d2μPDSCH2μPDCCH is added to timeDurationForQCL. It seems with this text, timeDurationForQCL now has inlucded d2μPDSCH2μPDCCH whenever applicable. If the TP is accepted, it means that timeDurationForQCL in the TP is the original value, i.e., no double additional of d2μPDSCH2μPDCCH. - The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If μPDCCH < μPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1; There are also several other places in TS 38.214 with wording like - and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, If we adopt the TP, should we change all the other places with timeDurationForQCL in the spec? |
| Ericsson | We agree with Qualcomm. Since timeDurationForQCL is a capability, it would seem inappropriate to add anything to it – it is constant. We support a review of the specification to include the offset d2μPDSCH2μPDCCH explicitly in the appropriate places. |
| MTK | Same view as QC and Ericsson. |
| vivo | Not fine. |
| OPPO | Companies may have different understandings on current description of 38.214. We are fine to current description in 38.214 if it is common understanding that timeDurationForQCL has considered d in all places if there is cross-carrier schelduling. |
| Lenovo/MOT | We agree with Qualcomm and Ericsson. |

May 28 status: Do not adopt the proposal

# 3	Conclusion after the 1st meeting week

Agreement:

Adopt the TPs in Sections 2.1, 2.4 and 2.5 of R1-2004640 for Clause 5.2.1.5.1a of TS38.214.

For issue in 2.2 of R1-2004640, develop a TP by 6/4 based on the two proposals in section 2.2 of DRAFT R1-2004640 - RAN1-101 7.2.10.4 V009_LenovoMM_Eric2

For issue in 2.6 of R1-2004640, develop TP by 6/4 based on the updated Ericsson proposal in DRAFT R1-2004640 - RAN1-101 7.2.10.4 V009_LenovoMM_Eric2

## 3.1	Issue #1

As agreed, adopt the TP in section 2.1 to TS38.214 subclause 5.2.1.5.1a

| Reason for change: | As agreed earlier, for cross-carrier A-CSI-RS triggering with different SCS, RRC parameter [enableDefaultBeamForCCS] should be used to enable the default QCL assumption |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | Add a clause conditioning the default QCL assumption behaviour to the RRC configuration for cross-carrier A-CSI-RS triggering with different SCSs |  |  |  |
| Consequences if not approved: | Functionality intended to be RRC configured becomes a default functionality that the network cannot control, and the specification is not according to the RAN1 agreements. |  |  |  |
| Clauses affected: | 5.2.1.5.1a |  |  |  |
|  | Y | N |  |  |
| Other specs |  | x | Other core specifications |  |
| affected: |  | x | Test specifications |  |
| (show related CRs) |  | x | O&M Specifications |  |
| Other comments: |  |  |  |  |

## 3.2	Issue #2

As agreed, develop a TP by 6/4 based on the two proposals in section 2.2 to TS38.214 subclause 5.2.1.5.1

June 2nd update: see the updated TP in the end of section 2.2

| Reason for change: | Introduce the the default QCL assumption for cross-carrier A-CSI-RS triggering with same SCS when the scheduling offset is smaller than the threshold and no CORESET is configured on A-CSI-RS carrier. RRC parameter [enableDefaultBeamForCCS] should be used to enable the default QCL assumption for cross-carrier A-CSI-RS triggering with same SCS |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | Introduce the the default QCL assumption for cross-carrier A-CSI-RS triggering with same SCS when the scheduling offset is smaller than the threshold and no CORESET is configured on A-CSI-RS carrier. Add a clause conditioning the default QCL assumption behaviour to the RRC configuration for cross-carrier A-CSI-RS triggering with the same SCS |  |  |  |
| Consequences if not approved: | The specification is not according to the RAN1 agreements wrt. the default QCL assumption for the cross-carrier A-CSI RS triggering with the same SCS |  |  |  |
| Clauses affected: | 5.2.1.5.1 |  |  |  |
|  | Y | N |  |  |
| Other specs |  | x | Other core specifications |  |
| affected: |  | x | Test specifications |  |
| (show related CRs) |  | x | O&M Specifications |  |
| Other comments: |  |  |  |  |

## 3.3	Issue #3

Not in scope

## 3.4	Issue #4

As agreed, adopt the TP in section 2.4 to TS38.214 subclause 5.2.1.5.1a

| Reason for change: | Fixing two incorrect RRC parameter names |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | CORESET ID controlResourseSetID [minimumSchedulingOffset] minimumSchedulingOffsetK0 |  |  |  |
| Consequences if not approved: | RRC parameters are incorrectly referenced |  |  |  |
| Clauses affected: | 5.2.1.5.1a |  |  |  |
|  | Y | N |  |  |
| Other specs |  | x | Other core specifications |  |
| affected: |  | x | Test specifications |  |
| (show related CRs) |  | x | O&M Specifications |  |
| Other comments: |  |  |  |  |

## 3.5	Issue #5

As agreed, adopt the TP in section 2.5 to TS38.214 subclause 5.2.1.5.1a

| Reason for change: | Incorrect indentation in the default QCL assumption determination |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | Correct the indentation of three bullets defining the default QCL assumption determination |  |  |  |
| Consequences if not approved: | the “if…else…” relationship that corresponds to the “If…Otherwise…” in the specification is not aligned with the agreement and lead to incorrect logic when interpreting the if…else…otherwise structure. |  |  |  |
| Clauses affected: | 5.2.1.5.1a |  |  |  |
|  | Y | N |  |  |
| Other specs |  | x | Other core specifications |  |
| affected: |  | x | Test specifications |  |
| (show related CRs) |  | x | O&M Specifications |  |
| Other comments: |  |  |  |  |

## 3.6	Issue #6

As agreed, develop a TP by 6/4 based on the updated Ericsson proposal in section 2.6 to TS38.214 subclause 5.2.1.5.1

June 2nd update: see the updated TP in the end of section 2.6

| Reason for change: | The new Rel-16 RRC parameter introducing a wider range of triggering offsets for the case where the PDCCH SCS < CSI-RS SCS cannot be used as both the parameter and the SCS condition are missing from the specification |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | Introduction of the Rel-16 RRC parameter name to provide the triggering offset. Addition of the SCS relationship condition to the new RRC parameter |  |  |  |
| Consequences if not approved: | The new Rel-16 RRC parameter with a larger range of triggering offsets cannot be used. |  |  |  |
| Clauses affected: | 5.2.1.5.1a |  |  |  |
|  | Y | N |  |  |
| Other specs |  | x | Other core specifications |  |
| affected: |  | x | Test specifications |  |
| (show related CRs) |  | x | O&M Specifications |  |
| Other comments: |  |  |  |  |

## 3.7	Issue #7

The proposal was not agreed

# References

- R1-2003412 Remaining issues on aperiodic CSI-RS triggering, vivo

- R1-2004058 Text proposals for aperiodic CSI-RS triggering with different numerologies, OPPO

- R1-2004150 Remaining issues on the A-CSI RS triggering with different numerology, Huawei, HiSilicon

- R1-2004204 Maintenance for cross-carrier CSI-RS triggering, Ericsson

- R1-2004475 Remaining issues for aperiodic CSI-RS triggering with different numerology, Qualcomm Inc.

- R1-2004638 FL summary on aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH, Moderator (Nokia)