3GPP TSG RAN WG1 #101	DRAFT R1-2004639

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.10.6

Source:	Moderator (Nokia)

Title:	FL summary on cross-carrier scheduling with different numerology

Document for:	Discussion and Decision

# 1	Introduction

This contribution is the pre-meeting feature-lead summary of the xxxxxxx documents submitted for the AI7.2.10.6 on cross-carrier scheduling with different numerologies.

< Companies are invited to provide comments on the FL proposals to this draft in section 3 >

# 2	Summary of issues addressed in the Tdocs

| Issue # | Description | Source |
| --- | --- | --- |
| 1 | Codebook for more than 1 DCI per monitoring occasion - The PDSCH starting time in addition to the existing MO and Cell index is introduced to order the HARQ-ACK feedback [ZTE] - Type-2 HARQ-ACK codebook and PUCCH resource determination: use CCE index [Huawei] - C-DAI is used as a third dimension of HARQ-ACK bits ordering in Type2 HARQ-ACK CB [Intel] - The (counter) DAI values need to be determined according to some factor such as CCE index used for the PDCCH resource conveying the DCI [LG] - Update HARQ-ACK codebook and PUCCH resource determination to enable reception of multiple valid DCIs at the same monitoring occasions and multiplexing of corresponding feedback in same HARQ codebook [Ericsson] | ZTE, Huawei, Intel, LG, Ericsson |
| 2 | Cross-carrier SPS PDSCH release: Need to update the type-1 HARQ-ACK codebook to determine HARQ-ACK bit-location? | vivo, CATT |
| 3 | The “HARQ codebook blocking” issue of UE supporting only one unicast PDSCH slot can be handled by gNB scheduler, no spec change is needed. | vivo |
| 4 | Whether to introduce separate UE capability to indicate the support of the DCI format 1_2 for cross-carrier scheduling (FG 18-5e). | MediaTek |

# 3	Discussion on the scope of the RAN1#101

Issue #1: [ZTE, Huawei, Intel, LG Electronics, Ericsson]

- Codebook for more than 1 DCI per monitoring occasion

FL proposal: Discuss how to handle the > 1 DCI per MO in RAN1#101

Comments on the FL proposal

| Company | Comment on FL proposal wrt. issue #1 |
| --- | --- |

Issue #2: [vivo, CATT]

- Type-1 HARQ-ACK codebook determination update for SPS-PDSCH release

FL proposal: The issue has been debated for several meetings with no consensus. Suggest no further discussions on the topic and NOT to discuss the issue in RAN1#101.

Comments on the FL proposal

| Company | Comment on FL proposal wrt. issue #1 |
| --- | --- |

Issue #3: [vivo]

- The “HARQ codebook blocking” issue of UE supporting only one unicast PDSCH slot can be handled by gNB scheduler, no spec change is needed.

FL proposal: The proposed solution for the raised issue does not require any specification changes, and no other proposals have been made. Thus suggest NOT to discuss the issue in RAN1#101.

Comments on the FL proposal

| Company | Comment on FL proposal wrt. issue #1 |
| --- | --- |

Issue #4: [MediaTek]

- Whether to introduce separate UE capability to indicate the support of the DCI format 1_2 for cross-carrier scheduling (FG 18-5e).

FL proposal: The issue is on the list of topics to discuss under the MR-DC UE feature and is not functional in nature. Suggest to defer the discussion to the MR-DC UE feature agenda and avoid parallel discussion threads.

Comments on the FL proposal

| Company | Comment on FL proposal wrt. issue #1 |
| --- | --- |

# 4	Conclusion on the scope of the RAN1#101

To be completed

# References

- R1-2003328, Remaining Issues on Cross-carrier Scheduling with Mixed Numerologies, ZTE

- R1-2003414, Remaining issues on cross-carrier scheduling with mix numerologies, vivo

- R1-2003508, Remaing issues on cross-carrier scheduling with different numerology, Huawei, HiSilicon

- R1-2003602, Discussion on HARQ-ACK feedback for SPS PDSCH release with cross-carrier scheduling, CATT

- R1-2003675, Remaining issues on cross-carrier scheduling with different numerology, MediaTek Inc.

- R1-2003751, Remaining issues on cross-carrier scheduling with different numerology, Intel Corporation

- R1-2004037, Remaining issue on cross-carrier scheduling with different numerology, LG Electronics

- R1-2004366, Remaining issues for cross-carrier scheduling with different numerologies, Ericsson

# Annex – Proposals of the submitted Tdocs

R1-2003328, Remaining Issues on Cross-carrier Scheduling with Mixed Numerologies, ZTE

Proposal 1: If the maximum number of unicast DCIs per MO per scheduled cell is increased to larger than one, the PDSCH starting time in addition to the existing MO and Cell index is introduced to order the HARQ-ACK feedback.

R1-2003414, Remaining issues on cross-carrier scheduling with mix numerologies, vivo

Proposal 1: Spec update to derive bit location in type-1 HARQ-ACK Codebook for cross-carrier SPS release is not necessary.

Proposal 2: The “HARQ codebook blocking” issue of UE supporting only one unicast PDSCH slot can be handled by gNB scheduler, no spec change is needed.

R1-2003508, Remaing issues on cross-carrier scheduling with different numerology, Huawei, HiSilicon

Proposal 1: CCE index is introduced to further determine the Type-2 HARQ-ACK codebook and PUCCH resource.

Proposal 2: Adopt the TP1 and TP2 in the appendix.

R1-2003602, Discussion on HARQ-ACK feedback for SPS PDSCH release with cross-carrier scheduling, CATT

Observation 1: Cross-carrier SPS PDSCH release will introduce scheduling restrictions for the carrier with SPS PDSCH release in case a UE doesn’t support more than one unicast PDSCHs within one slot.

Observation 2: If a UE doesn’t support more than one PDSCH receptions within a slot, the UE behavior is unclear when a unicast PDSCH is scheduled in a slot on the SPS PDSCH carrier and the HARQ-ACK of SPS release is associated with the slot.

Proposal: For cross-carrier SPS PDSCH release, the specification corresponding to HARQ-ACK generation of SPS PDSCH release should be updated. The above text proposal should be endorsed.

R1-2003675, Remaining issues on cross-carrier scheduling with different numerology, MediaTek Inc.

Proposal 1: Discuss whether to introduce separate UE capability to indicate the support of the DCI format 1_2 for cross-carrier scheduling (FG 18-5e).

R1-2003751, Remaining issues on cross-carrier scheduling with different numerology, Intel Corporation

Proposal 1: Assuming X>1 DL DCIs scheduling unicast PDSCHs per scheduled cell can be transmitted in a PDCCH MO, C-DAI is used as a third dimension of HARQ-ACK bits ordering in Type2 HARQ-ACK CB.

R1-2004037, Remaining issue on cross-carrier scheduling with different numerology, LG Electronics

Proposal: DAI counting rule needs to be defined for the multiple DCIs scheduling a same serving cell transmitted within a same MO, to avoid potential misalignment on the ordering of HARQ-ACK bits in the HARQ-ACK payload for dynamic codebook between UE and gNB.

- The (counter) DAI values need to be determined according to some factor such as CCE index used for the PDCCH resource conveying the DCI.

- The following is the TP for TS38.213 based on the CCE index.

R1-2004366, Remaining issues for cross-carrier scheduling with different numerologies, Ericsson

Proposal 1: Release-16 supports reception of multiple valid DCIs at the same monitoring occasions and multiplexing of corresponding feedback in same HARQ codebook.

Proposal 2: Update HARQ-ACK codebook and PUCCH resource determination to enable reception of multiple valid DCIs at the same monitoring occasions and multiplexing of corresponding feedback in same HARQ codebook.