3GPP TSG RAN WG1 #101			                                   					       												                  R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.11.3

Source:	Moderator (AT&T)

Title:	Summary of email discussion/approval [101-e-NR-IAB-UEFeatures-01]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [101-e-NR-IAB-UEFeatures-01] during RAN1 #101-e. According to the Chairman’s Notes:

[101-e-NR-IAB-UEFeatures-01] Email discussion/approval till 5/29 – Ralf (AT&T) - Whether and how to confirm the working assumption from RAN1 #100bis-e including clarifying the IAB class for the working assumption

The following was discussed and agreed during RAN1 #101-e within the scope of [101-e-NR-IAB-UEFeatures-01].

# Summary of email discussion/approval [101-e-NR-IAB-UEFeatures-01]

The following table represents the version of the NR UE feature list for IAB agreed by RAN1 during RAN1 #100bis-e Error: Reference source not found.

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20-2 | Inter-IAB-node discovery and measurements: SSB reception configuration | Support up to 4 SMTCs configured for an IAB node MT per frequency location, including IAB-specific SMTC window periodicities |  |  |  | Yes | N/A | Separate configuration of SMTC windows for Inter-IAB node discovery and measurement is not possible | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. [Devices supporting IAB backhaul must report this FG as supported] |
|  | 20-3 | Extension of RACH occasions and periodicities for backhaul RACH resources | Support RACH configuration for IAB-MT separately from the RACH configuration for UE access, including new IAB-specific offset and scaling factors |  |  |  | Yes | N/A | Separate configuration of RACH transmissions for access UEs and IAB nodes is not possible | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. [Devices supporting IAB backhaul must report this FG as supported] |
|  | 20-5a | UL-Flexible-DL slot formats | Support semi-static configuration/indication of UL-Flexible-DL slot formats for IAB-MT resources |  | 5-1a |  | Yes | N/A | Only Rel-15 slot formats can be configured for backhaul links | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
|  | 20-5b | UL-Flexible-DL slot formats | Support dynamic indication of UL-Flexible-DL slot formats for IAB-MT resources |  | 3-6 |  | Yes | N/A | Dynamic indication of UL-Flexible-DL slot formats for IAB-MT resources is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
|  | 20-6 | Dynamic indication of soft resource availability | Support monitoring DCI Format 2_5 scrambled by AI-RNTI for indication of soft resource availability to an IAB node |  |  |  | Yes | N/A | Explicit indication of soft resource availability is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. [Devices supporting IAB backhaul must report this FG as supported] |
|  | 20-7 | Case 1 OTA timing alignment | Support T_delta reception. |  |  |  | Yes | N/A | Case-1 OTA timing alignment is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. [Devices supporting IAB backhaul must report this FG as supported] |
|  | 20-8 | Guard symbols | 1) Support DesiredGuardSymbols reporting 2) Support ProvidedGuardSymbols reception |  |  |  | Yes | N/A | Guard symbols reporting and reception is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |

Companies are invited to provide their views on whether and how to confirm the following working assumption from RAN1 #100bis-e in the following table. Please also comment on whether/how to clarify the IAB class for the working assumption.

Working Assumption: - IAB-MTs support the following Rel. 15 layer-1 mandatory UE features (as defined in TR38.822) at least for wide-area IAB-nodes - Without capability - 0-1, 0-3, 0-4, 1-1 [FFS components], 2-1, [2-5], [2-6], 2-12, [2-16], [2-16a], 2-32 [FFS components], 2-50 [FFS components], 2-52 [FFS components], 3-1 [FFS components], [3-4], [4-1], [4-10], 5-1 [FFS components], 6-1, 7-1, [8-3] - With capability signaling which shall be set to '1' - [1-3] - FFS whether additional Rel. 15 layer-1 mandatory UE features (as defined in TR38.822) including list below are optional at least for IAB-MTs of wide-area IAB-nodes, remain mandatory, or have a default value - Without capability - [0-2], [4-3], [4-4], [5-6] - With capability signaling - [1-10], [2-2], [2-21], [2-22], [2-25] - Note: This doesn’t mean that the UE features under FFS will not be supported by wide-area IAB-MT at all if RAN1 does not reach a consensus on them

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | This discussion and possible resulting agreement should only concern wide area IAB nodes, i.e., nodes that have a planned and operator-controlled deployment, since we don’t see this subset of FGs properly functioning for the local area IAB node and discussing bot simultaneously will only complicate and confuse the discussion. Correspondingly, any discussion about mandatory features without a clear definition of basic operation will likely also be complicated. We support the above WA. Considering our definition of basic operation, i.e., reaching RRC_CONNECTED, we don’t see a need for any mandatory FGs other than what is in the WA above, excluding the bracketed ones, hence, these should be removed. Regarding the relation between FGs 2-5 and 2-6, on the one hand, and 2-16 and 2-16a, on the other hand, 3GPP does not specify conditionally mandatory FGs, i.e., mandating one out of equal alternatives. Hence, to be consistent with the above argumentation and 3GPP praxis, these FGs should be optional. In order to further progress, among the mandatory FGs, we can agree to making the following components mandatory: Feature group; Mandatory components. 1-1; 1 (at least one preamble). 2-32; 1-4. 2-50; 1. 2-52; 1. 3-1; 1, 4, 5. 5-1; 1-4, 6, 9, 10, 12. In RAN1 #100bis, some concern was expressed regarding the implications on the ASN.1 list if mandatory features without capability signaling are made optional. By using manufacturer declaration, it is possible to maintain the ASN.1 unchanged. As pointed out in the discussions, UE feature signaling is only needed if the IAB node is unknown to the network. That is not the case for wide area, operator deployed IAB nodes after sending RRCSetupComplete and hence, no changes to ASN.1 are needed for that class of IAB nodes. We acknowledge, however, that a different solution is likely needed for local area IAB nodes. Finally, we would like to point out that a parallel Rel-15 UE features discussion will be held in RAN2. Not foregoing that discussion, we think that it is highly desirable that different RAN WGs adopt the same basic principle when determining mandatory vs optional FGs. Furthermore, remaining topics to be discussed in RAN2 are, whether, in general, mandatory features with capability signaling are optional for IAB-MT and if UE capability signaling will be used at all for Wide Area MTs. Presumably, the latter may only be achieved after the MT being identified as an IAB node, i.e., upon reaching RRC_CONNECTED. Hence, such a definition of basic operation for wide area IAB nodes is likely to be agreeable also in RAN2. |
| AT&T | We believe that making a feature mandatory for a wide-area IAB-MT should be based on the criteria that the feature is essential for Initial Access, RRC Connection establishment, Topology Formation (e.g. HO/RLF), or basic backhaul link operation. Given that these requirements may vary for wide-area vs. local-area IAB-MTs based on deployment scenarios, we believe the Working Assumption can be confirmed and applies only to wide-area IAB-MTs. For the items in brackets we share our views below: Feature group; Mandatory components. 0-2; Not mandatory for wide-area IAB-MTs. 1-1; All components support either Initial Access or Topology Formation. 1-3; Essential for basic backhaul link operation and topology formation. 1-10; Essential for basic backhaul link operation and topology formation. 2-2; Essential for basic backhaul link operation at least in FR2 (keep conditioned on FR2 as in 38.822). 2-5; All components are essential for basic backhaul link operation (keep conditioned on scheduling as in 38.822). 2-6; All components are essential for basic backhaul link operation (keep conditioned on scheduling as in 38.822). 2-16; All components are essential for basic backhaul link operation (keep conditioned on scheduling as in 38.822). 2-16a; All components are essential for basic backhaul link operation (keep conditioned on scheduling as in 38.822). 2-21; All components are essential for basic backhaul link operation. 2-22; Not mandatory for wide-area IAB-MTs. 2-25; Essential for basic backhaul link operation at least in FR2 (candidate set Xi may be different for IAB-MTs than Rel-15 UEs). 2-32; {1-4 (FFS 5 or 6)} are essential for basic backhaul link operation. 2-50; {1, 2 (only a subset of TRS periodicity may be supported pending RAN4 requirements)} are essential for basic backhaul link operation. 2-52; {1, 2} are essential for basic backhaul link operation. 3-1; All components are essential for basic backhaul link operation. 3-4; Essential for basic backhaul link operation. 4-1; All components are essential for basic backhaul link operation. 4-3; Not mandatory for wide-area IAB-MTs. 4-4; Not mandatory for wide-area IAB-MTs. 4-10; All components are essential for basic backhaul link operation. 5-1; All components are essential for basic backhaul link operation except for components 5 and 8. 5-6; Not mandatory for wide-area IAB-MTs. 8-3; All components are essential for basic backhaul link operation. |
| Huawei | We think the discussion should first focus on wide-area IAB nodes. Wide-area IAB nodes and Local-area IAB nodes may require quite different features due to the distinctions in deployment scenarios, RF characteristics and controllability. Wide-area IAB nodes is essentially a network node. Typically the network node features are not specified since they mainly rely on the negotiation between the operator and the vendor. Despite of this, there are some features that are rather basic such as waveform and initial access for connection setup which may not be part of the negotiation. Hence the criteria for selecting the mandatory features for wide area IAB nodes is basic operation such as waveform and initial access for connection setup. We are supportive to confirm the working assumption for wide-area IAB nodes. The following features are mandatory for wide-area IAB nodes (include both the ones with and without brackets) - Without capability - 0-1 - 0-3 - 0-4 - 1-1 (components 1/3) - 2-1 - 2-12 - 2-32 (components 1/2/3/4) - 2-50 - 2-52 - 3-1 - 4-1 (components 1/2/3/4/5/6/8/10) - 5-1 (components 1/2/3/4/5/6/9/10/12) - 6-1 - 7-1 - 8-3 (components 1/2/3/4/5/6/7/8) - With capability signaling which shall be set to '1' - 4-10 The Rel-15 layer-1 UE features other than the ones listed above are either not applicable (optional) for wide-area IAB-nodes. How to capture the mandatory features for IAB-MT into specification is being discussed in RAN2 hence be decided there. |
| LG | Before making further decisions, it should be discussed first whether this capabilities could be known to the network before initial connection or not. Our view is that we should not make IAB-MT feature list with such assumption or only targeting to wide area IAB-MT since Rel-16 specifications should cover general IAB-MT nodes. Then, we think FG 0-2 (DFT-S-OFDM in UL) should be mandatory to support large coverage IAB nodes. Moreover, we think at least one of FG 2-21 (periodic beam report) or FG 2-22 (aperiodic beam report) should be mandatory to leverage NR’s essential features related to the multi-beam operation, where we prefer FG 2-22 if one of them should be selected. In addition, it should be clarified FG 1-10 is still mandatory conditioned on intra-band CA while intra-band CA itself is reported capability as in Rel-15 For the other parts of WA, we are ok with adding such FGs above, removing square brackets, removing ‘FFS per components’ and removing ‘wide area IAB nodes’ so that our overall proposal is as follows. - IAB-MTs support the following Rel. 15 layer-1 mandatory UE features (as defined in TR38.822) at least for wide-area IAB-nodes - Without capability - 0-1, 0-2, 0-3, 0-4, 1-1, 1-10, 2-1, 2-5, 2-6, 2-12, 2-16, 2-16a, 2-32, 2-50, 2-52, 3-1, 3-4, 4-1, 4-10, 5-1, 6-1, 7-1, 8-3 - With capability signaling which shall be set to '1' - 1-3, 2-22 |
| Nokia, NSB | As mentioned by other companies above, RAN2 is already currently discussing which signalling mechanisms can be used to identify wide-area IAB-MT devices and to communicate capabilities if needed, including the possibility of defining the features as optional by declaration. Hence, RAN1 can continue to work specifically on defining the subset of features that are mandatory for wide-area IAB-MT devices. We agree with the overall statements from Ericsson and Huawei above that features involving initial setup are the most critical ones to be considered by RAN1. We are fine to confirm the working assumption for the items not currently in brackets or which have FFS on the exact components that are mandatory. Our detailed proposals are as follows: - 0-2: Optional without capability signalling - 1-1: Okay to be Mandatory. Only 1 preamble and component 1) is needed - 2-5: Optional without capability signalling - 2-6: Optional without capability signalling - 2-16a: Optional without capability signalling - 2-32: Only a subset should be mandatory. Components 1-4 is a reasonable subset. The remaining should be optional without capability signalling - 2-50: TRS is mandatory. Not all periodicities and configuration in all bandwidth parts are required. These remaining configurations will be optional without capability signalling. - 2-52: Only 1-port Basic SRS need be mandatory. These remaining configurations will be optional without capability signalling. - 3-1: Only one configured CORESET per BWP per cell is necessary. These remaining configurations will be optional without capability signalling. - 3-4: Optional without capability signalling. The maximum number of TCI states need not be 64 for a stationary IAB node. - 4-1: Optional without capability signalling - 4-3: Optional without capability signalling - 4-4: Optional without capability signalling - 4-10: Optional without capability signalling - 5-1: Generally mandatory. It is okay for components 5, 7, 8, and 11 to be optional - 5-6: Optional without capability signalling - 8-3: Optional without capability signalling - 1-3: Okay to be mandatory with capability signalling which shall be set to '1' - FGs 1-10, 2-2, 2-21, 2-22, 2-25: optional with capability signalling |

…

# Conclusion

…

# References

- R1-2003073, RAN1 UE features list for Rel-16 NR after RAN1#100bis-E, AT&T & NTT DOCOMO, INC.