3GPP TSG RAN WG1 #101			                                   					       												                  R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.11.3

Source:	Moderator (AT&T)

Title:	Summary of email discussion/approval [101-e-NR-IAB-UEFeatures-03]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [101-e-NR-IAB-UEFeatures-03] during RAN1 #101-e. According to the Chairman’s Notes:

[101-e-NR-IAB-UEFeatures-03] Email discussion/approval till 5/29 – Ralf (AT&T) - Whether to inform RAN4 that if the RAN4 RF/RRM features 1-2, 1-3, 1-4, 1-6, 1-7, 1-8, and 2-1 are made optional for IAB-MTs, there is no impact from a RAN1 perspective assuming that for features without capability signaling the parent node is made aware of the IAB-MT’s support or lack-of-support of the features (see R1-2004282) - Note: This email discussion/approval will also address any potential impact from R1-2003275 (“LS on RAN4 IAB-MT feature list agreement”) per the conclusions agreed in “RAN1#101-e preparation phase on LSs” by the RAN1 Chairman

The following was discussed and agreed during RAN1 #101-e within the scope of [101-e-NR-IAB-UEFeatures-03].

# Summary of email discussion/approval [101-e-NR-IAB-UEFeatures-03]

The following table represents the version of the NR UE feature list for IAB agreed by RAN1 during RAN1 #100bis-e Error: Reference source not found.

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20-2 | Inter-IAB-node discovery and measurements: SSB reception configuration | Support up to 4 SMTCs configured for an IAB node MT per frequency location, including IAB-specific SMTC window periodicities |  |  |  | Yes | N/A | Separate configuration of SMTC windows for Inter-IAB node discovery and measurement is not possible | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. [Devices supporting IAB backhaul must report this FG as supported] |
|  | 20-3 | Extension of RACH occasions and periodicities for backhaul RACH resources | Support RACH configuration for IAB-MT separately from the RACH configuration for UE access, including new IAB-specific offset and scaling factors |  |  |  | Yes | N/A | Separate configuration of RACH transmissions for access UEs and IAB nodes is not possible | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. [Devices supporting IAB backhaul must report this FG as supported] |
|  | 20-5a | UL-Flexible-DL slot formats | Support semi-static configuration/indication of UL-Flexible-DL slot formats for IAB-MT resources |  | 5-1a |  | Yes | N/A | Only Rel-15 slot formats can be configured for backhaul links | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
|  | 20-5b | UL-Flexible-DL slot formats | Support dynamic indication of UL-Flexible-DL slot formats for IAB-MT resources |  | 3-6 |  | Yes | N/A | Dynamic indication of UL-Flexible-DL slot formats for IAB-MT resources is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
|  | 20-6 | Dynamic indication of soft resource availability | Support monitoring DCI Format 2_5 scrambled by AI-RNTI for indication of soft resource availability to an IAB node |  |  |  | Yes | N/A | Explicit indication of soft resource availability is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. [Devices supporting IAB backhaul must report this FG as supported] |
|  | 20-7 | Case 1 OTA timing alignment | Support T_delta reception. |  |  |  | Yes | N/A | Case-1 OTA timing alignment is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. [Devices supporting IAB backhaul must report this FG as supported] |
|  | 20-8 | Guard symbols | 1) Support DesiredGuardSymbols reporting 2) Support ProvidedGuardSymbols reception |  |  |  | Yes | N/A | Guard symbols reporting and reception is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |

Companies are invited to provide their views in the following table:

- Whether to inform RAN4 that if the RAN4 RF/RRM features 1-2, 1-3, 1-4, 1-6, 1-7, 1-8, and 2-1 are made optional for IAB-MTs, there is no impact from a RAN1 perspective assuming that for features without capability signaling the parent node is made aware of the IAB-MT’s support or lack-of-support of the features (see R1-2004282)

  - Note: This email discussion/approval will also address any potential impact  from R1-2003275 (“LS on RAN4 IAB-MT feature list agreement”) per the conclusions agreed in “RAN1#101-e preparation phase on LSs” by the RAN1 Chairman

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | We agree to inform RAN4 that the mentioned features can be made optional for IAB-MTs with no impact from a RAN1 perspective, provided the parent node is made aware of respective features’ support or lack-of-support. |

…

# Conclusion

…

# References

- R1-2003073, RAN1 UE features list for Rel-16 NR after RAN1#100bis-E, AT&T & NTT DOCOMO, INC.