3GPP TSG RAN WG1 #101			                                   					       												        R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.11.6

Source:	Moderator (AT&T)

Title:	Summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-01]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-01] during RAN1 #101-e. According to the Chairman’s Notes:

[101-e-NR-eMIMO-UEFeature-01] Email discussion/approval till 5/29 – Ralf (AT&T) - Finalize all FGs in the 16-1 family of features as in x4285

The following was discussed and agreed during RAN1 #101-e within the scope of [101-e-NR-eMIMO-UEFeature-01].

# Summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-01]

This email discussion/approval aims to finalize all FGs in the 16-1 family of features. Open issues are highlighted in yellow [1] and summarized in [2] based on the contributions submitted to RAN1 #101-e. Additional aspects can be raised as part of this email discussion.

Companies are asked to prioritize the following:

  - Any change to the number of rows, i.e., deletion of rows, merging of rows, splitting of rows …

  - Any change to a component that impacts signalling design, e.g., because the component requires candidate values to be signalled incl. {enabled, disabled}

  - Any change to the type

  - Any change to xDD/FRx differentiation

  - Any change to whether the gNB needs to know if the feature is supported

  - Any change to whether capability exchange between UEs (V2X only) is applicable

  - Any change to a note that impacts signalling design, e.g., because a component requires candidate values to be signalled incl. {enabled, disabled}

The following should only be discussed if relevant or necessary in order to finalize aspects with ASN.1 impact:

  - Any change to a component that does not impact signalling design

  - Any change to a note that does not impact signalling design

  - Any change to whether a feature group is mandatory or optional

  - Any change to consequences if a feature is not supported by a UE

  - Any change to prerequisite feature groups for a feature

The following tables represent the latest version of the FGs in the 16-1 family of features [1]. Companies are invited to provide their views on how to finalize the respective FG in the 16-1 family of features in the following tables.

| 16-1a-1 | SSB/CSI-RS for L1-SINR measurement | - The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-1 - The max number of NZP-IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR (including CMR and IMR) within a slot shall not exceed MB_1-3 - The max number of SSB/CSI-RS (1Tx)/CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR (including CMR and IMR) shall not exceed MC_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-1 - The max number of NZP IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MC_1-3 - The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_2 - Supported density of CSI-RS (CMR) - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 | 2-21, 2-22 or 2-23, 2-23a |  |  |  | Per band |  |  |  |  | Optional with capability signaling Component-1 to 4, candidate value set for MB_1 and MB_1-x is {0, 8, 16, 32, 64} Component-5 to 8, candidate value set for MC_1 and MC_1-x is {0, 4, 8, 16, 32, 64} Component-9, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-10: candidate value set: {"not supported", "1 only", "3 only", "both 1 and 3"} Component-11, candidate value set for MD_2 is {0, 1, 4, 8, 16, 32, 64} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - In our view the current list is too detailed, and organization could be improved to make the FG definition more clear. For example, components 4, 5, and 9 are not necessary and they could be removed. Regarding the value ranges for the components, they should not include ‘0’ and ‘not supported’, as per the guidance received by RAN2. Moreover, in Rel-15 minimum mandatory values have been defined for similar components, we should avoid indicating unnecessary component values to RAN2 this time. |
| ZTE | Firstly, the motivation of distinguishing 1-Tx CSI-RS and 2-Tx CSI-RS is unclear, considering that we also have the component “10. Supported density of CSI-RS (CMR)”. Secondly, we only need to consider either the maximum number of CMR/IMR separately (i.e. components 1,2,3), or the maximum number of CMR+IMR together (i.e. components 4 and 5). We don’t need both of them. Consequently, we have the following proposals for revising the components in 16-1a-1. Specifically, we have the following suggestions. - The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-1 - The max number of NZP-IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR (including CMR and IMR) within a slot shall not exceed MB_1-3 - The max number of SSB/CSI-RS (1Tx)/CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR (including CMR and IMR) shall not exceed MC_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-1 - The max number of NZP IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR (including CMR and IMR) shall not exceed MC_1-3 - The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_2 - Supported density of CSI-RS (CMR) - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 |
| Qualcomm | - Similar to CSI-RS as CMR in component 9, we prefer to have a new component for max # of SSB across all CCs configured as CMR, i.e. having a separate capability on the combination of SSB as CMR + CSI-RS as IMR. - Because this combination is harder for implementation due to different periodicities of CMR and IMR. Because SSB as CMR has fixed periodicity, which can be different from IMR. At each reporting time, UE needs to pair the IMR occasion with the closest SSB occasion as CMR among all stored SSB measurements before the reporting time. We have no issue for CSI-RS as both CMR and IMR, which are expected to have same periodicity, and UE does not need to do this dynamic pairing for every L1-SINR computation. 12. The max number of SSB across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_3; Candidate value set for MB_3 is {0, 4, 8, 16, 32, 64} - Also, component 5 should be changed as below to match the counterpart in component 1 - The max number of SSB/CSI-RS (1Tx)/CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR (including CMR and IMR) shall not exceed MC_1 |

| 16-1a-2 | Non-group based L1-SINR reporting | - Support of non-group based L1-SINR reporting with N_max L1-SINR values reported | [16-1a-1] |  |  |  | Per band | [No] | [No] |  |  | [Optional with capability signaling] Candidate value set is {1, 2, 4} FFS: default value equals 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | FG is OK. It should be mandatory for a UE supporting 16-1a-1. |
| ZTE | We share the same views with Nokia |
| Qualcomm | Fine with the FG |

| 16-1a-3 | Group based L1-SINR reporting | - Support of group based L1-SINR reporting | 16-1a-1 |  |  |  | Per band | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | FG is OK |
| ZTE | Support |
| Qualcomm | Fine with the FG |

| 16-1b | TCI state activation and spatial relation update | - Support of Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH - FFS: details on whether/how to indicate band pairs which can share the same DL TCI state | Component 1: 2-1, 2-4 |  | N/A |  | [Per BC or per band] | No | Yes |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Firstly, the title of “16-1b: TCI state activation and spatial relation update” should be replaced by “16-1b: TCI state activation and spatial relation update”, considering that it can be only applied to DL transmission. Then, we can support the FFS part in principle. - Sharing the same TCI state or not is purely up to gNB configuration, but using different TCI state can be seen as a further UE capability enhancement over the Rel-15 UE that only supports single active TCI state for DL and/or UL transmission. Instead, we only need to discuss which bands can be used with different TCI state/spatial relation. For instance, we prefer to update the FFS components as “indicating the group of band pairs that can be indicated with different DL TCI states” or adding one more note that the UE can be indicated different DL TCI state(s) for the different group(s) of band pairs. Finally, Per BC is slightly preferred. |
| Qualcomm | - Support the “FFS” in both 16-1b and 16-1b-2 with the following rewording 16-1b: FFS: details on whether/how to indicate groups of bands pairs which can share the same DL spatial filter TCI state 16-1b-2: FFS: details on whether/how to indicate groups of bands pairs which can share the same UL spatial filter relation info - Motivation of the FFS - As background, the simultaneous beam update per CC list feature is motivated by the common UE analog beam, at least that is our initial motivation. In the initial RAN1 agreement (the one with 2 examples), we have the wording of “UE is expected to have same QCL-Type D across the CCs if indicating common analog beam for those CCs”. But we didn’t insist to mandate this in spec due to gNB vendors’ preference for flexibility. Here, we only propose for UE to indicate common beam for a group of bands to recommend gNB to use same Tx beam on them. - Use of this FFS in this FG - This FFS can provide recommendation at least from UE point of view on the choice of CC lists. For example, if UE reports 2 bands sharing same analog beam, then gNB can configure both bands in one CC list, and use one MAC-CE to update this common beam link. If UE reports 2 bands having separate analog beams, then gNB can configure two bands in two CC lists, and use two MAC-CEs to optimize the two beam links independently, since the best beam per band can be different due to different interference and frequency selective fading. The FFS informs gNB that it can exploit the two degrees of freedom at UE. - Both 16-1b and 16-1b-2 should be per UE - The support of simultaneous beam update across CCs should be per UE, not per band or per BC. - The FFS in component 2 should also be per UE, e.g. UE can report each group of band indexes sharing same analog beam. For example, group 1 has band 1 & 2, group 2 has band 3, 4, 5, and group 1 & 2 have different analog beams. The report of band group sharing same analog beam is only needed per UE. No need to report per BC to reduce signaling - Restrict 16-1b and 16-1b-2 only to FR2 - As mentioned before, this feature is originally motivated by the common UE analog beam, which only exists in FR2. For FR1, each CC has independent digital beam. Therefore, gNB ideally should use one MAC-CE per CC to optimize each beam per CC. So we didn’t see strong use case in FR1. |

| 16-1b-2 | Spatial relation update across multiple CCs | - Support of Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - FFS: details on whether/how to indicate band pairs which can share the same UL spatial relation info | Component 1: 2-59, 2-60 |  | N/A |  | [Per BC or per band] | No | Yes |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support the FFS part in principle. - Sharing the same spatial relation or not is purely up to gNB configuration, but using different spatial relation can be seen as a further UE capability enhancement over the Rel-15 UE that only supports single active spatial relation for UL transmission. Instead, we only need to discuss which bands can be used with differentspatial relation. For instance, we prefer to update the FFS components as “indicating the group of band pairs that can be indicated with different UL spatial relation info” or adding one more note that the UE can be indicated different UL spatial relation info for the different group(s) of band pairs. Then, Per BC is slightly preferred. |
| Qualcomm | Support the FFS. Please see our comments in 16-1b |

| 16-1b-3 | Spatial relation update for PUCCH group | - Support of PUCCH resource groups per BWP for simultaneous spatial relation update | 2-53, 2-59, 4-24 |  | N/A |  | [Per BC or per band] | No | Yes |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Per band is slightly preferred. |
| Qualcomm | Prefer per band |

| 16-1c | Default spatial relation | Support of default spatial relation and pathloss reference RS for dedicated-PUCCH/SRS and PUSCH [scheduled by DCI format 0_0] | [Component 1: 2-20,] 2-53, 2-59 |  | N/A |  | [Per band] | [No] | Yes |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | OK with this FG. But it should be per UE with FR2 differentiation. We do not see a need for adding other pre-requisite FGs in addition 2-53 and 2-59. |
| ZTE | In our MIMO-BM1, there are some related discussion for PUSCH scheduled by DCI format 0_1 also. Meanwhile, even though there are some restrictions for PUSCH, it also can be specified in RAN1 spec, rather than this FG, as we did for SRS herein (which should NOT be SRS for BM). Consequently, we prefer to remove “[scheduled by DCI format 0_0]”. Meanwhile, we also need to consider that this approach can be also used for default path-loss RS determination in FR1. Therefore, the last “Yes” should be replaced by “No”, in terms of FR1 and FR2 differentiation. |
| Qualcomm | Fine with the FG with per band |

| 16-1d | MAC CE spatial relation update for AP-SRS | [Support of / The maximum number of] spatial relation update for AP-SRS via MAC CE | 2-53, 2-59 |  | N/A |  | [Per UE] | No | [No] |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | The FG description should be for “Support of spatial relation updated for AP-SRS via MAC CE”. Type “Per UE”, with FRx differentiation. |
| ZTE | We share the same views with Nokia. |
| Qualcomm | Prefer to remove the “maximum number of”, and restrict this feature to FR2 only |

| 16-1e | Pathloss reference RS activation via MAC CE | - The maximum number of configured pathloss reference RSs for PUSCH/SRS by RRC for MAC-CE based pathloss reference RS update - FFS: The maximum number of activated pathloss reference RS update for PUSCH/SRS/PUCCH [across CCs / within a slot across all CCs / per CC] - FFS: Number of measurement samples N to apply newly activated pathloss reference RS | 8-2, 8-3 |  | N/A |  | [Per UE] | No | No |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | OK to remove component 3 to be aligned with RAN1 decisions. Component 2 is not needed as it is already part of intrinsic feature operation. Type can be per UE without differentiation. |
| ZTE | Support to remove both component 2 and component 3. - We prefer to have a fixed value for maximum number of activated pathloss reference RS(s) rather than according to UE capability. As a baseline, up to 4 activated pathloss reference RS(s) can be supported mandatorily without capability signaling, which is the same as in Rel-15. As a result, the component of “FFS: The maximum number of activated pathloss reference RS update for PUSCH/SRS/PUCCH [across CCs / within a slot across all CCs / per CC]” should be removed. |
| Qualcomm | For component 2, remove the “FFS” and select the option for “within a slot across all CCs”, since the time density is most critical to UE. |

| 16-1f | SCell beam failure recovery | - The maximum number of SCells [configured] for SCell beam failure recovery [simultaneously] - FFS: Support of PUCCH-BFR - FFS: The maximum number of CSI-RS and/or SSB resources for new beam identification of SCell BFR [across all CCs / within a slot across all CCs / per CC] - FFS: Density of CSI-RS for new beam identification for SCell BFR | 2-31 |  | N/A |  | FFS | No |  |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support to remove component 2 and component 4 with the following reasons. - Firstly, the support of PUCCH-BFR shall be mandatory with capability signaling for FR2 and optional for FR1 (as we did for PCell-BFR in Rel-15), in order to guarantee the effectiveness/low-latency of SCell-BFR. Hence, this component 5 in 16-1f should be removed, and the UE supporting the basic feature group FG 16-1f should support both SR based and PUCCH based-BFR. - Then, regarding the component “FFS: Density of CSI-RS for new beam identification for Scell BFR”, we only need to reuse Rel-15 UE capability, and consequently it can be removed. |
| Qualcomm | - For component 2, remove “FFS” for PUCCH-BFR, i.e. as a separate capability. Because PUCCH-BFR needs to have additional priority rule when overlapped with normal SRs - For component 3, remove the “FFS” and select the option for “within a slot across all CCs”, since the time density is most critical to UE. - For component 4, prefer to remove it, since R15 has no such capability, and the candidate beam should have no difference from R15 |

| 16-1g | Resources for beam management, [pathloss measurement, BFD, and BFR] | - The maximum number of SSB/CSI-RS/[CSI-IM] resources [within a slot] across all CCs for any of L1-RSRP measurement, L1-SINR measurement, [pathloss measurement, BFD, and new beam identification] - FFS: The maximum number of SSB/CSI-RS resources within a slot across all CCs for pathloss measurement - FFS: The maximum number of SSB/CSI-RS resources within a slot across all CCs for BFD - FFS: The maximum number of SSB/CSI-RS resources across all CCs for new beam identification |  |  | N/A |  | FFS | No |  |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support in principle - Component 2~4 should be removed - “Path-loss measurement” in title and component 1 should be moved, since path-loss measurement is NOT relevant to this FG. |
| Qualcomm | - For component 1, remove the bracket on “within a slot”, since time density is most critical to UE, and remove bracket on PL RS, BFD RS, and NBI, since total RS time density is the fundamental bottle neck. - Add a new component, same as component 1 but without “within a slot”. - For other component 2, 3, 4, they can be removed, since the total limits in component 1 & the new component are sufficient. |

…

# Conclusion

…

# References

- R1-2003194, Summary of Email Approval [100e-b-NR-UEFeatures-Remaining] — MIMO Enhancement Aspects, Moderator (AT&T)

- R1-2004285, Summary of UE features for eMIMO, Moderator (AT&T)