3GPP TSG RAN WG1 #101			                                   					       												        R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.11.6

Source:	Moderator (AT&T)

Title:	Summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-02]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-02] during RAN1 #101-e. According to the Chairman’s Notes:

[101-e-NR-eMIMO-UEFeature-02] Email discussion/approval till 5/29 – Ralf (AT&T) - Finalize all FGs in the 16-2 family of features as in x4285

The following was discussed and agreed during RAN1 #101-e within the scope of [101-e-NR-eMIMO-UEFeature-02].

# Summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-02]

This email discussion/approval aims to finalize all FGs in the 16-2 family of features. Open issues are highlighted in yellow [1] and summarized in Error: Reference source not found based on the contributions submitted to RAN1 #101-e. Additional aspects can be raised as part of this email discussion.

Companies are asked to prioritize the following:

  - Any change to the number of rows, i.e., deletion of rows, merging of rows, splitting of rows …

  - Any change to a component that impacts signalling design, e.g., because the component requires candidate values to be signalled incl. {enabled, disabled}

  - Any change to the type

  - Any change to xDD/FRx differentiation

  - Any change to whether the gNB needs to know if the feature is supported

  - Any change to whether capability exchange between UEs (V2X only) is applicable

  - Any change to a note that impacts signalling design, e.g., because a component requires candidate values to be signalled incl. {enabled, disabled}

The following should only be discussed if relevant or necessary in order to finalize aspects with ASN.1 impact:

  - Any change to a component that does not impact signalling design

  - Any change to a note that does not impact signalling design

  - Any change to whether a feature group is mandatory or optional

  - Any change to consequences if a feature is not supported by a UE

  - Any change to prerequisite feature groups for a feature

The following tables represent the latest version of the FGs in the 16-2 family of features [1]. Companies are invited to provide their views on how to finalize the respective FG in the 16-2 family of features in the following tables.

| 16-2a | Multi-DCI based multi-TRP | - The maximum number of CORESETs configured per “PDCCH-Config” - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” - The value of R=[1,2] for BD/CCE - [Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency] [for FR1] FFS whether default value of R is needed | FFS |  | N/A |  | [per band / per FSPC] | No | FFS |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - Component 3: with current definition FG 16-2a-10 is redundant and should be removed. Alternatively, one could rephrase it to “Support R= 1 for BD/CCE”, without associated capability signalling. - Component 4: remove brackets and delete “[for FR1]” - Per band indication |

| 16-2a-0 | Overlapping PDSCHs in time and non-overlapping in frequency | - Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency [merge with 16-2a? Merge with 16-2a-8?] [FR2 only?] |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - Rename the FG to “Overlapping PDSCHs in time and non-overlapping in frequency” - Replace current description with the following: - “1. Support fully overlapping PDSCHs in frequency and partially/fully overlapping in time” - “2. Support partially overlapping PDSCHs in frequency and partially/fully overlapping in time” - Pre-requisites: 16-2a, 16-2a8 (for FR2), 16-2a-1 |

| 16-2a-1 | Simultaneous reception of Multiple PDSCHs overlapping in frequency | - Support fully overlapping PDSCHs at frequency and partially/fully overlapping at time - Support partially overlapping PDSCHs in frequency and partially/fully overlapping in time - The maximum number of MIMO layers of each scheduled PDSCH [if PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain] - The maximal number of PDSCH scrambling sequences per serving cell FFS whether default values of component 3/4 to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - Delete components 1 and 2 - Remove square brackets from component 3 - Component 4: “Support two PDSCH scrambling sequences per serving cell” |

| 16-2a-2 | Out-of-order operation for DL | 1. Support out-of-order operation for PDCCH to PDSCH 2. Support out-of-order operation for PDSCH to HARQ-ACK |  |  |  |  |  |  |  |  | Note: this FG is a WA in RAN1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - FG is OK |

| 16-2a-3 | Out-of-order operation for UL | 1. Support out-of-order operation for PDCCH to PUSCH |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - FG is OK |

| 16-2a-4 | HARQ-ACK for multi-DCI based multi-TRP - separate | - Support of separate HARQ-ACK - [Support of two TDMed long PUCCHs in a slot] |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | Component 2 is unclear |

| 16-2a-4a | HARQ-ACK for multi-DCI based multi-TRP - joint | - Support of joint HARQ-ACK |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | FG is OK |

| 16-2a-5 | Separate CRS rate matching | Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-2a-6 | Default QCL enhancement for multi-DCI based multi-TRP | Support of default QCL assumption per CORESETPoolIndex |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| 16-2a-6 | FG is OK |

| 16-2a-7 | Maximum number of activated TCI states | - Support of maximum per CORESETPoolIndex - Support of total maximum FFS whether default values to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - Clarify if this means the UE supports the maximum defined in specs or is the intention to indicate supported values - Remove “FFS whether default values to be included in 16-2a” |

| 16-2a-8 | Simultaneous reception with different Type-D | whether UE supports receiving time-overlapping [PDSCHs]/PDCCHs with different Type-D FFS whether default values to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | Remove “FFS whether default values to be included in 16-2a” as the capability is a yes/no indication anyway |

| 16-2a-9 | [Simultaneous reception across CCs with Multi-DCI] | [The maximum number of CCs supporting multi-DCI based multi-TRP simultaneously] |  |  |  |  |  |  |  |  | Note: If the type of 16-2a is agreed to be FSPC or FS this FG will be removed |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - It is OK to confirm this FG |

| 16-2a-10 | Value of BD factor | R=[(1,2] for BD/CCE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | As mentioned before this FG is not really needed, but alternatively it can be rephrased to “Support R= 2 for BD/CCE”. |

| 16-2b-0 | Two default beams for single-DCI based multi-TRP | Support of default QCL assumption with two TCI states | 16-2b |  | N/A |  | Per band | TDD only | FR2 only |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-2b-1 | Single-DCI based SDM scheme | - FFS: Support of DCI indication of of 2 TCI states by a codepoint and DMRS ports within two CDM groups - Support of two PTRS ports - FFS Support of DMRS entry {0, 2, 3} | 16-2b, FFS |  | N/A |  | Per band | No | FFS |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-2b-2 | Single-DCI based FDMSchemeA | Support of FDMSchemeA | 16-2b, FFS |  | N/A |  | Per band | No | FFS |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-2b-3 | Single-DCI based FDMSchemeB | - Support of FDMSchemeB - For FDMSchemeB, whether the UE can support CW soft combining | 16-2b, FFS |  | N/A |  | [per FSPC] | No | FFS |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-2b-4 | Single-DCI based TDMSchemeA | - Support of TDMSchemeA - Supported maximum TBS size for TDMSchemeA | 16-2b, FFS |  | N/A |  | Per band | No | FFS |  | Component 2 candidate values {10 CBs, FFS} | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-2b-5 | Single-DCI based inter-slot TDM | - Support of MAC CE to activate two TCI states for a TCI codepoint - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size according to RepNumR16 in PDSCH-TimeDomainResourceAllocation - FFS: TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping) | 16-2b, FFS |  | N/A |  | Per band | No | FFS |  | Component 1 candidate values: {8,16} Component 2 candidate values {10 CBs, FFS} | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

…

# Conclusion

…

# References

- R1-2003194, Summary of Email Approval [100e-b-NR-UEFeatures-Remaining] — MIMO Enhancement Aspects, Moderator (AT&T)

- R1-2004285, Summary of UE features for eMIMO, Moderator (AT&T)