3GPP TSG RAN WG1 #101			                                   					       												        R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.11.6

Source:	Moderator (AT&T)

Title:	Summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-02]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-02] during RAN1 #101-e. According to the Chairman’s Notes:

[101-e-NR-eMIMO-UEFeature-02] Email discussion/approval till 5/29 – Ralf (AT&T) - Finalize all FGs in the 16-2 family of features as in x4285

The following was discussed and agreed during RAN1 #101-e within the scope of [101-e-NR-eMIMO-UEFeature-02].

# Summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-02]

This email discussion/approval aims to finalize all FGs in the 16-2 family of features. Open issues are highlighted in yellow [1] and summarized in Error: Reference source not found based on the contributions submitted to RAN1 #101-e. Additional aspects can be raised as part of this email discussion.

Companies are asked to prioritize the following:

  - Any change to the number of rows, i.e., deletion of rows, merging of rows, splitting of rows …

  - Any change to a component that impacts signalling design, e.g., because the component requires candidate values to be signalled incl. {enabled, disabled}

  - Any change to the type

  - Any change to xDD/FRx differentiation

  - Any change to whether the gNB needs to know if the feature is supported

  - Any change to whether capability exchange between UEs (V2X only) is applicable

  - Any change to a note that impacts signalling design, e.g., because a component requires candidate values to be signalled incl. {enabled, disabled}

The following should only be discussed if relevant or necessary in order to finalize aspects with ASN.1 impact:

  - Any change to a component that does not impact signalling design

  - Any change to a note that does not impact signalling design

  - Any change to whether a feature group is mandatory or optional

  - Any change to consequences if a feature is not supported by a UE

  - Any change to prerequisite feature groups for a feature

The following tables represent the latest version of the FGs in the 16-2 family of features [1]. Companies are invited to provide their views on how to finalize the respective FG in the 16-2 family of features in the following tables.

| 16-2a | Multi-DCI based multi-TRP | - The maximum number of CORESETs configured per “PDCCH-Config” - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” - The value of R=[1,2] for BD/CCE - [Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency] [for FR1] FFS whether default value of R is needed | FFS |  | N/A |  | [per band / per FSPC] | No | FFS |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - Component 3: with current definition FG 16-2a-10 is redundant and should be removed. Alternatively, one could rephrase it to “Support R= 1 for BD/CCE”, without associated capability signalling. - Component 4: remove brackets and delete “[for FR1]” - Per band indication |
| Ericsson | - Component 3 can be removed as it is a separate FG in 16-2a-10. - For Component 4, remove the brackets around ‘Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency’. Note that this is one of the basic features for multi-dci based multi-TRP scheduling. If this feature is removed, then only TDM based scheduling can be done which will result in throughput degradation. We can support this feature for FR1 as part of 16-2a. - “The maximal number of PDSCH scrambling sequences per serving cell” currently captured under 16-2a-1 is a better fit as part of 16-2a. Hence, we suggest to move “The maximal number of PDSCH scrambling sequences per serving cell” from 16-2a-1 to 16-2a. - We prefer Per band indication. |
| Apple | - Component 3: This can be removed since it is overlapping with FG16-2a-10 - Component 4: for overlapping type of PDSCH, it should be - Non-overlapping RE and Non-overlapping in time - We need to discuss the limitation on MIMO layers, i.e. whether to reuse per FSPC maxNumberMIMO-LayersPDSCH - This FG is “optional with capability signally” |
| LG | - Component 3 can be removed as it is a separate FG in 16-2a-10. - Component 4 can be removed and merged with 16-2a-0. |
| Spreadtrum | - Component 3 is redundant for it has been a separate FG 16-2a-10. - ‘FFS whether default value of R is needed’ could be removed. - Component 4 could be deleted and merged into FG 16-2a-0. - This FG is optional. |
| NTT DOCOMO | - Component 3 can be removed since it is a separate FG in FG16-2a-10. - For component 4, remove the bracket. - Prefer per band indication. |
| QC | - The type of the FG should be per FSPC. Multi-DCI is a complicated feature from UE implementation point of view. FSPC allowed different UE implementation choices including using CA capability. The UE should be able to indicate the support of this feature per-CC of per-band and per-band-combination. Note that the FG 16-2a-9 cannot achieve the required flexibility that is needed for the UE to indicate the support of multi-DCI based mTRP. A 20MHz CC does not have the same complexity as a 400MHz CC. In addition, the number of CCs that are supported with multi-DCI depends on a band combination support. For example, in a first band combination where UE indicates support of 8 CCs, a larger number of CCs can be supported with multi-DCI based mTRP compared to a second band combination where UE indicates support of 16 CCs. Hence, FG 16-2a-9 should be removed while the type of FG 16-2a can be kept per FSPC. - We propose to add two components to the FG for “PDSCH processing capability” and for “maximum number of unicast PDSCHs per CORESETPoolIndex per slot”. This is because the UE should be able to indicate the support of multi-DCI feature per-CC of per-band and per-band-combination in the presence/absence of other features that impact the complexity (such as processing capability, or number of TBs per slot). Hence we propose to add two components to the basic multi-DCI support. Regarding “maximum number of unicast PDSCHs per CORESETPoolIndex per slot”, it should be noted that this is aligned with all the other features in multi-DCI based multi-TRP that is defined per CORESETPoolIndex value (e.g. # of CORESETs, # BD/CCE limit, etc.) - Component 3 can be removed as it this is now a new FG (FG 16-2a-10). |
| OPPO | - Prefer to remove component 3 - Prefer to describe component 4 by “overlapping PDSCHs in time with non-overlapping REs” - Optional with capability signaling |
| Intel | - Component 4: Remove all brackets - Prefer per band or per band per band combination signaling - Component 3 can be removed |
| ZTE | - Component 3 should be removed - Confirm component 4: Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency - One new component is needed to indicate the maximum number of PDSCHs across two TRPs in the same slot per CC. The candidates can be {2, 4, 8, 14}. - The reason is in NR Rel-15, for single TRP transmission, a UE capability signaling pdsch-ProcessingType1-DifferentTB-PerSlot to report the maximum number of TDMed PDSCHs in a slot, where the candidates include 2, 4 and 7. In case of multi-DCI based MTRP, PDSCHs scheduled by different TRPs in a slot can overlap in the time domain. It is natural to support more PDSCHs in one slot per CC. |

| 16-2a-0 | Overlapping PDSCHs in time and non-overlapping in frequency | - Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency [merge with 16-2a? Merge with 16-2a-8?] [FR2 only?] |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - Rename the FG to “Overlapping PDSCHs in time and non-overlapping in frequency” - Replace current description with the following: - “1. Support fully overlapping PDSCHs in frequency and partially/fully overlapping in time” - “2. Support partially overlapping PDSCHs in frequency and partially/fully overlapping in time” - Pre-requisites: 16-2a, 16-2a8 (for FR2), 16-2a-1 |
| Ericsson | - This feature group name needs to be changed to ‘PDSCHs with full Overlapping in frequency and full overlapping in time (fully overlapping REs)’. - A first component for this FG is ‘Support fully overlapping PDSCHs in frequency and fully overlapping in time (fully overlapping REs)’. - A second component for this FG is ‘The maximum number of MIMO layers of each scheduled PDSCH’. The text with ‘[if PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain]’ can be removed. - 16-2a is a prerequisite for this FG. - The feature type is ‘per band’. |
| Apple | - We are fine to have “the maximal number of PDSCH scrambling sequences per serving cell” or move to FG16-2a - For the other 2 overlapping types of PDSCH, we prefer to have - Fully-overlapping PDSCH in RE - All REs in PDSCH 1 overlap with PDSCH 2 in RE and All REs in PDSCH 2 overlap with PDSCH 1 in RE - Partially-overlapping PDSCH in RE - At least one RE in PDSCH 1 does not overlap with PDSCH 2 in RE or at least one RE in PDSCH 2 does not overlap with PDSCH 1 in RE |
| LG | - For the first component, remove “[merge with 16-2a? Merge with 16-2a-8?] [FR2 only?]” and restrict this FG for FR 1. For FR 2, we have another FG 16-2a-8 for time domain overlapping. |
| Spreadtrum | - Remove “[merge with 16-2a? Merge with 16-2a-8?] [FR2 only?]”. FG 16-2a-0 is more related to processing capability while FG 16-2a-8 is more related to reception. They are different. |
| NTT DOCOMO | - Component 1 can be merged with 16-2a-8 as time-domain overlapping in FR2 is a separate FG in FG16-2a-8. |
| QC | - Remove at least “Merge with 16-2a-8?] [FR2 only?]”. Agree with LG that we have another FG 16-2a-8 for time domain overlapping in FR2. |
| OPPO | - This feature group can be renamed to “support PDSCHs with fully overlapping REs” and modify corresponding description. - “The maximal number of PDSCH scrambling sequences per serving cell” can be one of the component - “The maximum number of MIMO layers of each scheduled PDSCH” can be one of the component |
| Intel | - Clarify what is “fully overlapping” – can we say same PRBs and same OFDM symbols (its allowed that the exact PDSCH REs are not equal for PDSCH-1 and PDSCH-2) - Clarify whether this is applicable for FR1 or FR2 or both. - We could use this FG for fully/partially overlapping in time and frequency in FR1 |
| ZTE | - This FG should be removed since it is the same as component 4 of FG 16-2a. |

| 16-2a-1 | Simultaneous reception of Multiple PDSCHs overlapping in frequency | - Support fully overlapping PDSCHs at frequency and partially/fully overlapping at time - Support partially overlapping PDSCHs in frequency and partially/fully overlapping in time - The maximum number of MIMO layers of each scheduled PDSCH [if PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain] - The maximal number of PDSCH scrambling sequences per serving cell FFS whether default values of component 3/4 to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - Delete components 1 and 2 - Remove square brackets from component 3 - Component 4: “Support two PDSCH scrambling sequences per serving cell” |
| Ericsson | - This feature group name needs to be changed to ‘PDSCHs with partial overlapping frequency and full/partial overlapping in time (partially overlapping REs)’. - A first component for this FG is ‘Support partially overlapping PDSCHs in frequency and partially/fully overlapping in time (partially overlapping REs)’. - A second component for this FG is ‘The maximum number of MIMO layers of each scheduled PDSCH’. The text with ‘[if PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain]’ can be removed. - 16-2a is a prerequisite for this FG. - The feature type is ‘per band’. |
| Apple | The same comment as 16-2a-0. All the compoents can be removed |
| OPPO | This feature group can be renamed to “support PDSCHs with partially overlapping REs” and modify corresponding description if we use 16-2a-0 to support fully overlapping REs. |
| Intel | Delete 1, 2 For component 3, some layer combinations can be optional depending on maxMIMO-Layers. Also interpret Rel-15 maxMIMO-Layers as the total number of layers across PDSCH in a slot. |
| ZTE | - Remove component 4. UE should support two scrambling ID if it supports M-DCI based MTRP where CORESET pool index and scrambling ID is one-to-one mapping. |

| 16-2a-2 | Out-of-order operation for DL | 1. Support out-of-order operation for PDCCH to PDSCH 2. Support out-of-order operation for PDSCH to HARQ-ACK |  |  |  |  |  |  |  |  | Note: this FG is a WA in RAN1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - FG is OK |
| Ericsson | - Ok to confirm WA. - we prefer the feature type to be ‘Per UE’ |
| Apple | Okay |
| Spreadtrum | OK |
| QC | Ok |
| OPPO | OK |
| Intel | OK |
| ZTE | OK |

| 16-2a-3 | Out-of-order operation for UL | 1. Support out-of-order operation for PDCCH to PUSCH |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - FG is OK |
| Ericsson | - we are ok with the FG. - Our preference for feature type is ‘per UE’ |
| Apple | Okay |
| Spreadtrum | OK |
| QC | We propose to add another FG that is related to this FG as shown below: 16-2a-3i; Out-of-order operation for UL with same closed loop index; 1. Support out-of-order operation for PDCCH to PUSCH with same closed loop index for power control across PUSCHs associated with different CORESETPoolIndex values; 16-2a, 16-2a-3; Per band. The reason is explained in our Tdoc with a Figure, and is summarize here: When out-of-order operation for PDCCH to PUSCH is allowed, and in the case that two closed loop indexes are not configured for PUSCH transmissions corresponding to the two values of CORESETPoolIndex, the ordering in which TPC commands in the DCI should be considered for each PUSCH transmission requires more complex handling at the UE side. These additional complexities are taken care of if the network uses different closed loop indices for PUSCHs corresponding to different CORESETPoolIndex values. |
| OPPO | Support additional FG for “OOO for UL with same close loop index” |
| Intel | OK |
| ZTE | OK |

| 16-2a-4 | HARQ-ACK for multi-DCI based multi-TRP - separate | - Support of separate HARQ-ACK - [Support of two TDMed long PUCCHs in a slot] |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | Component 2 is unclear |
| Ericsson | We prefer the feature type to be ‘per UE’. |
| Apple | - Component 2: We prefer to split into the following 3 cases. Note that Rel-15 does not support more than 1 HARQ-ACK in a slot - Two TDMed long PUCCHs within a slot for HARQ-ACK - TDMed short PUCCH and long PUCCH within a slot for HARQ-ACK - TDMed short PUCCH and short PUCCH within a slot for HARQ-ACK |
| Spreadtrum | - For component 2, share the same view with Apple |
| QC | - Type should be per band. - Component 2: A UE that supports separate HARQ-Ack feedback for multi-DCI based mTRP may not support two long PUCCHs within one slot. Hence, we propose to change the second component to maximum number of long PUCCHs within one slot for more flexible signalling so that UE can indicate either 1 or 2. 16-2a-4; HARQ-ACK for multi-DCI based multi-TRP - separate; - Support of separate HARQ-ACK - Supported maximum number of long PUCCHs within a slot for separate HARQ-Ack; 16-2a; Per band; Component 2 candidate values: {1, 2}. |
| OPPO | - Support to split component 2 into 3 cases. |
| ZTE | Support this FG. |

| 16-2a-4a | HARQ-ACK for multi-DCI based multi-TRP - joint | - Support of joint HARQ-ACK |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | FG is OK |
| Ericsson | We prefer the feature type to be ‘per UE’. |
| Apple | Okay |
| Spreadtrum | OK |
| QC | Ok. Type should be per band. |
| OPPO | OK |
| Intel | OK |
| ZTE | OK |

| 16-2a-5 | Separate CRS rate matching | Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | We prefer the feature type to be ‘per UE’. |
| Apple | - This FG is “optional with capability signalling” - UE can report {union, union + separate} - Union: UE rate matches around all configured CRS patterns in the scheduled cell - Separate: UE rate matches around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex |
| QC | Ok. Type should be per band. |
| OPPO | - “whether the UE shall rate…” can be modified to “UE can rate…” |
| ZTE | - Support this FG |

| 16-2a-6 | Default QCL enhancement for multi-DCI based multi-TRP | Support of default QCL assumption per CORESETPoolIndex |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| 16-2a-6 | FG is OK |
| Ericsson | - The feature type is ‘per band’. - The FR1/FR2 differentiation column can be set to ‘FR2 only’ |
| Apple | Okay |
| LG | 16-2a-8 is prerequisite feature groups of 16-2a-6. |
| QC | Agree with LG and Ericsson. |
| OPPO | OK |
| ZTE | Support this FG |

| 16-2a-7 | Maximum number of activated TCI states | - Support of maximum per CORESETPoolIndex - Support of total maximum FFS whether default values to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - Clarify if this means the UE supports the maximum defined in specs or is the intention to indicate supported values - Remove “FFS whether default values to be included in 16-2a” |
| Ericsson | - For component 1, we are ok to have component values of {1, 2, 4, 8} - For component 2, we are ok to have component values of {2, 4, 8, 16} |
| Apple | - Component 1: similar as FG2-4, this is per CC, {1, 2, 4, 8} - Component 2: {1, 2, 4, 8, 16} - This is per band reporting similar as FG2-4 |
| QC | - It should be clarified that both components represent the maximum number of active TCI states for both data and control, which is the similar to the Rel. 15 FG 2-4. - For component 2, the value cannot be arbitrary, e.g., it cannot be smaller than component 1. However, it is possible that the value is either the same as component 1 or it is twice as the value indicated by component 1, i.e. {same as component 1, twice of component 1} |
| OPPO | - Fine to have {1,2,4,8} and {2,4,8,16} as candidates for component 1 and component 2. |
| ZTE | - Component 1: {1, 2, 4, 8} - Component 2: { 2, 4, 8, 16} |

| 16-2a-8 | Simultaneous reception with different Type-D | whether UE supports receiving time-overlapping [PDSCHs]/PDCCHs with different Type-D FFS whether default values to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | Remove “FFS whether default values to be included in 16-2a” as the capability is a yes/no indication anyway |
| Ericsson | - Add a second component ‘Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency for FR2’ - Remove ‘[PDSCHs]’ from the first component. - The feature type is ‘per band’. |
| Apple | This is the same as group based reporting |
| LG | Square brackets should be removed. As we mentioned in 16-2a-0, time domain overlapping PDSCH for FR 1 and FR 2 needs to be defined as separate feature group. |
| NTT DOCOMO | Remove the bracket around PDSCHs. |
| QC | - Remove the bracket for PDSCH (also fine with Ericsson’s suggestion). - This should be per band and applicable to FR2 only. |
| OPPO | If 16-2a,16-2a-0 and 16-2a-1 are per band reporting, we can delete PDSCH here since UE would report the support of 16-2a,16-2a-0 and 16-2a-1 for FR2 band only when it supports simultaneous reception with different Type-D. Otherwise, the brackets should be removed and FR1 and FR2 can have different capabilities. |
| Intel | This FG should not be overlapping with FG 16-2a-0 (after removing brackets around PDSCH) Clarify overlap of PDSCH in frequency domain (is it non/partially/fully overlapping) |
| ZTE | Confirm this FG: whether UE supports receiving time-overlapping [PDSCHs]/PDCCHs with different Type-D |

| 16-2a-9 | [Simultaneous reception across CCs with Multi-DCI] | [The maximum number of CCs supporting multi-DCI based multi-TRP simultaneously] |  |  |  |  |  |  |  |  | Note: If the type of 16-2a is agreed to be FSPC or FS this FG will be removed |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - It is OK to confirm this FG |
| Apple | It depends on whether 16-2a is per FSPC |
| QC | Delete the FG. |

| 16-2a-10 | Value of BD factor | R=[(1,2) for BD/CCE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | As mentioned before this FG is not really needed, but alternatively it can be rephrased to “Support R= 2 for BD/CCE”. |
| Ericsson | We prefer the feature type for this FG to be ‘per UE’ |
| Apple | Okay |
| QC | The FG should be per band combination. It is possible that the UE supports R=2 for a given band combination (e.g. with smaller number of total supported CCs) while it supports R=1 for a different band combination (e.g. with larger number of total supported CCs). |
| OPPO | OK to have it to be per UE. |
| ZTE | OK |

| 16-2b-0 | Two default beams for single-DCI based multi-TRP | Support of default QCL assumption with two TCI states | 16-2b |  | N/A |  | Per band | TDD only | FR2 only |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Remove 16-2b from prerequisite as 16-2b does not exist. |
| Apple | Okay |
| Spreadtrum | - This FG is optional - Remove 16-2b from prerequisite |
| NTT DOCOMO | Agree with Ericsson and spreadtrum, remove 16-2b from prerequisite. |
| QC | Ok |
| ZTE | Agree with Ericsson |

| 16-2b-1 | Single-DCI based SDM scheme | - FFS: Support of DCI indication of of 2 TCI states by a codepoint and DMRS ports within two CDM groups - Support of two PTRS ports - FFS Support of DMRS entry {0, 2, 3} | 16-2b, FFS |  | N/A |  | Per band | No | FFS |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Remove 16-2b from prerequisite as 16-2b does not exist. |
| Apple | - Component 1: If DCI indicating 2 TCI states in a TCI codepoint is captured as basic component, it should be consistently captured for the other schemes including FDMSchemeA, FDMSchemeB and TDMSchemeA, i.e., FG 16-2b-2/3/4 - Component 2: Based on previous agreement, support of 2 port PTRS should be optional, we need to find a way to correct it such as use bitmap, i.e. {1 port PTRS, 2 port PTRS} |
| LG | Component 1 can be removed because 16-2b-1 already assumes that component 1 is supported. We slightly prefer to remove component 3 because it is hard to find reason of that component. |
| NTT DOCOMO | Remove FFS in component 1 and component 3. Remove 16-2b from prerequisite. |
| QC | - As commented by Apple, support of two PTRS ports should be optional. UE should be able to indicate support of SDM scheme without support of two PTRS ports, as already agreed. This can be achieved by adding a new FG or by using a bitmap. - Support of the new DMRS port {0,2,3} should be a separate capability. This entry is added for support of 1+2 layers. However, this feature is not essential to SDM scheme as rank combination 1+2 can be achived even with the Rel. 15 DMRS entry {0,1,2} as long as one TCI codepoint is used for (TCI state x, TCI state y) and another for (TCI state y, TCI state x) |
| OPPO | - We are fine to remove component 1 or add it to all the schemes. - The FFS for component 3 can be removed. - Component 2 should be separate optional FG or be modified to be “supported maximal number of PTRS ports”. |
| Intel | - Prefer to remove Component 3 and remove FFS from component 1 |
| ZTE | - Component 2: two PTRS ports should be optional as what we agreed. Then this FG can be further split into two FGs, or we should explicitly introduce one reporting bit for component 2. |

| 16-2b-2 | Single-DCI based FDMSchemeA | Support of FDMSchemeA | 16-2b, FFS |  | N/A |  | Per band | No | FFS |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Remove 16-2b from prerequisite as 16-2b does not exist. |
| Apple | Okay |
| NTT DOCOMO | Remove 16-2b from prerequisite. |
| QC | Ok |
| OPPO | OK |
| ZTE | OK |

| 16-2b-3 | Single-DCI based FDMSchemeB | - Support of FDMSchemeB - For FDMSchemeB, whether the UE can support CW soft combining | 16-2b, FFS |  | N/A |  | [per FSPC] | No | FFS |  |  | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Remove 16-2b from prerequisite as 16-2b does not exist. |
| Apple | - Component 2: Based on previous agreement, CW soft combining should be optional, we need to find a way to correct it such as use bitmap, i.e. {CW hard combining, CW soft combining} |
| NTT DOCOMO | Remove 16-2b from prerequisite. |
| QC | - Agree with Apple. We can either add a new FG or use bitmap. - For FDMSchemeB, the type should be per FSPC. This is because the UE may need to use two receive processing chains to demodulate the two copies at the same time, which is similar to the case of two overlapping PDSCHs in multi-DCI from UE processing point of view. If the type is per FSPC, it can address different UE implementation choices as the UE can indicate in which CC of a band of a band combination this feature is supported. |
| OPPO | Agree with Apple that CW soft combining should be optional for scheme 2b. Maybe we need a separate FG for it or use bitmap signaling. |
| ZTE | - Support this FG. For component 2, we agree with Apple. |

| 16-2b-4 | Single-DCI based TDMSchemeA | - Support of TDMSchemeA - Supported maximum TBS size for TDMSchemeA | 16-2b, FFS |  | N/A |  | Per band | No | FFS |  | Component 2 candidate values {10 CBs, FFS} | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Remove 16-2b from prerequisite as 16-2b does not exist. |
| Apple | Okay |
| LG | Component 2 doesn’t seem to have special meaning. This is because available REs for TDMSchemeA may be limited, so that large TB size cannot be supported regardless of this component. For example, when 7 symbols with one DMRS symbol, 100 RBs, and 2 layers are assumed for a single TO, maximum 14400 REs can be scheduled to UE. In this case, 86400 bits can be scheduled when 64QAM and coding rate 1 are assumed. As a result, limitation on payload size for TDMSchemeA (e.g., 10Kbytes) seems a corner case. So this component can be removed. |
| NTT DOCOMO | Remove 16-2b from prerequisite. |
| QC | Ok. Regarding the comment from LG: This depends on the choice of candidate value, e.g., 10 CBs (code blocks) can be easily exceeded in the example (also, # of RBs can be larger than 100). In any case, this is already agreed. |
| OPPO | We are fine to leave component 2 as it is. |
| ZTE | OK |

| 16-2b-5 | Single-DCI based inter-slot TDM | - Support of MAC CE to activate two TCI states for a TCI codepoint - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size according to RepNumR16 in PDSCH-TimeDomainResourceAllocation - FFS: TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping) | 16-2b, FFS |  | N/A |  | Per band | No | FFS |  | Component 1 candidate values: {8,16} Component 2 candidate values {10 CBs, FFS} | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | - Remove ‘Support of MAC CE to activate two TCI states for a TCI codepoint’ as this feature can also work for single TRP. - Remove last FFS ‘TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping)’ as this does not need to be a UE capability. - Remove 16-2b from prerequisite as 16-2b does not exist. |
| Apple | - Component 1: Inter-slot TDM has single TCI fall back mode to operate as DPS, therefore, strictly speaking, it is not a basic component for UE to support MAC CE to activate two TCI states for a TCI codepoint or DCI to indicate two TCI states |
| LG | Regarding component 2, {2,3,4,5,6,7,8,16} can be candidate values based on the previous agreement. Regarding component 3, the same maximum TB size can be used regardless of RepNumR16, so that component 3 can be modified as “Supported maximum TBS size according to RepNumR16 in PDSCH-TimeDomainResourceAllocation”. And FFS for the candidate values can be removed. Component 4 can be removed. |
| Spreadtrum | - For component 1, it could be removed for scheme 4 supporting fall back to DPS operation. - For component 4: given that the following agreement in RAN1#98b, as the highlight shows that whether both or one of the options is UE optional or not could be further discussed. In our opinion, cyclical mapping would require UE to switch beam per slot, to some degree, it will increase the UE complexity. In addition, the benefit for cyclical mapping is not clear. Thus, at least cyclical mapping should be optional. But if the majority think it could be removed, we are fine. Agreement(RAN1#98b) For single-DCI based M-TRP URLLC scheme 4, for TCI state mapping to PDSCH transmission occasions, - Both options 1 and 2 are supported and switched by RRC signalling - Option 1: support Cyclical mapping, e.g. TCI states #1#2#1#2 are mapped to 4 transmission occasions if 2 TCI stats are indicated - Option 2: support Sequential mapping, e.g. TCI states #1#1#2#2 are mapped to 4 transmission occasions if 2 TCI stats are indicated - For more than 4 transmission occasions, above is repeated (for example, 8 transmission occasion in case of option 2: #1#1#2#2#1#1#2#2) - FFS: The mapping between RV sequence and transmission occasions if the offset between the DCI and scheduled PDSCH is less than the threshold - FFS: Whether both or one of the options is UE optional or not. |
| NTT DOCOMO | Remove component 4. Remove 16-2b from prerequisite. |
| OPPO | - For component 1, it can be modified to be “the supported number of activated TCI states for a TCI codepoint by MAC CE” with candidates of {1,2}, since UE may only support scheme 4 with single TRP transmission. - For component 4, we prefer to remove FFS. UE doesn’t need to support both mapping patterns. |
| Intel | Remove component 4 |
| ZTE | Support this FG. We are fine to remove component 4. |

…

# Conclusion

…

# References

- R1-2003194, Summary of Email Approval [100e-b-NR-UEFeatures-Remaining] — MIMO Enhancement Aspects, Moderator (AT&T)

- R1-2004285, Summary of UE features for eMIMO, Moderator (AT&T)