3GPP TSG RAN WG1 #101			                                   					       												        R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.11.6

Source:	Moderator (AT&T)

Title:	Summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-03]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-03] during RAN1 #101-e. According to the Chairman’s Notes:

[101-e-NR-eMIMO-UEFeature-03] Email discussion/approval till 5/29 – Ralf (AT&T) - Finalize all FGs in the 16-3, 16-7, and 16-8 families of features as in x4285

The following was discussed and agreed during RAN1 #101-e within the scope of [101-e-NR-eMIMO-UEFeature-03].

# Summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-03]

This email discussion/approval aims to finalize all FGs in the 16-3, 16-7, and 16-8 families of features. Open issues are highlighted in yellow [1] and summarized in Error: Reference source not found based on the contributions submitted to RAN1 #101-e. Additional aspects can be raised as part of this email discussion.

Companies are asked to prioritize the following:

  - Any change to the number of rows, i.e., deletion of rows, merging of rows, splitting of rows …

  - Any change to a component that impacts signalling design, e.g., because the component requires candidate values to be signalled incl. {enabled, disabled}

  - Any change to the type

  - Any change to xDD/FRx differentiation

  - Any change to whether the gNB needs to know if the feature is supported

  - Any change to whether capability exchange between UEs (V2X only) is applicable

  - Any change to a note that impacts signalling design, e.g., because a component requires candidate values to be signalled incl. {enabled, disabled}

The following should only be discussed if relevant or necessary in order to finalize aspects with ASN.1 impact:

  - Any change to a component that does not impact signalling design

  - Any change to a note that does not impact signalling design

  - Any change to whether a feature group is mandatory or optional

  - Any change to consequences if a feature is not supported by a UE

  - Any change to prerequisite feature groups for a feature

The following tables represent the latest version of the FGs in the 16-3, 16-7, and 16-8 families of features [1]. Companies are invited to provide their views on how to finalize the respective FG in the 16-3, 16-7, and 16-8 families of features in the following tables.

| 16-3a | Regular eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=1 - Support of parameter combinations 1-6 - Support of rank 1,2 - FFS: CBSR with hard amplitude restriction | FFS |  | N/A |  | FFS: Per band or Per band per BC | No | No |  |  | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - On the feature type follow same approach as for Rel-15 in order to avoid future underreporting issues. - Component 4: no need for this component if the description below is adopted for 16-3a-4, as CBSR with beam restriction specified in 38.214 is not subject to UE capability |
| ZTE | “FFS” in the 4-th component should be removed per previous agreement. It should at least be per band. To make it per band AND per BC is better. It’s better to clarify only the 1-st component requires signaling. |
| CATT | - The type should be per band and per BC to avoid under-reporting problem discussed in past RAN1 meetings. - Either removing component #4 or just removing ‘FFS:’ is fine, as CBSR with hard amplitude restriction was agreed to be mandatory feature. - The fields ‘Need of FDD/TDD differentiation’ and ‘Need of FR1/FR2 differentiation’ shall be set to ‘N/A’. |
| Apple | - Similar as FG2-36/40/41/43, it is both per band and per BC, no for xDD and FRx differentiation - We need to discuss the dependency of the parameter setting on the triplets UE reported - For example, similar as Rel-15, UE can report the supported maximum L for each number of ports of CSI-RS for CMR. |
| Qualcomm | The prerequisite FG should be 2-35, similar to other codebooks. gNB should know this feature is supported or not. If not signaled, UE does not support Regular eType II. Reporting type should be per band AND per BC, similar to other codebooks so as to avoid underreporting issue. N/A for FRx differentiation. |

| 16-3a-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | It should at least be per band. To make it per band AND per BC is better. |
| CATT | Component#2, #3 and #4 of 16-3a shall also be added to the description field of 16-3a-1 to make it complete. The type shall also be per band and per BC. |
| Apple | If triplet is reported, we need to consider the ways to reduce the overhead. For example, for the triplets that UE reports UE support R=2, UE can also support R=1 so that the triplets do not need to be reported twice. |
| Qualcomm | The prerequisite FG should be 16-3a gNB should know this feature is supported or not. If not signaled, UE does not support R=2 Reporting type should be per band AND per BC, similar to all other triplet signaling to avoid underreporting issue. N/A for FRx differentiation. |

| 16-3a-2 | Support of parameter combinations 7-8 | Support of parameter combinations 7-8 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| CATT | This feature group shall be per band reported, and prerequisite of this feature group shall be ’16-3a or 16-3a-1’. |
| Apple | It can be per band |
| Qualcomm | The prerequisite FG should be 16-3a. gNB should know this feature is supported or not. If not signaled, UE does not support parameter combination 7-8 Reporting type should be per band. N/A for FRx differentiation. |

| 16-3a-3 | Support of rank 3,4 | Support of rank 3,4 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| CATT | This feature group shall be per band reported, and prerequisite of this feature group shall be ’16-3a or 16-3a-1’. |
| Apple | Per band is okay, but it is subject to MIMO layer capability which is per FSPC |
| Qualcomm | The prerequisite FG should be 16-3a. gNB should know this feature is supported or not. If not signaled, UE does not support rank 3-4 Reporting type should be per band. N/A for FRx differentiation. |

| 16-3a-4 | CBSR | ALT 1) CBSR with soft amplitude restriction (capture consequence if not supported  hard amplitude restriction is supported) ALT 2) CBSR |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | Essentially ALT1 is preferred, but the component should be reformulated to say as “support of amplitude subset restriction”. |
| ZTE | It should be Alt 1) per previous agreement. |
| CATT | ALT1 shall be selected per previous agreement. |
| Apple | We are fine with only limit to soft amplitude restriction |
| Qualcomm | The prerequisite FG should be 16-3a. gNB should know this feature is supported or not. If not signaled, UE does not support soft amplitude restriction. Reporting type should be per band. N/A for FRx differentiation. |

| 16-3b | Port selection eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=1 - 6 parameter combinations (combos with L=6 don’t apply) - Support of rank 1,2 | FFS |  | N/A |  | FFS: Per band or Per band per BC | No | No |  |  | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | - On the feature type follow same approach as for Rel-15 in order to avoid future underreporting issues. |
| ZTE | It should at least be per band. To make it per band AND per BC is better. It’s better to clarify only the 1-st component requires signaling. |
| CATT | 16-3b shall be reported per band and per BC based the discussion in past RAN1 meetings to resolve under-reporting issue. |
| Apple | - Similar as FG2-36/40/41/43, it is both per band and per BC, no for xDD and FRx differentiation - We need to discuss the dependency of the parameter setting on the triplets UE reported - For example, similar as Rel-15, UE can report the supported maximum L for each number of ports of CSI-RS for CMR. |
| Qualcomm | The prerequisite FG should be 2-35, similar to other codebooks. gNB should know this feature is supported or not. If not signaled, UE does not support eType II port selection. Reporting type should be per band AND per BC, similar to other codebooks so as to avoid underreporting issue. N/A for FRx differentiation. |

| 16-3b-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | It should at least be per band. To make it per band AND per BC is better. |
| CATT | Component#2 and #3 of 16-3b shall also be added to the description field of 16-3b-1 to make it complete. The type shall also be per band and per BC. The fields ‘Need of FDD/TDD differentiation’ and ‘Need of FR1/FR2 differentiation’ shall be set to ‘N/A’. |
| Apple | If triplet is reported, we need to consider the ways to reduce the overhead. For example, for the triplets that UE reports UE support R=2, UE can also support R=1 so that the triplets do not need to be reported twice. |
| Qualcomm | The prerequisite FG should be 16-3b gNB should know this feature is supported or not. If not signaled, UE does not support R=2 Reporting type should be per band AND per BC, similar to all other triplet signaling to avoid underreporting issue. N/A for FRx differentiation. |

| 16-3b-2 | Support of rank 3,4 | Support of rank 3,4 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| CATT | This feature group shall be per band reported, and prerequisite of this feature group shall be ’16-3b or 16-3b-1’. The fields ‘Need of FDD/TDD differentiation’ and ‘Need of FR1/FR2 differentiation’ shall be set to ‘N/A’. |
| Apple | Per band is okay, but it is subject to MIMO layer capability which is per FSPC |
| Qualcomm | The prerequisite FG should be 16-3a. gNB should know this feature is supported or not. If not signaled, UE does not support rank 3-4 Reporting type should be per band. N/A for FRx differentiation. |

| 16-7 | Extension of the maximum number of configured aperiodic CSI report settings | Extension of the maximum number of configured aperiodic CSI report settings for all codebook types |  |  |  |  | [Per band] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Per band should suffice. |
| CATT | The type shall be per band. The fields ‘Need of FDD/TDD differentiation’ and ‘Need of FR1/FR2 differentiation’ shall be set to ‘N/A’. |
| Qualcomm | The prerequisite FG should be 2-32. gNB should know this feature is supported or not. If not signaled, the maximum number of configured aperiodic CSI report settings for all codebook types should be based on 2-35. Reporting type should be per band. N/A for FRx differentiation. |

| 16-8 | Active CSI-RS resources and ports for mixed codebook types in any slot | FFS: the detailed component design |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | The new capability can be defined such that it indicates a certain number of combinations of codebooks and the associated capabilities for each combination. If any new rules on how to interpret the combination of capabilities need to be specified after the definition of the new capability, those should be clearly defined within 38.306 to avoid ambiguities and underreporting issues. |
| ZTE | For component design: - Report a list of codebook combinations as {codebook 1, codebook 2}. - For each codebook combination, report a list of {max number of ports per resource, max number of resources, max number of total ports}. It should at least be per band. To make it per band AND per BC is better. |
| Qualcomm | We think it is better to have a clear solution for all possible combinations, either signaling based (option 1) or rule-based (option 2) or avoid (option 3). So, we suggest to use the following table to ease the discussion. Number of codebooks in concurrency; Mixed codebook types; Solution. >2; Any types; Option 3. =2; Any pair of {Type II, Type II PS, eType II, eType II PS}; Option 3. Type I SP + Type I MP; Option 1/2/3. Type I SP/MP + any of {Type II, Type II PS, eType II, eType II PS}. Based on the table, for component design, we share similar view as Nokia and ZTE. We may consider the following as starting point for discussion - Report a list of codebook combinations as {codebook 1, codebook 2}. - Codebook 1 = {Type I SP, Type I MP}, codebook 2 = {Type II, Type II PS, eType II, eType II PS} - For each codebook combination, report a list of {max number of ports per resource, max number of resources, max number of total ports}. - For codebook combinations NOT signaled, we should define rules (option 2) to restrict their concurrent capabilities or avoid their currency. The rule and avoidance should be captured in 38.306 or 38.214. The prerequisite FG should be 2-35. gNB should know this feature is supported or not. If not signaled, all the codebook combinations should be subject to the rule specified in in 38.306 or 38.214 or avoided from triggering/configuring concurrently. Reporting type should be per band and per BC, same as all other codebooks. N/A for FRx differentiation. |

…

# Conclusion

…

# References

- R1-2003194, Summary of Email Approval [100e-b-NR-UEFeatures-Remaining] — MIMO Enhancement Aspects, Moderator (AT&T)

- R1-2004285, Summary of UE features for eMIMO, Moderator (AT&T)