3GPP TSG RAN WG1 #101			                                   					       												                  R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.11.6

Source:	Moderator (AT&T)

Title:	Summary of email discussion/approval [101-e-Post-NR-UE-Features-07]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [101-e-Post-NR-UE-Features-07] after RAN1 #101-e. According to the Chairman’s Notes:

[101-e-Post-NR-UE-Features-07] Email discussion/approval for remaining issues on UE features for eMIMO, till 6/10 – Ralf (AT&T) - Need for gNB to know where that field is empty - Type and/or xDD/FRx differentiation for FGs 16-1a-1, 16-1b-3, 16-1f, 16-1g, 16-2a, 16-2a-0, 16-2a-1, 16-2a-2, 16-2a-3, 16-2a-4, 16-2a-4a, 16-2a-5, 16-2a-6, 16-2a-7, 16-x, 16-2a-9, 16-2a-10, 16-2b-1, 16-2b-1a, 16-2b-2, 16-2b-3, 16-2b-3a, 16-2b-4, 16-2b-5, 16-4, 16-5a, 16-6a, 16-6b, 16-6c - Component descriptions and component candidate values for all FGs with open ASN.1 impact

The following was discussed and agreed after RAN1 #101-e within the scope of [101-e-Post-NR-UE-Features-07].

# Summary of email discussion/approval [101-e-Post-NR-UE-Features-07]

This email discussion/approval aims to finalize all ASN.1 impact for the NR UE features for Rel. 15 MIMO enhancements.

The following FGs represent the versions in the NR UE feature list for eMIMO agreed by RAN1 during RAN1 #101-e Error: Reference source not found.

For the  following fields, the moderator proposes a resolution based on the summaries in [2][3][4][5][6]:

- Need for gNB to know is “Yes”

- Applicability to capability signalling between UEs is “N/A”

- Type is “per FSPC”

- FRx differentiation is “No”

- xDD differentiation is “No”

The moderator’s proposals generally reflect the majority view, and, in many cases, further discussion may be needed.

In a first phase of the email discussion/approval, companies are invited to comment on the yellow highlighted items in the tables below.

| 16-1a-1 | SSB/CSI-RS for L1-SINR measurement | Per slot limitations: - The max number of [unique] SSB/CSI-RS [(1Tx)] for CMR - The max number of [CSI-IM/NZP-IMR] resources - [The max number of CSI-RS (2Tx) resources for CMR] Memory limitations: - The max number of SSB/CSI-RS resources as CMR - The max number of [CSI-IM/NZP IMR] resources Other limitations: - [Supported density of CSI-RS (CMR)] - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 - Supported SINR measurements: {SSB as CMR with dedicated IMR, CSI-RS as CMR with dedicated [CSI-IM/NZP IMR] configured, CSI-RS as CMR without dedicated IMR configured[, CMR+CSI-IM+NZP-IMR], [CSI-RS (2Tx) resources for CMR]} | 2-21, 2-22 or 2-23, 2-23a | Yes | N/A |  | Per band | No | No |  | Note: For Component 11, UE must at least report support of one [FFS: which one(s)] FFS: How CSI-RS is counted when it is configured as CMR without dedicated IMR | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | 1. Component #1, remove brackets, candidate values {8, 16, 32, 64} 2. Component #2, remove brackets, candidate values {8, 16, 32, 64} 3. Component #3, remove component (to be captured by component #8) 4. Component #4, candidate values {16, 32, 64, 128} 5. Component #5, remove brackets, candidate values {16, 32, 64, 128} 6. Component #6, remove brackets, candidate values {‘1 only’, ‘1 and 3’} 7. Component #7, candidate values {8, 16, 32, 64} 8. Component #8, remove all brackets, UE must report supporting ‘CSI-RS as CMR with dedicated IMR configured’ 9. Suggest adding a note as in Rel-15 that “If the UE sets a value other than n0 in an FR1 band, it shall set that same value in all FR1 bands. If the UE sets a value other than n0 in an FR2 band, it shall set that same value in all FR2 bands. The UE supports a total number of resources equal to the maximum of the FR1 and FR2 value, but no more than the FR1 value across all FR1 serving cells and no more than the FR2 value across all FR2 serving cells.” |
| Qualcomm | Prefer to remove “unique”, since same resource ID can appear twice in a 60kHz slot, and they should be counted twice. Prefer to merge “CSI-IM/NZP IMR” as IMR, i.e. count the total of the two. Support the density component Either “CSI-RS as CMR with CSI-IM IMR” or “CSI-RS as CMR with NZP IMR” can be mandated to support Does not support CMR+CSI-IM+NZP-IMR, which is not agreed Support the separate capability for CSI-RS(2Tx), either in component 3 or 8, one is sufficient |
| Samsung | - Component 1: remove [unique], and keep (1Tx) - Component 2: remove brackets at CSI-IM/NZP-IMR - Component 3: remove brackets (support the component) - Component 5: remove brackets at CSI-IM/NZP-IMR - Component 6: remove brackets (support the component) - Component 8: remove brackets at CSI-IM/NZP-IMR, delete [CMR+CSI-IM+NZP-IMR] |
| Apple | - Component 1: - Prefer to remove “unique”. It is impossible to ensure that there is no UE computational complexity increase when multiple resources are configurated with the same ID. This concept is not there in Rel-15. - Keep “1Tx”. - Candidate value set {[0], 4, 8, 16, 32, 64}. - Component 2: - CSI-IM and NZP-IMR can be combined only if UE can independently indicate whether UE support CSI-IM and/or NZP-IMR in component 8. - Candidate value set {[0], 4, 8, 16, 32, 64} - Component 3: - Keep it. - Candidate value set {[0], 1, 2, 4, 8, 16, 32, 64} - Component 4: - Candidate value set {[0], 4, 8, 16, 32, 64}. - Component 5: - CSI-IM and NZP-IMR can be combined only if UE can independently indicate whether UE support CSI-IM and/or NZP-IMR in component 8. - Candidate value set {[0], 4, 8, 16, 32, 64} - Component 6: Keep the component - Candidate value set {"not supported", "1 only", "3 only", "both 1 and 3"} - Component 7: - Candidate value set {0, 1, 2, 4, 8, 16, 32, 64} - Component 8: We need the following bit maps - 1 port CSI-RS as CMR without dedicated IMR - SSB as CMR with dedicated IMR - CMR + CSI-IM - CMR + NZP-IMR - 2 port CSI-RS for CMR - CMR+CSI-IM+NZP-IMR - No FDD/TDD differentiation - Potentially Yes for FR1/FR2 differentiation - Clarification of subcarrier spacing of per slot is needed for “within a slot” |
| LG | Although it is not our first preference, we suggest adopting the latest harmonized proposal discussed in [101-e-NR-eMIMO-UEFeature-01] summarized by Claes (Ericsson) last week for fast convergence. For the candidate values, we suggest adding ‘4’ for component 7 compared with the latest proposal from Claes. - Combine CSI-IM and NZP-IMR  Remove brackets of Component 2 and 5 - Keep CSI-RS density  Remove brackets of Component 6 - Remove ‘unique’ from Component 1 1. The max number of SSB/CSI-RS [(1Tx)] for CMR: {8, 16, 32, 64} 2. The max number of CSI-IM/NZP-IMR resources: {8, 16, 32, 64} 3. [The max number of CSI-RS (2Tx) resources for CMR]: {0, 8, 16, 32, 64} 4. The max number of SSB/CSI-RS(1Tx)/CSI-IM resources as CMR: {8, 16, 32, 64, 128} 5. The max number of CSI-IM/NZP IMR resources: {8, 16, 32, 64, 128} 7. The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1: {4, 8, 16, 32, 64} |
| Spreadtrum | - Component 1: remove ‘unique’, and keep ‘1Tx’ - Component 2: remove brackets - Component 3: remove brackets - Component 5: remove brackets - Component 6: remove brackets. The candidate values { “1”, “3”, “both 1 and 3”} - Component 8: support CSI-IM/NZP-IMR and CSI-RS (2Tx) resources for CMR, not support CMR+CSI-IM+NZP-IMR for it has not been agreed in Rel-16. - Note: For component 11, should be revised as For component 8 |
| OPPO | Componenet#1: remove the bracket Component #2: remove the bracket and change “CSI-IMR/NZP-IMR” to “IMR” Componenet#3: remove the bracket Component #5: remove the bracket and change “CSI-IM/NZP-IMR” to “IMR” Component #6: remove the bracket Component #8: UE at least reports one value but we do not mandate which one. |
| Nokia/NSB | - Component 1: UE would be restricted with number of resource to be measured as CMR, whether the resource is uniquely configured as CMR or not. The word ‘unique’ is not essential. In addition, we would like to note that no separation is required between 1Tx/2Tx CSI-RS for CMR - Component 2: O.K. with CSI-IM/NZP-IMR - Component 3: We do not need to separate 1Tx/2Tx CSI-RS. Component 3 should be removed - Component 5: O.K. with CSI-IM/NZP IMR - Component 6: We slightly prefer not to define, but can be open for the progress - Component 8: - We agree to define one default measurement type and have a priority on SSB as CMR with dedicated IMR. - We want to remove {CMR+CSI-IM+NZP-IMR, CSI-RS (2Tx)}. CMR+CSI-IM+NZP-IMR is not agreed to be supported yet, so it cannot be included, or we can leave it with square bracket as it as. As explained for component 1 and 3, we do not want to differentiate 1Tx/2Tx CSI-RS - For the FFS part, without dedicated IMR, UE should measure residual power of CMR as interference, but we do not need to define such operation in UE feature. - Though moderator invited discussion first on yellow highlighted items, we are not OK with FSPC for this FG. It should be Per Band, which follows the type of similar FG from Rel-15. |
| ZTE | Considering that there is similar complexity for CSI-IM and NZP-IMR, “CSI-RS as CMR with dedicated CSI-IM/NZP-IMR configured” is supported as a mandatory function for component-11. Selecting dedicated CSI-IM or NZP-IMR is determined according to the reporting value of the two new components related to component 2 and the two new ones related to component 5. Also, there is some redundancy between component 4, component 8 and component 10, we slightly prefer to remove the bracket of component 10 and delete the whole sentence of component 4 and 8. Then, the text of “[(1Tx)]” can be removed accordingly. Consequently, we have the following suggestions. Proposed changes: Per slot limitations: - The max number of [unique] SSB/CSI-RS [(1Tx)] for CMR - The max number of [CSI-IM/NZP-IMR] resources - The max number of NZP-IMR resources - [The max number of CSI-RS (2Tx) resources for CMR] Memory limitations: - The max number of SSB/CSI-RS resources as CMR - The max number of [CSI-IM/NZP IMR] resources - The max number of NZP-IMR resources Other limitations: - [Supported density of CSI-RS (CMR)] - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 - Supported SINR measurements: one or combination of {SSB as CMR with dedicated IMR, CSI-RS as CMR with dedicated [CSI-IM/NZP IMR] configured, CSI-RS as CMR without dedicated IMR configured[, CMR+CSI-IM+NZP-IMR], [CSI-RS (2Tx) resources for CMR]} |
| Ericsson | 1. Remove “unique”. Remove “(1Tx)” 2. Remove brackets around CSI-IM/NZP-IMR 3. Remove brackets. (The UE can report =0, since there is no risk for ambiguity.) 5. Remove brackets around CSI-IM/NZP-IMR 6. Remove brackets 8. Remove brackets around CSI-IM/NZP-IMR, or just write “IMR”. Remove “[, CMR+CSI-IM+NZP-IMR], [CSI-RS (2Tx) resources for CMR]” – neither of these options should be included. CSI-IM+NZP-IMR is not supported in specification (text is in brackets). The UE would advertise support for 2Tx CSI-RS CMR by signaling >0 in component 3. Need for the gNB to know: Yes Applicable to V2X: N/A Need for FDD/TDD differentiation: No Need for FR1/FR2 differentiation: No Notes: - For component 8, the UE must report support of “CSI-RS as CMR with dedicated CSI-IM/NZP IMR configured” - Remove “FFS: How CSI-RS is counted when it is configured as CMR without dedicated IMR” Value ranges: - {8, 16, 32, 64} - {8, 16, 32, 64} - {0, 8, 16, 32, 64} - {8, 16, 32, 64, 128} - {8, 16, 32, 64, 128} - {one, three, oneAndThree} – same as in R15. UE must report ‘three’ or ‘oneAndThree’ - {8, 16, 32, 64} - {SSB as CMR with dedicated IMR, CSI-RS as CMR with dedicated IMR, CSI-RS as CMR without dedicated IMR} – the UE must report support for at least CSI-RS as ‘CMR with dedicated IMR’. |
| MediaTek | - Component 1: Remove ‘[unique]’ and Remove the bracket for ‘[(1Tx)] for CMR. Candidate values {8, 16, 32, 64} The max number of [unique] SSB/CSI-RS [(1Tx)] for CMR - Component 2: Remove the bracket. Candidate values {0, 8, 16, 32, 64} The max number of [CSI-IM/NZP-IMR] resources - Component 3: Remove the bracket. Candidate values {0, 4, 8, 16, 32, 64} [The max number of CSI-RS (2Tx) resources for CMR] - Component 4: Candidate values {8, 16, 32, 64, 128} - Component 5: Remove the bracket. Candidate values {0, 8, 16, 32, 64, 128} The max number of [CSI-IM/NZP-IMR] resources - Component 6: Remove the bracket [Supported density of CSI-RS (CMR)] - Component 7: Candidate values {4, 8, 16, 32, 64} - Component 8: Delete CMR+CSI-IM+NZP-IMR, Remove the brackets for the rest. Supported SINR measurements: {SSB as CMR with dedicated IMR, CSI-RS as CMR with dedicated [CSI-IM/NZP IMR] configured, CSI-RS as CMR without dedicated IMR configured[, CMR+CSI-IM+NZP-IMR], [CSI-RS (2Tx) resources for CMR]} |
| Intel | Candidate values: - Component 1: Candidate values {8, 16, 32, 64} - Component 2: Candidate values {8, 16, 32, 64} - Component 3: Candidate values {[0], 8, 16, 32, 64} - Component 4: Candidate values {8, 16, 32, 64} - Component 5: Candidate values {8, 16, 32, 64} - Component 6: TBD. - Component 7: Candidate values {8, 16, 32, 64} No need of “unique” in component 1, however counting of CSI-RS resources needs to be clarified. Remove brackets in 1Tx Remove brackets in component 3. Remove [CSI-RS (2Tx) resources for CMR] in the candidate list. Remove brackets in component 5 Candidate 6 may not be needed, since UE is not required to estimate / store all CSI-RS REs subject o RAN4 requirements Mandatory combination CSI-RS as CMR with dedicated CSI-IM/NZP IMR configured |

| 16-1a-2 | Non-group based L1-SINR reporting | - Support of non-group based L1-SINR reporting with N_max L1-SINR values reported | 16-1a-1 | Yes | N/A |  | Per band | No | No |  | Note: Default value is N_max = 1 in case 16-1a-2 is not provided by the UE. | Optional with capability signalling Candidate value set is {1, 2, 4} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Support moderator proposal |
| Apple | Support |
| Spreadtrum | Support |
| Nokia/NSB | Support moderator proposal |
| ZTE | Support the highlighted parts. |
| Ericsson | Need for the gNB to know: Yes Applicable to V2X: N/A |
| MediaTek | Support |
| Intel | OK with moderator proposal |

| 16-1a-3 | Group based L1-SINR reporting | - Support of group based L1-SINR reporting | 16-1a-1 | Yes | N/A |  | Per band | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Support moderator proposal |
| Apple | Support |
| Spreadtrum | Support |
| Nokia/NSB | Support moderator proposal |
| ZTE | Support the highlighted parts. |
| Ericsson | Need for the gNB to know: Yes Applicable to V2X: N/A |
| MediaTek | Support |
| Intel | OK with moderator proposal |

| 16-1b-1 | TCI state activation across multiple CCs | - Support of Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH | Component 1: 2-1, 2-4 | Yes | N/A |  | per UE | No | Yes |  | Note: Whether a FG to indicate group(s) of bands that share the same DL spatial filters will be introduced is in RAN4 domain | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Support moderator proposal |
| Qualcomm | Prefer per BC instead of per UE |
| Apple | Prefer per BC |
| Nokia/NSB | Support moderator proposal |
| ZTE | Support the highlighted part. |
| Ericsson | Need for the gNB to know: Yes |
| MediaTek | Support |
| Intel | OK with moderator proposal |

| 16-1b-2 | Spatial relation update across multiple CCs | - Support of Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS | Component 1: 2-59, 2-60 | Yes | N/A |  | per UE | No | Yes |  | Note: Whether a FG to indicate group(s) of bands that share the same UL spatial filters will be introduced is in RAN4 domain | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Support moderator proposal |
| Apple | Prefer per BC |
| Nokia/NSB | Support moderator proposal |
| ZTE | Support the highlighted parts. |
| Ericsson | Need for the gNB to know: Yes |
| MediaTek | Support |
| Intel | Should be “Per UE” and “applicable to FR2 only” |

| 16-1b-3 | Spatial relation update for PUCCH group | - Support of PUCCH resource groups per BWP for simultaneous spatial relation update | 2-53, 2-59, 4-24 | Yes | N/A |  | [Per BC or per band] | No | Yes |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Support moderator proposal |
| Qualcomm | Prefer per band |
| Samsung | Prefer per band |
| Apple | Prefer per band |
| Nokia/NSB | Support moderator proposal |
| ZTE | Support the highlighted parts. |
| Ericsson | Need for the gNB to know: Yes Type: per band |
| MediaTek | Prefer per band |
| Intel | OK with moderator proposal |

| 16-1c | Default spatial relation | Support of default spatial relation and pathloss reference RS for dedicated-PUCCH/SRS and PUSCH | 2-53, 2-59 | Yes | N/A |  | per UE | No | FR2 only |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Can accept moderator proposal, although we prefer to enable default PL-RS in FR1 as well |
| Apple | Support |
| Nokia/NSB | Support moderator proposal |
| ZTE | Support the highlighted parts. |
| Ericsson | Need for the gNB to know: Yes |
| MediaTek | Support |
| Intel | OK with moderator proposal |

| 16-1d | MAC CE spatial relation update for AP-SRS | Support of spatial relation update for AP-SRS via MAC CE | 2-53, 2-59 | Yes | N/A |  | Per UE | No | FR2 only |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Support moderator proposal |
| Apple | Support |
| Nokia/NSB | Support moderator proposal |
| ZTE | Support the highlighted parts. |
| Ericsson | Need for the gNB to know: Yes |
| MediaTek | Support |
| Intel | OK with moderator proposal |

| 16-1e | Pathloss reference RS activation via MAC CE | - The maximum number of configured pathloss reference RSs for PUSCH/PUCCH/SRS by RRC for MAC-CE based pathloss reference RS update | 8-3 | Yes | N/A |  | Per UE | No | No |  | Candidate values for component (1): {FFS} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | 1. We are not sure whether RRC signaling structure allows NW to dedicatedly mark PL-RS for the purpose of MAC-CE-based update. If this is not possible, we don’t know how to interpret this UE feature, and suggest removing ‘for MAC-CE based pathloss reference RS update’. 2. Candidate values {8, 16, 32, 64} |
| Apple | - We prefer to be band, so component 1 is per band maximum number of configured pathloss RSs. - Candidate values {4, 8, 16, 32, 64} |
| ZTE | Support the first highlighted parts. The candidate value for this component: {4, 8, 16, 32, 64} |
| Ericsson | Need for the gNB to know: Yes Value range: {8,16,32,64} – 0 should not be included, that would indicate “no support” of the feature |
| Intel | Candidate values {8, 16, 32, 64} |

| 16-1f | SCell beam failure recovery | - The maximum number of SCells configured for SCell beam failure recovery simultaneously | 2-31 | Yes | N/A |  | [Per band] [Per BC] [Per UE] | No | No |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | 1. Suggest putting type as ‘per band’, and adding a note as in Rel-15 that “If the UE sets a value other than n0 in an FR1 band, it shall set that same value in all FR1 bands. If the UE sets a value other than n0 in an FR2 band, it shall set that same value in all FR2 bands. The UE supports a total number of resources equal to the maximum of the FR1 and FR2 value, but no more than the FR1 value across all FR1 serving cells and no more than the FR2 value across all FR2 serving cells” |
| Qualcomm | Prefer per band |
| Samsung | Support per band |
| Apple | Per band |
| LG | Per band |
| Spreadtrum | Per band |
| OPPO | Per band |
| Nokia/NSB | Per UE. As noted by Huawei above the UE is expected to repeat the same information for all FR1/FR2 bands anyway. FRx differentiation would be enough to address this issue. |
| ZTE | Support the first and third highlighted parts. For the second highlighted part, this FG is “per UE”. |
| Ericsson | Need for the gNB to know: Yes Type: Per UE or Per band Need for FR1/FR2 differentiation: No Value range: {1,2,4,8,16} |
| MediaTek | Per band |
| Intel | Per band. Candidate values {1,2,4,6,8} |

| 16-1g | Resources for beam management, [pathloss measurement, BFD, and BFR] | - The maximum number of [unique] SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs for any of L1-RSRP measurement, L1-SINR measurement, [pathloss measurement, BFD, RLM] and new beam identification - The maximum number of SSB/CSI-RS/CSI-IM resources configured across all CCs for any of L1-RSRP measurement, L1-SINR measurement, [pathloss measurement, BFD, RLM] and new beam identification | 2-24, 2-31 | Yes | N/A |  | [Per band] [Per BC] [Per UE] | No | No |  | Component-1: candidate value set is {4, 8, 16, 32, 64, 128, FFS} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | 1. Feature name: Remove brackets, add RLM 2. Component #1: Remove ‘[unique]’, change ‘configured’ as ‘required’, remove brackets on ‘pathloss measurement, BFD, RLM’, candidate values: {8:1:64} (step size of 1) 3. Component #2: Change ‘configured’ as ‘required to measure’, remove brackets on ‘pathloss measurement, BFD, RLM’, candidate values: {32:1:1024} (step size of 1) 4. Add a note that ‘If one signal is associated with multiple functionalities or with multiple reporting settings, the signal would be counted only once’, which is applicable for both component #1/2 5. Suggest putting type as ‘per band and adding a note as in Rel-15 that “If the UE sets a value other than n0 in an FR1 band, it shall set that same value in all FR1 bands. If the UE sets a value other than n0 in an FR2 band, it shall set that same value in all FR2 bands. The UE supports a total number of resources equal to the maximum of the FR1 and FR2 value, but no more than the FR1 value across all FR1 serving cells and no more than the FR2 value across all FR2 serving cells”. |
| Qualcomm | Prefer to remove the bracket, i.e. include PL RS, BFD, RLM. Prefer per band, same as FG 2-24 Fine to have 256 in candidate values |
| Samsung | - Type: per BC or per UE are reasonable. - Candidate value for component 1: we think the maximum value of 128 can be enough, but if companies have aligned view for more than 128, we can also live with that. |
| Apple | - Component 1: - More clarification of “unique” and motivation to include pathloss/BFD/RLM RS is needed - Candidate value set {4, 8, 16, 32, 64, 128}. - Component 2: - Candidate value set {4, 8, 16, 32, 64, 128}. - Per band and Per BC |
| LG | Although it is not our first preference, we suggest adopting the latest harmonized proposal discussed in [101-e-NR-eMIMO-UEFeature-01] last week for fast convergence, captured below. The component descriptions for both components need to be revised accordingly. The notes can be discussed in later meeting because it has no ASN.1 impact. - Component#1: The maximum number of SSB/CSI-RS/CSI-IM resources required to measure within a slot across all CCs for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, beam failure detection, radio link monitoring, and new beam identification - Component#2: The maximum number of SSB/CSI-RS/CSI-IM resources required to measure across all CCs for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, beam failure detection, radio link monitoring, and new beam identification |
| OPPO | - We support to include pathloss measurement RS, BFD and RLM in both component 1 and 2 and the FG title. - Support per band |
| Nokia, NSB | Support to exclude [pathloss measurement, BFD, RLM]. And we are O.K. with per band indication |
| ZTE | As our preference, we suggest to delete the whole text of “[pathloss measurement, BFD, and RLM]”, which may introduce some backward compatibility issue for Rel-15 gNB. But, if including pathloss measurement, BFD, and RLM, we need to have the following notes to reserve sufficient resources for backward compatibility as a condition for compromise. - Note-1: UE should report component 1 with value X1>0 greater than MB_1 in FG 2-24. FFS: X1 - Note-2: UE should report component 5 with value X2=16 greater than MC_1 in FG 2-24. The feature is “per UE”, and we have the following candidate value for component 1 and component 5. - Component-1: {4, 8, 16, 32, 64, 128, FFS} - Component-2: {4, 8, 16, 32, 64, 128, 256, FFS} |
| Ericsson | Component name: Rename the component “Resources for beam management and pathloss measurement” 1. Remove “[unique]”. Remove brackets around [pathloss measurement, BFD, RLM] 2. Remove brackets around [pathloss measurement, BFD, RLM] Need for the gNB to know: Yes Type: Per band Need for FR1/FR2 differentiation: No Value range: - {8,12,16,20, …, 256} - {16,24,32, 40, …,1024} |
| MediaTek | - Component 1: Remove the brackets for pathloss measurement, BFD, RLM. Candidate values {4, 8, 16, 32, 64, 128}, Need more clarification for ‘unique’ as Apple suggested - Component 2: Remove the brackets for pathloss measurement, BFD, RLM. Candidate values {4, 8, 16, 32, 64, 128, 256} |
| Intel | Candidate values: 1st component {16, 32, 64,128, 256, 512} Candidate values: 2nd component {16, 32, 64,128, 256, 512, 1024} |

| 16-2a | Multi-DCI based multi-TRP | - The maximum number of CORESETs configured per “PDCCH-Config” - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” - Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency - Maximum number of unicast PDSCHs per CORESETPoolIndex per slot - [PDSCH processing capability for CC] | FFS | Yes | N/A |  | [per band / per FSPC] | No | No |  | Note: A UE may assume that its maximum receive timing difference between the DL transmissions from two TRPs is within a CP FFS: component (4) only applies to UE processing capability #1 | FFS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Component 1: Candidate values {2,3,4,5} Component 2: Candidate values {1,2,3} Component 4: Candidate values {[1],2,4,7} which shall be per SCS, similar with Rel-15 as following Component 5: Agree with Nokia. It shall be removed and it shall be up to further discussion in RAN1 maintenance. |
| QC | - Candidate values for component 1: {2,3,4,5} - Candidate values for component 2: {1,2,3} - Candidate values for component 4: {1,2,4,7}. Agree with HW that it should be per SCS. - Regarding component 5: We have to have a common understanding whether processing capability 2 is supported or not in the case of multi-DCI. If supported, Alt1 (option1/option2) is needed. If not supported, Alt2 is fine with us. - Alt1: When at least one CC is configured with multi-DCI, Cap2 can be configured in the same CC or another CC - Option1: Keep the component with candidate values { Capability 1, Capability 2 with scheduling limitation, Capability 2} or - Option2: Add a description in Rel. 15 FGs 5-5a, 5-13, 5-13a, 5-13c (applicable only in Rel. 16 UEs supporting mDCI) that when # of configured CCs are counted to compare against X, each mDCI CC is counted as two times. - Alt2: Add a note that “Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex - Regarding type: Agree with “FSPC”. As explained before, anything other than FSPC is not acceptable for this FG since the BW of each CC with mDCI is also important (and not just the number of CCs) - No prerequisite is needed. - Last column: “Optional with capability signaling” |
| Samsung | - Type: FSPC signaling is necessary. UE complexity to support multi-TRP depends on the max BW of a CC which is already signaled per FSPC. Besides, 16-2a, 16-2a-0, and 16-2a-1 are parallel features (non/full/partial overlapping) which need to be the same type. - Component 5: If cap#2 is to be supported, okay with option2 in Alt1 from QC. |
| Apple | - Component 1: - Candidate value set {[1], 2, 3, 4, 5} - Component 2: - Candidate value set {1, 2, 3} - Component 4: We can reuse similar design as pdsch-ProcessingType1-DifferentTB-PerSlot as suggested by Huawei - Component 5: Remove it unless we first agree on the PDSCH processing capability #2 timeline. As of now, there is no PDSCH processing capability 2 for MDCI MTRP. - FFS removed from “FFS: component (4) only applies to UE processing capability #1” - It is optional with capability signalling |
| LG | - Candidate values for component 1,2,4: agree with QC - Regarding component 5: prefer not supporting Cap2 and fine with adding a note that “Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex - Regarding type: FSPC |
| Spreadtrum | - Component 1: candidate values {2,3,4,5} - Component 2: candidate values {1,2,3} - Component 5: remove it. In Rel-16, we have not agreed to support PDSCH processing capability 2 for multi-DCI based multi-TRP. - No prerequisite - Type : FSPC - Optional with capability signalling |
| OPPO | - Candidate values for component 1: {2,3,4,5} - Candidate values for component 2: {1,2,3} - Candidate values for component 4: {1,2,4,7} per SCS. - On component 5, we are fine with a note of “Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex” - Support “per FSPC” and “Optional with capability signaling” |
| Nokia/NSB | Component 1: Candidate values {2,3,4,5} Component 2: Candidate values {1,2,3} Component 4: Candidate values {[1],2,4,7} : agree with Huawei comment that this should be per SCS. FFS can be kept for the moment. Component 5: We have not had much discussion on this. Can be removed. Regarding FG type we do not support FSPC. This would imply significant reporting overhead and adds to fragmentation. Please note RAN2 has requested RAN1 to provide clear explanation for all FGs that are defined as FS or FSPC, and that explanation has not been provided yet. We understand that “per band” is sufficient here. |
| MediaTek | Component 1: Candidate values {2, 3, 4, 5} Component 2: Candidate values {1, 2, 3} Component 4: Candidate values {1, 2, 4, 7}; agree with Huawei’s comment that this should be per SCS. Component-5: prefer to remove it because we never discuss PDSCH processing capability 2 for M-DCI case. Type: FSPC |
| ZTE | - Per band is enough. Based on RAN2’s guidance, we should avoid FSPC unless very clear benefit is clarified. - The candidate values of component 1 are {2,3,4,5} - The candidate values of component 2 are {1,2,3} - Component 4 is only used for processing capability 1 based on Rel-15’s related capability. The candidates can be {1, 2, 4, 7} - Component 5 is not needed. We are fine not to support processing capability 2 in Rel-16. Optionally, we can reuse Rel-15 capability signaling with some spec change, e.g. Option 2 in Alt.1 as QC suggested. |
| Ericsson | - For the last column, we can agree ‘Optional with capability signaling’ - Regarding Component 5, we see no reason why multi-DCI feature should be limited only to processing capability 1. So our first preference is Al1 with Option 1 in QC’s response. With Alt1 option 1, if a UE is not capable of supporting multi-DCI with capability 2, it can indicate a component value of ‘Capability 1’. We cannot accept limiting Multi-DCI feature to only Capability 1. So Alt1 + Option 1 in Qualcomm’s response seems to be the most flexible way forward. - Regarding reporting type, most companies that propose FSPC only comment about baseband complexity. But there are overhead issues associating with FSPC reporting as pointed out by Nokia and some other companies. Note that we are discussing 3 different multi-DCI FGs and 5 different single-DCI FGs that could potentially end up as FSPC in this email thread. The overhead becomes a real issue when the UEs instantiate many FeatureSetDowlinkperCC IEs as it starts to advertise different combination of features. For instance, the UE may report that it supports for the following: - Multi-DCI (non-overlapping feature) on 4 CCs - Single-DCI scheme 4 on 8 CCs - Single-DCI scheme 1a on 4 CCs - Single-DCI scheme 4 on 4 CCs and Multi-DCI (non-overlapping feature) on 2 CCs For each of this combination, the UE would use one FeatureSetDowlinkperCC IEs. The above is only a small list of combinations. The multi-DCI and/or single-DCI features could also be combined with other features such as number of MIMO layers in which case the number of combinations will explode. This in turn may require a large number of FeatureSetDowlinkperCC IEs to reflect all possible combinations. Hence, we need to consider both UE complexity as well as feedback overhead when it comes to the design of reporting type. In an earlier email thread, Haitong (Apple) mentioned ‘The fundamental issue is that UE may have a total budget in terms of the number TRPs (CCs), or number of MIMO layers, we can support’. This reminds me of a cost + budget based UE capability design that was agreed in R1-1905577 & R1-1904509 to reduce the signaling overhead. I think we can consider a similar design here, but given the limited time we have, we can leave the detailed design to RAN2. So, as a compromise, we can live with FSPC under the condition, the following note is added: Note: To reduce signaling overhead reduction, UE capability design similar to R1-1905577 & R1-1904509 can be further considered in RAN2. Detailed signaling design is up to RAN2. If this compromise is not acceptable to companies, we prefer to stick to our original preference which is ‘Per Band’ reporting. |
| Intel | Component 1: clarification needed for counting of CORESET#0 Component 2: values {1, 2, 3} Component 4. {1, 2, 4, 7} per SCS (similar to Rel-15) but clarify if this is also equal to the maximum number of unicast PDSCHs per slot (all PDSCHs are TDM) Type: prefer to avoid FSPC signaling and consider per band (or per band per BC) we can consider max number of CC and max BW per CC reporting in 16-2a-9 Ok with Alt 2 |

| 16-2a-0 | Overlapping PDSCHs in time and fully overlapping in frequency and time | - Support PDSCHs with fully overlapping REs, i.e. the allocated REs for PDSCH scheduled by DCI in CORESET configured with CORESETPoolIndex = 0 and PDSCH scheduled by DCI in CORESET configured with CORESETPoolIndex = 1 are exactly the same REs |  | Yes | N/A |  | Per FSPC | No | No |  | Note: A UE may assume that its maximum receive timing difference between the DL transmissions from two TRPs is within a CP | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK |
| QC | Agree. Pre-requisite should be 16-2a. |
| Samsung | - Type: FSPC signaling is necessary. UE complexity to support multi-TRP depends on the max BW of a CC which is already signaled per FSPC. Besides, 16-2a, 16-2a-0, and 16-2a-1 are parallel features (non/full/partial overlapping) which need to be the same type. |
| Apple | Optional with capability signalling Per FSPC |
| LG | Agree with QC |
| Spreadtrum | Optional with capability signalling |
| OPPO | Agree all highlighted |
| Nokia/NSB | Pre-requisite should be 16-2a. Regarding FG type we do not support FSPC. This would imply significant reporting overhead and adds to fragmentation. Please note RAN2 has requested RAN1 to provide clear explanation for all FGs that are defined as FS or FSPC, and that explanation has not been provided yet. We understand that “per band” is sufficient here. |
| MediaTek | 16-2a should be prerequisite. Type: Per FSPC |
| ZTE | - Per band - Optional with capability signalling |
| Ericsson | Add 16-2a as Pre-requisite. Regarding reporting type, please see our detailed comments in feature 16-2a. In essence, we need to consider both UE complexity as well as feedback overhead when it comes to the design of reporting type. In an earlier email thread, Haitong (Apple) mentioned ‘The fundamental issue is that UE may have a total budget in terms of the number TRPs (CCs), or number of MIMO layers, we can support’. This reminds me of a cost + budget based UE capability design that was agreed in R1-1905577 & R1-1904509 to reduce the signaling overhead. I think we can consider a similar design here, but given the limited time we have, we can leave the detailed design to RAN2. So, as a compromise, we can live with FSPC under the condition, the following note is added: Note: To reduce signaling overhead reduction, UE capability design similar to R1-1905577 & R1-1904509 can be further considered in RAN2. Detailed signaling design is up to RAN2. If this compromise is not acceptable to companies, we prefer to stick to our original preference which is ‘Per Band’ reporting. |
| Intel | No need to repeat a note here as it is available pre-requisite is 16-2a |

| 16-2a-1 | Overlapping PDSCHs in time and partially overlapping in frequency | - Support PDSCHs with partially[/fully] overlapping REs [in frequency], i.e. the allocated REs for PDSCH scheduled by DCI in CORESET configured with CORESETPoolIndex = 0 and PDSCH scheduled by DCI in CORESET configured with CORESETPoolIndex = 1 are partially overlapped, with at least one RE - FFS: Restrictions on the maximum number of MIMO layers of each scheduled PDSCH [if PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain] - [The maximal number of PDSCH scrambling sequences per serving cell] FFS whether default values of component 3/4 to be included in 16-2a |  | Yes | N/A |  | Per FSPC | No | No |  |  | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Suggest to remove “fully” and “[frequency]” so that 16-2a-0 and 16-2a-1 can be orthogonal. A UE is free to support 16-2a-0 and/or 16-2a-1. Prefer to remove component 2 We agree that component 3 can be moved to 16-2a with candidate values of {1,2}. Note that Component 3 is the maximal number so that the value of 2 will cover both values of {1 and 2}. The last FFS is not needed anymore. |
| QC | - [in frequency] can be removed - Pre-requisite should be 16-2a and 16-2a-0 - Component 3: Can either be moved to 16-2a, or 16-2a-0 (especially if FG 16-2a-0 is a pre-requisite for this FG), or it can be a new FG. |
| Samsung | - Type: FSPC signaling is necessary. UE complexity to support multi-TRP depends on the max BW of a CC which is already signaled per FSPC. Besides, 16-2a, 16-2a-0, and 16-2a-1 are parallel features (non/full/partial overlapping) which need to be the same type. - Component 1: [/fully] can be removed. Partial overlapping already implies full overlapping. - Component 2: Not needed. There is already an existing UE feature to signal total MIMO layers for all PDSCHs. |
| Apple | Optional with capability signalling Both [/fully] and [in frequency] should be removed, this is not the right way to capture prerequisite, if there is any. We do not put prerequisite FG in the component description, otherwise, many FGs need update Component 2 needs further discussion, Firstly, if we need, it should be moved to 16-2a, Second, we prefer to report per CORESETPoolIndex (TRP) similar as maxNumberMIMO-LayersNonCB-PUSCH Component 3 can be moved to 16-2a (candidate value set {1, 1and2}) or a new FG |
| LG | - [fully] and [in frequency] can be removed. - Pre-requisite should be 16-2a and 16-2a-0 - Remove Component 2. |
| Spreadtrum | - Component 1: suggest to remove [/fully], [in frequency]; - Component 2: seems not needed. Reusing Rel-15 UE feature FG2-3 is enough. - Component 3: it should be moved into FG16-2a. - FFS whether default values of component 3/4 to be included in 16-2a, could be deleted - Optional with capability signalling |
| OPPO | - The name should be “Overlapping PDSCHs in time and partially overlapping in frequency and time” to be consistent with the description. It should include fully overlapping in frequency and partially overlapping in time. - Component 3 can be moved to 16-2a or deleted. - Support to remove all yellow highlighted in the component description. - Pre-requisite should be 16-2a - Support “per FSPC” and “Optional with capability signaling” |
| Nokia/NSB | - Remove “in frequency”. Keep the ‘Fully” but remove the brackets. - Pre-requisite should be 16-2a. - Component 2: ok to remove the FFS. - Component 3: remove brackets. - Also copy the component 2 and 3 to 16-2a-0. - Regarding FG type we do not support FSPC. This would imply significant reporting overhead and adds to fragmentation. Please note RAN2 has requested RAN1 to provide clear explanation for all FGs that are defined as FS or FSPC, and that explanation has not been provided yet. We understand that “per band” is sufficient here. |
| MediaTek | 16-2a and 16-2a-0 should be prerequisites Component-1: remove texts in yellow Component-2: Prefer to move “Restrictions on the maximum number of MIMO layers of each scheduled PDSCH” to 16-2a Component-3: Prefer to move “The maximal number of PDSCH scrambling sequences per serving cell” to 16-2a Type: Per FSPC |
| ZTE | - Per band - Optional with capability signalling - For component 1, the suggested change is Support PDSCHs with partially[/fully] overlapping REs [in frequency], i.e. the allocated REs for PDSCH scheduled by DCI in CORESET configured with CORESETPoolIndex = 0 and PDSCH scheduled by DCI in CORESET configured with CORESETPoolIndex = 1 are partially overlapped, with at least one RE - Component 2 is not needed since we can reuse Rel-15 capability signaling of maximum number of layers - Component 3 is not needed. As we agreed one-to-one mapping between two scrambling IDs and two CORESET pool indices. Once FG 16-2 is supported by UE, two scrambling sequences should be supported. - Remove FFS part: FFS whether default values of component 3/4 to be included in 16-2a |
| Ericsson | Add 16-2a as Pre-requisite. Regarding Component 1, we suggest to remove [infrequency]. Brackets around ‘fully’ can be removed and we can keep ‘fully’. Regarding Component 2, we are fine do remove this. Regarding Component 3, we think it is better to move this to feature 16-2a. The component values should be {1, 2}. The FFS can be removed. Regarding reporting type, please see our detailed comments in feature 16-2a. In essence, we need to consider both UE complexity as well as feedback overhead when it comes to the design of reporting type. In an earlier email thread, Haitong (Apple) mentioned ‘The fundamental issue is that UE may have a total budget in terms of the number TRPs (CCs), or number of MIMO layers, we can support’. This reminds me of a cost + budget based UE capability design that was agreed in R1-1905577 & R1-1904509 to reduce the signaling overhead. I think we can consider a similar design here, but given the limited time we have, we can leave the detailed design to RAN2. So, as a compromise, we can live with FSPC under the condition, the following note is added: Note: To reduce signaling overhead reduction, UE capability design similar to R1-1905577 & R1-1904509 can be further considered in RAN2. Detailed signaling design is up to RAN2. If this compromise is not acceptable to companies, we prefer to stick to our original preference which is ‘Per Band’ reporting. |
| Intel | Component 1: Keep fully and delete “in frequency” Component 2: Either delete or consider restrictions on following layer combinations: {1+3}, {1+4}, {2+4}, here if 1+3 is not supported 3+1 is also not supported Component 3: This is not in the right place (with FSPC granularity), may be better to delete. |

| 16-2a-2 | Out-of-order operation for DL | 1. Support out-of-order operation for PDCCH to PDSCH 2. Support out-of-order operation for PDSCH to HARQ-ACK |  | Yes | N/A |  | Per UE | No | No |  |  | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK We are also fine for “per band” as a compromise. |
| QC | - Type should be per band. UE may support this FG in licensed where Backhaul can be non-ideal while there may be no need for the support of this FG in unlicensed band (e.g. in a factory / indoor environment with fiber backhaul). - Pre-requisite should be 16-2a |
| Samsung | - Type: FSPC signaling is necessary. Out-of-order operation is challenging to support in a UE level considering its excessive complexity. |
| Apple | Optional with capability signalling Per FSPC, we only support this due to MDCI MTRP, and MDCI MTRP is agreed to be per FSPC |
| Spreadtrum | Type: per FSPC Optional with capability signalling |
| OPPO | - Pre-requisite should be 16-2a - Support “per band” or “per FSPC” instead of “per UE” |
| Nokia/NSB | OK in general. Add a note that “UE may need to support this FG with 16-2a, 16-2a-0, and 16-2a-1 when there is non-ideal backhaul conditions”. Regarding FG type we agree “per UE” is sufficient here. In addition, contrary to what has been stated by Apple above, there is no such agreement that MTRP is FSPC. Regarding the licensed/unlicensed differentiation mentioned by Qualcomm, please refer to the conclusions from RAN1#101e. |
| MediaTek | Type: FSPC. Since out-of-order operation requires higher UE processing capability, a UE may be able to support OOO operation together with 16-2a/16-2a-0/16-2a-1 in fewer carriers, compared to no OOO case. We are fine for “per band” as a compromise if 2a/2a-0/2a-1 can be FSPC. |
| ZTE | - Per UE. - Optional with capability signalling. |
| Ericsson | We are fine with Moderator’s proposal, and we prefer ‘Per UE’ signaling as proposed by the moderator. We cannot accept ‘per FSPC’ as reporting type for this Feature. |
| Intel | Ok with Type per-band |

| 16-2a-3 | Out-of-order operation for UL | 1. Support out-of-order operation for PDCCH to PUSCH |  | Yes | N/A |  | Per UE | No | No |  | Note: “Same closed loop index for power control across PUSCHs associated with different CORESETPoolIndex values is not supported by a UE indicating the support of this feature” | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK in principle but the wording of “a UE indicating the support of this feature this feature” shall be changed to “this FG” to be more precise. Secondly, we may need a new RRC parameter, i.e. “EnablingOOOForUL”, since the note itself has strong restrictions at NW side even if the gNB may not enable/need OOO for UL by following Rel-15. The note will mandate two different closed loop indices for gNB implementation by UE capability reporting of 16-2a-3, no matter whether the gNB needs it or not. If accumulation is disabled, the absolute TPC value is still obtained per loop index as in 213. To implement the note, the gNB has to enable twoPUSCH-PC-AdjustmentStates and configure two SRS resources with at least two SRI codepoints. We can further discuss in next RAN1 maintenance, i.e. whether a new RRC is needed. Prefer Per UE |
| QC | Same comments as in the case of 16-2a-2 (should be per band, pre-requisite should be 16-2a). Regarding HW’s comment: Network already has the tools, e.g. TPC accumulation can be disabled by RRC “tpc-Accumulation” |
| Samsung | - Type: FSPC signaling is necessary. Out-of-order operation is too challenging to support in a UE level considering its excessive complexity. |
| Apple | Optional with capability signalling Per FSPC, we only support this due to MDCI MTRP, and MDCI MTRP is agreed to be per FSPC |
| Spreadtrum | Type: per FSPC Optional with capability signalling |
| OPPO | - Pre-requisite should be 16-2a - Support “per band” or “per FSPC” instead of “per UE” - Fine to have a new RRC parameter proposed by Huawei. |
| Nokia/NSB | OK in general. Add a note that “UE may need to support this FG with 16-2a, 16-2a-0, and 16-2a-1 when there is non-ideal backhaul conditions”. Per UE signaling, see comment on 16-2a-2. |
| MediaTek | Type: Per FSPC We are fine for “per band” as a compromise if 2a/2a-0/2a-1 can be FSPC. |
| ZTE | - Per UE. - Optional with capability signalling. |
| Ericsson | We are fine with Moderator’s proposal, and we prefer ‘Per UE’ signaling as proposed by the moderator. We cannot accept ‘per FSPC’ as reporting type for this Feature. |
| Intel | Ok with Type per-band. We don’t see need for new RRC parameter |

| 16-2a-4 | HARQ-ACK for multi-DCI based multi-TRP - separate | - Support of separate HARQ-ACK - The maximum number of long PUCCHs within a slot for separate HARQ-Ack |  | Yes | N/A |  | Per UE | No | No |  | ALT 1) Candidate values for Component 2: {LongAndLong, LongAndShort, ShortAndShort} ALT 2) Candidate values for Component 2: Supported combinations are - short+short; - short+short and short+long short+short, short+long, and long+long | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Either Alt is fine for us. Prefer per UE. |
| QC | Given the majority view, we can accept the “per UE” type for this FG. Pre-requisite should be 16-2a. |
| Samsung | - Type: support per UE. - Candidate value: support Alt 1 |
| Apple | Optional with capability signalling Per UE Alt 1, 3-bit bitmap |
| Spreadtrum | Type: per UE Alt1 Optional with capability signalling |
| OPPO | Support. Pre-requisite should be 16-2a. Alt.1 is preferred |
| Nokia/NSB | Both alternatives are OK. Per UE signaling. |
| MediaTek | Type: Per UE Alt 1, bit-map |
| ZTE | - Per UE. - Optional with capability signalling. - Either fine with Alt1 or Alt2 |
| Ericsson | Per UE is fine. We don’t have a strong preference between Alt-1 and Alt-2. |
| Intel | Slightly prefer ALT2 as it provides balance between flexibility and overhead |

| 16-2a-4a | HARQ-ACK for multi-DCI based multi-TRP - joint | - Support of joint HARQ-ACK |  | Yes | N/A |  | Per UE | No | No |  |  | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK |
| QC | Given the majority view, we can accept the “per UE” type for this FG. Pre-requisite should be 16-2a. |
| Samsung | - Type: support per UE. - Candidate value: support Alt 1 |
| Apple | Optional with capability signalling Per UE |
| Spreadtrum | Optional with capability signalling |
| OPPO | Support. Pre-requisite should be 16-2a. |
| Nokia/NSB | Support moderator proposal |
| MediaTek | Type: Per UE |
| ZTE | - Per UE. - Optional with capability signalling. |
| Ericsson | Support Moderator’s proposal. |
| Intel | OK with moderator proposal |

| 16-2a-5 | Separate CRS rate matching | Whether the UE can rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex | FFS | Yes | N/A |  | Per band | No | No |  |  | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK, since TEI 14-1 has been agreed to be per band this meeting |
| QC | - Pre-requisite should be 16-2a and 14-1a (TEI FG) - We agree with “per band”. Note that Rel. 15 FG related to CRS rate matching (5-28) is also per band. - This should be “FR1 only” (not applicable to FR2 as there is no LTE) |
| Samsung | - Mandatory/optional: already agreed as an optional feature |
| Apple | Optional with capability signalling Per band FR1 only and only for 15kHz SCS |
| LG | Agree with “per band” and “FR1 only” |
| Spreadtrum | Per band FR1 only Optional with capability signalling |
| OPPO | Support. |
| Nokia/NSB | This is ok. No need to add any restrictions as FR1 and 15KHz, as that is anyways the general case. |
| MediaTek | Type: Per band (FR1 only) |
| ZTE | - Per band and FR1 only. - Optional with capability signalling. |
| Ericsson | Ok with Moderator’s proposal. |
| Intel | Pre-requisite should be 16-2a and 14-1a Type per-band is ok |

| 16-2a-6 | Default QCL enhancement for multi-DCI based multi-TRP | Support of default QCL assumption per CORESETPoolIndex |  | Yes | N/A |  | Per band | No | FR2 only |  |  | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK |
| QC | - Pre-requisite should be 16-2a and 16-x (or 16-2c if choose that name) - xDD differentiation should be “N/A” since this is only applicable to TDD (FR2 only) |
| Samsung | - Mandatory/optional: already agreed as an optional feature |
| Apple | Optional with capability signalling Per band |
| LG | Same view with QC |
| Spreadtrum | Per band Optional with capability signalling |
| OPPO | Pre-requisite should be 16-2a and 16-x |
| Nokia/NSB | Support moderator proposal. |
| MediaTek | Type: Per band |
| ZTE | - Per band. - Optional with capability signalling. |
| Ericsson | Support moderator’s proposal. |
| Intel | pre-requisite should be 16-2a and 16-x Type per band is ok |

| 16-2a-7 | Maximum number of activated TCI states | - The maximal number of activated TCI states per CORESETPoolIndex per BWP per CC including data and control - The maximal total number of activated TCI states across CORESETPoolIndex per BWP per CC including data and control FFS whether default values to be included in 16-2a |  | Yes | N/A |  | Per band | No | No |  | Candidate values for Component 1: {1,2,4,8} Candidate values for Component 2: {2,4,8,16} | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK |
| QC | - Pre-requisite should be 16-2a - FFS can be removed. If not signaled, Rel. 15 FGs (FGs 2-4, 2-4a) are followed. |
| Samsung | - Component 1: have default value as 1 if not signalled |
| Apple | Optional with capability signalling Per band |
| Spreadtrum | Per band Optional with capability signalling |
| OPPO | - Remove FFS. - Pre-requisite should be 16-2a |
| Nokia/NSB | Support moderator proposal. |
| MediaTek | Pre-requisite should be 16-2a. Type: Per band (the same as FG2-4). Regarding the FFS, if 16-2a-7 is not signaled, it is not trivial to directly apply FG 2-4 (Support number of active TCI states per BWP per CC, including control and data) to the per-TRP based variable in this FG. Thus we support the default value as 1 (for component-1) if not signaled as Samsung suggested. For component-2, default value is 2. |
| ZTE | - Per band. - Optional with capability signalling. |
| Ericsson | Support moderator’s proposal. |
| Intel | Type per-band is ok |

| 16-x | Simultaneous reception with different Type-D | Supports simultaneous reception with different Type-D [based on multiple spatial domain receiver filters]. This applies to [PDCCHs]/PDSCHs FFS whether default values to be included in 16-2a |  | Yes | N/A |  | Per band | No | FR2 only |  |  | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | 16-x shall be “16-2c” to cover both single DCI and Multi-DCI families, i.e. 16-2a and 16-2b. We can further discuss “[based .. filters]”, whether it is needed next meeting. No strong view. FFS can be removed |
| QC | - xDD differentiation should be “N/A” since this is only applicable to TDD (FR2 only) - FFS and [based on multiple spatial domain receiver filters] can be deleted |
| Samsung | Suggest to delete the FFS part |
| Apple | Optional with capability signalling Per band FR2 only |
| LG | - Revise 16-x as 16-2c” - We are fine with keeping ”based on multiple spatial domain receiver filters” to be clearer that this UE can support simultaneous reception in any case with multiple different Rx beams. If companies are not comfortable with capturing the specific UE implementation, instead of adding “based on multiple spatial domain receiver filters”, we can elaborate the current description as “Supports simultaneous reception with any different Type-D”. It may imply UE has multiple spatial domain receiver filters. |
| Spreadtrum | Optional with capability signalling |
| OPPO | - Remove all yellow highlighted in component description - Support to cover both S-DCI and M-DCI via 16-2c. |
| Nokia/NSB | Multiple spatial domain receive filters can be deleted. FFS can be deleted. Per band indication |
| MediaTek | Remove all yellow highlighted in component description |
| ZTE | Keep ‘based on multiple spatial domain receiver filters’. Otherwise, 16-x is the same as the component 4 in FR2, and it is unclear if MDCI can work in FR2 when UE supports FG16-2 but not support 16-x. |
| Ericsson | ok to remove FFS and [based on multiple spatial domain receiver filters]. |
| Intel | Remove [based on multiple spatial domain receiver filters] as it is UE implementation specific. Revise PDSCHs to PDSCH(s). Not a strong opinion but ok to delete “PDCCH” as its not supported now in specifications and we can revise later in Rel-16 based on discussions. |

| 16-2a-9 | [Simultaneous reception across CCs with Multi-DCI] | [The maximum number of CCs supporting multi-DCI based multi-TRP simultaneously] |  | FSS | FSS |  | FSS | FSS | FSS |  | Note: If the type of 16-2a is agreed to be FSPC or Fs this FG will be removed | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | It is fine to remove this FG, based on the majority view. |
| QC | This component is no longer needed (if type of 16-2a is agreed to be FSPC) |
| Samsung | Have 16-2a as FSPC and remove this FG. |
| Apple | Remove this FG |
| Spreadtrum | Remove |
| OPPO | Remove this FG. |
| Nokia/NSB | Remove this FG, though we note that 16-2a should not be FSPC. |
| ZTE | This FG is needed if FG16-2 is per band. |
| Ericsson | ok to remove this feature |
| Intel | possibly add max BW as another component if FSPC is not agreed for 16-2a |

| 16-2a-10 | Value of BD factor | Value of R for BD/CCE FFS whether default values to be included in 16-2a |  | Yes | N/A |  | Per BC | No | No |  | Component: {1,2} | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | It may have further spec changes, if two BC reports different values, i.e. R=1 in BC #1 and R=2 in BC #2. For the sake of avoiding further spec changes, it can be clearer to be “per UE” If it is agreed to per BC based on the majority, further RAN1 CR is needed next meeting. If FFS is removed, we need a note, e.g. from Samsung. |
| QC | Agree. FFS can be deleted. No need to have a default value similar to Rel. 15 FG 6-5a (pdcch-BlindDetectionCA) where is no default value assumed. |
| Samsung | Have a note to assume default value R as 1 when this component is not reported. |
| Apple | Optional with capability signalling Per BC. Since BDFactorR is configured per CG, when one CG covers multiple BC, it is just minimum of the reported R. UE may not use CA to support all the BC |
| Spreadtrum | FFS whether default values to be included in 16-2a should be removed. ‘Note: R=1 when UE does not report this FG’ is needed, for the sake that current TS38.213 has assumed that R exists no matter whether UE reported this FG. Optional with capability signalling |
| OPPO | Fine to have it as per BC. If agreed, maybe we needs a CR in next meeting. |
| Nokia/NSB | FFS can be deleted. |
| MediaTek | Type: per BC. Each BC may consist of different carrier number and a BC with more CCs requires higher UE PDCCH processing capability. So there could be under reporting issue if the signaling type is per UE. |
| ZTE | - Per UE. - The default value can be 1. Then, this FG can be changed to ‘support BD factor 2’. |
| Ericsson | We have similar view as Huawei on reporting type. |
| Intel | Type Per BC is ok, no need to include default in 16-2a (always reported) |

| 16-2b-0 | Two default beams for single-DCI based multi-TRP | Support of default QCL assumption with two TCI states |  | Yes | N/A |  | Per band | N/A | FR2 only |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK |
| QC | Pre-requisite should be 16-x (or 16-2c name is changed) |
| Samsung | Support |
| Apple | Support |
| LG | Same view with QC |
| Spreadtrum | Support |
| OPPO | Pre-requisite should be 16-2c if agreed |
| Nokia/NSB | Support |
| MediaTek | OK |
| ZTE | OK |
| Ericsson | Support |
| Intel | same view as QC: pre-req is 16-x |

| 16-2b-1 | Single-DCI based SDM scheme | - Support of single-DCI based SDM scheme - FFS Support of DMRS entry {0, 2, 3} |  | Yes | N/A |  | [Per band or per FSPC] per band per BC | N/A | N/A |  | [Candidate values for component (2): {0,2,3}] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | For the sake of design, we may rename component 2 as “Support of DMRS tables”, in a 4-bit bitmap corresponding to tables 7.3.1.2.2-1A, 7.3.1.2.2-2A, 7.3.1.2.2-3A, 7.3.1.2.2-4A |
| QC | - Component 2 can be a new FG. It is very unreasonable to mandate this new DMRS port for SDM scheme. All functionalities can be supported even without DMRS port entry {0,2,3}. We can also accept HW’s suggestion above - We prefer the type to be FSPC dues to the additional complexity. |
| Samsung | - Type: FSPC signaling is necessary considering the high burden to implement multi-TRP at UE. - Component 2: Candidate values need to be changed. Fine with either HW’s or QC’s suggestion. |
| Apple | Per FSPC |
| LG | Regarding component 2, we are ok with a new FG or HW’s suggestion. |
| Spreadtrum | Like FG 16-2a, it should also be per FSPC |
| OPPO | - Support component 2 as a separate FG or Huawei’s proposal. - Per FSPC is preferred. |
| Nokia, NSB | Fine to either removing component 2 or following Huawei’s suggestion. FG type should be per band. We do not agree with FSPC, see comments in 16-2a above. |
| MediaTek | Type: FSPC |
| ZTE | - Per band. If this feature group is per FSPC, gNB have to ensure TCI state of PDCCH is one of two default TCI states of PDSCH when time offset is small. This scheduling restriction is too limited since usually the beam of PDCCH is wider than PDSCH. - Remove component 2. We disagree with HW and QC. In the current spec, the new tables are used if MACCE contains at least one codepoint with two TCI states. That is, once SDM is supported, the new tables 7.3.1.2.2-1A, 7.3.1.2.2-2A, 7.3.1.2.2-3A, 7.3.1.2.2-4A should be supported automatically. |
| Ericsson | For component 2, we are fine with Huawei/HiSi suggestion. Regarding reporting type, please see our detailed comments in feature 16-2a. In essence, we need to consider both UE complexity as well as feedback overhead when it comes to the design of reporting type. In an earlier email thread, Haitong (Apple) mentioned ‘The fundamental issue is that UE may have a total budget in terms of the number TRPs (CCs), or number of MIMO layers, we can support’. This reminds me of a cost + budget based UE capability design that was agreed in R1-1905577 & R1-1904509 to reduce the signaling overhead. I think we can consider a similar design here, but given the limited time we have, we can leave the detailed design to RAN2. So, as a compromise, we can live with FSPC under the condition the following note is added: Note: To reduce signaling overhead, UE capability design similar to R1-1905577 & R1-1904509 can be further considered in RAN2. Detailed signaling design is up to RAN2. If this compromise is not acceptable to companies, we prefer to stick to our original preference which is ‘Per Band’ reporting. |
| Intel | Component 1: This description is unclear – better to go back to clearer description “Support of DCI indication of 2 TCI states by a codepoint and DMRS ports within two CDM groups” Type per band per BC is ok |

| 16-2b-1a | Downlink PTRS | - Support of 2-port DL PTRS |  | Yes | N/A |  | Per band per BC | N/A | N/A |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | It shall be “per band”, same with twoPortsPTRS-UL in Rel-15 |
| QC | Pre-requisite should be 16-2b-1. “Per band” would be also ok for us for this FG |
| Apple | Per band is fine |
| OPPO | - Pre-requisite should be 16-2b-1. - “per band” instead of “per band per BC” |
| Nokia, NSB | Per band |
| MediaTek | Type: Per band |
| ZTE | Per band |
| Ericsson | Per Band is fine. |
| Intel | Type can be per-band |

| 16-2b-2 | Single-DCI based FDMSchemeA | Support of single-DCI based FDMSchemeA |  | Yes | N/A |  | [Per band or per FSPC] per band per BC | No | No |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK |
| QC | We prefer the type to be FSPC dues to the additional complexity. |
| Samsung | - Type: FSPC signaling is necessary considering the high burden to implement multi-TRP at UE. |
| Apple | Per FSPC |
| Spreadtrum | Per FSPC |
| OPPO | Per FSPC is preferred. |
| Nokia, NSB | FG type should be per band. We do not agree with FSPC, see comments in 16-2a above. |
| MediaTek | Type: FSPC |
| ZTE | Per band |
| Ericsson | Regarding reporting type, please see our detailed comments in feature 16-2a. In essence, we need to consider both UE complexity as well as feedback overhead when it comes to the design of reporting type. In an earlier email thread, Haitong (Apple) mentioned ‘The fundamental issue is that UE may have a total budget in terms of the number TRPs (CCs), or number of MIMO layers, we can support’. This reminds me of a cost + budget based UE capability design that was agreed in R1-1905577 & R1-1904509 to reduce the signaling overhead. I think we can consider a similar design here, but given the limited time we have, we can leave the detailed design to RAN2. So, as a compromise, we can live with FSPC under the condition the following note is added: Note: To reduce signaling overhead, UE capability design similar to R1-1905577 & R1-1904509 can be further considered in RAN2. Detailed signaling design is up to RAN2. If this compromise is not acceptable to companies, we prefer to stick to our original preference which is ‘Per Band’ reporting. |
| Intel | We don’t see justification for FSPC signaling |

| 16-2b-3 | Single-DCI based FDMSchemeB | Support of single-DCI based FDMSchemeB |  | Yes | N/A |  | [per FSPC] per band per BC | No | No |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK |
| QC | The type should be FSPC. This is the most complicated single-DCI scheme as the UE needs to perform channel estimation / demodulation / RE de-mapping for two sets of RBs simultaneously. |
| Samsung | - Type: FSPC signaling is necessary considering the high burden to implement multi-TRP at UE. |
| Apple | Per FSPC |
| Spreadtrum | Per FSPC |
| OPPO | Per FSPC is preferred. |
| Nokia/NSB | FG type should be per band. We do not agree with FSPC, see comments in 16-2a above. |
| MediaTek | Per FSPC |
| ZTE | Per band |
| Ericsson | Regarding reporting type, please see our detailed comments in feature 16-2a. In essence, we need to consider both UE complexity as well as feedback overhead when it comes to the design of reporting type. In an earlier email thread, Haitong (Apple) mentioned ‘The fundamental issue is that UE may have a total budget in terms of the number TRPs (CCs), or number of MIMO layers, we can support’. This reminds me of a cost + budget based UE capability design that was agreed in R1-1905577 & R1-1904509 to reduce the signaling overhead. I think we can consider a similar design here, but given the limited time we have, we can leave the detailed design to RAN2. So, as a compromise, we can live with FSPC under the condition the following note is added: Note: To reduce signaling overhead, UE capability design similar to R1-1905577 & R1-1904509 can be further considered in RAN2. Detailed signaling design is up to RAN2. If this compromise is not acceptable to companies, we prefer to stick to our original preference which is ‘Per Band’ reporting. |
| Intel | We don’t see justification for FSPC signaling |

| 16-2b-3a | Single-DCI based FDMSchemeB CW soft combining | - For FDMSchemeB, Support CW soft combining that UE can support |  | Yes | N/A |  | [per FSPC] per band per BC | No | No |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK |
| QC | Pre-requisite should be 16-2b-3 |
| Apple | Support |
| OPPO | Pre-requisite should be 16-2b-3. If 16-2b-3 is per FSPC, it should be per FSPC too. |
| Nokia/NSB | FG type should be per band for consistency |
| MediaTek | Type: FSPC |
| ZTE | Per band |
| Ericsson | Regarding reporting type, please see our detailed comments in feature 16-2a. In essence, we need to consider both UE complexity as well as feedback overhead when it comes to the design of reporting type. In an earlier email thread, Haitong (Apple) mentioned ‘The fundamental issue is that UE may have a total budget in terms of the number TRPs (CCs), or number of MIMO layers, we can support’. This reminds me of a cost + budget based UE capability design that was agreed in R1-1905577 & R1-1904509 to reduce the signaling overhead. I think we can consider a similar design here, but given the limited time we have, we can leave the detailed design to RAN2. So, as a compromise, we can live with FSPC under the condition the following note is added: Note: To reduce signaling overhead, UE capability design similar to R1-1905577 & R1-1904509 can be further considered in RAN2. Detailed signaling design is up to RAN2. If this compromise is not acceptable to companies, we prefer to stick to our original preference which is ‘Per Band’ reporting. |
| Intel | We don’t see justification for FSPC signaling |

| 16-2b-4 | Single-DCI based TDMSchemeA | - Support of single-DCI based TDMSchemeA - Supported maximum TBS size for TDMSchemeA |  | Yes | N/A |  | [Per band or per FSPC] per band per BC | No | No |  | Component 2 candidate values {FFS} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK |
| QC | - Candidate values for component 2: {3, 5, 10, 20} KByte - We prefer the type to be FSPC dues to the additional complexity. |
| Samsung | - Type: FSPC signaling is necessary considering the high burden to implement multi-TRP at UE. |
| Apple | Per FSPC |
| Spreadtrum | Per FSPC |
| OPPO | Prefer per FSPC. |
| Nokia/NSB | FG type should be per band. We do not agree with FSPC, see comments in 16-2a above. |
| MediaTek | FSPC. We are fine with the candidate values suggested by QC. |
| ZTE | Per band |
| Ericsson | Regarding reporting type, please see our detailed comments in feature 16-2a. In essence, we need to consider both UE complexity as well as feedback overhead when it comes to the design of reporting type. In an earlier email thread, Haitong (Apple) mentioned ‘The fundamental issue is that UE may have a total budget in terms of the number TRPs (CCs), or number of MIMO layers, we can support’. This reminds me of a cost + budget based UE capability design that was agreed in R1-1905577 & R1-1904509 to reduce the signaling overhead. I think we can consider a similar design here, but given the limited time we have, we can leave the detailed design to RAN2. So, as a compromise, we can live with FSPC under the condition the following note is added: Note: To reduce signaling overhead, UE capability design similar to R1-1905577 & R1-1904509 can be further considered in RAN2. Detailed signaling design is up to RAN2. If this compromise is not acceptable to companies, we prefer to stick to our original preference which is ‘Per Band’ reporting. |
| Intel | We don’t see justification for FSPC signaling |

| 16-2b-5 | Single-DCI based inter-slot TDM | - Support of single-DCI based inter-slot TDM - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size [according to RepNumR16 in PDSCH-TimeDomainResourceAllocation] - [Maximum number of TCI states] |  | Yes | N/A |  | [Per band or per FSPC] per band per BC | No | No |  | Component 2 candidate values: {FFS} Component 3 candidate values {FFS} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK in general but prefer to remove component 4 |
| QC | - Candidate values for component 2: {2, 4,8,16} (if sub-sampling the RRC values) or {2,3,4,5,6,7,8,16} (if using all possible RRC values) - Candidate values for component 3: {3, 5, 10, 20} KByte - We prefer the type to be FSPC dues to the additional complexity. - In component 3, “[according to RepNumR16 in PDSCH-TimeDomainResourceAllocation]” can be deleted. - Component 4 can be kept without brackets in order to allow the UE to only implement single-TRP based repetition. Candidate values {1,2} |
| Samsung | - Type: FSPC signaling is necessary considering the high burden to implement multi-TRP at UE. - Component 4: Support with candidate values {1, 2} |
| Apple | Per FSPC - Component 2: candidate value set {2, 3, 4, 5, 6, 7, 8, 16} |
| LG | Regarding component 3, “[according to RepNumR16 in PDSCH-TimeDomainResourceAllocation]” can be deleted. |
| Spreadtrum | Type: per FSPC |
| OPPO | - Candidate values for component 2: {2, 4,8,16} - Delete [according to RepNumR16 in PDSCH-TimeDomainResourceAllocation] - Prefer to keep component 4 with candidate value {1,2} - Prefer per FSPC. |
| Nokia/NSB | Candidate values for component 2 can be {2,3,4,5,6,7,8,16} Candidate values for component 3: suggested values of QC are fine. We could also add no restriction maximum size. {3, 5, 10, 20, no restriction} KByte Brackets in component 3 can be removed. Component 4 can be deleted. FG type should be per band. We do not agree with FSPC, see comments in 16-2a above. |
| MediaTek | Type: FSPC Component-3: Add a value of 40 CBs to the set of candidate values for maximum TB size. We are fine with ‘no restriction’ suggested by Nokia. Texts in brackets can be removed Component 4 can be kept without brackets |
| ZTE | Component 4 should be removed. Support of two TCI states should be basic functionality for scheme 4. |
| Ericsson | For component 2, support candidate values {2,3,4,5,6,7,8,16} For component 3, we have similar views as Nokia. Candidate values are {3, 5, 10, 20, no restriction} KByte Component 4 can be deleted. Regarding reporting type, please see our detailed comments in feature 16-2a. In essence, we need to consider both UE complexity as well as feedback overhead when it comes to the design of reporting type. In an earlier email thread, Haitong (Apple) mentioned ‘The fundamental issue is that UE may have a total budget in terms of the number TRPs (CCs), or number of MIMO layers, we can support’. This reminds me of a cost + budget based UE capability design that was agreed in R1-1905577 & R1-1904509 to reduce the signaling overhead. I think we can consider a similar design here, but given the limited time we have, we can leave the detailed design to RAN2. So, as a compromise, we can live with FSPC under the condition the following note is added: Note: To reduce signaling overhead, UE capability design similar to R1-1905577 & R1-1904509 can be further considered in RAN2. Detailed signaling design is up to RAN2. If this compromise is not acceptable to companies, we prefer to stick to our original preference which is ‘Per Band’ reporting. |
| Intel | Component 4: beneficial to keep to allow 1-TCI state dynamic repetition |

| 16-3a | Regular eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=1 - Support of parameter combinations 1-6 - Support of rank 1,2 - [Number of beams L per CSI-RS ports] | 2-35 | Yes | N/A |  | Per band and per BC | N/A | N/A |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Component 4 is not needed due to following agreement. The value of L is a parameter of each parameter combination. Agreement For Rel.16 Type II codebook/CSI, the support for parameter combinations 1, 2, 3, 4, 5, and 6 is mandatory, while the support for parameter combinations 7 and 8 is optional |
| Qualcomm | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {2,4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} Support component 4 similar to Type II and Type II PS, also ok with removing component 4 per previous agreement. |
| Samsung | Component 4 is not needed and can be removed |
| Apple | - Component 1 - Size of the list 7 can be the starting point - Max # of Tx ports in one resource: {2,4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} - Component 4: support since this is allowed in FG2-41/43 - Update component as “Parameter "Lx" (number of beams) in codebook generation, where x is index of Tx ports, corresponding to 4,8,12,16,24 and 32 ports.” - Candidate value set {2, 4, 6} - We can add Note: UE is required to report 4 for at least one of the numbers of Tx ports |
| LG | Component 4 is not needed due to the basic component 2 including L value. |
| OPPO | OK with component 4 |
| Nokia/NSB | - Component 4 is not needed and should be removed as per previous agreement in RAN#98b Agreement [RAN1#98b-Chongqing] Mandatory support for L=2, 4 - Supported without additional UE capability signalling |
| ZTE | We think the supported number of parameter combinations can already inform NW the supported L value, i.e., whether maximum L=6 can be supported. Further, the two triplets in 16-3a and 16-3a-1 have indicated NW UE’s CSI-RS capability for eType II CSI. Hence we are not clear about extra information of this component 4 can provide. Adding this may further make UE’s support of eType II fractional. Hence we are not supportive of add this component. |
| Ericsson | Component 4 should be removed. It causes fragmentation of UEs with different support and will lead to a risk that only UEs with a single and same L value will be configured for Type II. |
| Intel | We support the intention to align with the Rel. 15 Type II codebook capability design. However, it was already agreed in RAN1 that such capability is not supported (see the below agreement from RAN1#98b). Agreement On UE capability issues: - For a UE capable of Rel.16 Type II codebook, agree on the following: - Mandatory support for L=2, 4 - Supported without additional UE capability signaling - Mandatory support for maximum rank of 1 and 2 - FFS whether the support for maximum rank 3 and 4 is mandatory or not - Supported without additional UE capability signaling - Separate UE capabilities for the “regular” Rel.16 Type II and Rel.16 Type II port selection codebooks - Note: for discussion purposes: - “Mandatory” implies that the (sub-)feature is always supported when the UE is capable of Rel.16 Type II codebook. In other words, this feature is considered basic. Rel.16 Type II codebook is a UE optional feature. - “Optional” implies that a separate UE (sub-)capability is needed (hence not necessarily supported) even when the UE is capable of Rel.16 Type II codebook. In other words, this feature is considered advanced. |
| vivo | Component 4 is not needed |

| 16-3a-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=2 | 16-3a | Yes | N/A | If this FG is not reported, UE does not support R=2 | Per band and per BC | N/A | N/A |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK |
| Qualcomm | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {2,4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} |
| Apple | - Component 1 - Size of the list 7 can be the starting point - Max # of Tx ports in one resource: {2,4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} |
| LG | Support |
| OPPO | OK |
| Ericsson | OK |
| vivo | Ok |

| 16-3a-2 | Support of parameter combinations 7-8 | Support of parameter combinations 7-8 | 16-3a | Yes | N/A | UE does not support parameter combination 7-8 | Per Band | N/A | N/A |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK |
| Qualcomm | Candidate value: {‘Support parameter combinations 7-8’, ‘Not support parameter combinations 7-8’} |
| Apple | Support |
| LG | Support |
| OPPO | OK |
| Ericsson | Support |
| vivo | ok |

k

| 16-3a-3 | Support of rank 3,4 | Support of rank 3,4 | 16-3a | Yes | N/A | UE does not support rank 3-4 | Per Band | N/A | N/A |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK |
| Qualcomm | Candidate value: {‘Support rank 3-4’, ‘Not support rank 3-4’} |
| Apple | Support |
| LG | Support |
| Spreadtrum | Support |
| OPPO | OK |
| Ericsson | OK |
| vivo | ok |

| 16-3a-4 | CBSR | 1) CBSR with soft amplitude restriction | 16-3a | Yes | N/A | Only CBSR with hard amplitude restriction is supported | Per Band | N/A | N/A |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK |
| Qualcomm | Candidate value: {‘Support amplitude subset restriction’, ‘Not support amplitude subset restriction’} |
| Apple | Support |
| LG | Support |
| OPPO | OK |
| Ericsson | OK |
| vivo | ok |

| 16-3b | Port selection eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=1 - 6 parameter combinations (combos with L=6 don’t apply) - Support of rank 1,2 - [Number of beams L per CSI-RS ports] | 2-35 | Yes | N/A |  | Per band and per BC | N/A | N/A |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Component 4 is not needed. |
| Qualcomm | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {2,4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} Support component 4 similar to Type II and Type II PS, also ok with removing component 4 per previous agreement. |
| Samsung | Component 4 is not needed and can be removed |
| Apple | - Component 1 - Size of the list 7 can be the starting point - Max # of Tx ports in one resource: {2,4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} - Component 4: support since this is allowed in FG2-41/43 - Update component as “Parameter "Lx" (number of beams) in codebook generation, where x is index of Tx ports, corresponding to 4,8,12,16,24 and 32 ports.” - Candidate value set {2, 4, 6} - We can add Note: UE is required to report 4 for at least one of the numbers of Tx ports |
| LG | Component 4 is not needed due to the basic component 2 including L value. |
| OPPO | OK |
| Nokia/NSB | - Component 4 is not needed and should be removed as per previous agreement in RAN#98b Agreement [RAN1#98b-Chongqing] Mandatory support for L=2, 4 - Supported without additional UE capability signalling |
| ZTE | eType II PS does not support L=6. We don’t see the need to indicate maximum L for L=2 or 4. Further, the two triplets in 16-3b and 16-3b-1 have indicated NW UE’s CSI-RS capability for eType II PS CSI. Hence we are not clear about extra information of this component 4 can provide. Adding this may further make UE’s support of eType II PS fractional. Hence we are not supportive of add this component. |
| Ericsson | Component 4 should be removed. It causes fragmentation of UEs with different support and will lead to a risk that only UEs with a single and same L value will be configured for Type II. |
| Intel | The same comment as for 16-3a |
| vivo | Component 4 is not needed |

| 16-3b-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=2 | 16-3b | Yes | N/A | UE does not support R=2 | Per band and per BC | N/A | N/A |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK |
| Qualcomm | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {2,4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} |
| Apple | - Component 1 - Size of the list 7 can be the starting point - Max # of Tx ports in one resource: {2,4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} |
| LG | Support |
| OPPO | OK |
| vivo | ok |

| 16-3b-2 | Support of rank 3,4 | Support of rank 3,4 | 16-3b | Yes | N/A | UE does not support rank 3-4 | Per Band | N/A | N/A |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK |
| Qualcomm | Candidate value: {‘Support rank 3-4’, ‘Not support rank 3-4’}. |
| Apple | Support |
| LG | Support |
| Spreadtrum | Support |
| OPPO | OK |
| vivo | ok |

| 16-4 | Low PAPR DMRS for DL | Low PAPR DMRS for PDSCH |  | Yes | N/A |  | Alt. 1) Per UE Alt. 2) Per Band | Alt. 1) No Alt. 2) N/A | Alt. 1) No Alt. 2) N/A |  |  | [Optional with capability signaling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Fine for the proposal. Support “per Band”, since CDM group based sequence generation is more complexity than before, we should leave flexible for a UE to implementation for different band, such as FR1 and FR2 can be different. |
| QC | This FG should be “per band”, as a UE may be able to support it in one band, but not in another band. For example, UE can support this feature in licensed band, but not in unlicensed band. This FG should be “optional with capability signaling”. This is a Rel-16 feature on DL DMRS PAPR optimization for several corner cases. We don’t see the need to make this mandatory. |
| Samsung | Type: Support per band. Mandatory/optional: Support optional with capability signaling. We prefer to have the same UE implementation flexibility through 16-4, 16-6a, 16-6b, 16-6c. |
| Apple | Optional with capability signalling |
| Spreadtrum | Optional with capability signaling. |
| OPPO | This FG should be “per band” and “optional with capability signaling” |
| Nokia/NSB | Per UE. For licensed/unlicensed differentiation see the conclusion from RAN1#101e. |
| ZTE | Per UE. Optional with capability signaling |
| Ericsson | The feature is per UE as we don’t see why it has to be per band? Can the proponent explain why it can be supported in one band and not in another. Unclear why unlicensed band have less issue with high PAPR at gNB?? The feature must be mandatory with capability signaling. The reason for mandatory is that this is a critical correction of a flaw in Rel.15 specifications, it is not an add-on enhancement. |
| MediaTek | Optional with capability signaling |
| Intel | Ok with per band and Optional with capability signaling |
| vivo | Per band is fine |

| 16-5a | UL full power transmission mode of fullpower | - Supported UL full power transmission mode of fullpower | 2-13, 2-14 | Yes | N/A |  | FFS: Per FS or Per band or Per band per BC | N/A | N/A |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Fine for it. |
| QC | Similar to what was agreed for mode 1 and mode 2, This FG (mode 0) need to be “per FS”, i.e. per band per band combination. A simple example is that, due to UE’s PA, a UE can only support full power on one band. Suppose this UE can support UL CA on two bands. Then depends the band combination is band A + band B, or band A + band C, UE may choose to do full power on different band. For example, for band A + band B, UE may choose to support UL full power on band A. While for band A + band C, UE may choose to support full power on band C. |
| Apple | Per FS |
| LG | Per FS is fine |
| Spreadtrum | Support |
| OPPO | This FG should be “per FS” |
| Nokia/NSB | Per FS is fine here, but just note that an explanation needs to be provided for RAN2 as for the reason to choosing this FG type. |
| ZTE | Support FL’s update. |
| Ericsson | Agree with Nokia that the explanation should be provided. |
| MediaTek | Per FS |
| Intel | Agree that the granularity for 16-5a should be ‘Per FS’. The granularity for 16-5a, 16-5b and 16-5c should be the same. |
| vivo | Per FS is fine |

| 16-5b | UL full power transmission [mode 1] | - Supported UL full power transmission [mode 1] - [Number of Tx to support mode 1: {2Tx, 4Tx, 2Tx_4Tx}] | 2-13, 2-14 | Yes | N/A |  | Per FS | No | No |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei,HiSilicon | - It’s fine to keep mode 1. - The second component is not needed. UE never report physic antennas to NW, only report the supported max number of ports. Max 2 ports and max 4 ports are already reported in 2-14, so no need to be reported. For 2Tx_4Tx, Mode-1 is not for virtualization based solution, no need for such case for 2Tx_4Tx. |
| QC | The square bracket around mode 1 can be removed. Support to keep component 2 with candidate values {2Tx, 4Tx, 2Tx_4Tx}, because a UE supporting mode 1 can be 2Tx only, 4Tx only, or 4Tx but can fallback to 2Tx. |
| Samsung | Replaced mode 1 with RRC parameter value fullpowerMode1 Component 2: remove the bracket |
| Apple | Remove bracket and keep “mode 1” Keep component 2 |
| LG | To align with 16-5a, suggest to change “fullpower Mode 1”. The second component is not needed. |
| Spreadtrum | Suggest to align with RRC parameter: mode 1 could be replaced with fullpowerMode1 Support component 2 |
| OPPO | 1 open to keep mode 1 or use RRC parameter fullpowerMode1 2 Open to keep Component 2 |
| ZTE | The component 2 “[Number of Tx to support mode 1: {2Tx, 4Tx, 2Tx_4Tx}]” should be removed. Besides, we can support the other FFS parts. |
| Ericsson | Square brackets should be removed around ‘mode 1’ Open to keep component 2 |
| MediaTek | Keep “mode 1” Keep component 2 |
| Intel | Change ‘[mode 1]’ to be ‘mode of fullpowerMode1’ to align with RRC parameters. Component 2 is not necessary. |
| vivo | Mode1 can be replaced with RRC parameter name fullpowerMode1 Support keeping component 2 |

| 16-5c | UL full power transmission mode 2 | - The maximum number of SRS resources in one SRS resource set with usage set to ‘codebook’ for Mode 2: {1, 2, 4} | 2-13, 2-14 | Yes | N/A |  | Per FS | No | No |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Fine |
| Samsung | Replace mode 2 in FG name with RRC parameter value fullpowerMode2 |
| Apple | Support |
| LG | To align with 16-5a, suggest to change “fullpower Mode 2”. |
| Spreadtrum | Suggest to align with RRC parameter: mode 2 could be replaced with fullpowerMode2 |
| ZTE | Support |
| Ericsson | OK |
| MediaTek | Support |
| Intel | Change ‘mode 2’ in the second column to be ‘mode of fullpowerMode2’ to align with RRC parameters. |
| vivo | Mode2 can be replaced with RRC parameter name fullpowerMode2 |

| 16-5c-2 | UL full power transmission mode 2 – SRS resources | - [Number of Tx to support mode 2: {2Tx, 4Tx, 2Tx_4Tx}] - The SRS configuration with different number of antenna ports for Mode 2: {[NULL,] 1_2, 1_4, [2_4], 1_2_4} | 16-5c | Yes | N/A |  | Per FS | No | No |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | - The Component-1 is not needed, since the Component-2 is sufficient to indicate UE capability for the SRS resources configuration for the ports. - [NULL] is not needed, since the FG is optional. - [2_4] should be kept, since UE need to let NW know 2 ports and 4 ports can be configured. If missing, it is difficult to let gNB know 2 different SRS can be 2 port SRS+4 port SRS; - Moreover, it should be clarified that 1_2_4 means that up to 3 different number of SRS resource with 1-port+2-port+4-port can be supported. To Ericsson, 2_4 and 1_2_4 are not same, 2_4 means to support both 2-port and 4-port SRS resources simultaneously, 1_2_4 means to support 1-port and 2-port and 4-port SRS resources simultaneously. I.e., 1-2-4 is higher requirement for a UE capability. So, [2-4] should be kept. |
| QC | Support component 1 with candidate values set {2Tx, 4Tx, 2Tx_4Tx}, because a UE can be 2Tx only, 4Tx only, or 4Tx but can fallback to 2Tx. In component 2, support keep “NULL” and [2_4] in candidate value set, as UE needs such flexibility due to different PAs implementation. |
| Samsung | Component 1: should be moved to 16-5c (similar to 16-5b) Component 2: [2_4] can be removed |
| Apple | Support to keep component 1 For component 2, support to keep NULL, and 2_4 |
| LG | 1. To align with 16-5a, suggest to change “fullpower Mode 2 – SRS resources”. 2. Keep [2_4] candidate value and remove [Null]. |
| Spreadtrum | Suggest to align with RRC parameter: mode 2 could be replaced with fullpowerMode2. Suggest to revise the number 16-5c-2 of FG to be 16-5c-1, only for orderly numbering. Component 1: not needed. The number of Tx could be deduced by component 2. Component 2: keep NULL, 2_4 |
| OPPO | Remove Component 1 since Component 2 can indicate more information. NULL is not needed as the FG is optional. 2_4 is needed |
| ZTE | The component “[Number of Tx to support mode 2: {2Tx, 4Tx, 2Tx_4Tx}]” should be removed, which is redundant considering the existing component-2. Besides, [NULL] and [2_4] can be removed. |
| Ericsson | Component 1 can be removed NULL is not needed: the capability is optional. 2_4 is not needed: why would a full power UE not be able to virtualize to single antenna port with full power? Name should be fixed: ‘UL full power transmission mode 2 – SRS port combinations’ |
| MediaTek | Component 1 can be removed Keep NULL and 2_4 |
| Intel | Change ‘mode 2’ in the second column to be ‘mode of fullpowerMode2’ to align with RRC parameters. Component 1: it is redundant and could be removed. Component 2: The value of component 2 is a bitmap. The value of [NULL] should be removed. The bracket of [2_4] should be removed. It’s better to list all the port combinations so that the gNB could clearly know which configuration is supported by the UE. |
| vivo | Keep component 1 Regarding component2, the agreement is about max number of SRS resources supported, copied below. Thus component 2 should be “maximum number for supported SRS resources for mode2: {2, 4}”, for example for an UE supporting 2 SRS resources gNB can configure either 1-port+2-port or 1-port+4-port or [2-port+4-port] (if gNB prefers to configure) Agreement - For 4 TX UEs, a maximum of 4 SRS resources are supported in Mode 2 for usage set to ‘codebook’ in a set - Depending on UE capability, either up to 2 or 4 SRS resources are supported - For 2 TX UEs, a maximum of 4 SRS resources are supported in Mode 2 for usage set to ‘codebook’ in a set - Depending on UE capability, either up to 2 or 4 SRS resources are supported - For mode 2 UEs, up to 2 different spatial relation info can be configured for all SRS resources with usage set to ‘codebook’ Note: it does not mean to support simultaneous transmission of multiple SRS resources usage is set to ‘codebook’ |

| 16-5c-3 | UL full power transmission mode 2 – fullpower TPMI groups | - TPMI group(s) which delivers full power: {2-port {2-bit bitmap}, 4-port non-coherent {G0~G3}, 4-port partial-coherent {G0~G6}, [FFS: 4-port full-coherent {G0~G6}]} | 16-5c | Yes | N/A |  | Per FS | No | No |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Two notes should be added, where the followings are common understanding from email discussion: - Note: For 4-port full coherent UE, the reporting is the same as 4-port partial coherent UE. For 2-port full coherent UE, the reporting is the same as 2-port non-coherent UE - Note: UE can report both 2-port and 4 port TPMIs simultaneously For “4-port full-coherent {G0~G3}”, it is preferred to be kept, which is more clear for full-coherent UE how to report. |
| QC | Support adding “4-port full-coherent {G0~G6}” in the candidate values set. The reason is that it was already agreed in RAN1 to allow full coherent UE to support mode 2. When a full coherent UE operates in mode 2, the way it reports TPMIs should be the same as a partial-coherent UE, meaning reporting {G0-G6} is sufficient, because by default those full coherent TPMIs can support full power. So there is no need to do additional TPMI reporting for full coherent TPMIs. |
| Samsung | FFS on full-coherent UE: as discussed last week, - 2 port: the TPMI group reporting is the same as that for 2Tx non-coherent - 4 port: the TPMI group reporting is the same as that for 2Tx partial-coherent Regarding TPMI group(s) reporting, in our view, a 4Tx UE can report a 2Tx TPMI group (in addition to a 4Tx TPMI group). Regarding a 4Tx PC UE reporting, 1 PC and 1 NC TPMI groups, there was no consensus in RAN1 (based on extensive discussion last week). As mentioned last week, we are still open to including two pairs (g1,g2) = (G1,G4), (G2,G4). |
| Apple | UE can report all three at the same time: 2-port {2-bit bitmap}, 4-port non-coherent {G0~G3}, 4-port partial-coherent {G0~G6}, |
| LG | 1. To align with 16-5a, suggest to change “fullpower Mode 2 – fullpower TPMI groups”. 2. Remove “[FFS: 4-port full-coherent {G0~G6}]”, since it is the same as 4-port partial coherent TPMI groups. |
| Spreadtrum | Suggest to align with RRC parameter: mode 2 could be replaced with fullpowerMode2. Suggest to revise the number 16-5c-3 of FG to be 16-5c-2, only for orderly numbering. Component 1: sharing the same view with HW. |
| OPPO | “4-port full-coherent {G0-G6}” is not needed as “4-port partial-coherent {G0~G6}” can be used for the full-coherent UE. But we can be open to keep it as well. |
| ZTE | The texting of [FFS: 4-port full-coherent {G0~G6}] should be removed, and, alternatively, we only need to have a conclusion about how to indicate the full power TPMI with lower coherent or lower port, e.g., for partial/ful-coherent 4-Tx UE, and reply to the RAN2 LS (R1-2004936) as given below. 2.1 Interpretation of the support of TMPI(s) for lower configurations in mode-2 operation RAN2 wonders whether the UL full power mode-2 supporting TPMIs for the lower configuration of coherency/port config can be deduced from the reported set of TPMIs, or does the UE need to explicitly report supported TPMIs for each coherency/port config the UE can support as part of it’s capability?. We suggest that the UE needs to explicitly report supported TPMIs for each coherency/port configuration the UE can support as part of its capability, according to the following views. - Regarding the the lower configuration of port config, for the different number of ports, the full power TPMIs for each number of ports cannot be deduced from the reported TPMIs, which needs to be reported separately. Therefore, the UE need to report supported TPMIs for each port config as part of its capability. - Regarding the lower configuration of coherency config, for the case of 4-port partial-coherency, the full power TPMIs exclude the Rank-1 non-coherent TPMIs, which will apparently downgrade the performance of this functionality. For example, if an UE reports G1 with G4 (which means PA = [23 17 23 17] dBm corresponds to antenna ports {1000, 1001, 1002, 1003}) as full power TPMIs. Then, the gNB can indicates the TPMI from G1 for the case of part of antenna ports (such as port {1002}) get hand-blockage. Equally, gNB also can indicate the TPMI from G4 for the case of obtaining up to 3 dB beam-forming gain by transmitting port {1000} and port {1002} with an appropriate TPMI. Therefore, the UE needs to report supported TPMIs for each coherency config as part of its capability. - Besides, in order to make progress, we can live with the existing G0~G6 for 4-TX UE without any further revision or deletion in Rel-16, but the reporting TPMI for non-coherent or partial coherent configuration in 4Tx can be indicated from G0 to G3, and from G0 to G6, independently. Proposal: The UE needs to explicitly report supported TPMIs for each coherency/port configuration the UE can support as part of its capability, in full power mode 2. - The text of “[FFS: 4-port full-coherent {G0~G6}]” is removed in component 1. - Note: The reporting TPMIs should be indicated from G0 to G6, which are as agreed in RAN1 #99 and without further modification in Rel-16. |
| Ericsson | Share Samsung’s understanding that there is no agreement yet to report combinations of 4 port non-coherent and 4 port partially-coherent TPMIs. Using G0-G6 for full coherent operation is fine for us to help close this issue, but 4 port fully coherent UEs should not also report 4 port non-coherent TPMIs. |
| MediaTek | UE should be able to report both 2-port and 4-port options at the same time. The use case is if when 2-port SRS resource and 4-port SRS resource are both configured in RRC for mode 2, it can indicate gNB which TPMIs (for 2-port and 4-port) support full power transmission. |
| Intel | Change ‘mode 2’ in the second column to be ‘mode of fullpowerMode2’ to align with RRC parameters. And ‘fullpower’ should be ‘full power’ Same view as Apple. In addition, remove ‘[FFS: 4-port full-coherent {G0~G6}]’. We can add a note to address the full coherent case. |
| vivo | Square bracket around and FFS can be removed per RAN1 agreement |

| 16-6a | Low PAPR DMRS for PUSCH without transform precoding | - For PUSCH without transform precoding |  | Yes | N/A |  | FFS: Per band | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | OK. Support “per Band”. As we discussed in 16-4, we should leave sufficient flexibility for UE to select the features in different bands to control the complexity. |
| QC | This FG should be “per band”, as a UE may be able to support it in one band, but not in another band. For example, UE can support this feature in licensed band, but not in unlicensed band. |
| Samsung | Type: Support per band. |
| Apple | Support |
| Spreadtrum | Support |
| OPPO | “per band” |
| Nokia/NSB | Per UE. For licensed/unlicensed differentiation see the conclusion from RAN1#101e. |
| ZTE | Agree with Nokia |
| Ericsson | Per UE |
| Intel | OK with “per-band” |
| vivo | Per band is fine |

| 16-6b | Low PAPR DMRS for PUCCH | For PUCCH format 3 and PUCCH format 4 with transform precoding and with pi/2 BPSK modulation | [FG 1-7, 4-4, 4-5] | Yes | N/A |  | FFS: Per band | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | The pre-requisite should be kept, since if no pi/2 BPSK configured, the feature can not be used. FG 1-7 is RAN4 UE feature list 1-7 “pi/2-BPSK for PUCCH format 3/4”. Support “per band”. We should leave sufficient flexibility for UE to select the features in different bands to control the complexity, while the CDM group based sequence generation is more complexity than before. |
| QC | This FG should be “per band”, as a UE may be able to support it in one band, but not in another band. For example, UE can support this feature in licensed band, but not in unlicensed band. Regarding the pre-requisite “[FG 1-7, 4-4, 4-5]”, we don’t see they are necessary and they should be deleted. It seems to us “1-7” is “CSI-RS based RLM”. We don’t see the connection between “FG 1-7” with this FG at all. Maybe proponents of adding these pre-requisites can clarify? “4-4” and “4-5” are PUCCH format 3/4 with frequency hopping enabled. We don’t see the motivation to limit this FG to PUCCH with frequency hopping enabled only. This FG should be applied to both with and without frequency hopping. |
| Samsung | Prerequisite: Support 1-7, 4-4 and 4-5 Type: Support per band. |
| Apple | Support |
| Spreadtrum | Prerequisite: haven’t seen the connection between FG 1-7 and FG 16-6b, but support 4-4 and 4-5; Type: support per band |
| OPPO | “per band” Regarding the pre-requisite, FG 1-7 should be RAN4 UE feature list 1-7 “pi/2-BPSK for PUCCH format 3/4”. |
| Nokia/NSB | Per UE. For licensed/unlicensed differentiation see the conclusions from RAN1#101e. |
| ZTE | OK |
| Ericsson | Per UE |
| Intel | OK with pre-requisites and “per-band” capability |
| vivo | Per band is fine [there are multiple instances of 1-7 in 38.882,e.g. for L1 and RF] |

| 16-6c | Low PAPR DMRS for PUSCH with transform precoding and with pi/2 BPSK | For PUSCH with transform precoding and with pi/2 BPSK modulation | [1-6 and 2-12] | Yes | N/A |  | FFS: Per band | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | The pre-requisite should be kept, since if no pi/2 BPSK configured, the feature cannot be used. FG 1-6 is RAN4 UE feature list 1-6 “pi/2-BPSK for PUSCH” Support “per band”. We should leave sufficient flexibility for UE to select the features in different bands to control the complexity, while the CDM group based sequence generation is more complexity than before. |
| QC | This FG should be “per band”, as a UE may be able to support it in one band, but not in another band. For example, UE can support this feature in licensed band, but not in unlicensed band. Regarding the pre-requisite “1-6 and 2-12”, we don’t see the need of any pre-requisite for this FG. Again, “1-6” is “CSI-RS based RS-SINR measurement”, we don’t see the connection between “1-6” and this FG. “2-12” is a “mandatory without capability signaling” Rel-15 FG. We don’t see the motivation to add it neither, as UE needs to support it anyway. |
| Samsung | Prerequisite: Support 1-6 and 2-12 Type: Support per band. |
| Apple | Support |
| Spreadtrum | Prerequisite: we haven’t seen the connection between FG 1-6 and FG 16-6c, but support 2-12; Type: support per band |
| OPPO | “per band” Regarding the pre-requisite, FG 1-6 should be RAN4 UE feature list 1-6 “pi/2-BPSK for PUSCH”. |
| Nokia/NSB | Per UE, with FRx differentiation. For licensed/unlicensed differentiation see the conclusions from RAN1#101e. |
| Ericsson | Per UE |
| Intel | OK with pre-requisites and “per-band” capability |
| vivo | Per band is fine [there are multiple instances of 1-6 and 2-12 in 38.822, e.g. for L1 and RF] |

| 16-7 | Extension of the maximum number of configured aperiodic CSI report settings | Extension of the maximum number of configured aperiodic CSI report settings for all codebook types | 2-32 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | Candidate values: {1 to 8} |
| Apple | Candidate value set: {1 to 8} |
| Ericsson | A small number of values is necessary to avoid fragmentation of UEs, e.g. 2 different values |

| 16-8 | Active CSI-RS resources and ports for mixed codebook types in any slot | - Report a list of codebook combinations as {codebook 1, codebook 2} - For each codebook combination, report a list of {max number of ports per resource, max number of resources, max number of total ports} | [2-35] | Yes | N/A |  | per band and per BC | N/A | N/A |  | Component-1 candidate values: Codebook 1 = {Type I SP, Type I MP} codebook 2 = {Type II, Type II PS, eType II R=1, eType II R=2, eType II PS R=1, eType II PS R=2 } FFS: whether introduce codebook 3, where codebook 3 is downselected from {Type II, Type II PS, eType II R=1, eType II R=2, eType II PS R=1, eType II PS R=2, NULL} Note 3：if a UE reports one or more codebook combinations in 16-8, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations Note 4: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8 and per-codebook capability 2-36/40/41/43 and 16-3a/b FFS: the max number of combinations can be signaled in component 1 FFS: the minimum requirement for component 2 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Prerequisite: 2-36/2-40/2-41/2-42 in Rel-15, and 16-3a, 16-3b in Rel-16, if applicable FFS the max number of combinations: 3 or 4 FFS the minimum requirement for component 2: max number of ports per resource>=16, max number of resources>=2 |
| Qualcomm | For codebook 3, support the following cases - {codebook 2, codebook 3} = {any, NULL}, {Type II, Type II PS}, {eType II R=1, eType II PS R=1} and {eType II R=2, eType II PS R=2} Support 16 triplets for each codebook combination - Max # of Tx ports in one resource: {2,4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} No minimum requirement for component 2. For component 1, consider 4 codebook combinations as a starting point for further discussion. |
| Samsung | Codebook 3: Our first preference is no support for codebook 3 since the use of configuring 2 Types II codebooks simultaneously is limited. However, if the group wants to support it then we should restrict the candidate values for codebook 3 to reasonable. For instance, one of the following can be considered. Alt1: When Codebook 3 = NULL, codebook 2 = {Type I MP, Type II, Type II PS, eType II R=1, eType II R=2, eType II PS R=1, eType II PS R=2} When Codebook 3 ≠ NULL, (Codebook 2, Codebook 3) = {(Type II, Type II PS)} Alt2: (Codebook 2, Codebook 3) = {(Type II, NULL), (Type II PS, NULL), (eType II R=1, NULL), (eType II R=2, NULL), (eType II PS R=1, NULL), (eType II PS R=2, NULL), (Type II, Type II PS)} Regarding the max number of combinations in component 1, we think try to keep it a min, for example, 8. Regarding the min requirement for component 2, the real system gain (and use case) of Type II CSI for MU MIMO with large number of ports (e.g. 16, 32). So, the min requirement should be such 16 and 32 port CSI-RS are included. Ericsson’s proposal from last week {max number of ports per resource, max number of resources, max number of total ports} = {32,3,72} is good starting point. |
| Apple | Prefer to remove codebook 3, or at least all NULL in codebook 3 Component 2 - A list size of at least 7 - Max # of Tx ports in one resource: {2,4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} No minimum requirement for component |
| LG | Prerequisite FGs: 2-36/40/41/42 and 16-3a/b For codebook 3, prefer not to support configuring 3 codebooks concurrently. For minimum requirement for component, support Ericsson’s proposal as a baseline. |
| Nokia/NSB | - Prerequisites: 2-36/40/41/43 and 16-3a/b - Codebook 3: our first preference is to remove it as the use cases for supporting concurrent codebooks of 3 different types are very limited. However, if the group decides to support it, the number of combinations should be limited to the useful ones without duplications. One simple way to achieve this is with the following candidate values (regular CBs in codebook 2, PS codebooks in codebook 3): CB 2: {Type II, eType II R=1, eType II R=2, NULL} CB 3: {Type II PS, eType II PS R=1, eType II PS R=2, NULL} - System performance gain and use for eType II is primarily MU-MIMO which requires large number of antennas. Hence a suitable minimum number of ports per resource should be introduced in FG16-3a/b |
| ZTE | We think the use case of having codebook 3 is not typical. Thus two codebooks seems sufficient. For the minimum requirement of component 2, the major use case of Type II/eType II related codebooks is for large number of active antennas. Hence we can have the following note. Note: for component 2, at least one triplet in the list support 32 ports per resource. |
| Ericsson | At least 16 or 32 ports per resource is the minimum value to make Type II feature useful. Having a smaller number will risk that Type II CSI is a paper product and make MU-MIMO performance questionable in FDD |
| Intel | - Regarding the prerequisite FG, we are OK to include 2-35. - We support the codebook combination with 3 codebooks at least for per BC signaling since 3 codebook types may be configured in inter-band CA scenario - Regarding the indication of the max number of combinations, we don’t see any use-case for indication of that parameter - Regarding the minimum component for component 2, in our view UE should be mandated to report at least {32, 3, 72} for codebook combinations {Type I, Type II}, {Type I, Type II PS}, {Type I, eType II}, {Type I, eType II PS} since this values corresponds to the main use case scenario for Type II codebooks |

…

# Conclusion

…

# References

- R1-2005097, RAN1 UE features list for Rel-16 NR after RAN1#101-e, Moderators (AT&T, NTT DOCOMO)

- R1-2004285, Summary of UE features for eMIMO, Moderator (AT&T)

- R1-2005033, Summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-01], Moderator (AT&T)

- R1-2005034, Summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-02], Moderator (AT&T)

- R1-2005035, Summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-03], Moderator (AT&T)

- R1-2005036, Summary of email discussion/approval [101-e-NR-eMIMO-UEFeature-04], Moderator (AT&T)