3GPP TSG RAN WG1 #101			                                   					       												                  R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.11.6

Source:	Moderator (AT&T)

Title:	Summary of email discussion/approval [101-e-Post-NR-UE-Features-10]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [101-e-Post-NR-UE-Features-10] after RAN1 #101-e. According to the Chairman’s Notes:

[101-e-Post-NR-UE-Features-10] Email discussion/approval for remaining issues on UE features for NR eMIMO till 8/4 – Ralf (ATT) - Finalize open issues of FG 16-1a-1: component descriptions, component candidate values, default values if any, how CSI-RS is counted when it is configured as CMR without dedicated IMR - Finalize open issues of FG 16-1e: component candidate values - Finalize open issues of FG 16-1g: component descriptions, component candidate values, type - Finalize open issues of FG 16-2a: component descriptions, component candidate values, type, prerequisites, FFS in component (5) - Finalize open issues of FG 16-2a-0: component candidate values, type - Finalize open issues of FG 16-2a-1: component descriptions, type, prerequisites - Finalize open issues of FG 16-2a-2: type - Finalize open issues of FG 16-2a-3: type - Finalize open issues of FG 16-2a-5: note - Finalize open issues of FG 16-2c: component descriptions - Finalize open issues of FG 16-2b-1: component descriptions, component candidate values, type - Finalize open issues of FG 16-2b-2: type - Finalize open issues of FG 16-2b-3: type - Finalize open issues of FG 16-2b-3a: type, prerequisites - Finalize open issues of FG 16-2b-4: component candidate values, type - Finalize open issues of FG 16-2b-5: component descriptions, type - Finalize open issues of FG 16-3a: component descriptions, component candidate values - Finalize open issues of FG 16-3a-1: component candidate values - Finalize open issues of FG 16-3b: component descriptions, component candidate values - Finalize open issues of FG 16-3b-1: component candidate values - Finalize open issues of FG 16-4: down-select between ALT1 and ALT2 - Finalize open issues of FG 16-5b: component descriptions - Finalize open issues of FG 16-5c-2: component descriptions - Finalize open issues of FG 16-5c-3: component descriptions - Finalize open issues of FG 16-6a: type - Finalize open issues of FG 16-6b: type, prerequisites - Finalize open issues of FG 16-6c: type, prerequisites - Finalize open issues of FG 16-7: component candidate values - Finalize open issues of FG 16-8: prerequisites and the following FFS - whether introduce codebook 3, where codebook 3 is downselected from {Type II, Type II PS, eType II R=1, eType II R=2, eType II PS R=1, eType II PS R=2, NULL} - the max number of combinations can be signaled in component 1 - the minimum requirement for component 2

The following was discussed and agreed after RAN1 #101-e within the scope of [101-e-Post-NR-UE-Features-10].

# Summary of email discussion/approval [101-e-Post-NR-UE-Features-10]

The following are the moderator’s proposals based on the views articulated in [2] and the email discussions on the RAN1 reflector prior to agreeing [1]. All proposals are made as revision marks (red with or without strikethrough). Note that all proposals are based on the latest version of the NR UE feature list for Rel. 16 in [1].

For convenience, FGs are at times grouped. Below each FG or group of FGs a table is presented for companies to comment on the moderator’s proposals.

For FG 16-2c, the remaining highlighting in yellow is proposed to be addressed after further progress in the eMIMO maintenance session.

For FG 16-1a-1, some FFS in yellow highlighting are left due to lack of companies’ views in [2]. For those, companies should express their views in this document.

Otherwise, it’s been the moderator’s intention to provide proposals for every outstanding open issue. Hence, the proposals below are either based on the majority view in [2] or based on the latest status of the email discussions in [101-e-Post-NR-UE-Features-07] for those items for which agreements were not possible in [101-e-Post-NR-UE-Features-07] due to lack of time.

Consequently, in many cases, these proposals do not yet have consensus and should thus be understood as starting point for further discussion. To that end, this document serves to collect initial views on the moderator’s proposals which can then lead to further discussions as part of [101-e-Post-NR-UE-Features-10].

| 16-1a-1 | SSB/CSI-RS for L1-SINR measurement | Per slot limitations: - The max number of [unique] SSB/CSI-RS [(1Tx)] for CMR - The max number of CSI-IM/NZP-IMR resources - The max number of CSI-RS (2Tx) resources for CMR Memory limitations: - The max number of SSB/CSI-RS resources as CMR - The max number of CSI-IM/NZP IMR resources Other limitations: - Supported density of CSI-RS (CMR) - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 - Supported SINR measurements: {SSB as CMR with dedicated IMR, CSI-RS as CMR with dedicated [CSI-IM/NZP IMR] configured, CSI-RS as CMR without dedicated IMR configured, [CSI-RS (2Tx) resources for CMR]} | 2-21, 2-22 or 2-23, 2-23a | Yes | N/A |  | Per band | No | No |  | Component 1: Candidate values {8, 16, 32, 64} Component 2: Candidate values {[0,] 8, 16, 32, 64} Component 3: Candidate values {[0, 4,] 8, 16, 32, 64} Component 4: Candidate values {[8,] 16, 32, 64 [, 128]} Component 5: Candidate values {[0,] 8, 16, 32, 64 [, 128]} Component 6: Candidate values {‘1 only’, ‘3 only’, ‘1 and 3’} Component 7: Candidate values {[0, 1, 2, 4,] 8, 16, 32, 64} Component 8: Candidate values FFS Note: For Component 8, UE must at least report support of one [FFS: which one(s)] FFS: How CSI-RS is counted when it is configured as CMR without dedicated IMR | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-1g | Resources for beam management, [pathloss measurement, BFD, and BFR] | - The maximum number of [unique] SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs for any of L1-RSRP measurement, L1-SINR measurement, [pathloss measurement, BFD, RLM] and new beam identification - The maximum number of SSB/CSI-RS/CSI-IM resources configured across all CCs for any of L1-RSRP measurement, L1-SINR measurement, [pathloss measurement, BFD, RLM] and new beam identification | 2-24, 2-31 | Yes | N/A |  | [Per band] [Per BC] [Per UE] | No | No |  | Component-1: candidate value set is {4, 8, 16, 32, 64, 128, FFS} [Component-2: candidate value set is {4, 8, 16, 32, 64, 128, 256, FFS}] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-2a | Multi-DCI based multi-TRP | - The maximum number of CORESETs configured per “PDCCH-Config” - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” - Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency - Maximum number of unicast PDSCHs per CORESETPoolIndex per slot - [PDSCH processing capability for CC] | FFS | Yes | N/A |  |  | No | No |  | Note: A UE may assume that its maximum receive timing difference between the DL transmissions from two TRPs is within a CP Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex FFS: component (5) only applies to UE processing capability #1 Component 1: Candidate values {[2,] 3,4,5} Component 2: Candidate values {1,2,3} Component 4: Candidate values {1,2,4,7} Note: per SCS, similar with Rel-15 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-2a-1 | Overlapping PDSCHs in time and partially overlapping in frequency | - Support PDSCHs with partially[/fully] overlapping REs, i.e. the allocated REs for PDSCH scheduled by DCI in CORESET configured with CORESETPoolIndex = 0 and PDSCH scheduled by DCI in CORESET configured with CORESETPoolIndex = 1 are partially overlapped, with at least one RE | [16-2a-0] | Yes | N/A |  |  | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-2a-5 | Separate CRS rate matching | Whether the UE can rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex | 16-2a and 14-1a | Yes | N/A |  |  | No | FR1 only |  | [Note: only applicable for 15kHz SCS] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-2c | Simultaneous reception with different Type-D | Supports simultaneous reception with different Type-D [based on multiple spatial domain receiver filters]. This applies to [PDCCHs]/PDSCHs |  | Yes | N/A |  |  | N/A | FR2 only |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-2b-1 | Single-DCI based SDM scheme | - Support of single-DCI based SDM scheme - FFS Support of DMRS entry {0, 2, 3} |  | Yes | N/A |  |  | N/A | N/A |  | [Candidate values for component (2): {0,2,3}] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-2b-3a | Single-DCI based FDMSchemeB CW soft combining | - For FDMSchemeB, Support CW soft combining that UE can support | [16-2b-3] | Yes | N/A |  |  | No | No |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-2b-5 | Single-DCI based inter-slot TDM | - Support of single-DCI based inter-slot TDM - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size - [Maximum number of TCI states] |  | Yes | N/A |  |  | No | No |  | Component 2 candidate values: {{2,3,4,5,6,7,8,16}} Component 3 candidate values {{3, 5, 10, 20, no restriction} KByte } Component 3 candidate values: {1,2} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-3a | Regular eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=1 - Support of parameter combinations 1-6 - Support of rank 1,2 - [Number of beams L per CSI-RS ports] | 2-35 | Yes | N/A |  | Per band and per BC | N/A | N/A |  | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {[2,] 4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-3b | Port selection eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=1 - 6 parameter combinations (combos with L=6 don’t apply) - Support of rank 1,2 - [Number of beams L per CSI-RS ports] | 2-35 | Yes | N/A |  | Per band and per BC | N/A | N/A |  | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {[2,] 4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-4 | Low PAPR DMRS for DL | Low PAPR DMRS for PDSCH |  | Yes | N/A |  | Alt. 1) Per UE Alt. 2) Per Band | Alt. 1) No Alt. 2) N/A | Alt. 1) No Alt. 2) N/A |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-5b | UL full power transmission fullpowerMode1 | - Supported UL full power transmission fullpowerMode1 - [Number of Tx to support mode 1: {2Tx, 4Tx, 2Tx_4Tx}] | 2-13, 2-14 | Yes | N/A |  | Per FS | No | No |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-5c-2 | UL full power transmission fullpowerMode2 – SRS resources | - [Number of Tx to support mode 2: {2Tx, 4Tx, 2Tx_4Tx}] - The SRS configuration with different number of antenna ports for Mode 2: {[NULL,] 1_2, 1_4, [2_4], 1_2_4} | 16-5c | Yes | N/A |  | Per FS | No | No |  |  | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-5c-3 | UL full power transmission fullpowerMode2 – full power TPMI groups | - TPMI group(s) which delivers full power: {2-port {2-bit bitmap}, 4-port non-coherent {G0~G3}, 4-port partial-coherent {G0~G6}, [FFS: 4-port full-coherent {G0~G6}]} | 16-5c | Yes | N/A |  | Per FS | No | No |  | Note: When a full coherent UE operates in mode 2, the way it reports TPMIs should be the same as a partial-coherent UE | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-6a | Low PAPR DMRS for PUSCH without transform precoding | - For PUSCH without transform precoding |  | Yes | N/A |  | FFS: Per band | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-6b | Low PAPR DMRS for PUCCH | For PUCCH format 3 and PUCCH format 4 with transform precoding and with pi/2 BPSK modulation | [FG 1-7 (RAN4), 4-4, 4-5] | Yes | N/A |  | FFS: Per band | N/A | N/A |  |  | Optional with capability signalling |
| 16-6c | Low PAPR DMRS for PUSCH with transform precoding and with pi/2 BPSK | For PUSCH with transform precoding and with pi/2 BPSK modulation | [1-6 (RAN4) and 2-12] | Yes | N/A |  | FFS: Per band | N/A | N/A |  |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-7 | Extension of the maximum number of configured aperiodic CSI report settings | Extension of the maximum number of configured aperiodic CSI report settings for all codebook types | 2-32 | Yes | N/A |  | Per band | N/A | N/A |  | Candidate values: {FFS 1 to 8} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-8 | Active CSI-RS resources and ports for mixed codebook types in any slot | - Report a list of codebook combinations as {codebook 1, codebook 2, codebook 3} - For each codebook combination, report a list of {max number of ports per resource, max number of resources, max number of total ports} | [2-35] 2-36/2-40/2-41/2-42 in Rel-15, and 16-3a, 16-3b in Rel-16 | Yes | N/A |  | per band and per BC | N/A | N/A |  | Component-1 candidate values: Codebook 1 = {Type I SP, Type I MP} codebook 2 = {Type II, Type II PS, eType II R=1, eType II R=2, eType II PS R=1, eType II PS R=2 } (Codebook 2, Codebook 3) = {(Type II, NULL), (Type II PS, NULL), (eType II R=1, NULL), (eType II R=2, NULL), (eType II PS R=1, NULL), (eType II PS R=2, NULL), (Type II, Type II PS)} FFS: whether introduce codebook 3, where codebook 3 is downselected from {Type II, Type II PS, eType II R=1, eType II R=2, eType II PS R=1, eType II PS R=2, NULL} Note 3：if a UE reports one or more codebook combinations in 16-8, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations Note 4: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8 and per-codebook capability 2-36/40/41/43 and 16-3a/b FFS: the max number of combinations can be signaled in component 1 Note 5: Up to 4 combinations for component 1 FFS: the minimum requirement for component 2 Component-2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {2,4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-2a | Multi-DCI based multi-TRP | - The maximum number of CORESETs configured per “PDCCH-Config” - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” - Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency - Maximum number of unicast PDSCHs per CORESETPoolIndex per slot - [PDSCH processing capability for CC] | FFS | Yes | N/A |  | FFS FSPC | No | No |  | Note: A UE may assume that its maximum receive timing difference between the DL transmissions from two TRPs is within a CP FFS: component (5) only applies to UE processing capability #1 Component 1: Candidate values {[2,] 3,4,5} Component 2: Candidate values {1,2,3} Component 4: Candidate values {1,2,4,7} Note: per SCS, similar with Rel-15 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-2a-0 | Overlapping PDSCHs in time and fully overlapping in frequency and time | - Support PDSCHs with fully overlapping REs, i.e. the allocated REs for PDSCH scheduled by DCI in CORESET configured with CORESETPoolIndex = 0 and PDSCH scheduled by DCI in CORESET configured with CORESETPoolIndex = 1 are exactly the same REs - The maximal number of PDSCH scrambling sequences per serving cell | 16-2a | Yes | N/A |  | FFS per band | No | No |  | Note: A UE may assume that its maximum receive timing difference between the DL transmissions from two TRPs is within a CP Component 2: Candidate values {[1,] 2} | Optional with capability signalling |
| 16-2a-1 | Overlapping PDSCHs in time and partially overlapping in frequency | - Support PDSCHs with partially[/fully] overlapping REs, i.e. the allocated REs for PDSCH scheduled by DCI in CORESET configured with CORESETPoolIndex = 0 and PDSCH scheduled by DCI in CORESET configured with CORESETPoolIndex = 1 are partially overlapped, with at least one RE | [16-2a-0] | Yes | N/A |  | FFS per band | No | No |  |  | Optional with capability signalling |
| 16-2a-2 | Out-of-order operation for DL | 1. Support out-of-order operation for PDCCH to PDSCH 2. Support out-of-order operation for PDSCH to HARQ-ACK | 16-2a | Yes | N/A |  | FFS per band | No | No |  |  | Optional with capability signalling |
| 16-2a-3 | Out-of-order operation for UL | 1. Support out-of-order operation for PDCCH to PUSCH | 16-2a | Yes | N/A |  | FFS per band | No | No |  | Note: “Same closed loop index for power control across PUSCHs associated with different CORESETPoolIndex values is not supported by a UE indicating the support of this feature” | Optional with capability signalling |
| 16-2a-5 | Separate CRS rate matching | Whether the UE can rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex | 16-2a and 14-1a | Yes | N/A |  | Per band | No | FR1 only |  | [Note: only applicable for 15kHz SCS] | Optional with capability signalling |
| 16-2c | Simultaneous reception with different Type-D | Supports simultaneous reception with different Type-D [based on multiple spatial domain receiver filters]. This applies to [PDCCHs]/PDSCHs |  | Yes | N/A |  | Per band | N/A | FR2 only |  |  | Optional with capability signalling |
| 16-2b-1 | Single-DCI based SDM scheme | - Support of single-DCI based SDM scheme - FFS Support of DMRS entry {0, 2, 3} |  | Yes | N/A |  | [Per band or per FSPC] per band | N/A | N/A |  | [Candidate values for component (2): {0,2,3}] | Optional with capability signaling |
| 16-2b-2 | Single-DCI based FDMSchemeA | Support of single-DCI based FDMSchemeA |  | Yes | N/A |  | [Per band or per FSPC] per band | No | No |  |  | Optional with capability signaling |
| 16-2b-3 | Single-DCI based FDMSchemeB | - Support of single-DCI based FDMSchemeB - Maximum number of CCs in the band (of the BC) in which this scheme is supported |  | Yes | N/A |  | [per FSPC] per band per BC’ | No | No |  | Candidate values for component (2): FFS | Optional with capability signaling |
| 16-2b-3a | Single-DCI based FDMSchemeB CW soft combining | - For FDMSchemeB, Support CW soft combining that UE can support | 16-2b-3 | Yes | N/A |  | [per FSPC] per band per BC’ | No | No |  |  | Optional with capability signaling |
| 16-2b-4 | Single-DCI based TDMSchemeA | - Support of single-DCI based TDMSchemeA - Supported maximum TBS size for TDMSchemeA |  | Yes | N/A |  | [Per band or per FSPC] per band | No | No |  | Component 2 candidate values {3, 5, 10, 20[, no restriction] } KByte | Optional with capability signaling |
| 16-2b-5 | Single-DCI based inter-slot TDM | - Support of single-DCI based inter-slot TDM - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size - [Maximum number of TCI states] |  | Yes | N/A |  | [Per band or per FSPC] per band | No | No |  | Component 2 candidate values: {{2,3,4,5,6,7,8,16}} Component 3 candidate values {{3, 5, 10, 20, no restriction} KByte } | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |

| 16-1e | Pathloss reference RS activation via MAC CE | - The maximum number of configured pathloss reference RSs for PUSCH/PUCCH/SRS by RRC for MAC-CE based pathloss reference RS update | 8-3 | Yes | N/A |  | Per UE | No | No |  | Candidate values for component (1): {[4,] 8, 16, 32, 64} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-2a | Multi-DCI based multi-TRP | - The maximum number of CORESETs configured per “PDCCH-Config” - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” - Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency - Maximum number of unicast PDSCHs per CORESETPoolIndex per slot - [PDSCH processing capability for CC] | FFS | Yes | N/A |  |  | No | No |  | Note: A UE may assume that its maximum receive timing difference between the DL transmissions from two TRPs is within a CP FFS: component (5) only applies to UE processing capability #1 Component 1: Candidate values {[2,] 3,4,5} Component 2: Candidate values {1,2,3} Component 4: Candidate values {1,2,4,7} Note: per SCS, similar with Rel-15 | Optional with capability signaling |
| 16-2a-0 | Overlapping PDSCHs in time and fully overlapping in frequency and time | - Support PDSCHs with fully overlapping REs, i.e. the allocated REs for PDSCH scheduled by DCI in CORESET configured with CORESETPoolIndex = 0 and PDSCH scheduled by DCI in CORESET configured with CORESETPoolIndex = 1 are exactly the same REs - The maximal number of PDSCH scrambling sequences per serving cell | 16-2a | Yes | N/A |  |  | No | No |  | Note: A UE may assume that its maximum receive timing difference between the DL transmissions from two TRPs is within a CP Component 2: Candidate values {[1,] 2} | Optional with capability signalling |
| 16-2b-4 | Single-DCI based TDMSchemeA | - Support of single-DCI based TDMSchemeA - Supported maximum TBS size for TDMSchemeA |  | Yes | N/A |  |  | No | No |  | Component 2 candidate values {3, 5, 10, 20[, no restriction] } KByte | Optional with capability signaling |
| 16-3a | Regular eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=1 - Support of parameter combinations 1-6 - Support of rank 1,2 - [Number of beams L per CSI-RS ports] | 2-35 | Yes | N/A |  | Per band and per BC | N/A | N/A |  | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {[2,] 4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} | Optional with capability signaling |
| 16-3a-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=2 | 16-3a | Yes | N/A | If this FG is not reported, UE does not support R=2 | Per band and per BC | N/A | N/A |  | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {[2,] 4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} | Optional with capability signaling |
| 16-3b | Port selection eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=1 - 6 parameter combinations (combos with L=6 don’t apply) - Support of rank 1,2 - [Number of beams L per CSI-RS ports] | 2-35 | Yes | N/A |  | Per band and per BC | N/A | N/A |  | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {[2,] 4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} | Optional with capability signaling |
| 16-3b-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=2 | 16-3b | Yes | N/A | UE does not support R=2 | Per band and per BC | N/A | N/A |  | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {[2,] 4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {2 to 256} | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |

…

# Conclusion

…

# References

- R1-2005110, RAN1 UE features list for Rel-16 NR updated after RAN1#101-e, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2005113, Summary of email discussion/approval [101-e-Post-NR-UE-Features-07], Moderator (AT&T)