3GPP TSG RAN WG1 Meeting #101-e                    	R1-2004744

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.2.2.3

Source:	Moderator (Huawei)

Title:	Feature lead summary#1 on 101-e-NR-unlic-NRU-HARQ-01 (enhanced Type-2 HARQ-ACK codebook)

Document for:	Discussion and Decision

# Introduction

This document provides updated proposals on issues A5, A7 and A18 that are prioritized for RAN1#101e among the issues identified for the NR-U enhanced Type-2 HARQ-ACK codebook [1].

[101-e-NR-unlic-NRU-HARQ-01] Email discussion/approval on issues A5, A18 and A7 (limited to clarification of “if any”) from R1-2004692 until 5/29; if necessary, endorse associated TPs by 6/4 – David (Huawei)

- Issue A5: nHARQ-ACK definition for power control with enhanced dynamic codebook is missing

- Issue A7: Clarification of whether “if any” refers to RRC configuration or DCI format detection for setting Vtemp2 according to T-DAI for the non-scheduled group when two sub-codebooks (for TB and CBG) are configured

- Issue A18: Handling of DCI format 1_0 indicating a SPS PDSCH release in enhanced dynamic HARQ-ACK codebook

Each sub-section per issue includes an initial FL proposal based on the summary of the submitted Tdocs, and provides a table for collecting companies’ views on the FL’s proposal.

# Issue A5

| A5 | TS38.213 clause 9.1.3.3: nHARQ-ACK definition for power control with enhanced dynamic codebook is missing |
| --- | --- |

FL summary: there are just small differences between the various TPs. Before discussing the detailed TP, companies are requested to provide views on the following principles.

Proposal:

- nHARQ-ACK should be defined for the cases where

  - UE is not provided PDSCH-CodeBlockGroupTransmission for any cell

  - UE is provided PDSCH-CodeBlockGroupTransmission for  <

- Type 2 CB rule for nHARQ-ACK is used separately for each PDSCH group, and SPS PDSCH is considered separately from the two PDSCH groups

- is given by VDAIg(g+1mod2)), when available, for the (g+1)mod2

- nHARQ-ACK should be defined for cases where q=1 and q=0

| Company | Comments on the proposal above |
| --- | --- |
| Nokia, NSB | We would prefer to keep language as close to specification, for example, instead of - nHARQ-ACK should be defined for the cases where - UE is not provided PDSCH-CodeBlockGroupTransmission for any cell - UE is provided PDSCH-CodeBlockGroupTransmission for < We could agree - nHARQ-ACK should be defined for the cases where - A UE is not provided PDSCH-CodeBlockGroupTransmission for each of the serving cells, or for PDSCH receptions scheduled by a DCI format that does not support CBG-based PDSCH receptions, or for SPS PDSCH reception, or for SPS PDSCH release, and if Furthermore, we would like to clarify - Type 2 CB rule for nHARQ-ACK is used separately for each PDSCH group, and SPS PDSCH is considered separately from the two PDSCH groups and is always counted in |
| QC | We can first focus on TB-based HARQ-Ack when feedback for both groups are included. For the following cases, we can simply refer to section 9.1.3.1: - For the case that feedback for only one group is included - For the case UE is provided PDSCH-CodeBlockGroupTransmission: Since the assumption is that TB-based vs CBG-based HARQ-Ack generation follows Section 9.1.3.1 (except that the minor part related to issue A-7), it is not clear if we need to explicitly describe the nHARQ-ACK determination in this case. |
| Samsung | P1. For “nHARQ-ACK should be defined for the cases”, the same cases as Rel-15 should be supported for , - If a UE is not provided PDSCH-CodeBlockGroupTransmission for each of the serving cells, or for PDSCH receptions scheduled by a DCI format that does not support CBG-based PDSCH receptions, or for SPS PDSCH reception, or for SPS PDSCH release, (2) If a UE - is provided PDSCH-CodeBlockGroupTransmission for serving cells; and - is not provided PDSCH-CodeBlockGroupTransmission, for serving cells where P2. Agree that “Type 2 CB rule for nHARQ-ACK is used separately for each PDSCH group, and SPS PDSCH is considered separately from the two PDSCH groups”. P3. Agree that “ is given by VDAIg(g+1mod2)), when available, for the (g+1)mod2” P4. Agree that nHARQ-ACK should include the case of q=1 and q=0, but one common equation would be sufficient, e.g. ， nHARQ-ACK, (g+1)mod 2 equals 0 if q=0 To avoid duplicated description in the standard, it is sufficient to add a summation formula in section 9.1.3.3, e.g. with some clarifications for SPS PDSCH and , and simply refer to section 9.1.3.1 for per PDSCH group handling (to address P1 and P2) |
| ZTE | We agree with the proposals in principle. For the TP, we can simply refer to section 9.1.3.1 for UE is provided PDSCH-CodeBlockGroupTransmission or not and the nHARQ-ACK can be separately defined for each PDSCH group. |
| vivo | Agree with the FL’s proposal in principle. For the determination of , when the last non-fallback DCI is for group 1 and triggers HARQ-ACK reporting for group 0, and there is at least one DCI format 1_0 detected after the last non-fallback DCI, which points to the same PUCCH occasion, may not be given by VDAIg(g+1mod2)), instead, it could be based on DCI format detection by UE. When q=0, nHARQ-ACK could be simply refer to section 9.1.3.1, with pointing out the necessary modification. |
| Intel | P1: OK with a correction. The second sub-bullet should be “≤” P2: Agree P3: Yes if scheduled group in last DCI is group 0. On the other hand, if scheduled group is 1, comments from vivo is valid for consideration. P3: Agree, a common wording to simplify the spec is preferred |
| Ericsson | Agree in principle, while keeping the same wording as in the spec and referring to 9.1.3.1 when only one group is requested. |
| LG | We prefer to simply refer the existing Rel-15 equation as much as possible. In this context, the TP from Samsung (R1-2003862) or the TP from Nokia (R1-2004257) is supportive and acceptable to us. |
| FL summary | Thank you for the feedback. There seems to be broad agreement on the principles, with clarifications on the wording to be closer to specification text, and to clarify that the case of q=0 can directly refer to section 9.1.3.1, and that the case for CBG-based HARQ can also refer to section 9.1.3.1. So it seems easier to try and progress directly based on a simple TP rather than spending much time on wordsmithing the bullet points. I propose to start from Nokia’s TP with some modifications to clarify how to handle SPS PDSCH and to more clearly highlight the cases of q=0 and q=1. Please provide your comments on the draft TP for TS38.213 below. If , the UE determines a number of HARQ-ACK information bits nHARQ-ACK for groups g and g+1mod2 separately as described in Clause 9.1.3.1, except that NSPS,c is counted only once as part of nHARQ-ACK for group g, with the following modification that if q=1 and VDAIg+1mod2≠∅, UE sets = VDAIg+1mod2 for generating nHARQ-ACK for group g+1mod2. If q=0, the UE sets nHARQ-ACK = nHARQ-ACK, g, else UE sets nHARQ-ACK = nHARQ-ACK, g+nHARQ-ACK, (g+1)mod2 for obtaining a transmission power for a PUCCH. |
| Qualcomm | We are basically ok with all three TPs for A5, A7, A18. |
| Nokia, NSB | nHARQ-ACK = nHARQ-ACK, g, The comma should not be part of equation, otherwise it looks like “g’” , otherwise looks good. |
| Intel | As commented above, we need to be careful about VDAIg+1mod2. Under q=1, if current group is 0, VDAIg+1mod2 is suitable; on the other hand, if current group is 1, UE needs to further check if there is a later DCI format 1_0. If so, C-DAI in the last DCI format 1_0 should serve as |
| FL | Corrected the typo pointed out by Nokia: Please provide your comments on the draft TP for TS38.213 below. If , the UE determines a number of HARQ-ACK information bits nHARQ-ACK for groups g and g+1mod2 separately as described in Clause 9.1.3.1, except that NSPS,c is counted only once as part of nHARQ-ACK for group g, with the following modification that if q=1 and VDAIg+1mod2≠∅, UE sets = VDAIg+1mod2 for generating nHARQ-ACK for group g+1mod2. If q=0, the UE sets nHARQ-ACK = nHARQ-ACK, g else UE sets nHARQ-ACK = nHARQ-ACK, g+nHARQ-ACK, (g+1)mod2 for obtaining a transmission power for a PUCCH. @Intel: how critical is it if there is a slight mismatch of in the calculation of group 0? |
| vivo | The determination for is not correct in the case we raised in our original response, i.e., there is at least one DCI format 1_0 detected after the last non-fallback DCI, which points to the same PUCCH occasion. We prefer that NSPS,c is counted for the PUCCH transmission occasion i(g), not as part of nHARQ-ACK for group g, to avoid ambiguity. Besides, “if q=1 and VDAIg+1mod2≠∅, UE sets = VDAIg+1mod2” could be modified as “if q=1 and VDAIg+1mod2≠∅, and if g=0, or, g=1 and no DCI format not including a PDSCH group index field is detected after the last DCI format providing the value of g, UE sets = VDAIg+1mod2” to solve the problem raised by us before. |
| LG | We are OK with the latest FL’s TP. |

| Company | Summary of proposals |
| --- | --- |
| Huawei (R1-2003514) | In NR-U, since the DAI are accumulated within each PDSCH group, and the SPS PDSCH does not belong to any group, if HARQ-ACK feedback for both groups are requested, and OACK+OSR+OCSI<11, UE should determine the number of HARQ-ACK information bits nHARQ-ACK for each group and SPS, separately. The TP is provided as following: TP#4 for TS 38.213 Clause 9.1.3.3 === Unchanged part omitted === If q=0, the UE includes only the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g) elseif q=1 if g = 1 appends the first HARQ-ACK information to the second HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g) else append the second HARQ-ACK information to the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g) end if end if If a UE is not provided PDSCH-CodeBlockGroupTransmission for each of the serving cells, or for PDSCH receptions scheduled by a DCI format that does not support CBG-based PDSCH receptions, or for SPS PDSCH reception, or for SPS PDSCH release, and if , the UE determines a number of HARQ-ACK information bits for obtaining a transmission power for a PUCCH, as nHARQ-ACK=nHARQ-ACK,TB=g=01VDAI,mlastDL(g)-c=0NcellsDL-1UDAI,c(g)modTDNTB,maxDL+c=0NcellsDL-1g=01m=0M-1Nm,creceived+NSPS,c where - NSPS,c is the number of SPS PDSCH receptions by the UE on serving cell for which the UE transmits corresponding HARQ-ACK information in the PUCCH - NTB,maxDL is defined in clause 9.1.3.1 - Nm,creceived is defined in clause 9.1.3.1 with m and M counted separately for each PDSCH group - VDAI,mlastDLg,and UDAI,c(g) are defined in clause 9.1.3.1 except that the numbers are counted separately for each PDSCH group. If VDAIg+1mod2≠∅, VDAI,mlastDL(g+1mod2)= VDAIg+1mod2. If a UE - is provided PDSCH-CodeBlockGroupTransmission for serving cells; and - is not provided PDSCH-CodeBlockGroupTransmission, for serving cells where If , the UE also determines for obtaining a PUCCH transmission power, as described in Clause 7.2.1, with nHARQ-ACK,CBG=g=01VDAI,mlastDL(g)-c=0NcellsDL,CBG-1UDAI,cCBG(g)modTDNHARQ-ACK,maxCBG/TB,max+c=0NcellsDL-1g=01m=0M-1Nm,creceived,CBG where - NHARQ-ACK,maxCBG/TB,max is defined in clause 9.1.3.1 - Nm,creceived, CBG is defined in clause 9.1.3.1 with m and M counted separately for each PDSCH group - VDAI,mlastDLg and UDAI,cCBG(g)are defined in clause 9.1.3.1 except that the numbers are counted separately for each PDSCH group. If VDAIg+1mod2≠∅, VDAI,mlastDL(g+1mod2)= VDAIg+1mod2. === Unchanged part omitted === |
| Samsung (R1-2003862) | Rel-15 equation is reused for each PDSCH group respectively, except the following revision: (1) for group (g+1)mod2, if VDAIg+1mod2≠∅, the last DCI contains T-DAI for group (g+1)mod2 and VDAI,mlastDL should be determined by VDAIg+1mod2. (2) The number of SPS PDSCH receptions is only calculated in group g and set to 0 for group (g+1)mod2 to avoid duplicated calculation. Proposal 1: For enhanced dynamic HARQ-ACK codebook using PUCCH format 2 or PUCCH format 3 or PUCCH format 4, if the number of UCI bits is smaller than or equal to 11, the HARQ-ACK information bits for power control should consist of the HARQ-ACK information bits for both PDSCH groups and SPS PDSCH reception(s) when gNB triggers HARQ-ACK feedback for both PDSCH groups.. TP for clause 9.1.3.3 ------------------ Unchanged part omitted ------------------------ The UE appends the HARQ-ACK information corresponding to SPS PDSCH receptions, if any, as described in Clause 9.1.3.1, after the first and second, if any, HARQ-ACK information. If , the UE determines a number of HARQ-ACK information bits for obtaining a transmission power for a PUCCH, as described in Clause 7.2.1, as where and are determined as in Clause 9.1.3.1 for PDSCH group g and , respectively, except that for group , NSPS,c=0 and if VDAIg+1mod2≠∅, VDAI,mlastDL=VDAIg+1mod2. ------------------ Unchanged part omitted ------------------------ |
| Vivo (R1-2003372) | Proposal 2: For enhanced dynamic codebook, nHARQ-ACK to apply should be the sum of nHARQ-ACK, g across all reported PDSCH group(s) in a PUCCH transmission occasion, i.e. nHARQ-ACK=gnHARQ-ACK, g, when the number of UCI bits for the PUCCH transmission occasion is smaller than or equal to 11. |
| Nokia (R1-2004257) | Proposal 1: Type 2 CB rule for nHARQ-ACK is used separately for each PDSCH group. is given by VDAIg(g+1mod2)), when available, for the non-scheduled PDSCH group. TP for TS38.213: #### 9.1.3.3 Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission ##### <unchanged text omitted > If q=0, the UE includes only the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g) elseif q=1 if g = 1 appends the first HARQ-ACK information to the second HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g) else append the second HARQ-ACK information to the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g) end if end if If , the UE determines a number of HARQ-ACK information bits nHARQ-ACK, k for groups g and g+1mod2 separately as described in Clause 9.1.3.1, with the following modification that if VDAIg+1mod2≠∅, UE sets = VDAIg+1mod2. If q=0, the UE sets nHARQ-ACK = nHARQ-ACK, g, else UE sets nHARQ-ACK = nHARQ-ACK, g+nHARQ-ACK, (g+1)mod2 for obtaining a transmission power for a PUCCH. <unchanged text omitted > |
| Qualcomm (R1-2004445) | If a UE is not provided PDSCH-CodeBlockGroupTransmission for each of the serving cells, or for PDSCH receptions scheduled by a DCI format that does not support CBG-based PDSCH receptions, or for SPS PDSCH reception, or for SPS PDSCH release, and if , and if the UE includes both first and second HARQ-ACK information for multiplexing in the PUCCH, the UE determines a number of HARQ-ACK information bits for obtaining a transmission power for a PUCCH, as described in Clause 7.2.1, as nHARQ-ACK=nHARQ-ACK,TB=j=01VDAI,mlastDL(j)-c=0NcellsDL-1UDAI,c(j)modTDNTB,maxDL+c=0NcellsDL-1j=01m(j)=0M(j)-1Nm(j),creceived+NSPS,c where - NTB,maxDL and NSPS,c are defined in Clause 9.1.3.1. - UDAI,c(j) and Nm(j),creceived are defined in Clause 9.1.3.1 except that the numbers are counted each PDSCH group index j separately. - VDAI,mlastDL(j) is defined in Clause 9.1.3.1 except that it is determined for each PDSCH group index j separately. If VDAIg+1mod2≠∅ and for j=g+1mod2, VDAI,mlastDLj=VDAIg+1mod2. |
| Ericsson (R1-2003845) | Adopt TP in R1-2002532 Proposals might have an issue with counting the HARQ-ACK information corresponding to SPS receptions twice or even not counting them at all since the generation of the codebook for each group excludes the generation of HARQ-ACK information for SPS PDSCH receptions. |
| OPPO | We are fine with the TP from Samsung (R1-2003862) |

# Issue A7

| A7 | TS38.213 clause 9.1.3.3: Clarification of whether “if any” refers to RRC configuration or DCI format detection for setting Vtemp2 according to T-DAI for the non-scheduled group when two sub-codebooks (for TB and CBG) are configured. |
| --- | --- |

Issues A7 addresses the following text in TS38.213 section 9.1.3.3:

If hg+1mod2g=∅ or hg+1mod2g=h(g+1mod2), generate second HARQ-ACK information for PUCCH transmission occasion i(g+1mod2) in a slot, as described in Clause 9.1.3.1, where […] - if VDAIg+1mod2≠∅, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, set Vtemp2=VDAIg+1mod2 for both sub-codebooks, if any.

Companies’ views on the clarification of the interpretation of “if any” are invited:

- Alt1: “if any” refers only to the configuration of PDSCH-CodeBlockGroupTransmission

  - Vtemp2=VDAIg+1mod2  is set for both sub-codebooks if PDSCH-CodeBlockGroupTransmission provided for at least one serving cell.

- Alt2: “if any” refers to DCI format detection for a cell configured with PDSCH-CodeBlockGroupTransmission

  - Vtemp2=VDAIg+1mod2  is set for a sub-codebook only if the UE has detected at least one DCI format scheduling a PDSCH for the sub-codebook.

| Company | Comments on the alternatives above |
| --- | --- |
| Nokia, NSB | We prefer Alt2 because it reduces CB size, however, it is not clear to us what does “at least one DCI format” in Alt2 means. Is it at least one DCI at PDCCH monitoring occasion m=0 or after? |
| QC | We prefer Alt1 as it is more robust against missing DCIs. We would be fine with Alt2 as well to close this issue. |
| Samsung | We prefer Alt 1 as it is more robust. |
| ZTE | We prefer Alt.1 as it is more robust. Alt2 may have some problem if the DCI is miss detected. |
| MediaTek | Prefer Alt 1 due to robustness |
| vivo | Alt1 is slightly preferred for us, because it is more robust against DCI miss-detection, though it may result in more overhead. |
| Lenovo, Motorola Mobility | We prefer Alt 1 for robustness |
| Sharp | We prefer Alt1 for less misalignments between gNB and UE. |
| Intel | Alt 1 is preferred, since Alt 2 may results in wrong codebook size if the only one DCI of the other sub-codebook is missed. |
| Ericsson | We prefer ALT2, we do not need to optimize for the case of all DCI being missed. |
| OPPO | Alt-1 |
| LG | Alt-1 is preferable to close this issue. |
| FL summary | Here is the summary of companies’ preferences: Alt1 (12): Qualcomm, Samsung, ZTE, Sanechips, MediaTek, vivo, Lenovo, Motorola Mobility, Sharp, Intel, OPPO, LG Alt2 (3~4): Nokia, NSB, Ericsson, [also acceptable to Qualcomm] There were questions on details of Alt2. We could try to clarify, but given the large majority of companies supporting Alt1, I would suggest closing the issue with Alt1. For Alt2, the intent was to detect at least one DCI among the monitoring occasions on the cells configured with CBG-based HARQ. Note that there could be cases where just a single DCI scheduled CBG-based PDSCH reception (in this case all DCIs missed means just one DCI missed). So I suggest directly looking at a possible TP without the need to make an agreement on Alt1 first. Looking at TPs corresponding to Alt1 from Huawei and Qualcomm, an even simpler TP could be: TP for TS 38.213 Clause 9.1.3.3 === Unchanged part omitted === - if VDAIg+1mod2≠∅, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, set Vtemp2=VDAIg+1mod2 . If the UE is provided PDSCH-CodeBlockGroupTransmission for NcellsDL,CBG serving cells, set Vtemp2=VDAIg+1mod2 for both sub-codebooks before appending the second sub-codebook to the first sub-codebook === Unchanged part omitted === Companies are invited to comment on the proposal to focus on the majority support for Alt1 and on the proposed TP above. |
| Qualcomm | We are basically ok with all three TPs for A5, A7, A18. |
| Nokia, NSB | 1 missed DCI was not an issue in B8, and suddenly it is an issue here? gNB can always schedule more than one for the sub-codebook. We appreciate that QC is consistent here. |
| vivo | Support the TP from FL. |
| Lenovo, Motorola Mobility | The new TP is fine with us. |
| LG | We are OK with the above FL’s TP. |

| Company | Summary of proposals |
| --- | --- |
| Huawei (R1-2003514) | A potential ambiguity remains in the interpretation of “if any”, which could be interpreted either as configuration of two sub-codebooks or detection of DCIs for both sub-codebooks in the set of monitoring occasions corresponding to the PUCCH occasion. TP#1 for TS 38.213 Clause 9.1.3.3(on top of R1-2003180) === Unchanged part omitted === - the PUCCH transmission occasion i(g+1mod2) is a last one for multiplexing second HARQ-ACK information and it is not after PUCCH transmission occasion i(g) - if VDAIg+1mod2≠∅, - if the UE is provided PDSCH-CodeBlockGroupTransmission for NcellsDL,CBG serving cells, and is not provided PDSCH-CodeBlockGroupTransmission for NcellsDL,TB serving cells , after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, set Vtemp2=VDAIg+1mod2 for both sub-codebooks before appending the second sub-codebook to the first sub-codebook. - Otherwise, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, set Vtemp2=VDAIg+1mod2. === Unchanged part omitted === |
| Vivo (R1-2003372) | It should be determined how to indicate and apply VDAIg+1mod2 when the second HARQ-ACK codebook contains two sub-codebooks In Figure below, when there are a number of consecutive DCI format(s) at the end of a sub-codebook and miss-detected by UE, with the number smaller than 4 which is supposed by NR Rel-15, applying a same total DAI of either the smaller or larger one to two sub-codebooks will cause misalignment between UE and gNB. Proposal: When the RRC parameter NFI-TotalDAI-Included-r16 = enable and two sub-codebooks may be applied, i.e., PDSCH-CodeBlockGroupTransmission is provided at least for a serving cell, indicating separate total DAIs for each sub-codebook respectively for the non-scheduled PDSCH group in a non-fallback DCI format. |
| OPPO (R1-2004087) | Proposal 6: Two T-DAIs for TB sub-codebook and CBG sub-codebook of the non-scheduled PDSCH group can be configured in DCI format 1_1 |
| LG (R1-2004015) | For the case when CBG based PDSCH transmission is configured and T-DAI indication for the non-scheduled PDSCH group is configured for DL DCI, Two T-DAI values are indicated for the non-scheduled PDSCH group: - One value corresponds to TB-based PDSCH. - The other value corresponds to CBG-based PDSCH For the case when CBG based PDSCH transmission is configured and T-DAI indication for both or one of two PDSCH groups is configured for UL DCI, the following is adopted, two T-DAI values are indicated per PDSCH group. - One value corresponds to TB-based PDSCH. - The other value corresponds to CBG-based PDSCH. |
| Mediatek (R1-2001904) | Introduce 2 additional bits for T-DAI field: DAI field in DCI format 1_1 has 8 bits for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and with NFI-TotalDAI-Included-r16 = enable. The 4 MSB bits are the counter DAI and the total DAI for the scheduled PDSCH group. The 4 LSB bits are the total DAI for the non-scheduled PDSCH group, where two bits apply separately for each HARQ-ACK sub-codebook. |
| Nokia (R1-2004257) | Proposal 2: Given that no consensus could be reached on the issue A7 in RAN1#100b, we propose not to discuss issue any more in RAN1#101 |
| Qualcomm (R1-2004445) | The procedures described in Section 9.1.1.3 should be done separately for the two sub-codebooks: --Unchanged part omitted------------------------ If hg+1mod2g=∅ or hg+1mod2g=h(g+1mod2), generate second HARQ-ACK information for PUCCH transmission occasion i(g+1mod2) in a slot, as described in Clause 9.1.3.1, where - the second HARQ-ACK information corresponds to detections of DCI formats each providing a same value of g+1mod2, of h(g+1mod2) and to detections of DCI formats that do not provide a value of g+1mod2, of h(g+1mod2), but are associated with a same value of g+1mod2, of h(g+1mod2), - at least one of the DCI formats provides a h(g+1mod2) value - m=0 corresponds to a PDCCH monitoring occasion, where the UE detects a DCI format that provides a value of g+1mod2 or that is associated with a value of g+1mod2, that is the first PDCCH monitoring occasion after a PDCCH monitoring occasion where the UE detects another DCI format that provides a value different than h(g+1mod2) - the PUCCH transmission occasion i(g+1mod2) is a last one for multiplexing second HARQ-ACK information and it is not after PUCCH transmission occasion i(g) - if VDAIg+1mod2≠∅, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, set Vtemp2=VDAIg+1mod2. --Unchanged part omitted------------------------ The UE appends the HARQ-ACK information corresponding to SPS PDSCH receptions, if any, as described in Clause 9.1.3.1, after the first and second, if any, HARQ-ACK information. If a UE is provided PDSCH-CodeBlockGroupTransmission for at least one serving cell, the procedures described in this Clause are applied separately for the first sub-codebook and the second sub-codebook, where the second sub-codebook is the CBG-based sub-codebook as described in Clause 9.1.3.1. If the HARQ-ACK information is multiplexed in a PUSCH transmission, the HARQ-ACK information is determined as --Unchanged part omitted------------------------ |
| Samsung (R1-2003862) | Regarding how to understand “if VDAIg+1mod2≠∅, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, set Vtemp2=VDAIg+1mod2 for both sub-codebooks, if any”, some companies have some concerns about “if any”. In our understanding, “if any” means a UE is provided PDSCH-CodeBlockGroupTransmission. The UE behaviour is clear that, UE determines HARQ-ACK bits for both CBG and TB sub-codebooks according to the single VDAIg+1mod2, if a UE is provided PDSCH-CodeBlockGroupTransmission for at least one serving cell. gNB may only schedules one sub-codebook for a PDSCH group for a PUCCH, the UE still has to report some bits of NACK for non-scheduled sub-codebook according to VDAIg+1mod2. Although additional UCI overhead may be required, DCI overhead can be reduced. In addition, gNB can control the UCI overhead through a proper scheduling. Observation: No need to further clarify the interpretation of T-DAI in DCI 1_1 for the non-scheduled group when two sub-codebooks (for TB and CBG) are configured. |

# Issue A18

| A18 | Handling of DCI format 1_0 indicating a SPS PDSCH release in enhanced dynamic HARQ-ACK codebook |
| --- | --- |

Companies are invited to provide their detailed comments on the possible TP starting from the proposal in R1-2003658.

| Company | Summary of proposals and further companies’ comments |
| --- | --- |
| MediaTek (R1-2003658) | In NR, DCI format 1_0 is possible to be used for indicating a DL SPS release. However, if UE detects a DCI format 1_0 indicating a DL SPS release, it is not clear in current specification how UE handle the DCI format 1_0 since only defines behavior for PDSCH reception scheduled by DCI format 1_0. We believe that the missing UE behaviour should be also completed in TS38.213 clause 9.1.3.3. Proposal 2: Text proposal 2 is adopted in TS38.213 clause 9.1.3.3 to complete UE behavior to DCI format 1_0 indicating a SPS PDSCH release in enhanced dynamic HARQ-ACK codebook. ====Text Proposal 2 Starts==== #### 9.1.3.3 Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission *** Unchanged text is omitted *** If a UE detects DCI formats with respective PDSCH-to-HARQ_feedback timing field values indicating a same PUCCH transmission occasion and none of the DCI formats that the UE detects after a last PUCCH transmission occasion for g=0 includes a New_Feedback indicator field for g=0, and at least one of the DCI formats is DCI format 1_0, the UE generates HARQ-ACK information only for PDSCH receptions scheduled by detections of DCI format 1_0 and SPS PDSCH releases indicated by detections of DCI format 1_0 by detections of DCI format, as described in Clause 9.1.3.1 or 9.1.3.2 for multiplexing in the PUCCH transmission occasion. If a DCI format indicating a slot for a PUCCH transmission occasion does not include a New_Feedback indicator field, a PDSCH reception scheduled by the DCI format or a SPS PDSCH release indicated by the DCI format is associated with PDSCH group 0 and a value of h(g) associated with the DCI format is set only if h(g) is provided by another DCI format that provides a value of h(g) for PDSCH group 0 and indicates the slot for the PUCCH transmission occasion. *** Unchanged text is omitted *** ===== Text Proposal 2 Ends==== |
| Qualcomm | This is editorial, and there may be easier ways, e.g. not mention “PDSCH reception” |
| Nokia, NSB | We prefer MTK clarification |
| QC | Fine with the TP in principle. Our preference is to simplify the text and not mention PDSCH. This is also aligned with other parts of the spec (note that in the previous part of the spec, the DCI format is associated with group 0, and not the scheduled PDSCH: e.g., “and to detections of DCI formats that do not provide a value of g and h(g) and are associated with a same value of g, of h(g)”) If a UE detects DCI formats with respective PDSCH-to-HARQ_feedback timing field values indicating a same PUCCH transmission occasion and none of the DCI formats that the UE detects after a last PUCCH transmission occasion for g=0 includes a New_Feedback indicator field for g=0, and at least one of the DCI formats is DCI format 1_0, the UE generates HARQ-ACK information only in response to detections of DCI format 1_0, as described in Clause 9.1.3.1 or 9.1.3.2 for multiplexing in the PUCCH transmission occasion. If a DCI format indicating a slot for a PUCCH transmission occasion does not include a New_Feedback indicator field, the DCI format is associated with group 0 and a value of h(g) associated with the DCI format is set only if h(g) is provided by another DCI format that provides a value of h(g) for group 0 and indicates the slot for the PUCCH transmission occasion. |
| Samsung | Fine with MTK’s proposal. |
| ZTE | We are fine with the TP. |
| vivo | It is just an editorial issue. We are fine with the TP from MTK, and the TP from QC can also be acceptable, as long as all necessary cases for HARQ-ACK of DCI format 1_0 are covered |
| Lenovo, Motorola Mobility | MTK’s proposal is fine with us. |
| Sharp | We are Okay with the proposed TP and we also think that the expressions in Qualcomm’s TP are clear and more concise. Anyway, it is editorial. |
| Intel | Fine with TP from MTK. |
| Ericsson | Fine with the TP |
| OPPO | Agee with MTK’s TP (R1-2003658) |
| LG | Fine with the TP |
| FL summary | Thank you for all the feedback. All companies are fine with a simple correction. I am not sure why it is referred to as editorial by several companies. The motivation from MediaTek is to include HARQ information for SPS release in enhanced Type-2 codebook. Without a correction, it would not be included so it is not simply editorial. If my recollection is correct, the reason (brought up by Sharp) we used “PDSCH receptions” in the TP endorsed at RAN1#100-e was to include SPS PDSCH receptions. So the TP from Qualcomm might have an issue with SPS PDSCH receptions. If that’s the case, can we simply go with MediaTek’s TP? MediaTek’s TP is re-written on top of the CR endorsed in R1-2003180: ====Text Proposal Starts==== #### 9.1.3.3 Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission *** Unchanged text is omitted *** If a UE detects DCI formats with respective PDSCH-to-HARQ_feedback timing field values indicating a same PUCCH transmission occasion and none of the DCI formats that the UE detects after a last PUCCH transmission occasion for g=0 includes a New_Feedback indicator field for g=0, and at least one of the DCI formats is DCI format 1_0, the UE generates HARQ-ACK information only for PDSCH receptions scheduled by detections of DCI format 1_0 and SPS PDSCH releases indicated by detections of DCI format 1_0, as described in Clause 9.1.3.1 or 9.1.3.2 for multiplexing in the PUCCH transmission occasion. If a DCI format indicating a slot for a PUCCH transmission occasion does not include a New_Feedback indicator field, a PDSCH reception scheduled by the DCI format or a SPS PDSCH release indicated by the DCI format is associated with PDSCH group 0 and a value of h(g) associated with the DCI format is set only if h(g) is provided by another DCI format that provides a value of h(g) for PDSCH group 0 and indicates the slot for the PUCCH transmission occasion. *** Unchanged text is omitted *** ====Text Proposal Starts==== |
| Qualcomm | We are basically ok with all three TPs for A5, A7, A18. Regarding your comment on A18, we still think that it is editorial since HARQ-Ack for SPS release has always been supported in enhanced type 2 codebook |
| Nokia, NSB | I suppose FL meant: “…information for SPS release in e-Type-32 codebook … or it is not clear to us, how 9.1.3.3 text is related to TYPE-3 CB. David, could please clarify? |
| FL | Correct, it was a typo. I meant enhanced type-2 codebook (fixed above) |
| vivo | We thought it as an editorial issue because HARQ-ACK for SPS release is supported in dynamic codebook and enhanced dynamic codebook as a common understanding. Anyway, we support the TP from MTK. |
| Lenovo, Motorola Mobility | We think including A/N for SPS release in HARQ-ACK codebook for PDSCH group 0 is simple. We support this proposal. One question from our side is this seems not a simple editorial change, so do we need to reach agreement firstly before discussing the TP? |
| LG | We are OK with the FL’s proposal. |

# Conclusions

Issue A5: nHARQ-ACK definition for power control with enhanced dynamic codebook is missing

Proposal 1: adopt TP1 for 38.213 v16.1.10

| Reason for change | nHARQ-ACK definition for power control with enhanced dynamic codebook is missing |
| --- | --- |
| Summary of changes | Introduction of nHARQ-ACK definition for power control with enhanced dynamic codebook |
| Specs/Sections impacted | 38.213 v16.1.10 section 9.1.3.3 |
| Consequences if not approved | The PUCCH power control when the number of HARQ-ACK information bits is less than or equal to 11 is undefined. |

================== Beginning of TP1 ===================

9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

*** Unchanged text omitted ***

If , the UE determines a number of HARQ-ACK information bits nHARQ-ACK for groups g and g+1mod2 separately as described in Clause 9.1.3.1, except that NSPS,c is counted only once as part of  nHARQ-ACK for group g, with the following modification that if q=1 and VDAIg+1mod2≠∅, UE sets = VDAIg+1mod2 for generating nHARQ-ACK for group g+1mod2. If q=0, the UE sets nHARQ-ACK = nHARQ-ACK,  g else UE sets nHARQ-ACK = nHARQ-ACK,  g+nHARQ-ACK,  (g+1)mod2 for obtaining a transmission power for a PUCCH.

*** Unchanged text omitted ***

================== End of TP1 ===================

Issue A7: Clarification of whether “if any” refers to RRC configuration or DCI format detection for setting Vtemp2 according to T-DAI for the non-scheduled group when two sub-codebooks (for TB and CBG) are configured

Proposal 2: adopt TP2 for 38.213 v16.1.10

| Reason for change | When two sub-codebooks (for TB and CBG) are configured and the UE is provided with pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16, it is unclear if UE should set Vtemp2 according to the condition that two sub-codebooks are configured or based on actual detection of DCI formats scheduling cells configured for each sub-codebook. |
| --- | --- |
| Summary of changes | Clarify that setting of Vtemp2 is done according to the condition that two sub-codebooks are configured. |
| Specs/Sections impacted | 38.213 v16.1.10 section 9.1.3.3 |
| Consequences if not approved | UE and gNB may not set Vtemp2 in the same way, resulting in mismatched sizes of reported and expected enhanced type-2 HARQ-ACK codebook. |

================== Beginning of TP2 ===================

9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

*** Unchanged text omitted ***

-	if VDAIg+1mod2≠∅, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, set Vtemp2=VDAIg+1mod2 . If the UE is provided PDSCH-CodeBlockGroupTransmission for  NcellsDL,CBG serving cells, set Vtemp2=VDAIg+1mod2  for both sub-codebooks before appending the second sub-codebook to the first sub-codebook

*** Unchanged text omitted ***

================== End of TP2 ===================

Issue A18: Handling of DCI format 1_0 indicating a SPS PDSCH release in enhanced dynamic HARQ-ACK codebook

Proposal 3: adopt TP3 for 38.213 v16.1.10, which is written on top of R1-2003180 endorsed CR_38.213_NRU_post RAN1#100b-e.

| Reason for change | DCI format 1_0 can be used for indicating a DL SPS release. However, if UE detects a DCI format 1_0 indicating a DL SPS release, it is not clear how UE handles the HARQ-ACK information in enhanced Type-2 codebook since a behavior is only defined for PDSCH reception scheduled by DCI format 1_0. |
| --- | --- |
| Summary of changes | Add a behavior for handling HARQ-ACK information for DL SPS release in enhanced Type-2c codebook when DL SPS release is indicated in a DCI format 1_0. |
| Specs/Sections impacted | 38.213 v16.1.10 section 9.1.3.3 |
| Consequences if not approved | The UE cannot report HARQ-ACK information for DL SPS release in enhanced Type-2 codebook when the DL SPS release is signaled in a DCI format 1_0. |

================== Beginning of TP3 ===================

9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

*** Unchanged text is omitted ***

If a UE detects DCI formats with respective PDSCH-to-HARQ_feedback timing field values indicating a same PUCCH transmission occasion and none of the DCI formats that the UE detects after a last PUCCH transmission occasion for g=0 includes a New_Feedback indicator field for g=0, and at least one of the DCI formats is DCI format 1_0, the UE generates HARQ-ACK information only for PDSCH receptions scheduled by detections of DCI format 1_0 and SPS PDSCH releases indicated by detections of DCI format 1_0, as described in Clause 9.1.3.1 or 9.1.3.2 for multiplexing in the PUCCH transmission occasion.

If a DCI format indicating a slot for a PUCCH transmission occasion does not include a New_Feedback indicator field, a PDSCH reception scheduled by the DCI format or a SPS PDSCH release indicated by the DCI format is associated with PDSCH group 0 and a value of h(g) associated with the DCI format is set only if h(g) is provided by another DCI format that provides a value of h(g) for PDSCH group 0 and indicates the slot for the PUCCH transmission occasion.

*** Unchanged text omitted ***

================== End of TP3 ===================

# References

R1-2004692 FL summary_1 for 72223 NRU HARQ moderator (Huawei), RAN1#101-e

R1-2003372	Remaining issues on HARQ operation for NR-U	vivo

R1-2003452	Remaining issues on the HARQ for NR-U	ZTE, Sanechips

R1-2003514	Maintenance on HARQ-ACK enhancement	Huawei, HiSilicon

R1-2003658	Remaining issues on HARQ operation for NR-U	MediaTek Inc.

R1-2003730	Enhancements to HARQ for NR-unlicensed	Intel Corporation

R1-2003823	Text proposals for HARQ enhancement for NR-U	Lenovo, Motorola Mobility

R1-2003845	HARQ enhancement	Ericsson

R1-2003862	HARQ enhancement for NR-U	Samsung

R1-2004015	Remaining issues of HARQ procedure for NR-U	LG Electronics

R1-2004087	Discussion on the remaining issues of HARQ enhancements	OPPO

R1-2004257	Remaining issues on NR-U HARQ scheduling and feedback	Nokia, Nokia Shanghai Bell

R1-2004325	Remaining issues and corrections on HARQ enhancement for NR-U	Sharp

R1-2004445	TP for Enhancements to Scheduling and HARQ Operation for NR-U	Qualcomm Incorporated

R1-2004529	Text proposal for enhanced dynamic HARQ procedures	Google Inc.