Proposal 8:

- Down selection on the following options for the link budget template in next meeting.

- Option 1: Adopt single link budget template based on IMT-2020 self-evaluation with necessary revisions, including adding/removing/revising some parameters.

  - FFS: The template provided by FL or Qualcomm.

- Option 2: Adopt both templates, i.e. link budget template in IMT-2020 self-evaluation and link budget template in TR 36.824.

- Option 3: Adopt single link budget template in TR 36.824 with necessary revisions, including adding/revising some parameters.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Nokia | Option 1. Two alternatives should have a Tdoc number maybe. This would simplify future discussions. Maybe we could also discuss if including the MIL or not. |
| ZTE | Option 1 is trying to combine the templates from ITU and TR 36.824. As a super set, it could include both Option 2 and Option 3. So, we think it should be ok to only keep Option 1 here. As for the FFS, we prefer the template from FL. However, I still don't understand why we need list MIL as an output. MCL can be easily converted to MIL w/ additionally considering transmitter/receiver antenna gains. So, it may no need to explicitly list MIL here. Of course, we can further discuss the FFS if we go with Option 1. |
| CATT | Option1. Option 1 has been well developed in IMT 2020 self-evaluation and is sufficient for coverage evaluation. |
| Samsung | Support option 1. Since we think HW link budget value used in IMT2020 is enough to identify the baseline coverage performance and target performance for this SI, we are not sure the necessity of MIL. In option 1, we prefer the template provided by FL without MIL. |
| Qualcomm | We are okay to go with Option 1. MIL is identified separately as some companies had concerns on how antenna gain is to be accounted for. Regarding Note 2 in Row 63 of FL table, our understanding on tx/rx array gain is that this reflects what we gain from operations like digital beamforming/combining. We are not sure if Note 2 is aligned with this understanding. The description “mapping between antenna elements and corresponding TXRU” seems to point to what is referred to as antenna gain in the IMT 2020 table. For example, we we group 8 antenna elements to form a single TXRU, this composite 8-element antenna has a certain antenna gain, and this is what is to be used as input to antenna gain in the IMT 2020 table. |
| Ericsson | We tend to prefer option 3 as a starting point. The ITU template has 67 rows, many containing free parameters that need to be identified. Rather than remove all the parameters that are not relevant to the NR coverage enhancement study from the ITU template, we prefer to add a small number of values that are needed to the 36.824 template. This should be more time efficient. |
| NTT DOCOMO | We prefer option 3 as in the same reason with Ericsson. |
| InterDigital | We also support option 3. |
| KDDI | We support option 3. |
| IITH, CEWIT, IITM, Tejas Networks, Reliance Jio | Option 1 with the FL proposal. |
| Huawei, HiSilicon | Support the proposal although we prefer Option 1. |

Proposal 8a:

Down selection on the following options for antenna array gain.

- Option 1: Antenna array gain is included in the link budget template.

- FFS: array gain = 10 * 1og10 (number of receive antennas/number of receive TxRUs)

- FFS: For TDL channel model

- Option 2: Antenna array gain is included in LLS.

- FFS: For CDL channel model

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Nokia | We think Option 1 is a good starting point, but may be very optimistic. We suggest to add a further FFS for option 1, such as: “FFS: receiver implementation margin to model non-idealities and imperfections”. Additionally, we should ensure different companies use same naming scheme and terminology. |
| ZTE | To be clear, we suggest to add ‘ for FR1 based on LLS based methodology’ at the end of the main bullet. As for the two options, we prefer Option 1. |
| CATT | Option 1. Not sure what the second sub-bullet under option 1 means. Open to discuss but need make it clear. |
| Samsung | We support Option 1 for at least FR1. |
| Qualcomm | We strongly recommend Option 2. It is important to not take array gain (from digital beamforming/combining) for granted, especially for cell-edge UEs. As Nokia notes above, it leads to very optimistic results. MMIMO deployments in the 4 GHz band form an important component of worldwide NR adoption. If there are coverage issues with such deployments it is better to uncover them now. Simulating 64 TXRUs at the gNB forms an important aspect of such an analysis. Due to the larger number of TXRUs, MMIMO gNBs can have very low SINRs per TXRUs, making channel estimation for beamforming/combining very challenging. It would be an over simplification if we go with a static/fixed gain. Diversity gains and channel hardening is another aspect that we will completely miss if we decide to go with Option 1. This decision has major implications for what enhancements we prioritize, and if we don’t do this right, we could come to misleading conclusions. Let us not reduce this to an academic exercise. |
| Ericsson | Option 1 should be used, but agree 100% with Nokia to avoid optimistic models. We’d propose something a bit more general than Nokia: ‘Values reflective of realistic implementation and network operation should be used’ |
| NTT DOCOMO | We support Option 1. |
| InterDigital | We support Option 1. |
| IITH, CEWIT, IITM, Tejas Networks, Reliance Jio | Option 1 and same views as Nokia and Ericsson. |
| Huawei, HiSilicon | Support the proposal. TDL channel is sufficient for FR1. |

Proposal 9:

Identify the target performance and coverage bottlenecks based on target performance metric.

- FFS: the target performance metric.

- Option 1: The target path loss is considered as the target performance.

  - Derived from the target ISD.

- Option 2: The target MCL is considered as the target performance.

  - Alt1: Derived from the target ISD, considering shadow fading margin, penetration loss, etc.

  - Alt2: Fixed target MCL, e.g. 147dB for VoIP

  - Alt3: Relative MCL

- Option 3 (optional): The target performance based on SLS is determined by the 5th percentile SINR value in CDF curve for different physical channels.

- Other target performance metrics are not precluded.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Nokia | Given that only one option is given for target performance based on SLS, shouldn’t it be better to remove Option3 and add a bullet that says: “If optional SLS is performed, the target performance for SLS is determined by the 5th percentile SINR value in CDF curve for different physical channels ” ? |
| Intel | We share similar view as Nokia that it is unclear to us why MIL is used in the FL link budget template. In proposal 9, none of the performance metrics is based on MIL. We suggest to only keep the MCL calculation in the FL link budget template. |
| ZTE | We are fine with Nokia’s suggestion. |
| CATT | We are fine with the proposal but think it’s better to make it clear that we don’t expect to use all of the three options. We still suggest to modify the main bullet as following: ‘Down selection on the following options to identify the target performance and coverage bottlenecks based on target performance metric’ |
| Samsung | We are fine with Nokia’s suggestion. |
| Qualcomm | We are fine with the proposal. It is not clear if down selection is way to go at this point. We suggest keeping it as it is. |
| Ericsson | Regarding MIL: We think it is critical to include antenna gain (as well as interference margin), so if we have MCL then including MIL is natural, while if we have MIL and antenna gain, MCL can come for free. MIL is anyway in the IMT template, so I wonder why this is an issue. Overall, we can agree to identify the coverage bottlenecks via relative performance measures at this stage, since that is needed to decide on enhancements. Measures of targets, i.e. absolute performance will naturally require more effort, and the suitability of the targets will need to be debated. So we would prefer to further discuss target performance & metrics. |
| NTT DOCOMO | We support relative metric for the target performance, since it may be difficult to define common parameters, so that absolute value may be difficult to define. |
| InterDigital | We support Option 2. We can rephrase Option 2 as “The target MCL An MCL or MCL based metric is considered as the target performance” since the original wording is more suitable for Alt1/2. We are also fine to postpone the discussion for the next meeting. |
| KDDI | We support Option 2. However, considering the situation at this stage, we are fine with Qualcomm’s suggestion. |
| IITH, CEWIT, IITM, Tejas Networks, Reliance Jio | Is there a down selection? If not the proposal is fine. |
| Huawei, HiSilicon | Support the proposal. Option 3 is not preferred because its 5%-ile SINR highly depends on traffic load level. |

Proposal 5:

- For link level simulation, adopt the following table for PUSCH for FR1.

| Parameters | Values |
| --- | --- |
| BLER for PUSCH | 10% iBLER for eMBB, 2% rBLER for voice. |
| Number of UE transmit antennas chains for PUSCH | 1 or 2 |
| Number of UE TRXUs for PUSCH | 1 or 2 |
| DMRS configuration for PUSCH | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For 30km/h and 120km/h: Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping, Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. |
| Waveform for PUSCH | DFT-s-OFDM |
| Number of repetitions for PUSCH | For eMBB, no repetition is assumed. For VoIP, the maximum number of repetitions can be is 2/4/8. |
| HARQ configuration for PUSCH | For eMBB, no retransmission is assumed. For VoIP, the maximum number of HARQ transmission can be reported by companies. can be 2/4/8 depends on the frame structure for TDD and latency requirements for VoIP. |
| PUSCH duration | 14 OS |

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Nokia | From our perspective one important factor is still missing from the proposal. Both the number of repetitions and retransmissions must be compliant with the considered TDD frame structure. For instance, when DDDSU is considered and we consider a 20ms arrival rate for VoIP packets only 2 repetitions and 4 retransmissions are possible for FR1, if we do not want to incur latency issues. For this reason, we fear that not considering these important limitations may yield very unreliable results with scarce practical relevance. Concerning the DMRS number we do not see why we should remove the possibility of considering 1 DMRS symbol for 3km/h. Why do we want to prevent ourselves from considering a case in which code rate can be lowered at the cost of lower channel estimation quality? There is clearly a trade-off to be harnessed there, and by setting a default static value is a very strange choice. Please note this does not mean we are suggesting that each company should present results for both values, it’s actually the converse. In our view, it is wise to limit the choices to 2 values per UE speed scenario, and each company can then present results for the best configurations according to the simulations the company itself has performed. As we discussed above, we suggest to consider also 1 DMRS symbol for the 3 km/h case. The benefit of having 2 DMRS in this case is not clear, if suitable LA is performed, i.e., if a suitable MCS index and number of PRBs is chosen. We also suggest to add that both the max number of repetitions and retransmissions shall be compatible with the limitations imposed by the considered frame structure. |
| Intel | As we commented in GTW meeting, we also need to consider DMRS positions for DMRS configurations. We think Type B mapping could be a good starting point. |
| ZTE | We think one DMRS should also be considered. A lower coding rate could be achieved with less DMRS overhead and potentially provide better performance. |
| CATT | We share the same views that 1 DMRS for 3 km/h is sufficient. As mentioned in GTW meeting, we don’t think we need to enable retransmission and repetition simultaneously, especially considering we have assumed 8 repetitions and the HARQ gain has been considering in the link budget template. |
| Samsung | Regarding the repetition for PUSCH, there are two repetition types for PUSCH such as PUSCH repetition type A and PUSCH repetition type B. For clarification, we suggest to only consider the PUSCH repetition type B. PUSCH repetition type B has better performance than type A in TDD scenario and almost same performance with type A in FDD scenario. To this end, we suggest to add the clarification followed as: “For VoIP, the maximum number of nominal repetitions is 8 for PUSCH repetition type B.” Regarding the number of DMRS symbol, we also suggest to consider 1 DMRS symbol for 3km/h without frequency hopping. In our understanding, 1 DMRS symbol is sufficient to estimate the channel information in case of 3km/h. |
| Qualcomm | We want to make sure a minimum of 2 DMRS symbols are included for all UE velocities. Cell-edge UEs operate at extremely low SINRs, uplink channel estimation is challenging independent of doppler. Also cell-edge UEs tend to have larger residual frequency errors due to poor downlink SINRs, in such scenarios, having 2 DMRS symbols provides gNB with some ability to account for such issues. Can we also evaluate CP-OFDM? Field logs indicate cell-edge UEs being configured with CP-OFDM in some NR deployments. This is also useful to have when considering potential enhancements, where applicability to both waveforms may need to be studied. |
| Ericsson | LLS of HARQ is needed for accurate HARQ gains. We would like to know how proponents will generate HARQ gain values for different speeds, channel conditions, and bandwidths, etc. HARQ should be used for eMBB. It is counter-intuitive to allow it for VoIP but not eMBB. Repetition should also be allowed for eMBB. At least 2 DMRS symbols are needed. Setups for CSI on PUSCH and Msg3 should be agreed as well; see below Msg3 PUSCH: PUSCH with 7 bytes payload, MCS 0, 2 PRBs, 3 DMRS symbols 11 symbols with PUSCH, With 7 re-transmissions (8 attempts), using different frequency for different attempts. No PDCCH errors. 1% and 10% error rate CSI on PUCCH: 5+2 bits for wideband CSI feedback for 2Tx 1 PRB, no HARQ ACK/NACKs PUCCH format 3 with 4 DMRS and frequency hopping PUSCH without multiplexing with data on PUSCH and no frequency hopping 1% and 10% error rate |
| NTT DOCOMO | We also support to use one symbol DMRS for at least low mobile speed. And also following setting may be clarified for common understanding. - PUSCH mapping type (as indicated by Intel) - Additional DM-RS symbol positions (It seems pos0 (no additional DMRS symbols). We prefer pos1 for two symbols DMRS and pos3 for one symbol DMRS, but we can fine for pos0.) |
| InterDigital | We prefer to have 2 DMRS symbols even at 3km/h. Our evaluation results suggest that 2 DMRS symbols in a slot provides the best tradeoff between throughput and BLER performance. As presented in the contribution, we also support PUSCH mapping type B. Corresponding DMRS positions are shown in our contribution for FR1. |
| IITH, CEWIT, IITM, Tejas Networks, Reliance Jio | We have always been saying that if repetitions and retx are not allowed for eMBB, then appropriate code rate tables must be specified. This is missing. |
| Huawei, HiSilicon | OK with the proposal. |

Proposal 5a:

- For eMBB, HARQ gain is included in the link budget template.

- For VoIP, HARQ gain is included in LLS.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Nokia | Ok |
| ZTE | Support |
| CATT | For VoIP, we don’t think we need to consider the HARQ gain in LLS. Considering K=8, the residual BLER will be very low. |
| Samsung | For clarification, second bullet means that HARQ retransmission is considered in LLS for VoIP. If it is right, we support proposal 5a. |
| Qualcomm | Need additional clarification here. In proposal 5, we are agreeing to study iBLER for eMBB. If so what does it mean to include HARQ gains in the link budget template? If we agree to study iBLER, shouldn’t we leave out HARQ gains? |
| Ericsson | LLS of HARQ is needed for accurate HARQ gains. We would like to know how proponents will generate HARQ gain values for different speeds, channel conditions, and bandwidths, etc. |
| NTT DOCOMO | We are fine with the proposal, on the other hand, we just wonder why HARQ gain for eMBB is included in the link budget template, although that for VoIP will be evaluated in LSS. |
| InterDigital | We would like to ask clarification for the proposal, “For VoIP, HARQ gain is included in LLS.” Does this mean we agree to simulate HARQ for VoIP? If so, we agree with the proposal for VoIP. For VoIP, we would like to have both HARQ and repetition as baseline. For eMBB we propose to have options on enabling/disabling HARQ in LLS. |
| IITH, CEWIT, IITM, Tejas Networks, Reliance Jio | Same views as Qualcomm. |
| Huawei, HiSilicon | OK with the proposal. |

Proposal 6:

- For link level simulation, adopt the following table for PUCCH for FR1.

| Parameters | Values |
| --- | --- |
| PUCCH format type | Format 1, 2bits UCI Format 3, [4]/11/22 bits UCI |
| BLER for PUCCH | For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%, ACK missed detection probability: 1%. For PUCCH format 3: Block error probability: 1% |
| Number of PRBs for PUCCH | 1 PRB |
| Number of UE antennas transmit chains for PUCCH | 1 |
| Number of UE TRXUs for PUCCH | 1 |
| Number of repetitions for PUCCH | w/ and w/o repetition for PUCCH. The maximum number of repetitions can be 2/4/ is 8. No repetition for PUCCH. |
| PUCCH duration | 14 OS |

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Nokia | Ok |
| ZTE | Support |
| CATT | Don’t see the motivation to simulate 4 bits for PF#3 as larger payload size is already supported. |
| Samsung | In our understanding, repetition is the most obvious method to enhance the coverage especially for PUCCH. We are not sure why we did not consider PUCCH repetition in coverage enhancement SI. |
| Qualcomm | We are okay with this. |
| Ericsson | [4] bits should be clarified to be Ack/Nack + SR. 11/12 is CSI. BLER for CSI should be 10%; 1% can be optional Agree with Samsung. PUCCH repetition should be simulated: CSI is likely a bottleneck |
| IITH, CEWIT, IITM, Tejas Networks, Reliance Jio | Agree with Samsung and Ericsson. Repetitions have to be considered for PUCCH. |
| Huawei, HiSilicon | OK with the proposal. |

Proposal 7:

- For link level simulation, adopt the following table for PUSCH and PUCCH for FR1.

| Parameters | Values |
| --- | --- |
| Number of receive antenna elements for BS | Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (2,4,2,1,1) 32 antenna elements for 2GHz and 700MHz (M,N,P,Mg,Ng) = (8,2,2,1,1) |
| Number of receive TxRUs for BS | Urban: 2/8/64 TxRUs for 4GHz and 2.6GHz, Rural: 2/4/8 TxRUs 8 TxRUs for 4GHz and 2.6GHz, and 4TxRUs for 2GHz and 700MHz. TDL: 2 or 4 TxRUs [CDL: urban: 64TxRUs, rural: 8 TxRUs for 4GHz and 2.6GHz, and 4TxRUs for 2GHz and 700MHz.] |
| Delay spread | Urban: 300ns, [240ns] Rural: 300ns Rural with long distance: 30ns |
| Latency requirements for voice | 50ms/100ms |
| PRBs/TBS/MCS for eMBB for PUSCH | Reported by companies. [30PRBs] for 1Mbps, [4 PRBs] for 100kbps. Other values of PRBs can be reported by companies. TBS and MCS can be calculated based on the number of PRBS, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP for PUSCH | [4 PRBs] for VoIP. QPSK |

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Nokia | Are the numbers between [] simple reference values or mandated values to study? Concerning TxRUs, we think that the starting point should be the differentiation between macro-cell DL and UL power limitations. What happens in actual macro-cells, at gNB, is that EIRP is (almost) always at the maximum allowable level (small fluctuations may occur, of course). In other words, in macro cell downlink the beam-based architecture certainly serves a UE multiplexing purpose (with corresponding interference reduction), however modeling its coverage enhancement impact (if by coverage we mean range) is more difficult due to EIRP limitations. In other words, irrespective of the choice of large or narrow beams, gNB must always comply with EIRP regulations and this implicitly sets the limit of how narrow DL beams you can transmit with the same output power. In our view, this complicates the modelling we are discussing here. UL situation is different, and so is indoor DL at FR2, since power limitations at the transmitter have an important impact and then different beamforming assumptions at the receive end have big impact. In this sense, from our perspective the problem is related to the definition of TxRUs and antenna elements. Shouldn’t we first agree on these definitions to ensure that companies’ understanding of the terminology is fully aligned? Discussion on how to model the gains may be much easier then. |
| Intel | We still think it would be more appropriate to align the TBS/MCS/number of PRBs among companies for the simulations. In our simulations, we observed quite different results if different combinations are chosen. To conduct meaningful study, it is good to conclude at least a set of parameters (TBS/MCS/number of PRBs) that all companies can agree. |
| ZTE | Our understanding is, there could be two ways to modeling the overall beamforming gain for LLS based methodology: Alt 1-1: Assuming larger # of TxRU while smaller # of antennas per TxRU, e.g. 64 TxRU and 3 antennas per TxRU. That is, we would have finer digital beamforming while a coarser analog beamforming. Alt 1-2: Assuming smaller # of TxRU while larger # of antennas per TxRU, e.g. 2 TxRU and 96 antennas per TxRU. That is, we would have coarser digital beamforming while a finer analog beamforming. Digital beamforming can be evaluated by enabling precoding matrix based on SRS. For analog beamforming gain, it is ok to use ideal analog beamforming gain for Alt 1-1 since anyway the deviation would be not much in case of using smaller # of antennas per TxRU. While for Alt 1-2, assuming ideal analog beamforming gain could be not that accurate. In addition, CDL is mainly used for evaluating analog beamforming gain, in other words, Alt 1-1 which is mainly for digital beamforming, should not limit to CDL channel. So, in this sense, we think we should keep 64 TxRU for TDL channel in urban scenario. But if we are minority, it's fine for us to make it as optional. |
| CATT | We don’t think the first two rows are needed to be captured here. It should be noted it is highly dependent with proposal 8-a. Secondly, even if CDL is used, the array gain could also be calculated in the template as the immense antenna configuration will significantly lower the simulation. |
| Samsung | Regarding the antenna configuration, we support this proposal. Regarding # of PRBs/TBS/MCS, we are fine to remove the bracket for both eMBB and VoIP. If there is no specific suggestion from other companies, it can be a starting point. |
| Qualcomm | On antenna elements for 4 GHz: Why are you reducing the number of antenna elements for rural scenarios? I am assuming that rural scenarios have better cell sites to accommodate larger antenna arrays compared to urban scenarios? If so shouldn’t we have more antenna elements for rural scenarios than urban scenarios? To keep things simple, I think it is best to assume a same panel for both urban and rural in this band. No need to consider a 64 antenna panel, 192 suffices for both scenarios in this band. On TXRUs for 4 GHz: ZTE lays out the argument on including 64 TXRUs in a pretty clear manner, so I wont repeat similar arguments. We think it is important to include 64 TXRUs for both TDL and CDL. We believe typical deployments in the 4 GHz band will have gNBs with a large number of TXRUs. We will need this to support MU-MIMO via spatial multiplexing with 12 layers or even more. If this reflects the ground reality, it becomes all the more important to evaluate the case with 64 TXRUs. Operator input on their deployments and gNB vendor comments on commercial products can act as an important guideline. CMCC (noticed their comment in an earlier email discussion) and ZTE both seem to indicate support for such configurations. We want there to exist at least one commercial product for any final configuration we assume. Else this just becomes an academic exercise. On antenna elements and TXRUs for 700 Mhz: I don’t think there is any deployment where we have 32 antennas in this band. 32 antennas in this band implies a 4x4 panel with dual-pol antennas. Considering that half-wavelength is around 0.5m, such an antenna panel would approximately be 3 meters x 3 meters in size. This is way too large to be practical. Wind loading would be a big issue, finding cell sites that accommodate such large panels would be extremely challenging. I think no more than 4 antennas ( 2 passive dual-pol antennas) are typical in these bands. There is a one-to-one mapping between antennas and TXRUs. If all of this is not convincing, we can check with at least one operator with spectrum in the sub-GHz band on what is the appropriate configuration. On PRB/TBS/MCS: Can we agree to at least the numbers in the brackets to represent a baseline configuration that we can all use as a quick calibration checkpoint? Having a config for calibration is useful I think. We all have made 3 dB accounting errors at some point in our careers. Companies should aim to find other settings that yield better performance, but as a simple baseline can we agree to a set of numbers proposed by the moderator? |
| Ericsson | In our view, (M,N,P,Mg,Ng) = (12,8,2,1,1) is a little large, and (M,N,P,Mg,Ng) = (8,8,2,1,1) is more likely at least in Europe and US. This may not affect bottlenecks, though. We should correct that 2 or 4 gNB antennas are used in the LLS with TDL based LLS, not 2 or 4 TXRUs at 4 GHz. A note like ‘Note: At least for TDL models, link simulations may use 2 or 4 gNB Rx with suitable values of effective antenna and array gain’ needs to be added. The number of TXRUs can be 32 in our view for 4 GHz, with 4 mandatory and 2 optional for 700 MHz, but companies should comment. Other factors may be used to determine MCS and PRBs, so we at least need an e.g. in the following: TBS and MCS can be calculated based on e.g. the number of PRBS, target data rate, frame structure and overhead. Moreover, we think the target SINR for a given BLER requirement and TBS should be used to determine the PRBs and MCS, so our first preference is that other wording is used or to just leave this detail to proponents. |
| InterDigital | Considering that there are contributions which evaluated coverage performance by defining # of RX and TX antennas, we need to specify the number of antennas that can be assumed in LLS. From our perspective, if we choose Option 1 in Proposal 8a, we can simply specify the number of RX at BS here. For that reason, we support to specify at least 2RX and 4RX at BS for TDL. We prefer to keep the values of PRB in square brackets. Another question for clarification is whether we are mandating QPSK as baseline? Another option for modulation is pi/2 BPSK. |
| IITH, CEWIT, IITM, Tejas Networks, Reliance Jio | Since 30kbps was agreed for proposal 1, we request to add 1 PRB evaluation scenario to capture this agreement in proposal 1. |
| Huawei, HiSilicon | OK. Better to remove CDL for FR1 |

Proposal 10:

- For link level simulation, adopt the following table for PDSCH for FR1.

For PDSCH, most simulation assumptions for PUSCH can be reused except for the following parameters.

- Waveform

- target data rate

- PRBs/MCS/TBS

- frequency hopping

- number of symbols

- other parameters

| Parameters | Values |
| --- | --- |
| Waveform | CP-OFDM |
| PRBs/MCS/TBS | Reported by companies. |
| PDSCH duration | 12 OS |
| Other parameters | FFS: localized/distributed mapping |

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Nokia | Ok. |
| ZTE | Support |
| CATT | A baseline PRBs/MCS/TBS should be defined as PUSCH, and the others can be reported by companies. |
| Samsung | We support the proposal. |
| Qualcomm | Can we mandate full BW allocation in downlink? Are considerations for msg4 (broadcast PDSCH) included under this proposal? It looks like msg 4 can become a bottleneck under certain scenarios. Note that msg 4 payloads can be as large as 3000 bits. When considering VoIP traffic, Softbank notes that issues with msg 4 have been observed in the field. This is very valuable information, and we need to follow up on this. |
| Ericsson | Parameters are also needed for Msg2, and perhaps Msg4. Msg2: PDSCH with 8 bytes payload, MCS 0 with transport block scale factor 0.25, 12 PRBs, 3 DMRS symbol, 9 symbols with PDSCH (and 2 symbols reserved for PDCCH) precoder cycling 1% and 10% error rate ‘Normal’ PDSCH: 4 GHz Link and rank adaption based on 20 slot wideband CSI feedback periodicity and HARQ with up to three retransmissions. 273 PRBs, 2 symbols with DMRS, PDSCH and DMRS mapped to overhead due to CSI-RS and TRS with 20ms period ‘Normal’ PDSCH: 700 MHz with 10 MHz bandwidth Link and rank adaption based on 20 slot wideband CSI feedback periodicity and HARQ with up to three retransmissions. 52 PRBs, 2 symbols with DMRS, PDSCH and DMRS mapped to overhead due to CSI-RS and TRS with 20ms period |
| InterDigital | We should agree on the number of DMRS for PDSCH. |
| Huawei, HiSilicon | OK |

Proposal 11:

- For link level simulation, adopt the following table for PDCCH for FR1.

| Parameters | Values |
| --- | --- |
| aggregation level | 8/16 |
| payload | 40 bits/ 24 bits |
| CORESET size | 2 symbols, 48 PRBs |
| CCE-to-REG mapping type | interleaved or non-interleaved mapping |
| Number of SSB beams | FFS |

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Nokia | What is the purpose of having the number of SSB beams in this table? |
| Intel | It is also unclear to us the number of SSBs. We would assume this is for broadcast PDCCH. It may be good to clarify. |
| ZTE | As commented, the # of RBs of the CORESET size is not needed. If it is set to 48 RBs, there is no difference between interleaved or non-interleaved mapping. If companies still prefer to have it. It could be 48RBs for non-interleaved mapping and 96RBs for interleaved mapping. |
| CATT | Same questions as Nokia. And same comments on the mapping type as ZTE. |
| Samsung | As commented in ZTE, if the CORESET size is 48 PRBs and the aggregation level is 16, there is no difference in CCE-to-REG mapping type. In case of 10MHz BW for FDD, the maximum number of PRBs is 52 with 15kHz SCS. To tackle this issue, it could leave the aggregation level 8. |
| Qualcomm | We prefer to explicitly include ’48 PRBs’. It reduces the number of combinations to evaluate. Also, as Nokia commented above, lets leave out SSB beams. Modelling broadcast beams requires a separate proposal and this issue has been brought up by Vivo and us even in earlier email discussions. |
| Ericsson | We propose 4 SSB beams for good coverage. Precoder cycling should be used with the non-interleaved mapping. Same comment as ZTE. |
| NTT DOCOMO | We also have the same question as NOKIA. |

Proposal 12:

- For link level simulation, adopt following TBS for Msg3 for FR1

- 56 bits

- [144bits]

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Nokia | Ok |
| ZTE | Fine with the proposal. |
| CATT | OK |
| Samsung | We support the proposal. |
| Qualcomm | We support the proposal. |
| Ericsson | Agree a 56 bit TBS is enough for Msg3 simulations |
| Huawei, HiSilicon | OK |