3GPP TSG RAN WG1 #101-e			R1-2004915

e-Meeting, May 25th – June 6th, 2020

Title:	LS to RAN2 on NR-U RSSI Measurement Duration

Release:	Rel-16

Work Item:	NR_unlic-core

Source:	RAN WG1

To:	RAN WG2

Cc:	RAN WG4

Contact Person:

Name:	Amitav Mukherjee

E-mail Address:	Amitav_dot_Mukherjee_at_charter_dot_com

1. Overall Description:

During RAN1#101-e, RAN WG1 has made the following agreement related to configuration of RSSI measurement duration for NR-U:

Agreement:

The set of values for the parameter measDuration-r16 is {sym1, sym14or12, sym28or24, sym42or36, sym70or60} which is in units of the reference numerology configured by ref-SCS-CP-r16

- “sym14or12” refers to 14 symbols of the reference numerology for NCP and 12 symbols for ECP, respectively, and so on

- Notes (Can be captured in specifications as needed):

  - The UE derives the RSSI measurement duration from a combination of measDuration-r16 and ref-SCS-CP-r16

  - At least for RSSI measurement confined within the active DL BWP, UE performs RSSI measurement using the numerology of the active DL bandwidth part during the derived measurement duration. Otherwise, the numerology used by the UE for measurements is up to UE implementation.

  - For RSSI measurements within the active DL BWP, the UE does not expect a non-integer number of symbol(s) with respect to the numerology of the active DL BWP.

- Inform RAN2 of this decision and cc RAN4

2. Actions:

To RAN WG2.

ACTION: RAN WG1 asks RAN WG2 to kindly take the above agreement into account for developing their specifications.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-WG1 Meeting #102-e		17th – 28th Aug 2020		Online

TSG-WG1 Meeting #102-bis-e		26th Oct – 13th Nov 2020		Online