3GPP TSG-RAN WG1 Meeting #101	       R1-2004965

e-Meeting, May 25th – June 5th, 2020

Title:	LS on UE capability on wideband carrier operation for NR-U

Release:	Rel-16

Work Item:	NR_unlic-core

Source:	RAN WG 1

To:	RAN WG 4, RAN WG2

Cc:

Contact Person:

#### Name:	Chiou-Wei Tsai

E-mail Address:	cw.tsai@mediatek.com

1. Overall Description:

RAN1 is discussing NR-U UE feature on wideband carrier operations about whether to add UE capabilities for Mode 1, Mode 2 and Mode 3 from [1].

The following DL wideband operation cases are discussed.

- DL Case 1: Intra-band CA

- DL Case 2: Wideband carrier operation Modes 2/3 without scheduling intra-cell guard bands

  - DL Case 2a: Mode 2 where single wideband carrier when LBT is successful in a subset of the LBT sub-bands which are contiguous [1]

  - DL Case 2b: Mode 3 where single wideband carrier when LBT is successful in a subset of the LBT sub-bands which are non-contiguous [1]

- DL Case 3: Wideband carrier operation Modes 2/3 with scheduling intra-cell guard bands between transmitted contiguous LBT sub-bands

- DL Case 4: Wideband carrier operation Mode 1 where single carrier wideband operation when LBT is successful in all LBT sub-bands [1]

For DL operation, RAN1 would like to ask RAN4 for feedbacks about the following questions.

- Question 1: Is there any difference in DL reception among DL Cases 1, 2a, 2b, 2, and 3 with respect to AGC when at least one of the sub-bands of a [BW or carrier] is not part of gNB’s acquired channel occupancy and contains interference from devices other than the UE’s serving gNB e.g. near-by WiFi AP? Does RAN4 think AGC issues may prevent UE to meet RAN4 requirements for Mode 2 and Mode 3?

- Question 2a: Is there a difference in UE capability between any of DL Cases 2a/2b and DL Case 3?

- Question 2b: Is there a difference in UE capability between any of DL Cases 2a/2b/3 and DL Case 4?

- Question 2c: Is there a difference in UE capability between any of DL Cases 2a/2b/3/4 and DL Case 1?

- Question 3: From RAN4 point of view, does “all LBT sub-bands” for Mode 1 refer to LBT sub-bands of configured carrier or BWP?

The following UL wideband operation cases are discussed.

- UL Case 1: UL wideband operation Mode 2A (UL-WB Mode 2A) where UE transmits if LBT passes for single scheduled LBT sub-band

- UL Case 2: UL wideband operation Mode 2B (UL-WB Mode 2B) where UE transmits if LBT passes for scheduled multiple contiguous LBT sub-bands

- UL Case 3: UL wideband operation Mode 1 (UL-WB Mode 1) where UE transmits only if LBT passes for all LBT sub-bands of BWP

For UL operation, RAN1 would like to ask RAN4 for feedbacks about the following questions.

- Question 4: Is change of transmit filtering required (as shown in Figure 1 for WB Mode 2B) to meet RAN4 requirements for any of UL Cases 1-3?

- Question 5: Is there any difference if intra-cell GBs between scheduled contiguous sub-bands are scheduled or not?

Finally, if the answer to any of Questions 2a/2b/2c/4/5 is yes and capabilities for any of the cases are deemed needed, RAN1 would like to request RAN4 to define the corresponding UE capabilities.

Figure 1: An example for Mode 2B

2. Actions:

To RAN WG 4 group

ACTION: 	RAN WG1 respectfully asks RAN WG4 for a response to the above questions and requests RAN4 to define the corresponding UE capabilities if the answer to any of Questions 2a/2b/2c/4/5 is yes and capabilities for any of the cases are deemed needed.

To RAN WG 2 group

ACTION: 	RAN WG1 respectfully requests RAN WG2 to reserve capability signalling bits for indication of Mode 1/2/3 for DL and Mode 1/2A/2B for UL as a precaution in case the capability discussion in RAN4 cannot be concluded by the end of RAN4#95-e. It is RAN WG1's understanding that the bits can be kept or reused as needed after the June deadline.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #102	17 – 28 August 2020	e-Meeting

4. Reference

[1]	R4-1905209, “LS reply on wideband carrier operation for NR-U,” RAN4 #90bis, April 2019