3GPP TSG-RAN WG1 #102-e	R1-2005313

e-Meeting, August 17th – 28th, 2020

Title:	Study Cases for System-Level Simulations in NTN WI

Source:	Nomor Research GmbH, Thales

Type:	Discussion

Document for:	Decision

Agenda Item: 	8.4.4 Others

Work Item: 	NR_NTN_Solutions: Solutions for NR to support non-terrestrial networks (NTN)

# Introduction

At RAN#86 meeting in Sitges, Spain, a new WI “Solutions for NR to support non-terrestrial networks (NTN)” [1] was agreed with the aim to specify the enhancements identified for NR NTN. It has been updated in RAN#88 e-meeting [3].

This document analyses the list of simulation study cases, used during the Study Item phase and tries to reduce the large set for system level simulations (SLS) during the Work Item phase.

# Study Cases for NTN System-Level Simulations

During Study Item phase a set of 30 study cases have been determined differentiating between satellite orbit, satellite parameter set, central beam elevation, type of terminal, frequency band and usage of frequency/polarization reuse, see Table 1 or Table 6.1.1.1-9 of [2]. Simulation parameters have been captured in section 6.1.1.1 of [2]. The baseline of the metrics for performance is defined as “UE throughput (5%, 50%, 95%) at 20% and [50 or 60]% RU”, see Table 6.1.1.1-7 of [2].

In this contribution, we consider for LEO-1200, S-Band scenario with satellite parameter set “Set 1” (Study Case 14 and 15) the following resource usage settings:

- 20% resource utilization, Frequency/Polarization reuse factor = (1/1) (Option 1)

- 20% resource utilization, Frequency/Polarization reuse factor = (3/1) (Option 2)

- 100% resource utilization, Frequency/Polarization reuse factor = (1/1) (Option 1)

- 100% resource utilization, Frequency/Polarization reuse factor = (3/1) (Option 2)

Further parameter settings applied for the SLS are listed in Table 2. It is worth to mention that in case of full frequency reuse (FFR, frequency reuse factor (FRF) equal to one), each beam uses a bandwidth of 30MHz, while in case of FRF=3, each beam uses a bandwidth of 10MHz as the bandwidth is divided into the three neighbouring beams to decrease interference.

Table 1 List of calibration study cases [2]

| Case | Satellite orbit | Satellite parameter set | Central beam elevation | Terminal | Frequency Band | Frequency/ Polarization Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | Set 1 | 45 deg | VSAT | Ka-band | Option 1 |
| 2 | GEO | Set 1 | 45 deg | VSAT | Ka-band | Option 2 |
| 3* | GEO | Set 1 | 45 deg | VSAT | Ka-band | Option 3 |
| 4* | GEO | Set 1 | 45 deg | Handheld | S-band | Option 1 |
| 5* | GEO | Set 1 | 45 deg | Handheld | S-band | Option 2 |
| 6 | LEO-600 | Set 1 | 90 deg | VSAT | Ka-band | Option 1 |
| 7 | LEO-600 | Set 1 | 90 deg | VSAT | Ka-band | Option 2 |
| 8* | LEO-600 | Set 1 | 90 deg | VSAT | Ka-band | Option 3 |
| 9 | LEO-600 | Set 1 | 90 deg | Handheld | S-band | Option 1 |
| 10 | LEO-600 | Set 1 | 90 deg | Handheld | S-band | Option 2 |
| 11* | LEO-1200 | Set 1 | 90 deg | VSAT | Ka-band | Option 1 |
| 12* | LEO-1200 | Set 1 | 90 deg | VSAT | Ka-band | Option 2 |
| 13* | LEO-1200 | Set 1 | 90 deg | VSAT | Ka-band | Option 3 |
| 14 | LEO-1200 | Set 1 | 90 deg | Handheld | S-band | Option 1 |
| 15 | LEO-1200 | Set 1 | 90 deg | Handheld | S-band | Option 2 |
| 16** | GEO | Set 2 | 45 deg | VSAT | Ka-band | Option 1 |
| 17** | GEO | Set 2 | 45 deg | VSAT | Ka-band | Option 2 |
| 18** | GEO | Set 2 | 45 deg | VSAT | Ka-band | Option 3 |
| 19** | GEO | Set 2 | 45 deg | Handheld | S-band | Option 1 |
| 20** | GEO | Set 2 | 45 deg | Handheld | S-band | Option 2 |
| 21** | LEO-600 | Set 2 | 90 deg | VSAT | Ka-band | Option 1 |
| 22** | LEO-600 | Set 2 | 90 deg | VSAT | Ka-band | Option 2 |
| 23** | LEO-600 | Set 2 | 90 deg | VSAT | Ka-band | Option 3 |
| 24** | LEO-600 | Set 2 | 90 deg | Handheld | S-band | Option 1 |
| 25** | LEO-600 | Set 2 | 90 deg | Handheld | S-band | Option 2 |
| 26** | LEO-1200 | Set 2 | 90 deg | VSAT | Ka-band | Option 1 |
| 27** | LEO-1200 | Set 2 | 90 deg | VSAT | Ka-band | Option 2 |
| 28** | LEO-1200 | Set 2 | 90 deg | VSAT | Ka-band | Option 3 |
| 29** | LEO-1200 | Set 2 | 90 deg | Handheld | S-band | Option 1 |
| 30** | LEO-1200 | Set 2 | 90 deg | Handheld | S-band | Option 2 |
| NOTE 1: no star = 1st priority, * = second priority scenario, ** = third priority scenario NOTE 2: Only 1st priority cases will be considered for calibration phase 1 |  |  |  |  |  |  |

Table 2 Parameter Settings for SLS

| Scenario | LEO-1200, S-band |
| --- | --- |
| Duplexing | FDD |
| Transmission direction | Downlink |
| Numerology | 15kHz, 14 OFDM symbol slot |
| System bandwidth | 30MHz |
| Resource Utilization (RU) | 20%, 100% |
| # of UEs per cell | 15 |
| User deployment scenario | Rural |
| LoS probability | Table 6.6.1-1 in TR 38.811 |
| Propagation delay (one way) | 16ms |
| Scheduling | Proportional Fair |
| Handover margin (dB) | 3 |
| UE attachment | Based on RSRP |

Figure 1 CDF of UE Throughput

Table 3 UE Throughput LEO-1200, S-band

| RU | FRF | 5% TPuser [Mbit/s] | 50% TPuser [Mbit/s] | 95% TPuser [Mbit/s] |
| --- | --- | --- | --- | --- |
| 20% | 1 | 0.16 | 0.27 | 0.40 |
| 20% | 3 | 0.18 | 0.26 | 0.29 |
| 100% | 1 | 0.29 | 0.66 | 1.10 |
| 100% | 3 | 0.35 | 0.76 | 0.88 |

Figure 1 presents the SLS results and Table 3 presents the 5%, 50% and 95%-tile of the UE throughput for the above discussed options of resource usage. The following observations can be made:

Observation 1: 	Considering a LEO-1200 S-Band scenario, a higher UE throughput can be achieved using a 100% RU instead of a 20% RU.

Observation 2: 	Considering a LEO-1200 S-Band scenario with 100% RU, the majority of the UEs show a higher throughput for the case with FRF=3 than for the case with FRF=1. Around 17% of the best users have better performance in case of FRF =1.

Although the used bandwidth per beam is 10MHz for FRF=3, while it is 30MHz for FRF=1, a higher UE throughput can be observed in case of FRF=3 except for the UEs with highest throughput for a RU of 100%. This results from the high interference limitation in single satellite systems where the interfering signal has the same path loss as the serving signal.

Observation 3:	Considering a LEO-1200 S-Band scenario with 100% RU and a FRF=1, the difference between the worst and best UEs is significant (5%-tile: 0.29Mbit/s and 95%-tile: 1.10Mbit/s, which is a factor of 3.8).

The high difference in terms of UE throughput for the scenario with 100% RU and FRF=1 is again an indication on the interference limitation. Although a proportional fair scheduler is applied, the system cannot achieve a similar throughput for all UEs.

Based on the high interference level in case of FRF=1, from our perspective it is reasonable to evaluate scenarios with FRF=3 because other effects will be more visible and can be interpreted more reliable.

Applying FRF=3 means that the PRBs of the same frequency are only used in each third beam. By this, the interference level is already decreased, and simulating with a RU of 100% makes sense for such a configuration. Of course, this conclusion may only hold for loaded systems. In case certain beams are lowly loaded, other beams could benefit from using the full 30 MHz instead of a 10 MHz portion only. Due to the large coverage area of satellite beams, we think a fully loaded system is a reasonable assumption for performance evaluation. If a fully loaded system is not reasonable, we should question if full buffer traffic and equal user distribution are a valid assumption. Therefore, we propose to use a frequency reuse factor of 3 and a resource utilization of 100% for the SLSs performed during Work Item Phase.

Proposal 1: 	Consider 100% RU and FRF=3 as prioritized scenario for SLS in this Work Item for LEO-1200 S-band scenario.

Without further simulation results, we propose the same configuration for all other satellite orbit scenarios.

Proposal 2:	Consider following study cases for system level simulations during the Work Item.

Table 3 List of study cases for SLS in WI phase

| Case | Satellite orbit | Satellite parameter set | Central beam elevation | Terminal | Frequency Band | Frequency/ Polarization Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | Set 1 | 45 deg | VSAT | Ka-band | Option 2 |
| 2* | GEO | Set 1 | 45 deg | Handheld | S-band | Option 2 |
| 3 | LEO-600 | Set 1 | 90 deg | VSAT | Ka-band | Option 2 |
| 4 | LEO-600 | Set 1 | 90 deg | Handheld | S-band | Option 2 |
| 5* | LEO-1200 | Set 1 | 90 deg | VSAT | Ka-band | Option 2 |
| 6 | LEO-1200 | Set 1 | 90 deg | Handheld | S-band | Option 2 |
| NOTE 1: no star = 1st priority, * = second priority scenario |  |  |  |  |  |  |

# Conclusions

In this document the study cases determined for SLS calibration during Study Item phase have been analysed. The following observations and proposals are made:

Observation 1: 	Considering a LEO-1200 S-Band scenario, a higher UE throughput can be achieved using a 100% RU instead of a 20% RU.

Observation 2: 	Considering a LEO-1200 S-Band scenario with 100% RU, the majority of the UEs show a higher throughput for the case with FRF=3 than for the case with FRF=1. Around 17% of the best users have better performance in case of FRF =1.

Observation 3:	Considering a LEO-1200 S-Band scenario with 100% RU and a FRF=1, the difference between the worst and best UEs is significant (5%-tile: 0.29Mbit/s and 95%-tile: 1.10Mbit/s, which is a factor of 3.8).

Proposal 1: 	Consider 100% RU and FRF=3 as prioritized scenario for SLS in this Work Item for LEO-1200 S-band scenario.

Proposal 2:	Consider following study cases for system level simulations during the Work Item.

# References

- RP-193234, “Solutions for NR to support non-terrestrial networks (NTN)”, Sitges, Spain, 3GPP TSG RAN meeting #86, Thales

- 3GPP TR 38.821 V16.0.0, “Solutions for NR to support non-terrestrial networks (NTN) (Release 16)”

- RP-201256, “Solutions for NR to support non-terrestrial networks (NTN)”, e-meeting, 3GPP TSG RAN meeting #88-e, Thales