3GPP TSG RAN WG1 #102-e			R1-2005945

e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.2.11

Source:	AT&T

Title:	Remaining details of UE features for IAB

Document for:	Discussion/Approval

# Introduction

This contribution discusses the remaining details of support of Rel-16 UE features by IAB-MTs.

# Mandatory IAB-MT Features

During RAN#88e the following agreements were reached:

T1-P1: RAN1&4 take into account the signaling framework used by RAN2 and finalize the IAB-MT feature list.

T1-P2: Local-area IAB-MTs support the same mandatory UE features (as already agreed) for wide-area IAB-MTs.

T2-P2’’: The following Rel-15 Layer-2 and Layer-3 UE Feature is mandatory with capability signaling for wide-area and local-area IAB-MTs (it is up to the IAB node to set the capability bit, and the feature will not be captured into the minimum set table specified in TS 38.306):

4-1          Intra-NR measurements and reports

The following Rel-15 Layer-2 and Layer-3 UE Feature is optional with capability signaling for wide-area and local-area IAB-MTs:

7-1          Handover:

1) Intra-frequency HO

2) Inter-frequency HO

T2-P3: RF/RRM Rel-15 UE Features related to topology adaptation (i.e. FG 3-1/3-2/3-3) should remain optional for IAB-MTs in Rel-16.

Additionally during RAN1#102 the following features were discussed, but not conclusion was reached on whether to remove the brackets around [Devices supporting IAB backhaul must report this FG as supported]:

Given the RAN plenary agreement that Intra-NR measurements and reports are mandatory with capability signaling for wide-area and local-area IAB-MTs, it would be consistent to apply the same agreement to FG 20-2.

Proposal 1: Update the Mandatory/Optional column for FG 20-2 as the following:

Mandatory with capability signaling for wide-area and local-area IAB-MTs (it is up to the IAB node to set the capability bit. Note: the feature will not be captured into the minimum set table specified in TS 38.306

While similar benefits of providing orthogonal access and backhaul link resources for RACH Tx/Rx is envisioned for FG 20-3, given that the need for different RACH periodicities may be scenario dependent (e.g. long-range IAB links), it is acceptable in Rel-16 that this FG is left optional for wide-area and local-area IAB-MTs.

Proposal 2: The square brackets (including the text) is removed for FG 20-3.

# Conclusion

The following proposals were made:

Proposal 1: Update the Mandatory/Optional column for FG 20-2 as the following:

Mandatory with capability signaling for wide-area and local-area IAB-MTs (it is up to the IAB node to set the capability bit. Note: the feature will not be captured into the minimum set table specified in TS 38.306

Proposal 2: The square brackets (including the text) is removed for FG 20-3.