3GPP TSG RAN WG1 Meeting #102-e	R1-2007211

# E-meeting, August 17–28, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1347 | rev | - | Current version: | V16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on L1 adjustment on the NPUSCH repetition number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Huawei) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Core |  |  |  |  |  | Date: |  |  | 2020-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - For PUR, the UE uses the NPUSCH repetition number from dedicated PUR ACK DCI or RRC configuration, whichever is received latest. And the L1 adjustment on the (N)PUSCH repetition number is intended to update the higher layer (i.e. RRC) configuration. In the current spec, the NPUSCH repetition number cannot be further updated by RRC configuration, and is against the intention of “intended to update the higher layer (i.e. RRC) configuration”. - In TS36.213 section 16.5.1.1, “Subcarrier indication field” and “resource assignment field” are used. However, the corresponding higher layer parameters in PUR-Config-NB are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Replace “determined by the NPUSCH repetition adjustment field according to Table 16.5.1.1-3 from the most recent NPDCCH DCI format N0 with CRC scrambled by PUR C-RNTI with the value of "modulation and coding scheme" field () set to '14' if detected, configured by higher layers otherwise.” with “configured by higher layers” - The higher layer parameters npusch-SubCarrierSetIndex and npusch-NumRUsIndex in PUR-Config-NB have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The NPUSCH repetition number cannot be further updated by RRC configuration, and is against the intention of “intended to update the higher layer (i.e. RRC) configuration”. - The parameter names are not aligned between TS 36.213 and TS 36.331. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.5.1.1, 16.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

#### 16.5.1.1	Resource allocation

The resource allocation information in uplink DCI format N0 for NPUSCH transmission or configured by higher layers for NPUSCH transmission using preconfigured uplink resource indicates to a scheduled UE

- a set of contiguously allocated subcarriers () of a resource unit determined by the Subcarrier indication field, or by the higher layer parameter npusch-SubCarrierSetIndex in PUR-Config-NB

- a number of resource units () determined by the resource assignment field according to Table 16.5.1.1-2, or by the higher layer parameter npusch-NumRUsIndex in PUR-Config-NB

- a repetition number () determined by the repetition number field according to Table 16.5.1.1-3. For a NPUSCH transmission using preconfigured uplink resource, the UE shall use the repetition number configured by higher layers.

- < Unchanged parts are omitted >

16.6.4	Preconfigured uplink resource ACK/fallback procedure

If a UE has initiated a NPUSCH transmission using preconfigured uplink resource on a given serving cell, and upon detection of a NPDCCH with DCI format N0 with CRC scrambled by PUR C-RNTI intended for the UE within the PUR search space window as defined in Subclause 16.6, and the value of "modulation and coding scheme" field () in the corresponding DCI set to '14', the UE shall deliver the PUR ACK/fallback indication and the NPUSCH repetition adjustment, as signaled on the NPDCCH, to the higher layers.

#### < Unchanged parts are omitted >