# 3GPP TSG RAN WG1 #102-e						R1-200xxxx

# e-Meeting, August 17th – 28th, 2020

Agenda item:	6.2.4

Source: 	Moderator (Qualcomm Incorporated)

Title: 	Summary of 102-e-LTE_TerrBcast-02

Document for:	Discussion and Decision

# Issue #1: Indexing of PBCH scrambling

In x6190, it is proposed to change the indexing from k to k’ in the PBCH scrambling as follows:

| Reason for change: | The current PBCH scrambling is given by θk,l'=ejπc2k2ejπc2k+1 However, k (aboslute subcarrier index) is not known before decoding PBCH. |
| --- | --- |
| Summary of change: | PBCH repetition is scrambled based on k’ (as defined in 6.6.4), not k |
| Consequences if not approved: | PBCH repetitions cannot be used until after decoding PBCH. |

<TP2, 36.211>

<Omitted in summary>

#### 6.6.4.1	PBCH repetition in the cell acquisition subframe

<Omitted in summary>

Resource elements already reserved or used for transmission of cell-specific reference signals in absence of repetition shall not be used for additional mapping of cell-specific reference signals.

The quantity θk,l' is given by

θk,l'=ejπc2k'2ejπc2k'+1

where the relationship between k' and k is defined in clause 6.6.4, and the pseudo-random sequence ci is given by clause 7.2 and initialized for each OFDM symbol l' with

cinit=213NIDcell+1NsymbDLns'+l'+1+24NIDcell+NsymbDLns'+l'

Table 6.6.4.1-1: Slot and symbol number pair for repetition of PBCH.

| l | Slot and symbol number pair ns',l' |  |
| --- | --- | --- |
|  | Normal cyclic prefix | Extended cyclic prefix |
| 0 | (0, 4) | - |
| 1 | (1, 4) | (0, 3) |
| 2 | (1, 5) | (1, 4) |
| 3 | (0, 3), (1, 6) | (1, 5) |

</TP2 >

Comments on TP2:

| Company | Input |
| --- | --- |
| Qualcomm | Agree the TP. |
| ZTE | We are ok with the TP. |
| BBC | We agree with the TP. |

# Issue #2: Corrections for 0.37 SCS (subframe/slot)

In x6416 the following TP is proposed to discuss the relationship between slots/subframes for 0.37 kHz SCS:

<TP3, 36.211>

# 6	Downlink

## 6.1	Overview

The smallest time-frequency unit for downlink transmission is denoted a resource element and is defined in clause 6.2.2.

A subset of the downlink subframes in a radio frame can be configured as MBSFN subframes by higher layers. Each MBSFN subframe is divided into a non-MBSFN region and an MBSFN region. For MBSFN slot with Δf=182944Ts≈0.37 kHz, the duration of the slot spans three consecutive MBSFN subframes.

-	For subframes using , the non-MBSFN region spans the first one or two OFDM symbols in an MBSFN subframe where the length of the non-MBSFN region is given according to Clause 6.7.

-	For subframes using , Δf=2.5 kHz, , or slot using Δf=182944Ts≈0.37 kHz, the non-MBSFN region is of zero size.

-	The MBSFN region in an MBSFN subframe is defined as the OFDM symbols not used for the non-MBSFN region. For Δf≈0.37 kHz the MBSFN region is defined as one slot of 3 ms.

For an MBMS-dedicated cell, subframes where PSS/SSS/PBCH or PDSCH carrying system information are transmitted with  are non-MBSFN subframes.

For frame structure type 3, MBSFN configuration shall not be applied to downlink subframes in which at least one OFDM symbol is not occupied or discovery signal is transmitted.

Unless otherwise specified, transmission in each downlink subframe shall use the same cyclic prefix length as used for downlink subframe #0.

### < Unchanged parts are omitted >

### 6.10.2	MBSFN reference signals

MBSFN reference signals shall be transmitted in the MBSFN region of MBSFN subframes/slots only when the PMCH is transmitted. MBSFN reference signals are transmitted on antenna port 4.

For an MBMS-dedicated carrier configured with a single MBSFN area, and for a PMCH transmitted with 0.37 kHz subcarrier spacing in slot n, which is indicated to contain MCCH by higher layer parameter MCCH-Config:

-	for MBSFN reference signal pattern type 1, the UE may assume that MBSFN reference signals associated with the same NIDMBSFN are present in the three preceding slots to slot n.

-	for MBSFN reference signal pattern type 2, the UE may assume that MBSFN reference signals associated with the same NIDMBSFN are present in the preceding slot to slot n.

MBSFN reference signals are defined for extended cyclic prefix only.

</TP3>

Comments on TP3:

| Company | Input |
| --- | --- |
| Qualcomm | On the first change “the duration of the slot spans three consecutive MBSFN subframes”, we think this is clear from TS 36.211, 4.1: For transmissions using Δf=182944Ts≈0.37 kHz, a slot has a length of 92160Ts=3 ms. There are 13 slots, numbered in increasing order from 0 to 12, in a 40 ms period starting at nf mod 4=0 with slot 0 starting at 30720Ts in the 40 ms period. On the second change, we agree with it with a minor change (slot -> slots): For subframes using , Δf=2.5 kHz, , or slots using Δf=182944Ts≈0.37 kHz, the non-MBSFN region is of zero size. The third change looks OK. |
| ZTE | We tend to agree with Qualcomm that the first change may be not needed. We are ok with the second change (with Qualcomm’s updates) and the third change. |
| BBC | We think either option is ok, but we have a slight preference to the changes suggested by Qualcomm and ZTE. |

# Conclusion

<To be completed after discussion>